IEEE P802.11
Wireless LANs

Summary of Features Proposed for the Enhanced-DCF Wireless MAC Protocol

Date:	January 2, 2001

Author:	Mathilde Benveniste
	AT&T Labs - Research
	180 Park Avenue, Florham Park, New Jersey 07932
	Phone: 973-761-0988
	e-Mail: benveniste@att.com

Abstract

This memo summarizes the author’s perception of features appearing in the various proposals made for the 802.11 Enhanced DCF MAC protocol. A brief description is given for each feature, followed by the proposals that include it.  The features are grouped in categories specified in terms of the function they provide. The table was put together in order to contribute to the understanding and evaluation of these features for possible inclusion into the Standard.

Table 1 summarizes the author’s perception of features appearing in the various proposals made for the 802.11 Enhanced DCF MAC protocol. A brief description is given for each feature, followed by the proposals that include it.  The features are grouped into categories specified in terms of the function they provide.  The categories cover the following functions:

- Basic Contention Resolution Approach

- Class Differentiation

- Packet Classification

- Averting Packet Aging

- Scheduling of Competing Traffic Streams at a Node

- Adaptation to Traffic Intensity: Control Mechanism

- Adaptation to Traffic Intensity: Frequency

The table was put together in order to contribute to the understanding and evaluation of these features for possible inclusion into the Standard.  It is the author’s belief that the final recommendation would benefit from the inclusion of features from various proposals, to the extent of course that mutually exclusive features are not selected. We suggest that a group of compatible features be selected first, to be followed by subsequent refinement of the implementation details for each of the selected features.

With this in mind, the last column of the table assigns labels to each feature as follows:

- “IN AGREEMENT“ indicates that a feature is common to all proposals

- “NO CONFLICT” indicates feature independence; i.e., the inclusion of the feature in the final proposal would not impact the inclusion of other features

- “EXCLUSIVE” indicates that other features are impacted by the inclusion of the feature in the final proposal

Selecting features, it should be kept in mind that, while for features labeled either as “IN AGREEMENT“ or “NO CONFLICT” a simple “include” or “not include” decision would suffice, features with the “EXCLUSIVE” label require a choice between conflicting alternatives; the competing alternatives are indicated in the table.

Table 1 Possible Features for E-DCF, by Purpose Category A-D (Page 1 of 2)

| Feature Name | Description | Included in Proposals | Comment |
| --- | --- | --- | --- |
| # A. Basic Contention Resolution |  |  |  |
| A.1 Backoff | A backoff counter is drawn from a random distribution | P-DCF TCMA V-DCF | IN AGREEMENT (present in all proposals) |
| # B. Class Differentiation |  |  |  |
| B.1 By Initial Backoff Parameters | The parameters (offset, window size) of the starting backoff distribution vary by class | P-DCF TCMA V-DCF | IN AGREEMENT (present in all proposals) |
| B.2 By Arbitration Time | The waiting time to start countdown after a transmission (DIFS for legacy) varies by class | TCMA | NO CONFLICT (compatible with all other features) |
| B.3 By Retrial Backoff Parameters | The rules to update the backoff distribution following collision vary by class | TCMA | NO CONFLICT (compatible with all other features) |
| B.4 By Dwell-Time Limit | The maximum time a packet may spend attempting transmission varies by class | TCMA | NO CONFLICT (compatible with all other features) |
| # C. Packet Classification |  |  |  |
| C.1 Priority/Urgency Classes | The eight 802.1d Annex H.2 classes are mapped into N priority/urgency classes | P-DCF TCMA V-DCF | IN AGREEMENT (present in all proposals) |
| # D. Averting Packet Aging |  |  |  |
| D.1 Urgency Class Upgrade | Packets in certain classes are upgraded dynamically [as their age increases] to reflect their transmission urgency | TCMA | NO CONFLICT (compatible with all other features) |
| D.2 Packet Expiration | Packets are eliminated from queue when queueing times reach class-specific thresholds | TCMA | NO CONFLICT (compatible with all other features) |

Table 1 Possible Features for E-DCF, by Purpose Category E-G (Page 2 of 2)

| Feature Name | Description | Included in Proposals | Comment |
| --- | --- | --- | --- |
| # E. Scheduling of Competing Traffic Streams at a Node |  |  |  |
| E.1 Parallel Queues | Class specific queues are maintained at a node with own backoff countdown | V-DCF TCMA | NO CONFLICT (compatible with all other features) |
| # F. Adaptation to Traffic Intensity: Control Mechanism |  |  |  |
| F.1 Point-coordinated | A point coordinator monitors traffic and sends adjustment parameters regularly | P-DCF V-DCF | EXCLUSIVE (select none or one option in category) |
| F.2 Distributed | A node (STA or AP) updates traffic from local information as needed | TCMA | EXCLUSIVE (select none or one option in category) |
| # G. Adaptation to Traffic Intensity: Frequency |  |  |  |
| G.1 On First Backoff | Backoff process is adjusted for traffic on first backoff | P-DCF TCMA V-DCF | IN AGREEMENT (present in all proposals) |
| G.2 On Retrial Backoff | Backoff process is adjusted on retrial, based on congestion estimates | P-DCF TCMA V-DCF | NO CONFLICT (compatible with all other features) |
| G.3 Continuous –Redraw Backoff | Before backoff expires, new backoff counter is drawn regularly (based on current congestion estimates) and backoff countdown restarts | P-DCF | EXCLUSIVE (select none or one of G3 or G4) |
| G.4 Continuous – Backoff Scaling | Current backoff counter is scaled up or down if traffic intensity changes substantially | (TCMA) | EXCLUSIVE (select none or one of G3 or G4) |

Proposal offered only if continuous adaptation to traffic is deemed necessary.