IEEE P802.11
Wireless LANs

802.11a and 802.11b Maximum Throughput for Simulation Model Conformance

Date:	January 16, 2001

Author:	Sunghyun Choi, Javier Del Prado, and Matthew Sherman*

Philips Research, Briarcliff Manor
	345 Scarborough Road, Briarcliff Manor, NY 10510 USA
	Phone: 914 945-6506 
	Fax: 914 945-6580
	e-Mail: sunghyun.choi@philips.com

*AT&T Labs - Research
180 Park Ave, Florham Park, NJ 07932 USA
Phone: 973 236-6791
e-Mail: mjsherman@att.com

Abstract

In this paper, we present the maximum throughput achievable in 802.11a and 802.11b using the Distributed Coordination Function (DCF) when there are one sender and one receiver. The results via analysis and simulations are provided. The results presented can be used as a conformance test to validate different 802.11a/b simulation models.

# Introduction

IEEE 802.11 TGe simulation ad hoc group is working on the development of 802.11 WLAN models using the network simulation tools including OPNET and Network Simulator 2 (ns2). Currently, there are a number of different models being developed by different individuals even in one simulation environment, e.g., OPNET.   This paper analyzes the maximum throughput (or goodput) which can be achieved through IEEE 802.11a and .11b WLAN systems in the simplest network environment. Then, we compare the analytical results with the simulation results. The simulation models which we used include: (1) 802.11a and 802.11b models developed by Philips based on the official OPNET model; and (2) 802.11a and 802.11b models developed by Philips based on the AT&T PCF OPNET model. The former is available to public while the latter is available under the NDA with AT&T.  Both codes were run using the Distributed Coodination Function (DCF).  We believe the considered maximum throughput can be used as the conformance test for different simulation models.

# Assumption

The wireless system under consideration is composed of two wireless stations (STAs); one sender and one receiver. Here is the communication scenario:

- The sender generates L-byte long data (or MSDU) with the infinite rate, where L <= 2304 bytes.

- The sender will make an MPDU with an MSDU in the MAC, and will try to send the MPDU.

- The receiver responds with an ACK per receiving an MPDU.

Now, recall that the transmission cycle under the DCF of the IEEE 802.11 MAC consists of the following phases, which are executed repetitively:

- DIFS deferral phase

- Back-off/contention

- Data (or MPDU) transmission phase

- SIFS deferral phase

- ACK transmission phase

The related characteristics for the IEEE 802.11a and .11b PHYs are listed in Table 1.

Table 1. IEEE 802.11a and .11b PHY CHARACTERISTICS

| Parameters | .11a | .11b | Comments |
| --- | --- | --- | --- |
| AslotTime | 9usec | 20usec | slot time |
| ADIFSTime | 34usec | 50usec | DIFS time |
| ACWmin | 15 | 31 | Min contention window size in unit of aSlotTime |
| tPLCPPreamble | 16usec | 144usec | PLCP preamble duration |
| tPLCPHeader | 4usec | 48usec | PLCP header duration (except the SERVICE field in case of 802.11a) |
| tSymbol | 4usec | N/A | OFDM symbol interval |

# Analysis

In this section, we analysize the throughput of the 802.11 network assumped above.

## 802.11b

The time to transmit a data frame or MPDU with L-byte long payload at m Mbps PHY rate is given by

Tdatam(L)=tPLCPPreamble+tPCLPHeader+8∗(28+L)m∗1000000

and the time to transmit an ACK frame at n Mbps PHY rate is given by

Tackn=tPLCPPreamble+tPCLPHeader+8∗14n∗1000000

## 802.11a

For m Mbps PHY rate, the number of bytes in a symbol is given by

BpS(m)=m2

Now, the time to transmit a data frame or MPDU with L-byte long payload at m Mbps PHY rate is given by

Tdatam(L)=tPLCPPreamble+tPCLPHeader+30.75+LBpS(m)∗tSymbol

and the time to transmit an ACK frame at n Mbps PHY rate is given by

Tackn=tPLCPPreamble+tPCLPHeader+16.75BpS(n)∗tSymbol

## Throughput

Now, the throughput of the system, when the data frames are transmitted at m Mbps and the ACK frames are transmitted at n Mbps, is given by

T(m,n)=8∗LaDIFSTime+Tbk+Tdatam(L)+aSIFSTime+Tackn (bps),

where the average backoff time is given by

Tbk=CWmin2∗aSlotTime

# Results

The figure and table below shows both analytic and simulation results for both 802.11a and 802.11b networks. The two different simulation models, specified in the introduction, for both 802.11a and 802.11b resulted in almost the same results. So only one set of results is ploted for each. The transmission rates of data and ACK are:

- 802.11a: 54 Mbps for data and 24 Mbps for ACK frames

- 802.11b: 11 Mbps for data and 2 Mbps for ACK frames

The rationale for choosing the specific transmission rates for the ACK frames is that the ACK, a control frame, can be transmitted at the highest rate in the BSS basic service rate set. We assuming that the basic service rate sets for 802.11a and 802.11b are {6,12,24} (Mbps) and {1,2} (Mbps), respectively. Note that {6,12,24} (Mbps) are the mandatory rates for 802.11a PHY while {1,2} (Mbps) are the rates supported by the legacy Direct Sequence (DS) PHY.

|  | Throughput (Mbps) |  |
| --- | --- | --- |
| MPDU size L (bytes) | .11b | .11a |
| 10 | 0.09550684 | 0.461095101 |
| 50 | 0.46150619 | 2.253521127 |
| 100 | 0.88584659 | 4.312668464 |
| 200 | 1.63964971 | 7.94044665 |
| 300 | 2.28888504 | 11.03448276 |
| 400 | 2.85389979 | 13.94335512 |
| 500 | 3.35008375 | 16.29327902 |
| 600 | 3.78929238 | 18.35564054 |
| 700 | 4.1808063 | 20.18018018 |
| 800 | 4.53199434 | 22.10708117 |
| 900 | 4.84878168 | 23.56792144 |
| 1000 | 5.13598693 | 24.88335925 |
| 1100 | 5.39756886 | 26.3868066 |
| 1200 | 5.63681008 | 27.46781116 |
| 1300 | 5.85645541 | 28.45417237 |
| 1400 | 6.05881774 | 29.66887417 |
| 1500 | 6.24585975 | 30.49555273 |
| 1600 | 6.41925777 | 31.25763126 |
| 1700 | 6.58045219 | 31.96239718 |
| 1800 | 6.73068752 | 32.91428571 |
| 1900 | 6.87104463 | 33.51708931 |
| 2000 | 7.00246678 | 34.07880724 |
| 2100 | 7.12578083 | 34.89096573 |
| 2200 | 7.24171467 | 35.37688442 |
| 2300 | 7.3509116 | 35.83252191 |

# Conclusion

In this paper, we provide the maximum throughput achieved by 802.11a and 802.11b networks when there are only one sender and one receiver utilizing the Distributed Coordination Function (DCF), which can be used to help check the conformance of different simulation models.