IEEE P802.11
Wireless LANs

TGh Functional Requirements

Date:

January 16, 2001

Author:

Evan Green, Intel

Phone: (503) 264-8456
Fax: (503) 264-3483
e-Mail: evan.r.green@intel.com

# Introduction

## This document specifies the minimum set of requirements that proposals must meet upon submission to task group H for consideration.

## All requirements listed in this document are to be met by IBSS and infrastructure BSSs, during DCF or optional PCF operation, unless specifically indicated otherwise in the requirement.

# General Requirements

## A device implementing TGh functionality shall meet the functional requirements of ERC/DEC/(99)23 (IEEE802.11-00/171), "ERC Decision of November 1999 on the harmonised frequency bands to be designed for the introduction of High Performance Radio Local Area Networks (HIPERLANS)"

## No functionality specified by TGh shall cause an existing IEEE 802.11 MAC and IEEE802.11a PHY mechanisms to become non-conformant.

## All devices that implement functionality specified by TGh shall remain compatible with legacy equipment (both APs and stations, and both DCF and PCF modes).

## Changes to the existing standard and new functionality introduced to meet TGh requirements shall be kept to a minimum.

# Transmit Power Control Requirements

## Applicable Regulatory Requirements

### Support transmit power control requirements as defined in ERC/DEC/(99)23, "ERC Decision of November 1999 on the harmonised frequency bands to be designed for the introduction of High Performance Radio Local Area Networks (HIPERLANS)".

## Functional Requirements

### Protocol mechanisms shall be defined to allow transmit power control to be implemented that meets the requirements in this document.

### The supported transmit power levels shall ensure a mitigation factor of at least 3dB.

## MAC Requirements

### Frame Definitions

#### Define new information elements within existing frames, and new management frames as required to support transmit power control

#### .

### MAC Protocol Elements

#### Define a mechanism at an STA to allow aCurrentTXPowerLevel to be set from the MLME on receipt of appropriate management information elements.

#### Define the BSS initial power level set at MLME-START.

#### PHY Requirements

### Modify the MIB parameter aNumberSupportedPowerLevels to match the number of possible power settings

### Define appropriate settings for use in the 5470-5725 MHz bands.

# Spectrum Management Requirements

## Applicable Regulatory Requirements

### Support dynamic frequency requirements as defined in ERC/DEC/(99)23, "ERC Decision of November 1999 on the harmonised frequency bands to be designed for the introduction of High Performance Radio Local Area Networks (HIPERLANS)"

## Functional Requirements

### Mechanisms shall be defined to allow an AP and an STA to assess channel state

### Mechanisms shall be defined to allow an STA to report the state of a given channel

### Mechanisms shall be defined to allow a request for a STA to report the state of a given channel

### Review and enhance if necessary mechanisms in existing MAC protocol that allow an IBSS or infrastructure BSS to change channel in a synchronised manner compatible with all other IEEE802.11 MAC protocols

### MAC Requirements

### Frame Definitions

#### Add a new management frame to request the STA to measure the state or quality of any admissible channel

#### Add a new management frame from an STA reporting to the requestor the state or quality of the requested channel

#### Add a new information element to indicate channel switch information; modify the definition of beacon and probe response to include the new definition.

### General MAC

#### Define constraints for new channel selection algorithm

#### Define new MAC-PHY primitives for communicating channel state information

#### Review and enhance mechanisms if required for BSS frequency agility

## PHY Requirements

### Add channels in the 5470-5725 MHz bands

### Specify mechanisms and PHY primitives for assessing  and communicating channel state to the MAC.