IEEE P802.11
Wireless LANs

IEEE 802.11f pre-Draft

Date:	January 19, 2001

Author:	Bob O’Hara
	IEEE 802.11f Editor
	Phone: (408) 986-9596
	e-Mail: bob@informed-technology.com

Abstract

The document attached is the current state of the work to reach consensus for a first draft of the IAPP Recommended Practice.

P802.11f/D0.2

March 14, 2001

# Draft 
Recommended Practice for Multi-Vendor Access Point Interoperability via an Inter-Access Point Protocol Across Distribution Systems Supporting IEEE 802.11 Operation

Sponsored by the

LAN/MAN Standards Committee

of the

IEEE Computer Society

Copyright © 2001 by the Institute of Electrical and Electronics Engineers, Inc.

345 East 47th Street

New York, NY 10017, USA

All rights reserved.

This is an unapproved draft of a proposed IEEE Standard, subject to change. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities. If this document is to be submitted to ISO or IEC, notification shall be given to the IEEE Copyright Administrator. Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for purposes of developing a national position. Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license. Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department

Copyright and Permissions

445 Hoes Lane, P.O. Box 1331

Piscataway, NJ 08855-1331, USA

# Introduction

(This introduction is not part of IEEE P802.11f, Recommended Practice for Multi-Vendor Access Point Interoperability via Inter-Access Point Protocol Across Distribution Systems Supporting IEEE 802.11 Operation.)

See 9.3 of the IEEE Standards Style Manual for information on the Introduction. Use the heading 1 style for the Introduction and the paragraph style for succeeding paragraphs of text. (See Clauses 1-3 in this template for information about styles.)

At the time this standard was completed, the working group had the following membership:

Stuart Kerry, Chair

David Bagby, Chair, Task Group f

Bob O'Hara, Editor, Task Group f

Put working group member names here

The following persons were on the balloting committee: (To be provided by IEEE editor at time of publication.)

Contents

Introduction	iii

1	Overview	5

1.1	Scope	5

1.2	Purpose	5

1.3	Inter-Access Point recommended practice overview	5

2	References	7

3	Definitions, abbreviations, and acronyms	7

4	Service definition	8

4.1	IAPP-INITIATE.request	8

4.2	IAPP-INITIATE.confirm	9

4.3	IAPP-TERMINATE.request	9

4.4	IAPP-TERMINATE.confirm	10

4.5	IAPP-ADD.request	10

4.6	IAPP-ADD.confirm	11

4.7	IAPP-ADD.indication	12

4.8	IAPP-REMOVE.request	12

4.9	IAPP-REMOVE.confirm	13

4.10	IAPP-REMOVE.indication	13

4.11	IAPP-MOVE.request	13

4.12	IAPP-MOVE.confirm	14

4.13	IAPP-MOVE.indication	15

4.14	IAPP-MOVE.response	16

4.15	IAPP-Config-READ.request	16

4.16	IAPP-Config-READ.confirm	17

5	Operation of the IAPP	17

5.1	Formation and maintenance of the ESS	17

5.2	Support for 802.11 authentication and pre-authentication	19

5.3	Secure operation of the IAPP	19

5.4	AP specific MIB	20

5.5	Single station association	20

6	Packet Formats	20

6.1	General IAPP Packet Format	20

6.2	ADD-notify Packet	21

6.3	MOVE-notify Packet	22

6.4	MOVE-response Packet	22

# Draft 
Recommended Practice for Multi-Vendor Access Point Interoperability via an Inter-Access Point Protocol Across Distribution Systems Supporting IEEE 802.11 Operation

# Overview

## Scope

The scope of this document is to describe recommended practices for implementation of a Distribution System supporting IEEE Standard 802.11 wireless LAN links.  The recommended Distribution System utilizes an Inter-Access Point Protocol (IAPP) that provides the necessary capabilities to achieve multi-vendor Access Point interoperability within the Distribution System. This IAPP is described for a Distribution System consisting of IEEE 802 LAN components supporting an IETF IP environment.

## Purpose

IEEE 802.11 specifies the MAC and PHY layers of a Wireless LAN system and includes the basic architecture of such systems, including the concepts of Access Points and Distribution Systems.  Implementations of these concepts were purposely not defined by P802.11 because there are many ways to create a Wireless LAN system. Additionally, many of the possible implementation approaches involve concepts from higher network layers.  While this leaves great flexibility in Distributions System and Access Point functional design, the associated cost is that physical Access Point devices from different vendors are unlikely to interoperate across a Distribution System due to the different approaches taken to Distribution System design.

As 802.11 systems have grown in popularity, this limitation has become an impediment to WLAN market growth. At the same, time it has become clear that there are a small number of Distribution System environments that comprise the bulk of the commercial WLAN system installations.

This recommended practice specifies the necessary information that needs to be exchanged between Access Points to support the 802.11 DS functions. The information exchanges required are specified for Distribution Systems built on the Internet Engineering Task Force (IETF) Internet Protocol (IP) in a manner sufficient to enable the interoperation of Distribution Systems containing Access Points from different vendors that adhere to the recommended practices.

## Inter-Access Point recommended practice overview

This recommended practice describes a service access point (SAP), service primitives, a set of functions and a protocol that will allow conformant APs from multiple vendors to interoperate on a common DS, using UDP/IP as the protocols to carry the IAPP.  The IAPP described in this recommended practice will function as described when the 802.11 stations maintain a network layer address or addresses that are valid for their point of connection to the network, i.e., when an 802.11 station associates or reassociates, the station must ascertain that its network layer address(es) are configured such that the normal routing functions of the network attaching to the BSS will correctly deliver the station’s traffic to the BSS to which it is associated.  If the station determines that the network layer address(es) is not configured so as to allow the normal routing functions of the network to deliver the station’s traffic to the BSS to which it is associated, the station must obtain such an address(es), before any network traffic can be delivered to it.  A station can meet this requirement is many ways.  Two mechanisms for a station to accomplish this are to renew a Dynamic Host Configuration Protocol (DHPC) lease for its IP address or to use Mobile IP to obtain or create a valid local IP address.  The IAPP is not a routing protocol.  It does not deal with the delivery of 802.11 data frames to the station, instead the IAPP utilizes existing network functionality for data frame delivery.

IAPP

UDP

APME/

SME

DS Services

IP

802.11

MAC

MLME

802.11

PHY

PLME

SAP

SAP

SAP

DSM

MAC

DSM

PHY

With the requirement that stations maintain a valid network layer address, APs function much the same as 802.1D bridges and the IAPP supports the following functions:

- Distribution System Services, as defined in ISO/IEC 8802-11 1999

- Address mapping of wireless medium addresses to distribution system addresses

These additional functions are also supported by the IAPP:

- Evolution of the IAPP through multiple versions

- Mapping of the wireless medium addresses of the “old AP” in an 802.11 reassociation frame to a distribution system address, in order to support the Reassociation Service

- Formation of a distribution system

- Maintenance of the distribution system

- Enforcement of the restriction of ISO/IEC 8802-11 1999 that a station may have only a single association at any given time

- Support for 802.11 authentication and privacy, including preauthentication

- Operation in a reasonably secure fashion

- Remote configuration, including AP attributes

This recommended practice makes use of several IETF RFCs to implement many of its functions.

# References

IEEE Standard 802.11-1999, with all current supplements

rfc2181 - Clarifications to the DNS Specification, Internet Engineering Task Force, www.ietf.org

rfc2541 - DNS Security Operational Considerations, Internet Engineering Task Force, www.ietf.org

rfc2608 - Service Location Protocol, Version 2, Internet Engineering Task Force, www.ietf.org

rfc2609 - Service Templates and Service: Schemes, Internet Engineering Task Force, www.ietf.org

rfc2782 - A DNS RR for specifying the location of services (DNS SRV) , Internet Engineering Task Force, www.ietf.org

rfc2929 - Domain Name System (DNS) IANA Considerations, Internet Engineering Task Force, www.ietf.org

rfc3007 - Secure Domain Name System (DNS) Dynamic Update, Internet Engineering Task Force, www.ietf.org

# Definitions, abbreviations, and acronyms

DNS

SLP

IAPP

UDP

IP

IETF

SAP

ESS

BSS

SSID

BSSID

MAC

AP

DS

DSM

URL

# Service definition

## IAPP-INITIATE.request

### Function

This service primitive causes the AP to initialize the IAPP data structures, functions, and protocols.  It also causes the AP to register itself with the distribution system.

### Semantics of the service primitive

The IAPP-INITIATE.request has the following semantics.

IAPP-INITIATE.request {

SSID;

BSSID;

Registration Service

}

The SSID is the name of the ESS with which the AP is attempting to register.

The BSSID is the MAC address of the 802.11 interface of the AP.

The Registration Service is the URL to be used to locate the server responsible for handling registrations for the ESS.

### When generated

This service primitive is generated by an external management entity when it is desired to reset /initiate the operation of the IAPP and cause it to join the distribution system.  The IAPP should be configured by the external management entity before issuing this service primitive.

### Effect of receipt

Upon receipt of this service primitive from an external management entity, the IAPP initializes its data structures, functions, and protocols.  The information in any IAPP data structures is lost.  It is recommended that the external management entity disassociate any stations with which it is associated before generating this primitive.  The IAPP will find the registration service for its distribution system, as identified in the Registration Service parameter, and register.  This causes a Service Location Protocol (SLP) registration handshake to occur between the AP and the registration service, involving the exchange of one or more packets on the DS.  This is further described in x.x.x.x  The AP should not accept any association requests from stations until the IAPP-INITIATE.confirm has been issued and the IAPP is configured which may be using IAPP-CONFIG.

## IAPP-INITIATE.confirm

### Function

This service primitive notifies an external management entity that the actions begun by an IAPP-INITIATE.request have been completed.  The AP should not accept association requests from stations in the time between an IAPP-INITIATE.request and its corresponding IAPP-INITIATE.confirm

### Semantics of the service primitive

The IAPP-INITIATE.confirm primitive has the following semantics.

IAPP-INITIATE.confirm {

Status

}

The Status parameter indicates the result of the corresponding IAPP-INITIATE. request.  The allowable values for the Status parameter are the SUCCESSFUL, REGISTRATION_SERVICE_NOT_FOUND, ESS_NOT_FOUND, MAC_ADDRESS_IN_USE, UNKNOWN_ERROR, and TIMEOUT.

### When generated

This service primitive is generated when the actions begun by an IAPP-INITIATE.request are completed.  This is generally the case when the SLP registration packet exchange has completed.

### Effect of receipt

Upon receipt of the IAPP-INITIATE.confirm corresponding to a previously issued IAPP-INITIATE.request, an external management entity should examine the status returned and take any further appropriate actions.  Only upon receipt of SUCCESSFUL status from this primitive should the external management entity initialize the operation of the AP by issuing an 802.11 MLME-START.request primitive to the AP.  If this primitive returns any other status, the cause of the failure to initiate the service should be corrected and the IAPP-INITIATE.request should be issued again.

## IAPP-TERMINATE.request

### Function

This service primitive causes the IAPP to cease operation of the IAPP functions, and protocols.  It also causes the AP to deregister itself with the distribution system.

### Semantics of the service primitive

The IAPP-TERMINATE.request primitive has the following semantics.

IAPP-TERMINATE.request {

Registration Service

}

### When generated

This service primitive is generated by an external management entity when it is desired to terminate the operation of the AP and cause it to be removed from the distribution system.

### Effect of receipt

The AP should disassociate any stations with which it is associated before ceasing operations.  The AP will find the registration service for its distribution system and deregister.  The AP should not accept any association requests from stations after the IAPP-TERMINATE.request has been issued.  The IAPP will find the registration service for its distribution system, as identified in the Registration Service parameter, and deregister.  This causes a Service Location Protocol (SLP) registration handshake to occur between the AP and the registration service, involving the exchange of one or more packets on the DS.  This is further described in x.x.x.x.

## IAPP-TERMINATE.confirm

### Function

This service primitive notifies an external management entity that the actions begun by an IAPP-TERMINATE.request have been completed.  The AP should not accept association requests from stations in the time between an IAPP-INITIATE.request and its corresponding IAPP-INITIATE.confirm

### Semantics of the service primitive

The IAPP-TERMINATE.confirm primitive has the following semantics.

IAPP-TERMINATE.confirm {

}

### When generated

This service primitive is generated when the actions begun by an IAPP-TERMINATE.request are completed.

### Effect of receipt

Upon receipt of the IAPP-TERMINATE.confirm corresponding to a previously issued IAPP-TERMINATE.request, an external management entity should examine the status returned and take any further appropriate actions.

## IAPP-ADD.request

### Function

This service primitive is used when a station associates with the AP using an 802.11 associate request frame.  It causes a frame to be sent to the distribution system that will update bridge forwarding tables for the newly associated station, and will notify the distribution system of the new association between the AP and station.

### Semantics of the service primitive

The IAPP-ADD.request primitive has the following semantics.

IAPP-ADD.request {

MAC Address;

Sequence Number;

}

The MAC Address is the address of the station that recently has successfully associated with the AP.

The Sequence Number is the value of the 802.11 Sequence Number field of the Association Request frame received from the associating station.

If provided by the MLME, the station’s path identifier must be included in the IAPP-ADD.request.  The path identifier is a sequence number maintained by the station to identify the sequence of associations for this station.  [There is no such sequence maintained by a station.  How can we require this here?] The higher path identifier represents the newest associated AP.  Regardless of the availability of a path identifier, the IAPP-ADD.request includes the age since the last successful communication with the station.  That will typically be very close to zero seconds.

### When generated

This service primitive is generated by an external management entity when an AP generates an 802.11 MLME-ASSOCIATE.indication.

### Effect of receipt

Receipt of this service primitive should cause the following actions to occur:

- Sending an IAPP ADD-notify packet to the distribution system, addressed such that it will cause the forwarding tables in any bridges that receive the frame to be updated so that all future traffic received by those bridges is forwarded to the port on which the frame was received,

- Notifying the distribution system of the association between the AP and station. [Since the DS can be spread over many subnets, how can this be accomplished?]

## IAPP-ADD.confirm

### Function

This service primitive is used to confirm that the actions initiated by an IAPP-ADD.request have been completed and inform an external management entity of the status of those actions.

### Semantics of the service primitive

The IAPP-ADD.confirm primitive has the following semantics.

IAPP-ADD.confirm {

Status

}

The Status parameter indicates the success or failure of the corresponding IAPP-ADD.request.  The allowable values for this parameter are SUCCESSFUL and ???.

### When generated

This service primitive is generated upon completion of the actions of the IAPP-ADD.request.

### Effect of receipt

Upon receipt of this service primitive by an external management entity with SUCCESSFUL status, the management entity should begin forwarding frames for the associated station.  The external management entity should disassociate the indicated station and correct the cause of the failure, if possible, if any status other than SUCCESSFUL is received.

## IAPP-ADD.indication

### Function

The IAPP-ADD.indication primitive is used to indicate to an external management entity that an association relationship has been established between a mobile station and another AP in the distribution systems.

### Semantics of the service primitive

The IAPP-ADD.indication primitive has the following semantics.

IAPP-ADD.indication {

MAC Address

}

The MAC Address is the address of the mobile station for which the AP is sending the IAPP ADD-request packet.

### When generated

This service primitive is generated upon receipt of an IAPP ADD-notify packet from the distribution system by an AP that is forwarding frames to the BSS for the station identified in the packet. [This, of course, is only effective on the immediate LAN segment and will not cross router boundaries.]

### Effect of receipt

Upon receipt of this service primitive the AP should determine if the station indicated by the MAC Address is shown to be associated with the AP.  If so, this service primitive should cause the generation of an 802.11 MLME-DISASSOCIATE.request by the external management entity.

## IAPP-REMOVE.request

### Function

This service primitive is used when a station is disassociated from an AP.  [Is this still necessary, now that the IAPP is not keeping a record of the locations of mobile stations any longer?  Isn’t this purely a local AP thing?]

### Semantics of the service primitive

The IAPP-REMOVE.request primitive has the following semantics.

IAPP-REMOVE.request {

MAC Address

}

The MAC Address parameter is the address of the 802.11 station that has disassociated from the AP.

### When generated

This service primitive should be generated by an external management entity when that management entity receives an 802.11 MLME-DISASSOCIATE.indication or MLME-DISASSOCIATE.confirm.

### Effect of receipt

Upon receipt of this service primitive, the following actions should occur:

- The distribution system should be notified of the termination of the association between the AP and the disassociated station. [Notify who?]

## IAPP-REMOVE.confirm

### Function

This service primitive is used to indicate that the actions of the corresponding IAPP-REMOVE.request have been completed. [Delete if the corresponding .request does not survive.]

### Semantics of the service primitive

### When generated

This service primitive is generated upon completion of the actions of an IAPP-REMOVE.request.

### Effect of receipt

Upon receipt of this service primitive, an external management entity should cease forwarding data frames for the disassociated station.

## IAPP-REMOVE.indication

### Function

This service primitive is used to indicate that an IAPP-REMOVE operation has been completed at another AP. [This should also be deleted if the .request does not survive.]

### Semantics of the service primitive

### When generated

This service primitive is generated when an xxxframexxx is received.

### Effect of receipt

Upon receipt of this service primitive…

## IAPP-MOVE.request

### Function

This service primitive is used when a station associates with the AP using an 802.11 reassociate request frame.  It causes a frame to be sent to the distribution system that will update bridge forwarding tables for the newly associated station, and will notify the distribution system of the new association between the AP and station.

### Semantics of the service primitive

The IAPP-MOVE.request primitive has the following semantics.

IAPP-MOVE.request {

MAC Address;

Sequence Number;

Old AP

}

The MAC Address is the address of the station that recently has successfully reassociated with the AP.

The Sequence Number is the value of the 802.11 Sequence Number field of the Reassociation Request frame received from the reassociating station.

Old AP is the MAC address (BSSID) of the AP with which the reassociating station was last associated.  This value is obtained from the Old AP Address field of the 802.11 Reassociation Request frame.

If provided by the MLME, the station’s path identifier must be included in the IAPP-ADD.request.  The path identifier is a sequence number maintained by the station to identify the sequence of associations for this station.  The higher path identifier represents the newest associated AP. [The station does not maintain such a sequence.  How can we require this?]  Regardless of the availability of a path identifier, the IAPP-ADD.request includes the age since the last successful communication with the station.  That will typically be very close to zero seconds.

### When generated

This service primitive is generated by an external management entity when an AP generates an 802.11 MLME-REASSOCIATE.indication.

### Effect of receipt

Receipt of this service primitive should cause the following actions to occur:

- Sending a frame to the distribution system, addressed such that it will cause the forwarding tables in any bridges that receive the frame to be updated so that all future traffic received by those bridges is forwarded to the port on which the frame was received,

- Notifying the distribution system of the association between the AP and station. [is this still needed?]

- Requesting any state stored at the AP with which the station was previously associated to be forwarded to the AP with which the station is currently associated by sending an IAPP MOVE-notify packet to the old AP.  This action may include querying the registration service for a mapping of the Old AP address to that AP’s IP address on the DSM.

## IAPP-MOVE.confirm

### Function

This service primitive is used to confirm that the actions initiated by an IAPP-MOVE.request have been completed and inform an external management entity of the status of those actions.

### Semantics of the service primitive

The IAPP-MOVE.confirm primitive has the following semantics.

IAPP-MOVE.confirm {

MAC Address;

Context Blob;

Status

}

The MAC Address is the address of the station from the corresponding IAPP-MOVE.request.

The Context Blob is the context returned by the Old AP, if the Status is SUCCESSFUL.  Otherwise, the Context Blob is null.

The Status parameter indicates the result of the corresponding IAPP-MOVE.request.  The allowable values for this parameter are SUCCESSFUL, OLD_AP_NOT_VALID, MAC_ADDRESS_EQUAL_TO_OLD_AP, TIMEOUT

### When generated

This service primitive is generated upon completion of the actions of the IAPP-MOVE.request, including receipt of context information from the Old AP in an IAPP MOVE-response packet as a result of its use of the IAPP-MOVE.response primitive.

### Effect of receipt

Upon receipt of this service primitive by an external management entity with SUCCESSFUL status, the management entity should begin forwarding frames for the associated station. Any frames received from the distribution system for the associated station should be buffered until receipt of the IAPP-MOVE.confirm.  The external management entity should disassociate the indicated station and correct the cause of the failure, if possible, if any status other than SUCCESSFUL is received.  Completion of the IAPP-MOVE.request includes receipt of station state that may have been received, when the Status is SUCCESSFUL.

## IAPP-MOVE.indication

### Function

This service primitive is used to indicate that a station has reassociated with another AP.

### Semantics of the service primitive

The IAPP-MOVE.indication primitive has the following semantics.

IAPP-MOVE.indication {

MAC Address;

AP Address

}

The MAC Address is the address of the 802.11 station that has reassociated with the AP sending the IAPP Move-notify packet.

The AP Address is the address of the AP sending the IAPP Move-notify packet.

### When generated

This service primitive is generated when an IAPP Move-notify packet is received.

### Effect of receipt

Upon receipt of this service primitive, the AP forwards all state related to the reassociated station to the AP with which the station is now associated by using the IAPP-MOVE.response primitive.

## IAPP-MOVE.response

### Function

This service primitive is used send any context resident in the AP issuing this primitive to another AP when a station has reassociated with another AP.

### Semantics of the service primitive

The IAPP-MOVE.response primitive has the following semantics.

IAPP-MOVE.response {

MAC Address;

AP Address;

Context Blob

}

The MAC Address is the address of the 802.11 station that has reassociated with the AP identified by the AP Address.

The AP Address is the address of the AP where the 802.11 station has reassociated.

The Context Blob is the context for the reassociated station.

### When generated

This service primitive should be generated by the external management entity when an IAPP-MOVE.indication is received.

### Effect of receipt

Upon receipt of this service primitive, the AP forwards all state related to the reassociated station to the AP with which the station is now associated by using the IAPP MOVE-response packet.

## IAPP-Config-READ.request

### Function

This service primitive is used when a request for configuration information from the AP.

### Semantics of the service primitive

Get and/or Set of Configuration Fields need to be allowed.

Configuration items: BSSID, SSID, IP Address, TimetoLive, status, etc.

### When generated

This service primitive is generated by an external management entity when it wishes to obtain some configuration information from the AP, or set a configurable parameter.  Configuration fields that can be obtained are defined as …………

### Effect of receipt

The AP obtains the requested configuration data and places it in the provided buffer, or sets the appropriate configurable field with the provided data.

## IAPP-Config-READ.confirm

### Function

This service primitive is used when the configuration information has been placed in the buffer provided by the IAPP-Config-Read.request.

### Semantics of the service primitive

### When generated

This service primitive is generated when the buffer has been filled with the requested configuration information.

### Effect of receipt

Upon receipt of this service primitive, an external management entity regains ownership of the buffer previously provided to the AP.  If the STATUS is successful, then the data is valid.  Error status can be indicated as well.  If the previous request was to set a configurable field, this indication signals that the requested fields were updated.

# Operation of the IAPP

The IAPP is a communication protocol, used by the management entity of an AP to communicate with other APs, when various local events occur in the AP.  It is an integral part of a communication system comprising a registration service, APs, 802.11 mobile stations, and an arbitrarily connected DS.  The function of the IAPP is to facilitate the creation and maintenance of the DS, support the transparent mobility of 802.11 stations, and enforce the requirement of a single association for each mobile station, as stated in ISO/IEC 8802-11/1999.

## Formation and maintenance of the ESS

The formation and maintenance of an ESS makes use of a registration service that is accessed via the Service Location Protocol (SLP, RFC 2608).  The registration service for each ESS maintains a list of all the APs in the ESS and a mapping of their BSSIDs to their IP addresses on the DSM.  To become part of an ESS, an AP must register itself and provide both its BSSID and DSM IP address to the registration service.  This action is performed in response to an IAPP-INITIATE.request at the IAPP SAP.

An ESS is a set of Basic Service Sets (BSSs) that form a single LAN, allowing an 802.11 mobile station to move transparently, from one BSS to another, throughout the ESS.  The establishment of the first AP in the DS accomplishes the formation of an ESS.  The AP is started, and an IAPP-INITIATE.request is issued. The use of 802.1X is required to authenticate with the DS.   A search is made in the registration service for the SSID.  If this is the first AP in the ESS, the AP will use the SSID that was passed into the service primitive to initialize the registration service for the ESS.  The AP provides its BSSID (MAC address) and UDP port number, as well as its IP address to the registration service.

Management Entities can query the registration service to obtain information about each of the APs in the registration service.  The registration service may be able to manage one or more ESSs, but only one registration service manages a given ESS.  The SSID may reflect the nature/boundaries of the ESS.  As Each AP deregisters, the registration service removes the entry for that AP.  When the last AP is removed for an ESS, the registration service removes the SSID entry as well.

What happens if the AP has more than one IP address, if the IP address is IPv6 which IPv6 addresses doyou register (link local, site local, global and which global if more than one address). Also if an AP is behind a NAT the address registered is the incorrect address i.e. the private address rather than the globally reachable address, except if there is more than one AP behind a NAT in which case the other APs behind the NAT require the private address and the APs on the global site of a NATrequire the global address.

Letter ballot commenters are requested to comment on the interaction between the ESS and DNS boundaries, i.e., does an ESS have to be completely contained inside a single DNS boundary.

### Locating the Registration Service:

The following algorithm should be used to locate the registration service:

- To find the ESS registration service, the AP should send a service request, using SLP, for the following service.

service:ess-registrar.1.0.en:register:  [is this complete and correct?]

- To locate the service, the AP should perform a DNS lookup for the ESS registration service “ess-registrar.1.0.en”, requesting the SRV record.

- If an ESS registration service is available, an SLP service reply will be received with the complete service: URL required to access the ESS registration service.  The following service: URL for the registration service should be used to register the SSID, BSSID, and DSM IP address of the AP.

service:ess-registrar.1.0.en:register://<registrar host>/<registration URL>   [I think we should specify the registration URL more completely.]

The <registrar host> is the host where the registration service resides.  The <registration URL> is the URL to be used to send the registration information to the registration service.

- If no service reply is received, there may not be an ESS registration service accessible on the network.  If there is no ESS registration service available to the AP, the AP should establish its own registration service and make this available to other APs through SLP.

### Changing information in the Registration Service

If an AP changes its IP address, it should notify the registration service.  This should be done by first terminating the IAPP service by issuing an IAPP-TERMINATE.request and subsequently starting the IAPP service with the new address through an IAPP-INITIATE.request.

service:ess-registrar.1.0.en:change://<registrar host>/<change URL>

### Deregistering from the Registration Service

If an AP leaves the ESS, it should notify the registration service.  This should be done by terminating the IAPP service by issuing an IAPP-TERMINATE.request.

When an AP is to be removed from an ESS in response to an IAPP-TERMINATE.request at the IAPP SAP, it should locate the deregistration service.  This service should be at the same host as the registration service.  The deregistration service should be used to remove the BSSID and DSM IP address from the list of APs registered for the ESS identified by the SSID.  The following service: URL should be used to deregister.

service:ess-registrar.1.0.en:deregister://<registrar host>/<deregistration URL>

### Registration Service

The Registration Service can be implemented in several ways.  The particular implementation of the registration service is identified in the service: URL for the registration service.  The registration service should be able to perform the following functions:

- Add a new entry mapping the BSSID of an AP to its DSM IP address in response to a registration request.

- Refresh entries mapping the BSSID of an AP to its DSM IP address in response to refresh requests.

- Remove entries mapping the BSSID of an AP to its DSM IP address in response to deregistration requests or because of entries not being refreshed in the required interval.

- Supply information mapping a BSSID to a DSM IP address in response to a query.

### Active AP Entries

The ESS registration service should require that each AP refresh its registration periodically.  The period for such refreshing should be 5 minutes.  The following service: URL should be used to refresh an AP’s entry with the registration service.

service:ess-registrar.1.0.en:refresh://<registrar host>/<refresh URL>

The <registrar host> is the host where the registration service resides.  The <refresh URL> is the complete URL required to refresh the directory entry for the AP in the registration service.

If an AP has not refreshed its entry in the registration service in 15 minutes, the registration service should remove the entry.

## Support for 802.11 authentication and pre-authentication

There are no requirements from the existing authentication mechanisms of IEEE 802.11-1999 or from the work of 802.11 Task Group e Security Subgroup that require the communication of authentication information between APs.  Thus, the IAPP makes no provision to carry such authentication information.

## Secure operation of the IAPP

Security of the message exchanges of the IAPP are not protected nor authenticated beyond the protection and authentication afforded other protocol traffic in the ESS, unless those services are provided by the protocol carrying the IAPP messages.  Usually, using IPSec to carry the IAPP messages would provide this protection.

- Designed for “reasonably” secure operation

    - Messages between APs must be authenticated, but not necessarily encrypted

    - Messages between APs need no more protection, i.e., encryption, than any other infrastructure configuration messages, such as router protocols like OSPF, EIGRP, or RIP

- Supporting 802.11 authentication and security, including preauthentication

    - Based on the current standard, there is no IAPP messaging required to support this functionality

    - Based on the work to date in the TGe security subgroup, there is no IAPP messaging required to support this functionality

      - In fact, the current work of the 802.11e Task Group Security subgroup will not allow preauthentication, since the 802.1x authentication exchange is carried in data frames and data frames are not allowed until a station is associated

c) IAPP should support an interchange of a security blob between APs; this allows support of other security protocols that may require this support.

## AP specific MIB

[The task group intends to delete this item, unless someone supplies something to fill this spot.  Letter ballot commenters are invited to provide definitions of attributes that belong in an AP-specific MIB.]

## Single station association

802.11 specifies that each Station may only be associated with a single AP at any given time.  When a station changes its association from one AP to another, the station issues a reassociate frame (as specified in the 802.11 standard).  Reception of the reassociate frame and granting of the association by the new AP causes the new AP to issue an IAPP-MOVE.request service primitive.  This causes an IAPP MOVE-notify packet to be sent to the Old AP, requesting the old AP to remove the station from its table, to forward any stored context for the station, and the new AP to add it to its table.  The IP address of the old AP should be obtained from the registration service using the following URL.

service:ess-registrar.1.0.en:lookup://<registrar host>/<lookup URL>   [I think we should specify the lookup URL more completely.]

When a roaming station associates with an AP, rather than reassociates, or when the registration service cannot be located, the AP attempts to enforce the single station association requirement by sending an IAPP ADD-notify packet to the distribution system.  Because this packet is addressed with the source MAC address equal to the MAC address of the associating station and the destination as a subnet-local broadcast address, this packet may not reach all APs in a distribution system.  It will reach at least those APs on the immediate LAN segment of the DSM.  If the old AP receives the IAPP ADD-notify packet, it should remove any context stored for the station.

# Packet Formats

## General IAPP Packet Format

The payload of a UDP packet passed from an AP that has just received a reassociate packet from a new station:

| IAPP Version | Command | Data |
| --- | --- | --- |
| 1 byte | 1 byte | 0-n bytes |

### IAPP Version Field

The IAPP Version Field indicates the protocol version of the IAPP, and thus the organization of the rest of the packet.

### Command Field

This is an 8-bit integer value that identifies the specific function of the packet.  The data field that is specific to that command follows each command field.

Table 1, Command field values

| Value | Command | Description |
| --- | --- | --- |
| 0 | ADD-notify |  |
| 1 | MOVE-notify |  |
| 2 | MOVE-response |  |
| 3-255 | Reserved |  |

### Data Field

The Data Field is a variable length field, the content of which is dependent on the value of the Command field.  The content of the Data Field is described in Error: Reference source not found.

## ADD-notify Packet

The ADD-notify packet is sent on the local LAN segment to notify any AP that receives it that the mobile station identified in the packet has associated at the AP sending the packet.  This packet must be sent using the MAC address of the mobile station that has associated, so that any layer 2 devices, e.g., bridges and other APs, can update their forwarding tables with the correct port to reach the new location of the mobile station.  Because the packet must be sent using the mobile station’s MAC address, it must not use the IP address of the AP, in order not to corrupt the mapping between MAC address and IP address for the AP that may exist in other devices to correctly deliver IP packets to the AP.  The packet is sent to the subnet broadcast address, using an IP source address of 0.0.0.0 so that it will reach every device on the DSM local subnet, even if the LAN is switched.

The ADD-notify packet carries MAC address and sequence number from the mobile station that has associated with the AP.  This packet is sent using the xxx protocol.  The format of the packet is shown in Figure 1.

| Address Length | Pad | MAC Address | Sequence Number |
| --- | --- | --- | --- |

Figure 1, ADD-notify Data Field Format

The Address Length is an 8-bit integer that indicates the number of octets in the MAC Address.  The Pad field is reserved in this version of the protocol and should be sent with a value of zero.  The MAC Address is the MAC address of the station that has associated.  The Sequence Number field contains the integer value of the sequence number of the associate request frame received by the AP from the station that has associated.  Allowable values for the Sequence number are between 0 and 4095.

## MOVE-notify Packet

The MOVE-notify packet is sent using the IAPP protocol, over UDP and IP.  This packet is sent from the AP directly to the old AP with which the reassociating mobile station was previously associated.

The data field of the MOVE-notify packet carries the MAC address and sequence number from the mobile station that has reassociated with the AP sending the packet.  The format of the data field for this packet is shown in Figure 2.

| Address Length | Pad | MAC Address | Sequence Number |
| --- | --- | --- | --- |

Figure 2, MOVE-notify Data Field Format

The Address Length is an 8-bit integer that indicates the number of octets in the MAC Address.  The Pad field is reserved in this version of the protocol and should be sent with a value of zero.  The MAC Address is the MAC address of the station that has associated.  The Sequence Number field contains the integer value of the sequence number of the associate request frame received by the AP from the station that has associated.  Allowable values for the Sequence number are between 0 and 4095.

## MOVE-response Packet

The MOVE-response packet is sent using the IAPP protocol, over UDP and IP.  This packet is sent directly to the AP from which the MOVE-notify packet was received..

The data field of the MOVE-response packet carries the MAC address of the reassociated station and the context information associated with that station.  The format of the data field for this packet is shown in Figure 2.

| Address Length | Pad | MAC Address | Length of Context Blob | Context Blob |
| --- | --- | --- | --- | --- |

Figure 3, MOVE-response Data Field Format

The Address Length is an 8-bit integer that indicates the number of octets in the MAC Address.  The Pad field is reserved in this version of the protocol and should be sent with a value of zero.  The MAC Address is the MAC address of the station that has associated.  The Length of Context Blob is a 16-bit integer that indicates the number of octets in the Context Blob field.  The Context Blob is a variable length field that contains the context information being forwarded for the reassociated station indicated by the MAC Address.  The content of the Context Blob should not be interpreted by the IAPP.

The format of the Context Blob is a series of information elements that are similar to the format of the Information Element defined in IEEE Std 802.11-1999.  The format of the Context Blob is shown in Figure 4.  The element identifiers and format of the information element content are defined by the standards that use the IAPP to transfer context form one AP to another.  Information elements are defined to have a common general format consisting of a 2 octet Element ID field, a 2 octet length field, and a variable-length element-specific information field. Each element is assigned a unique Element ID as defined in the standards that use the IAPP to transfer context between APs. The Length field specifies the number of octets in the Information field.

Users of the IAPP service should ignore information elements, the element identifier of which they do not understand, rather than dropping the entire IAPP MOVE-response packet.   Element identifiers are assigned by the IEEE Registration Service (http://www.ieee.org/???).

| Element Identifier | Length | Information |
| --- | --- | --- |

Figure 4, Information Element Format