IEEE P802.11
Wireless LANs

Proposed Baseline Text for MAC-Level FEC and FEC Frame Formats

Date:  	February, 26, 2001

Author:	John M. Kowalski
	Sharp Labs of America
	Address: 5750 NW Pacific Rim Bvd. Camas, WA 98607
	Phone: (360) 817-7520
	Fax: (360) 834-8696
	e-Mail:  kowalskj@sharplabs.com

Abstract

This document includs proposed baseline text for MAC-Level FEC and FEC Frame Formats

## 7.5 MAC-Level FEC and FEC Frame Formats

MAC-LayerFEC is an option to support low error rate frame transfer.  It is used in parameterized QoS (Level 3).  The following conditions apply to the use of MAC Level FEC:

- The MAC FEC operation  is contingent on the assumption that error correction  is expected to take longer than a SIFS time. As such, all FEC frames must be sent with an acknowledgment policy that does not include immediate acknowledgment.

- A valid FCS must result on an error corrected frame in order to forward the corrected MSDU to higher layers. When the FCS check is failed the MAC does not send up errored MSDUs to higher layers.

- The maximum MPDU that may be conveyed in  non-fragmented FEC frames is 2080 octets,.  However, if fragmentation is used, then larger MSDU sizes can be accommodated.

- Any encryption that may be performed is done prior to FEC encoding.

- MAC layer FEC is performed on a given traffic category. The MAC  MLME, based on requests for parameterized QoS, instantiates the use of MAC-Level FEC. STAs announce their FEC capability via bit 11 of the Capability Information Field (7.3.1.4 and Figure 27).  Activation of FEC for  a subset of streams to/from an FEC-capable ESTA  is accomplished  by  using  bit 4, of the Traffic Specification Element. (7.3.2.15 & figure42.8), via a management frame.

- The  AID, and the TCID are sufficient to indicate that the MPDU is FEC encoded.  The AID, identifies that the ESTA  is FEC capable, and the TCID indicates that a particular stream from the station is FEC encoded.  ESTAs that are not capable of FEC decoding will still be able to read the MPDU, since the code is a systematic code, and, via management frames, the receiving station is made aware of the flow’s encoding.    A non-FEC capable ESTA can therefore interpret the MAC header, and FCS, and discard frames based on an erroneous FCS result.  A non-FEC ESTA will ignore the parity check fields in the FEC frame.  A legacy station will not be able to interpret FEC fields, and therefore FEC encoding shall not be used to send frames to legacy STAs.

- The format of MAC-Level FEC MPDU’s is given  in Figure XXX.  The MAC Layer FEC uses a (224,208) Reed-Solomon code for decoding the MSDU’s, which are broken up into 208 octet blocks.  The code is capable of correcting up to 8 octet errors. The MAC header is encoded via a (48,32) Reed Solomon code, which is a shortened version of the (224, 208) code, and therefore 16 octets of parity are included.   For QoS data frames of types ToDS/From DS not equal to ’11,’,  (i.e., for QoS data frames that do not utilize an Address 4 field)  in order to facilitate decoding and separation of the header from the frame body, a 6 octet field of zeros is  used between the Sequence Control field and the TCID field.  The security header integrity vector (if present) and frame body, as well as the FCS are all encoded within the frame body.  The presence of the Frame Control field of the corrected MAC header can be used to determine whether the security field  is the first 4 or 32 octets of the frame body  This allows non-FEC capable ESTAs  to recognize the header and FCS.  The frame body is encoded in 208 octet blocks.  The ICV is coded as part of the frame body.  If the unencoded frame body is not an in integral multiple of  208 octets, then the last MSDU portion of the frame body is encoded with the remainder,  as though zeros were transmitted to fill in the remaining octets out of the 208, with 16 octets of parity.  Also, in order for non-FEC capable ESTAs to perform parity check, the FCS is done on the entire MPDU, parity check bits included.  However,  in the last  field  of the frame body (prior to FEC parity check), denoted “MSDUN  + ICV + ‘FEC FCS’” in the figure, the last 4 octets in that field denote the FCS of the fields of the frame body that are not  the parity check octets.  In this way, error correction can be performed, and a  FCS can be done on the error corrected  MSDU’s,  to determine whether to forward the error corrected frame up to the SAP.

- The RS  encoding and decoding on the header is done  as follows.  The received header is treated as though it were the first 32 octets of a 208 octet field (the remaining octets are assumed to be zeros; they are not transmitted).  This  208 octet sequence  is encoded with (224, 208 ) Reed-Solomon code, and decoded appropriately.

Figure XXX Proposed Coding Scheme

| MAC Header |  | Frame Body |  |  |  |  |  |  | FCS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | Header FEC |  | Security IV + MSDU1+ ICV | FEC | MSDU2 + ICV | ... | MSDUN + ICV + “FEC FCS” | FEC |  | FCS |
| 32 | 16 |  | 208 | 16 | 208 |  | 208 | 16 |  | 4 |