IEEE P802.11
Wireless LANs

802.11e Draft Standard D1.0 Letter Ballot 27 Comments, Clause 3

Date:	May 14th, 2001

Author:	John Fakatselis, Chair, Et Al

|  | 3.13 | Greg Parks | T | YES | Wording is unclear as to which coordination functions are optional or required and how they may be combined | Optionally allow PCF, mandatorilly require HCF, optionally require eDCF, all may co-exist on the same coordinator |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.2 | Matthew Sherman | T | YES | What is a portal? Is it the same as a bridge portal? If so, say so. If not, clarify. | Clarify text. |  |
|  | 3.25 | Matthew Sherman | T | YES | What is a portal? Is it the same as a bridge portal? If so, say so. If not, clarify. | Clarify text. |  |
|  | 3.52 | APS | T | yes | The concept of Bridge Portal (BP) is not adequately supported in the remainder of this specification. An AP needs to operate specific MAC-layer behavior related to learning what addresses can be reached via what BP MAC addresses in order to support a BP. In practice a user will only confidently be able to use a BP and an AP from the same manufacturer – thereby damaging the reputation of 802.11. | Either remove the BP or make support for it mandatory at all APs. |  |
|  | 3.53 | Bob O’Hara | T | Y | The “sufficiently short” interval mentioned in the definition must be SIFS in order to maintain backward compatibility with the current standard. The definition should be corrected to state this. | Replace the vague language with a proper reference to the SIFS interval. |  |
|  | 3.53 | Letanche | T | Y | The CFB has no maximum duration | Add a maximum duration of (TxOp limit + the overhead of a Poll) |  |
|  | 3.54 | Matthew Sherman | T | YES | More exact to say no contention within BSS. | Reword “inter-station” to read “intra-BSS”. |  |
|  | 3.56 | Greg Parks | T | YES | No need for separate CC defninition since RR can be sent at any time during CP | Eliminate this definition |  |
|  | 3.56 | Letanche | T | Y | The defined HPEPC is never referred to. The controlled contention is a reservation request retrieving mechanism that adds additional complexity to the already existing and sufficient mechanisms. | Delete this definition and related definitions and text |  |
|  | 3.56 | Matthew Sherman | T | YES | What does “enhanced point hybrid coordinator (HCEPC)” mean? | Clarify text. |  |
|  | 3.57 | Greg Parks | T | YES | First reference to Hybrid Coordination funtion, should include “(HCF)” | Add “(HCF)” to this definition |  |
|  | 3.57 | Matthew Sherman | T | YES | Needs clarification. | Change “and hybrid coordination function,” to “and potentially a hybrid coordination function (HFC)”. |  |
|  | 3.58 | Greg Parks | T | YES | There are no longer supposed to be any “levels” of QoS, only QoS and legacy STAs | Remove references to QoS levels 0 and 1 |  |
|  | 3.60 | Greg Parks | T | YES | QoS facility should be required as part of this draft | Change optional to mandatory |  |
|  | 3.61 | APS | T | Yes | It makes little sense to have the HC at a station that is not the AP: 1. because the AP has control of the DCF TxOp limit; and 2. because there is no negotiation or protocol defined for how multiple possible candidate non-AP HCs “elect” one of them to operate as the HC. | Require HC to be an EAP. |  |
|  | 3.61 | Greg Parks | T | YES | QoS facility should be required as part of this draft | Change optional to mandatory |  |
|  | 3.63 | Matthew Sherman | T | YES | Are non-QoS capable STA associated with an EAP considered a part of the QBSS since they do not support QoS? | Clarify text. |  |
|  | 3.64 | Bob O’Hara | T |  | I believe that the discussion of priorities in this definition has confused user priorities and channel access priorities. The priorities provided with the MSDU are user priorities and are monotonically decreasing in value from 7 to 0. The map in Annex H.2 of 802.1D maps the user priorities to channel access priorities and suggests (the Annex is informative) that for a MAC with 7 channel access priorities that a specific reordering be done to provide a “penalty box” for “less than best effort” traffic. If this is the desired mapping for 802.11 QoS enhancements, it belongs in a normative section, not a definition. | Delete the paragraph on priority. |  |
|  | 3.64 | Matthew Sherman | T | YES | Needs clarification. | Change “The priority value” to “The priority or traffic class parameter”. |  |
|  | 3.64 | Greg Parks | T | YES | This description is biased toward prioritization | Include simple description of the QoS parameters that may be provided: bandwidth, latency, jitter |  |
|  | 3.65 | APS | T | yes | My comments on 3.52 also apply here. Any use of 4-address-format DATA MPDUs requires that a node generating these frames learn the association between MAC-layer addresses of the BP or RHC that is providing MAC-layer bridging to either a DS or another BSS. This definition also confuses the issues of range extension (wireless repeater) with range extension + support for QoS. In the latter case, there is no support in this specification for 802.11 MAC-layer end-end negotiation of QoS parameters. Furthermore, 802.11e (S) needs to define how 802.1x operates in this configuration, and 802.11f (IAPP) needs to define how mobility between the RHC and an AP is supported. |  |  |
|  | 3.65 | Greg Chesson | T | No | Remote Hybrid Coordinator function is not justified. It’s not clear if the intention is to allow multiple HC’s or an HC-repeater function. Neither possibility is warranted by current practice. | Remove Remote HC definition (3.65) and all references: section 4 (RHC definition), 5.4.1.1, 7.1.3.3.3 (BSSID Field – undo the proposed changes), 7.2.3 (remove subclause 3) 7.4.6 (remove), 7.4.7 (remove). |  |
|  | 3.66 | Matthew Sherman | T | YES | How do a single DTIM interval and a single beacon interval compose a superframe? | Clarify. |  |
|  | 3.66 | Kenji Fujisawa | T | Yes | The meaning of “consisting of a single DTIM interval and single beacon interval” is unclear. Does that mean DTIM period and CFP period shall be 1? |  |  |
|  | 3.67 | Greg Parks | T | YES | This is unclear in isolation – how does this relate to traffic classes and traffic specifications, if at all? | TBD |  |
|  | 3.68 | Butch Anton | T | No | 5/23-27: Question | What happens when the MAC is no longer able to provide the requested bandwidth and/or service quality? Does the connection drop? Does the sky turn green? Is this an appropriate place to talk about it, or should we just make reference to someplace where we do address it? |  |
|  | 3.68 | Greg Parks | T | YES | This may be overly verbose and descriptive compared to 3.67 | TBD |  |
|  | 3.69 | Greg Parks | T | YES | What is an HPC? I seem to have missed this definition anywhere | Define HPC |  |
|  | 3.5.2 | Greg Chesson | T | no | Bridge Portals are a natural concept that is outside the scope of the 11e MAC protocol. The fact of whether or not a Bridge Portal is present, as indicated in a bitfield, does not affect the correct operation of the MAC – since there are no procedures that depend on the bit – nor is a bridge portal indication needed for interoperability of 802.11. | Remove the Bridge Portal definition from the Glossary (delete subclause 3.5.2). Remove all references to Bridge Portals from the text: Sections 3.2, 3.2.5, 3.5.2, 4., 5.2.4, 5.3.2, 5.4.1.2, 7.1.3.1.3, 7.1.3.1.3, 7.1.3.1.4, 7.3.1.4, 7.1.3.1.7, 7.2.2. |  |
|  | 3.70 | Matthew Sherman | T | YES | I believe a “wireless station” should include any non-AP non-bridge portal regardless of whether it is enhanced. | In first line replace “enhanced station (ESTA) with “station (STA)” and replace “enhanced access point (EAP)” with “access point (AP)”. |  |