IEEE P802.11
Wireless LANs

Proposed Text for EAP Authentication Suite Advertising

Date:	May 16, 2001

Author:	Jon Edney

Contact Info:	Nokia UK

Unit 6, Cambridge Bus. Park

Cambridge, UK

Jon.edney@nokia.com

Abstract

This contribution contains text for insertion into 802.11eD0, page 26 as per presentation IEEE802.11-01/306a.

TEXT:

| Page | Line | Changes proposed to add option for specific EAP advertising in Authentication Suite Selector |
| --- | --- | --- |
| 26 Element ID (TBD); Length; Authentication Suite List. | 20-23 | Modify figure as shown: |
| 26 OUI (3 Octets); Suite Specifier (1 Octet); Suite Parameter (0 – 4 Octets). | 26-29 | Modify text and Figure 3 as follows: “Each item in the Authentication Suite List consists of a field, comprised of a 3-octet OUI, followed by a 1 octet “suite specifier” field followed by 0 to 4 octets as required by the suite selector and specified in Table 1. Note that the OUI field is transmitted most significant byte first.” |
| 26 Table 1a Value of OUI in Authentication Suite Selectors OUI; Meaning. 00:00:00; Suites as defined in Table 1b. Other; Vendor specific. Table 1b Values of Suite Specifier for OUI of 00:00:00 Value; Meaning; Octets in Suite Parameter. 0; Open Authentication – default in a non-ESN; 0. 1; Shared Key Authentication; 0. 2; Kerberos over 802.1X; 0. 3; Unspecified authentication over 802.1X (802.1X to select authentication algorithm) – Enhanced Security Default; 0. 4; 802.1X using EAP; 2. 5 – 255; Reserved; 0. Table 1c Suite Parameter for Suite Specifier of 2 (802.1X using EAP) Value; Meaning. 1 – 65535; Value of EAP authentication protocol number as enumerated by IETF for each EAP method defined. | 31 | Table 1 is split into three tables indicating OUI values, Suite Specifier values and Suite parameter for 802.1X case. Shown below: |