IEEE P802.11
Wireless LANs

Normative Text for Acknowledgement of Multicast TCs

Date:	July 9, 2001

Author:	Srinivas Kandala
	Sharp Laboratories of America
	5750 NW Pacific Rim Blvd., CAMAS WA 98607
	Phone: (360) 817-7512
	Fax: (360) 834-8696
	e-Mail: srini@sharplabs.com

Abstract

This document contains the material proposed to Tge for inclusion in the draft in the form of insertions into and replacements for material in of IEEE std 802.11-1999, as updated by IEEE STd 802.11e/D1, March 2001.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Changes to existing text in current standard are shown underlined and in red for additions and red with strikethrough for deletions.

Change the contents of Clause 7.1.3.12, Table 1 as follows:

Table 1 - Valid type and subtype combinations
(numeric values in Table 1 are shown in binary)

| Type Value b3 b2 | Type Description | Subtype Value b7 b6 b5 b4 | Subtype Description |
| --- | --- | --- | --- |
| 00 | Management | 0000 | Association request |
| 00 | Management | 0001 | Association response |
| 00 | Management | 0010 | Reassociation request |
| 00 | Management | 0011 | Reassociation response |
| 00 | Management | 0100 | Probe request |
| 00 | Management | 0101 | Probe response |
| 00 | Management | 0110-0111 | Reserved |
| 00 | Management | 1000 | Beacon |
| 00 | Management | 1001 | ATIM |
| 00 | Management | 1010 | Disassociation |
| 00 | Management | 1011 | Authentication |
| 00 | Management | 1100 | Deauthentication |
| 00 | Management | 1101 | {generic} Action |
| 00 | Management | 1110 | Reserved |
| 00 | Management | 1111 | Container |
| 01 | Control | 0000-0011 | Reserved |
| 01 | Control | 0100 | Reservation Request (RR) |
| 01 | Control | 0101 | Delayed Acknowledgement (DlyAck) |
| 01 | Control | 0110 | Contention Control (CC) |
| 01 | Control | 0111 | Delayed Acknowledgement CF-Poll (DlyAck CF-Poll) |
| 01 | Control | 1000 | Contention-Free Multipoll (CF-Multipoll) |
| 01 | Control | 1001 | Reserved |
| 01 | Control | 1010 | Power Save Poll (PS-Poll) |
| 01 | Control | 1011 | Request To Send (RTS) |
| 01 | Control | 1100 | Clear To Send (CTS) |
| 01 | Control | 1101 | Acknowledgement (ACK) |
| 01 | Control | 1110 | Contention-Free End (CF-End) |
| 01 | Control | 1111 | CF-End + CF-Ack |
| 10 | Data | 0000 | Data |
| 10 | Data | 0001 | Data + CF-Ack |
| 10 | Data | 0010 | Data + CF-Poll |
| 10 | Data | 0011 | Data + CF-Ack + CF-Poll |
| 10 | Data | 0100 | Null (no data) |
| 10 | Data | 0101 | CF-Ack (no data) |
| 10 | Data | 0110 | CF-Poll (no data) |
| 10 | Data | 0111 | CF-Ack + CF-Poll (no data) |
| 10 | Data | 1000 | QoS Data |
| 10 | Data | 1001 | QoS Data + CF-Ack |
| 10 | Data | 1010 | QoS Data + CF-Poll |
| 10 | Data | 1011 | QoS Data + CF-Ack + CF-Poll |
| 10 | Data | 1100 | QoS Null (no data) |
| 10 | Data | 1101 | QoS CF-Ack (no data) |
| 10 | Data | 1110 | QoS CF-Poll (no data) |
| 10 | Data | 1111 | QoS CF-Ack + CF-Poll (no data) |
| 11 | Reserved | 0000-1111 | Reserved |

Add the following sub-clause immediately above Clause 7.2.1.8:

The frame format of the Delayed Acknowledgement CF-Poll (DlyAck CF-Poll) is defined in Figure XX.

| octets: 2 | 2 | 6 | 6 | 6 | 4 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Dur/ID | Address 1 | Address 2 | Address 3 | FCS |

Figure XX

The content of the Address 1 field is the address of the ESTA for which the poll is intended. The content of the Address 2 field is the address of the ESTA to which the ESTA with address 1 intends to sends its DlyAck. The content of Address 3 field is the BSSID.

Modify the last paragraph of Clause 7.2.1.8 as follows:

- TC-bitmap

A 2-octet field in which each bit indicates the reception status of an MSDU within the specified traffic category. TC-bitmap bit 0 indicates the reception status of the MSDU with the sequence number contained in the TC-seq field, and subsequent bits indicate the reception status of MSDUs within the same TC having the next 15 sequentially-ascending sequence numbers. In ACK mode, TC-bitmap bits set to 1 indicate MSDUs that have been received successfully, whereas bits set to 0 indicate MSDUs that have not been received (and which may have not yet been sent if fewer than 16 MSDUs are awaiting delayed acknowledgement for this TC). In Negative ack mode (when the Retry Inteval is set to 0), TC-bitmap bits set to 0 indicate MSDUs that have been not received or received with error, whereas bits set to 1 indicate other status (MSDUs have been received successfully or have not been received yet). Fragmented MSDUs which have been partially received (More Fragments=1 in the received MPDU with the highest fragment number) are reported as not received (=0) in the TC-bitmap.

Modify the following paragraph of clause 7.3.2.15 as follows:

…

The Ack policy sub-field indicates whether MAC acknowledgement is required for MSDUs belonging to this TC, and the desired form of those acknowledgements. Certain, selectable and/or optional facilities (e.g. FEC) may require the use of a particular ACK policy setting.

- Normal IEEE 802.11 acknowledgement.

The addressed recipient returns an ACK or (+)CF-Ack frame after a SIFS period, according to the procedures defined in clause 9.

- Alternate acknowledgement

Reserved for future use.

- Delayed acknowledgement

The addressed recipient returns a DlyAck frame, when polled by the HC. The HC shall send the poll to the addressed recipients (if there are more than one recipient, each of the ESTAs would be polled by the HC) by sending the DlyAck CF-Poll frame. The polled frame should be sent at the end of the TXOP unless there is a DTIM scheduled. The DlyAck CF-Poll frame shall be sent by HCwithin the retry period specified in this TSPEC.

- No acknowledgement

The recipient(s) shall not acknowledge the transmission, and the sender treats the transmission as successful without regard for the actual result.

Add the following sub-clause in clause 9 or any other clause that the editor deems appropriate:

### 9.10.6 Delayed Acknowledgement

The receiving ESTAs may not be able to generate acknowledgement frames withing SIFS duration of receiving for some QoS data frames. This may be because of the required processing that needs to be done before such an acknowledgement takes longer than SIFS or because the ESTA may have to compete for the channel access with other ESTAs who are on the multicast Destination Address of the TC. Only TCs that have been registered with the HC (through TSPEC by setting bits 2 and 3 of the TS Info field in the TSPEC to 1 and 0 respectively) may use the Delayed Acknowledgement.

Figure XX+1 shows the mechanism involved in delayed acknowledgement for data frames of a TC with one recipient (unicast TC). At the end of the TXOP, within a duration of PIFS, the HC shall send a DlyAck CF-Poll frame to the recipient. The recipient will respond by transmitting the DlyAck frame to the sender for the QoS data frame. Figure XX+2 shows the mechanism involved when there are multiple recipients of a data TC. The HC shall send the DlyAck CF-Poll to all the ESTAs on DA, which would be a multicast address.