IEEE P802.11
Wireless LANs

Suggested revision of TGh draft D1.0

Date:	9 November 2001

Author:	Andrew Myles
	Cisco Systems
	Level 2, 3 Innovation Road,
	North Ryde, 2113, Australia
	Phone: +61 2 84461010
	Fax: +61 2 84461001
	e-Mail: andrew.myles@cisco.com

Abstract

TGh recently submitted a draft (D1.0) to letter ballot (LB29).

The letter ballot responses indicate a wide range of issues and problems; some major and some minor. A fundamental issue highlighted by a number of voters was that D1.0 exceeds the scope set by TGh’s PAR for both DFS and TPC related functionality.

The author believes that the best way for TGh to achieve its goal of enabling 802.11a in Europe in the fastest time possible is to start by removing all DFS and TPC functionality exceeding the scope of the PAR. IEEE 802.11-01/542r0 discusses the detailed reasons for this conclusion.

This document represents a revision of the draft that removes all functionality exceeding the scope of the PAR. This proposed revision also draws on the letter ballot comments, while remaining within the spirit of the original TGh proposals, to:

- Add missing functionality required by the PAR

- Correct various protocol errors

- Improve text using style and layout compatible with 802.11 traditions

- Include templates for unwritten sections

The aim of this proposed revision is to assist TGh towards its primary goal; to enable the operation of 802.11a in Europe. It is offered in a spirit of progress. Any equivalent alternative revision from other authors are equally acceptable.

It should be noted that this proposed revision does NOT represent an official draft of TGh.

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

DFS and TPC extensions
in the 5GHz band in Europe

Sponsored by the
IEEE 802 Committee
of the
IEEE Computer Society

Copyright (c) 2001 by the Institute of Electrical and Electronics Engineers, Inc.
345 East 47th Street
New York, NY 10017, USA
All rights reserved.

This is an unapproved draft of a proposed IEEE Standard, subject to change. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities. If this document is to be submitted to ISO or IEC, notification shall be given to the IEEE Copyright Administrator. Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for purposes of developing a national position. Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license. Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department
Copyright and Permissions
445 Hoes Lane, P.O. Box 1331
Piscataway, NJ 08855-1331, USA

# Introduction

This introduction is not part of IEEE P802.11h, Draft Supplement to STANDARD FOR Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications: DFS and TPC extensions in the 5GHz band in Europe)

Example:

At the time this standard was completed, the working group had the following membership:

List of members of working group

The following persons were on the balloting committee: (To be provided by IEEE editor at time of publication.)

List of members of balloting committee

# Contents

Introduction	ii

Contents	iii

2. Normative References	2

3. Definitions	2

4. Abbreviations and acronyms	2

5.5 Relationships between services	3

5.7.2 Association	3

5.7.3 Reassociation	3

7.1.3.1.2 Type and Subtype fields	4

7.2.3.1 Beacon frame format	5

7.2.3.4 Association Request frame format	5

7.2.3.6 Reassociation Request frame format	5

7.2.3.8 Probe Request frame format	6

7.2.3.9 Probe Response frame format	6

7.2.3.12 Generic Action frame body format	7

7.3.1.4 Capability Information field	8

7.3.2 Information elements	9

7.3.2.22 Power Constraint element	10

7.3.2.23 Channel Switch Announcement element	12

7.3.2.24 Basic Channel Measurement Request element	13

7.3.2.25 Basic Channel Measurement Report element	14

7.3.2.26 Extended Channel Measurement Request element	16

7.3.2.27 Extended Channel Measurement Report element	17

7.3.2.28 Supported Channels element	18

7.4 DFS Actions	19

7.4.1 Channel Measurement Request frame format	20

7.4.2 Channel Measurement Report frame format	21

9.11 TPC procedures	23

9.11.1 Specification of a regulatory and local maximum transmit power levels	23

9.11.2 Selection of a transmit power	24

9.12 DFS procedures	25

9.12.1 Measuring of channels	25

9.12.2 Reporting channel measurements	26

9.12.3 Selection of channels	27

10.3 MLME SAP interface	29

17.2.2 TXVECTOR parameters	29

17.2.3 RXVECTOR parameters	29

Annex A – PICS	30

A.4.3 IUT configuration	30

A.4.11 TPC extensions	30

A.4.12 DFS extensions	30

Annex C – Formal description of MAC operation	31

Annex D – ASN.1 encoding of the MAC and PHY MIB	32

Annex E – Bibliography	33

Annex F – Extended Measurement Requests and Reports in DFS	34

F.1 RSSI Histogram Request	35

F.2 RSSI Histogram Report	35

Annex G – Using TPC and DFS facilities for other purposes	36

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

DFS and TPC extensions
in the 5GHz band in Europe

[The editing instructions contained in this supplement define how to merge the material contained herein into the current edition of IEEE Std 802.11 1999 Edition and the 802.11a, 802.11b and 802.11d supplements.

The editing instructions are shown in bold italic. Three editing instructions are used:

Change is used to make small corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed either by using strikethrough (to remove old material) or underscore (to add new material).

Delete removes existing material.

Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instructions.

Editorial notes will not be carried over into future editions.]

# 2. Normative References

Insert the following citation at the appropriate location in clause 2:

ERC/DEC/(99)23, ERC Decision of 29 November 1999 on the harmonized frequency bands to be designated for the introduction of High Performance Radio Local Area Networks (HIPERLANs).

# 3. Definitions

Insert the following new definitions at the appropriate locations in clause 3, renumbering as necessary:

3.51 dynamic frequency selection (DFS): Facilities mandated to satisfy requirements in some regulatory domains that specify precedence for licensed users and uniform channel spreading in the 5GHz band. DFS facilities may also be used for other purposes, such as network planning.

3.52 transmit power control (TPC): Facilities mandated to satisfy requirements in some regulatory domains for maximum transmit power and average transmit power mitigation in the 5GHz band. TPC facilities may also be used for other purposes, such as interference reduction, range control and power optimization.

3.53 transmit power: transmit power is the power measured at the antenna.

3.54 receive power: receive power is the power measured at the antenna.

# 4. Abbreviations and acronyms

Insert the following text in the appropriate location in clause 4:

- DFS	dynamic frequency selection

- TPC	transmit power control

## 5.5 Relationships between services

Add to the definition of class 1 management frames as follows:

vi) Generic Action

### 5.7.2 Association

Insert the following text under "Association request" just after entry for "ESS ID" (with appropriate leader character):

- Supported channels

### 5.7.3 Reassociation

Insert the following text under "Reassociation request" just after entry for "ESS ID" (with appropriate leader character):

- Supported channels

##### 7.1.3.1.2 Type and Subtype fields

Change the contents as follows:

Table 1 - Valid type and subtype combinations

| Type Value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 1101 | Generic Action |
| 00 | Management | 1110-1111 | Reserved |

#### 7.2.3.1 Beacon frame format

Change order 11 information field and add order 14 and 15 information fields as follows:

Table 5 - Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | Country element shall be included if dot11MultiDomainCapabilityEnabled is true or dot11TpcCapabilityEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be included if dot11TpcCapabilityEnabled is true |
| 15 | Channel Switch Announcement | Channel Switch Announcement element may be included if and only if dot11DfsCapabilityEnabled is true |
| 16 | Supported Channels | Supported Channels element may be included if and only if dot11DfsCapabilityEnabled is true |

Editors note: order numbers of all frame formats will need to be harmonized with the work in other working groups prior to sponsor ballot.

Editors note: the Supported Channels element might be useful in an IBSS to assist other STAs select a new channel

#### 7.2.3.4 Association Request frame format

Add order 5 information field as follows:

Table 7 - Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | Supported Channels | Supported Channels element may be included if and only if dot11DfsCapabilityEnabled is true |

#### 7.2.3.6 Reassociation Request frame format

Add order 6 information field as follows:

Table 9 - Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Supported Channels | Supported Channels element may be included if and only if dot11DfsCapabilityEnabled is true |

#### 7.2.3.8 Probe Request frame format

Add order 3 information field as follows:

Table 11 - Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 4 | Supported Channels | Supported Channels element may be included if and only if dot11DfsCapabilityEnabled is true |

#### 7.2.3.9 Probe Response frame format

Change order 10 information field and add order 14 and 15 information fields as follows:

Table 12 Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Country element shall be included if dot11MultiDomainCapabilityEnabled is true or dot11TpcCapabilityEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be included if dot11TpcCapabilityEnabled is true |
| 15 | Channel Switch Announcement | Channel Switch Announcement element may be included if and only if dot11DfsCapabilityEnabled is true |
| 16 | Supported Channels | Supported Channels element may be included if and only if dot11DfsCapabilityEnabled is true |

Editors note: the Country element is from 802.11d, only the notes change

Insert the following clause and renumber as necessary:

#### 7.2.3.12 Generic Action frame body format

The Generic Action frame indicates a management action from a particular category. The Generic Action frame body is shown in Figure 1.

|  | Category | Action | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | Generic Actions |  |  |  |
| Octets: | variable |  |  |  |

Figure 1 – Generic Action frame body

The Category field shall be set to one of the values shown in Table 1.

The Action field shall be set to a value that is allowed for the Category. The most significant bit shall normally be set equal to 0. However, if a STA receives a unicast Generic Action frame that it does not understand, the STA shall return the frame to the source with the most significant bit of the Action field set equal to 1.

The Action Specific field shall be set to a value that is specific to the Category and Action. It is normally used to carry a status value or an activation delay.

The Dialog Token field shall be set to a value that is specific to the Category and Action. It is normally used to associate a series of frames in a multi-management frame transaction.

The Generic Actions field shall contain fixed fields and elements that are specific to the Category and Action.

Table 1 - Category values

| Name | Value |
| --- | --- |
| DFS | 0 |
| Reserved | 1-255 |

Editors note: Category values will need to be harmonized with the work in other working groups prior to sponsor ballot.

Editors note: The style of the 802.11 MAC standard uses arrows to indicate the dimensions of each field in an element or frame figure. This draft does not yet contain these arrows

Editors note: The generic action frame here is a simplification of that in 802.11e. The 802.11e version is too complex. 802.11e and 802.11h need to work together to come up with a mutually agreeable definition

#### 7.3.1.4 Capability Information field

Insert two sub fields to the capability information field figure and change supporting text as follows:

The length of the Capability Information Field is two octets. The Capability Information Field consists of the following subfields: ESS, IBSS, CF-Pollable, CF-Poll Request, Privacy, Short Preamble, PBCC, Channel Agility, TPC and DFS. The format of the Capability Information field is as illustrated in Figure 27.

Change the contents of Figure 27 as shown:

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-field | ESS | IBSS | CF Pollable | CF Poll Request | Privacy | Short Preamble | PBCC | Channel Agility |
| Bit | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Sub-field | TPC | DFS | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) |

Figure 27 - Capability Information fixed field

Insert the following text at the end of clause 7.3.1.4:

A STA shall set the TPC subfield in the Capability Information field equal to 1 if the boolean MIB attribute dot11TpcCapabilityEnabled is true, otherwise it shall be set equal to 0.

A STA shall set the DFS subfield in the Capability Information field equal to 1 if the boolean MIB attribute dot11DfsCapabilityEnabled is true, otherwise it shall be set equal to 0.

Editors note: Bit fields will need to be harmonized with the work in other working groups prior to sponsor ballot.

Editors note: the primary purpose of these bits is to allow an AP to disallow association of STAs based on their capability – the use of this mechanism is not standardized.

### 7.3.2 Information elements

Insert the following information elements into Table 20 of clause 7.3.2 in place of the reserved IDs:

Table 20 - Element IDs

| Information element | Element ID |
| --- | --- |
| Power Constraint | 36 |
| Channel Switch Announcement | 37 |
| Basic Channel Measurement Request | 38 |
| Basic Channel Measurement Report | 39 |
| Extended Channel Measurement Request | 40 |
| Extended Channel Measurement Report | 41 |
| Supported Channels | 42 |
| Reserved | 11-35, 43-255 |

Editors note: Element IDs will need to be harmonized with the work in other working groups prior to sponsor ballot

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.22 Power Constraint element

The Power Constraint element contains the information necessary to allow a STA to determine the local maximum transmit power level that shall not be exceeded for transmissions in one or more channels. The format of the Power Constraint element is shown in Figure 2.

|  | Element ID (36) | Length (variable) |  |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |
|  | First Channel Number | Number of Channels | Local Power Constraint | This triplet is repeated zero or more times |
| Octets: | 1 | 1 | 1 |  |
|  | Pad (0) | Only included if required to make the length of the element even |  |  |
| Octets: | 1 |  |  |  |

Figure 2 - Power Constraint element format

The Element ID field shall be set equal to 36. The Length field is variable and depends on the number of triplets in the element and the requirement for a Pad field. The minimum value of the Length field is 4.

The First Channel Number field shall indicate the first channel in the sub-band described by the triplet. The field is coded as a positive integer. The First Channel Number field shall monotonically increase in sequential triplets.

The Number of Channels field shall indicate the number of channels in the sub-band described by the triplet. The field is coded as a positive integer. The Number of Channels field shall not contain a value that causes the sub-bands described by each triplet to overlap.

The Local Power Constraint field shall be set equal to a value greater than or equal to the minimum transmit power mitigation required to operate in the channels described by the triplet to satisfy regulatory requirements. The field is coded as an unsigned integer in units of dB. The local maximum transmit power for a channel is thus defined as the Maximum Transmit Power Level specified for the channel in the Country element minus the Local Power Constraint specified for the channel in the Power Constraint element.

The Pad field is included as necessary to make the length of the element even. The Pad field shall be set equal to zero.

Editors note: The Local Power Constraint field will normally be set equal to 3dB (default value of dot11MitigationRequirement for each channel). An AP might set it to a lesser value if it believes the average mitigation requirement can be satisfied with a lesser value (<3dB). An AP might set it to a greater value (>3dB) for range control.

Editors note: While it is possible to specify a 256dB range It is not intended that STAs should be capable of transmitting over the full range of 256dB.

Editors note: Per channel power constraints are provided to allow a flexible response to any  future regulatory  changes

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.23 Channel Switch Announcement element

The Channel Switch Announcement element allows an AP in a BSS or a STA in an IBSS to advertise when it is changing to a new channel and the channel number of the new channel. The format of the Channel Switch Announcement element is shown in Figure 3.

|  | Element ID (37) | Length (2) |
| --- | --- | --- |
| Octets: | 1 | 1 |
|  | New Channel Number | Channel Switch Count |
| Octets: | 1 | 1 |

Figure 3 - Channel Switch Announcement element format

The Element ID field shall be set equal to 37. The Length field shall be set equal to 2.

The New Channel Number field shall be set equal to the number of the channel to which the STA is moving.

The Channel Switch Count field shall be set equal to the number of TBTTs until the STA sending the Channel Switch Announcement element will switch to the new channel. A value of 1 indicates that the switch will occur immediately after the next TBTT. A value of 0 indicates that the switch will occur immediately.

Editors note: It is assumed that the channel numbers are defined by the various PHYs

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.24 Basic Channel Measurement Request element

The Basic Channel Measurement Request element contains a request that the receiving STA undertake a measurement for a specified duration to determine if a licensed user is operating in a particular channel. The format of the Basic Channel Measurement Request element is shown in Figure 4.

|  | Element ID (38) | Length (4) |  |
| --- | --- | --- | --- |
| Octets: | 1 | 1 |  |
|  | Channel Number | Measurement Token | Measurement Duration |
| Octets: | 1 | 1 | 2 |

Figure 4 - Basic Channel Measurement Request element format

The Element ID field shall be set equal to 38. The Length field shall be set equal to 4.

The Channel Number field shall be set equal to the channel number for which a measurement report is requested.

The Measurement Token shall be set to a non-zero number that is unique among the Basic Channel Measurement Request elements in a particular Channel Measurement Request frame.

The Measurement Duration field shall be set equal to the non-zero number of TUs for which the channel specified in the Channel Number field shall be measured.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.25 Basic Channel Measurement Report element

The Basic Channel Measurement Report element reports whether a licensed user is operating in a specified channel. The format of the Basic Channel Measurement Report element is shown in Figure 5.

|  | Element ID (39) | Length (4) |  |
| --- | --- | --- | --- |
| Octets: | 1 | 1 |  |
|  | Channel Number | Measurement Token | Measurement Summary |
| Octets: | 1 | 1 | 2 |

Figure 5 - Basic Channel Measurement Report element format

The Element ID field shall be set equal to 39. The Length field shall be set equal to 4.

The Channel Number field shall be set equal to the channel to which the Measurement Summary refers.

The Measurement Token shall be set equal to the Measurement Token in the corresponding Basic Channel Measurement Request element. If the Basic Channel Measurement Report is being provided autonomously then the Measurement Token shall be set equal to zero.

The Measurement Summary field shall contain a summary of the measurement results for the channel for the measurement period. The Measurement Summary field is coded as a bit field as shown in Figure 6 and includes a:

- BSS field, which indicates (when it is set equal to 1) at least one valid MPDU was decoded during the measurement period.

- Foreign PLCP Header field, which indicates (when it is set equal to 1) at least one PLCP preamble was detected during the measurement without a subsequent valid Signal field period, which may or may not indicate the presence of a foreign PLCP header such as Hiperlan2.

- Possible Licensed User field, which indicates (when it is set equal to 1) that the STA performing a measurement has detected transmissions in the channel that cannot be characterized as either from a licensed user or an unlicensed user.

- Licensed User field, which indicates (when it is set equal to 1) that the STA performing the measurement believes that it has detected a licensed user operating in the channel. The algorithm used to draw this conclusion is not standardized.

- Clean Channel field, which indicates (when it is set equal to 1) that the STA performing the measurement believes that there are no users (licensed or unlicensed) operating in the channel. The algorithm used to draw this conclusion is not standardized.

| Bits | 0 | 1 | 2 | 3 | 4 | 5-15 |
| --- | --- | --- | --- | --- | --- | --- |
| Name | BSS | Foreign PLCP Header | Possible Licensed User | Licensed User | Clean Channel | Reserved (0) |

Figure 6 - Measurement Summary bit field

Editors note: Need to define PHY/MAC interface signaling to enable Foreign PLCP Header bit

Editors note: There is contention as to the exact bits that should be included; some want exactly the same as in D1.0, while another suggestion is to reduce the signaling to one bit, with a name like “interference detected”

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.26 Extended Channel Measurement Request element

The Extended Channel Measurement Request element contains a request that the receiving STA undertake an extended measurement for a specified duration in a particular channel. The format of the Extended Channel Measurement Request element is shown in Figure 7.

|  | Element ID (40) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |
|  | Channel Number | Measurement Token | Measurement Duration | Measurement ID | Measurement Request |
| Octets: | 1 | 1 | 2 | 2 | variable |
|  | Pad (0) | Only included if required to make the length of the element even |  |  |  |
| Octets: | 1 |  |  |  |  |

Figure 7 - Extended Channel Measurement Request element format

The Element ID field shall be set equal to 40. The Length field is variable and depends on the length of the Measurement Request field and the requirement for a Pad field. The minimum value of the Length field is 6.

The Channel Number field shall be set equal to a channel number for which a measurement report is requested.

The Measurement Token shall be set to a non-zero number that is unique among the Extended Channel Measurement Request elements in a particular Channel Measurement Request frame.

The Measurement Duration field shall be set equal to the non-zero number of TUs for which the channel specified in the Channel Number field shall be measured.

The Measurement ID field shall be set to a number that identifies the format and contents of the Measurement Request field. The IEEE shall be responsible for allocating the number space.

The Measurement Request field shall contain the specification of a requested channel measurement. The Measurement ID field identifies the format of the Measurement Request field.

The Pad field is included as necessary to make the length of the element even. The Pad field shall be set equal to zero.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.27 Extended Channel Measurement Report element

The Extended Channel Measurement Report element contains extended measurement information for a specified channel. The format of the Extended Channel Measurement Report element is shown in Figure 8.

|  | Element ID (41) | Length (variable) |  |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |
|  | Channel Number | Measurement Token | Measurement ID | Measurement Report |
| Octets: | 1 | 1 | 2 | variable |
|  | Pad (0) | Only included if required to make the length of the element even |  |  |
| Octets: | 1 |  |  |  |

Figure 8 - Extended Channel Measurement Report element format

The Element ID field shall be set equal to 41. The Length field is variable and depends on the length of the Measurement Report field and the requirement for a Pad field. The minimum value of the Length field is 4.

The Channel Number field shall be set equal to the channel to which the Measurement Report element refers.

The Measurement Token shall be set equal to the Measurement Token in the corresponding Extended Channel Measurement Request element. If the Extended Channel Measurement Report is being provided autonomously then the Measurement Token shall be set equal to zero.

The Measurement ID field shall be set to a number that identifies the format and contents of the Measurement Report field. The IEEE shall be responsible for allocating the number space.

The Measurement Request field shall contain the specification of the channel measurement report. The Measurement ID field identifies the format of the Extended Measurement Request field.

The Pad field is included as necessary to make the length of the element even. The Pad field shall be set equal to zero.

Editors note: it would be possible to fold the basic and extended measurement request and report elements together if desired

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.28 Supported Channels element

The Supported Channels element contains a list of channels in which a STA is capable of operating. The format of the Supported Channels element is shown in Figure 9.

|  | Element ID (42) | Length (28) |
| --- | --- | --- |
| Octets: | 1 | 1 |
|  | Supported Channels |  |
| Octets: | 28 |  |

Figure 9 - Supported Channels element format

The Element ID field shall be set equal to 42. The Length field shall be set equal to 28.

The Supported Channels field is a 28-octet bit field, where bit 0 corresponds to channel 1 and bit 199 corresponds to channel 200. A bit in the field shall be set equal to 1 when the STA supports operation in the corresponding channel, otherwise it shall be set equal to 0. Bits 200-223 shall be set equal to 0.

Editors note: it is not clear that this element is required to satisfy the PAR

Insert clause 7.4 as follows. Renumber figures and tables as appropriate:

## 7.4 DFS Actions

The Action Codes within the DFS category of a Generic Action management frame are defined in Table 2. The details of the frame associated with each Action Code are described in the following sub-clauses.

Table 2 - DFS Action codes

| Code | Meaning |
| --- | --- |
| 0 | Channel Measurement Request |
| 1 | Channel Measurement Report |
| 2-255 | Reserved |

Insert clause 7.4.1 as follows. Renumber figures and tables as appropriate:

### 7.4.1 Channel Measurement Request frame format

The Channel Measurement Request frame uses the Generic Action frame format and is transmitted by a STA requesting another STA to measure one or more channels. The format of the Channel Measurement Request frame body is shown in Figure 10.

|  | Category (0) | Action Code (0) | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | Request elements | Shall contain one or more Basic or Extended Channel Measurement Request elements |  |  |
| Octets: | variable |  |  |  |

Figure 10 - Channel Measurement Request frame body

The Category field shall be set to 0 (representing DFS).

The Action Code field shall be set to 0 (representing a Channel Measurement Request frame).

The Action Specific field shall be set equal to an activation delay, which represents the number of TBTTs until the measurement specified by the first Basic or Extended Channel Measurement Request element should start. A value of 0 indicates that processing shall commence immediately. A value of 1 indicates that processing shall commence after the next TBBT. Measurements specified by subsequent Basic or Extended Channel Measurement Request elements shall occur sequentially.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the request to identify the request/report transaction. A pending request is superseded by a subsequent request from the same STA with the same Dialog Token value.

The Basic and/or Extended Channel Measurement Request elements shall be set as described in 7.3.2.25 and 7.3.2.27 respectively.

Insert clause 7.4.2 as follows. Renumber figures and tables as appropriate:

### 7.4.2 Channel Measurement Report frame format

The Channel Measurement Report frame uses the Generic Action frame format and is transmitted by a STA in response to a Channel Measurement Request frame or by a STA autonomously providing channel measurement information. The format of the Channel Measurement Report frame body is show in Figure 11.

|  | Category (0) | Action Code (1) | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | Report elements | Shall contain one or more Basic or Extended Channel Measurement Report elements |  |  |
| Octets: | variable |  |  |  |

Figure 11 - Channel Measurement Report frame body

The Category field shall be set to 0 (representing DFS).

The Action Code field shall be set equal to 1 (representing a Channel Measurement Report frame).

The Action Specific field shall contain the status of the response as shown in Table 3.

The Dialog Token field shall be set equal to the value in the corresponding Channel Measurement Request frame. If the Channel Measurement Report frame is not being transmitted in response to a Channel Measurement Request frame then the Dialog token shall be set equal to zero.

The Basic and/or Extended Channel Measurement Report elements shall be set as described in 7.3.2.26 and 7.3.2.28 respectively.

Table 3 - Channel Measurement Report Action Specific codes

| Value | Status |
| --- | --- |
| 0 | The frame contains one or more Basic or Extended Channel Measurement Report elements in response to one or more Basic or Extended Channel Measurement Request elements in a Channel Measurement Request. |
| 1 | The frame contains one or more Basic or Extended Channel Measurement Report elements provided autonomously. |
| 2 | The frame indicates that a STA received one or more Extended Channel Measurement Request elements in a Channel Measurement Request frame for which it is unable to provide a response. The frame contains the corresponding Extended Channel Measurement Request elements. This status is not available for use for a Basic Channel Measurement Request or for those Extended Channel Measurement Requests that are described as mandatory. |
| 3-255 | Reserved. |

Editors note: Need to put dimensioning arrows in figure

Insert sub-clause 9.11 as follows. Renumber figures and tables, as appropriate:

Editors note: We need to decide if 9.11 and 9.12 are part of clause 9 or clause 11

## 9.11 TPC procedures

The CEPT Member Administrations in Europe require transmit power control (TPC) for wireless LANs operating in the 5GHz band to reduce interference with licensed users. The current CEPT regulations specify a regulatory maximum transmit power and an average mitigation requirement in each channel (ERC/DEC/(99)23).

This clause describes TPC procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and may be useful for other purposes, such as interference reduction, range control and power optimization.

STAs shall use the TPC procedures defined in this clause if and only if the boolean MIB attribute dot11TpcEnabled is true. dot11TpcEnabled shall be set true when required by regulatory authorities. It may also be set true in other circumstances. The TPC procedures provide for the:

- Specification of regulatory and local maximum transmit power levels for channels using Country and Power Constraint elements in Beacons and Probe Responses (9.11.1).

- Selection of a transmit power for each MPDU as a function of the regulatory and local maximum transmit power levels for a channel and a defined set of rules (9.11.2).

Editor's note: we need to be careful not to put the detailed TPC regulations in the standard - they might change in the future

### 9.11.1 Specification of a regulatory and local maximum transmit power levels

Regulatory authorities are responsible for defining a regulatory maximum transmit power level and a mitigation requirement for each allowed channel in a regulatory domain. An AP in a BSS, a STA starting an IBSS or a STA actively scanning shall not transmit in a channel unless they know the TPC related regulatory requirements for the channel.

The AP in a BSS or the STA starting an IBSS shall determine a local maximum transmit power level for other STAs in the BSS or IBSS so that the average mitigation requirements are satisfied. A conservative approach is to set the local maximum transmit power level equal to the regulatory maximum transmit power level minus the average mitigation requirement. However, it may be possible to satisfy the average mitigation requirements using a higher local maximum transmit power level. A lower local maximum transmit power level may be used for other purposes, such as range control.

The regulatory maximum transmit power level is communicated to other STAs in a BSS or an IBSS using the Country element in Beacons and Probe Responses. The local maximum transmit power level is communicated to other STAs in the BSS or IBSS using a combination of the Country and Power Constraint elements in Beacons and Probe Responses.

In a BSS, the AP transmits the local maximum transmit power in all Beacons and Probe Responses. The AP may vary the local maximum transmit power level during the life of a BSS. However, it should consider network stability in deciding how often or by how much to change the local maximum transmit power level.

In an IBSS, the STA starting the IBSS shall set the local maximum transmit power level in any Beacons and Probe Responses it transmits. Other STAs shall adopt the same local maximum transmit power level for any Beacons and Probe Responses they transmit. The STAs in an IBSS shall not change the local maximum transmit power level during the life of an IBSS. A change to another channel will require a new IBSS to be established.

### 9.11.2 Selection of a transmit power

STAs shall select a transmit power for each MPDU transmitted in a channel as a function of the regulatory and local maximum transmit power levels for the channel and the following rules:

- An AP operating in a BSS shall use any transmit power less than the regulatory maximum transmit power level for the channel, with the constraint that the AP must ensure the regulatory mitigation requirement is always respected.

- STAs (non-AP) operating in a BSS or an IBSS shall use any transmit power less than the local maximum transmit power level for the channel. However, it is recommended that a STA operating in DCF mode use the highest possible transmit power within this constraint.

- STAs that are actively scanning for a BSS or an IBSS shall use any transmit power less than the regulatory maximum transmit power level for the channel, with the constraint that they must also ensure the regulatory mitigation requirement is always respected.

STAs may use various algorithms to select transmit power dynamically within the constraints imposed by these rules. However, these algorithms are not standardized.

Editor's note: Do we still want to keep the recommendation under DCF mode?

Insert clause 9.12 as follows. Renumber as appropriate.

## 9.12 DFS procedures

The CEPT Member Administrations in Europe require dynamic frequency selection (DFS) for wireless LANs operating in the 5GHz band to reduce interference with licensed users. The current CEPT regulations specify that systems should detect and avoid licensed users. Further, they require the use of channel selection algorithms that result in uniform channel spreading across a minimum number of channels (ERC/DEC/(99)23). Regulatory authorities may, in the future, specify a maximum time to detect a licensed user and a maximum time that a STA may continue transmitting in a channel after detecting a licensed user.

This clause describes DFS procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and may be useful for other purposes, such as network planning.

STAs shall use the DFS procedures defined in this clause if and only if the boolean MIB attribute dot11DfsEnabled is true. Dot11DfsEnabled shall be set true when required by regulatory authorities. It may also be set true in other circumstances. The DFS procedures provide for the:

- Measuring of channels directly by a STA or by a STA on behalf of an AP (9.12.1).

- Reporting of measurements by a STA as the result of a request or autonomously (9.12.2).

- Selection of channels and the advertising of channel switches (9.12.3).

A STA shall not scan for, start or join an IBSS when Dot11DfsEnabled is set equal to true.

Editor's note: we need to be careful not to put the detailed DFS regulations in the standard - they might change in the future

### 9.12.1 Measuring of channels

A STA may measure a channel itself. An AP may alsorequest other STAs in the same BSS to measure a channel on its behalf. The decision by a STA to measure a channel may result from an autonomous decision or regulatory requirements. The regulatory requirements will typically specify how quickly a licensed user should be detected.

Editors note: How frequently should measurements occur? Alternatively, what is the maximum time allowed by the European regulations to detect a licensed user?

An STA may attempt to use various mechanisms to quiet other STAs during a measurement it undertakes or requests another STA to undertake. For example, an AP could arrange for part of a CFP to contain no traffic from the AP’s BSS. An STA could use a CTS sent to itself. .

Editors note: Sending a CTS to itself is not currently an allowed sequence. Another suggested mechanism includes the use a Measurement Interval Control Frame (as suggested by Philips)

When requesting other STAs to measure one or more channels, an AP shall use one or more Channel Measurement Request frames containing one or more Basic or Extended Channel Measurement Request elements. The request frames may be sent using a unicast, multicast or broadcast destination address.

An AP that requests another STA to perform a channel measurement shall not transmit MPDUs to that STA while it is performing the measurement. The AP shall also not retransmit a measurement request until the measurement could be complete plus a reasonable period to account for access and processing delay.   In determining the maximum length of a measurement, a STA shall assume that any required channel switches take less than 2 ms per switch.

Editors note: Is 2 ms the right value? Should it really be a MIB varibale

### 9.12.2 Reporting channel measurements

A STA that receives a Channel Measurement Request frame from an AP in its BSS shall process each measurement request (as represented by a Basic or Extended Channel Measurement Request element) in the frame.

A STA shall carry out all measurements specified in all Basic Channel Measurement Request elements and those measurements in Extended Channel Measurement elements that are specified as mandatory. A STA may carry out measurements specified in Extended Channel Measurement elements that are specified as optional. The measurements shall be undertaken at the time specified in Channel Measurement Request frame's Action Specific field.

The results of all completed channel measurements shall be returned to the requesting AP in a corresponding number of Basic or Extended Channel Measurement Report elements, in one or more Channel Measurement Report frames with an Action Specific field equal to 0.

When a STA declines to carry out a measurement specified in an optional Extended Measurement Request element, it shall return the refused elements to the AP in one or more Channel Measurement Report frames with an Action Specific field equal to 2

A STA may autonomously report channel measurements to an AP in its  BSS using a Channel Measurement Report frame with an Action Specific field equal to 1 and containing one or more Basic or Extended Channel Measurement Report elements. The Extended Channel Measurement Report elements shall contain sufficient information for the STA receiving the frame to understand the measurement results that are being reported.

### 9.12.3 Selection of channels

An AP may use received Basic and Extended Channel Measurement Report elements, and other information, including local measurements, to decide to stay in the current channel or move to a new channel. The selection of the new channel may use information in Supported Channel elements. However, the selection shall satisfy regulatory uniform spreading rules.

A STA shall not actively scan for a BSS and an AP shall not transmit Beacons or Probe Responses in a channel unless it knows the DFS related regulatory requirements for the channel and it takes responsibility for respecting the regulations.

Editors note: the idea is that STAs may not propagate a BSS using Beacons or Probe Responses unless it respects the DFS regulatory requirements by arranging for appropriate channel measurements and unless it  leaves the channel if a licensed user is found

A STA that does not takes responsibility for respecting the DFS regulations for a channel shall not   transmit any MPDUs in a channel when it has not received any MPDUs from its AP in a BSS for a period of dot11MaxDfsQuietTime (for the channel). This restriction means that an STA waking up may have to wait until it hears a transmission from its AP before transmitting a frame.

Editors note: the idea is that other STAs can only continue to transmit in a channel while STAs taking responsibility for respecting the DFS regulations continue to transmitIn the case of a BSS any transmission from an AP (who by definition must be taking responsibility for respecting the DFS regulations) infers that it is still OK to transmit.

A STA that takes responsibility for respecting the DFS regulations shall ensure all its transmissions in a channel have stopped within a period of dot11MaxDfsContinueTime (for the channel) of detecting a licensed user in the channel.

Editors note: these mechanisms should ensure that no one is left transmitting in a channel dot11MaxDfsContinueTime + 2 * dot11MaxDfsQuietTime after a licensed user is detected.

Editors note: an underlying assumption is that we have at least a few TBTTs to leave a channel after discovering a licensed user. Is this true? If not then we have a fundamental problem.

An AP may use a Channel Switch Announcement element in Beacons and Probe Responses to advertise to other STAs when a channel switch will occur and the new channel. The channel switch should be scheduled, if possible, so that all STAs in the BSS, including those in sleep mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch. An AP operating a BSS that switches channels may either continue the operation of the BSS in the new channel or start a new BSS in the new channel.

Editors note: it is not always possible to notify a channel switch to others if the Beacons are lost or STAs are sleeping. Regulations may also limit the time a STA can transmit in a channel once a licensed user has been detected.

Editors note: we should probably put in some text somewhere about the use of the supported channels element. At the moment it is only described implicitly in 7.2.3.4, 7.2.3.6 and 7.3.2.28.

## 10.3 MLME SAP interface

Editors note: Need appropriate changes here – but probably not worth doing until the basic features in the draft are decided

### 17.2.2 TXVECTOR parameters

Editors note: Need appropriate changes here to allow specification of tx power for TPC and measurement requests for DFS

### 17.2.3 RXVECTOR parameters

Editors note: Need appropriate changes here to allow specification of measurement reports for DFS. This could be a little tricky given that we do not know what many of the measurements are likely to be.

# Annex A – PICS

Editors note: probably not worth completing until the basic details of the draft agreed

### A.4.3 IUT configuration

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF9 | Is TPC operation supported? | ? | O.4 | Yes, No |
| *CF10 | Is DFS operation supported? | ? | O.4 | Yes, No |

### A.4.11 TPC extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TP1 |  |  |  |  |
| TP2 |  |  |  |  |
| TP3 |  |  |  |  |
| … |  |  |  |  |
| TPn |  |  |  |  |

### A.4.12 DFS extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| DF1 |  |  |  |  |
| DF2 |  |  |  |  |
| DF3 |  |  |  |  |
| … |  |  |  |  |
| DFn |  |  |  |  |

# Annex C – Formal description of MAC operation

Editors note: the formal description in the 802.11 MAC is filled with syntactic and semantic errors. 802.11h are not proposing to add anything to this very flawed description unless it is verified first.

# Annex D – ASN.1 encoding of the MAC and PHY MIB

Editors note: initial attempt at ASN.1 coding

Add after dot11CountryString

dot11TpcCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“A STA will use the defined TPC procedures if and only if this attribute is TRUE. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 24 }

dot11DfsCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“A STA will use the defined DFS procedures if and only if this attribute is TRUE. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 25 }

Also need to add

- dot11MitigationRequirement for each channel – default 3dB – not sure on syntax yet

- dot11MaxDfsContinueTime for each channel – default not ecided yet – not sure on syntax yet

- dot11MaxDfsQuietTime for each channel – default not decided yet - not sure on syntax yet

- dot11SpectrumManagementImplemented

Who has the skills to construct the ASN.1 description?

# Annex E – Bibliography

Editors note: none so far

# Annex F – Extended Measurement Requests and Reports in DFS

Clause 7.3.2.26 defines an element in which a STA can request another STA to make an extended channel measurement request. Clause 7.3.2.27 defines an element in which an STA can report the results of an extended channel measurement, either as the result of a request or autonomously.

A Measurement ID identifies the format of the extended measurement requests and responses. The IEEE allocates the Measurement ID space. This annex defines the semantics of the currently defined Measurement ID’s as shown in Table 4.

Editors note: suggestions for other definitions invited

Table 4 – Measurement ID definitions

| Measurement Request or Report | Measurement ID | Purpose |
| --- | --- | --- |
| RSSI Histogram Request | 0 | Request measurement of a channel and return of an RSSI histogram |
| RSSI Histogram Report | 1 | Report the results of a channel measurement as an RSSI histogram |

## F.1 RSSI Histogram Request

A Measurement ID equal to 0 in the Extended Channel Measurement Report element indicates an RSSI Histogram Request. The Measurement Request field corresponding to an RSSI Histogram Request is null. A response to an RSSI Histogram Request is a RSSI Histogram Report. It is optional for a STA to respond to this request.

## F.2 RSSI Histogram Report

A Measurement ID equal to 1 in the Extended Channel Measurement Request element indicates an RSSI Histogram Report.

The format of the Measurement Report field corresponding to an RSSI Histogram Report is shown in Figure 12.

|  | RSSRI 0 density | RSSRI 1 density | RSSRI 2 density | RSSRI 3 density | RSSRI 4 density | RSSRI 5 density | RSSRI 6 density | RSSRI 7 density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 12 - Measurement Report field format for a RSSI Histogram Report

The RSSRI Histogram Report contains the densities over the measurement period for eight RSSRIs as defined in Table 2. The density for an RSSRI is defined as Ceiling (255 * [Period receiving at RSSRI / Measurement Period]).

Table 2 RSSRI definitions

| RSSRI | Power Observed at the Antenna (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 0 | Power  -87 | +5 |
| 1 | -87 < Power  -82 | ±5 |
| 2 | 82 < Power  -77 | ±5 |
| 3 | -77 < Power  -72 | ±5 |
| 4 | -72 < Power  -67 | ±5 |
| 5 | -67 < Power  -62 | ±5 |
| 6 | -62 < Power  -57 | ±5 |
| 7 | -57 < Power | -5 |

# Annex G – Using TPC and DFS facilities for other purposes

Editors note: it has been suggested that an informative annex be added which describes how TPC and DFS could be used by other PHYs or outside Europe.

ERC documents are available from European Radiocommunications Office, Midtermolen 1, DK-2100, Copenhagen, Denmark (http://www.ero.dk).