IEEE P802.11
Wireless LANs

Normative Text for TXOP Ending rule

Date:                                         November 13, 2001

Author:          S. Kandala1, S. Choi2, K. Nakashima1, Y. Ohtani1, S. Terada1, J. del Prado2

1Sharp Corporation, 2Philips
 
                       e-Mail: srini@sharplabs.com & sunghyun.choi@philips.com

Abstract

This document contains the material proposed to Tge for inclusion in the draft in the form of insertions into and replacements for material in of IEEE std 802.11-1999, as updated by IEEE STd 802.11e/D1.3, October 2001.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Changes to existing text in current standard are shown underlined and in red for additions and red with strikethrough for deletions.

Modify the last paragraph of Clause 9.10.2 immediately above clause 9.10.2.1 as follows:

Any frame exchange sequence containing a management type frame as well as any frame exchange sequence containing a data type frame of any subtype that does not include QoS, must the the sole or final frame exchange sequence in the TXOP. Multiple frame exchange sequences containing only control frames and QoS data type frames can be sent in a single TXOP, with the Non-final bit in the QoS control fields of all but the final such frame exchange sequence set to 1 to indicate that the ESTA intends to send another frame during the TXOP. If an expected Ack response does not occur to a frame with Non-Final set to 1, the ESTA awaiting the Ack should send either its next frame exchange sequence, or a QoS Null, starting a PIFS period after the end of the last transmission. When the Non-Final bit is set to 0 and the sender is a WSTA, the recipient of the frame shall be HC. If there are no frames to be sent to the HC, then the ESTA shall send a QoS Null with No Ack bit set to 0 to the HC.  If the WSTA does not receive the expected Ack response for this final frame, it shall retransmit the frame or transmit a QoS Null frame, with Non-Final bit set to 0 and No Ack bit set to 0, starting a PIFS period after the end of last transmission, until such time that it receives an acknowledgement or when there is not enough duration remaining in the TXOP for sending such a frame. This is to avoid the situation where the HC may not receive the frame and may result in an inefficient use of the channel. If the HC does not have any more ESTAs to be polled, it shall send a QoS CF-Poll with the RA matching the HC MACaddress and Duration/ID equal to zero.

Delete the third paragraph of clause 9.10.2.1 as the conditions that cause the behavior described therein will not occur.