IEEE P802.11
Wireless LANs

Recommended Normative Text for Burst Ack

Date: 	January 23, 2002

Author:  	Tge Burst Ack ad hoc group

leader: 	Srinivas Kandala (srini@sharplabs.com)

Sharp Labs of America, Inc.
5750 NW Pacific Rim Blvd., Camas, WA 98607 USA
Phone: (360) 817-7512 
Fax: (360) 834-8696
e-Mail: srini@sharplabs.com

Abstract

This document contains the material proposed to Tge for inclusion in the draft in the form of insertions into and replacements for material in of IEEE std 802.11-1999, as updated by IEEE STd 802.11e/2.0a, December 2001 and 02/004r4.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Changes to existing text in current standard are shown underlined and in red for additions and red with strikethrough for deletions.

Instruct the editor to make the following changes in clause 7.1.3.1

#### 7.1.3.1	Frame Control field

The Frame Control field consists of the following subfields: Protocol Version, Type, Subtype, To DS, From DS, More Fragments, Retry/No Ack, Power Management, More Data/Burst Ack Policy, and Wired Equivalent Privacy (WEP). Bit 15 is reserved and should be set to 0 in transmitted frames and ignored in received frames. The format of the Frame Control field is illustrated in Figure 13.  The frame control field shall always be taken as the first and second octets of any received frame.

| B0 | B1 | B2 | B3 | B4 |  |  | B7 | B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protocol Version |  | Type |  | Subtype |  |  |  | To DS | From DS | More Frag | Retry/ No Ack | Pwr Mgt | More Data/ Burst Ack | WEP | rsrv (0) |
| bits: 2 |  | 2 |  | 4 |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 13 – Frame Control field

Instruct the editor to make the following changes in clause 7.1.3.1.6

7.1.3.1.6 Retry/No Ack field

Bit 11 shall be interpreted as the Retry field for IEEE 802.11-1999 devices and as No Ack field for IEEE 802.11e.

For devices conformant to the IEEE 802.11-1999 standard, the Retry field is 1 bit in length and is set to 1 in any data or management type frame that is a retransmission of an earlier frame. It is set to 0 in all other frames. A receiving station uses this indication to aid in the process of eliminating duplicate frames.

For devices that are conformant to IEEE 802.11e, the No Ack field is 1 bit in length and is set to 0 to indicate that the frame shall be responded by the recipient with the ACK or QoS (+)CF-Ack frame after a SIFS period, according to the procedures defined in 9.2.8, 9.3.3 and 9.10.3. It is set to 0 in all other frames.

Instruct the editor to make the following changes in clause 7.1.3.1.8

##### 7.1.3.1.8	More Data/Burst Ack field

Bit 13 shall be interpreted as More Data field for for IEEE 802.11-1999 devices and as Burst Ack field for IEEE 802.11e.

##### 7.1.3.1.8.1 More Data field

For devices conformant to the IEEE 802.11-1999 standard, the More Data field is 1 bit in length and is used to indicate to a STA in power-save mode that more MSDUs, or MMPDUs are buffered for that STA at the AP.  The More Data field is valid in unicast data or management type frames transmitted by an AP to an STA in power-save mode.  A value of 1 indicates that at least one additional buffered MSDU, or MMPDU is present for the same STA.

The More Data field may be set to 1 in unicast data type frames transmitted by a contention-free (CF)-Pollable STA to the Point Coordinator (PC) in response to a CF-Poll to indicate that the STA has at least one additional buffered MSDU available for transmission in response to a subsequent CF-Poll.

The More Data field is set to 0 in all other unicast frames.

The More Data field is set to 1 in broadcast/multicast frames transmitted by the AP, when additional broadcast/multicast MSDUs, or MMPDUs, remain to be transmitted by the AP during this beacon interval.  The More Data field is set to 0 in broadcast/multicast frames transmitted by the AP when no more broadcast/multicast MSDUs, or MMPDUs, remain to be transmitted by the AP during this beacon interval and in all broadcast/multicast frames transmitted by non-AP stations.

##### 7.1.3.1.8.2 Burst Ack Field

For the devices that are conformant to the IEEE 802.11e, the Burst Ack field is 1 bit in length and is used to indicate the burst acknowledgement policy and is dependent on the value of the No Ack field in bit 11.

The permitted Ack/Burst Ack bit combination and their meanings in the QoS Data subtype are given in Table 2.5

Table 2. 5 - Ack/Burst Ack combination in QoS data frames

| Bit 11 (No Ack) | Bit 13 (Burst Ack) | Meaning |
| --- | --- | --- |
| 0 | 0 | Normal IEEE 802.11 acknowledgement. The addressed recipient returns an ACK or QoS (+) CF-ACK frame after a SIFS period, according to the procedures defined in 9.2.8, 9.3.3 and 9.10.3 |
| 0 | 1 | Delayed Burst Acknowledgement The addressed recipient returns an ACK or QoS (+) CF-ACK frame after a SIFS period, according to the procedures defined in 9.2.8, 9.3.3 and 9.10.3. The addressed recipient shall also return a BurstAck frame in a subsequent TXOP. |
| 1 | 1 | Immediate Burst Acknowledgement The addressed recipient returns either a BurstAck frame after a SIFS period, according to the procedures defined in 9.10.5 or shall return an ACK frame after SIFS period, with the Burst Ack field bit set to 1. |
| 1 | 0 | No Acknowledgement The addressed recipient takes no action. The user may however need to maintain a record if burst ack is set up for the traffic through management frame exchange. |

In the ACK frames, this bit shall be set to 1 to indicate the retransmission of the frame or transmission of a QoS Null with the same TID and both No Ack and Burst Ack bits set to 1 and 0 otherwise. In all other frames, this bit is reserved and shall be set to 0.

Instruct the editor to modify Table 3.5 as shown below:

Table 3.5 – QoS Control field

| Bits 0-8 | Bit 9 | Bit 10 | Bit 11 | Bits 12-15 | Usage |
| --- | --- | --- | --- | --- | --- |
| TXOP limit, units of 16 microseconds | FEC | Non-final | rsrv (0) | TID | QoS data type frames that include CF-Poll sent by the HC |
| reserved (0) | FEC | Non-final | rsrv (0) | TID | QoS data type frames without CF-Poll sent by the HC |
| Queue size, units of 128 octets | FEC | Non-final | rsrv (0) | TID | QoS data (non-null) frames sent by WSTAs |
| bit 15 =0: TXOP duration requested, units of 16 microseconds | 0 | Non-final | rsrv (0) | TID | QoS null frames sent by WSTAs |
| bit 15 =1: Queue size, units of 128 octets | 1 | Non-final | rsrv (0) | TID |  |
| bit 15 =0: TXOP duration requested, units of 16 microseconds | 0 | rsrv (0) | rsrv (0) | TID | RR frames |
| bit 15 =1: Queue size, units of 128 octets | 1 | rsrv (0) | rsrv (0) | TID |  |

Delete subclause 7.1.3.5.2 on No Ack.

Instruct the editor to make the following changes in subclause 7.2.1.3

When a QSTA receives a data frame with No Ack /Burst Ack subfield combination is set to 11, then the QSTA may respond with either an ACK frame or a Burst Ack frame. If the recipient responds with an ACK frame, it shall set the Burst Ack subfield to 1. The Burst Ack bit shall be set to 0 in all other frames.

Delete subclause 7.2.1.7.

Instruct the editor to make the add the following paragraph at the end of Clause 7.2.1.8:

#### 7.2.1.8 Burst  Acknowledgement (BurstAck) frame format

The frame format of the Burst Acknowledgement (BurstAck) frame is defined in Figure 21.4.

| Octets: 2 | 2 | 2 | 2 | 2 |  |  | 2 | 256 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | RA | TA | Bits: 0-7 | 8-10 | 12-15 | Starting Sequence Control | Ack Bitmap | FCS |
|  |  |  |  | Re-ordering Buffer Size | reserved | TID |  |  |  |

Figure 21.4 Burst Ack Frame Format

The Duration, RA and TA fields follow the rules defined in section 7.2.1.3 for the ACK frame. The Re-ordering buffer size indicates the number of of buffers of size 2304 octets available for bursting for this particular TC or TS (see 7.4.3.2). The TID field contains the value of the TCID or TSID for which the acknowledgement is being sent.

The Starting Sequence Control field contains the fragment number and sequence number of the first MPDU for traffic TID corresponding to bit 0 of the Ack Bitmap.  The Ack Bitmap field contains a bitmap that can acknowledge up to 128 MSDUs.

The Starting Sequence Control field defines an MPDU sequence number equal to ( Sequence Number * 16) + Fragment Number. Bit position n of the bitmap, if set to a 1 acknowledges MPDU with MPDU sequence number equal to (Starting Sequence Control + n).

Instruct the editor to make the following changes in the first paragraph of subclause 7.3.1.4:

The Capability Information field contains a number of subfields that are used to indicate requested or advertised capabilities. The length of the Capability Information field is 2 octets. The Capability Information field consists of the following subfields: ESS, IBSS, CF-Pollable, CF-Poll Request, Privacy, PBCC, Channel Agility, QoS, FEC, Bridge Portal, Burst Ack and Extended Capability Element. The remaining bits in the Capability Information field are reserved. The format of the Capability Information field is as illustrated in Figure 27.

| bits: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ESS | IBSS | CF- pollable | CF-poll request | Privacy | Short preamble | PBCC | Channel agility | QoS | FEC | Bridge Portal | rsrv (0) | rsrv (0) | Rsrv (0) | Burst Ack | Extended capability element |

Figure 27 – Capability Information fixed field

Insert the editor to insert the following above the last paragraph in clause 7.3.1.4:

WSTAs set the Burst Ack subfield to 1 within transmitted Association and Reassociation Request management frames if it supports burst ack. APs set the Burst Ack subfield to 1 within transmitted Beacon, Probe Response, Association Response and Reassociation Response. Otherwise, it shall be set to 0.

Instruct the editor to make the following changes in Table 20.1 of subclause 7.3.2.15:

Table 20.1 – Ack Policy field encoding

| Bit 2 | Bit 3 | Usage |
| --- | --- | --- |
| 0 | 0 | Normal IEEE 802.11 acknowledgement. The addressed recipient returns an ACK or QoS (+)CF-Ack frame after a SIFS period, according to the procedures defined in 9.2.8, 9.3.3 and 9.10.3. |
| 1 | 0 | Immediate Burst Acknowledgement The addressed recipient returns either a BurstAck frame after a SIFS period, or shall return an ACK frame after SIFS period, with the Burst Ack field bit set to 1, according to the procedures defined in 9.10.5. |
| 0 | 1 | Delayed Burst Acknowledgement The addressed recipient returns an ACK or QoS (+) CF-ACK frame after a SIFS period at the end of the burst, according to the procedures defined in 9.2.8, 9.3.3, 9.10.3 and 9.10.5. The addressed recipient shall also return a BurstAck frame in a subsequent TXOP. |
| 1 | 1 | No acknowledgement The recipient(s) shall not acknowledge the transmission, and the sender treats the transmission as successful without regard for the actual result. |

Instruct the editor to make the following changes in Table 20.3

Table 20.3 – QoS Action codes

| Code | Meaning |
| --- | --- |
| 0 | Add TS request |
| 1 | Add TS response |
| 2 | Delete TS |
| 3 | reserved |
| 4 | Define Burst Ack request |
| 5 | Define Burst Ack response |
| 6 | Delete Burst Ack request |
| 7 – 255 | Reserved |

Instruct the editor to add the following subclause at the end of subclause 7.4

### 7.4.3 Define Burst Ack QoS Action frame format

#### 7.4.3.1 Define Burst Ack request QoS Action frame format

The frame body of a Define Burst Ack request QoS Action frame is shown in 42.18.1. The dialog token can be of any value but should be used in all future QoS action frame exchanges. The Ack policy bit is set to 1 if an immediate Burst Ack is desired and 0 if a delayed Burst Ack is desired. TID contains the value of the TC or TS for which the burst ack is being requested. The Starting Sequence Control shall have the sequence number and the fragment number of the first MPDU for which the burst acknowledgement is desired.

| octets: 1 | 1 | 1 | 1 | 2 |  |  | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (Category QoS) | 4 (Define Burst Ack request) | 0 (no activation delay) | Dialog Token | Bits: 8-10 | 11 | 12-15 | Starting Sequence Control |
|  |  |  |  | reserved | Ack Policy | TID |  |

Figure 42.18.1 Define Burst Ack request QoS Action frame body

A Define Burst Ack request QoS Action frame is used to initiate burst acknowledgement for a specific TC or TS between the SA and RA in the header. A response is required for this frame from the recipient of the frame. A Define Burst Ack request QoS Action frame may also be used to reset the Starting Sequence Control field.

#### 7.4.3.2 Define Burst Ack response QoS Action frame format

The frame body of a Define Burst Ack response QoS Action frame format is shown in Figure 42.18.2. This frame is sent either in response to a Define Burst Ack request QoS Action frame or is sent unsolicited if the receiver of the traffic desires to change the parameters. The dialog token should be copied from the original Define burst Ack request Action frame format. The Reject subfield is set to 1 if the receiver is unable to accommodate the burst ack request and it is set to 0 indicating the receiver’s readiness to participate in the burst ack. The Ack policy bit is set to 1 for immediate Burst Ack and 0 for delayed Burst Ack. The Ack policy may either be same as in the Define Burst Ack Request QoS Action frame or may be changed.

| octets:1 | 1 | 1 | 1 | 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (Category QoS) | 5 (Define Burst Ack response) | 0 (no activation delay) | Dialog Token | Bit: 0-1 | 2 | 3 | 4-7 | 8-15 |
|  |  |  |  | reserved | Reject | Ack Policy | TID | Re-ordering Buffer Size |

Figure 42.18.2 Define Burst Ack response QoS Action frame body

TID contains the value of the TC or TS for which the burst ack is being requested. The Re-ordering buffer size indicates the number of of buffers of size 2304 octets available for bursting for this particular TC or TS.

Note: The receiver advertizes a single scalar number, N, which is the numberof full-size (2304 octets) buffers available for bursting. The semantics are that every MPDU will consume one of these full-length buffers whether the frame is an MSDU or whether it is a fragment. In other words, ten full-size MSDUs will consume the same amount of buffer space at the receiver as 10 small fragments.

#### 7.4.3.3 Delete Burst Ack request QoS Action frame format

The frame body of a Delete Burst Ack request QoS Action frame format is shown in Figure 42.18.3. This frame is sent to terminate the burst ack participation by either the originator of the traffic or the recipient. There is no response QoS action frame and the immediate acknowledgement that is sent by the receiver of this frame is considered as a positive response.

| octets: 1 | 1 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| 1 (Category QoS) | 4 (Define Burst Ack request) | 0 (no activation delay) | Dialog Token | TAID |

Figure 42.18.3 Define Burst Ack request QoS Action frame body

The dialog token should be copied from the original Define burst Ack request Action frame format. The TAID field indicates TID as well as the direction of the traffic for which the deletion is desired. No response is required for this frame.

Instruct the editor to make the following changes in Clause 9.10.5:

### 9.10.5 Burst Acknowledgment

#### 9.10.5.1 Introduction

The Burst Acknowledgement mechanism allows a burst of QoS DATA MPDUs to be transmitted separated by a SIFS period.  The mechanism provides solutions to two different problems : improve the channel efficiency by aggregrating several acknowledgements into one frame and allow for the decoding delay for FEC encoded frames.  Consequently, there are two types of burst ack mechanisms: immediate and delayed. Immediate burst ack is suitable for high-bandwidth, low latency traffic while the delayed burst ack is suitable for applications that can tolerate moderate latency.

Note: The delayed burst mechianism is primarily intended to allow the existing implementations to use this feature with minimal hardware changes or to allow inexpensive implementations that would use the processing power on the host.

The burst ack mechanism is set up by initialization through an exchange of Define Burst Ack QoS Action request/response frames. Once the initialization is done the MPDUs containing the QoS data frame bursts can be sent from the sender to the receiver. The burst can be started within a polled TXOP or by winning EDCF contention. The MPDUs within this exchange usually fit within a single TXOP and are all separated by a SIFS. The burst length is limited, and the amount of state that must be kept by the receiver of the DATA MPDUs is bounded.

It does not require a traffic-spec, however QSTAs using the TS facility may choose to signal their intention to use burst ack for the scheduler’s consideration in assigning transmission opportunities.  It should be noted that if a burst is sent after winning an EDCF contention, the burst within this EDCF TXOP can be composed of fragments of a single MSDU. However, acknowledgements of frames belonging to the same TID, transmitted during multiple TXOPs may also be combined.

The originator is given great flexibility regarding the ordering. The originator can split bursts across TXOPs, separate the burst and the burst acknowledgement exchange, interleave bursts for different TIDs or RAs.

Delete subclauses 9.10.5.2 and 9.10.5.3. Insert the following subclauses:

#### 9.10.5.2 Set up and modification of the Burst Ack parameters

The transmitting QSTA that has data to send and intends to use Burst Ack facility should first check if the receiver is capabile of participating. If the receiver is capable of participating, the transmitter sends a Define Burst Ack QoS Action request frame indicating the TID and the type of burst ack (immediate or delayed) that it desires. The receiving QSTA will respond through a Define Burst Ack QoS Action response frame. The receiving QSTA has the option of accepting or rejecting. If it accepts, then it also has the option to continue with the burst ack type that the transmitter desires or change it. This change may happen when the transmitter indicates that it expects an immediate Burst Ack but the receiver does not have the capability to support the immediate Burst Ack. When the receiver QSTA accepts, it also indicates the amount of resources in terms of buffer size that it will allocate for the support of this burst. If the QSTA rejects, then the transmitting shall not invoke burst ack mechanism and will use either the IEEE 802.11 acknowledgement or shall not rely on acknowledgements.

Once the burst ack has been set up then the data and acknowledgements are transferred following the procedure described in clause 9.10.5.3. During such transfer, however, the receiver has an option of sending the Define Burst Ack QoS response frame, unsolicited, to change either the type of burst ack or to change the amount of buffer that will be allocated.

#### 9.10.5.3 Data and acknowledgement transfer

After setting up the burst exchange following the procedure in 9.10.5.2, the transmitter shall transmit a burst of MPDUs separated by SIFS period, not exceeding the Re-ordering buffer size subfield in the associated Define Burst Ack response QoS action management frame. Each of the MPDUs except the last one shall have the No Ack/Burst Ack subfield combination set to 10. The last MPDU in the burst shall have the No Ack/Burst Ack subfield combination set to 11 if immediate Burst Ack policy is used, else the No Ack/Burst Ack subfield combination shall be set to 01. The receiver shall maintain a burst Ack record for the burst.

Subject to any constraints herein about permitted use of TXOP according to the channel access mechanism used: the originator can separate the burst and the BurstAckReq into separate TXOPs; the originator can split a burst across multiple TXOPs; the originator can sequence bursts with different TIDs in the same TXOP; the originator can interleave MPDUs from bursts with different TIDs within the same TXOP; the originator can sequence or interleave MPDUs for different RA within a TXOP.

The duration values of burst DATA MPDUs and any burst Ack exchange transmitted within a polled TXOP shall follow the rules defined in 9.2.2.2.

The duration rules during an EDCF TXOP shall be as follows: the duration field of any burst data shall protect any following transmitted MPDU and its response MPDU if there is one; the duration field of the final frame of the burst shall protect at least the BurstAck frame.  In this context “protect” means that the duration value causes the NAV to expire at the end of the protected MPDU.

If the immediate Burst Ack policy is used, the receiver shall respond to the final frame in the burst (the frame that has the No Ack/Burst Ack subfield combination is set to 11) with either the Burst Ack frame or with an ACK frame with the Burst Ack bit set to 1. If the receiver of the burst sends the Burst Ack, the transmitter will update its own record and will retry any frames that are not acknowledged in the Burst Ack either in another burst or individually. If the receiver of the burst sends an ACK frame with Burst Ack bit set to 1, then the transmitter will immediately transmit a QoS Null with the Ack/Burst Ack subfield combination set to 11 and will continue to seek the acknowledgement either in the same TXOP or in a subsequent TXOP if there is not enough time in the current TXOP.

If the delayed Burst Ack policy is used, the receiver shall respond to the final frame in the burst (the frame that has the No Ack/Burst Ack subfield combination is set to 01) with an ACK frame. The receiver shall then resend the Burst Ack in a subsequently obtained TXOP. The transmitter shall respond an IEEE 802.11 acknowledgement upon the receipt of the Burst Ack frame.

If a burst ends prematurely without the final frame or in the case one has been sent by the transmitters which has not been received at the receiver, the receiver has the option of sending a Burst Ack frame unsolicited.

The BurstAck contains acknowledgements for the MPDUs of up to 128 previous MSDUs.   If the BurstAck indicates that an MPDU was not received correctly, the originator shall retry that MPDU subject to that MPDU’s apprpriate retry limit.

A typical burst ACK frame exchange sequence using the immediate burst ack, for a single TID is shown in figure 62.3.

A typical burst ACK sequence in which the response is achieved subsequently is shown in Fig 62.4

If there is no response (i.e., neither a BurstAck nor an ACK) to final frame, the originator can retransmit the final frame of the burst within the current TXOP (if time permits) or within a subsequent TXOP.  This retransmission is subject to the short retry limit.  If this final frame is discarded due to reaching its retry limit, all MPDUs in the burst are considered to have failed transmission and are discarded.

Retransmitted burst data MPDUs shall preserve their original relative order.

#### 9.10.5.4 Tear down of the Burst Ack mechanism

#### When the transmitter has no data to send it shall signal the end of the bursts by sending the Delete Burst Ack QoS Action Management frame to the receiver of the burst. There will not be any response from the receiver except for the IEEE 802.11 acknowledgement to this management frame.