IEEE P802.11
Wireless LANs

TPC Status

Date	March 2002

Author	Andrew Myles (802.11h co-editor)
	Cisco Systems
	Level 2, 3 Innovation Road,
	North Ryde, 2113, Australia
	Phone: +61 2 84461010
	Fax: +61 2 84461001
	e-Mail: andrew.myles@cisco.com

Abstract

The TPC aspects of 802.11h are mostly agreed. The purpose of this submission is to provide a snap shot of the TPC text, without the distraction of DFS, to ensure all TPC agreements have been captured in the text. The group is requested to point out any errors before the St Louis meeting so that corrections can be made.

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

Spectrum and Transmit Power Management
extensions in the 5GHz band in Europe

Sponsored by the
IEEE 802 Committee
of the
IEEE Computer Society

Copyright (c) 2001 by the Institute of Electrical and Electronics Engineers, Inc.
345 East 47th Street
New York, NY 10017, USA
All rights reserved.

This is an unapproved draft of a proposed IEEE Standard, subject to change. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities. If this document is to be submitted to ISO or IEC, notification shall be given to the IEEE Copyright Administrator. Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for purposes of developing a national position. Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license. Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department
Copyright and Permissions
445 Hoes Lane, P.O. Box 1331
Piscataway, NJ 08855-1331, USA

# Introduction

(This introduction is not part of IEEE P802.11h, Draft Supplement to STANDARD FOR Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications: DFS and TPC extensions in the 5GHz band in Europe)

Example:

At the time this standard was completed, the working group had the following membership:

Chair of working group (to be provided by IEEE editor at time of publication)

List of members of working group (to be provided by IEEE editor at time of publication)

The following persons were on the balloting committee:

List of members of balloting committee (to be provided by IEEE editor at time of publication)

# Contents

Introduction	ii

Contents	iii

2. Normative References	2

3. Definitions	2

4. Abbreviations and acronyms	2

5.5 Relationships between services	2

5.7.2 Association	2

5.7.3 Reassociation	3

7.1.3.1.2 Type and Subtype fields	3

7.2.3.1 Beacon frame format	4

7.2.3.4 Association Request frame format	5

7.2.3.5 Reassociation Request frame format	5

7.2.3.9 Probe Response frame format	5

7.2.3.12 Action frame body format	6

7.3.1.4 Capability Information field	7

7.3.1.7 Reason Code field	8

7.3.1.9 Status Code field	8

7.3.2 Information elements	8

7.3.2.8 Power Constraint element	9

7.3.2.9 Power Capability element	10

7.3.2.10 TPC Request element	11

7.3.2.11 TPC Report element	11

7.4 Spectrum Management Actions	13

7.4.3 TPC Request frame format	14

7.4.4 TPC Report frame format	15

9.11 TPC procedures	16

9.11.1 Association based on transmit power capability	16

9.11.2 Specification of a regulatory and local maximum transmit power levels	17

9.11.3 Selection of a transmit power	17

9.11.4 Optimization of the transmit power	18

10.3 MLME SAP interface	19

17.2.2 TXVECTOR parameters	19

17.2.3 RXVECTOR parameters	19

17.3.8.3.3 Channelization	19

17.3.9.1 Transmit power levels	20

Annex A – PICS	21

A.4.3 IUT configuration	21

A.4.11 TPC extensions	21

A.4.12 DFS extensions	21

Annex C – Formal description of MAC operation	22

Annex D – ASN.1 encoding of the MAC and PHY MIB	23

Annex E – Bibliography	26

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

Spectrum and Transmit Power Management
extensions in the 5GHz band in Europe

[ The editing instructions contained in this supplement define how to merge the material contained herein into the current edition of IEEE Std 802.11 1999 Edition and the 802.11a, 802.11b, 802.11b-cor1 and 802.11d supplements

The editing instructions are shown in bold italic. Three editing instructions are used:

Change is used to make small corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed either by using strikethrough (to remove old material) or underscore (to add new material).

Delete removes existing material.

Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instructions.

Editorial notes will not be carried over into future editions.]

# 2. Normative References

Insert the following citation at the appropriate location in clause 2:

ERC/DEC/(99)23, ERC Decision of 29 November 1999 on the harmonized frequency bands to be designated for the introduction of High Performance Radio Local Area Networks (HIPERLANs) .

# 3. Definitions

Insert the following new definitions at the appropriate locations in clause 3, renumbering as necessary:

3.52 transmit power control (TPC): Facilities mandated to satisfy requirements in some regulatory domains for maximum transmit power and average transmit power mitigation in the 5GHz band. TPC facilities may also be used for other purposes, such as interference reduction, range control and power optimization.

3.53 transmit power: transmit power is the Effective Isotropic Radiated Power .

3.54 receive power: receive power is the mean power measured at the antenna connector.

# 4. Abbreviations and acronyms

Insert the following text in the appropriate location in clause 4:

TPC	transmit power control

## 5.5 Relationships between services

Add to the definition of class 1, management frames as follows:

vi) Action

### 5.7.2 Association

Insert the following text under "Association request" just after entry for "ESS ID":

- Power Capability

### 5.7.3 Reassociation

Insert the following text under "Reassociation request" just after entry for "ESS ID":

- Power Capability

##### 7.1.3.1.2 Type and Subtype fields

Change the contents of Table 1 as follows:

Table 1 - Valid type and subtype combinations

| Type Value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 1101 | Action |
| 00 | Management | 1110-1111 | Reserved |

#### 7.2.3.1 Beacon frame format

Change order 11 information field and add order 14 and order 15 information fields as follows:

Table 5 - Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11Spectrum ManagementCapabilityEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.4 Association Request frame format

Add order 5 and order 6 information fields as follows:

Table 7 - Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.5 Reassociation Request frame format

Add order 6 and order 7 information fields as follows:

Table 9 - Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.9 Probe Response frame format

Change order 10 information field and add order 14 and 15 information fields as follows:

Table 12 Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Country element shall be present if dot11MultiDomainCapabilityEnabled or dot11SpectrumManagementCapabilityEnabled is true |
| 13 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

Insert the following clause and renumber as necessary:

#### 7.2.3.12 Action frame body format

The Action frame indicates a management action from a particular category. The Action frame body is shown in Figure 1.

|  | Category | Action | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | Actions |  |  |  |
| Octets: | variable |  |  |  |

Figure 1 – Action frame body

The Category field shall be set to one of the values shown in Table 1. The details of the actions allowed in each category are described in the clause for the category referenced in Table 1.

The Action field shall be set to a value that is allowed in the Category. The most significant bit shall normally be set equal to 0. However, if a STA receives a unicast Action frame with an unrecognized Category field, Action field, Action Specific field, Dialog Token field or some other syntactic error and the most significant bit of the Action field set to 0 then the STA shall return the entire frame to the source with the most significant bit of the Action field set equal to 1.

The Action Specific field shall be set to a value that is specific to the particular Category and Action. The Action Specific field is often used to carry a status value or an activation delay.

The Dialog Token field shall be set to a value that is specific to the particular Category and Action. The Dialog Token field is normally used to associate a series of frames in a multi-management frame transaction.

The Actions field shall contain fixed fields and elements that are specific to the Category and Action.

Table 1 - Category values

| Name | Value | See clause |
| --- | --- | --- |
| Spectrum management | 0 | 7.4 |
| Reserved | 1-255 | - |

#### 7.3.1.4 Capability Information field

Change the supporting text as follows:

The length of the Capability Information field is two octets. The Capability Information field consists of the following subfields: ESS, IBSS, CF-Pollable, CF-Poll Request, Privacy, Short Preamble, PBCC, Channel Agility and Spectrum Management. The format of the Capability Information field is as illustrated in Figure 27.

Change the contents of Figure 27 as shown:

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESS | IBSS | CF Pollable | CF Poll Request | Privacy | Short Preamble | PBCC | Channel Agility |
| B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
| Spectrum Management | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) |

Figure 27 - Capability Information fixed field

Insert the following text at the end of clause 7.3.1.4:

A STA shall set the Spectrum Management subfield in the Capability Information field equal to 1 if the boolean MIB attribute dot11SpectrumManagementCapabilityEnabled is true, otherwise it shall be set equal to 0.

#### 7.3.1.7 Reason Code field

Change table 18 as follows:

Table 18 - Reason codes

| Reason Code | Meaning |
| --- | --- |
| 10 | Disassociated because the Power Capability element is missing or the Maximum Transmit Power Capability field in the Power Capability element is unacceptable |

#### 7.3.1.9 Status Code field

Change table 19 as follows:

Table 19 - Status codes

| Status Code | Meaning |
| --- | --- |
| 19 | Association request rejected because Spectrum Management capability is required |
| 20 | Association request rejected because the Power Capability element is missing or the Maximum Transmit Power Capability field in the Power Capability element is unacceptable |

### 7.3.2 Information elements

Insert the following information elements into Table 20 of clause 7.3.2 in place of the reserved IDs:

Table 20 - Element IDs

| Information element | Element ID |
| --- | --- |
| Power Constraint | 32 |
| Power Capability | 33 |
| TPC Request | 34 |
| TPC Report | 35 |
| Reserved | 11-15, 41-255 |

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.8 Power Constraint element

The Power Constraint element contains the information necessary to allow a STA to determine the local maximum transmit power level that shall not be exceeded for transmissions in various channels. The format of the Power Constraint element is shown in Figure 2.

|  | Element ID (32) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |
|  | First Channel Number | Number of Channels | Local Power Constraint | The element contains one or more triplets |  |
| Octets: | 1 | 1 | 1 |  |  |

Figure 2 - Power Constraint element format

The Element ID field shall be set equal to 32. The Length field is variable and depends on the number of triplets in the element. The minimum value of the Length field is 3.

The First Channel Number field shall indicate the first channel number in the sub-band described by the triplet. The field is coded as an unsigned integer. The First Channel Number field shall monotonically increase in successive triplets.

The Number of Channels field shall indicate the number of channels in the sub-band described by the triplet. The field is coded as a positive integer. The Number of Channels field shall not contain a value that causes the sub-bands described by successive triplets to overlap.

The Local Power Constraint field shall be set equal to a value greater than or equal to the minimum transmit power mitigation required to operate in the sub-band described by the triplet to satisfy regulatory requirements. The field is coded as an unsigned integer in units of dB with 0dB tolerance. The local maximum transmit power for a channel is thus defined as the Maximum Transmit Power Level specified for the channel in the Country element minus the Local Power Constraint specified for the channel in the Power Constraint element.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.9 Power Capability element

The Power Capability element specifies the maximum power with which a STA is capable of transmitting in the various channels. The format of the Power Capability element is shown in Figure 3.

|  | Element ID (33) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |
|  | First Channel Number | Number of Channels | Maximum Transmit Power Capability | The element contains one or more triplets |  |
| Octets: | 1 | 1 | 1 |  |  |

Figure 3 - Power Capability element format

The Element ID field shall be set equal to 33. The Length field is variable and depends on the number of triplets in the element. The minimum value of the Length field is 3.

The First Channel Number field shall indicate the first channel number in the sub-band described by the triplet. The field is coded as an unsigned integer. The First Channel Number field shall monotonically increase in successive triplets.

The Number of Channels field shall indicate the number of channels in the sub-band described by the triplet. The field is coded as a positive integer. The Number of Channels field shall not contain a value that causes the sub-bands described by successive triplets to overlap.

The Maximum Transmit Power Capability field shall be set equal to the nominal maximum transmit power with which a STA is capable of transmitting in the sub-band described by the triplet plus any transmit power tolerance. The field is coded as a signed integer in units of dBm.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.10 TPC Request element

The TPC Request element contains a request for a STA to report current transmit power and link margin information using a TPC Report element. The format of the TPC Request element is shown in Figure 4.

|  | Element ID (34) | Length (0) |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure 4 - TPC Request element format

The Element ID field shall be set equal to 34. The Length field shall be set equal to 0.

#### 7.3.2.11 TPC Report element

The TPC Report element contains current link margin information sent in response to a TPC Request element. The format of the TPC Report element is shown in Figure 5.

|  | Element ID (35) | Length (2) | Transmit Power | Link Margin |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure 5 - TPC Report element format

The Element ID field shall be set equal to 35. The Length field shall be set equal to 2.

The Transmit Power field shall be set equal to the transmit power used to transmit the frame containing the TPC Report element. The field is coded as a signed integer in units of dBm. The assumed tolerances of the Transmit Power values are shown in Table 2.

Table 2 – Tolerances for Transmit Power field in TPC Report element

| Transmit Power (dBm) | Tolerance (dB) |
| --- | --- |

The Link Margin field contains the estimated link margin at the time and for the rate at which the frame containing the TPC Request element was received. The link margin is defined as the ratio of the received signal power to that required for effective communication. The field is coded as a signed integer in units of dB. The tolerances of the Link Margin field values are shown in Table 3.

Table 3 – Tolerances for Link Margin field in TPC Report element

| Link Margin (dB) | Tolerance (dB) |
| --- | --- |

Insert clause 7.4 as follows. Renumber figures and tables as appropriate:

## 7.4 Spectrum Management Actions

The Action codes within the Spectrum Management category of an Action frame are defined in Table 4. The details of the frame bodies associated with each Action code are described in the following sub-clauses.

Table 4 – Spectrum Management Action codes

| Action code | Meaning |
| --- | --- |
| 2 | TPC Request |
| 3 | TPC Report |
| 4-255 | Reserved |

Insert clause 7.4.3 as follows. Renumber figures and tables as appropriate:

### 7.4.3 TPC Request frame format

The TPC Request frame uses the Action frame body format and is transmitted by a STA requesting another STA, with which it can communicate directly, for transmit power and link margin information. The format of the TPC Request frame body is shown in Figure 6.

|  | Category (0) | Action (2) | Action Specific (0) | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | TPC Request element |  |  |  |
| Octets: | 2 |  |  |  |

Figure 6 - TPC Request frame body format

The Category field shall be set to 0 (representing Spectrum Management).

The Action field shall be set to 2 (representing a TPC Request frame).

The Action Specific field shall be set equal to 0.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the request to identify the transaction. A pending request is superseded by a subsequent request from the same STA with the same Dialog Token.

The TPC Request element shall be set as described in 7.3.2.10.

Insert clause 7.4.4 as follows. Renumber figures and tables as appropriate:

### 7.4.4 TPC Report frame format

The TPC Report frame uses the Action frame body format and shall be transmitted by a STA in response to a TPC Request frame. The format of the TPC Report frame body is show in Figure 7.

|  | Category (0) | Action Code (1) | Action Specific (0) | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
|  | TPC Report element |  |  |  |
| Octets: | 4 |  |  |  |

Figure 7 - TPC Report frame body format

The Category field shall be set to 0 (representing Spectrum Management).

The Action field shall be set equal to 3 (representing a TPC Report frame).

The Action Specific field shall be set to 0.

The Dialog Token field shall be set equal to the value in the corresponding TPC Request frame.

The TPC Report element shall be set as described 7.3.2.11.

Insert sub-clause 9.11 as follows. Renumber figures and tables, as appropriate:

## 9.11 TPC procedures

The CEPT Member Administrations in Europe require the use of transmit power control (TPC) by wireless LANs operating in the 5GHz band to reduce interference with licensed users. The current CEPT regulations specify a regulatory maximum transmit power and an average mitigation requirement for each allowed channel (ERC/DEC/(99)23).

This clause describes TPC procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and may be useful for other purposes, such as interference reduction, range control and power optimization.

STAs shall use the TPC procedures defined in this clause if and only if the boolean MIB attribute dot11SpectrumManagementEnabled is true. dot11SpectrumManagementEnabled shall be set true when TPC is required by regulatory authorities. It may also be set true in other circumstances. The TPC procedures provide for the:

- Association of STAs with an AP in a BSS based on the STA’s power capability using Power Capability elements in Association Requests and Reassociation Requests (9.11.1).

- Specification of regulatory and local maximum transmit power levels for multiple channels using Country and Power Constraint elements in Beacons and Probe Responses (9.11.2).

- Selection of a transmit power for each MPDU transmitted in a channel within the constraints imposed by the rules (9.11.3).

- Optimization of transmit power based on a range of information, including transmit power and receive margin estimates (9.11.4).

For the purposes of TPC, a STA with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not operate in a BSS or IBSS unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons and Probe Responses received from the BSS or IBSS. Similarly, an AP with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not associate or reassociate a STA unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Association and Reassociation Requests received from the STA.

### 9.11.1 Association based on transmit power capability

A STA shall provide an AP with its maximum transmit power capability for at least the current channel when associating or reassociating using a Power Capability element in Association Request or Reassociation Request frames.

An AP may use the maximum transmit power capability of associated STAs to help determine the local transmit power constraint for any BSS it maintains.

An AP may reject an association or reassociation request from a STA if it considers the STA’s maximum transmit power capability is unacceptable or a transmit power capability is not provided.

### 9.11.2 Specification of a regulatory and local maximum transmit power levels

Regulatory authorities are responsible for defining a regulatory maximum transmit power level and a mitigation requirement for each allowed channel in a regulatory domain.

An AP operating a BSS, a STA starting an IBSS or a STA actively scanning shall not transmit in a channel unless  the TPC related regulatory requirements for the channel are available.

An AP in a BSS or a STA starting an IBSS shall determine a local maximum transmit power level for other STAs in the BSS or IBSS so that the average mitigation requirements are satisfied. Note that a conservative approach is to set the local maximum transmit power level equal to the regulatory maximum transmit power level minus the average mitigation requirement. However, it may be possible to satisfy the average mitigation requirement using a higher local maximum transmit power level. A lower local maximum transmit power level may be used for other purposes, such as range control or interference reduction.

The regulatory maximum transmit power level for at least the current channel shall be communicated to other STAs in a BSS or an IBSS using a Country element in Beacons and Probe Responses. The local maximum transmit power level for at least the current channel shall be communicated to other STAs in the a BSS or an IBSS using a combination of the Country and Power Constraint elements in Beacons and Probe Responses.

In a BSS, the AP shall specify the local maximum transmit power in all Beacons and Probe Responses it transmits. The AP may vary the local maximum transmit power level during the life of a BSS. However, it should consider network stability in deciding how often or by how much to change the local maximum transmit power level.

In an IBSS, the STA starting the IBSS shall specify the local maximum transmit power level in all Beacons and Probe Responses it transmits. Other STAs in the IBSS shall adopt the same local maximum transmit power level for all Beacons and Probe Responses they transmit. The STAs in an IBSS shall not change the local maximum transmit power level during the life of an IBSS.

### 9.11.3 Selection of a transmit power

A STA may select any transmit power for each MPDU transmitted in a channel within the constraints imposed by the following rules:

- An AP operating in a BSS shall use a transmit power less than or equal to the regulatory maximum transmit power level for the channel. However, the AP must also ensure the average regulatory mitigation requirement is respected.

- Non-AP STAs operating in a BSS or an IBSS shall use a transmit power less than or equal to the local maximum transmit power level for the channel.

- STAs that are actively scanning for a BSS or an IBSS shall use a transmit power less than or equal to the regulatory maximum transmit power level for the channel. However, the STA shall also ensure the average regulatory mitigation requirement is respected.

### 9.11.4 Optimization of the transmit power

A STA may use any information, and in particular any path loss and receive margin estimates, to dynamically optimize the transmit power for transmissions of an MPDU to another STA. The optimization methods or criteria are not standardized.

A STA may use a TPC Request frame to request another STA with which it directly communicates to respond with a TPC Report frame containing link margin and transmit power information.

A STA receiving a TPC Request Frame shall respond with a TPC Report frame containing the power used to transmit the response in the Transmit Power field and the estimated link margin in a Link Margin field.

## 10.3 MLME SAP interface

### 17.2.2 TXVECTOR parameters

### 17.2.3 RXVECTOR parameters

### 17.3.8.3.3 Channelization

Change  Table 88 as follows

Table 88 – Valid operation channel numbers by regulatory domain and band

| Regulatory domain | Band | Operating Channel Numbers | Channel Center Frequencies |
| --- | --- | --- | --- |
| USA CEPT | U-NII lower band 5.15-5.25 GHz | 36 40 44 48 | 5,180 MHz 5,200 MHz 5,220 MHz 5,240 MHz |
| USA CEPT | U-NII middle band 5.25-5.35 GHz | 52 56 60 64 | 5,260 MHz 5,280 MHz 5,300 MHz 5,320 MHz |
| CEPT | 5.47-5.725 GHz | 100 104 108 112 116 120 124 128 132 136 140 | 5,500 MHz 5,520 MHz 5,540 MHz 5,560 MHz 5,580 MHz 5,600 MHz 5,620 MHz 5,640 MHz 5,660 MHz 5,680 MHz 5,700 MHz |
| USA | U-NII upper band 5.725-5.825 GHz | 149 153 157 161 | 5,745 MHz 5,765 MHz 5,785 MHz 5,805 MHz |

Delete paragraph after Table 88

Change paragraph after Figure 119

In Figure 119, showing the frequency channel plan for the USA, the center frequency is indicated, however, no sub-carrier is allocated on the center frequency as described in Figure 117.

### 17.3.9.1 Transmit power levels

Replace first paragraph with following

The maximum allowable output power by regulatory domain is shown in Table 89.

Replace Table 89 with following

Table 89 – Transmit power level by regulatory domain

| Frequency band | USA (Maximum output power with up to 6dBi antenna gain) | CEPT (EIRP) |
| --- | --- | --- |
| 5.150-5.250 GHz | 40 mW (2.5 mW/MHz) | 200 mW |
| 5.250-5.350 GHz | 200 mW (12.5 mW/MHz) | 200 mW |
| 5.470-5.725 GHz | - | 1 W |
| 5.725-5.825 GHz | 800 mW (50 mW/MHz) | - |

In the CEPT regulatory domain every STA must be capable of transmitting using at least one transmit power less than or equal to those specified for each band in Table 90.

Table 90 – Declared maximum of minimum transmit power in CEPT countries

| Frequency band | CEPT (Mean EIRP) |
| --- | --- |
| 5.150-5.250 GHz | 17dBm |
| 5.250-5.350 GHz | 17dBm |
| 5.470-5.725 GHz | 24dBm |

# Annex A – PICS

### A.4.3 IUT configuration

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF9 | Is Spectrum Management operation supported? | ? | O.4 | Yes, No |

### A.4.11 TPC extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TP1 |  |  |  |  |
| TP2 |  |  |  |  |
| TP3 |  |  |  |  |
| … |  |  |  |  |
| TPn |  |  |  |  |

### A.4.12 DFS extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| DF1 |  |  |  |  |
| DF2 |  |  |  |  |
| DF3 |  |  |  |  |
| … |  |  |  |  |
| DFn |  |  |  |  |

# Annex C – Formal description of MAC operation

Editors note: the formal description in the 802.11 MAC is filled with syntactic and semantic errors. 802.11h are not proposing to add anything to this very flawed description unless it is verified first, which is not likely to occur in the near future.

# Annex D – ASN.1 encoding of the MAC and PHY MIB

In “Major Sections” of Annex D, add the following text to the end of the Station Management attributes

-- dot11SpectrumManagementCapabilityTable ::= {dot11smt 8}

In “SMT Station Config Table of Annex D, add the following text to the end of the dot11StationConfigEntry sequence list

dot11SpectrumManagementCapabilityImplemented TruthValue,

dot11SpectrumManagementCapabilityEnabled TruthValue

Add the following elements to the end of the dot11StationConfigEntry element definitions behind dot11AuthenticateFailStation

dot11SpectrumManagementCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station implementation is capable of supporting spectrum management. The capability is disabled otherwise. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 24 }

dot11SpectrumManagementCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“A STA will use the defined TPC and DFS procedures if and only if this attribute is TRUE. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 25 }

-- ********************************************************************

-- * dot11SpectrumManagementCapability TABLE

-- ********************************************************************

dot11SpectrumManagementCapabilityTable OBJECT-TYPE

SYNTAX SEQUENCE OF dot11SpectrumManagementCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCIPTION

“This (conceptual) table of attributes for spectrum management”

::= {dot11smt 8}

dot11SpectrumManagementCapabilityEntry OBJECT-TYPE

SYNTAX Dot11SpectrumManagementCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“An entry (conceptual row) in the Spectrum Management Capability Table.

IfIndex – Each 802.11 interface is represented by an ifEntry. Interface tables in this MIB are indexed by ifIndex.”

INDEX {ifIndex, dot11SpectrumManagementCapabilityIndex}

::= {dot11SpectrumManagementCapabilityTable 1}

dot11SpectrumManagementCapabilityEntry ::=

SEQUENCE {

dot11SpectrumManagementCapabilityIndex Integer32,

dot11FirstChannelNumber Integer32,

dot11NumberofChannels Integer32,

dot11MitigationRequirement Integer32}

dot11SpectrumManagementCapabilityIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar objects in the Spectrum Management Capability Table."

::= { dot11SpectrumManagementCapabilityEntry 1}

dot11FirstChannelNumber OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the lowest channel number in the sub-band for the associated domain country string. The default value of this attribute shall be zero."

::= { dot11SpectrumManagementCapabilityEntry 2}

dot11NumberofChannels OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the total number of channels allowed in the sub-band for the associated domain country string. The default value of this attribute shall be zero."

::= { dot11SpectrumManagementCapabilityEntry 3}

dot11MitigationRequirement OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the average mitigation requirement in dB required in the sub-band. The default value of this attribute shall be 3dB."

::= { dot11SpectrumManagementCapabilityEntry 4}

-- ******************************************************************

-- * End of dot11SpectrumManagementCapability TABLE

-- ******************************************************************

# Annex E – Bibliography

Editors note: none so far

ERC documents are available from European Radiocommunications Office, Midtermolen 1, DK-2100, Copenhagen, Denmark (http://www.ero.dk).