IEEE P802.11
Wireless LANs

Proposed TGi D2.2 Clause 8 AES-CTR CBC-MAC (CCM) text

Date:	July 10, 2002

Authors:		Onno Letanche
	Agere Systems
	Zadelstede 1 - 10
	3431 JZ  Nieuwegein, Netherlands
	Phone: +31 30-609-7454
	E-mail: oletanche@agere.com

Dorothy Stanley
	Agere Systems
	2000 North Naperville Rd
	Naperville, IL, 60566
	Phone: +1 630-979-1572
	E-mail: dstanley@agere.com

This document contains text describing the AES Counter Mode with CBC-MAC algorithm. It is based on the proposal documented in IEEE 802.11 document 11-02-001r1-I-AES-CTR-Mode-with-CBC-MAC (March 5 version), by Niels Ferguson, Russ Housley, and Doug Whiting. Some AES algorithm descriptive material was taken from the current working.11i draft, 802.11i-D1.8, edited by Jesse Walker.

## Advanced Encryption Standard (AES) based privacy

### Introduction

A protocol based on the Advanced Encryption Standard (AES) has been adopted to provide wired equivalent privacy in addition to RC4. This protocol is called AES-based privacy, and this clause defines it. AES-based privacy is a protocol that uses the AES algorithm in Counter Mode with CBC-MAC (CCM) mode to provide both privacy and data authenticity.

Clause 8.1.2 discusses features of AES encryption, the underlying cipher used by the AES-based privacy algorithm. Clause 8.1.2.1 describes Counter mode and CBC-MAC mode and how AES is used in these modes. Clause 8.1.2.1.5 gives the MPDU expansion used by the AES-based privacy algorithm. Implementation of AES-based privacy is mandatory in an RSN. Use of AES-based privacy, like any other privacy algorithm, always depends on local policies.

### The AES encryption algorithm

The AES encryption algorithm defines the NIST standard for block ciphers. The AES encryption algorithm is based on the iterated block cipher Rijndael. This cipher has both a variable length key and block size. AES keys can consist of 128, 196 or 256 bits. The NIST standard, however, specifies the use of 128-bit blocks only, even though the Rijndael algorithm itself can support 128, 196, and 256 bit blocks. The 802.11 AES-based privacy protocol restricts the use of AES encryption further by using only 128-bit keys with the cipher.

AES is a symmetric key iterated block cipher. Symmetric key means it uses the same key to encrypt and decrypt data. Iterated means it is applied to the same data over and over again to encrypt or decrypt data; 128-bit AES iterates over the data 10 times. A block cipher encrypts or decrypts a well-defined number of bits only; in the case of AES, this is 128-bits.

Every effort has been made to design AES-based privacy so as to maximize the chances of approval, by the U.S. Department of Commerce, of export from the U.S. of products containing an AES implementation. The current export policy may be found at http://www.bxa.doc.gov/Encryption/.

#### AES-CTR Privacy With AES-CBC-MAC  (CCM)

The AES encryption algorithm, like any block cipher, must be used with a mode of operation. A mode of operation is an algorithm employing the cipher to produce ciphertext from plaintext, and vice versa. Encryption provides a data privacy function only. In particular, it does not afford any protection against data modification. To provide data authenticity requires the use of a data authenticity mechanism. The standard way to accomplish this is to compute a tag, in the literature called a Message Authentication Code, using a keyed cryptographic function. The tag is transported with the data over an unprotected channel with the data it protects, and its value verified by the receiver, using the same key with the same cryptographic function. Since 802.11 already uses the acronym MAC for Medium Access Control, this specification deviates from established practice and refers to such a code as a Message Integrity Code, or MIC. The industry term “CBC-MAC” is retained, as the meaning of “MAC” is clear when paired with “CBC”.

The AES CCM mode of operation chosen as mandatory for 802.11 is a combination of the Counter (CTR) mode privacy and Cipher Block Chaining Message Authentication Code (CBC-MAC) authentication. These modes have been used and studied for a long time, with well-understood cryptographic properties, and no known patent encumbrances. They provide for good security and performance, whether implemented in hardware or software.

Both CTR mode and CBC-MAC use the same temporal key. The differences in both encryption cycles are due to a different IV generation of CTR and CBC-MAC. Figure 1 shows the key hierarchy and key derivation of the temporal key. [When the complete text is assembled, reference can be made to the appropriate figure number in the key/ re-key section]

Figure 1 – CCM Temporal Key Derivation

Definitions

The following variables are used in the description of the AES-CCM cryptographic mode.

| Name | Description | Field Size | Comments |
| --- | --- | --- | --- |
| C | Block number “counter” within a packet | 16 bits |  |
| Hlen | Length of unpadded Hdata | 16 bits |  |
| Dlen | Length of the input packet | 16 bits | Packet size |
| L | Number of bytes in Dlen or C field | 3 bits | Recommended value: L = 2 |
| M | Size (in octets) of MIC “tag” | 3 bits | Recommended value: M = 8 |
| Qos-TC | QoS Traffic Class | 8 bits |  |
| PN | Unique packet number | 48 bits | Similar to the WEP IV |
| Hdata | Additional authentication data | 24 - 48 octets | Immutable MAC header contents |
| P | Input packet | Dlen octets | Octets p[0], p[1], … , p[L-1] |
| A | Authentication (MIC) value, computed over P | M octets | Octets a[0], a[1], … , a[M-1] |
| Q | Output packet | Dlen+M octets | Octets q[0], q[1], … , q[L+M–1] |
| A1 | Address of the receiving STA | 48 bits | Octets A1[0], … , A1[5] |
| A2 | Address of the transmitting STA | 48 bits | Octets A2[0], … , A2[5] |
| K | AES Encryption Key | 128 bits | Octets k[0], k[1], … , k[15] |

Table 1– AES-CBC MIC Variable Descriptions

To ensure that the terminology is clear, consider an example of a 128-bit entry. The 128-bit entity can be represented as 16 octets.  The 128 bit entities can be represented as 16 octets, where the leftmost (and most significant) octet is sent and received first and the rightmost (and least significant) octet transferred last.

The unique packet number (PN) is a 48-bit  sequence counter for each QoS traffic class utilized by this association; the implementation uses each of these to construct unique per-packet nonces.

##### AES-CBC-MAC overview

The MIC value is calculated over the length of the MAC header, the MAC header and the data contents of the packet.

The MAC header authentication excludes the mutable Duration field and SequenceControl field. The FrameControl word is included in the authentication, but the retry bit must be forced to zero for this purpose.
Other mutable field like the Duration field and SequnceControl field are excluded from MAC header authentication. The Duration field is excluded to allow for automatic rate adjustment with a retry. 
The size of the MAC header part to be authenticated varies between 22 and 30 octets, and the MAC header part must be padded with zeroes to get a length that is a whole multiple of 16 octets. The resulting zero padded MAC header part is 32 octets (what can be extended when additional MAC header fields must be authenticated). 
Note that the Hlen field contains the size of the MAC header part (minus Duration and SequenceControl fields) before padding and without the Hlen field itself.

The length of the MIC is set to 64 bits, so the M field value in Table 3 is fixed to 8.

The first AES block of the MIC computation is used to generate the CBC IV. It does not cover any of the packet data, but instead includes the values shown below in Table 2 and Table 3.

| Field | Description | Size |
| --- | --- | --- |
| Flag | Flag byte as defined below (0x59) | 1 byte |
| QoS-TC | QoS Traffic Class | 1 byte |
| A2 | MAC header address 2 | 6 bytes |
| PN | Unique packet number | 6 bytes |
| Dlen | Length of data part | 2 bytes |

Table 2 – MIC Initial Block Fields

| Field | Bits | Description | Size |
| --- | --- | --- | --- |
| Reserved | 7 | Reserved (0) | 1 |
| Hdat | 6 | Header data field available | 1 |
| M | 5-3 | MIC length (3 = 8 octets) | 3 |
| L | 2-0 | Dlen length (1 = 2 octets) | 3 |

Table 3 – Flag Byte Definition for Initial Block

The Hdat bit signals whether the CBC-MAC calculation is performed over a part of the packet that will remain clear text, i.e. the MAC header part, for 802.11 CCM Hdat always is set to 1.

The shaded fields are retrieved from the MPDU as shown in Figure 2:

Figure 2 - Plaintext MPDU and MIC Fields

Figure 3 shows the MIC calculation process. The IV is calculated from the fields in Table 4 and the MAC header part is authenticated with 2 or 3 AES encryption cycles. Then the CBC-MAC over the data part of the packet is calculated, where the last data might be padded with zeroes. Since the MIC has a length of 8 octets and the result of the last encryption has a length of 16 octets (128 bits) the MIC consists of the leftmost (most significant) half of the encrypted result. The MIC is encrypted with the very first encryption preload value, where the counterfield is 0000.

Figure 3 - AES-CBC MIC Calculation Process

##### AES-CTR encryption overview

Counter mode encryption requires a counter that is encrypted to produce an output value. The encrypted output is exclusive-ored with the clear text data to produce encrypted data. AES-CTR mode has the property that only AES encryption is used, for both encryption and de-cryption of data. Also, the output data length does not need to be a whole multiple of the encryption block size (i.e. 16 bytes).

The first step in counter mode encryption consists of setting the counter plaintext preload value similar to the first step of the CBC-MAC. The initial counter values are determined as follows:

| Field | Description | Size |
| --- | --- | --- |
| Flag | Flag byte as defined below (0x01) | 1 byte |
| QoS-TC | QoS Traffic Class | 1 byte |
| A2 | MAC header address 2 | 6 bytes |
| PN | Unique packet number | 6 bytes |
| C | Counter (0x0000) | 2 bytes |

Table 4 – Encryption Initial Block Fields

| Field | Bits | Description | Size (bits) |
| --- | --- | --- | --- |
| Rsv | 7 | Reserved (0) | 1 |
| Rsv | 6 | Reserved (0) | 1 |
|  | 5-3 | 0 | 3 |
| L | 2-0 | Counter length (1 = 2 octets) | 3 |

Table 5 – Flag Byte Definition for Initial Block

The shaded fields are retrieved from the MPDU in its encrypted form as shown in Figure 4.

For encryption of the data part the C field is initially set to 0x0001 and is incremented by 1 at every next block that must be encrypted using counter mode. Depending on the amount of data, C ranges from 1 – 145 (0 – 2304 octets of data). The C field value 0x0000 is dedicated to the encryption of the MIC and is not used for data encryption.

Figure 4 - Plaintext MPDU and AES-CTR Initial Block Value

##### AES-CCM

Figure 5 gives an overview of how a transmit MPDU is authenticated and encrypted. Note that both the header and the data part are padded with zeroes for authentication and that the upper 64 bits of the CBC-MAC calculation are used for the MIC.

Figure 5 – AES CCM Encryption and Authentication

Figure 6 shows how the decryption and authentication of a received MPDU is performed. The decryption uses the encryption counter scheme, since only the AES generated random string is exclusive-ored with the data to either encrypt or decrypt. The clear text data is authenticated and the calculated MIC is compared with the received, and decrypted, MIC.

Figure 6 - AES CCM Decryption and Authentication

##### AES-CCM MPDU format

Figure 7 shows the encrypted MPDU when using AES-based privacy.

Figure 7 - Construction of Expanded AES MPDU

The AES-based privacy mechanism is invisible to entities outside the 802.11 MAC data path.

Bit 5 of the Mode field is used to signal an Extended Packet number field of 6 octets. For standard length Packet number/ IV fields (as with basic WEP, TKIP and AES-OCB) this bit is set to zero, for extended packet number field the bit must be set to one.

##### Key Derivation and Usage

Each pair of stations in the network that desire to communicate will have a unique pair wise key. A new key will also need to be negotiated prior to the exhaustion of the packet number field. With a 44 bit effective packet number size 16 Terra packets have to be transferred before exhausting the packet number space.

[Do we require unique pair wise keys with AES? Or will assemblage keys be supported for broadcast traffic and unicast traffic]

##### Implementation Notes and Options

Note that the upper 64 bits of the AES input blocks do not change during the lifetime of the connection. The purpose of these “salt” bits is to minimize the impact of certain classes of attacks (e.g., Hellman80, Biham96). For 802.11i, we have chosen them as addresses. Another approach would have been to choose these salt bits at random when the connection is established, possibly by hashing connection attributes, such as the receiver/transmitter addresses. These salt bits do not have to remain secret.

Annex A provides test vectors for verification of the correct implementation; annex B shows sample source code.

##### Security proof

Jakob Jonsson from RSA Laboratories is working on a security proof of CCM.  Work is still in progress, but current results show that CCM is provably secure to the same security bound as OCB mode. The proof is much simpler than the OCB proof of security. We are investigating whether this proof can be extended to prove better security bounds for the authentication.

##### Design Rationale of CCM

CCM mode of AES is specified for a wide range of applications beyond wireless LANS, including for example IPSec. Several of the design choices are described here.

The main difficulty in specifying the CCM mode is the trade-off between nonce size and counter size. For a general mode support of large messages is required. Some applications, such as wireless LANs use only small messages, but would rather have a larger nonce. Introducing the L parameter solves this issue. The parameter M gives the traditional trade-off between message expansion and probability of forgery. We recommend choosing the value of M to be at least 8.

The CBC-MAC is computed over a sequence of blocks that encode the relevant data in a unique way. Given the block sequence it is easy to recover PN, M, L, P, and Hdata.

The encryption is a straightforward application of CTR mode. As some implementations will support a variable length counter field, we have ensured that the least significant byte of the counter is at one end of the field. This also ensures that the counter is aligned on the block boundary.

By encrypting MIC we avoid all the collision attacks on CBC-MAC mode. If the block cipher behaves as a pseudo-random permutation then the key stream is indistinguishable from a random string. This implies that the attacker gets no information about the CBC-MAC results. The only avenue of attack that is left is a differential-style attack, which has no significant chance of success if the block cipher is a pseudo-random permutation.

We use the same block cipher key for the encryption and authentication functions. In our implementation this is not a problem. All the encryption inital blocks are different, and they are different from the authentication initial block. If the block cipher behaves like a random permutation, then the outputs are independent of each other, up to the insignificant limitation that they are all different. The only places where the inputs to the block cipher can overlap is an overlap between an intermediate value in the CBC-MAC and one of the other encryptions. As all the intermediate values of the CBC-MAC computation are essentially random (because the block cipher behaves like a random permutation) the probability of such a collision is very small. Even if there is a collision, these values only affect MIC, which is encrypted so that an attacker cannot deduce any information, or detect any collision.

Care has been taken to ensure that the blocks used by the authentication function match up with the blocks used by the encryption function. This should simplify hardware implementations, and reduce the amount of byte-shifting required by software implementations.

# Appendix A. Test Vectors

Notation

In the examples here, frames are represented as a stream of octets, each octet in hex notation, sometimes with text annotation.  The order of transmission for octets is left to right, top to bottom.  For example, consider the following representation of a frame:

| Description #1 | 00 01 02 03 |
| --- | --- |
|  | 04 05 |
| Description #2 | 06 07 08 |

The frame consists of nine octets, represented in hex notation as “00”, “01”, ..., “08”.  The octet represented by “00” is transmitted first, and the octet represented by “08” is transmitted last.  Similar tables are used for other purposes, such as describing a cryptographic operation.

In the text discussion outside of tables, integer values are represented in either hex notation using an “0x” prefix or in decimal notation using no prefix.  For example, the hex notation 0x12345 and the decimal notation 74565 represent the same integer value.

# AES-CCM encapsulation

The MPDU data, prior to AES-CCM encapsulation, is as follows:

| MPDU data | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| --- | --- |

The MAC header is as follows:

| MAC header | 08 43 12 34 ff ff ff ff ff ff 00 40 96 45 07 f1 08 00 46 17 62 3e 50 67 |
| --- | --- |

The muted MAC header, i.e., the MAC header with mutable fields cleared to 0, is as follows:

| Muted MAC header | 08 42 00 00 ff ff ff ff ff ff 00 40 96 45 07 f1 08 00 46 17 62 3e 00 00 |
| --- | --- |

The muting clears:  the retry bit, in the second octet (0x43  0x42); the 16-bit duration field, in the third and fourth octets (0x1234  0x0000); and the 12-bit sequence number, in the last two octets (0x567  0x000).

The following parameters are assumed:

- Replay counter value is 21,542,142,465= 0x0504030201  (40 bits).

- QOS traffic class is 4.

- KeyID is 2.

## MIC computation

The MIC is defined using AES-CBC-MAC.  It can also be computed using AES-CBC encryption.  At the time of this writing, NIST has published test vectors for AES-CBC encryption but not for AES-CBC-MAC.  Accordingly, the discussion here describes the MIC computation using AES-CBC encryption.

The discussion here represents an AES-CBC encryption using a table that shows the key, IV input, plaintext input, and ciphertext output.  For reference, here is a table that describes the test case “CBC-AES128.Encrypt” in Appendix F.2 of  [NIST Special Publication 800-38A, 2001 Edition].

| Key | 2b 7e 15 16 28 ae d2 a6 ab f7 15 88 09 cf 4f 3c |
| --- | --- |
| IV | 00 01 02 03 04 05 06 07 08 09 0a 0b 0c 0d 0e 0f |
| Plaintext | 6b c1 be e2 2e 40 9f 96 e9 3d 7e 11 73 93 17 2a ae 2d 8a 57 1e 03 ac 9c 9e b7 6f ac 45 af 8e 51 30 c8 1c 46 a3 5c e4 11 e5 fb c1 19 1a 0a 52 ef f6 9f 24 45 df 4f 9b 17 ad 2b 41 7b e6 6c 37 10 |
| Ciphertext | 76 49 ab ac 81 19 b2 46 ce e9 8e 9b 12 e9 19 7d 50 86 cb 9b 50 72 19 ee 95 db 11 3a 91 76 78 b2 73 be d6 b8 e3 c1 74 3b 71 16 e6 9e 22 22 95 16 3f f1 ca a1 68 1f ac 09 12 0e ca 30 75 86 e1 a7 |

The MIC is computed using the following AES-CBC encryption:

| Key | 00 01 02 03 04 05 06 07 08 09 0a 0b 0c 0d 0e 0f |
| --- | --- |
| IV | 59 04 00 40 96 45 07 f1 05 04 03 80 02 01 00 6e |
| Plaintext | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 18 08 42 00 00 ff ff ff ff ff ff 00 40 96 45 07 f1 08 00 46 17 62 3e 00 00 00 00 00 00 00 00 aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 00 00 00 00 00 00 00 00 00 00 |
| Ciphertext | 64 da d9 ad c1 5a 28 bf 0f 76 6e 37 56 0a 93 09 e7 f9 35 ea b8 7b c8 ad be fc 80 56 6c 48 bc 78 04 91 c6 0f c6 96 88 bc bf 63 b8 d3 41 5e 5c c2 cd 23 ab ca 6b 40 c9 b9 9c 47 bb 08 a5 13 13 a5 24 23 d8 d0 ad 6a 7e b2 bc 72 22 dd 64 cc 57 82 7f df bf 93 d9 41 e8 c1 84 2f 8f b0 2c 2a bf cb 5f 09 ca 9a 7d 2b 92 db 8d ac 38 79 dc 5f 65 18 5c 15 36 1f b5 42 49 0a 32 7e 83 b9 73 88 1c e9 22 68 e8 9f 4e a0 68 c0 44 d2 a3 29 68 55 b9 f1 |

The IV consists of 16 octets.  The first octet of the IV is “0x59”; this is a fixed value for AES-CCM encapsulation in 802.11.  The upper nibble of the second octet is 0.  The lower nibble of the second octet is the QOS traffic class.  The next six octets of the IV are address 2 of the MAC header.  The next six octets of the IV are the packet number (see below).  The last two octets of the IV show the sum of the lengths of the MAC header and the MPDU data; in this example, the value is 24 + 86 = 110 = 0x006e.

The packet number consists of six octets.  Octets one, two, three, five and six are the 40-bit replay counter.  Octet four is the concatenation of the 2-bit keyID value and six 0-bits.

The plaintext is constructed as follows.

- The first 16 octets of the plaintext are zero.  The reason for this is a slight difference in the definition of CBC encryption versus that of CBC-MAC.  In CBC encryption, the first ciphertext block c1 is computed as c1 = AES(IV ^ p1), where p1 is the first plaintext block and “^” means exclusive or; subsequent blocks are computed as cn = AES(cn-1 ^ pn) for n > 1.    In CBC-MAC, the zeroth ciphertext block is computed as c0 = AES(IV), with subsequent blocks computed as cn = AES(cn-1 ^ pn) for n > 0.  Thus, CBC-MAC can be computed using CBC encryption by inserting a block of zeroes at the start of the plaintext.

- The next two octets of the plaintext consist of the length of the muted MAC header.  In this example, the length is 24 = 0x18.

- The next octets of the plaintext are the muted MAC header.

- The MAC header is followed by 0 to 15 octets of zero, as needed to get to 16-octet alignment.  In this example, 6 octets of zero are needed.

- The next octets of the plaintext are the MPDU data.

- The MPDU data is followed by 0 to 15 octets of zero, as needed to get to 16-octet alignment.  In this example, 10 octets of zero are needed.

The MIC consists of the last block of ciphertext, truncated to 8 octets.  That is, the MIC value is

| MIC | 22 68 e8 9f 4e a0 68 c0 |
| --- | --- |

## Encapsulation

The MPDU and MIC data are encapsulated using AES-CTR encryption.  The discussion here represents an AES-CTR encryption using a table that shows the key, initial counter input, plaintext input, and ciphertext output.  For reference, here is a table that describes the test case “CTR-AES128.Encrypt” in Appendix F.5 of  [NIST Special Publication 800-38A, 2001 Edition].

| Key | 2b 7e 15 16 28 ae d2 a6 ab f7 15 88 09 cf 4f 3c |
| --- | --- |
| Init. Counter | f0 f1 f2 f3 f4 f5 f6 f7 f8 f9 fa fb fc fd fe ff |
| Plaintext | 6b c1 be e2 2e 40 9f 96 e9 3d 7e 11 73 93 17 2a ae 2d 8a 57 1e 03 ac 9c 9e b7 6f ac 45 af 8e 51 30 c8 1c 46 a3 5c e4 11 e5 fb c1 19 1a 0a 52 ef f6 9f 24 45 df 4f 9b 17 ad 2b 41 7b e6 6c 37 10 |
| Ciphertext | 87 4d 61 91 b6 20 e3 26 1b ef 68 64 99 0d b6 ce 98 06 f6 6b 79 70 fd ff 86 17 18 7b b9 ff fd ff 5a e4 df 3e db d5 d3 5e 5b 4f 09 02 0d b0 3e ab 1e 03 1d da 2f be 03 d1 79 21 70 a0 f3 00 9c ee |

AES-CTR encryption is performed on the MPDU, as follows:

| Key | 00 01 02 03 04 05 06 07 08 09 0a 0b 0c 0d 0e 0f |
| --- | --- |
| Init. Counter | 01 04 00 40 96 45 07 f1 05 04 03 80 02 01 00 00 |
| Plaintext | 22 68 e8 9f 4e a0 68 c0 XX XX XX XX XX XX XX XX aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| Ciphertext | 48 90 95 62 25 60 e3 b2 f3 bb 96 b5 41 cd 6e 6b 99 f6 31 09 22 86 21 dc 37 b5 ee 56 59 b5 d2 22 b2 41 b8 fe 54 8c 0d 93 9b f7 c7 0a e8 b8 88 ff 20 26 f3 31 6b 79 01 90 04 e9 52 fb 61 fe eb e0 1c ef fc b5 bc b3 ba 8c e5 c3 99 cd 43 8d b7 67 dc 72 ad c4 c4 56 e4 d9 af 98 c6 ec 85 22 4e 3f 14 60 7a b8 9c 99 |

The key is the same as used for the MIC calculation.  The initial counter value is nearly the same as the IV used for the MIC calculation, the difference being that the first octet is replaced by “01” and the last two octets by “00 00”; these are fixed values for AES-CCM encapsulation in 802.11.

The first eight plaintext octets consist of the MIC value.  The next eight octets of the plaintext are arbitrary values; the corresponding ciphertext is not used.  The remaining octets of the plaintext consist of the MPDU data.

The expanded MPDU, after AES-CCM encapsulation, is as follows:

| Packet number | 05 04 03 80 02 01 |
| --- | --- |
| MPDU data | 99 f6 31 09 22 86 21 dc 37 b5 ee 56 59 b5 d2 22 b2 41 b8 fe 54 8c 0d 93 9b f7 c7 0a e8 b8 88 ff 20 26 f3 31 6b 79 01 90 04 e9 52 fb 61 fe eb e0 1c ef fc b5 bc b3 ba 8c e5 c3 99 cd 43 8d b7 67 dc 72 ad c4 c4 56 e4 d9 af 98 c6 ec 85 22 4e 3f 14 60 7a b8 9c 99 |
| MIC | 48 90 95 62 25 60 e3 b2 |

The packet-number field consists of the 6-octet packet number.  The MPDU and MIC fields consist of those portions of the ciphertext corresponding to the MPDU and MIC data, respectively.  Note that the order of the MPDU and MIC fields is different in the encapsulated MPDU than in the ciphertext.

# Appendix B. Test Vector Generation Source Code

TBA