IEEE P802.11
Wireless LANs

Potential Draft Text

Date	March 2002

Author	Andrew Myles
	Cisco Systems
	Level 2, 3 Innovation Road,
	North Ryde, 2113, Australia
	Phone: +61 2 84461010
	Fax: +61 2 84461001
	e-Mail: andrew.myles@cisco.com

Simon Black
	Qosine
	e-Mail: simon.black@qosine.co.uk

Abstract

The purpose of this submission is to provide potential text for an official 802.11h draft.

The TPC clauses in this document are based strongly on the official draft D1.1.

The DFS clauses represent a very significant change to official draft D1.1.

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

Spectrum and Transmit Power Management
extensions in the 5GHz band in Europe

Sponsored by the
IEEE 802 Committee
of the
IEEE Computer Society

Copyright (c) 2001 by the Institute of Electrical and Electronics Engineers, Inc.
345 East 47th Street
New York, NY 10017, USA
All rights reserved.

This is an unapproved draft of a proposed IEEE Standard, subject to change. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities. If this document is to be submitted to ISO or IEC, notification shall be given to the IEEE Copyright Administrator. Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for purposes of developing a national position. Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license. Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department
Copyright and Permissions
445 Hoes Lane, P.O. Box 1331
Piscataway, NJ 08855-1331, USA

# Introduction

(This introduction is not part of IEEE P802.11h, Draft Supplement to STANDARD FOR Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications: DFS and TPC extensions in the 5GHz band in Europe)

Example:

At the time this standard was completed, the working group had the following membership:

Chair of working group (to be provided by IEEE editor at time of publication)

List of members of working group (to be provided by IEEE editor at time of publication)

The following persons were on the balloting committee:

List of members of balloting committee (to be provided by IEEE editor at time of publication)

# Contents

Introduction	ii

Contents	iii

2. Normative References	2

3. Definitions	2

4. Abbreviations and acronyms	2

5.5 Relationships between services	3

5.7.2 Association	3

5.7.3 Reassociation	3

7.1.3.1.2 Type and Subtype fields	3

7.2.3.1 Beacon frame format	4

7.2.3.4 Association Request frame format	4

7.2.3.5 Reassociation Request frame format	5

7.2.3.9 Probe Response frame format	5

7.3.1.4 Capability Information field	6

7.3.1.7 Reason Code field	7

7.3.1.9 Status Code field	7

7.3.1.12 Action Descriptor field	8

7.3.2 Information elements	9

7.3.2.8 Power Constraint element	10

7.3.2.9 Power Capability element	11

7.3.2.10 TPC Request element	12

7.3.2.11 TPC Report element	13

7.3.2.12 Supported Channels element	14

7.3.2.13 Channel Switch Announcement element	15

7.3.2.14 Measurement Request element	16

7.2.3.14.1 Basic Request	17

7.2.3.14.2 Proprietary Request	17

7.3.2.15 Measurement Report element	18

7.3.2.15.1 Basic Report	19

F.13 Proprietary Report	20

7.3.2.16 Quiet element	21

7.3.2.17 IBSS DFS element	22

7.4 Spectrum Management Actions	23

7.4.1 Measurement Request frame format	24

7.4.2 Measurement Report frame format	25

7.4.3 TPC Request frame format	26

7.4.4 TPC Report frame format	27

7.4.5 Channel Switch Announcement frame format	28

9.11 TPC procedures	29

9.11.1 Association based on transmit power capability	30

9.11.2 Specification of a regulatory and local maximum transmit power levels	31

9.11.3 Selection of a transmit power	32

9.11.4 Optimization of the transmit power	33

9.12 DFS procedures	34

9.12.1 Quieting channels for testing	35

9.12.2 Testing channels for primary users	36

9.12.3 Discontinuing operations after detecting primary users	37

9.12.4 Detecting primary users	38

9.12.5 Requesting and reporting of measurements	39

9.12.6 Selecting and advertising a new channel	40

10.3 MLME SAP interface	41

17.3.8.3.3 Channelization	42

17.3.9.1 Transmit power levels	43

Annex A – PICS	44

A.4.3 IUT configuration	44

A.4.11 TPC extensions	44

A.4.12 DFS extensions	44

Annex C – Formal description of MAC operation	45

Annex D – ASN.1 encoding of the MAC and PHY MIB	46

Annex E – Bibliography	54

Annex F – Measurement Requests and Reports in DFS	55

F.1 Basic Request	56

F.2 Basic Report	57

F.3 RSSRI Histogram Request	58

F.4 RSSRI Histogram Report	59

F.5 Extended CCA Request	60

F.6 Extended CCA Report	61

F.7 Unknown RSSRI Histogram Request	62

F.8 Unknown RSSRI Histogram Report	63

F.9 BSS Request	64

F.10 BSS Report	65

F.11 STA Request	66

F.12 STA Report	67

F.13 Proprietary Request	69

F.13 Proprietary Report	70

Draft Supplement to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

Spectrum and Transmit Power Management
extensions in the 5GHz band in Europe

[ The editing instructions contained in this supplement define how to merge the material contained herein into the current edition of IEEE Std 802.11 1999 Edition and the 802.11a, 802.11b, 802.11b-cor1 and 802.11d supplements

The editing instructions are shown in bold italic. Three editing instructions are used:

Change is used to make small corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed either by using strikethrough (to remove old material) or underscore (to add new material).

Delete removes existing material.

Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instructions.

Editorial notes will not be carried over into future editions.]

# 2. Normative References

Insert the following citation at the appropriate location in clause 2:

ERC/DEC/(99)23, ERC Decision of 29 November 1999 on the harmonized frequency bands to be designated for the introduction of High Performance Radio Local Area Networks (HIPERLANs) .

# 3. Definitions

Insert the following new definitions at the appropriate locations in clause 3, renumbering as necessary:

3.51 dynamic frequency selection (DFS): Facilities mandated to satisfy requirements in some regulatory domains that specify precedence for primary users and uniform channel spreading in the 5GHz band. DFS facilities may also be used for other purposes.

3.52 transmit power control (TPC): Facilities mandated to satisfy requirements in some regulatory domains for maximum transmit power and average transmit power mitigation in the 5GHz band. TPC facilities may also be used for other purposes, such as interference reduction, range control and power optimization.

3.53 transmit power: transmit power is the Effective Isotropic Radiated Power.

3.54 receive power: receive power is the mean power measured at the antenna connector.

# 4. Abbreviations and acronyms

Insert the following text in the appropriate location in clause 4:

DFS	dynamic frequency selection

TPC	transmit power control

## 5.5 Relationships between services

Add to the definition of class 1, management frames as follows:

vi) Action

### 5.7.2 Association

Insert the following text under "Association request" just after entry for "ESS ID":

- Power Capability

- Supported channels

### 5.7.3 Reassociation

Insert the following text under "Reassociation request" just after entry for "ESS ID":

- Power Capability

- Supported channels

##### 7.1.3.1.2 Type and Subtype fields

Change the contents of Table 1 as follows:

Table 1 - Valid type and subtype combinations

| Type Value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 1101 | Action |
| 00 | Management | 1110-1111 | Reserved |

#### 7.2.3.1 Beacon frame format

Change order 11 information field and add order 14 and order 15 information fields as follows:

Table 5 - Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11Spectrum ManagementCapabilityEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementCapabilityEnabled is true |
| 15 | Channel Switch Announcement | Channel Switch Announcement element may be present if dot11SpectrumManagementCapabilityEnabled is true |
| 16 | Quiet | Quiet element may be present if dot11SpectrumManagementCapabilityEnabled is true |
| 17 | IBSS DFS | IBSS DFS element shall be present if dot11SpectrumManagementCapabilityEnabled is true in an IBSS |

#### 7.2.3.4 Association Request frame format

Add order 5 and order 6 information fields as follows:

Table 7 - Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementCapabilityEnabled is true |
| 6 | Supported Channels | Supported Channels element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.5 Reassociation Request frame format

Add order 6 and order 7 information fields as follows:

Table 9 - Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementCapabilityEnabled is true |
| 7 | Supported Channels | Supported Channels element shall be present if dot11SpectrumManagementCapabilityEnabled is true |

#### 7.2.3.9 Probe Response frame format

Change order 10 information field and add order 14 and 15 information fields as follows:

Table 12 Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Country element shall be present if dot11MultiDomainCapabilityEnabled or dot11SpectrumManagementCapabilityEnabled is true |
| 13 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementCapabilityEnabled is true |
| 14 | Channel Switch Announcement | Channel Switch Announcement element may be present if dot11SpectrumManagementCapabilityEnabled is true |
| 15 | Quiet | Quiet element may be present if dot11SpectrumManagementCapabilityEnabled is true |
| 16 | IBSS DFS | IBSS DFS element shall be present if dot11SpectrumManagementCapabilityEnabled is true in an IBSS |
| 17-n | Requested information elements | Elements requested by the Request Information Element of the Probe Request Frame |

#### 7.3.1.4 Capability Information field

Change the supporting text as follows:

The length of the Capability Information field is two octets. The Capability Information field consists of the following subfields: ESS, IBSS, CF-Pollable, CF-Poll Request, Privacy, Short Preamble, PBCC, Channel Agility and Spectrum Management. The format of the Capability Information field is as illustrated in Figure 27.

Change the contents of Figure 27 as shown:

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESS | IBSS | CF Pollable | CF Poll Request | Privacy | Short Preamble | PBCC | Channel Agility |
| B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
| Spectrum Management | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) | Reserved (0) |

Figure 27 - Capability Information fixed field

Insert the following text at the end of clause 7.3.1.4:

A STA shall set the Spectrum Management subfield in the Capability Information field equal to 1 if the boolean MIB attribute dot11SpectrumManagementCapabilityEnabled is true, otherwise it shall be set equal to 0.

#### 7.3.1.7 Reason Code field

Change table 18 as follows:

Table 18 - Reason codes

| Reason Code | Meaning |
| --- | --- |
| 10 | Disassociated because the Power Capability element is missing or the Maximum Transmit Power Capability field in the Power Capability element is unacceptable |
| 11 | Disassociated because the Supported Channels element is missing or the Supported Channels field in the Supported Channels element is unacceptable |

#### 7.3.1.9 Status Code field

Change table 19 as follows:

Table 19 - Status codes

| Status Code | Meaning |
| --- | --- |
| 19 | Association request rejected because Spectrum Management capability is required |
| 20 | Association request rejected because the Power Capability element is missing or the Maximum Transmit Power Capability field in the Power Capability element is unacceptable |
| 21 | Association request rejected because the Supported Channels element is missing or the Supported Channels field in the Supported Channels element is unacceptable |

Insert the following clause after 7.3.1.11  and renumber as necessary:

#### 7.3.1.12 Action Descriptor field

The Action Descriptor field contains a number of subfields that are used in specifying management actions. The format of the Action Descriptor is shown in Figure 1.

|  | Category | Action | Action Specific | Dialog Token |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure 1 – Action Descriptor field

The Category field shall be set to one of the values shown in Table 1. The details of the actions allowed in each category are described in the clause for the category referenced in Table 1.

The Action field shall be set to a value that is allowed in the Category. The most significant bit shall normally be set equal to 0. However, if a STA receives a unicast Action frame with an unrecognized Category field, Action field, Action Specific field, Dialog Token field or some other syntactic error and the most significant bit of the Action field set to 0 then the STA shall return the entire frame to the source with the most significant bit of the Action field set equal to 1.

The Action Specific field shall be set to a value that is specific to the particular Category and Action. The Action Specific field is often used to carry a status value or an activation delay.

The Dialog Token field shall be set to a value that is specific to the particular Category and Action. The Dialog Token field is normally used to associate a series of frames in a multi-management frame transaction.

Table 1 - Category values

| Name | Value | See clause |
| --- | --- | --- |
| Spectrum management | 0 | 7.4 |
| Reserved | 1-255 |  |

### 7.3.2 Information elements

Insert the following information elements into Table 20 of clause 7.3.2 in place of the reserved IDs:

Table 20 - Element IDs

| Information element | Element ID |
| --- | --- |
| Power Constraint | 32 |
| Power Capability | 33 |
| TPC Request | 34 |
| TPC Report | 35 |
| Supported Channels | 36 |
| Channel Switch Announcement | 37 |
| Measurement Request | 38 |
| Measurement Report | 39 |
| Quiet | 40 |
| IBSS DFS | 41 |
| Reserved | 11-15, 42-255 |

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.8 Power Constraint element

The Power Constraint element contains the information necessary to allow a STA to determine the local maximum transmit power level that shall not be exceeded for transmissions in various channels. The format of the Power Constraint element is shown in Figure 2.

|  | Element ID (32) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |
|  | First Channel Number | Number of Channels | Local Power Constraint | The element contains one or more triplets |  |
| Octets: | 1 | 1 | 1 |  |  |

Figure 2 - Power Constraint element format

The Element ID field shall be set equal to 32. The Length field is variable and depends on the number of triplets in the element. The minimum value of the Length field is 3.

The First Channel Number field shall indicate the first channel number in the sub-band described by the triplet. The field is coded as an unsigned integer. The First Channel Number field shall monotonically increase in successive triplets.

The Number of Channels field shall indicate the number of channels in the sub-band described by the triplet. The field is coded as a positive integer. The Number of Channels field shall not contain a value that causes the sub-bands described by successive triplets to overlap.

The Local Power Constraint field shall be set equal to a value greater than or equal to the minimum transmit power mitigation required to operate in the sub-band described by the triplet to satisfy regulatory requirements. The field is coded as an unsigned integer in units of dB with 0dB tolerance. The local maximum transmit power for a channel is thus defined as the Maximum Transmit Power Level specified for the channel in the Country element minus the Local Power Constraint specified for the channel in the Power Constraint element.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.9 Power Capability element

The Power Capability element specifies the maximum power with which a STA is capable of transmitting in the various channels. The format of the Power Capability element is shown in Figure 3.

|  | Element ID (33) | Length (variable) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |
|  | First Channel Number | Number of Channels | Maximum Transmit Power Capability | The element contains one or more triplets |  |
| Octets: | 1 | 1 | 1 |  |  |

Figure 3 - Power Capability element format

The Element ID field shall be set equal to 33. The Length field is variable and depends on the number of triplets in the element. The minimum value of the Length field is 3.

The First Channel Number field shall indicate the first channel number in the sub-band described by the triplet. The field is coded as an unsigned integer. The First Channel Number field shall monotonically increase in successive triplets.

The Number of Channels field shall indicate the number of channels in the sub-band described by the triplet. The field is coded as a positive integer. The Number of Channels field shall not contain a value that causes the sub-bands described by successive triplets to overlap.

The Maximum Transmit Power Capability field shall be set equal to the nominal maximum transmit power with which a STA is capable of transmitting in the sub-band described by the triplet plus any transmit power tolerance. The field is coded as a signed integer in units of dBm.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.10 TPC Request element

The TPC Request element contains a request for a STA to report current transmit power and link margin information using a TPC Report element. The format of the TPC Request element is shown in Figure 4.

|  | Element ID (34) | Length (0) |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure 4 - TPC Request element format

The Element ID field shall be set equal to 34. The Length field shall be set equal to 0.

#### 7.3.2.11 TPC Report element

The TPC Report element contains current transmit power and link margin information sent in response to a TPC Request element. The format of the TPC Report element is shown in Figure 5.

|  | Element ID (35) | Length (2) | Transmit Power | Link Margin |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure 5 - TPC Report element format

The Element ID field shall be set equal to 35. The Length field shall be set equal to 2.

The Transmit Power field shall be set equal to the transmit power used to transmit the frame containing the TPC Report element. The field is coded as a signed integer in units of dBm. The assumed tolerances of the Transmit Power values are shown in Table 2.

Table 2 – Tolerances for Transmit Power field in TPC Report element

| Transmit Power (dBm) | Tolerance (dB) |
| --- | --- |

The Link Margin field contains the estimated link margin at the time and for the rate at which the frame containing the TPC Request element was received. The link margin is defined as the ratio of the received signal power to that required for effective communication. The field is coded as a signed integer in units of dB. The tolerances of the Link Margin field values are shown in Table 3implementation dependent.

– Tolerances for Link Margin field in TPC Report element

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.12 Supported Channels element

The Supported Channels element contains a list of channels in which a STA is capable of operating. The format of the Supported Channels element is shown in Figure 6.

|  | Element ID (36) | Length (28) |
| --- | --- | --- |
| Octets: | 1 | 1 |
|  | Supported Channels |  |
| Octets: | 28 |  |

Figure 6 - Supported Channels element format

The Element ID field shall be set equal to 36. The Length field shall be set equal to 28.

The Supported Channels field is a 28-octet bit field, where bit 0 corresponds to channel 1 and bit 199 corresponds to channel 200. A bit in the field shall be set equal to 1 when the STA supports operation in the corresponding channel, otherwise it shall be set equal to 0. Bits 200-223 shall be set equal to 0.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.13 Channel Switch Announcement element

The Channel Switch Announcement element allows an AP in a BSS or a STA in an IBSS to advertise when it is changing to a new channel and the channel number of the new channel. The format of the Channel Switch Announcement element is shown in Figure 7.

|  | Element ID (37) | Length (2) | New Channel Number | Channel Switch Count |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure 7 - Channel Switch Announcement element format

The Element ID field shall be set equal to 37. The Length field shall be set equal to 2.

The New Channel Number field shall be set equal to the number of the channel to which the STA is moving.

The Channel Switch Count field shall be set equal to the number of TBTTs until the STA sending the Channel Switch Announcement element will switch to the new channel. A value of 1 indicates that the switch will occur immediately before the next TBTT. A value of 0 indicates that the switch will occur immediately after the frame containing the element is transmitted.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.14 Measurement Request element

The Measurement Request element contains a request that the receiving STA undertake a measurement for a specified duration. The format of the Measurement Request element is shown in Figure 8.

|  | Element ID (38) | Length (variable) |  |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 |  |  |  |
|  | Measurement Token | Measurement Duration | Measurement ID | Measurement Request |
| Octets: | 1 | 2 | 2 | variable |

Figure 8 - Measurement Request element format

The Element ID field shall be set equal to 38. The Length field is variable and depends on the length of the Measurement Request field. The minimum value of the Length field is 5.

The Measurement Token shall be set to a non-zero number that is unique among the Measurement Request elements in a particular Measurement Request frame.

The Measurement Duration field shall be set equal to the measurement duration of the measurement request in TUs. A value of zero shall mean that the measurement shall use the same duration and starting time as the immediately previous Measurement Request element in the same Measurement Request frame. A value of zero shall not be used when the Measurement Request element is the first element in the Measurement Request frame

The Measurement ID field shall be set to a number that identifies the format and contents of the Measurement Request field. The IEEE shall be responsible for allocating the Measurement ID number space. Those Measurement ID’s that have been allocated are shown in Table 4 and the format of the corresponding Measurement Request field are described in the following clause.

The Measurement Request field shall contain the requested measurement.

Table 4 – Measurement ID definitions for Measurement Request element

| Name | Measurement ID |
| --- | --- |
| Basic Request | 0 |
| Reserved | 1-254 |
| Organization Specific Request | 255 |

##### 7.2.3.14.1 Basic Request

A Measurement ID equal to 0 in the Measurement Request element indicates a Basic Request. The Measurement Request field corresponding to a Basic Request is shown in Figure 9 and contains the Channel Number for which the request applies. The response to a Basic Request is a Basic Report. It is mandatory for a STA to respond to this request.

|  | Channel Number |
| --- | --- |
| Octets: | 1 |

Figure 9 - Measurement Request field format for a Basic Request

##### 7.2.3.14.2 Organization Specific Request

A Measurement ID equal to 255 in the Measurement Request element indicates a Organization Specific Request. The Measurement Request field corresponding to a Organization Specific Request is shown in Figure 10 and contains the information about a Organization Specific request. The response to a Organization Specific Request is a Organization Specific Report. It is optional for a STA to respond to this request.

|  | OID | Request |
| --- | --- | --- |
| Octets: | 1 | variable |

Figure 10 - Measurement Request field format for a Organization Specific Request

The IOD shall contain an organizationally unique identifier allocated by the IEEE.

The Request field contains a proprietary request. The length and format of the Request subfield is a function of the OID.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.15 Measurement Report element

The Measurement Report element contains a measurement report. The format of the Measurement Report element is shown in Figure 11.

|  | Element ID (39) | Length (variable) |  |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |
|  | Measurement Token | Measurement Duration | Measurement ID | Measurement Report |
| Octets: | 1 | 2 | 2 | variable |

Figure 11 - Measurement Report element format

The Element ID field shall be set equal to 39. The Length field is variable and depends on the length of the Measurement Report field. The minimum value of the Length field is 5.

The Measurement Token field shall be set equal to the Measurement Token in the corresponding Measurement Request element. If the Measurement Report element is being provided autonomously then the Measurement Token shall be set equal to zero.

The Measurement Duration field shall be set equal to the duration of the measurement in TUs. A value of zero when the Measurement Report element is the first element in the Measurement Report frame shall mean the measurement used the same duration and starting time as the last element in the last Measurement Report frame with the same values in the Dialog Token and Action Specific fields. Otherwise, a value of zero shall mean that the measurement used the same duration and starting time as the immediately previous Measurement Report element in the same Measurement Report frame.

The Measurement ID field shall be set to a number that identifies the format and contents of the Measurement Report field. The IEEE shall be responsible for allocating the Measurement ID number space. Those Measurement ID’s that have been allocated are shown in Table 5 and the format of the corresponding Measurement Report field are described in the following clauses.

The Measurement Report field shall contain the measurement report.

Table 5 – Measurement ID definitions for Measurement Report element

| Name | Measurement ID |
| --- | --- |
| Basic Report | 0 |
| Reserved | 1-254 |
| Organization Specific Report | 255 |

##### 7.3.2.15.1 Basic Report

A Measurement ID equal to 0 in the Measurement Report element indicates a Basic Report. The format of the Measurement Report field corresponding to a Basic Report is shown in Figure 12. The format of the Map subfield is shown in Figure 13.

| Name | Channel Number | Map |
| --- | --- | --- |
| Octets | 1 | 1 |

Figure 12 - Measurement Report field format for a Basic Report

| Name | BSS | Foreign PLCP Header | Unknown | Primary User | Unmeasured | Reserved (0) |
| --- | --- | --- | --- | --- | --- | --- |
| Bits | 0 | 1 | 2 | 3 | 4 | 5-8 |

Figure 13 - Map subfield format

Channel Number indicates the channel number to which the Basic Report applies.

The Map subfield is coded as a bit field and shall contain a:

- BSS field, which indicates (when set equal to 1) at least one valid MPDU was decoded in the channel during the measurement period.

- Foreign PLCP Header field, which indicates (when set equal to 1) at least one PLCP preamble was detected in the channel during the measurement period without a subsequent valid Signal field period. This may indicate the presence of a foreign PLCP header such as Hiperlan2.

- Unknown field, which indicates (when set equal to 1) transmissions were detected in the channel during the measurement period that cannot be characterized as from a primary user.

- Primary User field, which indicates (when set equal to 1) a primary user was detected operating in the channel during the measurement period. The algorithm to detect a primary user is not standardized but shall satisfy regulatory requirements.

- Unmeasured field (when set equal to 1), which indicates that this channel has not been measured. When the Unmeasured filed is set all the other fields shall be set equal to 0.

##### 7.3.2.15.2 Organization Specific Report

A Measurement ID equal to 255 in the Measurement Report element indicates a Organization Specific Report. The Measurement Report field corresponding to a Organization Specific Report is shown in Figure 14 and contains the information about a Organization Specific report.

|  | OID | Report |
| --- | --- | --- |
| Octets: | 1 | variable |

Figure 14 - Measurement Report field format for a Organization Specific Report

The IOD shall contain an organizationally unique identifier allocated by the IEEE.

The Report field contains a Organization Specific report. The length and format of the Report subfield is a function of the OID.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.16 Quiet element

The Quiet element defines a interval during which no transmission shall occur on the current operating channel. This may be used to assist in making accurate channel measurements. The format of the Quiet element is shown in Figure 15.

|  | Element ID (40) | Length (6) |  |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |
|  | Quiet Count | Quiet Period | Quiet Duration | Quiet Offset |
| Octets: | 1 | 1 | 2 | 2 |

Figure 15 - Quiet element format

The Element ID field shall be set equal to 40. The Length field shall be set equal to 4.

The Quiet Count field shall be set equal to the number of TBTT’s until the beacon interval during which the next quiet period shall start. A value of zero indicates the quiet period will start during the beacon interval starting at the next TBTT.

The Quiet Period field shall be set equal to the number of Beacon intervals between the start of quiet periods. A value of zero indicates that no periodic quiet period is defined.

The Quiet Duration field shall be set equal to the duration of the quiet period, expressed in TUs.

The Quiet Offset field shall be set equal to the offset of the start of the quiet period from the TBTT specified by the Quiet Count field, expressed in TUs. The value of the Quiet Offset field shall be less than one Beacon interval.

Insert the following sub-clause. Renumber figures and tables as appropriate.

#### 7.3.2.17 IBSS DFS element

The IBSS DFS element contains information useful for DFS operation in an IBSS. The format of the IBSS DFS element is shown in Figure 15.

|  | Element ID (41) | Length (variable) |  |
| --- | --- | --- | --- |
| Octets: | 1 | 1 |  |
|  | DFS Owner | DFS Recovery Interval | Channel Map |
| Octets: | 6 | 1 | 2*n |

Figure 16 – IBSS DFS element format

The DFS Owner ID field shall be set equal to 41. The Length field is variable.

The DFS Owner field shall be set equal to the individual IEEE MAC address of the STA that is the currently known DFS Owner in the IBSS.

The DFS Recovery Interval field indicates the time interval that shall be used for DFS Owner recovery, expressed as an integral number of beacon intervals. DFS Recovery Interval is static throughout the lifetime of the IBSS and is determined by a parameter to MLME-START.request at the STA that initiates the IBSS.

The Channel Map field shall contain a Basic Report subfield (see 7.3.2.15.1) for each channel supported by the STA transmitting the IBSS DFS element.

Insert clause 7.4 as follows. Renumber figures and tables as appropriate:

## 7.4 Spectrum Management Actions

The Action codes within the Spectrum Management category of an Action frame are defined in Table 6. The details of the frame bodies associated with each Action code are described in the following sub-clauses.

Table 6 – Spectrum Management Action codes

| Action code | Meaning |
| --- | --- |
| 0 | Measurement Request |
| 1 | Measurement Report |
| 2 | TPC Request |
| 3 | TPC Report |
| 4 | Channel Switch Announcement |
| 5-255 | Reserved |

Insert clause 7.4.1 as follows. Renumber figures and tables as appropriate:

### 7.4.1 Measurement Request frame format

The Measurement Request frame uses the Action frame body format and is transmitted by a STA requesting another STA to measure one or more channels. The format of the Measurement Request frame body is shown in Figure 17.

|  | Action descriptor |  |  |  |
| --- | --- | --- | --- | --- |
|  | Category (0) | Action (0) | Action Specific | Dialog Token |
| Octets: | 1 | 1 | 1 | 1 |
|  | Measurement Offset | Measurement Request Elements |  |  |
| Octets: | 1 | variable |  |  |

Figure 17 - Measurement Request frame body format

The Category field shall be set equal to 0 (representing Spectrum Management).

The Action field shall be set equal to 0 (representing a Measurement Request frame).

The Action Specific field shall be set equal to an activation delay, which represents the number of TBTTs, plus the offset specified by the Measurement Offset field, until the measurement specified by the first Measurement Request element should start. A value of 0 indicates that processing shall commence after the offset. A value of 1 indicates that processing shall commence after the next TBBT plus the offset. Processing of subsequent Measurement Request elements shall occur sequentially.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the request to identify the request/report transaction. A pending request is superseded by a subsequent request from the same STA with the same Dialog Token value.

The Measurement Offset field shall be set equal to the offset of the measurement specified by the first Measurement Request element from the time specified by the Action Specific field, expressed in TUs.

The Measurement Request Elements field shall contain one or more of the Measurement Request elements described in 7.3.2.14.

Insert clause 7.4.2 as follows. Renumber figures and tables as appropriate:

### 7.4.2 Measurement Report frame format

The Measurement Report frame uses the Action frame body format and shall be transmitted by a STA in response to a Measurement Request frame or by a STA autonomously providing measurement information. The format of the Measurement Report frame body is show in Figure 18.

|  | Action descriptor |  |  |  |
| --- | --- | --- | --- | --- |
|  | Category (0) | Action (1) | Action Specific | Dialog Token |
| Octets: | 1 | 1 | 1 | 1 |
|  | Measurement Report Elements |  |  |  |
| Octets: | variable |  |  |  |

Figure 18 - Measurement Report frame body format

The Category field shall be set equal to 0 (representing Spectrum Management).

The Action field shall be set equal to 1 (representing a Measurement Report frame).

The Action Specific field shall contain the status of the response as shown in Table 7.

The Dialog Token field shall be set equal to the value in the corresponding Measurement Request frame. If the Measurement Report frame is not being transmitted in response to a Measurement Request frame then the Dialog token shall be set equal to zero.

The Measurement Report Elements field shall contain one or more the Measurement Report elements described in 7.3.2.15.

Table 7 - Measurement Report Action Specific codes

| Value | Status |
| --- | --- |
| 0 | The frame contains one or more Measurement Report elements in response to one or more Measurement Request elements in a Measurement Request frame. |
| 1 | The frame contains one or more Measurement Report elements provided autonomously. |
| 2 | The status indicates that a STA received one or more Measurement Request elements in a Measurement Request frame for which it is unable to provide a response. The frame contains the corresponding Measurement Request elements. |
| 3-255 | Reserved. |

Insert clause 7.4.3 as follows. Renumber figures and tables as appropriate:

### 7.4.3 TPC Request frame format

The TPC Request frame uses the Action frame body format and is transmitted by a STA requesting another STA, with which it can communicate directly, for transmit power and link margin information. The format of the TPC Request frame body is shown in Figure 19.

|  | Action descriptor |  |  |  |
| --- | --- | --- | --- | --- |
|  | Category (0) | Action (2) | Action Specific (0) | Dialog Token |
| Octets: | 1 | 1 | 1 | 1 |
|  | TPC Request element |  |  |  |
| Octets: | 2 |  |  |  |

Figure 19 - TPC Request frame body format

The Category field shall be set equal to 0 (representing Spectrum Management).

The Action field shall be set equal to 2 (representing a TPC Request frame).

The Action Specific field shall be set equal to 0.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the request to identify the transaction. A pending request is superseded by a subsequent request from the same STA with the same Dialog Token.

The TPC Request element shall be set as described in 7.3.2.10.

Insert clause 7.4.4 as follows. Renumber figures and tables as appropriate:

### 7.4.4 TPC Report frame format

The TPC Report frame uses the Action frame body format and shall be transmitted by a STA in response to a TPC Request frame. The format of the TPC Report frame body is show in Figure 20.

|  | Action descriptor |  |  |  |
| --- | --- | --- | --- | --- |
|  | Category (0) | Action (3) | Action Specific (0) | Dialog Token |
| Octets: | 1 | 1 | 1 | 1 |
|  | TPC Report element |  |  |  |
| Octets: | 4 |  |  |  |

Figure 20 - TPC Report frame body format

The Category field shall be set equal to 0 (representing Spectrum Management).

The Action field shall be set equal to 3 (representing a TPC Report frame).

The Action Specific field shall be set equal to 0.

The Dialog Token field shall be set equal to the value in the corresponding TPC Request frame.

The TPC Report element shall be set as described 7.3.2.11.

Insert clause 7.4.5 as follows. Renumber figures and tables as appropriate:

### 7.4.5 Channel Switch Announcement frame format

The Channel Switch Announcement frame uses the Action frame body format and may be transmitted by a STA to advertise a channel switch. The format of the Channel Switch Announcement frame body is show in Figure 21.

|  | Action descriptor |  |  |  |
| --- | --- | --- | --- | --- |
|  | Category (0) | Action (4) | Action Specific (0) | Dialog Token |
| Octets: | 1 | 1 | 1 | 1 |
|  | Channel Switch Announcement element |  |  |  |
| Octets: | 4 |  |  |  |

Figure 21 – Channel Switch Announcement frame body format

The Category field shall be set equal  to 0 (representing Spectrum Management).

The Action field shall be set equal to 4 (representing a Channel Switch Announcement frame).

The Action Specific field shall be set equal to 0.

The Dialog Token field shall be set equal to 0.

The Channel Switch Announcement element shall be set as described 7.3.2.13.

Insert sub-clause 9.11 as follows. Renumber figures and tables, as appropriate:

## 9.11 TPC procedures

The CEPT Member Administrations in Europe require the use of transmit power control (TPC) by wireless LANs operating in the 5GHz band to reduce interference with primary users. The current CEPT regulations specify a regulatory maximum transmit power and an average mitigation requirement for each allowed channel (ERC/DEC/(99)23).

This clause describes TPC procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and may be useful for other purposes, such as interference reduction, range control and power optimization.

STAs shall use the TPC procedures defined in this clause if and only if the boolean MIB attribute dot11SpectrumManagementEnabled is true. dot11SpectrumManagementEnabled shall be set true when TPC is required by regulatory authorities. It may also be set true in other circumstances. The TPC procedures provide for the:

- Association of STAs with an AP in a BSS based on the STA’s power capability using Power Capability elements in Association Requests and Reassociation Requests (9.11.1).

- Specification of regulatory and local maximum transmit power levels for multiple channels using Country and Power Constraint elements in Beacons and Probe Responses (9.11.2).

- Selection of a transmit power for each MPDU transmitted in a channel within the constraints imposed by the rules (9.11.3).

- Optimization of transmit power based on a range of information, including transmit power and receive margin estimates (9.11.4).

For the purposes of TPC, a STA with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not operate in a BSS or IBSS unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons and Probe Responses received from the BSS or IBSS. Similarly, an AP with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not associate or reassociate a STA unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Association and Reassociation Requests received from the STA.

### 9.11.1 Association based on transmit power capability

A STA shall provide an AP with its maximum transmit power capability for at least the current channel when associating or reassociating using a Power Capability element in Association Request or Reassociation Request frames.

An AP may use the maximum transmit power capability of associated STAs to help determine the local transmit power constraint for any BSS it maintains.

An AP may reject an association or reassociation request from a STA if it considers the STA’s maximum transmit power capability is unacceptable or a transmit power capability is not provided.

### 9.11.2 Specification of a regulatory and local maximum transmit power levels

Regulatory authorities are responsible for defining a regulatory maximum transmit power level and a mitigation requirement for each allowed channel in a regulatory domain.

An AP operating a BSS, a STA starting an IBSS or a STA actively scanning shall not transmit in a channel unless the TPC related regulatory requirements for the channel are available.

An AP in a BSS or a STA starting an IBSS shall determine a local maximum transmit power level for other STAs in the BSS or IBSS so that the average mitigation requirements are satisfied. Note that a conservative approach is to set the local maximum transmit power level equal to the regulatory maximum transmit power level minus the average mitigation requirement. However, it may be possible to satisfy the average mitigation requirement using a higher local maximum transmit power level. A lower local maximum transmit power level may be used for other purposes, such as range control or interference reduction.

The regulatory maximum transmit power level for at least the current channel shall be communicated to other STAs in a BSS or an IBSS using a Country element in Beacons and Probe Responses. The local maximum transmit power level for at least the current channel shall be communicated to other STAs in the a BSS or an IBSS using a combination of the Country and Power Constraint elements in Beacons and Probe Responses.

In a BSS, the AP shall specify the local maximum transmit power in all Beacons and Probe Responses it transmits. The AP may vary the local maximum transmit power level during the life of a BSS. However, it should consider network stability in deciding how often or by how much to change the local maximum transmit power level.

In an IBSS, the STA starting the IBSS shall specify the local maximum transmit power level in all Beacons and Probe Responses it transmits. Other STAs in the IBSS shall adopt the same local maximum transmit power level for all Beacons and Probe Responses they transmit. The STAs in an IBSS shall not change the Local Power Constraint  during the life of an IBSS.

### 9.11.3 Selection of a transmit power

A STA may select any transmit power for each MPDU transmitted in a channel within the constraints imposed by the following rules:

- An AP operating in a BSS shall use a transmit power less than or equal to the regulatory maximum transmit power level for the channel. However, the AP must also ensure the average regulatory mitigation requirement is respected.

- Non-AP STAs operating in a BSS or an IBSS shall use a transmit power less than or equal to the local maximum transmit power level for the channel.

- STAs that are actively scanning for a BSS or an IBSS shall use a transmit power less than or equal to the regulatory maximum transmit power level for the channel. However, the STA shall also ensure the average regulatory mitigation requirement is respected.

### 9.11.4 Optimization of the transmit power

A STA may use any information, and in particular any path loss and receive margin estimates, to dynamically optimize the transmit power for transmissions of an MPDU to another STA. The optimization methods or criteria are not standardized.

A STA may use a TPC Request frame to request another STA with which it directly communicates to respond with a TPC Report frame containing link margin and transmit power information.

A STA receiving a TPC Request Frame shall respond with a TPC Report frame containing the power used to transmit the response in the Transmit Power field and the estimated link margin in a Link Margin field.

Insert clause 9.12 as follows. Renumber as appropriate.

## 9.12 DFS procedures

The CEPT Member Administrations in Europe require dynamic frequency selection (DFS) for wireless LANs operating in the 5GHz band to reduce interference with primary users, e.g. radar. The current CEPT regulations specify that systems should detect and avoid primary users. Further, they require the use of channel selection algorithms that result in uniform channel spreading across a minimum number of channels (ERC/DEC/(99)23).

This clause describes DFS procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and may be useful for other purposes.

STAs shall use the DFS procedures defined in this clause if and only if the boolean MIB attribute dot11SpectrumManagementEnabled is true when starting or joining a BSS or IBSS. dot11SpectrumManagementEnabled shall be set true when DFS is required by regulatory authorities. It may also be set true in other circumstances. The DFS procedures provide for:

- Quieting channels for testing (9.12.1)

- Testing channels for primary users (9.12.2).

- Discontinuing operations after detecting primary users (9.12.3)

- Detecting primary users (9.12.4)

- Requesting and reporting of measurements (9.12.5)

- Selecting and advertising a new channel (9.12.6)

For the purposes of DFS, a STA with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not operate in a BSS or IBSS unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons and Probe Responses received from the BSS or IBSS. Similarly, an AP with the boolean MIB attribute dot11SpectrumManagementEnabled set equal to true shall not associate or re-associate a STA unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Association and Reassociation Requests received from the STA.

### 9.12.1 Quieting channels for testing

All STAs in a BSS or IBSS shall ensure no part of any MPDU is transmitted during a specified common period to allow a channel to be tested more easily for primary users without interference. This period is known as a quiet period. All STAs shall consider a quiet period to be equivalent to a dwell time and adhere to all of the provisions relating to dwell time in clause 9.

An AP in a BSS may schedule quiet periods by transmitting Quiet elements in Beacons and Probe Responses. The AP may stop scheduling quiet periods or change the value of the Quiet Period field and the value of the Quiet Duration field in Quiet elements as required.

Only the STA starting an IBSS may start a schedule of quiet periods for testing, by transmitting a Quiet element in the Beacon establishing the IBSS. All STAs in an IBSS shall continue to schedule the quiet periods by including a Quiet element in any Beacons or Probe Responses, with the same Quiet Period, Quiet Duration and Quiet Offset, if any previous Beacons or Probe Responses in the IBSS contained a Quiet element.

Other clauses define events that should occur at TBTT. However, the specification of a quiet period may make TBTT unavailable for normal operations. Those clauses should consider the TBTT to be temporarily offset to the end of the quiet period.

### 9.12.2 Testing channels for primary users

A STA shall not use a channel that it knows contains primary users or has not been tested recently for the presence of primary users.  A STA shall test for the presence of primary users for at least:

- dot11StatupTestPeriod before operating in a new channel if the channel has not been tested for primary users for at least dot11StatupTestPeriod during the last dot11StatupTestValid.

- dot11StatupTestPeriod before operating in a new channel if a channel was previously determined to contain primary users during the last dot11StatupTestValid.

- dot11OperatingTestPeriod (where the period is only cumulated during testing) of each dot11OperatingTestCycle period while operating in a channel. Testing may occur in quiet periods or during normal operation.

A STA may start operating in a new channel without following the above start-up testing procedures if:

- The STA moves to the channel as a result of the receipt of a a Channel Switch Announcement from another STA in the STA’s IBSS or from the AP in the STA’s BSS.

- The STA is joining a BSS or IBSS that is not currently advertising, using Channel Switch Announcement elements, that it is about to move to a new channel.

### 9.12.3 Discontinuing operations after detecting primary users

If a STA is operating in a channel and detects radar in the channel then the STA shall discontinue all transmissions of:

- Data and Control type frames  within dot11MaxDataOperationsPeriod.

- MMPDUs within do11MaxManagementOperationsPeriod.

### 9.12.4 Detecting primary users

A STA shall use any method to detect primary users operating in a channel that satisfies regulatory requirements.

### 9.12.5 Requesting and reporting of measurements

A STA may measure one or more channels itself or a STA may request other STAs in the same BSS or IBSS to measure one or more channels on its behalf, either in a quiet period or during normal operation.

When requesting other STAs to measure one or more channels, a STA shall use a Measurement Request frame containing one or more Measurement Request elements. The measurement request may be sent to a unicast, multicast or broadcast destination address.

A STA that requests another STA to perform a measurement shall not transmit MPDUs to that STA or retransmit the measurement request until a enough time has passed for any required channel switches to occur, the measurement to be undertaken and the results reported. In determining this period, a STA shall assume that any required channel switches take less than dot11ChannelSwitchTime per switch.

A STA that receives a Measurement Request frame from a STA in its BSS or IBSS shall process the frame’s Measurement Request elements sequentially, starting at the time specified by the Action Specific field and the Measurement Offset field. A STA shall carry out at least those measurements that are requested by its AP in a BSS and are specified as mandatory.

The results of all requested measurements shall be returned to the requesting STA in one or more Measurement Report elements, in one or more Measurement Report frames with an Action Specific field equal to 0.

When a STA declines to carry out measurements it shall return the elements containing the refused measurement requests to the requesting STA in one or more Measurement Report frames with an Action Specific field equal to 2.

A STA may autonomously report measurements to another STA in its BSS or IBSS using a Measurement Report frame with an Action Specific field equal to 1 containing one or more Measurement Report elements. The Measurement Report elements shall contain sufficient information for the STA receiving the frame to understand the reported measurement results. A STA in a BSS detecting a licensed user shall attempt to use this mechanism to notify its AP. A STA in an IBSS detecting a licensed user shall attempt use this mechanism to broadcast at least one  notification to other  STAs in the IBSS.

An STA in an IBSS shall also autonomously report measurements to other STAs in the IBSS using the IBSS Basic Reports fields in the IBSS DFS element in a Beacon or Probe Response.

#### 9.12.6 Selecting and advertising a new channel

A STA may decide to stop operating in a channel at any time. The algorithm used to decide to stop operating in a channel is outside the scope of the standard but shall satisfy any regulatory requirements.

A STA may use a variety of information, including information in Supported Channel elements (BSS only) and IBSS DFS elements (IBSS only) and information gathered from measurements undertaken by the STA and other STAs to assist the selection of the new channel. The algorithm to choose a new channel is not standardized but shall satisfy any regulatory requirements, including uniform spreading rules and channel testing rules.

An AP in a BSS or a STA in an IBSS may attempt to move the other STAs associated with its BSS or IBSS to the selected new channel by advertising the imminent switch sending Channel Switch Announcement elements in Beacons, Probe Response and Channel Announcement frames. The channel switch should be scheduled, if possible, so that all STAs in the BSS or IBSS, including those in sleep mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch. The AP in a BSS or the STA in an IBSS should attempt to select a new channel that is supported by all the STAs in the BSS orIBSS.

The DFS Owner may attempt to move the members of the IBSS to a new operating channel selected according to the rules in this clause. The DFS Owner shall announce a channel switch by sending an IBSS Channel Switch Announcement frame. The Address 1 field of this frame shall be set to the broadcast address. The Activation Delay field shall indicate the number of TBTTs until the intended channel switch. The DFS Owner may send the Channel Switch Announcement frame more than once before the intended channel switch time. The DFS Owner shall also include the Channel Switch Announcement element in each Beacon, or Probe Response frame transmitted until the intended channel switch time. The activation delay shall be set to indicate the appropriate switch time and shall decrement at each TBTT.

A STA that receives a channel switch announcement and intends to move channel shall update the Channel Switch Count field and relay that announcement in Beacon and Probe Response management frames that it transmits. The STA shall switch to the specified channel immediately before the announced TBTT.

A STA may perform DFS Owner recovery if it has not received a Channel Switch Announcement frame, or a Beacon or Probe Response frame containing a Channel Switch Announcement element within the DFS Recovery Time from the end of the frame within which it received a valid notification of the primary user. During DFS recovery any STAs that sends a Beacon for the first TBTT after the DFS Recovery Interval expires shall assume that they are a DFS Owner and shall immediately schedule a channel switch.

A DFS Owner STA that receives a valid DFS element from a STA in the same IBSS containing a DFS Owner address that differs from its own individual MAC address shall immediately cease to be DFS Owner. The STA shall adopt the DFS Owner address and DFS Recovery Interval from the received DFS element. If a Channel Switch Announcement element was received within the same frame, any pending channel switch shall be cancelled and a new channel switch scheduled based on the received information. This is necessary to provide resolution between multiple STAs that have assumed DFS ownership in DFS Recovery through beacon transmissions that have not been received correctly, collisions, or coalescing.

## 10.3 MLME SAP interface

Editors note: Need appropriate changes here – base on Simon Black’s proposal

### 17.3.8.3.3 Channelization

Change  Table 88 as follows

Table 88 – Valid operation channel numbers by regulatory domain and band

| Regulatory domain | Band | Operating Channel Numbers | Channel Center Frequencies |
| --- | --- | --- | --- |
| USA CEPT | U-NII lower band 5.15-5.25 GHz | 36 40 44 48 | 5,180 MHz 5,200 MHz 5,220 MHz 5,240 MHz |
| USA CEPT | U-NII middle band 5.25-5.35 GHz | 52 56 60 64 | 5,260 MHz 5,280 MHz 5,300 MHz 5,320 MHz |
| CEPT | 5.47-5.725 GHz | 100 104 108 112 116 120 124 128 132 136 140 | 5,500 MHz 5,520 MHz 5,540 MHz 5,560 MHz 5,580 MHz 5,600 MHz 5,620 MHz 5,640 MHz 5,660 MHz 5,680 MHz 5,700 MHz |
| USA | U-NII upper band 5.725-5.825 GHz | 149 153 157 161 | 5,745 MHz 5,765 MHz 5,785 MHz 5,805 MHz |

Delete paragraph after Table 88

Change paragraph after Figure 119

In Figure 119, showing the frequency channel plan for the USA, the center frequency is indicated, however, no sub-carrier is allocated on the center frequency as described in Figure 117.

### 17.3.9.1 Transmit power levels

Replace first paragraph with following

The maximum allowable output power by regulatory domain is shown in Table 89.

Replace Table 89 with following

Table 89 – Transmit power level by regulatory domain

| Frequency band | USA (Maximum output power with up to 6dBi antenna gain) | CEPT (EIRP) |
| --- | --- | --- |
| 5.150-5.250 GHz | 40 mW (2.5 mW/MHz) | 200 mW |
| 5.250-5.350 GHz | 200 mW (12.5 mW/MHz) | 200 mW |
| 5.470-5.725 GHz | - | 1 W |
| 5.725-5.825 GHz | 800 mW (50 mW/MHz) | - |

# Annex A – PICS

### A.4.3 IUT configuration

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF9 | Is Spectrum Management operation supported? | ? | O.4 | Yes, No |

### A.4.11 TPC extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TP1 |  |  |  |  |
| TP2 |  |  |  |  |
| TP3 |  |  |  |  |
| … |  |  |  |  |
| TPn |  |  |  |  |

### A.4.12 DFS extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| DF1 |  |  |  |  |
| DF2 |  |  |  |  |
| DF3 |  |  |  |  |
| … |  |  |  |  |
| DFn |  |  |  |  |

# Annex C – Formal description of MAC operation

Editors note: the formal description in the 802.11 MAC is filled with syntactic and semantic errors. 802.11h are not proposing to add anything to this very flawed description unless it is verified first, which is not likely to occur in the near future.

# Annex D – ASN.1 encoding of the MAC and PHY MIB

In “Major Sections” of Annex D, add the following text to the end of the Station Management attributes

-- dot11SpectrumManagementCapabilityTable ::= {dot11smt 8}

In “SMT Station Config Table of Annex D, add the following text to the end of the dot11StationConfigEntry sequence list

dot11SpectrumManagementCapabilityImplemented TruthValue,

dot11SpectrumManagementCapabilityEnabled TruthValue

Add the following elements to the end of the dot11StationConfigEntry element definitions behind dot11AuthenticateFailStation

dot11SpectrumManagementCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station implementation is capable of supporting spectrum management. The capability is disabled otherwise. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 24 }

dot11SpectrumManagementCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“A STA will use the defined TPC and DFS procedures if and only if this attribute is TRUE. The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 25 }

-- ********************************************************************

-- * dot11SpectrumManagementCapability TABLE

-- ********************************************************************

dot11SpectrumManagementCapabilityTable OBJECT-TYPE

SYNTAX SEQUENCE OF dot11SpectrumManagementCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCIPTION

“This (conceptual) table of attributes for spectrum management”

::= {dot11smt 8}

dot11SpectrumManagementCapabilityEntry OBJECT-TYPE

SYNTAX Dot11SpectrumManagementCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“An entry (conceptual row) in the Spectrum Management Capability Table.

IfIndex – Each 802.11 interface is represented by an ifEntry. Interface tables in this MIB are indexed by ifIndex.”

INDEX {ifIndex, dot11SpectrumManagementCapabilityIndex}

::= {dot11SpectrumManagementCapabilityTable 1}

dot11SpectrumManagementCapabilityEntry ::=

SEQUENCE {

dot11SpectrumManagementCapabilityIndex Integer32,

dot11FirstChannelNumber Integer32,

dot11NumberofChannels Integer32,

dot11MitigationRequirement Integer32,

dot11StartupTestPeriod Integer32,

dot11StartupTestValid Integer32,

dot11OperatingTestPeriod Integer32,

dot11OperatingTestCycle Integer32,

dot11MaxDataOperationsPeriod Integer32,

do11MaxManagementOperationsPeriod Integer32,

dot11ChannelSwitchTime Integer32}

dot11SpectrumManagementCapabilityIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar objects in the Spectrum Management Capability Table."

::= { dot11SpectrumManagementCapabilityEntry 1}

dot11FirstChannelNumber OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the lowest channel number in the sub-band for the associated domain country string. The default value of this attribute shall be zero."

::= { dot11SpectrumManagementCapabilityEntry 2}

dot11NumberofChannels OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the total number of channels allowed in the sub-band for the associated domain country string. The default value of this attribute shall be zero."

::= { dot11SpectrumManagementCapabilityEntry 3}

dot11MitigationRequirement OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the average mitigation requirement in dB required in the sub-band. The default value of this attribute shall be 3dB."

::= { dot11SpectrumManagementCapabilityEntry 4}

dot11StartupTestPeriod OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the start up test period in seconds of this channel. The default value of this attribute shall be 10 seconds"

::= { dot11SpectrumManagementCapabilityEntry 5}

dot11StartupTestValid OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the validity of start up test in seconds for this channel. The default value of this attribute shall be 24*60*60"

::= { dot11SpectrumManagementCapabilityEntry 6}

dot11OperatingTestPeriod OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the aggregate period of an operating test in TUs for this channel each cycle. The default value of this attribute shall be 20 TUs"

::= { dot11SpectrumManagementCapabilityEntry 7}

dot11OperatingTestCycle OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the cycle period for an operating test in TUs for this channel. The default value of this attribute shall be 100 TUs "

::= { dot11SpectrumManagementCapabilityEntry 8}

dot11MaxDataOperationsPeriod OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum period an STA may transmit MPDUs in a channel after detecting a primary user. The default value of this attribute shall be 200 TUs"

::= { dot11SpectrumManagementCapabilityEntry 9}

do11MaxManagementOperationsPeriod OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum period (only measured while transmission is occurring) an STA may transmit MMPDUs in a channel after detecting a primary user. The default value of this attribute shall be 500 TUs"

::= { dot11SpectrumManagementCapabilityEntry 10}

dot11ChannelSwitchTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate assumed channel switch time, measured in us. The default value of this attribute shall be 2000us"

::= { dot11SpectrumManagementCapabilityEntry 11}

-- ******************************************************************

-- * End of dot11SpectrumManagementCapability TABLE

-- ******************************************************************

# Annex E – Bibliography

Editors note: none so far

ERC documents are available from European Radiocommunications Office, Midtermolen 1, DK-2100, Copenhagen, Denmark (http://www.ero.dk).