IEEE P802.11
Wireless LANs

Proposed TKIP 48bit IV Frame Format

Date:	June 11, 2002

Authors:		Onno Letanche
	Agere Systems
	Zadelstede 1 - 10
	3431 JZ  Nieuwegein, Netherlands
	Phone: +31 30-609-7454
	E-mail: oletanche@agere.com

Dorothy Stanley
	Agere Systems
	2000 North Naperville Rd
	Naperville, IL, 60566
	Phone: +1 630-979-1572
	E-mail: dstanley@agere.com

##### TKIP MPDU 48-bit IV Format

Figure 1 below shows the proposed layout of the encrypted MPDU when using TKIP-based privacy and an additional 4 octets for the Extended IV field.

Figure 1 - Construction of Expanded TKIP MPDU to Include Extended IV

The ExtIV bit in the KeyId byte indicates the presence or absence of an extended IV. If the ExtIV bit is ‘0’ only the old-style non-extended IV is transferred. If the ExtIV bit is ‘1’ an extended IV of 4 octets follows the original IV.

TKIP uses a 48 bit IV that start with all zeroes (0x0000 0000 0000) and will be incremented monotonically upon sending an MSDU. The 48 bit IV is divided into two parts the 32 bit upper part IV32 and the 16 bit lower part IV16.

RC4KEY[0-2] are used in the TKIP Phase2 key hashing. RC4KEY[0-2] are formed by expanding IV16.

The expansion of IV16 is as follows:( see doc. IEEE 802.11-2/282r2)

RC4KEY[0] =  Hi8(IV16)

RC4KEY[1] = (Hi8(IV16) | 0x20) & 0x7F

RC4KEY[2] =  Lo8(IV16)

Octets IV[2-5] (IV32) are used in the TKIP phase 1 key hashing. As soon as an IV16 sequence number roll-over occurs (0xFFFF --> 0x0000) the other part of the extended IV (IV32) must be incremented.

Frame Construction Rationale and Issues

- Align on word boundaries to allow ease of implementation on legacy devices

- Add 4 bytes of extended IV, an additional 32 bits after the existing IV/KeyID Field. This removes IV exhaustion as a reason to re-key.

- Retain IV/Key-ID of 4 octets in the form as defined with basic WEP, the ‘old style’ IV bytes now contain RC4KEY[0-2]. The receiving station directly can feed these octets into the decryption hardware.

- Key ID Byte changes – Use one bit (bit 5) to indicate that an extended IV is present. This allows the receiver/transmitter to know that the extended mode is present. The receiver/transmitter processes the following 4 bytes as the extended IV. The receiving/transmitting station also uses the value of IV16 to detect that a key rollover has occurred.  When a key rollover has occurred, a new Phase 1 value with an incremented IV32 is calculated, and used to decrypt the received/transmitted frame.

- The extended IV field is not encrypted