IEEE P802.11
Wireless LANs

Example FEC Frame Encoding

Date:	May 13, 2002

Authors:	Sid Schrum, Matthew Shoemake, Steve Pope,
 	Lior Ophir; Texas Instruments. John Kowalski, Sharp Labs
	Address: 3000 Aerial Center Parkway, Suite 160, Morrisville, NC  27560
	Phone: (919) 463-1043
	Fax: (919) 463-4939
	e-Mail:

Abstract

Description of and draft standard text for example encoding of a FEC frame for 802.11 TGe.

# 1.0  Introduction

This document contains proposed draft text for the IEEE 802.11e draft standard that specifies an example 802.11e frame before and after FEC encoding.  The purpose of the normative text is to illustrate by example how to FEC encode a frame according to the 802.11e draft standard.

The example frame is not designed to cover all possible encoding situations.  The draft standard normative text that describes how to encode an FEC frame should be sufficient to correctly implement FEC encoding and decoding. The example frame provided here is designed to augment the draft standard normative text by showing a clarifying encoding example.

# 2.0  Example Frame

## Overview of Example Frame

The example frame prior to FEC encoding has the following characteristics:

- The frame type / subtype will be QoS Data, with To DS = 0 and From DS = 1, FEC = 1, and all other Frame Control fields equal to zero.

- The frame will have three address fields

- The RA field will be unicast

- The QoS Control Field will specify an ack policy of burst (delayed) ack, FEC = 1, and all other fields will be zero.

- The frame body will consist of 206 bytes, not including CRC, consisting of the squence 01’h, 02’h, 03’h etc.

- The frame will not be encrypted.

“Buffer Order” below refers to a natural bit within byte presentation, where the least significant bit in a byte is shown on the right.  “Transmit Order” refers to the order in which bytes are transmitted on the medium.

## Example Frame Description Prior to FEC Encoding

| Byte Number | Value, Buffer Order | Value, Transmit Order | ### Description |
| --- | --- | --- | --- |
| 1 | 88’h | 11’h | Frame Control byte 0. QoS Data frame |
| 2 | 82’h | 41’h | Frame Control byte 1. FEC, From DS. |
| 3 | 00’h | 00’h | Duration / ID byte 0 |
| 4 | 80’h | 01’h | Duration / ID byte 1 |
| 5 | 10’h | 08’h | Address 1.0 |
| 6 | 11’h | 88’h | Address 1.1 |
| 7 | 11’h | 88’h | Address 1.2 |
| 8 | 11’h | 88’h | Address 1.3 |
| 9 | 11’h | 88’h | Address 1.4 |
| 10 | 11’h | 88’h | Address 1.5 |
| 11 | 20’h | 88’h | Address 2.0 |
| 12 | 22’h | 04’h | Address 2.1 |
| 13 | 22’h | 44’h | Address 2.2 |
| 14 | 22’h | 44’h | Address 2.3 |
| 15 | 22’h | 44’h | Address 2.4 |
| 16 | 22’h | 44’h | Address 2.5 |
| 17 | 30’h | 0C’h | Address 3.0 |
| 18 | 33’h | CC’h | Address 3.1 |
| 19 | 33’h | CC’h | Address 3.2 |
| 20 | 33’h | CC’h | Address 3.3 |
| 21 | 33’h | CC’h | Address 3.4 |
| 22 | 33’h | CC’h | Address 3.5 |
| 23 | 00’h | 00’h | Sequence Control 0 |
| 24 | 00’h | 00’h | Sequence Control 1 |
| 25 | 78’h | 00’h | QoS Control 0. TID=8’h, FEC, final, no ack |
| 26 | 00’h | 51’h | QoS Control 1. |
| 27 | 01’h | 80’h | Frame Body 1 |
| 28 | 02’h | 40’h | Frame Body 2 |
| 29 | 03’h | C0’h | Frame Body 3 |
| … | … | … | … |
| 205 | CD’h | B3’h | Frame Body 205 |
| 206 | CE’h | 73’h | Frame Body 206 |

# 3.0  Draft Text – FEC encoding without scrambler

# Annex H

(informative)

Add Annex H (a new annex)

# An Example of Encoding a Frame for FEC

### H.1 Introduction

The purpose of this annex is to show an example encoding of a frame with the QoS FEC option, as described in Clause 7 of IEEE Standard 802.11e.  The example frame is a QoS Data frame with no Address 4 field prior to encoding, and with a 206 byte MAC payload.  After encoding, the last RS block consists of the last two bytes of FEC FCS plus 16 RS parity bytes.

### H.2 Example Frame Prior to FEC Encoding

The example frame prior to FEC Encoding is shown in Table H.1.  The example frame is show prior to padding of Address Field 4 and prior to cacluation and appending of FEC FCS.  The bit ordering of bits within bytes is consistent with the way the MAC might buffer frame bytes prior to transmission; that is, least significant bit in each byte is shown to the right.

| ## | Val | Val | Val | Val | Val | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1..10 | 88 | 82 | 00 | 80 | 10 | 11 | 11 | 11 | 11 | 11 |
| 11..20 | 20 | 22 | 22 | 22 | 22 | 22 | 30 | 33 | 33 | 33 |
| 21..30 | 33 | 33 | 00 | 00 | 78 | 00 | 01 | 02 | 03 | 04 |
| 31..40 | 05 | 06 | 07 | 08 | 09 | 0a | 0b | 0c | 0d | 0e |
| 41..50 | 0f | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| 51..60 | 19 | 1a | 1b | 1c | 1d | 1e | 1f | 20 | 21 | 22 |
| 61..70 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 2a | 2b | 2c |
| 71..80 | 2d | 2e | 2f | 30 | 31 | 32 | 33 | 34 | 35 | 36 |
| 81..90 | 37 | 38 | 39 | 3a | 3b | 3c | 3d | 3e | 3f | 40 |
| 91..100 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 4a |
| 101..110 | 4b | 4c | 4d | 4e | 4f | 50 | 51 | 52 | 53 | 54 |
| 111..120 | 55 | 56 | 57 | 58 | 59 | 5a | 5b | 5c | 5d | 5e |
| 121..130 | 5f | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 |
| 131..140 | 69 | 6a | 6b | 6c | 6d | 6e | 6f | 70 | 71 | 72 |
| 141..150 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 7a | 7b | 7c |
| 151..160 | 7d | 7e | 7f | 80 | 81 | 82 | 83 | 84 | 85 | 86 |
| 161..170 | 87 | 88 | 89 | 8a | 8b | 8c | 8d | 8e | 8f | 90 |
| 171..180 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 9a |
| 181..190 | 9b | 9c | 9d | 9e | 9f | a0 | a1 | a2 | a3 | a4 |
| 191..200 | a5 | a6 | a7 | a8 | a9 | aa | ab | ac | ad | ae |
| 201..210 | af | b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 |
| 211..220 | b9 | ba | bb | bc | bd | be | bf | c0 | c1 | c2 |
| 221..230 | c3 | c4 | c5 | c6 | c7 | c8 | c9 | ca | cb | cc |
| 231..240 | cd | ce | --- | --- | --- | --- | --- | --- | --- | --- |

### H.3 Example Frame After FEC Encoding

The example frame after FEC encoding is shown in Table H2.  The bit ordering of the bits with bytes is consistent with transmission order; that is, the least significant bit in each byte is shown to the left.

| ## | Val | Val | Val | Val | Val | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1..10 | 11 | 41 | 00 | 01 | 08 | 88 | 88 | 88 | 88 | 88 |
| 11..20 | 04 | 44 | 44 | 44 | 44 | 44 | 0c | cc | cc | cc |
| 21..30 | cc | cc | 00 | 00 | ff | ff | ff | ff | ff | ff |
| 31..40 | 1e | 00 | c3 | 24 | d6 | 48 | 9c | ae | a7 | c7 |
| 41..50 | df | d9 | be | 52 | cc | a2 | 4a | 92 | 80 | 40 |
| 51..60 | c0 | 20 | a0 | 60 | e0 | 10 | 90 | 50 | d0 | 30 |
| 61..70 | b0 | 70 | f0 | 08 | 88 | 48 | c8 | 28 | a8 | 68 |
| 71..80 | e8 | 18 | 98 | 58 | d8 | 38 | b8 | 78 | f8 | 04 |
| 81..90 | 84 | 44 | c4 | 24 | a4 | 64 | e4 | 14 | 94 | 54 |
| 91..100 | d4 | 34 | b4 | 74 | f4 | 0c | 8c | 4c | cc | 2c |
| 101..110 | ac | 6c | ec | 1c | 9c | 5c | dc | 3c | bc | 7c |
| 111..120 | fc | 02 | 82 | 42 | c2 | 22 | a2 | 62 | e2 | 12 |
| 121..130 | 92 | 52 | d2 | 32 | b2 | 72 | f2 | 0a | 8a | 4a |
| 131..140 | ca | 2a | aa | 6a | ea | 1a | 9a | 5a | da | 3a |
| 141..150 | ba | 7a | fa | 06 | 86 | 46 | c6 | 26 | a6 | 66 |
| 151..160 | e6 | 16 | 96 | 56 | d6 | 36 | b6 | 76 | f6 | 0e |
| 161..170 | 8e | 4e | ce | 2e | ae | 6e | ee | 1e | 9e | 5e |
| 171..180 | de | 3e | be | 7e | fe | 01 | 81 | 41 | c1 | 21 |
| 181..190 | a1 | 61 | e1 | 11 | 91 | 51 | d1 | 31 | b1 | 71 |
| 191..200 | f1 | 09 | 89 | 49 | c9 | 29 | a9 | 69 | e9 | 19 |
| 201..210 | 99 | 59 | d9 | 39 | b9 | 79 | f9 | 5 | 85 | 45 |
| 211..220 | c5 | 25 | a5 | 65 | e5 | 15 | 95 | 55 | d5 | 35 |
| 221..230 | b5 | 75 | f5 | 0d | 8d | 4d | cd | 2d | ad | 6d |
| 231..240 | ed | 1d | 9d | 5d | dd | 3d | bd | 7d | fd | 03 |
| 241..250 | 83 | 43 | c3 | 23 | a3 | 63 | e3 | 13 | 93 | 53 |
| 251..260 | d3 | 33 | b3 | 73 | b3 | 55 | c1 | 25 | 11 | 35 |
| 261..270 | 6b | 74 | 23 | 5a | 6e | c8 | af | 95 | b9 | 07 |
| 271..280 | 8c | 3b | 30 | ca | d7 | 0b | ce | 3c | 5a | 73 |
| 281..290 | 7d | ad | 6e | 5f | 84 | 82 | d3 | 9d | 51 | 62 |
| 291..300 | 97 | dc | 05 | 1a | --- | --- | --- | --- | --- | -- |

# 4.0  Draft Text – FEC encoding with scrambler

# Annex H

(informative)

Add Annex H (a new annex)

# An Example of Encoding a Frame for FEC

### H.1 Introduction

The purpose of this annex is to show an example encoding of a frame with the QoS FEC option, as described in Clause 7 of IEEE Standard 802.11e.  The example frame is a QoS Data frame with no Address 4 field prior to encoding, and with a 206 byte MAC payload.  After encoding, the last RS block consists of the last two bytes of FEC FCS plus 16 RS parity bytes.

### H.2 Example Frame Prior to FEC Encoding

The example frame prior to FEC Encoding is shown in Table H.1.  The example frame is show prior to padding of Address Field 4 and prior to cacluation and appending of FEC FCS.  The bit ordering of bits within bytes is consistent with the way the MAC might buffer frame bytes prior to transmission; that is, least significant bit in each byte is shown to the right.

| ## | Val | Val | Val | Val | Val | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1..10 | 88 | 82 | 00 | 80 | 10 | 11 | 11 | 11 | 11 | 11 |
| 11..20 | 20 | 22 | 22 | 22 | 22 | 22 | 30 | 33 | 33 | 33 |
| 21..30 | 33 | 33 | 00 | 00 | 78 | 00 | 01 | 02 | 03 | 04 |
| 31..40 | 05 | 06 | 07 | 08 | 09 | 0a | 0b | 0c | 0d | 0e |
| 41..50 | 0f | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| 51..60 | 19 | 1a | 1b | 1c | 1d | 1e | 1f | 20 | 21 | 22 |
| 61..70 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 2a | 2b | 2c |
| 71..80 | 2d | 2e | 2f | 30 | 31 | 32 | 33 | 34 | 35 | 36 |
| 81..90 | 37 | 38 | 39 | 3a | 3b | 3c | 3d | 3e | 3f | 40 |
| 91..100 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 4a |
| 101..110 | 4b | 4c | 4d | 4e | 4f | 50 | 51 | 52 | 53 | 54 |
| 111..120 | 55 | 56 | 57 | 58 | 59 | 5a | 5b | 5c | 5d | 5e |
| 121..130 | 5f | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 |
| 131..140 | 69 | 6a | 6b | 6c | 6d | 6e | 6f | 70 | 71 | 72 |
| 141..150 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 7a | 7b | 7c |
| 151..160 | 7d | 7e | 7f | 80 | 81 | 82 | 83 | 84 | 85 | 86 |
| 161..170 | 87 | 88 | 89 | 8a | 8b | 8c | 8d | 8e | 8f | 90 |
| 171..180 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 9a |
| 181..190 | 9b | 9c | 9d | 9e | 9f | a0 | a1 | a2 | a3 | a4 |
| 191..200 | a5 | a6 | a7 | a8 | a9 | aa | ab | ac | ad | ae |
| 201..210 | af | b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 |
| 211..220 | b9 | ba | bb | bc | bd | be | bf | c0 | c1 | c2 |
| 221..230 | c3 | c4 | c5 | c6 | c7 | c8 | c9 | ca | cb | cc |
| 231..240 | cd | ce | --- | --- | --- | --- | --- | --- | --- | --- |

### H.3 Example Frame After FEC Encoding

The example frame after FEC encoding is shown in Table H2.  The bit ordering of the bits with bytes is consistent with transmission order; that is, the least significant bit in each byte is shown to the left.

| ## | Val | Val | Val | Val | Val | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1..10 | 11 | 41 | 00 | 01 | 08 | 08 | 08 | 08 | 08 | 08 |
| 11..20 | 04 | 44 | 44 | 44 | 44 | 44 | 0c | 4c | 4c | 4c |
| 21..30 | 4c | 4c | 00 | 00 | 7f | 7f | 7f | 7f | 7f | 7f |
| 31..40 | 1e | 00 | 7e | 4e | 0b | 59 | 59 | 10 | 45 | 55 |
| 41..50 | 48 | 4d | 28 | 15 | 4d | 27 | 10 | 38 | 4f | 66 |
| 51..60 | 42 | 2 | 66 | 4e | 7b | 08 | 3b | 1e | 67 | 11 |
| 61..70 | 45 | 3b | 12 | 06 | 77 | 38 | 56 | 65 | 2c | 2c |
| 71..80 | 64 | 45 | 2e | 68 | 0e | 24 | 57 | 3a | 12 | 13 |
| 81..90 | 41 | 59 | 3a | 45 | 19 | 7e | 6d | 1c | 0d | 6e |
| 91..100 | 39 | 54 | 18 | 4d | 2a | 09 | 59 | 63 | 46 | 17 |
| 101..110 | 50 | 2f | 17 | 28 | 0e | 4c | 6e | 48 | 66 | 3d |
| 111..120 | 25 | 70 | 3e | 49 | 68 | 7d | 36 | 14 | 1a | 15 |
| 121..130 | 65 | 3b | 76 | 12 | 56 | 1a | 47 | 09 | 39 | 2f |
| 131..140 | 33 | 3c | 7e | 54 | 43 | 76 | 6b | 55 | 34 | 69 |
| 141..150 | 73 | 3a | 32 | 57 | 6d | 40 | 20 | 6c | 55 | 4b |
| 151..160 | 4e | 6b | 44 | 0e | 35 | 29 | 6a | 51 | 65 | 0f |
| 161..170 | 1f | 6d | 19 | 23 | 62 | 7b | 09 | 45 | 4e | 24 |
| 171..180 | 7b | 0f | 79 | 41 | 46 | 4e | 27 | 43 | 63 | 67 |
| 181..190 | 0f | 7a | 79 | 3a | 5f | 66 | 70 | 44 | 7a | 13 |
| 191..200 | 7f | 76 | 79 | 57 | 4 | 2d | 6d | 65 | 34 | 2f |
| 201..210 | 29 | 0f | 45 | 56 | 7b | 13 | 6e | 40 | 18 | 3b |
| 211..220 | 24 | 18 | 3f | 6c | 6d | 0c | 2f | 38 | 35 | 19 |
| 221..230 | 0c | 2b | 70 | 58 | 22 | 47 | 76 | 51 | 6e | 16 |
| 231..240 | 59 | 0f | 0d | 6f | 29 | 67 | 7c | 24 | 0f | 3f |
| 241..250 | 08 | 69 | 1c | 37 | 55 | 1b | 64 | 64 | 7a | 77 |
| 251..260 | 73 | 57 | 5b | 46 | 2d | 2d | 3 | 50 | 0e | 68 |
| 261..270 | 70 | 42 | 1d | 21 | 5b | 62 | 48 | 61 | 64 | 45 |
| 271..280 | 46 | 62 | 3e | 43 | 5c | 1c | 41 | 25 | 4b | 21 |
| 281..290 | 58 | 1e | 43 | 54 | 7a | 07 | 14 | 3b | 3d | 58 |
| 291..300 | 46 | 9e | df | 04 | --- | --- | --- | --- | --- | --- |