IEEE P802.11
Wireless LANs

Proposed Test Vectors for IEEE 802.11 TGi

Date:	September 10, 2002

Authors: 	Paul Lambert and Mike Sabin

Airgo Networks, Inc.
		900 Arastradero, Palo Alto, CA, 94304 USA
		Phone: 650 475 2006
		Fax: 650 475 1990
		e-Mail: Paul@AirgoNetworks.Com

Abstract

This document contains test vectors for inclusion into the base IEEE 802.11Tgi specification.  Test vectors are included for WEP, TKIP, AES-OCB (WRAP), AES-CCM (CCMP),  PRF and key hierarchy functions.

# Appendix:  Test vectors (informative)

This appendix presents numerical examples which may be useful to implementers of the specification.  The examples illustrate frame encapsulation using WEP, TKIP, and AES-OCB. Test vectors are also provided for the key generation process.

## Notation

In the examples here, frames are represented as a stream of octets, each octet in hex notation, sometimes with text annotation.  The order of transmission for octets is left to right, top to bottom.  For example, consider the following representation of a frame:

| Description #1 | 00 01 02 03 |
| --- | --- |
|  | 04 05 |
| Description #2 | 06 07 08 |

The frame consists of nine octets, represented in hex notation as “00”, “01”, ..., “08”.  The octet represented by “00” is transmitted first, and the octet represented by “08” is transmitted last.  Similar tables are used for other purposes, such as describing a cryptographic operation.

In the text discussion outside of tables, integer values are represented in either hex notation using an “0x” prefix or in decimal notation using no prefix.  For example, the hex notation 0x12345 and the decimal notation 74565 represent the same integer value.

## WEP Encapsulation

The discussion here represents an RC4 encryption using a table that shows the key, plaintext input, and ciphertext output.  For reference, here is a table that describes test vector “Commerce” of  <draft-kaukonen-cipher-arcfour-03.txt>, a work-in-progress.

| Key | 61 8a 63 d2 fb |
| --- | --- |
| Plaintext | dc ee 4c f9 2c |
| Ciphertext | f1 38 29 c9 de |

The MPDU data, prior to WEP encapsulation, is as follows:

| MPDU data | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| --- | --- |

RC4 encryption is performed as follows:

| Key | fb 02 9e 30 31 32 33 34 |
| --- | --- |
| Plaintext | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 1b d0 b6 04 |
| Ciphertext | f6 9c 58 06 bd 6c e8 46 26 bc be fb 94 74 65 0a ad 1f 79 09 b0 f6 4d 5f 58 a5 03 a2 58 b7 ed 22 eb 0e a6 49 30 d3 a0 56 a5 57 42 fc ce 14 1d 48 5f 8a a8 36 de a1 8d f4 2c 53 80 80 5a d0 c6 1a 5d 6f 58 f4 10 40 b2 4b 7d 1a 69 38 56 ed 0d 43 98 e7 ae e3 bf 0e 2a 2c a8 f7 |

The plaintext consists of the MPDU data, followed by a 4-octet CRC-32 calculated over the MPDU data.

The expanded MPDU, after WEP encapsulation, is as follows:

| IV | fb 02 9e 80 |
| --- | --- |
| MPDU data | f6 9c 58 06 bd 6c e8 46 26 bc be fb 94 74 65 0a ad 1f 79 09 b0 f6 4d 5f 58 a5 03 a2 58 b7 ed 22 eb 0e a6 49 30 d3 a0 56 a5 57 42 fc ce 14 1d 48 5f 8a a8 36 de a1 8d f4 2c 53 80 80 5a d0 c6 1a 5d 6f 58 f4 10 40 b2 4b 7d 1a 69 38 56 ed 0d 43 98 e7 ae e3 bf 0e |
| ICV | 2a 2c a8 f7 |

The IV consists of the first three octets of the RC4 key, followed by an octet containing the KeyID value in the upper two bits.  In this example, the KeyID value is 2.  The MPDU data consists of the ciphertext, excluding the last four octets.  The ICV consists of the last four octets of the ciphertext, which is the encrypted CRC-32 value.

## TKIP encapsulation

The discussion here represents a Michael calculation using a table that shows the key, input data, and MIC output.  For reference, here is a table that describes a the test vector for input string “Michael” shown in Annex F:

| Key | d5 5e 10 05 10 12 89 86 |
| --- | --- |
| Input data | 4d 69 63 68 61 65 6c |
| MIC | 0a 94 2b 12 4e ca a5 46 |

The discussion represents calculation of phase 2 of the temporal key mixing function using a table that shows the TTAK key, the IV input, and the RC4-key output.  For reference, here is a table that describes test vector #4 shown in Annex F:

| TTAK | a2 db 10 2a 3e a3 56 82 99 56 c4 5d 7b 11 fc 54 |
| --- | --- |
| IV | 55 c6 |
| RC4 key | 55 75 c6 a5 04 2b 11 29 25 1e 22 f4 5a 25 c7 d6 |

The MSDU data, prior to TKIP encapsulation, is as follows:

| MSDU data | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| --- | --- |

The MIC is computed using Michael, as follows:

| Key | d5 5e 10 05 10 12 89 86 |
| --- | --- |
| Input data | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| MIC | 31 2d 0f fb 8c d6 58 30 |

The input to the MIC calculation is the MSDU data.

The MSDU and MIC are concatenated, and if necessary, the concatenated result is fragmented into several MPDUs.  In this example, it is fragmented into two MPDUs, as follows:

| MPDU #1 data | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 |
| --- | --- |

| MPDU #2 data | 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 31 2d 0f fb 8c d6 58 30 |
| --- | --- |

To encrypt the first MPDU, the RC4 key is derived using phase 2 of the temporal key mixing function, as follows:

| TTAK | a2 db 10 2a 3e a3 56 82 99 56 c4 5d 7b 11 fc 54 |
| --- | --- |
| IV | 5b a0 |
| RC4 key | 5b 7b a0 d7 9a ee c2 2e 0d d1 a9 14 bd b8 42 30 |

In this example, the IV has value 23456, or 0x5ba0.

RC4 encryption of the first MPDU is performed as follows:

| Key | 5b 7b a0 d7 9a ee c2 2e 0d d1 a9 14 bd b8 42 30 |
| --- | --- |
| Plaintext | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 99 22 5f 4e |
| Ciphertext | e0 3f 0e 76 ce dd d5 54 cb 7d af 74 41 8f 9f db 86 ed 6a 46 f1 1c e0 6a 64 53 3e 95 76 43 3a 93 ac e5 5d 65 ac f0 8e ec 87 88 e7 a8 ad f6 04 ee 4b 64 6e |

The plaintext consists of the MPDU data, followed by a 4-octet CRC-32 calculated over the MPDU data.

The expanded first MPDU, after encapsulation, is as follows:

| IV | 5b 7b a0 40 |
| --- | --- |
| MPDU #1 data | e0 3f 0e 76 ce dd d5 54 cb 7d af 74 41 8f 9f db 86 ed 6a 46 f1 1c e0 6a 64 53 3e 95 76 43 3a 93 ac e5 5d 65 ac f0 8e ec 87 88 e7 a8 ad f6 04 |
| ICV | ee 4b 64 6e |

The IV field consists of the first three octets of the RC4 key, followed by an octet containing the KeyID field in the upper two bits.  In this example, the KeyID has value 1.  The MPDU data consists of the ciphertext, excluding the last four octets.  The ICV consists of the last four octets of the ciphertext, which is the encrypted CRC-32 value.

To encrypt the second MPDU, the RC4 key is derived using phase 2 of the temporal key mixing function, as follows:

| TTAK | a2 db 10 2a 3e a3 56 82 99 56 c4 5d 7b 11 fc 54 |
| --- | --- |
| IV | 5b a1 |
| RC4 key | 5b 7b a1 2c 67 9b cb 70 e7 c3 d6 5e 14 d5 2a c7 |

The IV for the second MPDU is the value of the IV for the first MPDU, plus one.

RC4 encryption of the second MPDU is performed in the same manner as for the first MPDU, as follows:

| Key | 5b 7b a1 2c 67 9b cb 70 e7 c3 d6 5e 14 d5 2a c7 |
| --- | --- |
| Plaintext | 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 31 2d 0f fb 8c d6 58 30 b6 d3 a7 06 |
| Ciphertext | 9f 26 25 79 b8 bf 49 9e 27 bc a6 a9 2c 4d 21 95 4b 3b 84 45 c0 77 33 11 f1 78 ff 14 57 83 15 3c a0 93 31 81 ac 2d bb 1c 81 cc 0e 0b e3 60 06 04 98 9c dc |

The expanded second MPDU, after encapsulation, is similar to that of the first MPDU, as follows:

| IV | 5b 7b a1 40 |
| --- | --- |
| MPDU #2 data | 9f 26 25 79 b8 bf 49 9e 27 bc a6 a9 2c 4d 21 95 4b 3b 84 45 c0 77 33 11 f1 78 ff 14 57 83 15 3c a0 93 31 81 ac 2d bb 1c 81 cc 0e 0b e3 60 06 |
| ICV | 04 98 9c dc |

# AES-CCMP

## AES-CCMP Encapsulation Example

The MPDU parameters and data, prior to AES-CCM encapsulation, is as follows:

Type     = 2   SubType  = 11

ToDS     = 1   FromDS   = 1

MoreFrag = 1   Retry    = 1

PwrMgt   = 1   moreData = 1

WEP      = 1

Order    = 1

Duration = 200

A1 = a1:a1:a1:a1:a1:a1

A2 = a2:a2:a2:a2:a2:a2

A3 = a3:a3:a3:a3:a3:a3

seqNum = 4000  fraqNum = 1

A4 = a4:a4:a4:a4:a4:a4

QC = 0xff77

QoS-TID  = 7  QoS-FEC = 1

QoS-AckP = 3  QoS-TXOP/QL = 0xff

Algorithm = AES_CCM

KeyId = 1

PN    = 0x000000000001  (decimal = 1 )

Data  =

69 6e 6f 76 61 74 69 6f  6e 73 20 69 6e 20 77 69

72 65 6c 65 73 73

The calculation of the encrypted MPDU is as follows:

| CCM additional auth data (muted header) | b8 c7 a1 a1 a1 a1 a1 a1 a2 a2 a2 a2 a2 a2 a3 a3 a3 a3 a3 a3 01 fa a4 a4 a4 a4 a4 a4 07 00 |
| --- | --- |
| CCM Nonce Value | 07 a2 a2 a2 a2 a2 a2 00 00 00 00 00 01 |
| Encryption Header Note PN is big-endian!! | 00 00 00 60 00 00 00 01 |
| CBC Input Blocks | 59 07 a2 a2 a2 a2 a2 a2 00 00 00 00 00 01 00 16 00 1e b8 c7 a1 a1 a1 a1 a1 a1 a2 a2 a2 a2 a2 a2 a3 a3 a3 a3 a3 a3 01 fa a4 a4 a4 a4 a4 a4 07 00 69 6e 6f 76 61 74 69 6f 6e 73 20 69 6e 20 77 69 72 65 6c 65 73 73 00 00 00 00 00 00 00 00 00 00 |
| CBC MIC Value | a6 de 98 74 73 da 55 34 5b f0 26 e6 f0 b8 d9 27 |
| CTR Mode Preload (0) | 01 07 a2 a2 a2 a2 a2 a2 00 00 00 00 00 01 00 00 |
| CCM Final MIC Value | 7d 63 5f d0 d8 3f 8b 6c |
| CTR Mode Data to Encrypt | 69 6e 6f 76 61 74 69 6f 6e 73 20 69 6e 20 77 69 72 65 6c 65 73 73 |
| CTR Mode Encrypted Data | 6b 64 99 64 53 85 64 f1 28 69 08 ab fb 12 41 ed 10 04 d4 44 da 3f |
| CCM Encrypted MPDU with FCS | b8 ff c8 00 a1 a1 a1 a1 a1 a1 a2 a2 a2 a2 a2 a2 a3 a3 a3 a3 a3 a3 01 fa a4 a4 a4 a4 a4 a4 77 ff 00 00 00 60 00 00 00 01 6b 64 99 64 53 85 64 f1 28 69 08 ab fb 12 41 ed 10 04 d4 44 da 3f 7d 63 5f d0 d8 3f 8b 6c d5 67 81 13 |

## Additional CCMP Vest Vectors

The following CCMP test vectors are full 802.11 CCMP encrypted MPDUs.  The MPDUs and CCMP processing can be tested by using the supplied key to decrypt the MPDU and checking the MIC value.

| Description | ccm#0001 : Data Packet, no A4 and no QC |
| --- | --- |
| Key | c0 c1 c2 c3 c4 c5 c6 c7 c8 c9 ca cb cc cd ce cf |
| CCMP Encrypted MPDU with FCS | 08 41 02 01 00 06 25 a7 c4 36 00 02 2d 49 97 b4 00 06 25 a7 c4 36 e0 00 06 05 00 a0 04 03 02 01 1e e5 2d 13 b1 be 3f 20 42 5b 3f de dd d4 55 2b 98 71 d8 7b 65 8c fd 57 f7 96 ad 71 87 |

| Description | ccm#0002 : Data Packet, no A4 and no QC, retry |
| --- | --- |
| Key | c0 c1 c2 c3 c4 c5 c6 c7 c8 c9 ca cb cc cd ce cf |
| CCMP Encrypted MPDU with FCS | 08 49 02 01 00 06 25 a7 c4 36 00 02 2d 49 97 b4 00 06 25 a7 c4 36 e0 00 06 05 00 e0 04 03 02 01 1e e5 2d 13 b1 be 3f 20 42 5b 3f de dd d4 55 2b 98 71 d8 7b 65 8c fd 57 f7 67 c5 18 73 |

| Description | ccm#0003 : Data Packet,A4 with no QC |
| --- | --- |
| Key | c0 c1 c2 c3 c4 c5 c6 c7 c8 c9 ca cb cc cd ce cf |
| CCMP Encrypted MPDU with FCS | 08 43 02 01 00 06 25 a7 c4 36 00 02 2d 49 97 b4 00 06 25 a7 c4 36 e0 00 41 42 43 44 45 46 00 00 00 20 00 00 00 01 3b e9 b2 46 c6 fc 7a 51 55 1e 14 c6 a8 85 28 bc 06 56 67 c8 ef 30 b3 12 69 14 6c 3b c3 |

| Description | ccm#0004 : Data Packet,A4 and QC |
| --- | --- |
| Key | c0 c1 c2 c3 c4 c5 c6 c7 c8 c9 ca cb cc cd ce cf |
| CCMP Encrypted MPDU with FCS | 88 43 02 01 00 06 25 a7 c4 36 00 02 2d 49 97 b4 00 06 25 a7 c4 36 e0 00 41 42 43 44 45 46 04 00 00 00 00 20 00 00 00 01 46 72 f2 9e 41 54 e9 11 58 47 c2 a9 ae dc 10 0c e8 82 53 bd a2 04 ae 1d 33 05 af 02 1e |

| Description | ccm#0005 : Data Packet,QC no A4 |
| --- | --- |
| Key | c0 c1 c2 c3 c4 c5 c6 c7 c8 c9 ca cb cc cd ce cf |
| CCMP Encrypted MPDU with FCS | 88 41 02 01 00 06 25 a7 c4 36 00 02 2d 49 97 b4 00 06 25 a7 c4 36 e0 00 04 00 00 00 00 20 00 00 00 01 46 72 f2 9e 41 54 e9 11 58 47 c2 a9 ae dc 10 0c e8 dc 91 98 bf 6a 52 c8 03 67 12 0b 83 |

| Description | ccm#0006 : Data Packet, no A4, No QC, look out for the C9 |
| --- | --- |
| Key | 00 01 02 03 04 05 06 07 08 c9 0a 0b 0c 0d 0e 0f |
| CCMP Encrypted MPDU with FCS | 08 41 02 01 00 06 25 a7 c4 36 00 02 2d 49 97 b4 00 06 25 a7 c4 36 e0 00 06 05 00 a0 04 03 02 01 de bf 2c c9 94 e6 5a 70 2c ee e3 19 84 21 39 c3 f2 9a 2e 12 63 11 74 5f 3c 20 3d fd 4e |

| Description | ccm#0007 : Data Packet, same as 144r4 data, odd a4 alignment |
| --- | --- |
| Key | 00 01 02 03 04 05 06 07 08 09 0a 0b 0c 0d 0e 0f |
| CCMP Encrypted MPDU with FCS | 08 43 12 34 ff ff ff ff ff ff 00 40 96 45 07 f1 08 00 46 17 62 3e 50 67 aa aa 03 00 00 00 00 05 00 a0 04 03 02 01 22 3b 8c 39 95 b0 d0 c5 81 d7 19 2f e4 4a ad 02 76 61 30 fe 1a 2c 1d 54 0b e2 ce 2f 4d 53 03 1b 62 68 8f 9d 75 81 08 ff 6d 35 e5 a0 75 f4 c2 0a 95 d2 f2 c7 45 94 b6 9e 64 63 3a fa 6e 5c 97 57 ea 49 24 66 f4 e5 3e e9 81 77 d2 0b f9 d9 82 15 ac ce 8f e8 7b 7e f1 ef ae cc 9b ac |

| Description | ccm#0008 : All flag bits set with QC |
| --- | --- |
| Key | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 |
| CCMP Encrypted MPDU with FCS | b8 ff c8 00 a1 a1 a1 a1 a1 a1 a2 a2 a2 a2 a2 a2 a3 a3 a3 a3 a3 a3 01 fa a4 a4 a4 a4 a4 a4 77 ff 00 00 00 60 00 00 00 01 6b 64 99 64 53 85 64 f1 28 69 08 ab fb 12 41 ed 10 04 d4 44 da 3f 7d 63 5f d0 d8 3f 8b 6c d5 67 81 13 |

## AES-OCB encapsulation

The discussion here represents an AES-OCB encryption using a table that shows the key, nonce input, plaintext input, ciphertext output, and tag output.  For reference, here is a table that describes test case “OCB-AES-128-34B” available at http://www.cs.ucdavis.edu/~rogaway/ocb/.

| Key | 00 01 02 03 04 05 06 07 08 09 0a 0b 0c 0d 0e 0f |
| --- | --- |
| Nonce | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 |
| Plaintext | 00 01 02 03 04 05 06 07 08 09 0a 0b 0c 0d 0e 0f 10 11 12 13 14 15 16 17 18 19 1a 1b 1c 1d 1e 1f 20 21 |
| Ciphertext | 01 a0 75 f0 d8 15 b1 a4 e9 c8 81 a1 bc ff c3 eb d4 90 3d d0 02 5b a4 aa 83 7c 74 f1 21 b0 26 0f a9 5d |
| Tag | cf 83 41 bb 10 82 0c cf 14 bd ec 56 b8 d7 d6 ab |

The MSDU data, prior to AES-OCB encapsulation, is as follows:

| MSDU data | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| --- | --- |

In this example, the following parameters will be used:

- Replay counter value is 123456789 = 0x75bcd15.

- QOS traffic class is 4.

- KeyID is 2.

- Source MAC address is 0x123456789abc

- Destination MAC address is 0x23456789abcd

AES-OCB encryption is performed as follows:

| Key | 00 01 02 03 04 05 06 07 08 09 0a 0b 0c 0d 0e 0f |
| --- | --- |
| Nonce | 75 bc d1 54 12 34 56 78 9a bc 23 45 67 89 ab cd |
| Plaintext | aa aa 03 00 00 00 08 00 45 00 00 4e 66 1a 00 00 80 11 be 64 0a 00 01 22 0a ff ff ff 00 89 00 89 00 3a 00 00 80 a6 01 10 00 01 00 00 00 00 00 00 20 45 43 45 4a 45 48 45 43 46 43 45 50 46 45 45 49 45 46 46 43 43 41 43 41 43 41 43 41 43 41 41 41 00 00 20 00 01 |
| Ciphertext | 3f 42 a7 b6 8c 4d ea 07 e1 2b 2d c6 82 e4 72 81 70 45 a3 02 21 ee b6 bf 0a f0 26 a6 8d 73 e6 f3 55 dc 15 60 8e 7e 92 52 0b 4e 11 73 a3 a5 ce 7c 39 93 65 70 a1 16 40 79 02 a1 85 17 f6 5e 9d eb 35 bf 2c 18 49 6c 2c 2b 33 8b d4 0f d5 4e 1c 50 eb fd fa 56 95 56 |
| Tag | 06 9e e2 41 c6 1e 60 2d b3 05 76 53 03 a5 5f 5e |

The first three octets of the nonce are the upper 24 bits of the replay counter value.  The upper nibble of the fourth octet of the nonce consists of the least significant 4 bits of the replay counter value.  The lower nibble of the fourth octet of the nonce is the QOS traffic class.  Octets five through ten of the nonce are the source MAC address.  Octets eleven through sixteen of the nonce are the destination MAC address.  The plaintext consists of the MSDU data.

The expanded MSDU, after AES-OCB encapsulation, is as follows:

| Replay | 75 bc d1 85 |
| --- | --- |
| MSDU data | 3f 42 a7 b6 8c 4d ea 07 e1 2b 2d c6 82 e4 72 81 70 45 a3 02 21 ee b6 bf 0a f0 26 a6 8d 73 e6 f3 55 dc 15 60 8e 7e 92 52 0b 4e 11 73 a3 a5 ce 7c 39 93 65 70 a1 16 40 79 02 a1 85 17 f6 5e 9d eb 35 bf 2c 18 49 6c 2c 2b 33 8b d4 0f d5 4e 1c 50 eb fd fa 56 95 56 |
| MIC | 06 9e e2 41 c6 1e 60 2d |

The first three octets of the replay field are the upper 24 bits of the replay counter value.  The fourth octet of the replay field is the concatenation of:  the 2-bit keyID value; two 0-bits; and the least significant 4 bits of the replay counter value.  The MSDU data consists of the ciphertext. The MIC is the first eight octets of the tag value.

## The PRF Function - PRF(key, prefix, data, length).

A set of test vectors are provided for each size of PRF function used in this specification.  The input to the PRF function are strings for ‘key’, ‘prefix’ and ‘data’.  The length can be any multiple of 8, but the values: 192, 256, 384, 512 and 768 are used in this specification. The test vectors were taken from RFC2202 with additional vectors added to test larger key and data sizes.

| Test_case | 1 |
| --- | --- |
| key | 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b |
| prefix | "prefix" |
| data | "Hi There" |
| length | 192 |
| PRF-192 | bc d4 c6 50 b3 0b 96 84 95 18 29 e0 d7 5f 9d 54 b8 62 17 5e d9 f0 06 06 |

| Test_case | 2 |
| --- | --- |
| key | 'Jefe' |
| prefix | "prefix-2" |
| data | "what do ya want for nothing?" |
| length | 256 |
| PRF-256 | 47 c4 90 8e 30 c9 47 52 1a d2 0b e9 05 34 50 ec be a2 3d 3a a6 04 b7 73 26 d8 b3 82 5f f7 47 5c |

| Test_case | 3 |
| --- | --- |
| key | aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa |
| prefix | "prefix-3" |
| data | "Test Using Larger Than Block-Size Key - Hash Key First" |
| length | 384 |
| PRF-384 | 0a b6 c3 3c cf 70 d0 d7 36 f4 b0 4c 8a 73 73 25 55 11 ab c5 07 37 13 16 3b d0 b8 c9 ee b7 e1 95 6f a0 66 82 0a 73 dd ee 3f 6d 3b d4 07 e0 68 2a |

| Test_case | 4 |
| --- | --- |
| key | 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b |
| prefix | "prefix-4" |
| data | "Hi There Again" |
| length | 512 |
| PRF-512 | 24 8c fb c5 32 ab 38 ff a4 83 c8 a2 e4 0b f1 70 eb 54 2a 2e 09 16 d7 bf 6d 97 da 2c 4c 5c a8 77 73 6c 53 a6 5b 03 fa 4b 37 45 ce 76 13 f6 ad 68 e0 e4 a7 98 b7 cf 69 1c 96 17 6f d6 34 a5 9a 49 |

| Test_case | 5 |
| --- | --- |
| key | aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa aa |
| prefix | "prefix-5" |
| data | "Test Using Larger Than Block-Size Key and Larger Than One Block-Size Data" |
| length | 768 |
| PRF-768 | 67 27 a3 e8 d5 2c f2 70 08 ce 4d 68 3e 45 99 25 c6 23 5b e0 0c 8c 13 03 77 26 af fc bc 02 29 17 a5 94 1c 0c 77 4b 00 25 7f 77 c6 e2 4c 81 02 87 8e 04 b7 2c f6 c7 88 a7 ba ec 4f 69 68 7b eb d6 30 15 59 ca 1f c2 6f 93 04 2e 1e 82 ba 28 9a 05 2c a8 51 ef cd 4e 15 a1 5d d0 4c bb e1 f6 94 58 |

## Key Hierarchy Test Vectors

The following test vectors provide an example of key derivation for:

- TKIP Pairwise Keys

- TKIP Group Keys

- WRAP Pairwise Keys

- WRAP Group Keys

| PMK | 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b 0b |
| --- | --- |
| GMK | 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c 0c |
| AA | 01 01 01 01 01 01 |
| SA | 02 02 02 02 02 02 |
| SNonce | 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 |
| ANonce | 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 |
| GNonce | 30 30 30 30 30 30 30 30 30 30 30 30 30 30 30 30 30 30 30 30 |
| TKIP Pairwise EAP_MIC_key | 86 f5 6f fd 2d b9 9b b8 e8 72 26 09 7b 16 0a 42 |
| TKIP Pairwise EAP_encr_key | eb ff 51 15 ae aa 72 f1 d7 c0 64 e3 c9 4d 7c f5 |
| TKIP PTEK | ef 14 31 fb 94 8e 99 97 de 0f 4f 3d 46 37 f0 e9 |
| TKIP P_TX_MIC_key | 6d dd bc db bd 34 ab 3f |
| TKIP P_RX_MIC_key | a7 de b3 7c 16 85 e0 35 |
| TKIP GTEK | 3c 2b 0f 83 76 46 89 aa 89 33 b4 e8 4e ca a7 6a |
| TKIP G_TX_MIC_key | d3 74 14 d4 4b 7d 3b dc |
| TKIP G_RX_MIC_key | cb b1 7b c2 30 bf 91 49 |
| WRAP PTEK | ef 14 31 fb 94 8e 99 97 de 0f 4f 3d 46 37 f0 e9 |
| WRAP GTEK | 3c 2b 0f 83 76 46 89 aa 89 33 b4 e8 4e ca a7 6a |

## PRF Reference Code

"""  prf.py

Paul A. Lambert

Paul@AirgoNetworks.Com

The PRF function is used in a number of places in 802.11i. It is used

to create psuedo random bit strings of length:

128bits, 192bits, 254bits, 384bits, 512bits and 768bits.

The parameter 'prefix' is a string that shall be a unique label for each

different purpose of the PRF.

"""

import hmac

import sha

from binascii_plus import b2a_p

def PRF(key,prefix,data,length_in_bits):

""" Key, prefix and data are arbitrary strings, the parameter

length_in_bits and must be multiple of 8.

"""

R = ''

for i in range((length_in_bits+159)/160):

hmac_sha_1 = hmac.new(key, prefix + chr(0x00) + data + chr(i),sha)

R = R + hmac_sha_1.digest()       # concatenate latest hast to result string

return L(R,0,length_in_bits)          # return R truncated to 'length'

def L(I,F,L):

""" Take from I starting from the left, bit F for L bits

moving to the right using 7.1.1 bit convention from IEEE 802.11.

"""

#  This implementation only works on byte boundaries.

assert (F % 8 == 0) and (L % 8 == 0), 'can only truncate on byte boundaries'

return I[(F/8):((F/8)+(L/8))]

def PRF_128(key,A,B): return PRF(key,A,B,128)

def PRF_192(key,A,B): return PRF(key,A,B,192)

def PRF_256(key,A,B): return PRF(key,A,B,256)

def PRF_384(key,A,B): return PRF(key,A,B,384)

def PRF_512(key,A,B): return PRF(key,A,B,512)

def PRF_768(key,A,B): return PRF(key,A,B,768)

## Key Hierarchy Reference Code

""" key_hierarchy_80211tgi.py

Classes supporting reference implementation of

IEEE 802.11 TGi Key Hierarchies

"""

from   crypto.keyedHash.prf import PRF_128,PRF_256,PRF_512,PRF_384,L

from   binascii_plus        import b2a_hex, a2b_hex, b2a_p

class TKIP_pairwise_keys:

def __init__(self,PMK,AA,SA,SNonce,ANonce):

""" TKIP pairwise keys are derived from:

PMK, AA, SA, ANonce and SNonce

"""

assert (len(PMK) == 32), 'PMK - PreMaster Key must be 256bits'

PTK = PRF_512(PMK,'Pairwise key expansion',

min(AA,SA)+max(AA,SA) + min(SNonce,ANonce)+

max(SNonce,ANonce))

self.EAP_MIC_key = L(PTK,0,128)   #  First 16 bytes

self.EAP_encr_key = L(PTK,128,128) #  Second 16 bytes

self.PTEK = L(PTK,256,128) #  Third 16 bytes

self.P_TX_MIC_key = L(PTK,384,64)  #  Next 8 bytes

self.P_RX_MIC_key = L(PTK,448,64)  #  Final 8 bytes

def __repr__(self):

return '---- TKIP Pairwise Keys ----\n'+\

'  EAP_MIC_key ='  + b2a_p(self.EAP_MIC_key)+'\n'+\

'  EAP_encr_key =' + b2a_p(self.EAP_encr_key)+'\n'+\

'  PTEK ='         + b2a_p(self.PTEK)+'\n'+\

'  P_TX_MIC_key =' + b2a_p(self.P_TX_MIC_key)+'\n'+\

'  P_RX_MIC_key =' + b2a_p(self.P_RX_MIC_key)

class TKIP_group_keys:

def __init__(self,GMK,AA,GNonce):

""" TKIP group keys are derived from: GMK, AA and GNonce """

assert (len(GMK) == 32), 'GMK - Group Master Key must be 256bits'  #not in spec!

GTK = PRF_256(GMK,'Group key expansion',AA+GNonce)

self.GTEK = L(GTK,0,128)   #  First 16 bytes

self.G_TX_MIC_key = L(GTK,128,64)  #  Next 8 bytes

self.G_RX_MIC_key = L(GTK,192,64)  #  Final 8 bytes

def __repr__(self):

return '---- TKIP Group Keys ----\n'+\

'  GTEK ='         + b2a_p(self.GTEK)+'\n'+\

'  G_TX_MIC_key =' + b2a_p(self.G_TX_MIC_key)+'\n'+\

'  G_RX_MIC_key =' + b2a_p(self.G_RX_MIC_key)

class WRAP_pairwise_keys:

def __init__(self,PMK,AA,SA,SNonce,ANonce):

""" WRAP pairwise keys are derived from: PMK, AA, SA, ANonce and SNonce """

assert (len(PMK) == 32), 'PMK - PreMaster Key must be 256bits'

PTK = PRF_384(PMK,'Pairwise key expansion',min(AA,SA)+max(AA,SA)+min(SNonce,ANonce)+max(SNonce,ANonce))

self.EAP_MIC_key = L(PTK,0,128)   #  First 16 bytes

self.EAP_encr_key = L(PTK,128,128) #  Second 16 bytes

self.PTEK = L(PTK,256,128) #  Third 16 bytes

def __repr__(self):

return '---- WRAP Pairwise Keys ----\n'+\

'  PTEK =' + b2a_p(self.PTEK)

class WRAP_group_keys:

def __init__(self,GMK,AA,GNonce):

""" WRAP group keys are derived from: GMK, AA and GNonce """

assert (len(GMK) == 32), 'GMK - Group Master Key must be 256bits'

GTK = PRF_128(GMK,'Group key expansion',AA+GNonce)

self.GTEK = GTK

def __repr__(self):

return '---- WRAP Group Key ----\n'+\

'  GTEK =' + b2a_p(self.GTEK)

""" key_hierarchies_test_vectors.py

Generate example test vectors for the IEEE 802.11 TGi Key Hierarchies

"""

from crypto.keyedHash.key_hierarchy_80211tgi import TKIP_pairwise_keys, TKIP_group_keys, WRAP_pairwise_keys, WRAP_group_keys

from   binascii_plus        import b2a_hex, a2b_hex, b2a_p

if __name__ == "__main__": # Make this module runable from the command prompt

print 'IEEE 802.11TGi Key Hierarchy Reference Implementation Test Vectors'

PMK    = 32*chr(0x0b)  # 32 bytes of 0x0b

GMK    = 32*chr(0x0c)  # 32 bytes of 0x0c

AA     =  6*chr(0x01)  #  6 bytes of 0x01

SA     =  6*chr(0x02)  #  6 bytes of 0x02

SNonce = 20*chr(0x10)  # 20 bytes of 0x10

ANonce = 20*chr(0x20)  # 20 bytes of 0x20

GNonce = 20*chr(0x30)  # 20 bytes of 0x30

tkip_PKeys = TKIP_pairwise_keys(PMK,AA,SA,SNonce,ANonce)

tkip_Gkeys = TKIP_group_keys(GMK,AA,GNonce)

wrap_PKeys = WRAP_pairwise_keys(PMK,AA,SA,SNonce,ANonce)

wrap_GKeys = WRAP_group_keys(GMK,AA,GNonce)

print '  PMK    =' + b2a_p(PMK)

print '  GMK    =' + b2a_p(GMK)

print '  AA     =' + b2a_p(AA)

print '  SA     =' + b2a_p(SA)

print '  SNonce =' + b2a_p(SNonce)

print '  ANonce =' + b2a_p(ANonce)

print '  GNonce =' + b2a_p(GNonce)

print    tkip_PKeys

print    tkip_Gkeys

print    wrap_PKeys

print    wrap_GKeys

# References

- RFC-2202, Test Cases for HMAC-MD5 and HMAC-SHA-1.