IEEE P802.11
Wireless LANs

Direct Link Protocol Specification

Date:	September, 2002

Authors:	The Side Link Ad Hoc Group:

Srinivas Kandala (Sharp Labs)

Keith Amman (Spectralink)

Wim Diepstraten (Agere)

Menzo Wentink (Intersil)

Abstract

This document contains normative text for the Direct Link Protocol (DLP).

DLP lets stations in an infrastructure BSS start a direct link to a receiver in the same BSS. When the direct link is active, the sender may send frames directly to the receiver, instead of via the AP. This text replaces the Wireless Address Resolution Protocol in P802_11E-D3_0.

This text is based on:

02/421 Direct Stream Request Protocol (Menzo Wentink, Intersil)

02/465 WiSP - Wireless Side Channel Protocol (Wim Diepstraten, Gerrit Hiddink, Agere Systems)

02/438 Direct Link Protocol (S. Kandala, Sharp Labs)

Editorial notes appear in bold italic Times New Roman font.

_____________________________________________________________________________________________

Add the following definition in clause 4:

DLP		direct link protocol

Add the following text to clause 5:

## Direct Link Protocol

### Overview

To send frames from one WSTA to another directly in an infrastructure QBSS requires the setup of a Direct Link. The Direct Link Protocol (DLP) allows stations to do so, in an easy manner. The need for this protocol is motivated by the fact that the intended recipient may be in Power Save Mode, in which case it can only be woken up via the AP. The second feature of DLP is to exchange rate set and other information between the sender and the receiver. Finally, DLP messages can be used to attach security information elements. The security elements and the associated security handshake are not part of this section.

This protocol assumes that stations do not go into powersave for the active duration of the Direct Stream.

DLP does not apply in an IBSS, where frames are always sent directly from one STA to another.

The DLP handshake is illustrated in Figure 10.1.

AP

QSTA

-

1

QSTA

-

2

1a

2b

2a

1b

3

Figure 1. The four steps that are involved in the Direct Link handshake.

First, the sending station QSTA-1 sends a DLP-request frame to the AP (1a). This request contains the rate set, and (extended) capabilities of QSTA-1, as well as the MAC addresses of QSTA-1 and QSTA-2.

If QSTA-2 is associated in the BSS, direct streams are allowed in the policy of the BSS and QSTA-2 is indeed a QSTA (i.e. not a regular STA), the AP shall forward the DLP-request to the recipient, QSTA-2 (1b).

If QSTA-2 accepts the direct stream, it shall send a DLP-response frame to the AP (2a), which contains the rate set, (extended) capabilities of QSTA-2 and the MAC addresses of QSTA-1 and QSTA-2. The AP shall forward the DLP-response to QSTA-1 (2b), after which the direct link becomes active. From this time on, QSTA-1 may send frames directly to QSTA-2.

QSTA-2 shall not go into power save for a duration of aDLPIdleTimeout, after it sent the DLP-action response frame (2a).

When the direct link is active, QSTA-1 may use DLP-probes (3) to gauge the quality of the link between QSTA-1 and QSTA-2.

The direct link becomes inactive when no frames have been exchanged as part of the direct link for a duration of aDLPIdleTimout. After the timeout, frames with destination QSTA-2 shall be sent via the AP.

By omitting the first step (1a), this protocol can also be invoked by the AP to set up a Direct Link between two associated QSTAs.

Add the following MIB element to annex D

aDLPIdleTimeout	integer (0-65535)		read-write	default: 500 TU

Replace the “meaning” of category code 2 in table 15.1, clause 7.2.3.12 as follows:

Replace “WARP” with “Direct Link Protocol (DLP)”.

Add a clause 7.3.1.11

### 7.3.1.11 Random Data Field

The Random Data Field is a field with an arbitrary bit pattern and with an arbitrary length, but upper bounded such that the containing MPDU size does not exceed aFragmentationThreshold.

The Random Data Field shall be discarded by the receiving MAC entity.

Add DLP-request and DLP-response action codes to tabe 20.6 in section 7.4, as shown below:

Table 20.6 – QoS Action codes

| Code | Meaning |
| --- | --- |
| 0 | ADDTS request |
| 1 | ADDTS response |
| 2 | DELTS |
| 3 | Reserved |
| 4 | Define Burst Ack request |
| 5 | Define Burst Ack response |
| 6 | Delete Burst Ack request |
| 7 | Reserved |
| 8 | QAPC-STA assertion request |
| 9 | QAPC-STA assertion response |
| 10 | DLP-request |
| 11 | DLP-response |
| 12 | DLP-probe |
| 13 – 255 | Reserved |

Renumber 7.5 as 7.4.4.

Delete subclauses 7.5.1, 7.5.2 and 7.5.3 (now renumbered as 7.4.4.1, 7.4.4.2 and 7.4.4.3) and renumber the following subclauses starting with 7.4.4.1)

Add the following clauses:

### 7.4.4.1 DLP-request

The Action body of the DLP-request QoS Action frame body is defined in Figure 42.18.5.

| Usage | Order | Information | Note |
| --- | --- | --- | --- |
| Always present | 1 | Destination MAC Address |  |
|  | 2 | Source MAC Address |  |
|  | 3 | Capability Information |  |
|  | 4 | Supported rates |  |
| QBSS | 5 | Extended Capabilities | The Extended Capabilities information element is only present in DLP Request frames generated by QSTAs with Capability Information bit 15=1. |

Figure 42.18.5 – DLP-request action body

The Destination MAC address shall be the target destination address, QSTA-2.

The Source MAC address shall be the MAC address of the originator, QSTA-1.

The Capability information shall be the capability information of the originator of the request, QSTA-1.

The supported rates information element shall contain the supported rates information of the originator, QSTA-1.

The Extended Capabilities shall be the extended capabilities information element corresponding to those extended capabilities supported by the originator of the request, QSTA-1.

### 7.4.4.2	DLP-response

The action-specific status codes of a DLP-response QoS action frame are defined in Table 20.8.1. The action body of a DLP-response frame is defined in Figure 42.18.5.

Table 20.8.1 DLP-response QoS action frame status field

| Status Code | Result Code | Definition |
| --- | --- | --- |
| 0 | Action completed successfully | The WSTA is willing to participate. |
| 1 | Unrecognized Action Code | This should not occur. |
| 2 | Not Allowed | Direct Link is not enabled in the BSS policy |
| 3 | Not Present | The Destination WSTA is not present within this QBSS. |
| 4 | Not a QSTA | The Destination WSTA is not a QSTA. |
| 5 | Refused | The WSTA is not willing to participate. |
| 6-255 | Reserved |  |

| Usage | Order | Information | Note |
| --- | --- | --- | --- |
| Always present | 1 | Destination MAC Address |  |
|  | 2 | Source MAC Address |  |
| DLP (Present if required) | 3 | Capability Information |  |
|  | 4 | Supported rates |  |
| QBSS (Present if required) | 5 | Extended Capabilities | The Extended Capabilities information element is only present in DLP Request frames generated by QSTAs with Capability Information bit 15=1. |

Figure 42.18.5 DLP-response action body

The Destination MAC address in the DLP-response action body when the frame is sent by the sender shall be the target destination address, QSTA-2.

The Source MAC address shall be the MAC address of the originator, QSTA-1.

The Capability information shall be the capability information of the target destination, QSTA-2.  This information shall only be included if the action response status code corresponds to Successful (status code 0).

The supported rates information element shall contain the supported rates information of the target destination, QSTA-2.  This information shall only be included if the action response status code corresponds to Successful (status code 0).

The Extended Capabilities shall be the extended capabilities information element corresponding to those extended capabilities supported by the originator of the response, QSTA-2.  This information shall only be included in the response if the action response status code corresponds to Successful (status code 0).

### 7.4.4.2	DLP-probe

The action body of the DLP-probe is defined in Figure 42.18.6.

| Usage | Order | Information | Note |
| --- | --- | --- | --- |
| Always present | 1 | Destination MAC Address |  |
|  | 2 | Source MAC Address |  |
| DLP | 3 | Random Data |  |

Figure 42.18.6 DLP-probe action body

Add clause 10.3.13

### 10.3.13	Management of Direct Links

This clause describes the management procedures associated with Direct Links. The following DLP management primitives are defined:

- MLME-DLP.request

- MLME-DLP.indication

- MLME-DLP.confirm

This section is a functional duplicate of the DLP descriptions earlier in this document. In case of dicrepancies, the earlier sections shall lead over this section. Also, this section does not add to the understanding of the protocol.

The extended DLP message flow is illustrated in the following figure:

SME

MAC

MAC

MAC

SME

SME

MLME

-

DLP.request*

MLME

-

DLP.indication*

MLME

-

DLP.request

DLP-request*

DLP-request

MLME

-

DLP.indication

MLME

-

DLP.confirm

DLP-response

MLME

-

DLP.confirm

QSTA-1

AP

QSTA-2

DLP-response

MLME

-

DLP.confirm

MLME

-

DLP.confirm

X

DLP-probes

(optional)

X

Figure 2. Direct Link Protocol message flow. (Messages indicated with an asterix are omitted when the AP is the initiator of the Direct Link.)

### 10.3.13.1	MLME-DLP.request

### 10.3.13.1.1 Function

This primitive requests that the MAC entity set up a direct link between two QSTAs.

### 10.3.13.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-DLP.request 	(

Sender MAC Address

Receiver MAC Address

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Sender MAC Address | MACAddress | Any valid individual MAC address | Specifies the MAC address of the WSTA that is the sender of the data flow. The initiating WSTA shall use the own MAC address. The AP shall use the address which it received in the MLME-DLP.indication, or the address of the intended sender. |
| Receiver MAC Address | MACAddress | Any valid individual MAC address | Specifies the MAC address of the WSTA that is the intended immediate recipient of the data flow. |

### 10.3.13.1.3 When generated

This primitive is generated by the SME at a QSTA that wishes to set up a Direct Link to another WSTA.

### 10.3.13.1.4 Effect of receipt

This request intiates the DLP handshake.

### 10.3.13.2		MLME-DLP.indication

### 10.3.13.2.1 Function

This primitve indicates the reception of a DLP-request frame.

### 10.3.13.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-DLP.request 	(

Sender MAC Address

Receiver MAC Address

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Sender MAC Address | MACAddress | Any valid individual MAC address | Specifies the MAC address of the WSTA that is the sender of the data flow. |
| Receiver MAC Address | MACAddress | Any valid individual MAC address | Specifies the MAC address of the WSTA that is the intended immediate recipient of the data flow. |

### 10.3.13.2.3 When generated

This primitive is generated when the MAC received a DLP-request.

### 10.3.13.2.4 Effect of receipt

This primitive informs the receiver about the pending Direct Link handshake.

### 10.3.13.3		MLME-DLP.confirm

### 10.3.13.3.1 Function

This primitive informs the node about the decision that was taken regarding the pending Direct Link request.

### 10.3.13.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-DLP.confirm 	(

Sender MAC Address

Receiver MAC Address

Result Code

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Sender MAC Address | MACAddress | Any valid individual MAC address | Specifies the MAC address of the WSTA that is the sender of the data flow. |
| Receiver MAC Address | MACAddress | Any valid individual MAC address | Specifies the MAC address of the WSTA that is the intended immediate recipient of the data flow. |
| Result Code | Enumeration | SUCCESS, INVALID_, NOT_ALLOWED, NOT_PRESENT, NOT_QSTA, REFUSED | Specifies the MAC address of the WSTA that is the intended immediate recipient of the data flow. |

### 10.3.13.3.3 When generated

This primtive is generated by the MLME after a decision was taken regarding the pending Direct Link request.

### 10.3.13.3.4 Effect of receipt

If the AP receives a positive confirmation, it will send a confirmation to the sender WSTA, QSTA-1. At the sender, the confirmation indicates the point at which the Direct Link becomes active.