IEEE P802.11
Wireless LANs

Security MIB Extensions

Date:	September 10, 2002

Author:	                        Arnoud Zwemmer 
	2000 North 3720 AH Bilthoven
	Bilthoven, The Netherlands
	Phone: 31-30-229-60-84
	Fax: 31-30-229-60-61
e-mail: Arnoud.Zwemmer@intersil.com

Frank Ciotti
	24900 Pitkin Road Suite 307
	The Woodlands, Texas 77386
	Phone: 281-298-9922
	Fax: 281-419-9205
	e-mail: ciottif@lincom.com

Dorothy Stanley
	2000 North Naperville Rd, Room 2F-415
	Naperville IL 60566
	Phone: 630-979-1572
	Fax: 630-979-1572
	e-mail: dstanley@agere.com

Abstract

This document provides text for MIB sections that are currently not provided in TGi D2, together with proposed modifications to the existing 802.11 MIB.  Motions to add/modify the text are also included.

Issue 1:

Unlike legacy WEP, all RSN keying is now done internally in the MAC. The MLME-SetKeys primitive can be used to configure all keys internally. There is no need anymore to expose these keys in the MIB. The only key that needs to be exposed in the MIB for RSN is the pre-shared key, if RSN PSK is used as the authentication suite.

Motion: Remove section 8.3.2.3 referring to TKIP state that augments the dot11WEPDefaultKeysTable and the dot11WEPKeymappingsTable.

Issue 2:

Need variable to indicate whether RSN is implemented and whether it’s activated.

Motion: Add the following attribute to the dot11StationConfigTable:

dot11RSNOptionImplemented OBJECT-TYPE

SYNTAX		TruthValue

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"This variable indicates whether the entity is RSN-capable."

::= { dot11StationConfigEntry 24 }

Motion: Add the following attribute to the dot11PrivacyTable

dot11RSNEnabled OBJECT-TYPE

SYNTAX		TruthValue

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"When this object is set to TRUE, this shall indicate that

RSN is enabled on this entity. The entity will	 advertise the

RSN Information Element in its Beacons and

Probe Responses. Configuration variables for RSN operation

are found in the dot11RSNConfigTable.

This object requires that dot11PrivacyInvoked also be set

to TRUE.“

Issue 3:

802.11-1999 has a variable dot11PrivacyInvoked, which currently means that WEP is invoked. In current 802.11 implementations, the privacy bit in Beacon/Probe response and the WEP bit in the Capability Information follow directly from the value of this variable. This means we cannot change it and we must assume dot11RSNEnabled also requires dot11PrivacyInvoked to be set to TRUE.

This introduces a problem, because if dot11RSNEnabled is set to TRUE, we only want RSN (no WEP) and we want to explicitly use another variable to indicate that legacy WEP is also allowed (dot11RSNConfigTSNEnabled is used for this purpose). The description of dot11PrivacyInvoked still suggests WEP is used though.

Solution:

Propose to change the description of dot11PrivacyInvoked to not mean that WEP is invoked, but that ‘some level of security’ is invoked. In combination with dot11RSNEnabled this would mean RSN security is invoked. It is not allowed to change the semantics of a variable in a MIB definition, but for legacy equipment the semantics do not change, as the only security mechanism they are aware of is WEP.

Motion: Change the DESCRIPTION clause of object dot11PrivacyInvoked from:

"When this attribute is true, it shall indicate that the IEEE

802.11 WEP mechanism is used for transmitting frames of type

Data. The default value of this attribute shall be false."

To:

“When this attribute is TRUE, it shall indicate that some level of

security is invoked for transmitting frames of type Data. For

802.11-1999 clients, the security mechanism used is WEP.

For RSN-capable clients, an additional variable dot11RSNEnabled indicates whether RSN is enabled. If dot11RSNEnabled is FALSE,

the security mechanism invoked  is WEP; if dot11RSNEnabled is TRUE, RSN security mechanisms invoked are configured in the dot11RSNConfigTable. The default value of this attribute shall be FALSE.”

Issue 4: We need a MIB to configure RSN, when RSN is enabled

# Motion: Remove all MIB additions currently defined in Annex F of Tgi draft 2.3.3 and incorporate the following text as the IEEE 802.11i MIB (in the correct Annex: D)

--

--  IEEE 802.11i MIB

--

IEEE802dot11i-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, Integer32, Unsigned32, Counter32

FROM SNMPv2-SMI

DisplayString, MacAddress, TruthValue

FROM SNMPv2-TC

ieee802dot11

FROM IEEE802dot11-MIB

InterfaceIndexOrZero

FROM IF-MIB;

ieee802dot11i MODULE-IDENTITY

LAST-UPDATED "0209100000Z"

ORGANIZATION "IEEE 802.11"

CONTACT-INFO

"WG E-mail: stds-802-11@ieee.org

Chair: 	Stuart J. Kerry

Postal: Philips Semiconductors, Inc.

1109 McKay Drive

M/S 48 SJ

San Jose, CA 95130-1706  USA

Tel: +1 408 474 7356

Fax: +1 408 474 7247

E-mail: stuart.kerry@philips.com

TGi Chair: David Halasz

Postal:

Tel:

Fax:

E-mail: dhala@cisco.com

Technical Editor: Jesse R. Walker

Postal:

Tel:

Fax:

Email: jesse.walker@intel.com

"

DESCRIPTION

"The MIB module for 802.11 entities implementing 802.11i (RSN/TSN)."

::= { ieee802dot11 7 }

--

--  Robust Security Network (RSN (and TSN)) Configuration

--

dot11RSNConfigTable OBJECT-TYPE

SYNTAX		SEQUENCE OF Dot11RSNConfigEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"The table containing RSN/TSN configuration objects."

::= { ieee802dot11i 1 }

dot11RSNConfigEntry OBJECT-TYPE

SYNTAX		Dot11RSNConfigEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"An entry in the dot11RSNConfigTable."

INDEX { dot11RSNConfigIndex }

::= { dot11RSNConfigTable 1 }

Dot11RSNConfigEntry ::=

SEQUENCE {

dot11RSNConfigIndex			InterfaceIndexOrZero,

dot11RSNConfigVersion			Integer32,

dot11RSNConfigPairwiseKeysSupported	TruthValue,

dot11RSNConfigMulticastCipher		INTEGER,

dot11RSNConfigGroupRekeyMethod		INTEGER,

dot11RSNConfigGroupRekeyTime		Unsigned32,

dot11RSNConfigGroupRekeyPackets		Unsigned32,

dot11RSNConfigGroupRekeyStrict		TruthValue,

dot11RSNConfigPSKValue			OCTET STRING,

dot11RSNConfigPSKPassPhrase		DisplayString,

dot11RSNConfigTSNEnabled		TruthValue

}

dot11RSNConfigIndex OBJECT-TYPE

SYNTAX		InterfaceIndexOrZero

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"Each 802.11 interface is represented by an entry in the

ifTable. If this index is zero, the information in this

table shall apply to all 802.11 interfaces."

::= { dot11RSNConfigEntry 1 }

dot11RSNConfigVersion OBJECT-TYPE

SYNTAX		Integer32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The highest RSN version this entity supports."

::= { dot11RSNConfigEntry 2 }

dot11RSNConfigPairwiseKeysSupported OBJECT-TYPE

SYNTAX		TruthValue

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"This object indicates whether the entity supports pairwise keys

for RSN. When FALSE, it only supports (four) default keys."

::= { dot11RSNConfigEntry 3 }

dot11RSNConfigMulticastCipher OBJECT-TYPE

SYNTAX		INTEGER { wep(1), tkip(2), wrap(3), ccmp(4) }

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"This object indicates the multicast cipher suite the entity is

currently using. The multicast cipher suite in the RSN

Information Element shall take its value from this variable.

With the information from the unicast cipher suites table and

the variable dot11RSNConfigTSNEnabled, the entity will often be

able to deduce the least common denominator and set the multicast

cipher suite accordingly. Therefore, this object may change

value when either the unicast cipher suites table is modified

or when the variable dot11RSNTSNEnabled is modified.

The network administrator can always override the automatically

selected multicast cipher suite by writing this object."

::= { dot11RSNConfigEntry 4 }

dot11RSNConfigGroupRekeyMethod OBJECT-TYPE

SYNTAX		INTEGER { disabled(1), timeBased(2), packetBased(3) }

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"This object selects a mechanism for rekeying the RSN Group

Key. The default is time-based, once per day."

DEFVAL		{ timeBased }

::= { dot11RSNConfigEntry 5 }

dot11RSNConfigGroupRekeyTime OBJECT-TYPE

SYNTAX		Unsigned32 (1..4294967295)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"The time in seconds after which the RSN group key must be

refreshed. The timer shall start at the moment the group key

was set using the MLME-SetKeys primitive."

DEFVAL		{ 86400 } -- once per day

::= { dot11RSNConfigEntry 6 }

dot11RSNConfigGroupRekeyPackets OBJECT-TYPE

SYNTAX		Unsigned32 (1..4294967295)

UNITS		"1000 packets"

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"A packet count (in 1000s of packets) after which the

RSN group key shall be refreshed. The packet counter shall

start at the moment the group key was set using the MLME-

SetKeys primitive and it shall count all packets encrypted

using the current group key."

::= { dot11RSNConfigEntry 7 }

dot11RSNConfigGroupRekeyStrict OBJECT-TYPE

SYNTAX		TruthValue

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"This object signals that the group key shall be refreshed

whenever a Station leaves the BSS."

::= { dot11RSNConfigEntry 8 }

dot11RSNConfigPSKValue OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(32))

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"The Pre-Shared Key (PSK) for when RSN in PSK mode is the

selected authentication suite. In that case, the PMK will

obtain its value from this object.

This object is logically write-only. Reading this variable

shall return unsuccessful status or null or zero."

::= { dot11RSNConfigEntry 9 }

dot11RSNConfigPSKPassPhrase OBJECT-TYPE

SYNTAX		DisplayString

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"The PSK, for when RSN in PSK mode is the selected

authentication suite, is configured by dot11RSNConfigPSKValue.

An alternative manner of setting the PSK uses the password-

to-key algorithm defined in section XXX. This variable

provides a means to enter a passphrase. When this object

is written, the RSN entity shall use the password-to-key

algorithm specified in section XXX to derive a pre-shared

and populate dot11RSNConfigPSKValue with this key.

This object is logically write-only. Reading this variable

shall return unsuccessful status or null or zero."

::= { dot11RSNConfigEntry 10 }

dot11RSNConfigTSNEnabled OBJECT-TYPE

SYNTAX		TruthValue

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"When dot11PrivacyInvoked and dot11RSNEnabled are both set to

TRUE, signaling that RSN is enabled on this entity, this object

shall indicate the entity also supports pre-RSN clients (with

or without an IEEE 802.1X supplicant), also referred to as a

Transitional Security Network (TSN)."

::= { dot11RSNConfigEntry 11 }

--

--  Unicast Cipher Suite configuration table

--

dot11RSNConfigUnicastCiphersTable OBJECT-TYPE

SYNTAX		SEQUENCE OF Dot11RSNConfigUnicastCiphersEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"This table lists the unicast ciphers supported by this entity.

It allows enabling and disabling of each unicast cipher by

network management. The Unicast Cipher Suite list in the RSN

Information Element is formed using the information in this

table."

::= { ieee802dot11i 2 }

dot11RSNConfigUnicastCiphersEntry OBJECT-TYPE

SYNTAX		Dot11RSNConfigUnicastCiphersEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"The table entry, indexed by the interface index (or all

interfaces) and the unicast cipher."

INDEX { dot11RSNConfigIndex, dot11RSNConfigUnicastCipherIndex }

::= { dot11RSNConfigUnicastCiphersTable 1 }

Dot11RSNConfigUnicastCiphersEntry ::=

SEQUENCE {

dot11RSNConfigUnicastCipherIndex	Unsigned32,

dot11RSNConfigUnicastCipher		INTEGER,

dot11RSNConfigUnicastCipherEnabled	TruthValue  }

dot11RSNConfigUnicastCipherIndex OBJECT-TYPE

SYNTAX		Unsigned32 (1..4294967295)

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"The auxiliary index into the dot11RSNConfigUnicastCiphersTable."

::= { dot11RSNConfigUnicastCiphersEntry 1 }

dot11RSNConfigUnicastCipher OBJECT-TYPE

SYNTAX		INTEGER { tkip(2), wrap(3), ccmp(4) }

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"A supported unicast cipher."

::= { dot11RSNConfigUnicastCiphersEntry 2 }

dot11RSNConfigUnicastCipherEnabled OBJECT-TYPE

SYNTAX		TruthValue

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"This object enables or disables the unicast cipher."

::= { dot11RSNConfigUnicastCiphersEntry 3 }

--

--  The Authentication Suites Table

--

dot11RSNConfigAuthenticationSuitesTable OBJECT-TYPE

SYNTAX 		SEQUENCE OF Dot11RSNConfigAuthenticationSuitesEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"This table lists the authentication suites supported by this

entity. Each authentication suite can be individually enabled

and disabled. The Authentication Suite List in the RSN IE

is formed using the information in this table."

::= { ieee802dot11i 3 }

dot11RSNConfigAuthenticationSuitesEntry OBJECT-TYPE

SYNTAX 		Dot11RSNConfigAuthenticationSuitesEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"An entry (row) in the dot11RSNConfigAuthenticationSuitesTable."

INDEX { dot11RSNConfigAuthenticationSuiteIndex }

::= { dot11RSNConfigAuthenticationSuitesTable 1 }

Dot11RSNConfigAuthenticationSuitesEntry ::=

SEQUENCE {

dot11RSNConfigAuthenticationSuiteIndex		Unsigned32,

dot11RSNConfigAuthenticationSuite		INTEGER,

dot11RSNConfigAuthenticationSuiteEnabled	TruthValue }

dot11RSNConfigAuthenticationSuiteIndex OBJECT-TYPE

SYNTAX		Unsigned32 (1..4294967295)

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"The auxiliary variable used as an index into the

dot11RSNConfigAuthenticationSuitesTable."

::= { dot11RSNConfigAuthenticationSuitesEntry 1 }

dot11RSNConfigAuthenticationSuite OBJECT-TYPE

SYNTAX		INTEGER { dot1X(1), psk(2) }

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"An authentication suite. Note that dot1X(1) and PSK(2) may

not be enabled simultaneously. These are mutually exclusive."

::= { dot11RSNConfigAuthenticationSuitesEntry 2 }

dot11RSNConfigAuthenticationSuiteEnabled OBJECT-TYPE

SYNTAX		TruthValue

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"This variable indicates whether the corresponding

authentication suite is enabled/disabled."

::= { dot11RSNConfigAuthenticationSuitesEntry 3 }

--

--  RSN/TSN statistics

--

dot11RSNStatsTable OBJECT-TYPE

SYNTAX		SEQUENCE OF Dot11RSNStatsEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"This table maintains per-STA statistics for SN. The entry

with dot11RSNStatsSTAAddress set to FF-FF-FF-FF-FF-FF shall

contain statistics for broadcast/multicast traffic."

::= { ieee802dot11i 4 }

dot11RSNStatsEntry OBJECT-TYPE

SYNTAX		Dot11RSNStatsEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"An entry in the dot11RSNStatsTable."

INDEX { dot11RSNConfigIndex, dot11RSNStatsIndex }

::= { dot11RSNStatsTable 1 }

Dot11RSNStatsEntry ::=

SEQUENCE {

dot11RSNStatsIndex			Unsigned32,

dot11RSNStatsSTAAddress			MacAddress,

dot11RSNStatsVersion			Unsigned32,

dot11RSNStatsSelectedUnicastCipher	INTEGER,

dot11RSNStatsTKIPICVErrors		Counter32,

dot11RSNStatsTKIPMICFailures		Counter32,

dot11RSNStatsWRAPFormatErrors		Counter32,

dot11RSNStatsWRAPReplays		Counter32,

dot11RSNStatsWRAPDecryptErrors		Counter32,

dot11RSNStatsCCMPFormatErrors		Counter32,

dot11RSNStatsCCMPReplays		Counter32,

dot11RSNStatsCCMPDecryptErrors		Counter32  }

dot11RSNStatsIndex OBJECT-TYPE

SYNTAX		Unsigned32 (1..4294967295)

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"An auxiliary index into the dot11RSNStatsTable."

::= { dot11RSNStatsEntry 1 }

dot11RSNStatsSTAAddress OBJECT-TYPE

SYNTAX		MacAddress

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The MAC address of the station the statistics in this

conceptual row belong to."

::= { dot11RSNStatsEntry 2 }

dot11RSNStatsVersion OBJECT-TYPE

SYNTAX		Unsigned32 (1..4294967295)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The RSN version which the station associated with."

::= { dot11RSNStatsEntry 3 }

dot11RSNStatsSelectedUnicastCipher OBJECT-TYPE

SYNTAX		INTEGER { tkip(2), wrap(3), ccmp(4) }

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The unicast cipher the station selected during association."

::= { dot11RSNStatsEntry 4 }

dot11RSNStatsTKIPICVErrors OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Counts the number of TKIP ICV errors encountered when

decrypting packets for the station."

::= { dot11RSNStatsEntry 5 }

dot11RSNStatsTKIPMICFailures OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Counts the number of Michael MIC failure encountered when

checking the integrity of packets received from the station."

::= { dot11RSNStatsEntry 6 }

dot11RSNStatsWRAPFormatErrors OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The number of MSDUs received with an invalid WRAP format."

::= { dot11RSNStatsEntry 7 }

dot11RSNStatsWRAPReplays OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The number of received unicast fragments discarded by the replay

mechanism."

::= { dot11RSNStatsEntry 8 }

dot11RSNStatsWRAPDecryptErrors OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The number of received fragments discarded by the OCB decryption

mechanism."

::= { dot11RSNStatsEntry 9 }

dot11RSNStatsCCMPFormatErrors OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The number of MSDUs received with an invalid CCMP format."

::= { dot11RSNStatsEntry 10 }

dot11RSNStatsCCMPReplays OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The number of received unicast fragments discarded by the

replay mechanism."

::= { dot11RSNStatsEntry 11 }

dot11RSNStatsCCMPDecryptErrors OBJECT-TYPE

SYNTAX		Counter32

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The number of received fragments discarded by the CCMP

decryption algorithm."

::= { dot11RSNStatsEntry 12 }

--

--   TBD: OBJECT-GROUPs and MODULE-COMPLIANCE statements

--

END