IEEE P802.11
Wireless LANs

RSN Capabilities Field

Date:	September 11, 2002

Author:	David Halasz
	Cisco Systems, Inc.
	320 Springside Dr. Suite 350
	Akron, Ohio 44333
	Phone: 330-664-7389
	Fax: 330-664-7301
	e-Mail: dhala@cisco.com

Abstract

IEEE 802.11i draft provides for a Transition Security Network (TSN). A TSN provides for migration to a Robust Security Network (RSN). This submission introduces a capability field to provide for advertisement of optional functions and to promote migration to an RSN.

The following changes to 7.3.2.17 are to provide for an RSN capabilities field.

Change Figure 6 to the following,

| Element ID 1 octet | Length 1 octet | Version 2 octets | Group Key Cipher Suite 4 octets | Pairwise Key Cipher Suite Count 2 octets | Pairwise Key Cipher Suite List 4n octets | Authenticated Key Suite Count 2 octets | Authenticated Key Suite List 4n octets | RSN Capabilities 2n octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Change Example 1 to the following,

- 802.1X authentication, CCMP pairwise and group key cipher suites (WEP and TKIP not allowed).

30, // information elment id, 48 expressed as Hex value

14,  // length in bytes, 20 expressed as Hex value

01 00, // Version 1

00 00 00 04, // CCMP as group key cipher suite

01 00, // pairwise key cipher suite count

00 00 00 04, // CCMP as pairwise key cipher suite

01 00, // authentication count

00 00 00 01 // 802.1X authentication

00 00 //No capabilities

Add the following text in 7.3.2.17,

RSN Capability Information

The RSN Capability Information field contains a number of subfields that are used to indicate requested or advertised capabilities. The length of the RSN Capability Information field is two octets. The RSN Capability Information field consists of the following subfield: Pre-Authentication Support. The remaining part of the RSN Capability Information field is reserved. The format of the Capability Information field is as illustrated in Figure XX.

APs set the Pre-Auth subfield to 1 within the RSN Capability field if the AP supports RSN pre-authentication. If the AP does not support RSN pre-authentication the Pre-Auth subfield is set to 0. Non-AP STAs should set the Pre-Auth subfield to 0.