IEEE P802.11
Wireless LANs

Suggested Frame Exchange Draft Text for Clause 5.4.3

Date:	November 3, 2002

Author:	Nancy Cam-Winget
	Cisco Systems, Inc.
	3625 Cisco Way 
	San Jose, Ca. 95134
	Phone: 408-853-0532
	e-mail:ncamwing@cisco.com  
	
	Thomas Maufer
	NVIDIA Corporation
	2701 San Tomas Expressway
	Santa Clara, Ca. 95050-2519
	Phone: 408-486-2637
	e-mail: tmaufer@nvidia.com

Abstract

This document provides proposed text for Clause 5 to provide a general description for how to negotiate and achieve a secure 802.11 link.

Motion:

Instruct the editor to add the text below as clause 5.9.3.1.

### Clause 5.9.3.1

Before 802.11 packets can be protected, the STA must open 802.11 authenticate and associate to the AP.  These steps allow the STA and AP to negotiate the security association including: the authenticated key management, unicast and multicast cipher suite. 802.11 Open authentication and association are used to retain legacy 802.11 state flow.   Figure 1  depicts  how a client discovers an AP and negotiates security policy prior to establishing a security association.

Figure 1 Establishing the 802.11 connection and negotiation

Client

AP

Probe Request

Probe Response + RSN IE (AP supports MCast/Ucast: CCMP, WRAP, TKIP, WEP and 802.1X EAP Authentication)

802.11 Open Authentication

802.11 Open Auth (success)

Association Req + RSN IE (Client requests TKIP and 802.1X EAP Authentication)

Association Response (success)

802.1X controlled port blocked for client AID

Once the client and AP have successfully negotiated the security policy, both parties filter both 802.11 and 802.1X frames to only allow 802.1X EAP authentication frames.  The next phase is for the client to successfully authenticate with an Authentication Server (AS) as shown in Figure 2.

Figure 2  802.1X EAP Authentication

AS

802.1X/EAP-Request Identity

802.1X/EAP-Response Identity            (EAP type specific)

EAP Access Request/Identity

EAP type specific mutual authentication

Derive Pairwise Master Key (PMK)

EAP ACCEPT  (with PMK via MS-MPPE)

802.1X/EAP-SUCCESS

802.1X controlled port still blocked for client AID

AP

Client

Derive Pairwise Master Key (PMK)

Since 802.1X EAP authentication is achieved between the AS and client, the client must then establish mutual authentication and liveness with the AP.  Both the AP is still block general 802.11 data packets and only allow 802.1X EAP packets to flow in order to achieve mutual authentication with the client and derive the required link keys to protect both unicast and multicast traffic with the client.  This is achieved by the use of a 4-way handshake shown in Figure 3 and described in more detail in Clause 8.4.1.1.

Client

AP

PMK

Derive ANonce

Derive SNonce

EAPoL-Key(Unicast, SNonce, MIC, STA RSN IE)

EAPoL-Key(Reply Required, Install PTK, Unicast, ANonce, MIC, AP RSN IE)

Derive PTK

Derive PTK

EAPoL-Key(Unicast, ANonce, MIC)

Install Keys

802.1X controlled port still blocked for client AID

PMK

EAPoL-Key(Reply Required, Unicast, ANonce)

Install Keys

Figure 3 Establishing Unicast Keys

Once the client and AP have established authentication and liveness of unicast keys, the AP can use these keys to deliver the keys required to protect multicast traffic.  This last phase is achieved with a two message exchange.  Upon it’s success, both client and AP can open the 802.1X port for protected communications.  The last phase is shown in Figure 4.

Client

AP

EAPoL-Key(All Keys Installed, Reply Required, Group Rx, Key Index, Group, GNonce, MIC, GTK)

GMK

Derive Gnonce & GTK

EAPoL-Key(Group, MIC)

Encrypt GTK field

Decrypt GTK field

802.1X controlled port unblocked for client AID

Figure 4 Multicast Key Delivery - Final Phase