IEEE P802.11
Wireless LANs

MLME-DELETEKEYS

Date:	November 11th, 2002

Authors:		David Johnston
	Mobilian Corporation
	7431 NW Evergreen Parkway,
	Suite 220,
	Hillsboro, OR, 97006
	Phone: +1 503-681-6800 x282
	E-mail: dj@mobilian.com; david.johnston@ieee.org

This document describes modifications to sections 8.7 of IEEE Std 802.11i/D2.4.2, edited by Jesse Walker.

Introduce text detailing the MLME-DELETEKEYS primitive:

### 10.3.12 DeleteKeys

#### 10.3.12.1  MLME-DELETEKEYS.request

##### 10.3.12.1.1  Function

This primitive causes the keys identified in the parameters of the primitive to be deleted from the MAC and thus disabled for use.

##### 10.3.12.1.2  Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-DELETEKEYS.request 	(
Keylist,
)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Keylist | A set of KeyIdentifiers | N/A | The list of keys to be deleted from the MAC |

Each KeyIdentifier consists of the following elements:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Address | MAC Address | Any valid individual MAC address | This parameter is valid only when the key type is Pairwise and contains an 802 address |

##### 10.3.11.1.3  When Generated

This primitive is generated by the SME at any time when keys for a security association are to be deleted in the MAC.

##### 10.3.11.1.4  Effect of Receipt

Receipt of this primitive causes the MAC to

- Delete the appropriate keys, both group and pairwise and to cease using them.

- Set aHaveGTK to FALSE

- Set aHavePTK to FALSE

#### 10.3.12.2  MLME-DELETEKEYS.confirm

##### 10.3.12.2.1  Function

This primitive confirms that the action of the associated MLME-DELETEKEYS.request has been completed.

##### 10.3.12.2.2  Semantics of the service primitive

This primitive has no parameters.

#### 10.3.12.2.3  When Generated

This primitive is generated by the MAC in response to receipt of a MLME-DELETEKEYS.request primitive. This primitive is issued when the action requested has been completed.

#### 10.3.12.2.4  Effect of Receipt

The SME is notified that the requested action of the MLME-DELETEKEYS.request is completed.