IEEE P802.11
Wireless LANs

Addition of Authentication Suite to TGi MIB

Date:	November 11, 2002

Author:	Frank Ciotti
	<Company>
	118 Frosted Pond Place
	The Woodlands, TX  77381
	Phone: 281-419-0050
	Fax: 
	e-Mail: frankc@msn.com

Motion:

Instruct the editor to insert the following MIB object into the 802.11i MIB as dot11RSNStatsEntry 5, and renumber the objects dot11RSNStatsEntry 5-14 as dot11RSNStatsEntry 6-15.

dot11RSNStatsSelectedAuthenticationSuite OBJECT-TYPE

SYNTAX            OCTET STRING (SIZE(4))

MAX-ACCESS        read-only

STATUS            current

DESCRIPTION

"The Authentication Suite the station selected during association.  The value consists of a three octet OUI followed by a one octet Type as follows:

OUI      Value   Authentication Type          Key Management Type

--------    -----   -------------------          -------------------

00:00:00      0     Reserved                     Reserved

00:00:00      1     Unspecified authentication   802.1X Key Management
                        over 802.1X

00:00:00      2     None                         802.1X Key Management
                                                     using pre-shared Key

00:00:00    3-255   Reserved                     Reserved

Vendor      any    Vendor Specific              Vendor Specific

other      any    Reserved                     Reserved"

::= { dot11RSNStatsEntry 5 }