IEEE 802.11
Wireless LANs

Proposed PICS for P802.11e

Date:	November 13, 2002

Author:	Srini Kandala (Sharp)
Srini@SharpLabs.com

Keith Amann (SpectraLink)
Mathilde Benveniste (Avaya)
Peter Johansson (Congruent Software)
Duncan Kitchin (Intel)
	John Kowalski (Sharp)
	Isaac Lim Wei Lih (Panasonic)
Luke Ludeman (SpectraLink)
Yoshihiro Ohtani (Sharp)
Javier del Prado (Philips)
Syd Schrum (Texas Instruments)
Matthew Sherman (AT & T)
Amjad Soomro (Philips)
Shugong Xu (Sharp)

Abstract

This document contains a proposed Protocol Implementation Conformance Statement (PICS) for P802.11e

Add the following entries to the table in A.4.3:

### A.4.3 IUT configuration

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| CF12 | QoS supported | 9.10, 9.11 | O | Yes, No |
| CF13 | Enhanced distributed channel access | 9.10.1 | CF12:M | Yes, No |
| CF14 | QoS Parameterized - channel access | 9.10.2 | CF12:M | Yes, No |

Insert the following at the end of A.4:

### A.4.x QoS Base Functionality

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QB1 | QoS frame format | 7.1.3..1, 7.1.3.2, 7.1.3.4 - 7.1.3.6, 7.2.2, 7.2.3.1, 7.2.3.4 - 7.2.3.9, 7.2.3.12, 7.3.1.4, 7.3.1.7, 7.3.1.9, 7.3.1.4, 7.3.2.13 -7.3.2.20, 7.5 | CF12:M | Yes, No |
| QB2 | Per TID duplicate detection | 7.1.3.4, 7.1.3.5, 9.10.1.6 | CF12:M | Yes, No |
| QB3 | Decode of no-ack policy in QoS data frames | 7.1.3.5.2, 9.10.1.4, 9.10.1.5, 9.10.1.7.1, 9.10.1.7.2 | CF12:M | Yes, No |
| QB4 | Group acknowledgements | 7.2.1.7, 7.2.1.8, 7.4.1.5, 7.4.1.6, 7.4.1.7, 9.11, 11.5 | CF12:O | Yes, No |
| QB5 | Automatic Power Save Delivery | 7.3.2.18, 7.4.1.6, 11.2.3 | CF12:O | Yes, No |
| QB6 | Direct Link Protocol | 7.3.2.20, 7.4.2, 10.3.13 | (CF1 AND CF12):M (CF2 AND CF12):O | Yes, No |

### A.4.y QoS Enhanced Distributed Channel Access

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QD1 | Support for four transmit queues with a separate channel access entity associated with each | 9.10.1.1 | CF13:M | Yes, No |
| QD2 | Per-channel access function differentiated channel access | 9.10.1.2, 9.10.1.3, 9.10.1.5 | CF13:M | Yes, No |
| QD3 | Continuation TXOP support | 9.10.1.4 | CF13:M | Yes, No |
| QD4 | Maintenance of within-queue ordering, exhaustive retransmission when sending non-QoS data frames | 9.10.1.6 | CF13:M | Yes, No |
| QD5 | Distributed admission control | 9.10.1.7 | CF13:M | Yes, No |

### A.4.z QoS Parameterized Channel Access

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QP1 | TSPEC and associated frame formats | 7.4.1.1 – 7.4.1.4 | CF14:M | Yes, No |
| QP2 | HCF controlled channel access rules | 9.1.3.2, 9.10.2.1 - 9.10.2.3 | CF14:M | Yes, No |
| QP3 | HCF controlled channel access schedule generation and management | 9.10.2.4 | CF14:M | Yes, No |
| QP4 | HCF frame exchange sequences | 9.10.3 | CF14:M | Yes, No |
| QP5 | TS Management | 11.4 | CF14:M | Yes, No |
| QP6 | Minimum TSPEC parameter set | 9.10.2.4 | CF14:M | Yes, No |
| QP7 | Power management | 11.2.1.10 | CF14:M | Yes, No |