IEEE P802.11
Wireless LANs

Roaming Counter Proposal

Date:	May 9, 2003

Author:	Lars Falk, Åse Jevinger
	TeliaSonera Mobile Networks AB
	Box 94, SE-201 20 Malmö, Sweden
	Phone: +46 (0)40 105133 
	Fax: +46 (0)40 307029  
	e-Mail: Lars.P.Falk@teliasonera.com

This submission proposes to add a counter for the number of handovers that the STA has done, a Roaming Counter.

# Introduction

In an environment where a terminal is located within the coverage area of several IEEE 802.11 APs and WWAN base stations (e.g. GPRS), a decision must be taken of whether a multi-access enabled terminal should associate with the WWAN or the WLAN.  For a terminal that is associated to the WLAN, but makes frequent handovers between the APs (e.g. the user is a passenger in a vehicle), it might be a good idea to change to the WWAN.

Therefore we propose that a counter (RoamingCount) keeps track on the number of handovers done within a certain time period (RoamingCountPeriod). The process of selecting the optimum network is called the “access selection process”, which may be located within the network, within the terminal or in both as a cooperation between the network and the terminal. Therefore the RoamingCount parameters should be available both in the terminal and on the network side. Irrespective of where the process is located, the parameters can be of great value as input parameters to the access selection process.

# Proposal

Add one row to table 0-2:

| Measurement Name | Measurement Type |
| --- | --- |
| Unused | 0-2 |
| Channel Load Request | 3 |
| Noise Histogram Request | 4 |
| Beacon Request | 5 |
| Frame Request | 6 |
| Hidden Node Request | 7 |
| Roaming Counter Request | 8 |
| Reserved | 9-255 |

Add one row to table 0-5:

| Measurement Name | Measurement Type |
| --- | --- |
| Unused | 0-2 |
| Channel Load Report | 3 |
| Noise Histogram Report | 4 |
| Beacon Report | 5 |
| Frame Report | 6 |
| Hidden Node Report | 7 |
| Roaming Counter Report | 8 |
| Reserved | 9-255 |

Add the following two sub-sections to the TGk draft and renumber sections and figure as necessary:

##### 7.3.2.19.X Roaming Counter Request

The Measurement Request field corresponding to a Roaming Counter Request is empty. A response to a Roaming Counter Request is a Roaming Counter Report.

##### 7.3.2.20.X Roaming Counter Report

The format of the Measurement Report field of a Roaming Counter Report is shown in Figure 0-X.

|  | Number of Roaming Counts | Roaming Count Period |  |
| --- | --- | --- | --- |
| Octets: | 4 | 4 |  |

Figure 0-X – Measurement Report field format for a Roaming Counter Report

Number of Roaming Counts contains the number of times the STA has roamed from one AP to another.

# Roaming Count Period is the time period, expressed in TUs, during which the Number of Roaming Counts have been measured.

Add the following, where appropriate (e.g. to the Counters Table), in 11-03/171r4 or the TGk draft:

dot11staRoamingCount       Counter32,

dot11RoamingCountPeriod    INTEGER

…

dot11RoamingCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented every time the STA associates to a new AP."

::= { dot11CountersEntry ? }

dot11RoamingCountPeriod OBJECT-TYPE

SYNTAX INTEGER (0..4296967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs elapsed

since the STA for the first time associated to an AP."

::= { dot11CountersEntry ? }