IEEE P802.11
Wireless LANs

Draft 5.1 Edits

Date:	Sept 16, 2003

Author:	Tim Moore
	Microsoft
	1 Microsoft Way, Redmond, WA, 98052
	Phone: 1-425-703-9861
	Fax: 
	e-Mail: timmoore@microsoft.com

Abstract

Summary

8.3.2.3.3

Change

WEP seed  Phase2(TTAK, TA, TSC)

To

WEP seed  Phase2(TTAK, TK, TSC)

8.4.3.1

If an AP operating within a TSN receives a (Re)association request without an RSN IE, it shall allow communications only if a WEP key has been configured or if the key will be provisioned by IEEE 802.1X to secure communication. If a WEP key is not installed, and if IEEE 802.1X authentication and key management is not enabled, the AP shall reject the association request; if a WEP key is configured or if the AP is configured to provide IEEE 802.1X authenticated key management, the AP may accept the request.

5.4.3.2

In a RSNA ESS, Open System Authentication is required for MAC layer authentication.  In a RSNA ESS, Deauthentication results in termination of any association for the deauthenticated station. It also results in the IEEE 802.1X controlled port for that STA being disabled. The Deauthentication notification is provided to IEEE 802.1X via the MAC layer.

In a RSNA IBSS, Open System Authentication is optional but a STA is required to recognize Deauthentication frames. Deauthentication results in the IEEE 802.1X controlled port for that STA being disabled.

5.6

In an IBSS a STA must be prepared for other STAs to initiate communications. Each pair of STAs in an IBSS negotiates the security algorithms to be used. In an ESS the STA initiates all associations.  In an ESS the STA and AP negotiate the security suite, and the AP enforces the security suite to be used.

In a RSNA ESS, the AS makes the authentication decision. The AS is typically separate from the AP, but may be integrated into the AP. In a RSNA IBSS, each STA must make its own authentication decision regarding each peer.

8.4.3

Informative Note: This can be enforced by configuring the AP to use only RSNA cipher and authentication and key management suites, i.e., by disabling WEP and pre-RSNA IEEE 802.1X key management.

7.2.3.9

The cipher suite selectors 00:00:00:1 “WEP-40” and 00:00:00:5 “WEP-104” are only valid as a group cipher suite when communicating with pre-RSNA devices.

7.3.2.9.5 4th example

802.1X authentication, Use CCMP for unicast cipher suite, CCMP group key cipher suites, pre-authentication and a PMKID.

5.4.3.3

Replace “Confidentiality is an SS.” with “Confidentiality is a SS.”

6.1.2

Replace “The security services provided by the WEP” with “The security services provided by WEP”.

8.1.3

Replace “It may optionally perform MAC authentication and if such authentication is utilized it shall be Open System Authentication.”

with “It may optionally perform MAC authentication and, if such authentication is utilized, it shall be Open System Authentication.”.

5.7.5

Replace Clause 5.7.5 with:

“For a STA to invoke the WEP privacy confidentiality algorithm (as controlled by the related MIB attributes, see Clause 11), the confidentiality service selects WEP encryption and sets the Protected Frame bit appropriately (see Clause 7).”

8.5.2

Replace the first bullet item in Clause 8.5.2

“4-Way Handshake, to confirm that the PMK between associated STAs are the same and is live and to transfer the GTK to the STA.”

With

“4-Way Handshake, to confirm that the PMK between associated STAs is the same and live and to transfer the GTK to the STA.”

# Annex D (normative) ASN.1 encoding of the MAC and PHY MIB

Change dot11Compliance as follows:

dot11Compliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the IEEE 802.11 MIB."

MODULE  -- this module

MANDATORY-GROUPS {

dot11SMTbase24,

dot11MACbase, dot11CountersGroup,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID, dot11PhyOperationComplianceGroup

}

Delete the  following text::

--

--  IEEE 802.11 RSN MIB

--

IEEE802dot11RSN-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, Integer32, Unsigned32, Counter32

FROM SNMPv2-SMI

DisplayString, MacAddress, TruthValue

FROM SNMPv2-TC

ieee802dot11

FROM IEEE802dot11-MIB

InterfaceIndexOrZero

FROM IF-MIB;

ieee802dot11RSN MODULE-IDENTITY

LAST-UPDATED "0209100000Z"

ORGANIZATION "IEEE 802.11"

CONTACT-INFO

"WG E-mail: stds-802-11@ieee.org

Chair:		Stuart J. Kerry

Postal:	Philips Semiconductors, Inc.

1109 McKay Drive

M/S 48 SJ

San Jose, CA 95130-1706  USA

Tel:		+1 408 474 7356

Fax:		+1 408 474 7247

E-mail:	stuart.kerry@philips.com

TGi Chair:	David Halasz

Postal:

Tel:

Fax:

E-mail:	dhala@cisco.com

Technical Editor:	Jesse R. Walker

Postal:	Intel Corporation

JF3-466

2111 N.E. 25th Avenue

Hillsboro, OR 97124-5961 USA

Tel:		+1 503 712 1849

Fax:

Email:		jesse.walker@intel.com

"

DESCRIPTION

"The MIB module for 802.11 entities implementing 802.11 RSN (Robust Security Network Associations)."

::= { ieee802dot11 7 }

Add after dot11RandomTableFieldNumber the following text:

--

--  Robust Security Network Association (RSNA and TSN) Configuration

--

dot11RSNAConfigTable OBJECT-TYPE

SYNTAX		SEQUENCE OF Dot11RSNAConfigEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"The table containing RSNA configuration objects."

::= { ieee802dot11 5 }

--

--  RSNA statistics

--

dot11RSNAStatsTable OBJECT-TYPE

SYNTAX		SEQUENCE OF Dot11RSNAStatsEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"This table maintains per-STA statistics for SN. The entry with dot11RSNAStatsSTAAddress set to FF-FF-FF-FF-FF-FF shall contain statistics for broadcast/multicast traffic."

::= { ieee802dot11 6 }