IEEE P802.11
Wireless LANs

PMK sharing

Date:	Sept 17, 2003

Author:	Tim Moore
	Microsoft
	1 Microsoft Way, Redmond, WA, 98052
	Phone: 1-425-703-9861
	Fax: 
	e-Mail: timmoore@microsoft.com

Abstract

Summary

Powerpoint is 11-03-0769-00-000i-PMKsharing.ppt

Add definitions to Clause 3

Supplicant Group Address: A MAC Address for a group of Supplicants that are a secure group and can share a MSK between them.

Authenticator Group Address: A MAC Address for a group of Authenticators that are a secure group and can share a MSK between them.

Clause 7.3.2.9

Change Figure 7

| Element ID 1 octet | Length 1 octet | Version 2 octets | Group Key Cipher Suite 4 octets | Pairwise Key Cipher Suite Count 2 octets | Pairwise Key Cipher Suite List 4m octets | Authentication and Key Management Suite Count 2 octets | Authentication and Key Management Suite List 4n octets | RSN Capabilities 2 octets | Group Addr 6 octets | PMKID Count 2 octets | PMKID List 16s octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Add

7.3.2.9.4 Group Addr

Group Addr is the Supplicant Group MAC address or the Authenticator Group MAC address. It is supplied if the Supplicant or Authenticator is a member of a group.

Informational Note: The Group MAC address may be one of the individual MAC addresses.

The Authenticator Group MAC address is used when the RSN IE used in a Beacon or Probe response. The Supplicant Group MAC address is used when the RSN IE is used in a (re)association request.

Renumber 7.3.2.9.4 PMKID to 7.3.2.9.5 PMKID

Clause 5.9.5

Add the following at the end of 5.9.5

Supplicant can only use PMKs that were obtained from an IEEE 802.1X authentication via Authenticators in the same Authenticator Group Address.

Authenticators can only use PMKs that were obtained from an IEEE 802.1X authentication with Supplicants in the same Supplicant Group Address.

8.5.1.2

The PMK is derived from the Master Session Key. The MSK is available directly to the Supplicant. The MSK is available to the Authenticator as the AAA Key.

PMK = PRF-256(MSK(0-255), “PMK Key”|Supplicant-MAC-Address|Authenticator-MAC-Address)

Where MSK(0-255) is the first 256bits of the MSK.