IEEE P802.11
Wireless LANs

Usage Models

Date:	May 10, 2004

Authors/Contributors:

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P. Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2°, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Bruno Jechoux | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | jechoux@tcl.ite.mee.com |
| Eldad Perahia | Cisco |  |  |  | eperahia@cisco.com |
| Hervé Bonneville | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | 1elecom1le@tcl.ite.mee.com |
| Javier del Prado | Philips | 345 Scarborough Rd, Briarcliff Manor, NY, 10510, USA | +1 914 945 6000 | +1 914 945 6580 | javier.delprado@philips.com |
| Mary Cramer | Agere Systems | 1110 American Parkway NE, Allentown, PA 18109-9138, USA | +1 610-712-6112 | +1 610 712 1182 | mecramer@agere.com |
| Paul Feinberg | Sony | 1 Sony Drive MD TA1-5 Park Ridge, NJ 07656 | +1 201 930-6316 | +1 201 930-6397 | Paul.Feinberg@am.sony.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Sanjeev Sharma | Samsung | 75 W. Plumeria Dr. , San Jose, CA, 95134 | +1 408- 544 5978 |  | ssharma@sisa.samsung.com |
| Timothy P Wakeley | Hewlett Packard Corporation | 8000 Foothills Blvd, Roseville, CA 95747 | 916-785-1619 |  | Tim.Wakeley@hp.com |
| Tomer Bentzion | Metalink | Yakum Business Park, 60972 Yakum Israel | +972 9 960 5365 | +972 9 960 5399 | tomerb@metalink.co.il |
| Vinko Erceg | Zyray Wireless |  |  |  | Verceg@zyraywireless.com |
| Youngsoo Kim | Samsung | Mt. 14-1 Nongseo-Ri, Giheung-Eup, Yongin-Si, Gyeonggi-Do, Korea 449-712 | +82-31-280-9614 | +82-31-280-9555 | kimyoungsoo@samsung.com |
| George Vlantis | ST Micro-electronics | 1060 E. Brokaw Road San Jose, CA 95131 | +1 408-451-8109 |  | George.Vlantis@st.com |
| Valerio Filauro | ST Micro-electronics | 1060 E. Brokaw Road San Jose, CA 95131 | +1 408-451-8109 |  | Valerio.Filauro@st.com |

Abstract

This document defines usage models for 802.11 TGn, intended to be used as part of the selection process to generate simulation results for specified well-defined simulation scenarios.

# Revision History of Document 11-03-0802

| ### Revision | ### Comments | ### Date | ### Author |
| --- | --- | --- | --- |
| R0 | Document 11-03-355r11-htsg was approved as the Usage Model for TGn at the September, 2003 meeting of TGn. R0 was created from that document in order to give it a TGn document number with a number of minor editorial changes. | September 19, 2003 | Adrian Stephens |
| R1 draft 2 | Simulation Scenarios modified to match the updated usage models. | September 25, 2003 | Adrian Stephens |
| R2 draft 3 | Removed STA2 from Simulation Scenario 1 | October 27, 2003 | Adrian Stephens |
| R2 | Merged in Mary Cramer's changes to enterprise, adding Appendix 1. Consistency updates to the simulation scenarios. | November 10, 2003 | Adrian Stephens |
| R3 | Updated Appendix1 based on new text supplied by Mary | November 11, 2003 | Adrian Stephens |
| R4 | Modified usage model 4 to remove top half and note. Usage model not yet approved by FRCC. | November 11, 2003 | Adrian Stephens |
| R5 | Revised linkage to channel models to conform to 03/940 | November 12, 2003 | Colin Lanzl |
| R6 | Note added during TGn session at top of section “Common Conditions”. | November 12, 2003 | Adrian Stephens |
| R7 | Missing revision history for R6 added at request of FRCC telecon and document up-issued. | November 18, 2003 | Adrian Stephens |
| R8 | Reviewed by SS Group at January 2004 TGn session | January 13, 2004 | Adrian Stephens, Andy Friefeld, Sunghyun Choi, Philip Kossin, Mohammad Ikram |
| R9 | Additions from Iterop/Coex Group at Jan 2004 TGn session | January 13, 2004 | John Ketchum |
| R10 | Specification of shadowing added to conditions section. Scenario 19 added to support CC 27-28 Scenario 19 moved to 16 as this SS is the number is used by the FR to refer to a point-point link throughput test. Also suitable for CC58 | January 14, 2004 | Adrian Stephens, Sanjiv Nanda |
| R11 | Updated table 5 to refer to new simulation scenarios | January 25, 2004 | Adrian Stephens |
| R12 | Table of channel model to simulation scenario mapping added | February 5, 2004 | Adrian Stephens |
| R13 | Relationship of application and QoS just before table 4 added. | March 9, 2004 | Sanjiv Nanda |
| R14 | Updated During TGn March 2004 session | March 16, 2004 | Adrian Stephens |
| R15 | Editorial Changes in 11-04-323r1 implemented Changes agreed at March 23, 2004 FRCC telecon implemented. Mandatory/Optional labels removed as the comparison criteria document [6] specifies this. | March 17, 2004 March 24, 2004 | Adrian Stephens |
| R16 | Changes agreed at April 6, 2004 FRCC telecon implemented. | April 6, 2004 | Adrian Stephens |
| R17 | Changes agreed at April 20, 2004 FRCC telecom implemented | April 21, 2004 | Adrian Stephens |
| R18 | Changes agreed at May 4, 2004 FRCC telecom implemented | May 5, 2004 | Adrian Stephens |
| R19 | Tracking removed from changes made at the March TGn session. | May 10, 2004 | Adrian Stephens |
| R20 | Updated during May TGn FRCC session | May 10, 2004 | Adrian Stephens |
| R21 | Updated during final review of changes | May 10, 2004 | Adrian Stephens |
| R22 | Modified during TGn session section 8.1. | May 10, 2004 | Adrian Stephens |

# Revision History of Document 11-03-355

| ### Revision | ### Comments | ### Date | ### Author |
| --- | --- | --- | --- |
| R0 Draft 0 | This document is in its beginning phases. The initial target is to generate a concept that can be reviewed and commented on. The first draft has definitions of some terms along with an initial stab at a few use cases. This is intended to start discussion and review. | July 8, 2003 | Mary Cramer |
| R0 Draft 1 | APS additions | July 9, 2003 | Adrian Stephens |
| R0 | Merged in comments and changes from the group of authors and made public via the .11 reflector. | July 11, 2003 | Adrian Stephens |
| R1 Draft 1 | Javier's contribution merged in Lalit's Contribution merged in Changes made to implement telecon discussion on 14 July 2003 Application table added Usage Models adjusted to reference applications named in the application table | July 14, 2003 | Adrian Stephens |
| R1 Draft 2 | Paul Feinberg's contribution merged in | July 18, 2003 | Adrian Stephens |
| R2 Draft 3 | Chiu Ngo's comments merged in | 21 July 2003 | Adrian Stephens |
| R2 | Results of Use Case voting added and table of use cases sorted by score Printing application and use cases added (Tim Wakeley) | 22 July 2003 | Adrian Stephens |
| R3 | Updates made at face-to-face session of the special committee, (8:00 am, 24 July 2003). Validation of usage models against use cases partially considered. | 24 July 2003 | Adrian Stephens |
| R4 | Merged in comments from Tiger Team (Rahul Malik, Bjorn Bjerke, Eldad Perahia, Paul Feinberg, Youngsoo Kim) to complete use case coverage in usage models. | 1 Aug 2003 | Adrian Stephens |
| R5 draft 1 | Merged in comments by Vinko Erceg on appropriate channel models to use. Merged in comment by Pratik Mehta. Merged in changes by Tim Wakely (addition of printing) | 22 August 2003 | Adrian Stephens |
| R5 draft 2 | Added comments to coexistence section to action changes requested in previous telecon. | 22 August 2003 | Adrian Stephens |
| R5 | Added new section – a sample simulation scenario for discussion | 22 August 2003 | Adrian Stephens |
| R6 | Actioned comments from: - August 26th Telecon - George Vlantis email of 29/08/2003 - Eldad Perahia email of 29/08/2003 (Scenario 4) - Hervé Bonneville email of 28/08/2003 (submitted as 11-03-0696r0) - Rahul Malik email of 3/9/2003 | 5 September 2003 | Adrian Stephens |
| R7 | Some additional scenarios added (George & Adrian) | 8 September 2003 | Adrian Stephens |
| R8 | Clarified meaning of Mean Rate / UDP/TCP using text supplied by John Ketchum Scenarios 6 and 7 contributed by Rahul Malik. | 13 September 2003 | Adrian Stephens |
| R9 | Edited at meeting held during September 2003 interim – Tuesday pm. | 16 September 2003 | Adrian Stephens |
| R9 | Edited at meeting held during September 2003 interim – Wednesday am | 17 September 2003 | Adrian Stephens |
| R10 | Edited at meeting held during September 2003 interim, Wednesday pm | 17 September 2003 | Adrian Stephens |
| R11 | Edited at meeting held during September 2003 interim – Thursday am | 18 September 2003 | Adrian Stephens |

# Introduction

To support the definition of a higher throughput WLAN standard (which will incorporate changes to both the MAC and the PHY) within the IEEE (to be published eventually as the 802.11n amendment), this document attempts to define usage models based on various market-based use-cases. The usage models are intended to support the definitions of network simulations that will allow 802.11 TGn to evaluate the performance of various proposals in terms of, for example,  network throughput, delay, packet loss and other metrics. It is anticipated that the outputs of this document will aid in the subsequent development of the evaluation and selection criteria used by TGn.

Note - These usage models that the usage model committee develops here are subject to the following constraints :

C1:	They are relevant to the expected uses of the technology

C2:	They require higher throughput than can be achieved with existing 802.11 technology

C3:	They are capable of being turned into an unambiguous simulation scenario

# Process going forward

The 802.11 TGn Functional Requirements and Comparison Criteria (FRCC) special committee have been given responsibility for maintaining this document.

The simulation scenarios need to be validated through an implementation.

# Definitions

This section defines some of the terms used in this document.

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video. VOIP.

Environment – The type of place a WLAN system is deployed in.  Initial examples:  home, large office.

Use case – A use case is a description of how an end user uses a system that exercises that system’s deployment of WLAN.  A use case includes an application in a deployment environment with details regarding the user activity and both sides of the link.

Examples: Watching television remote from the cable or set-top box within the home.  Talking on the telephone remote from one’s desk at work.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage models are created to "cover" use cases.

Simulation Scenario – A simulation scenario is a description of a usage model that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage model to the simulation scenario: environment linked to a channel model, position of the AP (console or ceiling mounted), position of STAs w.r.t. AP, uplink and downlink traffic (# packets, size of packets, interference (number and types of users on the same WLAN channel – adjacent cells, the same cell, number and types of users on alternate channels, BT, baby monitors, GPRS or other systems).  A simulation scenario is created from a Usage Model by characterizing the traffic profile of the applications and possibly merging multiple applications together to reduce simulation time.

# Mappings between Application, Environment, Channel Model, Use case, Usage Model and Simulation Scenario

Understanding and defining the application, environment, channel model, use case, usage model and simulation scenario are all necessary to create comparative results from 802.11 TGn proposals.

Channel models have been defined in [5], with 6 channel models.  Each environment will map to a pair of channel models..

Each use case involves the use of one or more applications and is defined for one or more environments.  It represents a single type of use of a system using the technology.

Each application reflects a source or sink of data.   They will eventually be characterized in terms of a traffic profile that allows a simulation of the application to be created.

Each usage model contains a representative mixture of applications and channel models designed to adequately cover the important use cases.  There is a many to many mapping between use cases and usage models (i.e., the same use case may contribute to multiple usage models and the same usage model may include applications from multiple use cases).

There will be a one-to-one mapping between usage models and simulation scenarios.   The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs not present in the usage model.

# Environments

The channel models identified in [5] are described in Table 1.

Table 1 - Environment to Channel Model Mapping

| Model | Environment |
| --- | --- |
| A | Flat fading (no multipath) |
| B | Residential |
| C | Residential / Small Office |
| D | Typical Office |
| E | Large Office |
| F | Large Space (indoors / outdoors) |

- Use of Channel Models by Simulation Scenarios

The list of environments being considered is shown in Table 3.  This list is here to allow this documens to relate an environment to a channel model. .

Table 3 - Environment Definitions

| Environment | Includes | Applicable Channel Model |
| --- | --- | --- |
| Residential, Domestic or Home | Intra-room Room to room Indoor to outdoor Large multi-family dwelling. Note: one or more PCs in the home may be notebooks or other portable devices that come home with the user. these wireless devices may have more than one wireless technology included. | B |
| House to house | One main house has AP with uplink connection, Another house holds single or multiple STA(s), Guest house, garage or studio. In garage model, STA may be embed inside a car. | F |
| Small Enterprise | Enclosed offices Meeting room / conference room Classroom | C |
| Medium/Large Enterprise | Enclosed offices Meeting room / conference room Classroom Sea of cubes Multi-story office environment Campus | E |
| Hotspot | Airport Library Convention center Hotel Shopping mall Arcade Train station / bus terminal Drive-in window | F |
| Outdoor | Outdoor sport event Campus City Square Public park Amusement park | F |
| Industrial | Indoor Large factory floor Hospital Warehouse Concert hall / auditorium Movie theatre | F |
| Other custom environments | Wireless backhaul Fixed wireless access: outside to multiple STA inside outside to multiple STA outside | F |
| Mobile | Train Bus Plane Roadside APs for data-service in-car (fast roaming) | C |

# Applications

Table 5 lists the applications that are referred to from the usage models, together with relevant traffic parameters.

The parameters used to define the application are defined in Table 4.

Table 4 - Application Parameter Definitions

| Parameter | Definition |
| --- | --- |
| MSDU size | Packet size at the top of the MAC |
| Maximum PLR | Maximum packet loss rate at the top of the MAC. This is defined by the loss rate that can be tolerated by the application. |
| Maximum Delay | Maximum transport delay at the top of the MAC – i.e. between matching MA-UNITDATA.request and .indication. |
| Protocol | Indicates the network-layer protocol running between the data source and the MAC. It takes one of two values: TCP or UDP. These two protocols are intended to represent a generic acknowledged and a generic unacknowledged network-layer protocol. |
| Offered Load | This parameter is interpreted differently according to the Protocol parameter: Applications identified as being carried by UDP are assumed to generate MSDUs at a fixed rate, as identified in the "Offered load" column. Inability to carry the traffic generated by a UDP application, due to insufficient throughput capability, results in lost MSDUs, which is reported in simulation results as a packet loss rate, or an outage, associated with the application. The comparison criteria include a measure of whether this packet loss rate exceeds the maximum specified for the application in this table. Traffic carried by TCP is assumed to be served on a best-effort basis, and applications using TCP are assumed to generate MSDUs at rates up to the value given in the "Offered load" column. Being an acknowledged protocol with a constrained window size, TCP responds to congestion in the BSS by reducing application throughput without losing MSDUs. This effect is reflected in simulation results by reporting achieved throughput for applications using TCP. Note: Acknowledgement traffic is generated by TCP sinks, this is not explicitly specified in the simulation scenarios, but it is included in the count of non-QoS flows and measurement of goodput. |

The delay and PLR requirements for each application are specified in Table 4. Simulations may map applications and flows to specific QoS classes as necessary to satisfy these requirements. Proposers shall clearly state how applications and flows are mapped to specific QoS classes in their simulations.

Table 5 - Application Definitions

| Number | Application | Offered Load (Mbps) | Protocol | MSDU Size (B) | Maximum PLR | Maximum Delay (ms) | Source [ref] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DV Audio/video | 28.8 | UDP | 1024, Is this true? All other video/audio (SDTV, HDTV) transfers has this parameter set to 1500. | 10^-7 (corresponds to a rate of 0.5 loss/hour) | 200 | SD Specifications of Consumer-Use Digital VCRs Max PLR: 15-03-276r0 |
| 2 | VoD control channel | 0.06 | UDP | 64 | 10^-2 | 100 | Guess |
| 3 | SDTV | 4-5 | UDP | 1500 | 5*10^-7 | 200 | 1 |
| 4 | HDTV (Video/Audio) | 19.2-24 | UDP | 1500 | 10^-7 | 200 | 1 |
| 5 | DVD | 9.8 peak | UDP | 1500 | 10^-7 | 200 | 1 |
| 6 | Video Conf | 0.128 - 2 | UDP | 512 | 10^-4 | 100 | 1 |
| 7 | Internet Streaming video/audio | 0.1 – 4 | UDP | 512 | 10^-4 | 200 | 1 |
| 8 | Internet Streaming audio | 0.064~0.256 | UDP | 418 | 10^-4 | 200 | Group guess |
| 9 | VoIP | 0.096 | UDP | 120 | 5% | 30 | ITU-T G.114 300ms round-trip delay G.711 Codec |
| 10 | Reserved |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |
| 12 | MP3 Audio Other formats are taking over (AAC/MPEG-4, OggVorbis, etc) | 0.064 – 0.32 | UDP | 418 | 10^-4 | 200 | 1 |
| 12.5 | Reserved |  |  |  |  |  |  |
| 13 | Content download (photo camera) | 11 | TCP | 1500 | n/a |  | Corresponds to USB and flash speed |
| 14 | Internet File transfer (email, web, chat) | 1 | TCP | 300 | n/a |  |  |
| 15 | Local File transfer, printing | 30 | TCP | 1500 | n/a |  | Aps guess |
| 16 | Interactive Gaming [Controller to Console x 1] | 0.5 | UDP | 50 | 10^-4 | 16 | 2 |
| 17 | Interactive Gaming [Console to Display] | 100+ | UDP | 1500 | 10^-2 | 10 | 2 video image: 320x240x15 @ 60Hz |
| 18 | Interactive Gaming [Console to Internet Access] *NOTE : Depends on Game Type | 1 | UDP | 512 | 10^-4 | 50ms | Group consensus |
| 19 | Netmeeting application/desktop sharing | 0.5 | TCP | 512 | n/a |  | Group guess |
| 20 | Reserved |  |  |  |  |  |  |
| 21 | Reserved |  |  |  |  |  |  |
| 22 | Reserved |  |  |  |  |  |  |
| 23 | Video phone | 0.5 | UDP | 512 | 10^-2 | 100 | Aps guess |
| 24 | Remote user interface (X11, Terminal Server Client) (remote display/keyboard/mouse) | 0.5-1.5 (peak) | UDP | 700 | n/a | 100 | 11-03-0696r0 |

# Use Cases

Table 6 contains a definition of the use cases used in this document.

The score relates to the results reported in [4] from the vote on 21 July 2003.  This scores 3 for high, 2 for medium and 1 for low priority.  The "Devn" column shows the weighted absolute deviation in the votes (0 shows complete agreement and 1 shows complete disagreement).

Table 6 - Use Case Definitions

| Number | Covered by model # | Use case | Application | Environment | Score | Devn. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1, 3, 4, 5, 6, 7 | One personal phone everywhere – home, office. Each person has a phone that works everywhere, home, office – same number. An extension of the cell phone into the office building. This includes cordless phone over VoIP. | VOIP integrated with other wireless WAN technologies | Residential, Enterprise – large and small, conference room | 2.12 | 0.84 |
| 2 | 1 | Multiplayer Internet gaming anywhere within the home / Internet Café. | Interactive gaming (console to internet), internet gaming (controller to console) | Residential/small enterprise (internet cafes) | 1.69 | 0.77 |
| 3 | 1 | Multiple TVs running throughout the home getting their content from a single remotely located AV-server/AP/set top box. Local control of the content (changing channels, etc). | HDTV, SDTV, VoD control channel | Residential | 2.87 | 0.23 |
| 4 | 2 | Link the home digital camera/video to the TV/display for display of pictures and movies taken. | DV Audio/Video | Residential | 2.59 | 0.55 |
| 5 | 6 (HDTV not covered) | Watch a movie of your choice, when you want it, it your hotel room. | Internet streaming audio/video, SDTV, HDTV | Hotspot | 1.11 | 0.19 |
| 6 | 7 | Watch a clear replay of an event from your seat in a sporting arena. | Internet Streaming Video | Outdoor | 1.51 | 0.50 |
| 7 | 1 | Remotely located security cameras transmitting video signal to a monitoring location. | SDTV | Outside/Inside Residential, Small office building (not covered) | 1.55 | 0.55 |
| 8 | 1 | Music real time on multiple receivers throughout the home from a remotely located AV-server/AP/set top box receiver. | PCM Audio, MP3 Audio | Residential | 2.65 | 0.49 |
| 9 | 5 | Net meeting in a conference/class room to share someone’s display. 30 participants/students | Netmeeting application/desktop sharing | Conference room/class room |  | 0.53 |
| 10 | 1, 4 (hotel not covered) | Reconfigurable / temporary office space, Ethernet cable replacement (similar throughput to wired cable). Back up files, email, web surfing, printing, etc. | Local File Transfer, printing | Enterprise – sea of cubes, home, hotel | 2.50 | 0.70 |
| 11 | 1, 2 | Download video, music and other data files to a device in an automobile in the home garage or driveway. Broadband file transfer – at HT rates. | Internet File Transfer | Residential / Outdoor | 1.78 | 0.68 |
| 12 | 1, 2 | Backup/transfer files between PCs located throughout the home, printing. Access point router. | Local File Transfer | Residential, Residential IBSS. | 2.06 | 0.63 |
| 13 | 6, 7 (Airplane & train not covered) | Synchronize your local device with the server – email, calendar, etc. Hot spot/airport/airplane | Internet File Transfer | Large open area – hot spot, airport, train station, bus terminal. Airplane, Train | 2.40 | 0.69 |
| 14 | 2 | Download digital pictures and home movies to a PC/AV-server | Content download | Residential – same room, Residential IBSS. | 2.34 | 0.71 |
| 15 | 2 | Exchange files between PCs or between CE devices – ad hoc (no access point). Using IBSS mode of operation. | Local File Transfer | Residential IBSS | 1.60 | 0.65 |
| 16 | Not covered | Update inventory from the warehouse and the retail floor. | Local File Transfer | Industrial | 1.24 | 0.38 |
| 18 | 5 | Access of networked software from the classroom. 30 participants, simultaneously signing on. | Local File Transfer | Conference room/class room | 1.76 | 0.73 |
| 19 | Not covered | Update/view medical records from patient rooms. | Local File Transfer | Industrial. | 1.53 | 0.66 |
| 20 | 7 | Obtain real time interactive player and game stats from your seat at a sporting event. | Internet File Transfer | Outdoor | 1.07 | 0.12 |
| 20.5 | 15 | View broadcast SDTV video/audio at a sporting event | SDTV | Outdoor / Arena | 2 | 0 |
| 20.6 | Not covered | View Video on demand at a sporting event / concert-hall /hotspot | SDTV | Outdoor / Arena | 2 | 0.36 |
| 21 | 1, 2 (train station not specifically covered) | Interactive multi-person gaming – ad hoc. | Interactive gaming | Home, train station | 1.39 | 0.62 |
| 22 | 14 | Point-point link for wireless backhaul | Backhaul traffic | Outdoor. | 2.00 | 0.73 |
| 23 | 8 (outdoor only covered) | Point-multipoint link for wireless backhaul | Backhaul traffic. | Arena? Hotspot? Could be both outdoor and indoor | 2.46 | 0.61 |
| 24 | 8 | Point-multipoint link for Fixed Wireless Access | FWA traffic | Outdoor | 2.32 | 0.65 |
| 25 | 9 | Mixed mode AP has legacy and HT STA | Legacy: file transfer HT: File transfer + SDTV | Large Enterprise | 2.89 | 0.20 |
| 26 | 11 | Co-channel legacy BSS interference | File transfer + SDTV | Small Enterprise | 2.23 | 0.65 |
| 27 | Does not need a usage model | Legacy mode operation in legacy BSS | Internet/Local File transfer | Small Enterprise | Not voted |  |
| 28 | Not covered | Real-time streaming of ultrasound video and real-time viewing of x-ray/MRI/CT images as well as medical diagnostics signal streams / patient monitoring data | SDTV, local file transfer | Hospital (Industrial) similar to Large Enterprise (?) | 1.74 | 0.56 |
| 29 | 1, 3, 4 | Online distance learning/broadcasting locally | Internet File Transfer, Internet Streaming Audio/Video | Residential, small/large enterprises | 1.75 | 0.63 |
| 30 | 5 | Video conferencing with headset | Internet Streaming video/audio + headset interference | Small Enterprise | 1.08 | 0.15 |
| 31 | 3, 4 | Enterprise high stress. Surfing the web, e-mail, printing, file transfers within the intranet. | Internet File Transfer. Printing. Local File Transfer | Small/Large Enterprise | 2.45 | 0.5 |
| 32 | 1 | Portable /Internet AV Devices. MP3 or other player playing music directly from an internet through a residential gateway. | Internet Streaming Audio | Residential | 2.06 | 0.33 |
| 33 | 1, 2, 4 | AV Communication Video Phone: Peer to peer AV communication. Video Conferencing: AV conference between multiple devices | Internet Streaming Video/Audio (multicast/broadcast) | Residential, Small/large Enterprise | 2.31 | 0.64 |
| 34 | 2 | Ad-hoc mode example | Local File Transfer | Residential IBSS | 1.74 | 0.72 |
| 36 | 5 | Enterprise conference room – 20 to 30 users | Local file transfer, internet file transfer, printing | Enterprise | 2 | 0.29 |
| 37 | Not covered | Lightweight terminal wirelessly connected to a remote computer | Remote user interface | Residential, Industrial, Enterprise | 1.8 | 0.43 |

# Usage Models

Table 7 defines the usage models defined by this document.

The purpose of these models is to merge representative use cases to create a small number of credible worst-case mixtures of applications.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities and capable of implementation in proposed 802.11n technology.

The number of usage models needs to be limited because each usage model adds simulation time to the preparation of results for submission against the 802.11n comparison criteria.

Table 7 - Usage Model Definitions

| Model # | Usage Model | Covers Use Cases | Score (ave/devn) | Application mix | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Residential | 3, 8, 10, 33, 1, 32, 11, 29, 7, 21 | 2.8/0.3 | 1 x AP STA 1: 19.2 Mbps HDTV (A4) , VoD control channel (A2) STA 3: 24 Mbps HDTV (A4) , VoD control channel (A2) STA 4: SDTV (A3), internet file transfer (A14), local file transfer (A15) STA 5 & 6: Video Phone (A23) STA 7,8 & 9: VoIP (A9) STA 10: Internet streaming video (A7), MP3 Audio (A12), Video gaming, console to internet (A18), local file transfer (A15) STA 11: Video gaming, controller to console (A16) | This scenario should be room to room or indoor/outdoor. The exact spatial distribution and mobility as well as the desired number of simultaneous connections can be referred to [2]. 20m range. |
| 2 | Residential IBSS | 4, 33, 11, 34, 15, 21 | 2.3/0.6 | 0 AP STA 1: peer-peer DV Audio/Video (A1) sink STA 2,3: local file transfer (A15) sink, local file transfer (A15) source STA 4,5,6,7: Video Phone (A23) sink, Video Phone (A23) source STA 8: 4 x Video gaming, controller to console (A16), sink STA 9: content download (A13) sink STA 10: peer-peer DV Audio/Video (A1) (rate, range combination > current technology) source STA 11,12,13,14: Video gaming, controller to console (A16) source STA 15: content download (A13) source | Note, all these devices are operating on the same channel. |
| 4 | Large Enterprise | 10, 31, 33, 1, 29 | 2.7/0.4 | 1 AP. 6 STA (1-6): internet file transfer (A14), 2 STA (7-8): Video Conferencing (A6) 2 STA (9-10): internet streaming video/audio (A7), 14 STA (11-24): local file transfer (A15) (10 downlink (STA 11-20), 4 uplink (STA 21-24)) 6 STA (25-30): VoIP (A9) | Minimum 15 – 20 meter range |
| 5 | Conference room | 9, 36, 1, 18, 30 | 2.4/0.5 | 1 AP 30 STA (1-30): netmeeting, desktop sharing (A19), Local File Transfer (A15) 10 STA (31-40): VoIP (A9) 1 STA: SDTV (A3) | LOS small enterprise 15m |
| 6 | Hot spot | 1, 13, 5 | 2.8/0.4 | 1 AP 10 STA: Internet file transfer (A14) 7 STA: internet streaming video/audio (A7) 2 STA: SDTV (A3) 15 STA: VoIP (A9) | Indoors: Large Enterprise channel model In hot spot area, most of traffic goes through internet, and the duration of the session is limited (less than 2 hours). |
| 9 | Mixed-mode (containing Legacy + HT STAs) BSS | 1 | 2.5/0.6 | 1 HT AP legacy STA 1-3 associated with the HT AP: internet file transfer (A14) 1 HT STA (STA4): SDTV (A3) 2 HT STA (STA5,6): internet file transfer (A14) Note, legacy system means 802.11a in the 5GHz bands and .11g in the 2.4GHz band. | Will apply to large enterprise 20m range |
| 11 | Co-channel legacy BSS | 26 | 2.6/0.6 | 1 legacy AP with 6 legacy STA. 1 HT AP and 6 STA. Co-channel with legacy AP. All STA doing internet file transfer (A14). 2 HT STA: internet streaming audio (A8). 1 HT STA: SDTV (A3) Note, the BSSs are overlapping, without any STA/AP being exactly on top of another STA/AP. | Will apply to large enterprise 20m range |
| 16 | Point-point link rate test | CC 27,28,58 and FR |  | One HT AP and one STA. The AP has a data source that is a synthetic application intended to ensure that it always has data to send. This is a UDP application (to avoid any effects from TCP protocol) with an MSDU size of 1500B and a mean rate of 400Mbps. | Residential and typical office environments over the range 0 to 200m. |
| 17 | Point-point goodput test | CC15 |  | One HT AP and one HT STA operating in 20MHz channel. The STA has a synthetic data source providing 100Mbps offered load using UDP protocol and an MSDU size of 1500B. | B-NLOS 10m range |
| 18 | Point-point legacy throughput test | CC15 |  | One HT AP and one legacy STA operating in 20MHz channel. The STA has a synthetic data source providing 100Mbps offered load using UDP protocol and an MSDU size of 1500B. | B-NLOS 10m range |
| 19 | Point-point legacy sharing throughput test | CC15 |  | One HT AP, one HT STA and one legacy STA operating in 20MHz channel. Each STA has a synthetic data source providing 100Mbps offered load using UDP protocol and an MSDU size of 1500B. | B-NLOS 10m range |

Usage models 3,8,7,10,13,14 and 15 are reserved (they were defined in an earlier revision of this document, but have now been deleted);

## Coexistence (Informative)

802.11 TGn will liaise with 802.19 TAG regarding coexistence requirements.

# Simulation Scenarios

## Common Conditions

Table 1 defines conditions that are common to all simulation scenarios.

Table 9 - Common Simulation Conditions

| Condition | Description |
| --- | --- |
| Shadowing | In generating a channel realization, the shadowing term of the channel model shall be set to 0dB. |
| Channel Model Breakpoint | When creating a channel realization between two STA, the distance between the STA is used to select between LOS and NLOS models according to the breakpoint distance defined in the channel model [5]. |

## Scenario 1

Use channel model B for this scenario.

| STA Name: AP |  | Role: Access Point |  |  | Location: 0, 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1 | 19.2 Mbps |  | Constant, UDP | 1500 |  | 200 |  | HDTV |
| STA3 | 24 Mbps |  | Constant, UDP | 1500 |  | 200 |  | HDTV |
| STA4 | 4 Mbps |  | Constant, UDP | 1500 |  | 200 |  | SDTV |
| STA4 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA7 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA8 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA9 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA10 | 2 Mbps |  | UDP | 512 |  | 200 |  | Internet Streaming video |
| STA 11 | 0.128 Mbps |  | UDP | 418 |  | 200 |  | MP3 audio |

| STA Name: STA1 |  | Role: HTDV Display |  |  | Location: 0, 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 60kbps |  | Constant, UDP | 64 |  | 100 |  | VoD control channel |

Note, STA2 name reserved.

| STA Name: STA3 |  | Role: HDTV Display |  |  | Location: 5, 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 60kbps |  | Constant, UDP | 64 |  | 100 |  | VoD control channel |

| STA Name: STA4 |  | Role: SDTV Display, Gaming & Printing |  |  | Location: -7, -7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA10 | 30 Mbps |  | Constant, TCP | 1500 |  |  |  | Local file transfer |

| STA Name: STA5 |  | Role: Video Phone |  |  | Location: -15,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA6 | 0.5 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video Phone |

| STA Name: STA6 |  | Role: Video Phone + internet upload |  |  | Location: 0,-15 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA5 | 0.5 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video Phone |

| STA Name: STA7 |  | Role: VoIP Phone |  |  | Location: 20, 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA8 |  | Role: VoIP Phone |  |  | Location: 0, 20 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA9 |  | Role: VoIP Phone |  |  | Location: 0, -20 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA10 |  | Role: Video Console + Internet Entertainment |  |  | Location: 10,10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 1 Mbps |  | Constant, UDP | 512 |  | 50ms |  | Console to Internet |

| STA Name: STA11 |  | Role: Video Gaming Controller |  |  | Location: 10, 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA10 | 0.5 Mbps |  | Constant, UDP | 50 |  | 16 |  | Controller to Console |

## Scenario 2 (Residential IBSS)

Use channel model B for this scenario.

| STA Name: STA1 |  | Role: DV Audio/Video sink |  |  | Location: 4, -4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| Note: this STA is a sink only, no applications generate data from this STA. |  |  |  |  |  |  |  |  |

| STA Name: STA2 |  | Role: Local File Transfer |  |  | Location: 5, 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA3 | 30Mbps |  | TCP | 1500 |  |  |  | Local file transfer |

| STA Name: STA3 |  | Role: Local file transfer |  |  | Location: -5, 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this STA is a sink only, no applications generate data from this STA. |  |  |  |  |  |  |  |  |

| STA Name: STA4 |  | Role: Video Phone |  |  | Location: 3, 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA5 | 0.5 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video Phone |

| STA Name: STA5 |  | Role: Video Phone |  |  | Location: 0, 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA4 | 0.5 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video Phone |

| STA Name: STA6 |  | Role: Video Phone |  |  | Location: -2, 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA7 | 0.5 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video Phone |

| STA Name: STA7 |  | Role: Video Phone |  |  | Location: -5, 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA6 | 0.5 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video Phone |
| The above four tables assume the (two-way) combination between stations 4-5 and stations 6-7. Note that this is only one out of the three possible groupings. |  |  |  |  |  |  |  |  |

| STA Name: STA8 |  | Role: Video Gaming Controller |  |  | Location: 1, 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this STA is a sink only. |  |  |  |  |  |  |  |  |

| STA Name: STA9 |  | Role: Content Download - sink |  |  | Location: 2, -5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this STA is a sink only. |  |  |  |  |  |  |  |  |

| STA Name: STA10 |  | Role: Content Download - sink |  |  | Location: -2, -5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1 | 28.8 Mbps |  | Constant, UDP | 1024 |  | 200 |  | Peer-2-peer DV audio/video |

| STA Name: STA11 |  | Role: Video Gaming Controller |  |  | Location: 0, -3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA8 | 0.5 Mbps |  | Constant, UDP | 50 |  | 16 |  | Controller to Console |

| STA Name: STA12 |  | Role: Video Gaming Controller |  |  | Location: 1, -2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA8 | 0.5 Mbps |  | Constant, UDP | 50 |  | 16 |  | Controller to Console |

| STA Name: STA13 |  | Role: Video Gaming Controller |  |  | Location: 1, -3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA8 | 0.5 Mbps |  | Constant, UDP | 50 |  | 16 |  | Controller to Console |

| STA Name: STA14 |  | Role: Video Gaming Controller |  |  | Location: 0, -2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA8 | 0.5 Mbps |  | Constant, UDP | 50 |  | 16 |  | Controller to Console |

| STA Name: STA15 |  | Role: Content Download - source |  |  | Location: 0, -4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA9 | 11Mbps |  | TCP | 1500 |  |  |  | Content Download |

## Scenario 4 (Large Enterprise)

Use Channel model D for this scenario.

The stations are contained in a +/-10 by +/-10 grid.

Note, items marked "**" are not strictly specified by the Usage Model, but are a reasonable extrapolation intended to achieve a range of offered loads.

| STA Name: AP |  | Role: Access Point |  |  | Location: 0, 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA2 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA3 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA4 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA5 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA6 | 10 Mbps. |  | TCP | 300 |  |  |  | Internet file, downloading large email attachments ** |
| STA7 | 1 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video conferencing |
| STA8 | 1 Mbps |  | Constant, UDP | 512 |  | 100 |  | Video conferencing |
| STA9 | 2 Mbps |  | UDP | 512 |  | 200 |  | Internet Streaming video + MP3 audio |
| STA10 | 2 Mbps |  | UDP | 512 |  | 200 |  | Internet Streaming video + MP3 audio |
| STA11 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA12 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA13 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA14 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA15 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA16 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA17 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA18 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA19 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA20 | 30 Mbps |  | TCP | 1500 |  |  |  | Local File transfer |
| STA25 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA26 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA27 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA28 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA29 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA30 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |

| STA Name: STA1 |  | Role: |  |  | Location: 5, -9.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 0.256 Mbps |  | TCP | 64 |  |  |  | ** Clicking on web link? |

| STA Name: STA2 |  | Role: |  |  | Location: 3.5, 7.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 0.256 Mbps |  | TCP | 64 |  |  |  | ** Clicking on web link? |

| STA Name: STA3 |  | Role: |  |  | Location: 7.5, -9.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | .256 Mbps |  | TCP | 64 |  |  |  | ** Clicking on web link? |

| STA Name: STA4 |  | Role: Internet file upload |  |  | Location: -4.5, 0.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 5Mbps |  | TCP | 1000 |  |  |  | ** Internet file transfer in the uplink direction. |

| STA Name: STA5 |  | Role: Internet file upload |  |  | Location: -1.5, 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 10Mbps |  | TCP | 1500 |  |  |  | Uploading large email attachments |

| STA Name: STA6 |  | Role: |  |  | Location: -5.5, 4.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | Delay / Jitter limit |  | Notes |
| AP1 | 0.256 Mbps |  | TCP | 64 |  |  |  | Clicking on web link? |

| STA Name: STA7 |  | Role: Video Conferencing |  |  | Location: -9, -5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 1Mbps |  | Constant, UDP | 512 |  | 100 |  |  |

| STA Name: STA8 |  | Role: Video Conferencing |  |  | Location: -8.5, 8.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 1Mbps |  | Constant, UDP | 512 |  | 100 |  |  |

| STA Name: STA9 |  | Role: Media player |  |  | Location: 7, -7.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |

| STA Name: STA10 |  | Role: Media player |  |  | Location: -3, 0.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA11 |  | Role: |  |  | Location: -0.5,8 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA12 |  | Role: |  |  | Location: 7,7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA13 |  | Role: |  |  | Location: -4,-4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA14 |  | Role: |  |  | Location: 7.5,-1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA15 |  | Role: |  |  | Location: 3,-0.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA16 |  | Role: |  |  | Location: 8,-6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA17 |  | Role: |  |  | Location: 0,-7.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA18 |  | Role: |  |  | Location: 10,0.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA19 |  | Role: |  |  | Location: -2.5,-4.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA20 |  | Role: |  |  | Location: 0.5,-2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources– none. This STA is a sink only. |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 |  |  |  |  |  |  |  |  |

| STA Name: STA21 |  | Role: local file transfer source |  |  | Location: -6.5, -3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA22 |  | Role: local file transfer source |  |  | Location: 0, -4.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA23 |  | Role: local file transfer source |  |  | Location: -1.5, 7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA24 |  | Role: local file transfer source |  |  | Location: 3,2.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  |  |

| STA Name: STA25 |  | Role: VoIP Phone |  |  | Location: 3.5, -5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA26 |  | Role: VoIP Phone |  |  | Location: 9,9.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA27 |  | Role: VoIP Phone |  |  | Location: -6,2.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA28 |  | Role: VoIP Phone |  |  | Location: -8,-5.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA29 |  | Role: VoIP Phone |  |  | Location: 1.5,3.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA30 |  | Role: VoIP Phone |  |  | Location: 9.5,3.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP Phone |

## Scenario 5 (Conference Room)

Use channel model C for this scenario.

| STA Name: AP1 |  | Role: AP |  |  | Location: 0,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA 1-30 (replicated) | 0.5 Mbps |  | TCP | 512 |  |  |  | Netmeeting |
| STA 1-20 (replicated) | 30Mbps |  | TCP | 1500 |  |  |  | Local File Transfer |
| STA 31-40 (replicated) | 0.096 |  | UDP | 120 |  | 30 |  | VoIP Phone |
| STA 41 | 5Mbps |  | UDP | 1500 |  | 200 |  | STDV |

| STA Name: STA1 – STA20 |  | Role: laptop (local file download) |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA21 – STA30 |  | Role: laptop (local file upload) |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 30Mbps |  | TCP | 1500 |  |  |  | Local File Transfer |

| STA Name: STA31 - STA40 |  | Role: VoIP phone |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| VoIP Phone | 0.096 |  | UDP | 120 |  | 30 |  | VoIP Phone |

| STA Name: STA41 |  | Role: SDTV display |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – none, this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

Table 10 - STA locations for SS5

| STA | X | Y |
| --- | --- | --- |
| 1 | -3 | -3 |
| 2 | 5 | 10 |
| 3 | -4 | -2 |
| 4 | 3 | 2 |
| 5 | 6 | 0 |
| 6 | 8 | 0 |
| 7 | -9 | 6 |
| 8 | 14 | 9 |
| 9 | 6 | 7 |
| 10 | -12 | 5 |
| 11 | -4 | -11 |
| 12 | 2 | 10 |
| 13 | 5 | 15 |
| 14 | 14 | -13 |
| 15 | -4 | 1 |
| 16 | -7 | 3 |
| 17 | -14 | 2 |
| 18 | 6 | 14 |
| 19 | 8 | 7 |
| 20 | -2 | 4 |
| 21 | 9 | -12 |
| 22 | 13 | 12 |
| 23 | 3 | -7 |
| 24 | 11 | 0 |
| 25 | 7 | -2 |
| 26 | 14 | -13 |
| 27 | 2 | -6 |
| 28 | 11 | -5 |
| 29 | 5 | -13 |
| 30 | -4 | 0 |
| 31 | -2 | 2 |
| 32 | 3 | -12 |
| 33 | 12 | 8 |
| 34 | 9 | 9 |
| 35 | 5 | -9 |
| 36 | -7 | 4 |
| 37 | 1 | -13 |
| 38 | -12 | -7 |
| 39 | -3 | -1 |
| 40 | 12 | 3 |
| 41 | -5 | -1 |

## Scenario 6: Hot spot

Use channel model E for this scenario.

| STA Name: AP1 |  | Role: AP |  |  | Location: 0,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1-10 (replicated) | 2 |  | TCP | 300 |  |  |  | Internet File Transfer |
| STA11-14 (replicated) | 2 |  | UDP | 512 |  | 200ms |  | Mid quality streaming audio/video |
| STA15-17 (replicated) | 8 |  | UDP | 512 |  | 200ms |  | High quality streaming audio/video |
| STA18-19 (replicated) | 5 |  | UDP | 1500 |  | 200ms |  | SDTV broadcast |
| STA20-34 (replicated) | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |

| STA Name: STA20 – STA34 |  | Role: VoIP Phone |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |

Note, the other STA are sinks only.

Locations (m)

Table 11 - STA locations  for SS6

| STA | X | Y |
| --- | --- | --- |
| 1 | 21 | 23 |
| 2 | 21 | -16 |
| 3 | 13 | 1 |
| 4 | 7 | 6 |
| 5 | 21 | 20 |
| 6 | -1 | -5 |
| 7 | -12 | 22 |
| 8 | -11 | -5 |
| 9 | -15 | 6 |
| 10 | -23 | -2 |
| 11 | 13 | 19 |
| 12 | 13 | -19 |
| 13 | -16 | 16 |
| 14 | -18 | -17 |
| 15 | 16 | 2 |
| 16 | 3 | -10 |
| 17 | -5 | 15 |
| 18 | 4 | -1 |
| 19 | -6 | 0 |
| 20 | 21 | 22 |
| 21 | 19 | -11 |
| 22 | 15 | 10 |
| 23 | 9 | -5 |
| 24 | 4 | 12 |
| 25 | -2 | -22 |
| 26 | -8 | 18 |
| 27 | -14 | -18 |
| 28 | -17 | -8 |
| 29 | -23 | 10 |
| 30 | 21 | 0 |
| 31 | 19 | -22 |
| 32 | 8 | 21 |
| 33 | 7 | -14 |
| 34 | 2 | 3 |

## Scenario 9 (Mixed-Mode BSS)

Use channel model D for this scenario

Note, legacy STA means .11g in the 2.4GHz band and .11a in the 5GHz band as appropriate.

| STA Name: AP1 |  | Role: HT Mixed-mode AP |  |  | Location: 0,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1 | 1 |  | TCP | 300 |  |  |  | Internet File Transfer |
| STA2 | 1 |  | TCP | 300 |  |  |  | Internet File Transfer |
| STA3 | 1 |  | TCP | 300 |  |  |  | Internet File Transfer |
| STA4 | 5 |  | UDP | 1500 |  | 200 |  | SDTV |
| STA5 | 1 |  | TCP | 300 |  |  |  | Internet File Transfer |

| STA Name: STA1 |  | Role: Legacy STA |  |  | Location: -9,-12 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – note this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA2 |  | Role: Legacy STA |  |  | Location: -19,10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – note this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA3 |  | Role: Legacy STA |  |  | Location: -2,17 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – note this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA4 |  | Role: HT SDTV display |  |  | Location: -1,-3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – note this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA5 |  | Role: HT STA |  |  | Location: 14,1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – note this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA6 |  | Role: HT STA |  |  | Location: -12,7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 1 Mbps |  | TCP | 300 |  |  |  | Internet File transfer |

## Scenario 11 (Co-channel legacy BSS)

Use channel model C for this scenario.

Table 12 - STA locations for SS11

| STA | X(m) | Y(m) |
| --- | --- | --- |
| 1 | -6 | 1 |
| 2 | 9 | -8 |
| 3 | 14 | 3 |
| 4 | -5 | 8 |
| 5 | 2 | -2 |
| 6 | 8 | 5 |
| 7 | 12 | 18 |
| 8 | 1 | 15 |
| 9 | -13 | 19 |
| 10 | -9 | -10 |
| 11 | 15 | 9 |
| 12 | -15 | -20 |

Note, the two APs are operating on the same channel (or overlapping channels if the channel width is not the same).

| STA Name: AP1 |  | Role: Legacy AP |  |  | Location: 0,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1 – STA4 replicated | 1Mbps |  | TCP | 300 |  |  |  | Internet file transfer |

| STA Name: STA1 – STA4 |  | Role: Legacy STA (download) |  |  | Location: See Table |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – none, these STA act as sinks only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA5 – STA6 |  | Role: Legacy STA (upload) |  |  | Location: See Table |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 1Mbps |  | TCP | 300 |  |  |  | Internet file transfer |

| STA Name: AP2 |  | Role: HT AP |  |  | Location: 5,5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA7 – STA10 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file transfer |
| STA7 | 0.256Mbps |  | UDP | 418 |  | 200 |  | Internet Streaming audio |
| STA8 | 0.256Mbps |  | UDP | 418 |  | 200 |  | Internet Streaming audio |
| STA9 | 5 Mbps |  | UDP | 1500 |  | 200 |  | SDTV |

| STA Name: STA7 – STA10 |  | Role: HT STA (download) |  |  | Location: See table |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – none, these STA act as sinks only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

| STA Name: STA11 – STA12 |  | Role: HT STA (upload) |  |  | Location: See table |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP2 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file transfer |

(Scenarios 12 to 15 are reserved)

# Simulation Scenarios related to Comparison Criteria

The simulation scenarios in this section are referenced from the comparison criteria document [6].

## Scenario 16 (Point-to-Point Throughput Test for CC 27-28,58)

Unlike other UDP sources, these UDP sources have no timeout value specified (it can be considered to be infinite).

This simulation is to be repeated using channel models B and D.

The simulation is repeated with the STA at locations in the range (0,0 to 0,200).

The UDP delay is not relevant, it can be considered to be infinite.

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA | 400Mbps |  | UDP | 1500 |  | N/A |  |  |

| STA Name: STA |  | Role: HI Sink |  |  | Location: (varies, see text) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – none, this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

## Scenario 17 (Point-to-Point HT Goodput Test for CC 15)

Use channel model B for this scenario.

The channelization for this scenario is 20 MHz. Goodput is measured.

Unlike other UDP sources, these UDP sources have no timeout value specified (it can be considered to be infinite).

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this AP is a sink only, no apps generate data from this STA |  |  |  |  |  |  |  |  |

| STA Name: STA |  | Role: HT UDP Source |  |  | Location: (0, 10) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP | 100 Mbps |  | UDP | 1500 |  |  |  |  |

## Scenario 18 (Point-to-Point Legacy Throughput Test for CC 15)

Use channel model B for this scenario.

The channelization for this scenario is 20 MHz. Goodput is measured.

Unlike other UDP sources, these UDP sources have no timeout value specified (it can be considered to be infinite).

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this AP is a sink only, no apps generate data from this AP |  |  |  |  |  |  |  |  |

| STA Name: STA |  | Role: Legacy UDP Source |  |  | Location: (0, 10) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP | 100 Mbps |  | UDP | 1500 |  |  |  |  |

## Scenario 19 (Point-to-Point Legacy Sharing Throughput Test for CC 15)

Use channel model B for this scenario.

The channelization for this scenario is 20 MHz. Goodput is measured.

Unlike other UDP sources, these UDP sources have no timeout value specified (the timeout can be considered to be infinite).

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this AP is a sink only, no apps generate data from this AP |  |  |  |  |  |  |  |  |

| STA Name: STA1 |  | Role: Legacy UDP Source |  |  | Location: (-10 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP | 100 Mbps |  | UDP | 1500 |  |  |  |  |

| STA Name: STA2 |  | Role: HT UDP Source |  |  | Location: (10, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP | 100 Mbps |  | UDP | 1500 |  |  |  |  |

References

[1]	11-03-364r0, Application Characteristics for HT Usage Scenarios,  Javier del Prado et al., Philips

[2]	11-03-489r0, Home Usage Model for HT WLAN Systems, Chiu Ngo et al., Samsung

[3]	11-03-327r0 A Proposed Usage Model Methodology for the High Throughput Task Group, Adrian Stephens, Intel

[4]	11-03-354, Usage Model Special Committee Cumulative Minutes, Adrian Stephens, Intel

[5]	11-03-940, TGn Channel Models

[6]	11-03-0814, TGn Comparison Criteria

Note, this corresponds to a loss of a 1024B MSDU per hour.   The TS PDU PLR is higher than this.  It is not known what is the effect to the decoder of giving it burst packet losses.

Note, a PLR of 10^-7 will not be measurable in our simulation technologies.

14File Transfer at HT at the minimum figure in the PAR should be a requirement for these applications. We may want to emphasize the differentiation between regular FTP and “Broadband FTP” (e.g. very short download time for audio files, “large” email message instantaneous sync, enterprise-class FTP etc)

This condition is specified because the shadowing term is a slowly varying random variable.  It varies slowly compared to realistic simulation durations (i.e. 10s).  This simulation specification has two alternatives to incorporate this effect properly: perform a large number of simulation runs and average the results or incorporate a static shadowing term.  It is not feasible to multiple the number of simulations by a large number, so shadowing is viewed as a static property of the simulation scenario that is already incorporated into the topology of the scenario (i.e. randomized placement of the STA).