IEEE P802.11
Wireless LANs

dot11 Stats

Date:	September 17, 2003

Author:	Gary Spiess
	Intermec Technologies
	550 Second St SE, Cedar Rapids, IA 52401
	Phone: 319 369 3580
	Fax: 319 369 3804
	e-Mail: Gary.Spiess@Intermec.com

#### ABSTRACT

#### This is a proposal to add Action Frames to support the reporting of measurements recorded while using the medium to exchange frames with another station.  The mechanism is designed as an extensible framework to permit a station read-only access of statistics that are kept in a peer’s MIB.

#### A new table is added to the MIB to capture information about how the link is utilizing and is affected by the data rate.

#### Some corrections to the MIB specified in the existing draft are addressed.

#### 7.3.2.19 Measurement Request Element

Insert following measurement type into Table 0-1:

| dot11mac Stats Request | 9 |
| --- | --- |

Insert the following new clause after clause 7.3.1.19.6 and renumber figures and tables as necessary:

##### 7.3.2.19.7 dot11mac Stats Request

The dot11mac Stats Request can request different types of information, depending of the content of the Stats Identifier field in the request.  The format of the Measurement Request field corresponding to a dot11mac Stats Request is shown in Error: Reference source not found. The response to a dot11mac Stats Request is a dot11mac Stats Report.

|  | Stats Identifier | Peer Address |
| --- | --- | --- |
| Octets: | 1 | 6 |

Figure 0-5 – Measurement Request field format for a dot11mac Stats Request

Stats Identifier is a 4, indicating a dot11mac Stats Report for a Peer is requested, or a 5 to indicate that dot11mac Stats Report for Peer Rates is requested.

Peer STA Address indicates the STA address for which a report is requested.

#### 7.3.2.20 Measurement Report element

Insert following measurement type into Table 0-6:

| dot11mac Stats Report | 9 |
| --- | --- |

Insert the following new clause after clause 7.3.1.20.6 and renumber figures and tables as necessary:

##### 7.3.2.20.7 dot11mac Stats Report

The dot11mac Stats Request can request different types of information, depending of the content of the Stats Identifier field in the request.  The format of the Measurement Report field corresponding to a dot11mac Stats Report is shown in Error: Reference source not found

The dot11mac Stats Report for a Peer is sent in response to a dot11mac Stats Request for a Peer.

|  | Stats Identifier | Peer Address | Transmitted Fragment Count | Failed Count | Retry Count | Multiple Retry Count | Frame Duplicate Count | RTS Success Count |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 6 | 4 | 4 | 4 | 4 | 4 | 4 |

| (Continued) | RTS Failure Count | ACK Failure Count | Received Fragment Count | Multicast Received Frame Count | Transmitted Frame Count | WEP Undecryptable Count | Received RCPI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

Figure 0.-13 – Measurement Request field format for a dot11mac Stats Report for a Peer

Stats Identifier is a 4, indicating the dot11mac Stats report for a Peer.

Peer Address indentifies the MAC address for which the report was requested.  If there is no Stats for this Peer STA Address, the dot11mac Stats Report shall only contain this field.

Transmitted Fragment Count is the number of MMPDUs and MPDUs acknowledged by the peer.

Failed Count is the number of undelivered MSDUs sent to the peer.

Retry Count is the number of MSDUs delivered with one or more retransmissions.

Multiple Retry Count is the number of MSDUs delivered with more than one retransmission.

Frame Duplicate Count is the number of MMPDUs and MPDUs successfully received where the sequence control field is identifies it as a duplicate fragment.

RTS Success Count is the number of CTSs received in response to an RTS.

RTS Failure Count is the number of RTSs that received no response.

ACK Failure Count is the number of MMPDUs and MPDUs unacknowledged by the peer.

Received Fragment Count is the number of MMPDUs and MPDUs successfully received from the peer.

Multicast Received Frame Count is the number of MSDUs received with the multicast bit set in the destination MAC address.

Transmitted Frame Count is the number of MSDUs successfully transmitted.

WEP Undecryptable Count is the number of MMPDUs and MPDUs that were discarded because an encrypted frame was received and this station was unprepared to attempt decryption.

Received RCPI is the running average of the RCPI for the four most recently received frames, or 0 if no frames have been received.

The dot11mac Stats Report for Peer Rates is sent in response to a dot11mac Stats Request for Peer Rates.

|  | Stats Identifier | Peer Address | Number of Rate Entries | Rate Table |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 6 | 1 | 6*Number of Rate Entries |

Figure 0.-14 – Measurement Request field format for a dot11mac Stats Report for Peer Rates

|  | Phy | Rate | Tx Good Count | Tx Error Count | Rx Good Count | Rx Error Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 0-15 – Rate Stats Table Entry format in a dot11mac Stats Report

Stats Identifier is a 5, indicating the dot11mac Stats report for Peer Rates.

Peer Address indentifies the MAC address for which the report was requested.  If there is no Stats for this Peer STA Address, the dot11mac Stats Report shall only contain this field.

Number of Rate Entries indicates the number of entries in the Rate Table.

Rate Table shall contain a list of one or more entries for Rates in the Rate Set used by the Peer.  Entries are ordered by ascending Phy values.  Within each Phy, the entries are ordered by ascending Rate values.

Phy shall contain the dot11PHYtype of the Rate Stats Entry.  A Rate Stats Table does not need to contain an entry for all Phys supported.

Rate shall contain a dot11SupportedDataRatesRxValue for the Phy.  A Rate Stats Table does not need to contain an entry for all rates supported by the Phy.

Tx Good Count is the running average number of acknowledged MPDUs and MMPDUs sent with a TA of the reporting station and an RA of the Peer STA Address since this Peer was established.

Tx Error Count is the running average number of unacknowledged MPDUs and MMPDUs sent with a TA of the reporting station and an RA of the Peer STA Address since this Peer was established.

Rx Good Count is the running average number of MPDUs and MMPDUs sent with a TA of the Peer STA Address and an RA of the reporting station, regardless of the retry bit, since this Peer was established.

Rx Error Count is the running average number of MPDUs and MMPDUs sent with a TA of the Peer STA Address and an RA of the reporting station, where the retry bit was set, since this Peer was established.

Insert the following new clause after clause 11.7.7.6 and renumber figures and tables as necessary:

#### 11.7.7.7 dot11mac Stats Report

A STA receiving a dot11mac Stats Request shall respond with a Radio Measurement Report frame containing one Measurement (dot11mac Stats) Report element. The dot11mac Stats Report in the Radio Measurement category shall include information accumulated since the became operational.  When a Peer is (re)established, such as when a (re)association is completed, the dot11mac Stats counts are cleared to zero.

In an ESS, a STA connecting to an AP should maintain several Peer histories for the APs to which it associates, although it shall maintain no less than two such histories.  An AP shall maintain one entry for each currently associated STA, and it is desirable for the Peer histories to remain available after the association has ended.

In an IBSS, a STA should maintain several Peer histories for the stations with which it communicates, although it shall maintain no less than two such histories.

During normal operation of a Peer, the Counts in the dot11mac Stats are incremented for the associated event.  Whenever an increment would overflow a Good Count or Error Count, the value shall be set to 128 and all other Good Counts and Error Counts for the Peer are divided by two.

| Event | RA | TA | Increment |
| --- | --- | --- | --- |
| Transmitted MPDU or MMPDU is acknowledged | Peer STA | Local STA | Tx Good Count |
| Transmitted MPDU or MMPDU is not acknowledged | Peer STA | Local STA | Tx Error Count |
| Received MPDU or MMPDU with Retry flag clear | Local STA | Peer Address | Rx Good Count |
| Received MPDU or MMPDU with Retry flag set | Local STA | Peer Address | Rx Good Count and Rx Error Count |

Figure 0-16 – Count Handling of Transmit or Receive Events

# Annex D

After the following line…

--  dot11peerStatsTable		::= { dot11mac 4 }

…Insert the following line:

--  dot11peerRatesTable		::= { dot11mac 5 }

Change the description of dot11peerStatsTableBSSaging from

DESCRIPTION

"The peerStatsTableBSSaging indicates the period of time

which entries for peer STAs that are in the peerStatsTable

because the peer STA was authenticated or associated with the

local STA must be kept in the table after the peer STA

becomes disassociated. The peer STA entry may be kept in the

peerStatsTable for longer than this time."

To

DESCRIPTION

"The peerStatsTableBSSaging indicates the minimum number of

seconds entries shall be kept in the peerStatsTable and

the peerRatesTable after deauthentication of peers that were

authenticated or associated with the local STA.  The default

value is 60 seconds."

Change the description of dot11peerStatsTableIBSSaging from

DESCRIPTION

"The peerStatsTableIBSSaging indicates the period of time

which entries for peer STAs that are in the peerStatsTable

because the peer STA was in an IBSS that the local STA was

joined to, and send a data frame to the local STA, or the

local STA attempted to send any frame to the peer STA

must be kept in the table after the peer STA

last communicated with the local STA. The peer STA entry

may be kept in the

peerStatsTable for longer than this time."

To

DESCRIPTION

"The peerStatsTableIBSSaging indicates the minimum number of

seconds entries shall be kept in the peerStatsTable and

the peerRatesTable after the last communication between the

local STA and the peer member of the IBSS.  The default value

is 60 seconds."

Change the description of dot11peerStatsEntry from

DESCRIPTION

"An entry in the Dot11peerStaStatsEntry Table.

Indexed by ifIndex and the mac address of the remote STA - dot11peerStaAddress"

To

DESCRIPTION

"An entry in the Dot11peerStaStatsEntry Table.

Indexed by ifIndex and the mac address of the remote STA - dot11peerAddress"

Remove the following incomplete attributes from Dot11peerStatsEntry and renumber the remaining attributes consecutively:

dot11peerWEPDecryptionFailureCount
dot11peerReceivedByteCount
dot11peerTransmittedByteCount
dot11peerReceivedBeaconCount

Change the description of the dot11peerFrameDuplicateCount attribute from

"This counter shall increment when a frame is received

that the Sequence Control field indicates is a

duplicate."

To

"This counter shall increment when an MPDU of type Data or Management is received

where the Sequence Control field indicates it is a

duplicate."

Change the description of the dot11peerWEPUndecryptableCount attribute from

"This counter shall increment when a frame is received with

the WEP subfield of the Frame Control field set to one and the

WEPOn value for the key mapped to the TA's MAC address

indicates that the frame should not have been encrypted or

that frame is discarded due to the receiving STA not

implementing the privacy option."

To

"This counter shall increment when an MMPDU or MPDU is received with

the WEP subfield of the Frame Control field set to one and the

WEPOn value for the key mapped to the TA's MAC address

indicates that the frame should not have been encrypted or

that PDU is discarded due to the receiving STA not

implementing the privacy option."

Change the description of the dot11peerLastReceiveRCPI attribute from

"The RCPI from the most recently received frame, or 0 if no

frames have been received."

To

"The running average of the RCPI for the four most recently received frames, or 0

if no frames have been received."

Insert the following  after the definitions describing the attributes of dot11peerStatsTable and before the Resource Type Attribute Templates

-- **********************************************************************

-- *    dot11peerRatesTable TABLE

-- **********************************************************************

dot11peerRatesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11peerRatesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains rate statistics for peer STAs. The following

peer STAs appear in this table:

1. Authenticated peer STAs must appear in this table. Their

entries shall not be removed until at least

dot11peerRatesTableBSSaging seconds have passed after they are

marked as deauthenticated.

2. Peer STAs joined to the same IBSS as this STA that have

sent any data frame to this STA must appear in this table.

Entries for these peer STAs may be removed when this STA

is no longer joined to the same IBSS, or if

dot11peerRatesTableIBSSaging seconds have passed since a data

frame was exchanged with the peer STA.

3. Other peer STAs may be included in this table."

::= { dot11mac 4 }

dot11peerRatesEntry OBJECT-TYPE

SYNTAX Dot11peerRatesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the Dot11peerStaStatsEntry Table.

Indexed by ifIndex, the mac address of the remote STA - dot11peerRtAddress,

the PHY type – dot11peerRt, and the data rate – dot11peerRtDataRateValue"

INDEX { ifIndex, dot11peerRtAddress, dot11peerRtPHYtype, dot11peerRtDataRateValue }

::= { dot11peerRatesTable 1 }

Dot11peerRatesEntry ::= SEQUENCE {

dot11peerRtAddress				MacAddress,

dot11peerRtPHYtype				INTEGER,

dot11peerRtDataRateValue				Integer32,

dot11peerRtPDUTxGood				INTEGER,

dot11peerRtPDUTxError				INTEGER,

dot11peerRtPDURxGood				INTEGER,

dot11peerRtPDURxError				INTEGER }

dot11peerRtAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address identifying a peer STA."

::= { dot11peerRatesEntry 1 }

dot11peerRtPHYType OBJECT-TYPE

SYNTAX INTEGER {fhss(1), dsss(2), irbaseband(3), ofdm(4), hrdsss(5), erp(6)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that identifies the PHY type

supported by the attached PLCP and PMD. Currently defined

values and their corresponding PHY types are:

FHSS 2.4 GHz = 01 , DSSS 2.4 GHz = 02, IR Baseband = 03,

OFDM 5GHz = 04, HRDSSS = 05, ERP = 06 "

::= { dot11peerRatesEntry 2 }

dot11peerRtDataRateValue OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This value represents the bit rate, as defined for the PHY specified

in dot11peerRtPHYType."

::= { dot11peerRatesEntry 2 }

dot11peerRtPDUTxGood OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that represents the number of

acknowledged PDUs sent to the STA in dot11peerRtAddress from

the local STA. This is the output of a running average, and

is relative to all dot11peerRtPDUTxGood and dot11RtPDUTxError values

with a matching dot11peerRtAddress.”

::= { dot11peerRatesEntry 3 }

dot11peerRtPDUTxError OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that represents the number of

acknowledged PDUs sent to the STA in dot11peerRtAddress from

the local STA. This is the output of a running average, and

is relative to all dot11peerRtPDUTxGood and dot11RtPDUTxError values

in a dot11peerRateEntry with a matching dot11peerRtAddress.”

::= { dot11peerRatesEntry 4 }

dot11peerRtPDURxGood OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that represents the number of

acknowledged PDUs sent by the STA in dot11peerRtAddress to the local

STA. This is the output of a running average, and

is relative to all dot11peerRtPDURxGood and dot11RtPDURxError values

with a matching dot11peerRtAddress.”

::= { dot11peerRatesEntry 5 }

dot11peerRtPDURxError OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that approximates the number of

retried PDUs sent by the STA in dot11peerRtAddress to the local

STA. Because multiple retries of a PDU or retries for a frame

that was never delivered will not be recorded, this value is generally

understated.  This is the output of a running average, and

is relative to all dot11peerRtPDURxGood and dot11RtPDURxError values

in a dot11peerRateEntry with a matching dot11peerRtAddress.”

::= { dot11peerRatesEntry 6 }