IEEE P802.11
Wireless LANs

Radio Resource Measurements MIB – Seattle Ad Hoc

Date:	October 2003

Authors:

Tim Olson
	Cisco Systems
	Phone: 408-853-4527
	e-Mail: tolson@cisco.com

Abstract

This document presents MIB tables for entering report requests, retrieving beacon reports, setting AP Channel Report information, and setting Site Report information. Most variables are one-to-one mapping from the fields in the request and report frame formats.

Insert the following in the ASN.1 encoding in the section of extual conventions from the 802 defintions section in Annex D..

-- **********************************************************************

-- * Textual conventions from 802 definitions

-- **********************************************************************

TSFtype ::= OCTET STRING (SIZE (8))

Insert the following in the ASN.1 encoding of the SMT MIB, dot11smt in Annex D.

dot11RadioResourceManagement OBJECT IDENTIFIER ::= { dot11smt 8 }

-- ********************************************************************

-- * Radio Resource Measurement Requests

-- ********************************************************************

dot11RRMRequest OBJECT IDENTIFIER ::= { dot11RadioResourceManagement 1 }

-- ********************************************************************

-- * dot11RRMRequest	TABLE

-- ********************************************************************

dot11RRMRequestMaxTableSize OBJECT-TYPE

SYNTAX		Unsigned32(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Identifies the number of measurement requests that can be entered

into RRMRequestTable.  A value of zero to indicates that external

requests are not supported."

::= { dot11RRMRequest 1 }

dot11RRMRequestTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RRMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of requests for RRM

report to be issued and have been issued. A network manager

adds a RRM request by updating a row and setting its status to

active. The request becomes ready to be issued when the row status

is set to Active. The rows can not be deleted. Once the MLME issues

the requested measurement the row status is changed to not active."

::= { dot11RRMRequest 2 }

dot11RRMRequestEntry OBJECT-TYPE

SYNTAX Dot11RRMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RRMRequestTable

Indexed by dot11RRMRqstIndex."

INDEX { dot11RRMRqstIndex }

::= { dot11RRMRequestTable 1 }

Dot11RRMRequestEntry ::=

SEQUENCE {

dot11RRMRqstIndex					Unsigned32,

dot11RRMRqstRowStatus				RowStatus,

dot11RRMRqstToken					OCTET STRING,

dot11RRMRqstIfIndex				INTEGER,

dot11RRMRqstType					INTEGER,

dot11RRMRqstTargetAdd				MacAddress,

dot11RRMRqstTimeStamp				TimeTicks,

dot11RRMRqstChanNumber			INTEGER,

dot11RRMRqstChanBand				INTEGER,

dot11RRMRqstRndInterval			Unsigned32,

dot11RRMRqstDuration				Unsigned32,

dot11RRMRqstParallel				TruthValue,

dot11RRMRqstEnable				TruthValue,

dot11RRMRqstRequest				TruthValue,

dot11RRMRqstReport				TruthValue,

dot11BeaconRqstMode				INTEGER,

dot11RRMRqstBssid					MacAddress,

dot11RRMRqstMeasPeriodUnit		INTEGER,

dot11RRMRqstMeasurementPeriod	INTEGER,

dot11RRMRqstMeasIntervalUnit 	INTEGER,

dot11RRMRqstMeasurementInterval INTEGER,

dot11RRMRqstReportingCondition  INTEGER,

dot11RRMRqstThresholdOffset		INTEGER,

dot11RRMRqstHysteresis 			INTEGER,

dot11MedSenseType					INTEGER,

dot11MedSenseRPIThreshold			INTEGER,

dot11MedSenseBinOffset			INTEGER,

dot11MedSenseBinInterval			INTEGER,

dot11MedSenseNumBin				INTEGER,

dot11STAStatRqstGroupID			INTEGER

}

dot11RRMRqstIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for RRM Request elements in dot11RRMRequestTable, greater

than 0."

::= { dot11RRMRequestEntry 1 }

dot11RRMRqstRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Row Status column of the current row, used for

tracking status of an individual request. When this attribute

is set to Active, AND a measurement request can be unambiguously

created based on the parameters in the row according to Section

<< TGk references here>>, then MLME may proceed to issue the

request to its intended targets. If not, this attribute may be

set to Not-ready immediately to indicate parametric errors.

However, it is the network managers responsibility to correct

the error"

::= { dot11RRMRequestEntry 2 }

dot11RRMRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A unique string to identy a group of rows to be issued as parallel

or back to back measurements. To guarantee the uniqueness of this

token across multiple network managers, it is recommended that this

token be prefixed with the IP address of the network manager

creating this row. This token is not necessarily equivalent to the

measurement tokens in RRM request frames. If this attribute is null string, then this row of request is independent from other requests."

::= { dot11RRMRequestEntry 3 }

dot11RRMRqstIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of RRM Request to be issued on."

::= { dot11RRMRequestEntry 4 }

dot11RRMRqstType OBJECT-TYPE

SYNTAX INTEGER (3..9)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The measurement type of this RRM request row.

3: Channel Load Request

4: Noise Histogram Request

5: Beacon Request

6: Frame Request

7: Hidden Node Request

8: Medium Sensing Time Histogram Request

9: STA statistics Request

The rest of the numbers are reserved."

::= { dot11RRMRequestEntry 5 }

dot11RRMRqstTargetAdd OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MAC address of STA for this row of RRM Request is to be issued

to. If this attribute matches the MAC address of the

dot11RRMRqstIfIndex, then measurement request is for this STA

itself to be carried out."

::= { dot11RRMRequestEntry 6 }

dot11RRMRqstTimeStamp OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The SysUpTime Value when the dot11RRMRqstRowStatus is set to

active."

::= { dot11RRMRequestEntry 7 }

dot11RRMRqstChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The channel number on which this request instructs the target STA

to report detected measurements and probe responses. This attribute

is ignored if dot11RRMRqstType = 9: STA statistics Request. However, even in that case, the manager should set this attribute to the current channel for this interface, to be compliant with MIB row creation procedures."

::= { dot11RRMRequestEntry 8 }

dot11RRMRqstChanBand OBJECT-TYPE

SYNTAX INTEGER(0..1)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Channel Band indicates the frequency band, taken from Table 0-2,

TGk Spec, in which the receiving STA shall conduct its measurement.

0 : 2.4 GHz

1 : 5 GHz

Other values are reserved.

This attribute is ignored if dot11RRMRqstType = 9: STA statistics

Request. However, even in that case, this value should set to the

current channel band for this interface, to be compliant with MIB

row creation procedures."

::= { dot11RRMRequestEntry 9 }

dot11RRMRqstRndInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This value helps set the Randomization Interval in the measurement

Request Frame for this RRM Request. dot11RRMRqstRndInterval is

expressed in TUs. The default value is 0. This attribute is ignored

if dot11RRMRqstType = 9: STA statistics Request."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 10 }

dot11RRMRqstDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This value helps set the Measurement Duration in the measurement

Request Frame for this RRM Request, expressed in TUs."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 11 }

dot11RRMRqstParallel OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute corresponds to the parallel bit of the request

frame that contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 12 }

dot11RRMRqstEnable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute corresponds to the enable bit of the request frame

that contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 13 }

dot11RRMRqstRequest OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute corresponds to the request bit of the request frame

that contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 14 }

dot11RRMRqstReport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute corresponds to the report bit of the request frame

that contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 15 }

dot11BeaconRqstMode OBJECT-TYPE

SYNTAX INTEGER(0..2)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"dot11BeaconRqstMode corresponds to the Scan Mode for Beacon

Request element. This attribute is only valid if the

dot11RRMRqstType  is 5, indicating a beacon report. Otherwise this

attribute is ignored.

0 : Passive Mode

1 : Active Mode

2 : Beacon Table Mode:

Other values are reserved as of now

In Passive Scan mode, the measuring STA shall passively receive on

the specified channel and return a Beacon Report containing one

information element for each STA from which it detects a beacon or

probe response. If the measuring channel is also the serving

channel, the STA shall concurrently carry out its normal data

traffic operation.

In Active Scan mode, the measuring STA shall transmit a probe

Request with the broadcast SSID. The measuring STA's Beacon Report

shall contain one information element for each STA from which it

detects a beacon or probe response, regardless of whether the probe

response was triggered by the measuring STA's own probe request.

In Beacon Table mode, the measuring STA shall return a Beacon

Report 	containing the current contents of its beacon table without

performing additional measurements."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 16 }

dot11RRMRqstBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"BSSID indicates the BSSID of the particular AP for which this

measurement is requested.  The BSSID shall be set to the broadcast

BSSID when the measurement is to be performed on any AP(s) on this

channel."

::= { dot11RRMRequestEntry 17 }

dot11RRMRqstMeasPeriodUnit OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Measurement Period Unit subfield defines the time unit for the

Measurement Period subfield using the following values:

0: TU

1: msec

2: sec

3: reserved "

::= { dot11RRMRequestEntry 18 }

dot11RRMRqstMeasurementPeriod OBJECT-TYPE

SYNTAX INTEGER (0.. 16383)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Measurement Period indicates whether this measurment is a

single measurement event or is a periodic measurment which is

scheduled and repeated each Measurment Period.  The Measurement

Periods is an unsigned 14 bit integer number representing the

repeating time interval for this periodic measurment.  The time

unit used for Measurment Period is defined in Measurement Period

Unit.  A Measurement Period value of 0 shall indicate that the

measurment is not periodic but is a single measurement.  A

Measurement Period value of 16383 (3FFF Hex) shall indicate that

the measurement is periodic with no requested measurement period;

in this case the measurment is performed on a best effort basis and

as frequently as conditions will permit.  If scheduling conflicts

prevent the STA from executing a periodic measurement at the

requested measurement time, the STA may delay the periodic

measurement, but shall execute the measurement as soon thereafter

as possible.  A delayed periodic measurment shall not change the

scheduled measurement time for any subsequent periodic

measurements."

::= { dot11RRMRequestEntry 19 }

dot11RRMRqstMeasIntervalUnit OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Measurement Period Unit subfield defines the time unit for the

Measurement Period subfield using the following values:

0: TU

1: msec

2: sec

3: reserved "

::= { dot11RRMRequestEntry 20 }

dot11RRMRqstMeasurementInterval OBJECT-TYPE

SYNTAX INTEGER (0..16383)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Measurement Interval indicates the time interval during which

this periodic measurement shall be implemented. Periodic

measurements shall begin at the indicated start time.  Periodic

measurements shall continue for the Measurement interval time, then

periodic measurments shall end. Measurement Interval uses a 14 bit

unsigned integer number representing the repeating time interval

for this periodic measurment.  The time unit used for Measurment

Interval is defined in Measurement Interval Unit.  A Measurement

Interval value of 0 shall terminate any currently active periodic

Beacon Request measurement for the indicated BSSID."

::= { dot11RRMRequestEntry 21 }

dot11RRMRqstReportingCondition OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Reporting Condition defines when the measured results are to

be reported to the requesting STA.  The Reporting Condition values

are as follows:

0: Report to be issued  after each measurment

1: Report to be issued when the RCPI level of the measured AP

crosses above an absolute threshold with hysteresis

2: Report to be issued when the RCPI level of the measured AP

crosses below an absolute threshold with  hysteresis.

3: Report to be issued when the RSSI level of the measured AP

crosses above an absolute threshold with hysteresis.

4: Report to be issued when the RSSI level of the measured AP

crosses below an absolute threshold with hysteresis.

5: Report to be issued when the RCPI level of the measured AP

crosses above a threshold defined by an offset (with

hysteresis) from the serving AP's RCPI.

6: Report to be issued when the RCPI level of the measured AP

crosses below a threshold defined by an offset (with

hysteresis) from the serving AP's RCPI.

7: Report to be issued when the RSSI level of the measured AP

crosses above a threshold defined by an offset (with

hysteresis) from the serving AP's RSSI.

8: Report to be issued when the RSSI level of the measured AP

crosses below a threshold defined by an offset (with

hysteresis) from the serving AP's RSSI.

9: Periodic reports (one per measurement) to begin when the

RCPI level of the measured AP enters and remains in a range

bound by the serving AP's RCPI and an offset (with

hysteresis) from the serving AP's RCPI.

10: Periodic reports (one per measurement) to begin when the

RSSI level of the measured AP enters and remians in a range

bound by the serving AP's RSSI and an offset (with hysteresis) from the serving AP's RSSI.

11-255:  reserved"

::= { dot11RRMRequestEntry 22 }

dot11RRMRqstThresholdOffset OBJECT-TYPE

SYNTAX INTEGER (-128..127)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Threshold/Offset provides either the threshold value or the offset

value to be used for conditional reporting. A threshold value is an

unsigned 8 bit integer having units equivalent to RSSI or RCPI. An

offset value is a signed 8 bit integer in the range [-128, +127]."

::= { dot11RRMRequestEntry 23 }

dot11RRMRqstHysteresis OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Hysteresis provides an unsigned 8 bit integer hysteresis value

having 	units equivalent to the units used in the Threshold/Offset

field."

::= { dot11RRMRequestEntry 24 }

dot11MedSenseType	OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute specifies the medium sense histogram request subtype

if dot11RRMRqstType = 8: Medium Sensing Time Histogram Request.

This attribute shall be ignored otherwise.

0: RCPI Time Histogram

1: CCA Idle Time Histogram

2: CCA Busy Time Histogram

3: CCA NAV Time Histogram

Other valued are reserved"

DEFVAL { 0 }

::= { dot11RRMRequestEntry 25 }

dot11MedSenseRPIThreshold OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute corresponds to the Medium Sense RPI Threshold if

dot11RRMRqstType = 8: Medium Sensing Time Histogram Request. This

attribute shall be ignored otherwise."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 26 }

dot11MedSenseBinOffset OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute corresponds to the Medium Sense Bin Offset if

dot11RRMRqstType = 8: Medium Sensing Time Histogram Request.

This attribute shall be ignored otherwise."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 27 }

dot11MedSenseBinInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute corresponds to the Medium Sense Bin Interval if

dot11RRMRqstType = 8: Medium Sensing Time Histogram Request. This

attribute shall be ignored otherwise."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 28 }

dot11MedSenseNumBin OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute corresponds to the Medium Sense Number of Bins if

dot11RRMRqstType = 8: Medium Sensing Time Histogram Request. This

attribute shall be ignored otherwise."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 29 }

dot11STAStatRqstGroupID OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute corresponds to the group identity if

dot11RRMRqstType = 9: STA statistics Request. This attribute

shall be ignored otherwise.

0 = STA Counters from dot11CountersTable

The rest are reserved."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 30 }

-- **********************************************************************

-- *    End of dot11RRMRequest	TABLE

-- **********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Reports

-- ********************************************************************

dot11RRMReport OBJECT IDENTIFIER ::= { dot11RadioResourceManagement 2 }

-- ********************************************************************

-- * dot11ChannelLoadReport	TABLE

-- ********************************************************************

dot11RRMChannelLoadReportMaxTableSize OBJECT-TYPE

SYNTAX		Unsigned32(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Identifies the number of measurement reports that can be entered

into ChannelLoadReportTable."

::= { dot11RRMReport 1 }

dot11ChannelLoadReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Channel Load reports that

have been received by the MLME. The report tables shall be

maintained as FIFO to preserve freshness, thus this attribute can

be overridden by memory constraints or other implementation constraints

determined by the vendor."

::= { dot11RRMReport 2 }

dot11ChannelLoadReportEntry OBJECT-TYPE

SYNTAX Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ChannelLoadReportTable

Indexed by dot11ChannelLoadRprtIndex."

INDEX { dot11ChannelLoadRprtIndex }

::= { dot11ChannelLoadReportTable 1 }

Dot11ChannelLoadReportEntry ::=

SEQUENCE {

dot11ChannelLoadRprtIndex					Unsigned32,

dot11ChannelLoadRprtRqstToken 			OCTET STRING,

dot11ChannelLoadRprtIfIndex				INTEGER,

dot11ChannelLoadMeasuringSTAAddr			MacAddress,

dot11ChannelLoadRprtChanNumber			INTEGER,

dot11ChannelLoadRprtChanBand				INTEGER,

dot11ChannelLoadRprtActualStartTime		TSFtype,

dot11ChannelLoadRprtMeasurementDuration	Unsigned32,

dot11ChannelLoadRprtCcaBusyFraction		INTEGER

}

dot11ChannelLoadRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Channel Load Report elements in

dot11ChannelLoadReportTable, greater than 0."

::= { dot11ChannelLoadReportEntry 1 }

dot11ChannelLoadRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report.  This

should 	be an exact match to the original dot11RRMRqstToken

attribute.  Note that there may be multiple entries in the table

that match this value since a single request may generate multiple

measurement reports."

::= { dot11ChannelLoadReportEntry 2 }

dot11ChannelLoadRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of ChannelLoad Report has been

received on."

::= { dot11ChannelLoadReportEntry 3 }

dot11ChannelLoadMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Channel Load

report."

::= { dot11ChannelLoadReportEntry 4 }

dot11ChannelLoadRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The channel number for this row of Channel Load Report Table."

::= { dot11ChannelLoadReportEntry 5 }

dot11ChannelLoadRprtChanBand OBJECT-TYPE

SYNTAX INTEGER(0..1)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" Channel Band indicates the frequency band, taken from Table 0-2,

TGk Spec, in which this row of ChannelLoad report has been conducted.

0 : 2.4 GHz

1 : 5 GHz

Other values are reserved."

::= { dot11ChannelLoadReportEntry 6 }

dot11ChannelLoadRprtActualStartTime OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11ChannelLoadReportEntry 7 }

dot11ChannelLoadRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the

ChannelLoad Report was measured, expressed in TUs."

::= { dot11ChannelLoadReportEntry 8 }

dot11ChannelLoadRprtCcaBusyFraction OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"CCA Busy Fraction contains the fractional duration over which CCA

indicated tha the channel was busy during the measurement duration.

It is defined as Ceiling (255 * [Duration CCA indicated channel was

busy] / [Measurement duration])."

::= { dot11ChannelLoadReportEntry 9 }

-- ********************************************************************

-- * End of dot11ChannelLoadReport	TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11NoiseHistogramReport	TABLE

-- ********************************************************************

dot11RRMNoiseHistogramMaxTableSize OBJECT-TYPE

SYNTAX		Unsigned32(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Identifies the number of measurement reports that can be entered into

NoiseHistogramReportTable."

::= { dot11RRMReport 3 }

dot11NoiseHistogramReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Beacon report that have been

received by the MLME. The report tables shall be maintained as FIFO

to preserve freshness, thus this attribute can be overridden by

memory 	constraints or other implementation constraints determined

by the vendor."

::= { dot11RRMReport 4 }

dot11NoiseHistogramReportEntry OBJECT-TYPE

SYNTAX Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11NoiseHistogramReportTable

Indexed by dot11NoiseHistogramRprtIndex."

INDEX { dot11NoiseHistogramRprtIndex }

::= { dot11NoiseHistogramReportTable 1 }

Dot11NoiseHistogramReportEntry ::=

SEQUENCE {

dot11NoiseHistogramRprtIndex					Unsigned32,

dot11NoiseHistogramRprtRqstToken 			OCTET STRING,

dot11NoiseHistogramRprtIfIndex				INTEGER,

dot11NoiseHistogramMeasuringSTAAddr			MacAddress,

dot11NoiseHistogramRprtChanNumber			INTEGER,

dot11NoiseHistogramRprtChanBand				INTEGER,

dot11NoiseHistogramRprtActualStartTime		TSFtype,

dot11NoiseHistogramRprtMeasurementDuration	Unsigned32,

dot11NoiseHistogramRprtRPIDensity0			INTEGER,

dot11NoiseHistogramRprtRPIDensity1			INTEGER,

dot11NoiseHistogramRprtRPIDensity2			INTEGER,

dot11NoiseHistogramRprtRPIDensity3			INTEGER,

dot11NoiseHistogramRprtRPIDensity4			INTEGER,

dot11NoiseHistogramRprtRPIDensity5			INTEGER,

dot11NoiseHistogramRprtRPIDensity6			INTEGER,

dot11NoiseHistogramRprtRPIDensity7			INTEGER

}

dot11NoiseHistogramRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Beacon Request elements in

dot11NoiseHistogramReportTable, greater than 0."

::= { dot11NoiseHistogramReportEntry 1 }

dot11NoiseHistogramRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report.  This

should be an exact match to the original dot11RRMRqstToken

attribute.  Note that there may be multiple entries in the table

that match this value since a single request may generate multiple

measurement reports."

::= { dot11NoiseHistogramReportEntry 2 }

dot11NoiseHistogramRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of Noise Histogram Report has been

received on."

::= { dot11NoiseHistogramReportEntry 3 }

dot11NoiseHistogramMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Noise

Histogram report."

::= { dot11NoiseHistogramReportEntry 4 }

dot11NoiseHistogramRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this measurement report was made."

::= { dot11NoiseHistogramReportEntry 5 }

dot11NoiseHistogramRprtChanBand OBJECT-TYPE

SYNTAX INTEGER(0..1)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" Channel Band indicates the frequency band, taken from Table 0-2,

TGk Spec, in which this row of noise histogram report has been

conducted.

0 : 2.4 GHz

1 : 5 GHz

Other values are reserved."

::= { dot11NoiseHistogramReportEntry 6 }

dot11NoiseHistogramRprtActualStartTime OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11NoiseHistogramReportEntry 7 }

dot11NoiseHistogramRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Noise

Histogram Report was measured, expressed in TUs."

::= { dot11NoiseHistogramReportEntry 8 }

dot11NoiseHistogramRprtRPIDensity0 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from Power <= -87."

::= { dot11NoiseHistogramReportEntry 9 }

dot11NoiseHistogramRprtRPIDensity1 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from -87 < Power <= -82."

::= { dot11NoiseHistogramReportEntry 10 }

dot11NoiseHistogramRprtRPIDensity2 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from -82 < Power <= -77."

::= { dot11NoiseHistogramReportEntry 11 }

dot11NoiseHistogramRprtRPIDensity3 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from -77 < Power <= -72."

::= { dot11NoiseHistogramReportEntry 12 }

dot11NoiseHistogramRprtRPIDensity4 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from -72 < Power <= -67."

::= { dot11NoiseHistogramReportEntry 13 }

dot11NoiseHistogramRprtRPIDensity5 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from -67 < Power <= -62."

::= { dot11NoiseHistogramReportEntry 14 }

dot11NoiseHistogramRprtRPIDensity6 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from -62 < Power <= -57."

::= { dot11NoiseHistogramReportEntry 15 }

dot11NoiseHistogramRprtRPIDensity7 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11

signals from Power -57 <= Power."

::= { dot11NoiseHistogramReportEntry 16 }

-- ********************************************************************

-- * End of dot11NoiseHistogramReport	TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11BeaconReport	TABLE

-- ********************************************************************

dot11RRMBeaconReportMaxTableSize OBJECT-TYPE

SYNTAX		Unsigned32(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Identifies the number of measurement reports that can be entered

into BeaconReportTable."

::= { dot11RRMReport 5 }

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Beacon report that have been

received by the MLME. The report tables shall be maintained as FIFO

to preserve freshness, thus this attribute can be overridden by

memory 	constraints or other implementation constraints determined

by the vendor."

::= { dot11RRMReport 6 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable

Indexed by dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex					Unsigned32,

dot11BeaconRprtRqstToken 				OCTET STRING,

dot11BeaconRprtIfIndex				INTEGER,

dot11BeaconMeasuringSTAAddr			MacAddress,

dot11BeaconRprtChanNumber				INTEGER,

dot11BeaconRprtChanBand				INTEGER,

dot11BeaconRprtActualStartTime		TSFtype,

dot11BeaconRprtMeasurementDuration	Unsigned32,

dot11BeaconRprtPhyType				INTEGER,

dot11BeaconRprtRCPI					INTEGER,

dot11BeaconRprtBSSID					MacAddress,

dot11BeaconRprtParentTSF				TSFtype,

dot11BeaconRprtTargetTSF				TSFtype,

dot11BeaconRprtBeaconInterval		Unsigned32,

dot11BeaconRprtCFStatus				INTEGER,

dot11BeaconRprtPrivacy				TruthValue,

dot11BeaconRprtBSSMode				INTEGER,

dot11BeaconRprtReceivedElements		OCTET STRING

}

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable,

greater than 0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in

the measurement request that generated this measurement report.

This should be an exact match to the original dot11RRMRqstToken

attribute.  Note that there may be multiple entries in the table

that match this value since a single request may generate multiple

measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of Beacon Report has been received

on."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Beacon

report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this row of report detected

beacons and probe responses."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtChanBand OBJECT-TYPE

SYNTAX INTEGER(0..1)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Channel Band indicates the frequency band, taken from Table 0-2,

TGk Spec, in which this row of beacon report has been conducted.

0 : 2.4 GHz

1 : 5 GHz

Other values are reserved."

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon

Report was measured, expressed in TUs."

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER(1..6)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Phy Type for this row of Beacon

Report.

Phy Type is defined in Table 0-7 in TGk as..

1: FH

2: DSS

3: unused

4: OFDM

5: High Rate DSS

6: ERP

other values are reserved."

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This value corresponds to the Received Signal Power in the

measurement Report Frame for this row of Beacon Report, expressed

in RCPI."

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the beacon for this

row of Beacon report."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value of the serving

measuring STA's TSF value at the time the measuring STA received

the beacon or probe response frame."

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtTargetTSF OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value contained in the

beacon 	or probe response received by the measuring STA."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtBeaconInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the beacon interval value contained

in the beacon or probe response received by the measuring STA."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtCFStatus OBJECT-TYPE

SYNTAX INTEGER(0..2)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the combination of CF pollable, and

CF-Poll Request bits in the capability information contained in

this row of Beacon report, as in Table 17, thus indicating the

contention-free capabilities of the STA that this row of beacon report corresponds to.

0: No Point Coordinator in the AP

1: Point Coordinator for delivery only

2: Point Coordinator for delivery and polling

Other values are reserved."

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtPrivacy OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Privacy subfield of the

capability information contained in the beacon report element

corresponding to this row."

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtBSSMode OBJECT-TYPE

SYNTAX INTEGER(0..2)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether the BSS in the beacon report

element corresponding to this row is in the Ad-Hoc mode or not,

based on the information 	available from the beacon report corresponding to this row.

0: Unknown

1: Infrastructure Mode

2: Ad-hoc Mode

Other values are reserved."

::= { dot11BeaconReportEntry 17 }

dot11BeaconRprtReceivedElements OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains a variable number of information elements.

All information elements, except Timestamp, Beacon interval and

Capability information, that are present in the measured beacon or

probe response must be included if the measured BSSID is different

from the STA's current BSS. The Beacon Report will hereby enclose

information about the 802.11 enhancements supported by the STA that

transmitted the beacon or probe response. All reported TIM elements

shall be truncated to 4 octets."

::= { dot11BeaconReportEntry 18 }

-- ********************************************************************

-- * End of dot11BeaconReport		TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport	TABLE

-- ********************************************************************

dot11FrameReportMaxTableSize OBJECT-TYPE

SYNTAX 	Unsigned32(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the number of measurement reports that can be entered into FrameReportTable."

::= { dot11RRMReport 7 }

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Frame reports that have been

received by the MLME. The report tables shall be maintained as FIFO

to preserve freshness, thus this attribute can be overridden by

memory constraints or other implementation constraints determined

by the vendor."

::= { dot11RRMReport 8 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable

Indexed by dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex					Unsigned32,

dot11FrameRprtIfIndex					INTEGER,

dot11FrameRprtRqstToken				Unsigned32,

dot11FrameRprtChanNumber				INTEGER,

dot11FrameRprtChanBand				INTEGER,

dot11FrameRprtActualStartTime		TSFtype,

dot11FrameRprtMeasurementDuration	Unsigned32,

dot11FrameRprtMeasuringSTAAddr		MacAddress,

dot11FrameRprtBSSID					MacAddress,

dot11FrameRprtRCPI					INTEGER,

dot11FrameRprtNumberFrames			INTEGER,

}

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameReportTable, greater

than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of Frame Report has been received

on."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameRequestTable that

corresponds to this row of frame report. Since a single frame

request can generate multiple rows in the frame report table, one

per BSSID, this dot11FrameRprtRqstToken indicates which request this particular row corresponds to. If this row of report is received without a particular request, this attribute should be 0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this row of report detected frame

request responses."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtChanBand OBJECT-TYPE

SYNTAX INTEGER(0..1)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Channel Band indicates the frequency band, taken from Table 0-12,

TGk Spec, in which this row of frame report has been conducted.

0 : 2.4 GHz

1 : 5 GHz

Other values are reserved."

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualStartTime OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when

measurement started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon

Report was measured, expressed in TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of STA for this row of Frame report that it has

been received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the STA that

transmitted this frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This value corresponds to the Received Signal Power in the

measurement Report Frame for this row of Frame Report, expressed in

RCPI."

::= { dot11FrameReportEntry 10 }

dot11FrameRprtNumberFrames OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This value corresponds to the number of received frames in the

measurement Report Frame for this row of Frame Report, expressed

in integers."

::= { dot11FrameReportEntry 11 }

-- ********************************************************************

-- * End of dot11FrameReport	TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11HiddenNodeReport	TABLE

-- ********************************************************************

dot11RRMHiddenNodeMaxTableSize OBJECT-TYPE

SYNTAX		Unsigned32(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Identifies the number of measurement reports that can be entered

into HiddenNodeReportTable."

::= { dot11RRMReport 9 }

dot11HiddenNodeReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11HiddenNodeReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Hidden Node Reports that have

been received by the MLME. The report tables shall be maintained as

FIFO to preserve freshness, thus this attribute can be overridden

by 	memory constraints or other implementation constraints

determined by 	the vendor."

::= { dot11RRMReport 10 }

dot11HiddenNodeReportEntry OBJECT-TYPE

SYNTAX Dot11HiddenNodeReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11HiddenNodeReportTable

Indexed by dot11HiddenNodeRprtIndex."

INDEX { dot11HiddenNodeRprtIndex }

::= { dot11HiddenNodeReportTable 1 }

Dot11HiddenNodeReportEntry ::=

SEQUENCE {

dot11HiddenNodeRprtIndex					Unsigned32,

dot11HiddenNodeRprtRqstToken 			OCTET STRING,

dot11HiddenNodeRprtIfIndex				INTEGER,

dot11HiddenNodeMeasuringSTAAddr			MacAddress,

dot11HiddenNodeRprtChanNumber			INTEGER,

dot11HiddenNodeRprtChanBand				INTEGER,

dot11HiddenNodeRprtActualStartTime		TSFtype,

dot11HiddenNodeRprtMeasurementDuration	Unsigned32,

dot11HiddenNodeRprtHiddenSTAAddr			MacAddress,

dot11HiddenNodeRprtNumFrames				INTEGER

}

dot11HiddenNodeRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Beacon Request elements in dot11HiddenNodeReportTable,

greater than 0."

::= { dot11HiddenNodeReportEntry 1 }

dot11HiddenNodeRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report.  This

should be an exact match to the original dot11RRMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multiple

measurement reports."

::= { dot11HiddenNodeReportEntry 2 }

dot11HiddenNodeRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of Hidden Node Report has been

received on."

::= { dot11HiddenNodeReportEntry 3 }

dot11HiddenNodeMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Hidden Node

report."

::= { dot11HiddenNodeReportEntry 4 }

dot11HiddenNodeRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this measurement report was made."

::= { dot11HiddenNodeReportEntry 5 }

dot11HiddenNodeRprtChanBand OBJECT-TYPE

SYNTAX INTEGER(0..1)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Channel Band indicates the frequency band, taken from Table 0-2,

TGk Spec, in which this row of hidden node report has been

conducted.

0 : 2.4 GHz

1 : 5 GHz

Other values are reserved."

::= { dot11HiddenNodeReportEntry 6 }

dot11HiddenNodeRprtActualStartTime OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11HiddenNodeReportEntry 7 }

dot11HiddenNodeRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Hidden

Node Report was measured, expressed in TUs."

::= { dot11HiddenNodeReportEntry 8 }

dot11HiddenNodeRprtHiddenSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the destination of the 802.11 frame received by

the measuring STA without detecting the subsequent acknowledgement."

::= { dot11HiddenNodeReportEntry 9 }

dot11HiddenNodeRprtNumFrames OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Number of frames is a count of the frames received from the hidden

station address."

::= { dot11HiddenNodeReportEntry 10 }

-- ********************************************************************

-- * End of dot11HiddenNodeReport	TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11MedSenseTimeHistogramReport	TABLE

-- ********************************************************************

dot11RRMMedSenseTimeHistogramReportMaxTableSize OBJECT-TYPE

SYNTAX		Unsigned32(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Identifies the number of measurement reports that can be entered into

MedSenseTimeHistogramReportTable."

::= { dot11RRMReport 12 }

dot11MedSenseTimeHistogramReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MedSenseTimeHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Medium Sensing Time Histogram

report entries that have been received by the MLME. The report

tables 	shall be maintained as FIFO to preserve freshness, thus

this attribute can be overridden by memory constraints or other

implementation constraints determined by the vendor."

::= { dot11RRMReport 13 }

dot11MedSenseTimeHistogramReportEntry OBJECT-TYPE

SYNTAX Dot11MedSenseTimeHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MedSenseTimeHistogramReportTable

indexed by dot11MedSenseTimeHistogramReportIndex."

INDEX { dot11MedSenseTimeHistogramReportIndex }

::= { dot11MedSenseTimeHistogramReportTable 1 }

Dot11MedSenseTimeHistogramReportEntry ::=

SEQUENCE {

dot11MedSenseTimeHistogramReportIndex         			Unsigned32,

dot11MedSenseTimeHistogramReportRqstToken     			OCTET STRING,

dot11MedSenseTimeHistogramReportIfIndex       			INTEGER,

dot11MedSenseTimeHistogramMeasuringSTAAddr    			MacAddress,

dot11MedSenseTimeHistogramReportChanNumber    			INTEGER,

dot11MedSenseTimeHistogramReportChanBand      			INTEGER,

dot11MedSenseTimeHistogramReportActualStartTime     	TSFtype,

dot11MedSenseTimeHistogramReportMeasurementDuration 	Unsigned32,

dot11MedSenseTimeHistogramReportSubtype       			INTEGER,

dot11MedSenseTimeHistogramReportRPIThreshold  			INTEGER,

dot11MedSenseTimeHistogramReportBinOffset     			INTEGER,

dot11MedSenseTimeHistogramReportBinInterval   			INTEGER,

dot11MedSenseTimeHistogramReportNumBin        			INTEGER,

dot11MedSenseTimeHistogramReportNumEvent   	      		INTEGER,

dot11MedSenseTimeHistogramReportHistogram      		OCTET STRING

}

dot11MedSenseTimeHistogramReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Medium Sensing Time Histogram Report table entries in

dot11MedSenseTimeHistogramReportTable, greater than 0."

::= { dot11MedSenseTimeHistogramReportEntry 1 }

dot11MedSenseTimeHistogramReportRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report.  This

should be an exact match to the original dot11RRMRqstToken attribute."

::= { dot11MedSenseTimeHistogramReportEntry 2 }

dot11MedSenseTimeHistogramReportIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of Medium Sensing Time Histogram

report has been received on."

::= { dot11MedSenseTimeHistogramReportEntry 3 }

dot11MedSenseTimeHistogramMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Medium

Sensing Time Histogram report."

::= { dot11MedSenseTimeHistogramReportEntry 4 }

dot11MedSenseTimeHistogramReportChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this row of report has been detected."

::= { dot11MedSenseTimeHistogramReportEntry 5 }

dot11MedSenseTimeHistogramReportChanBand OBJECT-TYPE

SYNTAX INTEGER(0..1)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Channel Band indicates the frequency band, taken from Table 0-2,

TGk Spec, in which this row of beacon report has been conducted.

0 : 2.4 GHz

1 : 5 GHz

Other values are reserved."

::= { dot11MedSenseTimeHistogramReportEntry 6 }

dot11MedSenseTimeHistogramReportActualStartTime OBJECT-TYPE

SYNTAX TSFtype

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11MedSenseTimeHistogramReportEntry 7 }

dot11MedSenseTimeHistogramReportMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Medium

Sensing Time Historgram Report was measured, expressed in TUs."

::= { dot11MedSenseTimeHistogramReportEntry 8 }

dot11MedSenseTimeHistogramReportSubtype OBJECT-TYPE

SYNTAX INTEGER(0..3)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Measurement SubType for this row

of MedSenseTimeHistogramReport. SubType is defined as:

0: RPI Time Histogram

1: CCA Idle Time Histogram

2: CCA Busy Time Histogram

3: NAV Busy Time Histogram

other values are reserved."

::= { dot11MedSenseTimeHistogramReportEntry 9 }

dot11MedSenseTimeHistogramReportRPIThreshold OBJECT-TYPE

SYNTAX INTEGER(0..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the RPI threshold that was used for

generating this row of MedSenseTimeHistogramReport.

0: -87dBm observed at antenna

1: -82dBm observed at antenna

2: -77dBm observed at antenna

3: -72dBm observed at antenna

4: -67dBm observed at antenna

5: -62dBm observed at antenna

6: -57dBm observed at antenna

other values are reserved."

::= { dot11MedSenseTimeHistogramReportEntry 10 }

dot11MedSenseTimeHistogramReportBinOffset OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the bin offset in microseconds."

::= { dot11MedSenseTimeHistogramReportEntry 11 }

dot11MedSenseTimeHistogramReportBinInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the bin interval duration in multiples of

aSlotTime."

::= { dot11MedSenseTimeHistogramReportEntry 12 }

dot11MedSenseTimeHistogramReportNumBin OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of bins."

::= { dot11MedSenseTimeHistogramReportEntry 13 }

dot11MedSenseTimeHistogramReportNumEvent OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of medium sensing events.

The purpose of this value is to allow the calculation of absolute

times, as well as providing a confidence indicator."

::= { dot11MedSenseTimeHistogramReportEntry 14 }

dot11MedSenseTimeHistogramReportHistogram OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..1000))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The histogram contains a variable number octet indicating the

probability of medium sensing events during the corresponding bin

interval."

::= { dot11MedSenseTimeHistogramReportEntry 15 }

-- ********************************************************************

-- * End of dot11MedSenseTimeHistogramReport	TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11STAStatisticsReport		TABLE

-- ********************************************************************

dot11RRMSTAStatisticsReportMaxTableSize OBJECT-TYPE

SYNTAX Unsigned32(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the number of measurement reports in

STAStatsReportTable."

::= { dot11RRMReport 14 }

dot11STAStatisticsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of STA Statistics reports that

have been received by the MLME. The report tables shall be

maintained as FIFO to preserve freshness, thus this attribute can

be overridden 	by memory constraints or other implementation constraints determined by the vendor."

::= { dot11RRMReport 15 }

dot11STAStatisticsReportEntry OBJECT-TYPE

SYNTAX Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11STAStatisticsReportTable

Indexed by dot11STAStatisticsReportIndex."

INDEX { dot11STAStatisticsReportIndex }

::= { dot11STAStatisticsReportTable 1 }

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex                    Unsigned32,

dot11STAStatisticsReportToken                    OCTET STRING,

dot11STAStatisticsIfIndex                        INTEGER,

dot11STAStatisticsSTAAddress                     MacAddress,

dot11STAStatisticsMeasurementDuration            Unsigned32,

dot11STAStatisticsTransmittedFragmentCount       Integer32,

dot11STAStatisticsMulticastTransmittedFrameCount Integer32,

dot11STAStatisticsFailedCount                    Integer32,

dot11STAStatisticsRetryCount                     Integer32,

dot11STAStatisticsMultipleRetryCount             Integer32,

dot11STAStatisticsFrameDuplicateCount            Integer32,

dot11STAStatisticsRTSSuccessCount                Integer32,

dot11STAStatisticsRTSFailureCount                Integer32,

dot11STAStatisticsACKFailureCount                Integer32,

dot11STAStatisticsReceivedFragmentCount          Integer32,

dot11STAStatisticsMulticastReceivedFrameCount    Integer32,

dot11STAStatisticsFCSCount                       Integer32,

dot11STAStatisticsTransmittedFrameCount          Integer32

}

dot11STAStatisticsReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for STA STatistics Report elements in

dot11STAStatisticsReportTable, greater than 0."

::= { dot11STAStatisticsReportEntry 1 }

dot11STAStatisticsReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the token that was specified in the

measurement request that generated this measurement report.  This

should be an exact match to the original dot11RRMRqstToken attribute.  Note that there may be multiple entries in the table that match this value since a single request may generate multiple measurement reports."

::= { dot11STAStatisticsReportEntry 2 }

dot11STAStatisticsIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of STA Statistics Report

has been received on"

::= { dot11STAStatisticsReportEntry 3 }

dot11STAStatisticsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this STA Statistics

report"

::= { dot11STAStatisticsReportEntry 4 }

dot11STAStatisticsMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the STA

Statistics was measured, expressed in TUs."

::= { dot11STAStatisticsReportEntry 5 }

dot11STAStatisticsTransmittedFragmentCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFragmentCount returned from the STA

in this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 6 }

dot11STAStatisticsMulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastTransmittedFrameCount returned from the

STA in this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 7 }

dot11STAStatisticsFailedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FailedCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 8 }

dot11STAStatisticsRetryCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RetryCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 9 }

dot11STAStatisticsMultipleRetryCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MultipleRetryCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 10 }

dot11STAStatisticsFrameDuplicateCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FrameDuplicateCount returned from the STA in

this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 11 }

dot11STAStatisticsRTSSuccessCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSSuccessCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 12 }

dot11STAStatisticsRTSFailureCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSFailureCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 13 }

dot11STAStatisticsACKFailureCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ACKFailureCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 14 }

dot11STAStatisticsReceivedFragmentCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ReceivedFragmentCount returned from the STA in

this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 15 }

dot11STAStatisticsMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastReceivedFrameCount returned from the

STA in this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 16 }

dot11STAStatisticsFCSCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FCSErrorCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 17 }

dot11STAStatisticsTransmittedFrameCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFrameCount returned from the STA in

this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 18 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport	TABLE

-- ********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Configuration Information

-- ********************************************************************

dot11RRMConfig OBJECT IDENTIFIER ::= { dot11RadioResourceManagement 3 }

-- ********************************************************************

-- * dot11APChannelReport	TABLE

-- ********************************************************************

dot11APChannelReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"AP Channel Report information. In tablular form to

allow for multiple instances on an agent."

::= { dot11RRMConfig 1 }

dot11APChannelReportEntry OBJECT-TYPE

SYNTAX Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11APChannelReportTable.  Each entry in the

table represents the information provided in the Probe Response for

a given IEEE 802.11 interface.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11APChannelReportTable 1 }

Dot11APChannelReportEntry ::=

SEQUENCE {

dot11APChannelRprtRegulatoryDomain	INTEGER,

dot11APChannelRprtNumberofChannels	INTEGER,

dot11APChannelRprtChannelMap			OCTET STRING

}

dot11APChannelRprtRegulatoryDomain OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Regulatory Domain is the domain in which the channel map is valid.

The reserved bits on the channel map will reflect the PHY frequency

channel plan as in Table 64 for 802.11b and Table 88 for 802.11a."

::= { dot11APChannelReportEntry 1 }

dot11APChannelRprtNumberofChannels OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This corresponds to the number of channels the ESS supports."

::= { dot11APChannelReportEntry 2 }

dot11APChannelRprtChannelMap OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(16))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute corresponds to the specific channels that are

supported in the ESS as shown in Table 0-2. Each bit corresponds to

a different channel"

::= { dot11APChannelReportEntry 3 }

-- ********************************************************************

-- * End of dot11APChannelReportTable	TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RRMSiteReport	TABLE

-- ********************************************************************

dot11RRMSiteReportMaxTableSize OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS	read-only

STATUS current

DESCRIPTION

"Identifies the number of entries that can be entered into

RRMSiteReportTable.  Could use a value of zero to indicate that

there are no external BSSID's"

::= { dot11RRMConfig 3 }

dot11RRMSiteReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RRMSiteReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains pertinent information on a collection of BSSID's

that are candidates to which STA's can roam."

::= { dot11RRMConfig 4 }

dot11RRMSiteReportEntry OBJECT-TYPE

SYNTAX Dot11RRMSiteReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RRMSiteReportTable

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex, dot11RRMSiteReportIFIndex }

::= { dot11RRMSiteReportTable 1 }

Dot11RRMSiteReportEntry ::=

SEQUENCE {

dot11RRMSiteReportIFIndex				INTEGER,

dot11RRMSiteReportBSSIDReporter 		MacAddress,

dot11RRMSiteReportMatchStatus		Unsigned32,

dot11RRMSiteReportCurentChannel		INTEGER,

dot11RRMSiteReportPhyType				INTEGER,

}

dot11RRMSiteReportIFIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for RRM Site Report elements in dot11RRMSiteReportTable,

greater than 0."

::= { dot11RRMSiteReportEntry 1 }

dot11RRMSiteReportBSSIDReporter OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Site Report BSSID Reporter MAC address corresponding to the

site report row"

::= { dot11RRMSiteReportEntry 2 }

dot11RRMSiteReportMatchStatus OBJECT-TYPE

SYNTAX Unsigned32 (0..26)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Is a match to the STA sending the site report.

Valid values are

00 = No Match

02 = Match on ESS

08 = Match on capabilities

10 = Match on ESS and capabilities

16 = Match on supported rates

18 = Match on ESS and supported rates

24 = Match on capabilities and supported rates

26 = Match on ESS, capabilities, and supported rates."

::= { dot11RRMSiteReportEntry 3 }

dot11RRMSiteReportCurentChannel OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is the current operating channel of the STA returning the

report"

::= { dot11RRMSiteReportEntry 4 }

dot11RRMSiteReportPhyType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Returns the PHY type of the reporting STA."

::= { dot11RRMSiteReportEntry 5 }

-- ********************************************************************

-- * End of dot11APChannelReport 	TABLE

-- ********************************************************************