IEEE P802.11
Wireless LANs

802.11 TGn Functional Requiremenets

Date:	November 13, 2003

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

Abstract

This document defines functional requirements that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document.)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P. Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2A, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Hervé Bonneville | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | bonneville@tcl.ite.mee.com |
| Javier del Prado | Philips | 345 Scarborough Rd, Briarcliff Manor, NY, 10510, USA | +1 914 945 6000 | +1 914 945 6580 | javier.delprado@philips.com |
| Paul Feinberg | Sony | 1 Sony Drive MD TA1-5 Park Ridge, NJ 07656 | +1 201 930-6316 | +1 201 930-6397 | Paul.Feinberg@am.sony.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Skeleton document created | 20 October 2003 |
| R1 | New content partially presented to discussed at the FRCC telecon. Includes updates agreed at the telecon. | 21 October 2003 |
| R2 | Comments on R1 merged in. This involved rewording and re-organisation of the comments so that the resulting combined document made sense. | 31 October 2003 |
| R3 | Changes approved at the 4 Nov 2003 FRCC telecon merged in. Approved changes are shown without change tracking. Comments made by the FRCC which have not been considered are shown as tracked changes. | 10 November 2003 |
| R4 | Comments by WFA merged in | 12 November 2003 |
| R5 | Updated during 12 Nov 2003 TGn session |  |
| R6 | Completed survey of FR at 13 Nov 2003 TGn session |  |
| R7 | Votes on FR | 13 November 2003 |
| R8 | Cleaned up – FRs failing votes removed | 13 November 2003 |
| R9 | Cleaned up at telecon – obsolete notes removed | 18 November 2003 |

# Introduction

## Purpose of document (Informative)

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in this document.

A partial proposal may meet some of the requirements stated here.

## Form of Disclosure

A proposal shall declare its compliance with these functional requirements using the template defined below in clause 3.

## Relationship to Comparison Criteria and Usage Models

Individual functional requirements may reference terms or metrics defined in the 802.11 TGn comparison criteria document [2], or the simulation scenarios defined in the 802.11 TGn Usage Models document [3].

## Requirements for this document

(This section may be removed at a later date.  It is really only relevant while we are writing this document)

The functional requirements document shall include a set of requirement statements.  Each of these:

- Shall be defined unambiguously

- Can be verified from an examination of the proposed submission or a reasonable simulation environment.

- Are compliant to the 802.11 HT PAR [1] and 5C [2]

Note, at the September TGn meeting, it was decided that this document should relate only to mandatory features of the proposal.  Optional features, qualities, performances are considered in [2].

Functional Requirements

| Number |  | Name |  |  | Requirement |  |  |  |  | Status of Requirement | Straw Poll on 13 Nov 2003 | Notes (informative) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Single Link HT rate supported |  |  | Proposal supports a single link throughput of 100Mbps at the top of the MAC SAP measured in the context of the simulation scenario #16. |  |  |  |  | Agreed at 4 Nov 2003 telecon, but see note. 20+20/2+7 Add usage model for single-link case and reference from here. | 40/8 passes |  |  |
| 2 |  | HT rate supported in 20MHz channel |  |  | Proposal supports at least one mode of operation that supports 100Mbps throughput at the top of the MAC SAP in a 20MHz channel. |  |  |  |  | Rahul proposal Agreed at 4 Nov 2003 telecon Agreed at 12 Nov TGn. | 45/5 passes | The regulatory bodies of many countries do not allow operation of a single device using several channels |  |
| 3 |  | Supports 5GHz bands |  |  | Protocol supports 5GHz bands (including those supported by .11a) |  |  |  |  | Straw poll: 92/1 on 12 Nov 2003 | 48/0 passes |  |  |
| 4 |  | .11a backwards compatibility |  |  | Some of the modes of operation defined in the proposal shall be backwards compatible and interoperable with 802.11a. |  |  |  |  | 34+43/1 on 12 Nov 2003 | 48/1 passes | Note: backwards compatibility means that it supports all the mandatory modes of the .11a standard. (vote 54/2) |  |
| 5 |  | .11g backwards compatibility |  |  | If it supports 2.4 GHz operation, some of the modes of operation defined in the proposal shall be backwards compatible and interoperable with 802.11g. |  |  |  |  | 25+44/1+8 | 50/0 passes | Note: backwards compatibility means that it supports all the mandatory modes of the .11g standard. (vote: 54/2) |  |
| 6 |  | Control of support for legacy STA from .11n AP |  |  | A .11n AP can be configured to reject or accept associations from legacy STA because they are legacy STA. |  |  |  |  | WFA proposal 49/17 on 12 Nov 2003 | 33/9 passes |  |  |
| 7 |  | .11e QoS support |  |  | The proposal shall permit implementation of the 802.11e options within a .11n STA |  |  |  |  | John K proposal 70/6 on 12 Nov 2003. | 39/0 passes |  |  |
| 8 |  | Spectral Efficiency |  |  | The highest throughput mode of the proposal shall achieve a spectral efficiency of at least 3 bps/Hz for the PSDU |  |  |  |  | Vote 73/0 on 13 Nov 2003 | 53/0 passes |  |  |
| 9 |  | Compliance to PAR |  |  | The proposal complies with all the mandatory requirements of the PAR [5] and 5 Criteria [6] |  |  |  |  | Voted yes 71/0 13 Nov 2003 | 56/0 passes |  |  |

# Template for Complete Submissions

The results for a complete submission shall include a statement of coverage of the functional requirements and results for simulation scenarios following the outline presented here.

## Coverage of Functional Requirements

Each requirement shall be declared to be covered or not covered.  For each requirement relating to measurements, a reference to the document/page showing the appropriate performance measurements should be added.

| Number | Name | Requirement | Coverage | Results Reference |
| --- | --- | --- | --- | --- |
| R1 | TBD | TBD |  |  |
| Rest of table will be filled in when the functional requirements are agreed… |  |  |  |  |

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/814, TGn Comparison Criteria

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/161r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)