IEEE P802.11
Wireless LANs

Beacon Request for Scanning All Channels

Date:	May 2004

Author:	Emily Qi, Mustafa Demirhan, Mousumi Hazra, Nandakishore Kushalnagar

Intel Corporation

2111 N.E 25th Avenue

Hillsboro, OR 97124

e-Mail: {emily.h.qi, Mustafa.demirhan, Mousumi.m.Hazra, Nandakishore.Kushalnagar } @intel.com

Abstract

This paper proposes normative text for submission 11-04-0493-00-000k-beacon-request-for-scanning-all-channels.ppt.

#### Proposed Normative Text:

7.3.2.21.6 Beacon Request

[Add the following texts following the second paragraph of 7.3.2.21.6]

If Channel Number is 0, it indicates that the receiving STA shall pursue iterative measurements for all channels in the specified channel band.

[Add the following texts following figure X-2 – Measurement Interval field:]

If channel Number is 0 and scan mode is set to Active mode or Passive mode, the receiving STA shall iteratively conduct measurements for all channels in the specified channel band. The two consecutive measurements are separated by the measurement period. The receiving STA shall randomly select a channel number to start with and pursue iterative measurements for all channels during the measurement interval. As an example, the receiving STA may start with channel 4 (a random channel number in the channel band) to conduct the measurement. The receiving STA shall pursue measurement at channel 5, 6, etc. in the next measurement periods.

If channel Number is 0 and scan mode is Beacon Table mode, the measuring STA shall return beacon reports containing the current contents of all stored beacon information for all channels without performing additional measurements.

7.3.2.22.6 Beacon Report

[Add the following texts following the second paragraph of 7.3.2.22.6:]

If Channel Number is 0, the receiving STA shall send Beacon Report with multiple Beacon Report IEs to cover all channel information.

11.7.7 Specific measurement usage

11.7.7.1 Beacon Report

[Add the following texts following the last paragraph of 11.7.7.1:]

If Channel number is 0 and measurement mode is Active scan mode or Passive Scan mode, the receiving station shall perform the following procedures as described in figure X-4. In Beacon Table mode, the measuring STA shall return beacon reports containing the current contents of all stored beacon information for all channels without performing additional measurements.