IEEE P802.11
Wireless LANs

Conference Call Minutes Task Group K April 21, 2004

Date:

April 21, 2004

Author:

Paul Gray
AirWave Wireless, Inc.
1700 El Camino Real Suite 500

San Mateo, CA 94025
Phone: 650-286-6107
Fax: 650-286-6101
e-Mail: paul@airwave.com

Abstract

Teleconference Minutes for Task Group K.

Detailed minutes follow:

Wednesday, April 21, 2004 – 8:00 AM PST

- Chair calls the conference call to order at 8:00 AM

- Attendance Paine, Gray, Olson, Barber, Black, Worstell, Pope, Johnson, Bagby, Klein, Qi

- Agenda

  - Comment Resolution

Resolution of Comments

Clause 10.3 - Black

Problem – There are no MLME primitives for site reporting. How is a site report request initiated?

Remedy – Consider adding primitives for site reporting.

Resolution – Open – Assigned to Simon Black, because editor needs more text.

Clause 10.3.14.3.2 – Simon Black

Problem - P35, L6 Valid range of Dialog Token should be 1 - 255, not 0 - 255 (0 is reserved for autonomous reporting)

Remedy – Correct Range

Comment – The dialog token text in 802.11h clearly spells this out.

Resolution – Accept – Instruct editor to make change as described above.

Clause - 10.3.14.3.2 – Simon Black

Problem - P37, L1 Description column for Measurement Report Set. There is some text here which says 'Only present if Status = 0, or 1'

Remedy - Clarify, or delete

Comment – Never seen in the text before.

Resolution – Accept - Instruct editor to delete

Clause - 10.3.2.2.2 – Simon Black

Problem - P32, L16 'The BSSMeasurement elements are in the same order as the BSSDescription elements and map one to one with each of the BSSDescription elements'  I think what this is trying to achieve is mapping BSSDescriptions to BSSMeasurements one-one, but the wording is wrong since it refers to the elements in the descriptions/measurements.

Remedy - Suggest: 'There is one BSSMeasurement for each BSSDescription. BSSMeasurements within the BSSMeasurementSet are in the same order as the BSSDescriptions within the BSSDescriptionSet'.

Comment – This section was updated for RCPI measurement as probe request or probe response.

Resolution – accept – Instruct editor to make change as described above.

Clause - 10.3.2.2.2 – Simon Black

Problem - BSSMeasurementSet is a .11k addition and should only be present if dot11RadioMeasurementEnabled is true.

Remedy - Add conditional text to description.

Comment – Is it implied that this must be present on both the client as well as the AP.

Comment – The capability bit was only for the air interface and not intended for the internal interface.

Resolution – open – take off-line for additional research – assigned to Simon Black

Clause - 11.1.3 - Black

Problem - P37, L10 'will be set to 255 for a passively scanning STA' and P37, L14 'will be set to the RCPI value'

Remedy – Use 'shall' not 'will' in both cases

Resolution – Accept – Instruct editor to make change as described above.

Clause – 11.1.3 - Johnson

Problem - p37, l15 - remove space

Remedy - Remove space between response and the

Resolution – Accept – Instruct editor to make change as described above.

Clause - 11.1.3.2.1 – Black

Problem - P37, L20 'In case a DS Parameter Set…'

Remedy - Better as 'If a DS Parameter Set…

Resolution – Accept – Instruct the editor to make change as described above.

Clause 11.1.3.2.1 – Johnson

Problem – p37, l21 - add comma

Remedy – Add comma between request and a

Resolution – Accept – Instruct the editor to accept the change as described above.

Clause 11.13.2.2 – Black

Problem – P38 L5, for a .11k STA a BSSMeasurementSet is also returned with the BSSDescriptionSet. The text here just specifies what to do if the RCPI measurement (result) is not available.

Remedy - Add text to specify that a BSSMeasurementSet is also returned if dot11RadioMeasurementEnabled is true. Also add ‘result’ as in the comment.

Comment – There is no conditional on this behavior.

Resolution – open –

Comment – we should table the difficult comments and address in May meeting.

Comment – only handle editorial comments via cc.

Comment – find volunteers to work these more difficult comments.

Comment – lump editorial comments together and technical sections

Chair - officially ends meeting at 9:06.