IEEE P802.11
Wireless LANs

Measurement Duration

Date:	July 12th, 2004

Author:	Simon Black, Jon Edney
	Nokia

e-Mail: simon.black@intalk2k.com; jon.edney@intalk2k.com

Abstract

This paper provides normative text to address the ambiguity in measurement duration in IEEE802.11k/D0.16. Editing instructions in [bold] are for inclusion of this text into D0.16 and should not be included with the normative text.

## Normative Text

#### 7.3.2.21 Measurement Request element

[Replace Figure 46h with the following:]

|  | Reserved (0) Parallel | Enable | Request | Report | Duration Mandatory | Reserved (0) |
| --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3 | 4 | 45-7 |

Figure 46h—Measurement Request Mode field

[Insert the following text following the description of the Report bit (bit 3):]

- Duration Mandatory bit (bit 4) indicates whether the measurement duration contained within the Measurement Request should be interpreted as mandatory by the STA receiving the request. A value of 0 shall indicate that the duration requested is a target duration, but the requesting STA will accept measurement results taken over a shorter duration. A value of 1 shall indicate that the duration requested is a mandatory duration.

#### 7.3.2.21.4 Channel Load Request

[Change the final paragraph of 7.3.2.21.4 as follows:]

The Measurement Duration field shall be set to the duration of the requested measurement, expressed in TUs.

The Measurement Duration field shall be set equal to the preferred duration of the requested measurement, expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this shall be interpreted as a target measurement duration. See 11.7.4.

#### 7.3.2.21.5 Noise Histogram Request

[Change the final paragraph of 7.3.2.21.5 as follows:]

The Measurement Duration field shall be set to the duration of the requested measurement, expressed in TUs.

The Measurement Duration field shall be set equal to the preferred duration of the requested measurement, expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this shall be interpreted as a target measurement duration. See 11.7.4.

#### 7.3.2.21.6 Beacon Request

[Change the fifth paragraph of 7.3.2.21.6 as follows:]

The Measurement Duration field shall be set to the duration of the requested measurement, expressed in TUs.

The Measurement Duration field shall be set equal to the preferred duration of the requested measurement, expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this shall be interpreted as a target measurement duration. See 11.7.4.

#### 7.3.2.21.7 Frame Request

[Change the final paragraph of 7.3.2.21.7 as follows:]

The Measurement Duration field shall be set to the duration of the requested measurement, expressed in TUs.

The Measurement Duration field shall be set equal to the preferred duration of the requested measurement, expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this shall be interpreted as a target measurement duration. See 11.7.4.

#### 7.3.2.21.8 Hidden Node Request

[Change the final paragraph of 7.3.2.21.8 as follows:]

The Measurement Duration field shall be set to the duration of the requested measurement, expressed in TUs.

The Measurement Duration field shall be set equal to the preferred duration of the requested measurement, expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this shall be interpreted as a target measurement duration. See 11.7.4.

#### 7.3.2.21.9 Medium Sensing Time Histogram Request

[Change the fifth paragraph of 7.3.2.21.9 as follows:]

The Measurement Duration field shall be set to the duration of the requested measurement, expressed in TUs.

The Measurement Duration field shall be set equal to the preferred duration of the requested measurement, expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this shall be interpreted as a target measurement duration. See 11.7.4.

#### 7.3.2.21.10 STA Statistics Request

[Change the final paragraph of 7.3.2.21.10 as follows:]

The Measurement Duration field shall be set to the duration of the requested measurement, expressed in TUs. The Measurement, if accepted, shall be performed over the requested duration without regard to the Duration Mandatory bit in the Measurement Request Mode field. If a measurement over the requested duration is not possible, the request shall be refused.

### 11.7 Radio Measurement Procedures

[Insert new 11.7.4 and renumber all following sections:]

#### 11.7.4 Measurement Duration

If the Duration Mandatory bit is set to 1 in the Measurement Request mode field of a measurement request, the requested STA, if it accepts the request, shall perform the measurement over the Measurement Duration specified in the request. If the STA is unable to commit to making the measurement over the requested duration it shall refuse the request by sending a measurement report with the refused bit set in the Measurement Report Mode field. The measurement duration in the measurement report shall be equal to the requested measurement duration.

If the Duration Mandatory bit is set to 0 in the Measurement Request mode field of a measurement request, the actual measurement duration shall be at the discretion of the measuring STA, but shall not not exceed the Measurement Duration specified in the request. The duration over which the measurement was made shall be included in the measurement duration field of the measurement report.

All measurements shall be performed over one continuous time period.

## Annex A – PICS

### A.4 PICS proforma–IEEE Std 802.11, 1999 Edition9

[Insert new item RRM3.4 in Annex A, A.4.13 and renumber all following items in RRM3:]

#### A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM3 RRM3.1 RRM3.2 RRM3.3 RRM3.4 RRM3.45 RRM3.56 | General protocol for requesting and reporting of measurements Parallel Measurements Enable Measurement Requests Enable Autonomous Report Duration Mandatory Incapable Indication Refused Indication | 7.3.2.21, 7.3.2.22, 11.7, 11.7.6 7.3.2.21, 7.3.2.22 7.3.2.21, 11.7.6 7.3.2.21, 11.7.6 7.3.2.21, 11.7.4 7.3.2.22 7.3.2.22 | M M M M M M M | Yes, No Yes, No Yes, No Yes, No Yes, No Yes, No Yes, No |

## Annex D

[Amend Dot11RRMRequestEntry as follows:]

Dot11RRMRequestEntry ::=

SEQUENCE {

dot11RRMRqstIndex			Unsigned32,

dot11RRMRqstRowStatus		RowStatus,

dot11RRMRqstToken			OctetString,

dot11RRMRqstIfIndex		InterfaceIndex,

dot11RRMRqstType			INTEGER,

dot11RRMRqstTargetAdd		MacAddress,

dot11RRMRqstTimeStamp		TimeTicks,

dot11RRMRqstChanNumber		INTEGER,

dot11RRMRqstChanBand		INTEGER,

dot11RRMRqstRndInterval		Unsigned32,

dot11RRMRqstDuration		Unsigned32,

dot11RRMRqstParallel		TruthValue,

dot11RRMRqstEnable		TruthValue,

dot11RRMRqstRequest		TruthValue,

dot11RRMRqstReport		TruthValue,

dot11RRMRqstDurationMandatory	Truthvalue,

dot11BeaconRqstMode		INTEGER,

dot11RRMRqstBssid			MacAddress,

dot11RRMRqstMeasPeriodUnit	INTEGER,

dot11RRMRqstMeasurementPeriod	INTEGER,

dot11RRMRqstMeasIntervalUnit 	INTEGER,

dot11RRMRqstMeasurementInterval INTEGER,

dot11RRMRqstReportingCondition  INTEGER,

dot11RRMRqstThresholdOffset	INTEGER,

dot11RRMRqstHysteresis 		INTEGER,

dot11MedSenseType			INTEGER,

dot11MedSenseRCPIThreshold	INTEGER,

dot11MedSenseBinOffset		INTEGER,

dot11MedSenseBinInterval	INTEGER,

dot11MedSenseNumBin		INTEGER,

dot11STAStatRqstGroupID		INTEGER

}

[Add attribute Dot11RRMRqstDurationMandatory as follows, renumbering attributes in dot11RRMRequestEntry as required:]

dot11RRMRqstDurationMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the duration mandatory bit of the request frame that contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 13 }