IEEE P802.11
Wireless LANs

Draft Terms and Definitions for 802.11s

Date:   01 July 2004

Authors/Contributors:

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Tricci So | Nortel Networks | 3500 Carling Ave., Ottawa ON K2H 8E9, Canada | +613-763-9639 |  | tso@nortelnetworks.com |
| Kevin Dick | Nortel Networks | P.O.Box 3511 Station C, Ottawa ON K1Y 4H7 Canada | +613-763-4366 |  | kdick@nortelnetworks.com |
| Jonathan Agre | Nortel Networks | College Park, MD, USA | +301-486-0978 |  | jagre@fla.fujitsu.com |
| W.Steven Conner | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. | +503-264-8036 |  | w.steven.conner@intel.com |
| Guido R. Hiertz | Aachen University | Kopernikusstr. 16 52064 Aachen Germany | +49-241-80-25-82-9 |  | hiertz@ieee.org |
| Donald E. Eastlake III | Motorola Laboratories | 111 Locke Dr., Marlboro, MA 01752, U.S.A. | +508-786-7554 |  | Donald.eastlake@motorola.com |
| Tyan-Shu Jou | Janusys Networks | 502 Lowell Ave., Palo Alto, CA 94301, U.S.A. | +919-656-2945 |  | tsjou@janusysnetworks.com |
| Ted Kuo | Janusys Networks | 502 Lowell Ave., Palo Alto, CA 94301, U.S.A. | +650-387-0589 |  | ted@janusysnetworks.com |
| Juan Carlos Zuniga | InterDigital Canada Ltd, | 1000 Sherbrooke W., 10th Fl., Montreal QC, H3A 3G4, Canada | +514-904-6251 |  | juanCarlos.zuniga@interdigital.com |
| Lily Yang | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. | +503-264-8813 |  | lily.l.yang@intel.com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave., Sunnyvale, CA 94085 | +408-331-6814 |  | audeh@tropos.com |
| Narasimha Chari | Tropos Networks | 555 Del Rey Ave., Sunnyvale, CA 94085 | +408-331-6814 |  | chari@tropos.com |
| Kazuyuki Sakoda | Sony Corporation | Oval Court Ohsaki MW 2-17-1 Higashigotanda Shinagawa-ku Tokyo 141-0022 Japan | +81-3-6409-3201 |  | sako@wcs.sony.co.jp |
| Hidenori Aoki | NTT DoCoMo. Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-6526 |  | aokihid@nttdocomo.co.jp |
| Yoichi Matsumoto | NTT DoCoMo. Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-3894 |  | matsumotoyou@nttdocomo.co.jp |
| Koji Omae | NTT DoCoMo. Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-3890 |  | omae@mlab.yrp.nttdocomo.co.jp |
| Kue Wong | Nortel Networks | 3500 Carling Ave., Ottawa ON K2H 8E9, Canada | +613-763-2515 |  | jkwong@nortelnetworks.com |

Abstract

This document defines terminology for 802.11 TGs.

# Terminology and Definitions

# Revision History

| ### Revision | ### Comments | ### Date | ### Authors | ### Editor |
| --- | --- | --- | --- | --- |
| R0 | Initial Ad-Hoc Team draft for Core Terms & Definitions | 01 July 2004 | See above author list…. | Tricci So |
| R1 | Updated draft as the result of July04 face-to-face pre-meeting for finalizing the joint contribution | 11 July 2004 | See above author list…. | Tricci So |
| R2 | Updated draft as the result of July04 task group meeting for this presentation | 13 July 2004 | See above author list… | Tricci So |

# Abstract

This document includes a collection of  “core” terms and definitions related to IEEE 802.11s that was agreed among the members of the ad-hoc team.  The purpose of this document is to promote consistent use of new terminologies to describe 802.11s technology.  The definitions in this document will eventually be integrated into the TGs amendment draft.

# Issues

|  | ### Issue | ### Status |
| --- | --- | --- |
| 1 |  |  |

# Terminology and Definitions

This section defines some of the “core” terms and definitions that are to be used to describe 802.11s core concepts.

Introduction

This document includes a collection of terms and definitions related to IEEE 802.11s. The purpose of this document is to promote consistent use of new terminology to describe 802.11s technology. The definitions in this document will eventually be integrated into the TGs amendment draft.

The following two figures describe the concepts and a sample configuration of the core terms and definitions that are proposed by the ad-hoc team, respectively.

Figure 1:  Proposed  802.11s Terminology Diagram

Figure 2:  Example of the Mesh Portal Relationship with the WLAN Mesh and with the 802.11 Functional and Logical Components

The following core terms are used to describe IEEE 802.11s basic concepts.

- WLAN Mesh – A WLAN Mesh is an IEEE 802.11-based WDS which is part of a DS, consisting of a set of two or more Mesh Points interconnected via IEEE 802.11 links and communicating via the WLAN Mesh Services. A WLAN Mesh may support zero or more entry points (Mesh Portals), automatic topology learning and dynamic path selection (including across multiple hops).

- WLAN Mesh Services – The set of services provided by the WLAN Mesh that support the control, management, and operation of the WLAN Mesh, including the transport of MSDUs between Mesh Points within the WLAN Mesh. WLAN Mesh Services supplement DSS (Distribution System Services).

- Mesh Point - Any IEEE 802.11 entity that contains an IEEE 802.11–conformant Medium Access Control (MAC) and Physical Layer (PHY) interface to the Wireless Medium (WM), is within a WLAN Mesh, and supports WLAN Mesh Services.

- Mesh AP  - Any Mesh Point that is also an Access Point.

- Mesh Portal  - A point at which MSDUs exit and enter a WLAN Mesh to and from other parts of a DS or to and from a non-802.11 network. A Mesh Portal can be collocated with an IEEE 802.11 portal.

- Mesh Link - A direct IEEE 802.11 link between two Mesh Points.

- Mesh Path - A concatenated set of connected Mesh Links from a source Mesh Point to a destination Mesh Point.

- Mesh Path Selection – The process of selecting Mesh Paths.

- Path Metric –Criteria used for Mesh Path Selection.

- Mesh Topology – A graph consisting of the full set of Mesh Points and Mesh Links in a WLAN Mesh.

- Mesh Neighbor - Any Mesh Point that is directly connected to another Mesh Point with a Mesh Link.

- Mesh Unicast - Frame forwarding mechanism for transporting MSDUs to an individual Mesh Point within a WLAN Mesh.

- Mesh Multicast - Frame forwarding mechanism for transporting MSDUs to a group of Mesh Points within a WLAN Mesh.

- Mesh Broadcast - Frame forwarding mechanism for transporting MSDUs to all Mesh Points within a WLAN Mesh.