IEEE P802.11
Wireless LANs

Neighbor Report Generation

Date:	14th July 2004

Author:	Peyush Agarwal
	STMicroelectronics
	Phone: +91 9818686345
	e-Mail: peyush.agarwal@st.com

Abstract

This submission provides draft text for a mechanism, for APs, to generate the Neighbor report during their initialization process.

Neighbor Report generation

This clause defines a procedure by which the Neighbor APs can generate Neighbor Report based on the 802.11 networks available within their range. The Neighbor Report generation is performed only during the BSS initialization. The procedure utilizes synchronization mechanism, with modifications, along with the active scan procedure to collect the neighbor information.

During MAC initialization or the MAC reset process, the AP shall send a Beacon frame with an additional Information Element, ‘Original channel Number’, on all the channels supported in the regulatory domain. The ‘Original Channel Number’ information element shall identify the channel number that the AP will finally operate on. The AP shall then follow the “Active scan” procedure, as defined by IEEE–802.11: clause 11.1.3.2.2, to send a Probe Request with broadcast BSSID and DA.

An AP receiving a Beacon frame with ‘Original channel Number’ information element shall use channel number value as present in the ‘Original channel Number’ IE, to store relevant information received in the Beacon frame. The initializing AP shall use received Probe Response frames to store relevant information. APs shall store the information in the Neighbor Report Table (dot11RRMNeighborReportTable).

Add the following information element at the end of Table 20 Element IDs

| Information element | Element ID |
| --- | --- |
| Original Channel Number | To be assigned |

Add the following at the end 7.3.2.*

#### 7.3.2.* Original Channel Number

This Information element shall be included in the Beacon frame only on the receipt of MLME-START.request or MLME-RESET.request.

|  | Element ID | Length | Channel |
| --- | --- | --- | --- |
| Octets: | To be assigned during letter ballot | 1 | 1 |

The Channel identifies the channel number on which the AP is finally supposed to start its BSS.