IEEE P802.11
Wireless LANs

TGn MIMO-OFDM PHY Partial Proposal-Specifications

Date:	August 13, 2004

Authors: Sumei SUN, Chin Keong Ho, Patrick Fung, Yuan Li, Yan Wu, Zhongding Lei, Woon Hau Chin, Ying-Chang Liang, Francois Chin

Institute for Infocomm Research
	21 Heng Mui Keng Terrace, Singapore 119613
	Phone: 65-68747588
	Fax: 65-67768109
	e-Mail: sunsm@i2r.a-star.edu.sg

Abstract

In this document, the multiple-input multiple-output (MIMO) orthogonal frequency division multiplexing (OFDM) physical layer (PHY) is proposed for the high throughput wireless local area networks (WLAN). The key points in the proposal are:

- OFDM modulation over 40MHz channel with FFT size of 128;

- Peak data rate of 216 Mbps;

- 2×2 MIMO supporting both spatial multiplexing and orthogonal STBC in mandatory mode;

- 4×2 MIMO from access point to terminal station (downlink) supporting groupwise STBC (GSTBC), orthogonal STBC, antenna beamforming, and antenna selection in optional mode;

- Linear pre-transform in both frequency and spatial domain to exploit the frequency and spatial diversities;

- Efficient training signal design (preambles) that supports robust frequency and timing synchronization and channel estimation;

- Bit-interleaved coded modulation (BICM) with K=7 convolutional code as mandatory mode, and low-density parity check (LDPC) code as optional mode;

- Backward compatible with IEEE 802.11a;

- Support of concurrent transmissions of two 11a signals in downlink with novel subcarrier arrangement.

# Table of Contents

Table of Contents	2

List of Tables	4

List of Figures	5

References	6

Definitions, symbols and abbreviations	6

Definitions	6

Symbols	6

Abbreviations	6

1. Introduction	8

1.1 Scope	9

2. MIMO-OFDM PLCP Sublayer	9

2.1 Introduction	9

2.2 PLCP Frame Format	9

2.2.1 Overview of the PPDU encoding process	10

2.2.2 Rate-dependent parameters	12

2.2.3 Timing related parameters	15

2.2.4  Mathematical conventions in the signal description	16

2.2.5 Discrete time implementation consideration	17

2.2.5.1 Mandatory	17

2.2.5.2 Optional	18

2.3 PLCP preamble (SYNC)	19

2.3.1 Preambles for the Mandatory Mode Subcarrier Arranment	20

2.3.2 Preambles for the Optional Mode Subcarrier Arrangement	22

2.4 Signal field (SIGNAL)	24

2.4.1 Data RATE	25

2.4.2 PLCP length field (LENGTH)	25

2.4.3 Parity (P), Reserved (R), and Signal tail (SIGNAL TAIL)	25

2.4.4 Service field (SERVICE)	25

2.5 Data field	25

2.5.1 PPDU tail bit field (TAIL)	25

2.5.1.1 When convolutional code is used	25

2.5.1.2 When LDPC code is used	26

2.5.2 Pad bits (PAD)	26

2.5.3 PLCP DATA scrambler and descrambler	26

2.5.4 FEC	27

2.5.4.1 Convolutional Encoder	27

2.5.4.2 LDPC Encoder	28

2.5.5 Data interleaving	29

2.5.5.1 When convolutional code is used	30

2.5.5.2 When LDPC code is used	32

2.5.6 Subcarrier modulation mapping	33

2.5.7 Pilot subcarriers	35

2.5.8 Pre-transform	36

2.5.9 OFDM modulation	38

2.6 MIMO architectures	40

2.6.1 2×2 spatial multiplexing	40

2.6.2 2×2 STBC	41

2.6.2.1 Time-Domain STBC Implementation	41

2.6.3 4×2 GSTBC	42

2.6.4 4×2 STBC	43

2.6.4.1 Time-Domain STBC Implementation for 4×2 STBC	43

2.6.5 Beamforming	44

2.6.6 Antenna selection	45

ANNEX A Generation of short training preamble for mandatory mode of subcarrier arrangement	47

ANNEX B Generation of long training preamble for mandatory mode of subcarrier arrangement	49

ANNEX C Generation of short training preamble for optional mode of subcarrier arrangement	51

ANNEX D Generation of long training preamble for optional mode of subcarrier arrangement	53

ANNEX E H1 matrix of LDPC	55

# List of Tables

Table 1 - Rate-dependent parameters for 2×2 SM MIMO-OFDM	13

Table 2 - Rate-dependent parameters for 2×2 STBC MIMO-OFDM	14

Table 3 - Rate-dependent parameters for 4×2 GSTBC MIMO-OFDM	14

Table 4 - Rate-dependent parameters for 4×2 STBC MIMO-OFDM	15

Table 5 - Timing-related parameters	15

Table 6 - Construction of long preamble for mandatory mode subcarrier arrangement	22

Table 7 - Modulation Dependent Normalization Factor KMOD	34

Table 8 - BPSK Encoding Table	34

Table 9 - QPSK Encoding Table	35

Table 10 - 16QAM Encoding Table	35

Table 11 - 64QAM Encoding Table	35

Table 12 - Conventional STBC OFDM Encoding	41

Table 13 - Modified STBC OFDM Encoding	42

Table 14 - Time domain STBC OFDM Encoding	42

Table 15 - Antenna Selection Index	46

Table 16 - Frequency domain representation of the short training preamble for mandatory mode subcarrier arrangement	47

Table 17 - One period of IFFT of the short training preamble for mandatory mode subcarrier arrangement	48

Table 18 - Time domain representation of the long training symbol (sequences P+L) for mandatory mode subcarrier arrangement	49

Table 19 - Time domain representation of the long training symbol (sequences P-L) for mandatory mode subcarrier arrangement	50

Table 20 - Frequency domain representation of the short training preamble for optional mode of subcarrier arrangement	51

Table 21 - One period of IFFT of the short training preamble for optional mode of subcarrier arrangement	52

Table 22 - Time domain representation of the long training symbol (sequences P+L)	53

Table 23 - Time domain representation of the long training symbol (sequences P-L)	54

Table 24 - H1 matrix of PD-LDPC (1152, 864)	55

# List of Figures

Figure 1 - PPDU frame format	10

Figure 2 - Subcarrier arrangement of mandatory mode	18

Figure 3 - Subcarrier arrangement of optional mode	19

Figure 4 - OFDM training structure at each transmit antenna	19

Figure 5 - Data scrambler	27

Figure 6 - Convolutional Encoder (k=7)	28

Figure 7 - PD-LDPC encoder	28

Figure 8 - 2D Bit Interleaver and Subcarrier mapper	29

Figure 9 - 2D Bit Deinterleaver and demapper	30

Figure 10 - Bit permutation block I for LDPC	32

Figure 11 - Bit permutation block II for LDPC	32

Figure 12 - Transmitter processing for 2×2 Spatial Multiplexing PT-FEC-OFDM	40

Figure 13 Transmitter processing for 2×2 STBC PT-OFDM	41

Figure 14 - Transmit processing of 4×2 GSTBC PT-FEC-OFDM	43

Figure 15 Transmit processing of 4×2 STBC PT-FEC-OFDM	43

Figure 16 - Block diagram of a beam multiplexing transmitter	44

Figure 17 - Illustration of Antenna Selection for the 4×2 System	45

# References

[1] S. M. Alamouti, “A simple transmit diversity technique for wireless communications”, IEEE JSAC, vol. 16, no. 8, pp. 1451 – 1458, October 1998

[2] V. Tarokh, H. Jafarkhani, and A. R. Calderbank, “Space-time block codes from orthogonal designs,” IEEE Trans. Inform. Theory, vol. 45, pp. 1456–1467, July 1999.

[3] IEEE std 802.11a-1999 (Supplement to IEEE Std 802.11 -1999), “Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: High-speed Physical Layer in the 5GHz Band”.

# Definitions, symbols and abbreviations

## Definitions

Parallel Stream: The independent data stream carrying different information that are processed (scrambled and FEC encoded) and transmitted through different antennas.

## Symbols

## Abbreviations

CONV		Convolutional Code

CP		Cyclic Prefix

FDM		Frequency Division Multiplexing

FFT		Fast Fourier Transform

GI		Guard Interval

GSTBC		Groupwise Space-Time Block Code

IFFT		Inverse Fast Fourier Transform

LDPC		Low-Density Parity Check

MIMO		Multiple-Input Multiple-Output

OFDM		Orthogonal Frequency Division Multiplexing

PSK		Phase Shift Keying

PT		Pre-Transform

QAM		Quadrature Amplitude Modulation

SM		Spatial Multiplexing

STBC		Space-Time Block Code

# 1. Introduction

This proposal specifies the physical layer (PHY) entity for the high throughput wireless local area network (LAN). An orthogonal frequency division multiplexing (OFDM) system employing multiple antennas at both the transmitter and the receiver (otherwise known as multiple-input multiple-output or MIMO-OFDM) is specified. The PHY layer is backward compatible with IEEE Std 802.11a-1999 PHY.

The radio frequency LAN system uses a 40MHz bandwidth, providing a wireless LAN with data payload communication capabilities of 12, 18, 24, 36, 48, 72, 96, 108, 144, 192, and 216 Mbit/s and all the data rates supported by IEEE Std 802.11a-1999 using a bandwidth of 20MHz. The support of transmitting and receiving at data rates of 12, 24, 48, 96 and 216 Mbps is mandatory.

The system uses 104 subcarriers that are modulated using binary or quadrature phase shift keying (BPSK/QPSK), 16-quadrature amplitude modulation (QAM), or 64-QAM. Bit-interleaved coded modulation (BICM) is used with the forward error correction (FEC) coding rate of 1/2, 2/3, or 3/4. Two FEC coding schemes are specified, with the convolutional code (CONV) as the mandatory mode, and low density parity check (LDPC) code optional. The bit interleaver is implemented in both the frequency and spatial domain. To further exploit the frequency and spatial domain diversities, a two-dimensional linear pre-transform is applied before the OFDM modulation.

Two antennas shall be implemented at the terminal station and two or four antennas shall be implemented at the access point (AP). Support of two antennas at both the AP and the station is mandatory (hereafter referred as 2×2), and four antennas at the AP is optional (hereafter referred as 4×2). For the 2×2 MIMO-OFDM system, both the transmitting and receiving devices shall support the spatial multiplexing (SM) transmission mode, and the orthogonal space-time block code (STBC) mode. The STBC shall fulfil the features as given in [1]. For the 4×2 MIMO-OFDM system, the transmitting and receiving devices shall support the group-wise STBC (GSTBC) mode the orthogonal STBC mode, beamforming mode, and antenna selection mode. The STBC shall follow the orthogonal design for four transmit antennas as given in [2].

A total of 12 efficient training signals (preambles) are used, among which 8 are short preambles to provide robust timing and coarse frequency synchronization, and 4 are long preambles to provide robust channel estimation and fine frequency synchronization. The preamble will occupy 20 μs, equivalent to 5 OFDM symbols.

## 1.1 Scope

This sub-clause describes the PHY services provided to the IEEE 802.11n medium access control (MAC) sub-layer, The MIMO-OFDM PHY layer consists of two protocol functions, as follows:

- A PHY convergence function, which adapts the capabilities of the physical medium dependent (PMD) system to the PHY service. This function is supported by the physical layer convergence procedure (PLCP), which defines a method of mapping the IEEE 802.11n PHY sublayer service data units (PSDU) into a framing format suitable for sending and receiving user data and management information between two or more stations using the associated PMD system.

- A PMD system whose function defines the characteristics and method of transmitting and receiving data through a wireless medium between two or more stations, each using the MIMO-OFDM system.

In this partial proposal, we will focus on the functions in the MIMO-OFDM PLCP sublayer.

# 2. MIMO-OFDM PLCP Sublayer

## 2.1 Introduction

This subclause provides a convergence procedure in which PSDUs are converted to and from PPDUs. During transmission, the PSDU shall be provided with a PLCP preamble and header to create the PPDU. At the receiver, the PLCP preamble and header are processed to aid in demodulation and delivery of the PSDU.

The IEEE 802.11n shall support a MIMO structure. Implementation of two antennas at both the transmitter and the receiver is mandatory. Optionally, four antennas can be implemented at the access point and two antennas be implemented at the station.

## 2.2 PLCP Frame Format

Figure 1 shows the format for the PPDU including the MIMO-OFDM PLCP preamble, MIMO-OFDM PLCP header, PSDU, tail bits, and pad bits. The PLCP header contains the following fields: LENGTH, RATE, a reserved bit, an even parity bit, SERVICE, and the tail bits. In terms of modulation, the LENGTH, RATE, reserved bit, parity bit, and 6 “zero” tail bits constitute a separate single OFDM symbol, denoted SIGNAL, which is transmitted with the most robust combination of BPSK modulation, a convolutional code with coding rate of  R = ½, and orthogonal STBC. The SERVICE field of the PLCP header and the PSDU (with 6 “zero” tail bits and pad bits appended in each parallel stream when convolutional code is used and only pad bits appended when LDPC code is used), denoted as DATA, are transmitted at the data rate described in the RATE field and may constitute multiple OFDM symbols. The tail bits in the SIGNAL symbol enable decoding of the RATE and LENGTH fields immediately after the reception of the tail bits. The RATE and LENGTH are required for decoding the DATA part of the packet.

Figure 1 - PPDU frame format

### 2.2.1 Overview of the PPDU encoding process

The encoding process is composed of many detailed steps, which are described fully in later subclauses, as noted below. The following overview intends to facilitate understanding the details described in these subclauses:

- Produce the PLCP preamble field, composed of eight repetitions of a “short training sequence” (used for AGC convergence, timing and coarse frequency acquisition in the receiver) and four repetitions of a “long training sequence” (used for channel estimation and fine frequency acquisition in the receiver), preceded by a guard interval (GI). Refer to 2.3 for details.

- Produce the PLCP header field from the RATE, LENGTH, and SERVICE fields of the TXVECTOR by filling the appropriate bit fields. The PLCP header are encoded by a convolutional code at a rate of R = 1/2, and are subsequently mapped onto two BPSK STBC-encoded OFDM symbol, denoted as the SIGNAL symbols. In order to facilitate a reliable and timely detection of the PLCP header, 6 “zero” tail bits are inserted into the PLCP header. The encoding of the SIGNAL field into two STBC-OFDM symbols follows the same steps for convolutional encoding, interleaving, BPSK modulation, pilot insertion, Fourier transform, prepending a GI, and time-domain STBC encoding, as described subsequently for data transmission at 12 Mbit/s. The contents of the SIGNAL field are not scrambled. Refer to 2.4 for details.

- Calculate from RATE field the transmission scheme in use (IEEE 802.11a compliant or IEEE 802.11n compliant), the number of antennas in use from the transmitter (2 or 4), the transmission mode (one STBC encoded single stream, or two parallel streams), the beamforming or antenna selection is in use or not for the 4 transmit antenna setup,  the number of data bits per OFDM symbol (NDBPS), the coding rate (R), the FEC (CONV or LDPC) scheme used, the number of bits in each OFDM subcarrier (NBPSC), and the number of coded bits per OFDM symbol (NCBPS). Refer to 2.4.1 for details.

- When SM or GSTBC is used, the PSDU will be de-multiplexed into two parallel streams, each with equal length (when the number of bits in the PSDU is even) or differ by one bit (when the number of bits in the PSDU is odd).

- The PSDU in each parallel stream shall be extended with “zero” bits (at least 6 bits when CONV is used) so that the resulting length will be a multiple of NDBPS (for 2×2 STBC and 4×2  GSTBC mode, an even multiple of NDBPS, for 4×2  STBC mode, a multiple of 4×NDBPS). The resulting bit strings constitutes the DATA part of the packet. Refer to 2.5.1 and 2.5.2 for details.

- Initiate the scrambler in each parallel stream with a pseudorandom non-zero seed, generate a scrambling sequence, and XOR it with the extended string of data bits. Refer to 2.5.3 for details.

- When convolutional code is used, replace the six scrambled “zero” bits following the “data” in each parallel stream with six non-scrambled “zero” bits. (Those bits return the convolutional encoder to the “zero state” and are denoted as “tail bits.”). Refer to 2.5.1.1 for details.

- In each parallel stream, encode the extended, scrambled data string with the FEC code (CONV or LDPC). Omit (puncture) some of the encoder output string (chosen according to “puncturing pattern”) to reach the desired “coding rate” when CONV is used. Refer to 2.5.4 for details.

- Perform a two-dimensional (2D) interleaver (reordering) of the coded bits from the parallel streams according to a rule corresponding to the desired RATE. Refer to 2.5.5 for details.

- Divide the resulting coded and interleaved data string into groups of NCBPS bits in each parallel stream. For each of the bit groups, convert the bit group into a complex number according to the modulation encoding tables. Refer to 2.5.6 for details.

- Divide the complex number string into groups of 96 complex numbers for each parallel stream. Each such group from the parallel streams will be applied a linear pre-transform, according to 2.5.8. The output from the linear transform will be numbered 0 to 95 and mapped hereafter into OFDM subcarriers numbered -52 to -48, -46 to -34, -32 to -27, -26 to -21, -19 to -7, -5 to -1, 1 to 5, 7 to 19, 21 to 26, 27 to 32, 34 to 46, and 48 to 52. The subcarriers -47, -33, -20, -6, 6, 20, 33, and 47 are skipped and, subsequently, used for inserting pilot subcarriers. The “0” subcarrier, associated with center frequency, are omitted and filled with zero value. The subcarriers between -27 to -26 and between 26 and 27 are reserved subcarriers used for noise variance estimation. They are also omitted and filled with zero value. Refer to 2.5.9 for details.

- Eight subcarriers are inserted as pilots into positions -47, -33, -20, -6, 6, 20, 33, and 47. Please refer to 2.5.7 for details. So the total number of the subcarriers is 104 (96 + 8), and they are converted to time domain through inverse Fourier transform. Refer to 2.5.9 for details.

- Prepend to the Fourier-transformed waveform a circular extension of itself thus forming a GI, and truncate the resulting periodic waveform to a single OFDM symbol length by applying time domain windowing. Refer to 2.5.9 for details.

- Apply STBC encoding, or beamforming according to the transmission mode. Refer to 2.5.8, 2.6.2 and 2.6.5 for details.

- Append the OFDM symbols one after another, starting after the SIGNAL symbol describing the RATE and LENGTH. Refer to 2.2 for details.

- Up-convert the resulting “complex baseband” waveform to an RF frequency according to the center frequency of the desired channel and transmit

### 2.2.2 Rate-dependent parameters

The data rates 6, 9, 12, 18, 24, 36, 48, 54 Mbit/s are supported via the legacy mode as specified in IEEE Std 802.11a-1999. Six additional data rates are provided through the use of a 40MHz bandwidth using 2 receive antennas. The modulation parameters dependent on the data rate used shall be set according to Table 1 when 2 ×2 SM MIMO-OFDM is used. This is also the mandatory mode the station shall support.

When 2 ×2 STBC MIMO-OFDM is used, the modulation parameters dependent on the data rate used shall be set according to Table 2; When 4 ×2 GSTBC MIMO-OFDM is used, the modulation parameters dependent on the data rate used shall be set according to Table 3; When 4 ×2 STBC MIMO-OFDM is used, the modulation parameters dependent on the data rate used shall be set according to Table 4.

Table 1 - Rate-dependent parameters for 2×2 SM MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | BPSK | 1/2 | 1 | 96 | 48 | 192 | 96 |
| 36 | BPSK | 3/4 | 1 | 96 | 72 | 192 | 144 |
| 48 | QPSK | 1/2 | 2 | 192 | 96 | 384 | 192 |
| 72 | QPSK | 3/4 | 2 | 192 | 144 | 384 | 288 |
| 96 | 16-QAM | 1/2 | 4 | 384 | 192 | 768 | 384 |
| 144 | 16-QAM | 3/4 | 4 | 384 | 288 | 768 | 576 |
| 192 | 64-QAM | 2/3 | 6 | 576 | 384 | 1152 | 768 |
| 216 | 64-QAM | 3/4 | 6 | 576 | 432 | 1152 | 864 |

Table 2 - Rate-dependent parameters for 2×2 STBC MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | BPSK | 1/2 | 1 | 96 | 48 | 96 | 48 |
| 18 | BPSK | 3/4 | 1 | 96 | 72 | 96 | 72 |
| 24 | QPSK | 1/2 | 2 | 192 | 96 | 192 | 96 |
| 36 | QPSK | 3/4 | 2 | 192 | 144 | 192 | 144 |
| 48 | 16-QAM | 1/2 | 4 | 384 | 192 | 384 | 192 |
| 72 | 16-QAM | 3/4 | 4 | 384 | 288 | 384 | 288 |
| 96 | 64-QAM | 2/3 | 6 | 576 | 384 | 576 | 384 |
| 108 | 64-QAM | 3/4 | 6 | 576 | 432 | 576 | 432 |

Table 3 - Rate-dependent parameters for 4×2 GSTBC MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | BPSK | 1/2 | 1 | 96 | 48 | 192 | 96 |
| 36 | BPSK | 3/4 | 1 | 96 | 72 | 192 | 144 |
| 48 | QPSK | 1/2 | 2 | 192 | 96 | 384 | 192 |
| 72 | QPSK | 3/4 | 2 | 192 | 144 | 384 | 288 |
| 96 | 16-QAM | 1/2 | 4 | 384 | 192 | 768 | 384 |
| 144 | 16-QAM | 3/4 | 4 | 384 | 288 | 768 | 576 |
| 192 | 64-QAM | 2/3 | 6 | 576 | 384 | 1152 | 768 |
| 216 | 64-QAM | 3/4 | 6 | 576 | 432 | 1152 | 864 |

Table 4 - Rate-dependent parameters for 4×2 STBC MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | BPSK | 1/2 | 1 | 96 | 48 | 96 | 48 |
| 18 | BPSK | 3/4 | 1 | 96 | 72 | 96 | 72 |
| 24 | QPSK | 1/2 | 2 | 192 | 96 | 192 | 96 |
| 36 | QPSK | 3/4 | 2 | 192 | 144 | 192 | 144 |
| 48 | 16-QAM | 1/2 | 4 | 384 | 192 | 384 | 192 |
| 72 | 16-QAM | 3/4 | 4 | 384 | 288 | 384 | 288 |
| 96 | 64-QAM | 2/3 | 6 | 576 | 384 | 576 | 384 |
| 108 | 64-QAM | 3/4 | 6 | 576 | 432 | 576 | 432 |

### 2.2.3 Timing related parameters

Table 5 - Timing-related parameters

| Parameters | Value |
| --- | --- |
| NSD: Number of data subcarriers | 96 |
| NSP: Number of pilot subcarriers | 8 |
| NST: Number of subcarriers, total | 104 (NSD+ NSP) |
| F: Subcarrier frequency spacing | 0.3125 MHz (= 40 MHz/128) |
| TFFT: IFFT/FFT period | 3.2µs (1/∆F) |
| TPREAMBLE: PLCP preamble duration | 20µs (TSHORT+ TLONG) |
| TSIGNAL: Duration of the SIGNAL BPSK-OFDM symbol | 8.0µs (2×(TGI+ TFFT)) |
| TGI: GI duration | 0.8µs (TFFT/4) |
| TSYM: Symbol interval | 4µs (TGI+ TFFT) |
| TSHORT: Short training sequence duration | 6.4µs (8× TFFT/4) |
| TLONG: Long training sequence duration | 13.6µs (TGI+ 4×TFFT) |

### 2.2.4 Mathematical conventions in the signal description

The transmitted signals will be described in a complex baseband signal notation. The actual transmitted signal is related to the complex baseband signal by the following relation:

r(RF)<t>=Re	(1)

where

Re(.) represents the real part of a complex variable;

fc denotes the carrier center frequency.

The transmitted baseband signal is composed of contributions from several OFDM symbols.

rPACKET(t)= rPREAMBLE(t)+ rSIGNAL(t- tSIGNAL)+ rDATA(t - tDATA)	(2)

The subframes of which Equation (2) are composed are described in 2.3, 2.4, and 2.5.9. The time offsets tSUBFRAME determines the starting time of the corresponding subframe; tSIGNALis equal to 20µs, and tDATA is equal to 24µs.

All the subframes of the signal are constructed as an inverse Fourier transform of a set of coefficients, Ck, with Ck defined later as data, pilots, or training symbols in 2.3 through 2.5.

(3)

The parameters F and NST are described in Table 5. The resulting waveform is periodic with a period of TFFT = 1/∆F. Shifting the time by TGUARD creates the “circular prefix” used in OFDM to avoid ISI from the previous frame. Two kinds of TGUARD are defined: 0 µs for the short training sequence, TGI  for both the long training sequence and for data OFDM symbols  (Refer to Table 5.) The boundaries of the subframe are set by a multiplication by a time-windowing function, wTSUBFRAME(t), which is defined as a rectangular pulse, wT(t), of duration T, accepting the valueTSUBFRAME. The time-windowing function, wT(t), depending on the value of the duration parameter, T, may extend over more than one period, TFFT. In particular, window functions that extend over multiple periods of the Fast Fourier Transform (FFT) are utilized in the definition of the preamble.

wT(t)=sin2π2(0.5+tTTR),−TTR2<t<TTR21,TTR2≤t<T−TTR2sin2π2(0.5−(t−T)TTR),T−TTR2≤t<T+TTR2	(4)

### 2.2.5 Discrete time implementation consideration

Two modes of mapping the 96 complex numbers in an OFDM symbol can be implemented. The mandatory mode treats the 96 complex numbers as two 52 IEEE Std 802.11a-1999 PHY compliant OFDM symbols. This allows concurrent transmission to 2 IEEE Std 802.11a-1999 PHY devices. To achieve this compatibility, 11 nulls are inserted around the DC.

The optional mode inserts one null at the DC. This allows a larger guard band and eases the requirements of the upsampling and downsampling filters.

The following descriptions of the discrete time implementation are informational. Both the mandatory and optional modes are described.

#### 2.2.5.1 Mandatory

The common way to implement the inverse Fourier transform, as shown in Equation (3), is by an inverse Fast Fourier Transform (IFFT) algorithm. The coefficients 1 to 26 are mapped to IFFT inputs 6 to 31, the coefficients 27 to 52 are mapped to IFFT inputs 33 to 58, the coefficients -52 to -27 are mapped to IFFT inputs 70 to 95, and the coefficients -26 to -1 are mapped to IFFT inputs 97 to 122. The rest of the inputs, 0 to 5, 59 to 69, 123 to 128, the 32 and 96 (reserved for noise variance calculations) inputs are set to zero. This mapping is illustrated in Figure 2. After performing an IFFT, the output is cyclically extended to the desired length.

Figure 2 - Subcarrier arrangement of mandatory mode

#### 2.2.5.2 Optional

Alternatively, the subcarrier assignment can be implemented as follows. The coefficients 1 to 26 are mapped to the same numbered IFFT inputs, while the coefficients 27 to 52 are mapped to IFFT inputs 28 to 53, the coefficients -52 to -27 are mapped to IFFT inputs 75 to 100, and the coefficients -26 to -1 are mapped to IFFT inputs 102 to 127. The rest of the inputs, 54 to 74, the 0 (dc) input, the 27 and 101 (reserved for noise variance calculation)  inputs are set to zero. This mapping is illustrated in Figure 3. After performing an IFFT, the output is cyclically extended to the desired length.

Figure 3 - Subcarrier arrangement of optional mode

## 2.3 PLCP preamble (SYNC)

The PLCP preamble is used for synchronization. It comprises the short training preamble (SP) and the long training preamble (LP). The short training preamble consists of 8 short symbols and the long training preamble of 4 long symbols at each transmit antenna. They are shown in Figure 4 and described in this subclause.

17  0.8µs = 13.6µs

8  0.8µs = 6.4µs

SP5

SP1

SP2

SP3

SP4

SP7

SP6

SP8

CP2

LP1

LP2

LP4

LP3

6.4 + 13.6 = 20µs

0.8 + 3.2 = 4.0µs

0.8 + 3.2 = 4.0µs

0.8 + 3.2 = 4.0µs

0.8 + 3.2 = 4.0µs

Signal Detect,

AGC, Diversity

Selection

Freq. Offset

Estimation,

Timing Synch

Channel Estimation,

Residual Frequency Offset Estimation

CP

Data 1

CP

Data 2

CP

SF2

SF1



CP

RATE, LENGTH

DATA

DATA

Figure 4 - OFDM training structure at each transmit antenna

Figure 4 shows the OFDM training structure, where SP1 to SP8 denote short training preambles and LP1 to LP4 long training preambles. They are followed by two signal field symbols (SF1 and SF2) and then the data symbols (Data 1, Data 2, ). The total training length is 20µs. The dashed boundaries around the short training preamble in the figure denote repetition due to periodicity of the inverse Fourier transform.

### 2.3.1 Preambles for the Mandatory Mode Subcarrier Arrangement

For the mandatory mode subcarrier arrangement, a short training OFDM symbol consists of 26 subcarriers, which are modulated by the elements of the sequence S, given by

S-59:59=136×0,0,0,-1-j,     0,0,0,1+j,     0,0,0,1+j,    0,0,0,1+j,   0,0,0,-1-j,     0,0,0,-1-j,       0,0,0,0,       0,0,0,1+j,     0,0,0,1+j,    0,0,0,1+j,   0,0,0,1+j,     0,0,0,1+j,   0,0,0,1+j,       0,0,0,0,         0,0,0,0,       0,0,0,0,     0,0,0,-1-j,    0,0,0,-1-j,  0,0,0,1+j,     0,0,0,-1-j,     0,0,0,-1-j,    0,0,0,1+j,      0,0,0,0,     0,0,0,-1-j,      0,0,0,1+j,     0,0,0,-1-j,     0,0,0,1+j,    0,0,0,-1-j,      0,0,0,1+j,     0,0,0

The factor  is for normalization of the average power in the resulting OFDM symbol which utilizes 24 out of 104 subcarriers. The signal shall be generated in the time domain according to the following equation:

rSHORT(t)=WTSHORT(t)k=−NST2NST2Skexp(j2πkΔFt)	(5)

The fact that only spectral lines of S-59:59 with indices that are a multiple of 4 have nonzero amplitude results in a periodicity of TFFT/4 = 0.8µs. The interval TSHORT is equal to eight 0.8µs periods (i.e. 6.4µs). The short training preamble is the same for every transmit antenna. Generation of the short training sequence at each transmit antenna is illustrated in Annex A.

Unlike the short training preamble, the long training preamble is different for different antennas, although the same structure as in Figure 4 is adopted. The long training preamble at each antenna is composed of 4 long symbols. They are non-repetitive, and share one common cyclic prefix CP2 for all transmit antennas. Each long symbol is the result of either the sum or difference of two sequences L and P. The sequence L for the mandatory mode subcarrier arrangement is represented in the time domain as

L0:127(t)=-2.424,       0.181,       2.492,      -0.082,      0.936,      -0.201,      0.498,     -0.665,  -1.250,       0.864,       0.461,       0.214,     -0.974,      -0.034,     -1.495,      0.227,   1.155,      -0.461,      -0.969,      -0.075,     1.944,       -0.997,     -0.123,      0.593,   1.514,       0.867,      -1.273,      -0.471,    -0.294,        0.597,     -0.037,      0.548,  -0.905,     -1.781,      -0.323,       0.029,      0.966,        0.727,     -0.887,     -0.671,   1.679,       0.496,      -1.758,       1.311,      1.113,       -0.986,     1.592,      -0.489,  -0.316,     -0.364,      -1.237,      -0.161,     -1.257,        1.413,     0.584,      -0.352,   0.112,       0.332,       0.614,       0.002,      -1.273,      -1.562,     0.669,       0.343,   0.601,       0.604,       0.426,      -0.500,       1.025,       0.886,    -0.347,       0.728,  -1.341,     -1.103,      -1.197,      -0.201,      -0.243,       0.331,     0.771,      -0.733,   0.791,      -0.089,      -0.298,       0.882,       0.211,       0.412,     1.699,      -0.328,   0.480,      -0.387,      -2.473,      -0.137,      -2.250,      0.255,      2.610,     -0.013,                0,              0,               0,              0,               0,             0,             0,             0,          0,              0,               0,              0,               0,             0,             0,             0,          0,              0,               0,              0,               0,             0,             0,             0,          0,              0,               0,              0,               0,             0,             0,             0

The sequence P in the frequency domain is described as

P-59:59,freq=0,0,0,0,      0,0,1,0,      0,0,0,0,     0,0,0,0,     0,0,0,0,     1,0,0,0,     0,0,0,0,       0,0,0,0,      0,0,1,0,      0,0,0,0,     0,0,0,0,     0,0,0,0,    -1,0,0,0,     0,0,0,0,       0,0,0,0,      0,0,0,0,      0,0,0,0,     0,0,1,0,     0,0,0,0,      0,0,0,0,     0,0,0,0,       1,0,0,0,      0,0,0,0,      0,0,0,0,     0,0,1,0,     0,0,0,0,      0,0,0,0,     0,0,0,0, -1,0,0,0,      0,0,0

from which its time domain counterpart can be generated according to the equation

P(t)=wLONGk=−NST2NST2P−59:59,freqexp(j2πkΔFt)	(6)

where the interval TLONG is equal to 3.2µs. The long training symbols at each time (OFDM symbol index) and space (transmit antenna index) is of 3.2µs duration and is constructed based on Table 6.

Table 6 - Construction of long preamble for mandatory mode subcarrier arrangement

|  |  | Space (transmit antenna index) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 |
| Time (OFDM symbol index) | 1 | P+L | P+L | P+L | P+L |
|  | 2 | P+L | P-L | P+L | P-L |
|  | 3 | P+L | P+L | P-L | P-L |
|  | 4 | P+L | P-L | P-L | P+L |

The last 32 values of each long symbol at each transmit antenna at different time is equal to the values in the cyclic prefix which is of 0.8µs duration and represented in the time domain as

CP2=       0 + 0.3540i,              0,              0.2240 + 0.1670i,              0,-0.3470 - 0.1960i,              0,               0.3120 - 0.3520i,              0,-0.1350 - 0.1350i,              0,             -0.1030 + 0.2240i,              0,0.2940 + 0.3470i,              0,             -0.3520 + 0.1030i,              0, 0.2500 - 0.2500i,              0,              -0.0350 - 0.3380i,              0,-0.1960 - 0.0690i,              0,               0.3380 + 0.2730i,             0,-0.3270 + 0.3270i,             0,               0.1670 + 0.0350i,             0,  0.0690 - 0.2940i,             0,              -0.2730 - 0.3120i,              0

An illustration of the long training preamble generation is given in Annex B.

It is interesting to note that for the mandatory mode of the subcarrier arrangement:

- The imaginary part of the two sequences P+L and P-L are identical and has the property

[Im(P+L)]n=[Im(P+L)]64-n=−[Im(P+L)]64+n= -[Im(P+L)]128-n[Im(P-L)]n  =[Im(P-L)]64-n = -[Im(P-L)]  = -[Im(P-L)]128-n,

for n=0,1,2,…31.

- The last 32 values of the two sequences P+L and P-L are identical to the cyclic prefix (CP2). Therefore no explicit cyclic prefix is required for individual long symbol except the first one in each transmit antenna as depicted in Figure 4.

### 2.3.2 Preambles for the Optional Mode Subcarrier Arrangement

The counterparts for the optional mode subcarrier arrangement are given below:

S-52:52=2×       1+j,     0,0,0,1+j,     0,0,0,-1-j,    0,0,0,-1-j,   0,0,0,-1-j,     0,0,0,1+j,   0,0,0,1+j,    0,0,0,-1-j,     0,0,0,-1-j,     0,0,0,1+j,    0,0,0,-1-j,    0,0,0,-1-j,     0,0,0,1+j       0,0,0,0       0,0,0,1+j,      0,0,0,1+j,    0,0,0,1+j,     0,0,0,1+j,    0,0,0,1+j,    0,0,0,1+j,     0,0,0,-1-j,     0,0,0,-1-j,     0,0,0,1+j,    0,0,0,-1-j,   0,0,0,1+j,    0,0,0,-1-j,     0,0,0,1+j

L0:127(t)=-2.124,      -1.851,       1.790,      0.215,     -0.955,       0.203,      -0.965,      1.053,    1.784,       -0.361,     -0.162,      0.282,      0.683,       0.187,      -0.789,      -0.642,   0.485,        0.239,      -0.903,     0.675,     -0.044,      -1.348,      -0.426,      -0.765,  -0.059,      -1.198,      -1.419,     0.964,       0.570,       0.766,       0.383,      -0.185,   0.378,       0.164,        0.925,     0.274,     -1.264,       -1.868,     -0.342,       0.919,  -0.992,        0.040,       1.297,      0.907,      1.307,        0.058,     -0.165,      -0.787, -1.359,      -1.177,       -2.058,     0.373,      0.778,       -2.212,     -0.599,       0.767, -1.126,      -0.531,        0.023,    -1.047,     0.241,        0.713,      1.130,        2.268,  0.485,      -0.837,       -1.395,    -0.387,     0.864,        0.644       2.049,        0.781,  0.360,       0.815,       -1.172,     0.971,      1.370,        0.118,      0.505,      -0.356,  0.786,       0.189,       -0.234,     0.853,      0.046,        0.857,      1.047,        0.482,-0.989,      -0.992,         1.893,    -0.343,    -0.889,        1.736     -0.750        -1.627,         0,              0,                0,             0,             0,               0,            0,               0,         0,              0,                0,             0,             0,               0,            0,               0,         0,              0,                0,             0,             0,               0,            0,               0,         0,              0,                0,             0,             0,               0,            0,               0

P-52:52,freq=0,0,0,0,      1,0,0,0,      0,0,0,0,     0,0,0,0,     0,0,1,0,     0,0,0,0,     0,0,0,0,       0,0,0,0,      1,0,0,0,      0,0,0,0,     0,0,0,0,    0,0,-1,0,     0,0,0,0,     0,0,0,0,       0,0,1,0,      0,0,0,0,      0,0,0,0,     0,0,0,0,     1,0,0,0,      0,0,0,0,     0,0,0,0,       0,0,1,0,      0,0,0,0,      0,0,0,0,     0,0,0,0,    -1,0,0,0,      0

CP2 =                       0,        0.1160 - 0.1760i,        0.1060 + 0.0790i,     -0.2250 - 0.2620i,-0.2880 - 0.1630i,       0.1180 - 0.0510i,         0.3100 - 0.3500i,      0.0350 - 0.0310i,-0.1250 - 0.1250i,      -0.0100 - 0.2080i,      -0.0650 + 0.1420i,     -0.1900 - 0.1080i, 0.0570 + 0.0680i,      0.3430 + 0.2370i,        0.1020 - 0.0300i,     -0.2740 + 0.2940i,-0.1770 + 0.1770i,      0.0780 + 0.0440i,       0.0330 + 0.3240i,       0.0040 - 0.0130i, 0.1930 + 0.0680i,      0.1210 + 0.1420i,       -0.2620 - 0.2110i,     -0.2840 + 0.0420i,      0.1250 - 0.1250i,      0.2590 - 0.2810i,        0.0160 + 0.0030i,      -0.0490 - 0.3010i,    0.0380 - 0.1630i,     -0.1220 - 0.0120i,       -0.2410 - 0.2750i,       0.0810 + 0.0740i

An illustration of the short and long training preamble generation for the optional mode of subcarrier arrangement is given in Annex C and Annex D.

It is interesting to note that for the optional mode of the subcarrier arrangement:

- The imaginary part of the two sequences P+L and P-L are identical and has the property

[Im(P+L)]n= -[Im(P+L)]64-n=[Im(P+L)]64+n= -[Im(P+L)]128-n[Im(P-L)]n  = -[Im(P-L)]64-n  =[Im(P-L)]64+n  = -[Im(P-L)]128-n,

for n=0,1,2,…31.

- Same as for the mandatory mode of subcarrier arrangement, the last 32 values of the two sequences P+L and P-L are identical to the cyclic prefix (CP2). Therefore no explicit cyclic prefix is required for individual long symbol except the first one in each transmit antenna as depicted in Figure 4.

## 2.4 Signal field (SIGNAL)

The MIMO-OFDM training symbols shall be followed by the SIGNAL field, which contains the RATE, LENGTH and SERVICE fields of the TXVECTOR. The RATE field conveys information about the number of antennas and the MIMO-encoding process applied at the transmitter, the type of modulation and coding scheme, the coding rate, the use of pre-transform in OFDM modulation, as used in the rest of the packet. The encoding of the SIGNAL single OFDM symbol shall be performed with BPSK modulation of the subcarriers and using convolutional coding at R = 1/2. The encoding procedure, which includes convolutional encoding, interleaving, modulation mapping processes, pilot insertion, OFDM modulation, and STBC encoding, follows the steps described in 2.5.4.1, 2.5.5, 2.5.6, 2.5.7, 2.5.9 and 2.6.2, as used for transmission of data at a 12 Mbit/s rate with 2×2 STBC code. The contents of the SIGNAL field are not scrambled.

The SIGNAL field shall be composed of 2×48 bits. The bit assignment will be determined later with the MAC.

### 2.4.1 Data RATE

The bits giving the RATE information will be determined later with the MAC.

### 2.4.2 PLCP length field (LENGTH)

The PLCP length field shall indicates the number of octets in the PSDU that the MAC is currently requesting the PHY to transmit. This value is used by the PHY to determine the number of octet transfers that will occur between the MAC and the PHY after receiving a request to start transmission. The number of bits in the LENGTH field will be determined later with the MAC.

### 2.4.3 Parity (P), Reserved (R), and Signal tail (SIGNAL TAIL)

The parity, reserved bit position will be determined later with the MAC. The bits 42-47 constitute the SIGNAL TAIL field, and all 6 bits shall be set to zero.

### 2.4.4 Service field (SERVICE)

The bits in the service field shall be used to synchronize the descrambler in the receiver. The number of bits in the service field, and their values shall be determined later with MAC.

## 2.5 Data field

The DATA field contains the SERVICE field, the PSDU, the TAIL bits, and the PAD bits. Depending on the MIMO encoding scheme adopted at the transmitter, the DATA field may contain one or two parallel streams. All bits in the DATA field are scrambled.

### 2.5.1 PPDU tail bit field (TAIL)

#### 2.5.1.1 When convolutional code is used

The PPDU tail bit field shall be six bits of “0”, which are required to return the convolutional encoder to the “zero state”. This procedure improves the error probability of the convolutional decoder, which relies on future bits when decoding and which may not be available past the end of the message. The PLCP tail bit field shall be produced by replacing six scrambled “zero” bits following the message end with six non-scrambled “zero” bits. The tail bit field shall be implemented in both streams when 2×2 SM and 4×2 GSTBC is used, which are described in 2.6.1 and 2.6.3.

#### 2.5.1.2 When LDPC code is used

The PPDU tail bit field is not required.

### 2.5.2 Pad bits (PAD)

The number of bits in the DATA field in each parallel stream shall be a multiple of NCBPS(for 2×2 STBC and 4×2  GSTBC mode, an even multiple of NDBPS, for 4×2  STBC mode, a multiple of 4×NDBPS), the number of data bits per OFDM symbol (96, 192, 384, or 576 bits). To achieve that, the length of the message is extended so that it becomes a multiple of NDBPS(for 2×2 STBC and 4×2  GSTBC mode, an even multiple of NDBPS, for 4×2  STBC mode, a multiple of 4×NDBPS). When convolutional code is used, at least 6 bits shall be appended to the message, in order to accommodate the TAIL bits. The number of OFDM symbols, NSYM; the number of bits in the DATA field, NDATA, and the number of pad bits, NPAD, are computed from the length of the PSDU (LENGTH) as follows:

NSYM=k×(Nservice+8×LENGTHs+Ntail)k×NDBPS,NDATA=NSYM×NDBPS,NPAD=NDATA−(Nservice+8×LENGTHs+Ntail),	(7)

where s=1 for 2×2 and 4×2 STBC modes, and s=2 for 2×2 SM, 4×2  GSTBC, 4×2  beamforming, and 4×2  antenna selection modes, k=1 for 2×2 SM, 4×2  beamforming, and 4×2  antenna selection modes, K=2 for 2×2 STBC and 4×2  GSTBC modes, and k=4 for 4×2  STBC mode.

The function  is a function that returns the smallest integer value greater than or equal to its argument value. As described in 2.5.1, Ntail = 6 when convolutional code is used, and Ntail = 0 when LDPC code is used.  The appended bits (“pad bits”) are set to “zeros” and are subsequently scrambled with the rest of the bits in the DATA field.

### 2.5.3 PLCP DATA scrambler and descrambler

The DATA field in each parallel stream in the MIMO-OFDM system, composed of SERVICE, PSDU, tail, and pad parts, shall be scrambled with a length-127 frame-synchronous scrambler. The octets of the PSDU are placed in the transmit serial bit stream, bit 0 first and bit 7 last. The frame synchronous scrambler uses the generator polynomial S(x) as follows, and is illustrated in Figure 5:

S(x)=x7+x4+1

The 127-bit sequence generated repeatedly by the scrambler shall be (leftmost used first), 00001110 11110010 11001001 00000010 00100110 00101110 10110110 00001100 11010100 11100111 10110100 00101010 11111010 01010001 10111000 1111111, when the “all ones” initial state is used. The same scrambler is used to scramble transmit data and to descramble receive data. When transmitting, the initial state of the scrambler will be set to a pseudo random non-zero state. The seven LSBs of the SERVICE field will be set to all zeros prior to scrambling to enable estimation of the initial state of the scrambler in the receiver.

Figure 5 - Data scrambler

### 2.5.4 FEC

A convolutional encoder of coding rate of R=1/2, 2/3, or 3/4, shall be used to encode the DATA field, which is composed of SERVICE, PSDU, tail, and pad parts. Support of LDPC is optional.

#### 2.5.4.1 Convolutional Encoder

The convolutional encoder shall use the generator polynomials, g0=1338 and g1=1718, of rate R=1/2, as shown in Figure 6. The bit denoted as "A" shall be output from encoder before the bit denoted as "B". Higher rates are derived from it by employing "puncturing". Puncturing is a procedure for omitting some of the encoded bits in the transmitter (thus reducing the number of transmitted bits and increasing the coding rate) and inserting a dummy "zero" metric into the convolutional decoder on the receive side in place of the omitted bits. The puncturing matrix for generating the R=2/3 code from the R=1/2 mother code is

P=1110,

and the puncturing matrix for generating the R=3/4 code is

P=110101.

Figure 6 - Convolutional Encoder (k=7)

#### 2.5.4.2 LDPC Encoder

Low-density parity-check (LDPC) shall be used as an optional channel encoder when the system has high throughput and/or has higher QoS requirement.  A simple partial-differential (PD)-LDPC encoder, as shown in Figure 7, shall be used.

Figure 7 - PD-LDPC encoder

The PD-LDPC encoder is a systematic encoder.  The codeword of size n consists of two parts, , where  of size k is the information sequence, and  of size n-k is the parity-check sequence, which is generated by the concatenation of a sparse block encoder, , followed by an differential encoder.

The corresponding parity-check matrix is composed of two parts, , where H2 is the differential matrix,

,	(8)

and H1 is a sparse matrix, with a uniform column weight of four.  The H1 matrix of a PD-LDPC(1152, 864), as shown in Annex C, is designed for the transmission type of rate-3/4 and 64QAM based on the progressive edge growth (PEG) method .

The designed PD-LDPC encoder is flexible to reduce the coding rate by appending dummy zeros to the transmitted sequence.  The following example illustrates this process.  For the setup of rate-2/3 and 64QAM (NMDBPS=768 and NMCBPS=1152), the information sequence of 768 bits is appended by 96 dummy zeros and the PD-LDPC(1152, 864) is used to generate the coded bits of size 1152, and transmitted by one OFDM symbol.  Note that at the LDPC decoder, the logarithm-likelihood ratio (LLR) of these dummy zeros is pre-set to - as prior information.

### 2.5.5 Data interleaving

Data interleaving shall be composed of two steps: bit-interleaving at the output of the FEC encoder, in each parallel streams, and symbol interleaving across the two parallel streams, as illustrated in Figure 8. It is therefore called a 2D interleaver. A symbol mapper shall be implemented between the two steps of interleavers, in each parallel stream. The symbol mapper shall follow the specification in 2.5.6. The position of the 2D interleaver in the MIMO-OFDM system can be referred to Figure 12 through Figure 15. The two bit interleavers are compliant with IEEE Std 802.11a-1999 PHY.

Figure 8 - 2D Bit Interleaver and Subcarrier mapper

The 2D bit deinterleaver is the inverse operation of the 2D bit interleaver and is shown in Figure 9.

Figure 9 - 2D Bit Deinterleaver and demapper

#### 2.5.5.1 When convolutional code is used

IEEE Std 802.11a-1999 PHY compliant bit interleaver/ deinterleaver

The convolutionally encoded bits shall be interleaved by a block interleaver with a block size corresponding to the number of bits in a single OFDM symbol, NCBPS. The interleaver is defined by a two-step permutation. The first permutation ensures that adjacent coded bits are mapped onto nonadjacent subcarriers. The second ensures that adjacent coded bits are mapped alternately onto less and more significant bits of the constellation and, thereby, long runs of low reliability (LSB) bits are avoided.

Let the number k denote the index of the coded bit before the first permutation; the number i denote the index after the first and before the second permutation, and j the index after the second permutation, just prior to modulation mapping, then the first permutation is defined by

(9)

The function  denotes the largest integer not exceeding the parameter.

The second permutation is defined by the rule

(10)

The value s is determined by the number of coded bits per subcarrier, NBPSC, according to

.	(11)

The deinterleaver, that performs the inverse relation, is also defined by two permutations. Here, j denotes the index of the original received bit before the first permutation; i shall be the index after the first and before the second permutation, and k shall be the index after the second permutation, just prior to delivering the coded bits to the convolutional (Viterbi) decoder.

The first permutation is defined by the rule

(12)

where s is defined in above Equation. This permutation is the inverse of the second permutation described in interleaver.

The second permutation is defined by the rule

(13)

This permutation is the inverse of the first permutation as described in Interleaver.

2D symbol interleaver / deinterleaver

Define:

NSD =96 as the number of data symbols,

NT =2 as the number of data streams

Ti(k), where i=1, …, NT, and k =1,2,…, Ns the mapped symbol before symbol interleaving at stream i, data symbol k,

Sm(n), where m=1, …, NT,  and n =1,2,…, Ns the symbol after symbol interleaving at stream m, data symbol n,

The symbol interleaver defines the mapping operation

Ti(k)(m,n)=Πs(i,k)Sm(n)

where the (stream, symbol) index pair after interleaving, (m,n), can be generated as follows:

m=(k−1)(NsNT)+1n=1+NT(k−1)+(i−1)modNs	(14)

The symbol deinterleaver defines the inverse mapping operation

Sm(n)(i,k)=Π−1s(m,n)Ti(k)

where the (stream, symbol) index pair after deinterleaving, (i,k), can be generated as follows:

i=1+(n−1)modNTk=(m−1)NsNT+(n−1)NT+1.	(15)

#### 2.5.5.2 When LDPC code is used

2D bit permutation

Although a bit interleaver is not required physically when LDPC code used, the following permutation rules are necessary to map coded bits into symbols: 1) map the adjacent coded bits to nonadjacent subcarriers; 2) map as many information bits as possible into MSB in order to achieve better protection to information bits than to parity-check bits.  The following three steps shall be processed for 2-D bit permutation.

Step 1: Coded bits are written into a rectangular block I of , where , as shown in Figure 10.

Figure 10 - Bit permutation block I for LDPC

Step 2: rectangular block I is reshaped as block II of  as shown in  Figure 11.

Figure 11 - Bit permutation block II for LDPC

Step 3: the columns of block II is permuted by the rule

(16)

where .

Note that the 2D bit de-permutation at the receiver is combined in the LDPC decoder.

2D symbol interleaver / deinterleaver

The LDPC encoder generates coded bits for two OFDM symbols duration.  The 2D symbol interleaver/deinterleaver is designed as follows.

Define:

Ns =192 as the number of data symbols,

NT =2 as the number of data streams

Ti(k) is the mapped symbol before symbol interleaving at stream i and data symbol k, where i=1, …, NT,  and k =1,2,…, Ns, and

Sm(n) is the symbol after symbol interleaving at stream m and data symbol n, where m=1, …, NT,  and n =1,2,…, Ns.

The symbol interleaver defines the mapping operation

Ti(k)(m,n)=Πs(i,k)Sm(n)

where the (stream, symbol) index pair after interleaving, (m,n), can be generated as follows:

(17)

The symbol deinterleaver defines the inverse mapping operation

Sm(n)(i,k)=Π−1s(m,n)Ti(k)

where the (stream, symbol) index pair after deinterleaving, (i,k), can be generated as follows:

i=1+(n−1)modNTk=(m−1)NsNT+(n−1)NT+1.	(18)

### 2.5.6 Subcarrier modulation mapping

The OFDM subcarriers shall be modulated by using BPSK, QPSK, 16-QAM or 64-QAM modulation, depending on the RATE requested. The encoded and interleaved binary serial input data shall be divided into groups of NBPSC (1, 2, 4, or 6) bits and converted into complex numbers representing BPSK, QPSK, 16-QAM or 64-QAM constellation points. The conversion shall be performed according to Gray-coded constellation mappings, illustrated in Table 8 for BPSK, Table 9 for QPSK, Table 10 for 16-QAM, and Table 11 for 64-QAM, with the input bit b0, being the earliest in the stream. The output values, d, are formed by multiplying the resulting (I+jQ) value by a normalization factor KMOD, as described in following equation:

The normalization factor, KMOD, depends on the base modulation mode and the number of transmit antennas, as prescribed in Table 7. The purpose of the normalization factor is to achieve the same average power for all mappings and all MIMO architectures.

Table 7 - Modulation Dependent Normalization Factor KMOD

| Modulation | Transmit Antenna Number | KMOD |
| --- | --- | --- |
| BPSK | 2 | 12 |
|  | 4 | 12 |
| QPSK | 2 | 12 |
|  | 4 | 122 |
| 16-QAM | 2 | 125 |
|  | 4 | 1210 |
| 64-QAM | 2 | 1221 |
|  | 4 | 1242 |

For BPSK, b0 determines the I value, as illustrated in Table 8. For QPSK, b0 determines the I value and b1 determines the Q value, as illustrated in Table 9. For 16-QAM, b0b1 determines the I value and b2b3 determines the Q value, as illustrated in Table 10. For 64-QAM, b0b1b2 determines the I value and b3b4b5 determines the Q value, as illustrated in Table 11.

- Table 8 - BPSK Encoding Table

| Input Bit (b0) | I-out | Q-out |
| --- | --- | --- |
| 0 | -1 | 0 |
| 1 | 1 | 0 |

- Table 9 - QPSK Encoding Table

| Input bit (b0) | I-out |  | Input bit (b1) | Q-out |
| --- | --- | --- | --- | --- |
| 0 | -1 |  | 0 | -1 |
| 1 | 1 |  | 1 | 1 |

- Table 10 - 16QAM Encoding Table

| Input bit (b0b1) | I-out |  | Input bit (b2b3) | Q-out |
| --- | --- | --- | --- | --- |
| 00 | -3 |  | 00 | -3 |
| 01 | -1 |  | 01 | -1 |
| 11 | 1 |  | 11 | 1 |
| 10 | 3 |  | 10 | 3 |

- Table 11 - 64QAM Encoding Table

| Input bit (b0b1b2) | I-out |  | Input bit (b3b4b5) | Q-out |
| --- | --- | --- | --- | --- |
| 000 | -7 |  | 000 | -7 |
| 001 | -5 |  | 001 | -5 |
| 011 | -3 |  | 011 | -3 |
| 010 | -1 |  | 010 | -1 |
| 110 | 1 |  | 110 | 1 |
| 111 | 3 |  | 111 | 3 |
| 101 | 5 |  | 101 | 5 |
| 100 | 7 |  | 100 | 7 |

### 2.5.7 Pilot subcarriers

In each long training preamble, signal field and OFDM symbol, eight of the subcarriers are dedicated to pilot signals in order to make the coherent detection robust against frequency offset and phase noise. These pilot signals shall be put in subcarriers -53, -39, -25, -11, 11, 25, 39, 53 for the mandatory subcarrier arrangement mode and  –48, -34, -20, -6, 6, 20, 34, and 48 for the optional subcarrier arrangement mode. The pilots in the long training preamble are not modulated over time but those in the signal field and the OFDM data symbols shall be BPSK modulated by a pseudo binary sequence to prevent the generation of spectral lines. The contribution of the pilot subcarriers to each OFDM symbol is described in 2.5.9.

### 2.5.8 Pre-transform

The 2D pre-transform module shall be performed such that the mapped modulated symbols are spread over frequency, space or both. The pre-transformation operation is represented in the following matrix form:

S1 = [S1(1), S1(2), …, S1(96)]T: data symbol vector of the first data stream;

S2 = [S2(1), S2(2), …, S2(96)]T: data symbol vector of the second data stream;

y=y1

where T is the transformation matrix which satisfied unitary condition, i.e. TTH=I.

In this partial proposal, we only consider the case of frequency spreading, which shall be performed as

y=y1.

The design and implementation of 2D (spatial and frequency) pre-transform matrix will be finalised later after extensive studies.

Two transforms can be used:

- Walsh-Hadamard Transform

- Rotated Discrete Fourier Transform

The columns of the transform matrix can be permuted according to a pre-defined order.

The order-96 Walsh Hadamard Transform can be generated from order-8 Hadamard matrix

T8=18×1     1     1     1     1     1     1     11   -1     1   -1     1   -1     1   -11     1   -1   -1     1     1   -1   -11   -1   -1     1     1   -1   -1     11     1     1     1   -1   -1   -1   -11   -1     1   -1   -1     1   -1     11     1   -1   -1   -1   -1     1     1

and order-12 Hadamard matrix

T12=112×   1      1     1       1     1     1     1      1      1      1      1      1   1    -1     1     -1     1     1     1    -1    -1    -1      1    -1   1    -1   -1       1   -1     1     1      1    -1    -1    -1      1   1      1   -1     -1     1   -1     1      1      1    -1    -1    -1   1    -1     1     -1   -1     1   -1      1      1      1    -1    -1   1    -1   -1       1   -1   -1     1    -1      1      1      1    -1   1    -1   -1     -1     1   -1   -1      1    -1      1      1      1   1     1    -1     -1   -1     1   -1    -1      1    -1      1      1   1     1      1     -1   -1   -1     1    -1    -1      1    -1      1   1     1      1       1   -1   -1   -1      1    -1    -1      1    -1   1   -1      1       1     1   -1   -1    -1      1    -1    -1      1

by performing the Kronecker product as

Tf=T8⊗T12.

The rotated DFT transform matrix is generated by

Tf=F×exp(jθ0)00...00exp(jθ1)0...000exp(jθ2)...0...............000...exp(jθNSD−1)

where F is the FFT matrix of size NSD, and θiis the phase rotation value.

### 2.5.9 OFDM modulation

The stream of complex numbers is divided into groups of NSD = 96 complex numbers. We shall denote this by writing the complex number dk,n, which corresponds to subcarrier k of OFDM symbol n, as follows:

dk,n≡dk+NSD×n,k = 0, …NSD-1, n = 0, … NSYM-1,

The number of OFDM symbols, NSYM, was introduced in 2.5.2.

An OFDM symbol, rDATA,n(t), is defined as

rDATA,n(t)=wTSYM(t)k=0NSD−1dk,nexp((j2πM(k)Δ(t−TGI))+pn+1k=−NST2NST2pkexp(j2πkΔF(t−TGI)))

where the function, M(k), defines a mapping from the logical subcarrier number 0 to 95 into frequency offset index -58 to 58 for mandatory mode or -53 to 53 for optional mode, while skipping the reserved subcarrier locations, pilot subcarrier locations and the 0th (dc) subcarrier.

Mandatory mode

M(k)=k−580≤k≤4k−575≤k≤17k−5618≤k≤23k−5524≤k≤29k−5430≤k≤42k−5343≤k≤47k−4248≤k≤52k−4153≤k≤65k−4066≤k≤71k−3972≤k≤77k−3878≤k≤90k−3791≤k≤95

Optional mode

M(k)=k−530≤k≤4k−525≤k≤17k−5118≤k≤23k−5024≤k≤29k−4930≤k≤42k−4843≤k≤47k−4748≤k≤52k−4653≤k≤65k−4566≤k≤71k−4472≤k≤77k−4378≤k≤90k−4291≤k≤95

The contribution of the pilot subcarriers for the nth OFDM symbol is produced by Fourier transform of sequence P-58,58for mandatory mode and P-53,53for optional mode given below.

First, define the sequence (used in IEEE Std 802.11a-1999)

P-26,26=0,0,0,0,0,1,0,0,0,0,0,0,0,0,0,0,0,0,0,1, 0,0,0,0,0,0,0,0,0,0,0,0,0,1,0,0,0,0,0,0, 0,0,0,0,0,0,0,-1,0,0,0,0,0.

then

P-53,53= P-26,26, 0, P-26,26,P-58,58= P-26,26, 0,0,0,0,0,0,0,0,0,0,0, P-26,26.

The polarity of the pilot subcarriers is controlled by the sequence, pn, which is a cyclic extension of the 127 elements sequence and is given by

P0..126=1,1,1,1, -1,-1,-1,1, -1,-1,-1,-1, 1,1,-1,1, 1,1,1,1, 1,1,1,1, 1,1,1,1, 1,1,1,1,                1,1,-1,1, 1,-1,-1,1, 1,1,-1,1, -1,-1,-1,1, -1,1,-1,1, 1,-1,-1,1, 1,1,1,1, -1,-1,1,1,                -1,-1,1,-1, 1,-1,1,1, -1,-1,-1,1, 1,-1,-1,-1, -1,1,-1,-1, 1,-1,1,1, 1,1,-1,1, -1,1,-1,1,                -1,-1,-1,-1, -1,1,-1,1, 1,-1,1,-1, 1,1,1,-1, -1,1,-1,1, -1,1,1,1, -1,-1,-1,-1, -1,1,-1

The sequence, pn, can be generated by the scrambler defined by Figure 5 when the "all ones" initial state is used, and by replacing all "1's" with -1 and all "0's" with 1. Each sequence element is used for one OFDM symbol. The first and second elements, p0 and p1, multiply the pilot subcarriers of the first and second SIGNAL symbol, respectively, while the elements from p2 on are used for the DATA symbols.

The subcarrier frequency allocation is shown in Figure 2 and Figure 3. To avoid difficulties in D/A and A/D converter offsets and carrier feed through in the RF system, the subcarrier falling at DC (0th subcarrier) is not used.

The concatenation of NSYM OFDM symbols can now be written as

rDATA(t)=n=0NSYM−1rDATA,n(t−nTSYM).

## 2.6 MIMO architectures

The IEEE 802.11n shall support a MIMO structure. Two antennas shall be implemented at both the transmitter and the receiver as the mandatory mode. Support of 2×2 SM and STBC at the receiver (AP and the station) is mandatory.

Optionally, four antennas can be implemented at AP considering AP’s potential tolerance for larger size and higher power consumption than the station. A GSTBC transmitter is proposed to increase the throughput and extend the transmission range of the downlink, and orthogonal STBC can be applied to improve the wireless link quality and extend the range of communication. Alternatively, two antennas can be selected for transmission based on the signal power. A fixed beam-forming weighting structure can also be implemented to listen to the RTS (request to transmit) signals from the station far from the AP. Taking into account the potential small size and the low power consumption requirement of the station, only two antennas shall be implemented for the station.

### 2.6.1 2×2 spatial multiplexing

Figure 12 illustrates the steps involved in transmit processing in a 2×2 SM PT-FEC-coded OFDM system. Source data shall first be de-multiplexed into two parallel streams, and then scrambled. The information bits in the data field of the two parallel streams shall be FEC-encoded, bit-interleaved, and mapped to the specific modulation symbols, according to 2.5.4, 2.5.5, and 2.5.6. The modulation symbols shall be transformed through the linear transformation matrix T, according to 2.5.8, and OFDM modulated which includes the inverse Fast Fourier Transform (IFFT) and cyclic prefix (CP) insertion, as specified in 2.5.9.

When SM is used, the data rate is two times that of a single stream.

Figure 12 - Transmitter processing for 2×2 Spatial Multiplexing PT-FEC-OFDM

### 2.6.2 2×2 STBC

When the channel conditions deteriorate, STBC can be used to improve the signal link quality. In this case, only one single stream of data is processed, and no data rate is gained through the MIMO structure. Figure 13 illustrates the steps involved in transmit processing in a 2×2 STBC PT-FEC-OFDM. STBC as given in [1] shall be used, but its time-domain implementation shall be adopted to reduce the implementation complexity.

The bits in the DATA field shall be FEC-encoded, bit-interleaved, and mapped to the specific modulation symbols, according to 2.5.4, 2.5.5, and 2.5.6. The modulation symbols shall be transformed through matrix T, according to 2.5.8, and OFDM modulated which includes the inverse Fast Fourier Transform (IFFT) and cyclic prefix (CP) insertion, as specified in 2.5.9.

Figure 13 Transmitter processing for 2×2 STBC PT-OFDM

#### 2.6.2.1 Time-Domain STBC Implementation

The conventional STBC encoding in an OFDM system shall be performed as shown in Table 12, where SF,i(k) denotes the transmitted frequency domain signal on subcarrier k at OFDM symbol interval i.

Table 12 - Conventional STBC OFDM Encoding

|  | Antenna 0 | Antenna 1 |
| --- | --- | --- |
| time t | SF,0 = [SF,0(0), SF,0(1),… SF,0(N - 1)] | SF,1 = [SF,1(0), SF,1(1),… SF,1(N - 1)] |
| time t+NT | -S*F,1 | S*F,0 |

To enable backward compatibility to 802.11a systems, we modify the STBC encoding such that the original OFDM signal is always transmitted on antenna 0 and STBC coded OFDM signal transmitted on antenna 1. The modified encoding is summarized in Table 13.

Table 13 - Modified STBC OFDM Encoding

|  | Antenna 0 | Antenna 1 |
| --- | --- | --- |
| time t | SF,0 = [SF,0(0), SF,0(1),… SF,0(N - 1)] | -S*F,1 |
| time t+NT | SF,1 = [SF,1(0), SF,1(1),… SF,1(N - 1)] | S*F,0 |

Making use of the following IFFT property, we can perform the equivalent STBC encoding in time domain, which reduces the implementation complexity:

sF(k)⇔FTs(n)sF(k)⇔FTs(N−n)

In this case, the time domain STBC encoding shall be performed according to Table 14.

Table 14 - Time domain STBC OFDM Encoding

|  | Antenna 0 | Antenna 1 |
| --- | --- | --- |
| time t | St,0=[ S t,0(0), S t,0(1),…, S t,0(N-1)] | [ -S t,1* (0), -S t,1* (N-1),…,-S t,1* (1)] |
| time t+NT | S t,1=[ S t,1(0), S t,1(1),…, S t,1(N-1)] | [ S t,0*(0), S t,0*(N-1),…,S t,0*(1)] |

### 2.6.3 4×2 GSTBC

Figure 14 illustrates the steps involved in transmit processing in a 4×2 GSTBC PT-FEC-OFDM system. The four antennas are used to transmit two parallel streams, with each stream implementing a 2×2 STBC PT-FEC-OFDM. Therefore, the data rate attained is the same as the 2×2 SM PT-FEC-OFDM, but transmit diversity of order is achieved. The bits in the DATA field of the two parallel streams shall be FEC-encoded, bit-interleaved, and mapped to the specific modulation symbols, according to 2.5.4, 2.5.5, and 2.5.6. The modulation symbols shall then be transformed through matrix T, according to 2.5.8, and OFDM modulated which includes the inverse Fast Fourier Transform (IFFT) and cyclic prefix (CP) insertion, as specified in 2.5.9. The time domain data from individual stream shall then be encoded by STBC, according to 2.6.2.1. Four streams of data shall be transmitted simultaneously.

Figure 14 - Transmit processing of 4×2 GSTBC PT-FEC-OFDM

### 2.6.4 4×2 STBC

When the channel conditions deteriorate, STBC with four transmit antennas can be used to improve the signal link quality. Figure 15 illustrates the steps involved in transmit processing in a 4×2 STBC PT-FEC-OFDM. STBC as given in [2] shall be used. Same as the 2×2 STBC PT-FEC-OFDM, time-domain STBC encoding shall be performed in order to reduce the transmitter complexity.

Figure 15 Transmit processing of 4×2 STBC PT-FEC-OFDM

The bits in the DATA field shall be FEC-encoded, bit-interleaved, and mapped to the specific modulation symbols, according to 2.5.4, 2.5.5, and 2.5.6. The modulation symbols shall be transformed through matrix T, according to 2.5.8, and OFDM modulated which includes the inverse Fast Fourier Transform (IFFT) and cyclic prefix (CP) insertion, as specified in 2.5.9.

As shown in [2], two STBC designs can be implemented for the 4 transmit antenna system. Further study needs to be carried out to decide on the optimal combination of the code rate in STBC and FEC.

#### 2.6.4.1 Time-Domain STBC Implementation for 4×2 STBC

To be given later.

### 2.6.5 Beamforming

Figure 16 - Block diagram of a beam multiplexing transmitter

To achieve both high throughput and beamforming gain in downlink when the spatial channel at AP is highly correlated, the fixed beam multiplexing transmission mode can be used at AP for the 4×2 MIMO system, where AP has four transmit antennas and station has two receive antennas. The block diagram of the fixed beam multiplexing transmission using four antennas at AP is illustrated in Figure 16. The time domain signals after OFDM modulation s1 and s2 stand for two data streams at AP to be transmitted simultaneously to a station with two receiving antennas. The two data streams are first passed into two transmit beamformers w1 and w2 for beamforming processing, then passed on to a signal combiner which performs a simple summing function of the two beamforming processed inputs to produce a signal vector x for transmission through four antennas.

The beamforming processing at each beamformer is simply to multiply each symbol of the corresponding input data stream with a 4  1 complex weight vector w1 or w2 and transfer each scalar symbol into a 4  1 vector. The values for the beamforming weights w1 and w2 have been pre-stored in a vector set table in the AP’s memory. However, the AP needs to determine which vectors in the pre-stored set table to be used on line.

The selection of the two vectors from the pre-stored vector set table is based on the measurement from uplink, where the four antennas are used to form a plurality of fixed beams for multiple fixed-beam reception. In the uplink, two fixed beams should be identified with most reception power of signals from station. Based on the identified two fixed beams, the beamforming weights w1 and w2 for downlink multiplexing transmission are determined by looking up the pre-stored vector set table, where each pair of fixed beams are mapped to two beamforming vectors.

The pre-stored mapping table for mapping from each pair of fixed beams to two beamforming vectors is set up as follows.

- For each pair of uplink fixed beams i and j (i  j), the main beam direction are  i and  j respectively. Based on the direction angle  i and  j, a 4  4 matrix can be formed as

R=12[a(ϕi)⋅aH(ϕi)+a(ϕj)⋅aH(ϕj)]

where a(θp) (p = i or j) is the downlink 4  1 steering vector at DOA p and a(θp)=[1,ej2πcos(θp)λd…ej2π(M−1)cos(θp)λd]T for a uniform linear antenna array with antenna spacing d,  is the wavelength of downlink center frequency, M is the number of antennas, i.e. M = 4 here, and superscript ‘T’ denotes transpose operation.

- Perform eigen-decomposition to the matrix R, and the two eigen-vectors corresponding to the largest two eigen-values are the target pair of beamforming vectors mapped from the fixed beam i and j.

### 2.6.6 Antenna selection

Figure 17 - Illustration of Antenna Selection for the 4×2 System

For an AP with complexity constraint to deploy  four RF chains, antenna selection transmission can be employed to achieve full (4 ) transmit diversity gain with four antenna elements but fewer (two for the multiplexing case or one for space time coding) RF parts, as illustrated in Figure 17.

The selection of the antenna is determined by station and the indexes of the selected antenna to be used for transmission are fed back to the AP. In the downlink of the 4  2 system setup, station measures the channel based on the channel estimation and determines which two antennas are to be used in the later multiplexing transmission. The indexes of the selected antenna are fed back to station via uplink transmission (inserted into signal field). Three bits altogether (for all subcarriers) are required for the feedback as indicated in Table 15.

Table 15 - Antenna Selection Index

| Bits | Selected antennas |
| --- | --- |
| 000 | #1 and #2 |
| 001 | #1 and #3 |
| 010 | #1 and #4 |
| 011 | #2 and #3 |
| 100 | #2 and #4 |
| 101 | #3 and #4 |
| 110 | reserved |
| 111 | reserved |

# ANNEX A Generation of short training preamble for mandatory mode of subcarrier arrangement

The frequency-domain and time-domain short training preambles for the mandatory mode are given in Table 16 and Table 17, respectively.

Table 16 - Frequency domain representation of the short training preamble for mandatory mode subcarrier arrangement

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -64 | 0.000 | 0.000 |  | -32 | 0.000 | 0.000 |  | 0 | 0.000 | 0.000 |  | 32 | 0.000 | 0.000 |
| -63 | 0.000 | 0.000 |  | -31 | 0.000 | 0.000 |  | 1 | 0.000 | 0.000 |  | 33 | 0.000 | 0.000 |
| -62 | 0.000 | 0.000 |  | -30 | 0.000 | 0.000 |  | 2 | 0.000 | 0.000 |  | 34 | 0.000 | 0.000 |
| -61 | 0.000 | 0.000 |  | -29 | 0.000 | 0.000 |  | 3 | 0.000 | 0.000 |  | 35 | 0.000 | 0.000 |
| -60 | 0.000 | 0.000 |  | -28 | 1.472 | 1.472 |  | 4 | 0.000 | 0.000 |  | 36 | -1.472 | -1.472 |
| -59 | 0.000 | 0.000 |  | -27 | 0.000 | 0.000 |  | 5 | 0.000 | 0.000 |  | 37 | 0.000 | 0.000 |
| -58 | 0.000 | 0.000 |  | -26 | 0.000 | 0.000 |  | 6 | 0.000 | 0.000 |  | 38 | 0.000 | 0.000 |
| -57 | 0.000 | 0.000 |  | -25 | 0.000 | 0.000 |  | 7 | 0.000 | 0.000 |  | 39 | 0.000 | 0.000 |
| -56 | -1.472 | -1.472 |  | -24 | 1.472 | 1.472 |  | 8 | -1.472 | -1.472 |  | 40 | 1.472 | 1.472 |
| -55 | 0.000 | 0.000 |  | -23 | 0.000 | 0.000 |  | 9 | 0.000 | 0.000 |  | 41 | 0.000 | 0.000 |
| -54 | 0.000 | 0.000 |  | -22 | 0.000 | 0.000 |  | 10 | 0.000 | 0.000 |  | 42 | 0.000 | 0.000 |
| -53 | 0.000 | 0.000 |  | -21 | 0.000 | 0.000 |  | 11 | 0.000 | 0.000 |  | 43 | 0.000 | 0.000 |
| -52 | 1.472 | 1.472 |  | -20 | 1.472 | 1.472 |  | 12 | -1.472 | -1.472 |  | 44 | -1.472 | -1.472 |
| -51 | 0.000 | 0.000 |  | -19 | 0.000 | 0.000 |  | 13 | 0.000 | 0.000 |  | 45 | 0.000 | 0.000 |
| -50 | 0.000 | 0.000 |  | -18 | 0.000 | 0.000 |  | 14 | 0.000 | 0.000 |  | 46 | 0.000 | 0.000 |
| -49 | 0.000 | 0.000 |  | -17 | 0.000 | 0.000 |  | 15 | 0.000 | 0.000 |  | 47 | 0.000 | 0.000 |
| -48 | 1.472 | 1.472 |  | -16 | 1.472 | 1.472 |  | 16 | 1.472 | 1.472 |  | 48 | 1.472 | 1.472 |
| -47 | 0.000 | 0.000 |  | -15 | 0.000 | 0.000 |  | 17 | 0.000 | 0.000 |  | 49 | 0.000 | 0.000 |
| -46 | 0.000 | 0.000 |  | -14 | 0.000 | 0.000 |  | 18 | 0.000 | 0.000 |  | 50 | 0.000 | 0.000 |
| -45 | 0.000 | 0.000 |  | -13 | 0.000 | 0.000 |  | 19 | 0.000 | 0.000 |  | 51 | 0.000 | 0.000 |
| -44 | 1.472 | 1.472 |  | -12 | 1.472 | 1.472 |  | 20 | -1.472 | -1.472 |  | 52 | -1.472 | -1.472 |
| -43 | 0.000 | 0.000 |  | -11 | 0.000 | 0.000 |  | 21 | 0.000 | 0.000 |  | 53 | 0.000 | 0.000 |
| -42 | 0.000 | 0.000 |  | -10 | 0.000 | 0.000 |  | 22 | 0.000 | 0.000 |  | 54 | 0.000 | 0.000 |
| -41 | 0.000 | 0.000 |  | -9 | 0.000 | 0.000 |  | 23 | 0.000 | 0.000 |  | 55 | 0.000 | 0.000 |
| -40 | -1.472 | -1.472 |  | -8 | 1.472 | 1.472 |  | 24 | -1.472 | -1.472 |  | 56 | 1.472 | 1.472 |
| -39 | 0.000 | 0.000 |  | -7 | 0.000 | 0.000 |  | 25 | 0.000 | 0.000 |  | 57 | 0.000 | 0.000 |
| -38 | 0.000 | 0.000 |  | -6 | 0.000 | 0.000 |  | 26 | 0.000 | 0.000 |  | 58 | 0.000 | 0.000 |
| -37 | 0.000 | 0.000 |  | -5 | 0.000 | 0.000 |  | 27 | 0.000 | 0.000 |  | 59 | 0.000 | 0.000 |
| -36 | -1.472 | -1.472 |  | -4 | 0.000 | 0.000 |  | 28 | 1.472 | 1.472 |  | 60 | 0.000 | 0.000 |
| -35 | 0.000 | 0.000 |  | -3 | 0.000 | 0.000 |  | 29 | 0.000 | 0.000 |  | 61 | 0.000 | 0.000 |
| -34 | 0.000 | 0.000 |  | -2 | 0.000 | 0.000 |  | 30 | 0.000 | 0.000 |  | 62 | 0.000 | 0.000 |
| -33 | 0.000 | 0.000 |  | -1 | 0.000 | 0.000 |  | 31 | 0.000 | 0.000 |  | 63 | 0.000 | 0.000 |

Table 17 - One period of IFFT of the short training preamble for mandatory mode subcarrier arrangement

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.520 | 0.520 |  | 32 | 0.520 | 0.520 |  | 64 | 0.520 | 0.520 |  | 96 | 0.520 | 0.520 |
| 1 | 0.823 | -0.620 |  | 33 | 0.823 | -0.620 |  | 65 | 0.823 | -0.620 |  | 97 | 0.823 | -0.620 |
| 2 | 0.736 | -0.736 |  | 34 | 0.736 | -0.736 |  | 66 | 0.736 | -0.736 |  | 98 | 0.736 | -0.736 |
| 3 | 0.323 | -0.901 |  | 35 | 0.323 | -0.901 |  | 67 | 0.323 | -0.901 |  | 99 | 0.323 | -0.901 |
| 4 | -0.520 | -0.520 |  | 36 | -0.520 | -0.520 |  | 68 | -0.520 | -0.520 |  | 100 | -0.520 | -0.520 |
| 5 | 0.024 | 0.842 |  | 37 | 0.024 | 0.842 |  | 69 | 0.024 | 0.842 |  | 101 | 0.024 | 0.842 |
| 6 | 0.736 | -0.736 |  | 38 | 0.736 | -0.736 |  | 70 | 0.736 | -0.736 |  | 102 | 0.736 | -0.736 |
| 7 | -0.654 | -0.367 |  | 39 | -0.654 | -0.367 |  | 71 | -0.654 | -0.367 |  | 103 | -0.654 | -0.367 |
| 8 | 0.520 | 0.520 |  | 40 | 0.520 | 0.520 |  | 72 | 0.520 | 0.520 |  | 104 | 0.520 | 0.520 |
| 9 | 0.367 | 0.654 |  | 41 | 0.367 | 0.654 |  | 73 | 0.367 | 0.654 |  | 105 | 0.367 | 0.654 |
| 10 | -0.736 | 0.736 |  | 42 | -0.736 | 0.736 |  | 74 | -0.736 | 0.736 |  | 106 | -0.736 | 0.736 |
| 11 | -0.842 | -0.024 |  | 43 | -0.842 | -0.024 |  | 75 | -0.842 | -0.024 |  | 107 | -0.842 | -0.024 |
| 12 | -0.520 | -0.520 |  | 44 | -0.520 | -0.520 |  | 76 | -0.520 | -0.520 |  | 108 | -0.520 | -0.520 |
| 13 | 0.901 | -0.323 |  | 45 | 0.901 | -0.323 |  | 77 | 0.901 | -0.323 |  | 109 | 0.901 | -0.323 |
| 14 | -0.736 | 0.736 |  | 46 | -0.736 | 0.736 |  | 78 | -0.736 | 0.736 |  | 110 | -0.736 | 0.736 |
| 15 | 0.620 | -0.823 |  | 47 | 0.620 | -0.823 |  | 79 | 0.620 | -0.823 |  | 111 | 0.620 | -0.823 |
| 16 | 0.520 | 0.520 |  | 48 | 0.520 | 0.520 |  | 80 | 0.520 | 0.520 |  | 112 | 0.520 | 0.520 |
| 17 | -0.823 | 0.620 |  | 49 | -0.823 | 0.620 |  | 81 | -0.823 | 0.620 |  | 113 | -0.823 | 0.620 |
| 18 | 0.736 | -0.736 |  | 50 | 0.736 | -0.736 |  | 82 | 0.736 | -0.736 |  | 114 | 0.736 | -0.736 |
| 19 | -0.323 | 0.901 |  | 51 | -0.323 | 0.901 |  | 83 | -0.323 | 0.901 |  | 115 | -0.323 | 0.901 |
| 20 | -0.520 | -0.520 |  | 52 | -0.520 | -0.520 |  | 84 | -0.520 | -0.520 |  | 116 | -0.520 | -0.520 |
| 21 | -0.024 | -0.842 |  | 53 | -0.024 | -0.842 |  | 85 | -0.024 | -0.842 |  | 117 | -0.024 | -0.842 |
| 22 | 0.736 | -0.736 |  | 54 | 0.736 | -0.736 |  | 86 | 0.736 | -0.736 |  | 118 | 0.736 | -0.736 |
| 23 | 0.654 | 0.367 |  | 55 | 0.654 | 0.367 |  | 87 | 0.654 | 0.367 |  | 119 | 0.654 | 0.367 |
| 24 | 0.520 | 0.520 |  | 56 | 0.520 | 0.520 |  | 88 | 0.520 | 0.520 |  | 120 | 0.520 | 0.520 |
| 25 | -0.367 | -0.654 |  | 57 | -0.367 | -0.654 |  | 89 | -0.367 | -0.654 |  | 121 | -0.367 | -0.654 |
| 26 | -0.736 | 0.736 |  | 58 | -0.736 | 0.736 |  | 90 | -0.736 | 0.736 |  | 122 | -0.736 | 0.736 |
| 27 | 0.842 | 0.024 |  | 59 | 0.842 | 0.024 |  | 91 | 0.842 | 0.024 |  | 123 | 0.842 | 0.024 |
| 28 | -0.520 | -0.520 |  | 60 | -0.520 | -0.520 |  | 92 | -0.520 | -0.520 |  | 124 | -0.520 | -0.520 |
| 29 | -0.901 | 0.323 |  | 61 | -0.901 | 0.323 |  | 93 | -0.901 | 0.323 |  | 125 | -0.901 | 0.323 |
| 30 | -0.736 | 0.736 |  | 62 | -0.736 | 0.736 |  | 94 | -0.736 | 0.736 |  | 126 | -0.736 | 0.736 |
| 31 | -0.620 | 0.823 |  | 63 | -0.620 | 0.823 |  | 95 | -0.620 | 0.823 |  | 127 | -0.620 | 0.823 |

# ANNEX B Generation of long training preamble for mandatory mode of subcarrier arrangement

The long training symbols for antenna 1 at time 1,2,3,4, antenna 2 at time 1,3, antenna 3 at time 1,2 and antenna 4 at time 1,4 for the mandatory mode subcarrier arrangement are described in the time domain in Table 18 and Table 19, respectively.

Table 18 - Time domain representation of the long training symbol (sequences P+L) for mandatory mode subcarrier arrangement

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -2.071 | 0.000 |  | 32 | -0.905 | -0.354 |  | 64 | 0.248 | 0.000 |  | 96 | 0.000 | 0.354 |
| 1 | 0.181 | 0.000 |  | 33 | -1.781 | 0.000 |  | 65 | 0.604 | 0.000 |  | 97 | 0.000 | 0.000 |
| 2 | 2.219 | 0.312 |  | 34 | -0.548 | -0.167 |  | 66 | 0.699 | -0.312 |  | 98 | 0.224 | 0.167 |
| 3 | -0.082 | 0.000 |  | 35 | 0.029 | 0.000 |  | 67 | -0.500 | 0.000 |  | 99 | 0.000 | 0.000 |
| 4 | 1.005 | 0.294 |  | 36 | 1.313 | 0.196 |  | 68 | 0.956 | -0.294 |  | 100 | -0.347 | -0.196 |
| 5 | -0.201 | 0.000 |  | 37 | 0.727 | 0.000 |  | 69 | 0.886 | 0.000 |  | 101 | 0.000 | 0.000 |
| 6 | 0.665 | -0.035 |  | 38 | -1.199 | 0.352 |  | 70 | -0.513 | 0.035 |  | 102 | 0.312 | -0.352 |
| 7 | -0.665 | 0.000 |  | 39 | -0.671 | 0.000 |  | 71 | 0.728 | 0.000 |  | 103 | 0.000 | 0.000 |
| 8 | -1.577 | -0.327 |  | 40 | 1.815 | 0.135 |  | 72 | -1.014 | 0.327 |  | 104 | -0.135 | -0.135 |
| 9 | 0.864 | 0.000 |  | 41 | 0.496 | 0.000 |  | 73 | -1.103 | 0.000 |  | 105 | 0.000 | 0.000 |
| 10 | 0.800 | -0.273 |  | 42 | -1.655 | -0.224 |  | 74 | -1.535 | 0.273 |  | 106 | -0.103 | 0.224 |
| 11 | 0.214 | 0.000 |  | 43 | 1.311 | 0.000 |  | 75 | -0.201 | 0.000 |  | 107 | 0.000 | 0.000 |
| 12 | -1.171 | 0.069 |  | 44 | 0.819 | -0.347 |  | 76 | -0.047 | -0.069 |  | 108 | 0.294 | 0.347 |
| 13 | -0.034 | 0.000 |  | 45 | -0.986 | 0.000 |  | 77 | 0.331 | 0.000 |  | 109 | 0.000 | 0.000 |
| 14 | -1.530 | 0.338 |  | 46 | 1.944 | -0.103 |  | 78 | 0.806 | -0.338 |  | 110 | -0.352 | 0.103 |
| 15 | 0.227 | 0.000 |  | 47 | -0.489 | 0.000 |  | 79 | -0.733 | 0.000 |  | 111 | 0.000 | 0.000 |
| 16 | 1.405 | 0.250 |  | 48 | -0.566 | 0.250 |  | 80 | 0.541 | -0.250 |  | 112 | 0.250 | -0.250 |
| 17 | -0.461 | 0.000 |  | 49 | -0.364 | 0.000 |  | 81 | -0.089 | 0.000 |  | 113 | 0.000 | 0.000 |
| 18 | -1.321 | -0.103 |  | 50 | -1.202 | 0.338 |  | 82 | 0.054 | 0.103 |  | 114 | -0.035 | -0.338 |
| 19 | -0.075 | 0.000 |  | 51 | -0.161 | 0.000 |  | 83 | 0.882 | 0.000 |  | 115 | 0.000 | 0.000 |
| 20 | 2.238 | -0.347 |  | 52 | -1.060 | 0.069 |  | 84 | -0.083 | 0.347 |  | 116 | -0.196 | -0.069 |
| 21 | -0.997 | 0.000 |  | 53 | 1.413 | 0.000 |  | 85 | 0.412 | 0.000 |  | 117 | 0.000 | 0.000 |
| 22 | -0.226 | -0.224 |  | 54 | 0.246 | -0.273 |  | 86 | 1.801 | 0.224 |  | 118 | 0.338 | 0.273 |
| 23 | 0.593 | 0.000 |  | 55 | -0.352 | 0.000 |  | 87 | -0.328 | 0.000 |  | 119 | 0.000 | 0.000 |
| 24 | 1.379 | 0.135 |  | 56 | 0.438 | -0.327 |  | 88 | 0.615 | -0.135 |  | 120 | -0.327 | 0.327 |
| 25 | 0.867 | 0.000 |  | 57 | 0.332 | 0.000 |  | 89 | -0.387 | 0.000 |  | 121 | 0.000 | 0.000 |
| 26 | -0.961 | 0.352 |  | 58 | 0.448 | -0.035 |  | 90 | -2.785 | -0.352 |  | 122 | 0.167 | 0.035 |
| 27 | -0.471 | 0.000 |  | 59 | 0.002 | 0.000 |  | 91 | -0.137 | 0.000 |  | 123 | 0.000 | 0.000 |
| 28 | -0.641 | 0.196 |  | 60 | -1.342 | 0.294 |  | 92 | -1.903 | -0.196 |  | 124 | 0.069 | -0.294 |
| 29 | 0.597 | 0.000 |  | 61 | -1.562 | 0.000 |  | 93 | 0.255 | 0.000 |  | 125 | 0.000 | 0.000 |
| 30 | 0.187 | -0.167 |  | 62 | 0.942 | 0.312 |  | 94 | 2.385 | 0.167 |  | 126 | -0.273 | -0.312 |
| 31 | 0.548 | 0.000 |  | 63 | 0.343 | 0.000 |  | 95 | -0.013 | 0.000 |  | 127 | 0.000 | 0.000 |

Table 19 - Time domain representation of the long training symbol (sequences P-L) for mandatory mode subcarrier arrangement

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 2.778 | 0.000 |  | 32 | 0.905 | -0.354 |  | 64 | -0.955 | 0.000 |  | 96 | 0.000 | 0.354 |
| 1 | -0.181 | 0.000 |  | 33 | 1.781 | 0.000 |  | 65 | -0.604 | 0.000 |  | 97 | 0.000 | 0.000 |
| 2 | -2.765 | 0.312 |  | 34 | 0.099 | -0.167 |  | 66 | -0.153 | -0.312 |  | 98 | 0.224 | 0.167 |
| 3 | 0.082 | 0.000 |  | 35 | -0.029 | 0.000 |  | 67 | 0.500 | 0.000 |  | 99 | 0.000 | 0.000 |
| 4 | -0.867 | 0.294 |  | 36 | -0.619 | 0.196 |  | 68 | -1.094 | -0.294 |  | 100 | -0.347 | -0.196 |
| 5 | 0.201 | 0.000 |  | 37 | -0.727 | 0.000 |  | 69 | -0.886 | 0.000 |  | 101 | 0.000 | 0.000 |
| 6 | -0.332 | -0.035 |  | 38 | 0.575 | 0.352 |  | 70 | 0.180 | 0.035 |  | 102 | 0.312 | -0.352 |
| 7 | 0.665 | 0.000 |  | 39 | 0.671 | 0.000 |  | 71 | -0.728 | 0.000 |  | 103 | 0.000 | 0.000 |
| 8 | 0.923 | -0.327 |  | 40 | -1.544 | 0.135 |  | 72 | 1.668 | 0.327 |  | 104 | -0.135 | -0.135 |
| 9 | -0.864 | 0.000 |  | 41 | -0.496 | 0.000 |  | 73 | 1.103 | 0.000 |  | 105 | 0.000 | 0.000 |
| 10 | -0.123 | -0.273 |  | 42 | 1.861 | -0.224 |  | 74 | 0.859 | 0.273 |  | 106 | -0.103 | 0.224 |
| 11 | -0.214 | 0.000 |  | 43 | -1.311 | 0.000 |  | 75 | 0.201 | 0.000 |  | 107 | 0.000 | 0.000 |
| 12 | 0.778 | 0.069 |  | 44 | -1.407 | -0.347 |  | 76 | 0.439 | -0.069 |  | 108 | 0.294 | 0.347 |
| 13 | 0.034 | 0.000 |  | 45 | 0.986 | 0.000 |  | 77 | -0.331 | 0.000 |  | 109 | 0.000 | 0.000 |
| 14 | 1.460 | 0.338 |  | 46 | -1.240 | -0.103 |  | 78 | -0.737 | -0.338 |  | 110 | -0.352 | 0.103 |
| 15 | -0.227 | 0.000 |  | 47 | 0.489 | 0.000 |  | 79 | 0.733 | 0.000 |  | 111 | 0.000 | 0.000 |
| 16 | -0.905 | 0.250 |  | 48 | 0.066 | 0.250 |  | 80 | -1.041 | -0.250 |  | 112 | 0.250 | -0.250 |
| 17 | 0.461 | 0.000 |  | 49 | 0.364 | 0.000 |  | 81 | 0.089 | 0.000 |  | 113 | 0.000 | 0.000 |
| 18 | 0.617 | -0.103 |  | 50 | 1.271 | 0.338 |  | 82 | 0.650 | 0.103 |  | 114 | -0.035 | -0.338 |
| 19 | 0.075 | 0.000 |  | 51 | 0.161 | 0.000 |  | 83 | -0.882 | 0.000 |  | 115 | 0.000 | 0.000 |
| 20 | -1.650 | -0.347 |  | 52 | 1.453 | 0.069 |  | 84 | -0.505 | 0.347 |  | 116 | -0.196 | -0.069 |
| 21 | 0.997 | 0.000 |  | 53 | -1.413 | 0.000 |  | 85 | -0.412 | 0.000 |  | 117 | 0.000 | 0.000 |
| 22 | 0.021 | -0.224 |  | 54 | -0.923 | -0.273 |  | 86 | -1.596 | 0.224 |  | 118 | 0.338 | 0.273 |
| 23 | -0.593 | 0.000 |  | 55 | 0.352 | 0.000 |  | 87 | 0.328 | 0.000 |  | 119 | 0.000 | 0.000 |
| 24 | -1.649 | 0.135 |  | 56 | 0.215 | -0.327 |  | 88 | -0.344 | -0.135 |  | 120 | -0.327 | 0.327 |
| 25 | -0.867 | 0.000 |  | 57 | -0.332 | 0.000 |  | 89 | 0.387 | 0.000 |  | 121 | 0.000 | 0.000 |
| 26 | 1.585 | 0.352 |  | 58 | -0.781 | -0.035 |  | 90 | 2.161 | -0.352 |  | 122 | 0.167 | 0.035 |
| 27 | 0.471 | 0.000 |  | 59 | -0.002 | 0.000 |  | 91 | 0.137 | 0.000 |  | 123 | 0.000 | 0.000 |
| 28 | -0.052 | 0.196 |  | 60 | 1.204 | 0.294 |  | 92 | 2.597 | -0.196 |  | 124 | 0.069 | -0.294 |
| 29 | -0.597 | 0.000 |  | 61 | 1.562 | 0.000 |  | 93 | -0.255 | 0.000 |  | 125 | 0.000 | 0.000 |
| 30 | 0.261 | -0.167 |  | 62 | -0.395 | 0.312 |  | 94 | -2.834 | 0.167 |  | 126 | -0.273 | -0.312 |
| 31 | -0.548 | 0.000 |  | 63 | -0.343 | 0.000 |  | 95 | 0.013 | 0.000 |  | 127 | 0.000 | 0.000 |

# ANNEX C Generation of short training preamble for optional mode of subcarrier arrangement

The short training preamble is described in the frequency domain as in Table 20.

Table 20 - Frequency domain representation of the short training preamble for optional mode of subcarrier arrangement

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -64 | 0.0 | 0.0 |  | -32 | 1.414 | 1.414 |  | 0 | 0.0 | 0.0 |  | 32 | -1.414 | -1.414 |
| -63 | 0.0 | 0.0 |  | -31 | 0.0 | 0.0 |  | 1 | 0.0 | 0.0 |  | 33 | 0.0 | 0.0 |
| -62 | 0.0 | 0.0 |  | -30 | 0.0 | 0.0 |  | 2 | 0.0 | 0.0 |  | 34 | 0.0 | 0.0 |
| -61 | 0.0 | 0.0 |  | -29 | 0.0 | 0.0 |  | 3 | 0.0 | 0.0 |  | 35 | 0.0 | 0.0 |
| -60 | 0.0 | 0.0 |  | -28 | 1.414 | 1.414 |  | 4 | 1.414 | 1.414 |  | 36 | 1.414 | 1.414 |
| -59 | 0.0 | 0.0 |  | -27 | 0.0 | 0.0 |  | 5 | 0.0 | 0.0 |  | 37 | 0.0 | 0.0 |
| -58 | 0.0 | 0.0 |  | -26 | 0.0 | 0.0 |  | 6 | 0.0 | 0.0 |  | 38 | 0.0 | 0.0 |
| -57 | 0.0 | 0.0 |  | -25 | 0.0 | 0.0 |  | 7 | 0.0 | 0.0 |  | 39 | 0.0 | 0.0 |
| -56 | 0.0 | 0.0 |  | -24 | -1.414 | -1.414 |  | 8 | 1.414 | 1.414 |  | 40 | -1.414 | -1.414 |
| -55 | 0.0 | 0.0 |  | -23 | 0.0 | 0.0 |  | 9 | 0.0 | 0.0 |  | 41 | 0.0 | 0.0 |
| -54 | 0.0 | 0.0 |  | -22 | 0.0 | 0.0 |  | 10 | 0.0 | 0.0 |  | 42 | 0.0 | 0.0 |
| -53 | 0.0 | 0.0 |  | -21 | 0.0 | 0.0 |  | 11 | 0.0 | 0.0 |  | 43 | 0.0 | 0.0 |
| -52 | 1.414 | 1.414 |  | -20 | -1.414 | -1.414 |  | 12 | 1.414 | 1.414 |  | 44 | 1.414 | 1.414 |
| -51 | 0.0 | 0.0 |  | -19 | 0.0 | 0.0 |  | 13 | 0.0 | 0.0 |  | 45 | 0.0 | 0.0 |
| -50 | 0.0 | 0.0 |  | -18 | 0.0 | 0.0 |  | 14 | 0.0 | 0.0 |  | 46 | 0.0 | 0.0 |
| -49 | 0.0 | 0.0 |  | -17 | 0.0 | 0.0 |  | 15 | 0.0 | 0.0 |  | 47 | 0.0 | 0.0 |
| -48 | 1.414 | 1.414 |  | -16 | 1.414 | 1.414 |  | 16 | 1.414 | 1.414 |  | 48 | -1.414 | -1.414 |
| -47 | 0.0 | 0.0 |  | -15 | 0.0 | 0.0 |  | 17 | 0.0 | 0.0 |  | 49 | 0.0 | 0.0 |
| -46 | 0.0 | 0.0 |  | -14 | 0.0 | 0.0 |  | 18 | 0.0 | 0.0 |  | 50 | 0.0 | 0.0 |
| -45 | 0.0 | 0.0 |  | -13 | 0.0 | 0.0 |  | 19 | 0.0 | 0.0 |  | 51 | 0.0 | 0.0 |
| -44 | -1.414 | -1.414 |  | -12 | -1.414 | -1.414 |  | 20 | 1.414 | 1.414 |  | 52 | 1.414 | 1.414 |
| -43 | 0.0 | 0.0 |  | -11 | 0.0 | 0.0 |  | 21 | 0.0 | 0.0 |  | 53 | 0.0 | 0.0 |
| -42 | 0.0 | 0.0 |  | -10 | 0.0 | 0.0 |  | 22 | 0.0 | 0.0 |  | 54 | 0.0 | 0.0 |
| -41 | 0.0 | 0.0 |  | -9 | 0.0 | 0.0 |  | 23 | 0.0 | 0.0 |  | 55 | 0.0 | 0.0 |
| -40 | -1.414 | -1.414 |  | -8 | -1.414 | -1.414 |  | 24 | 1.414 | 1.414 |  | 56 | 0.0 | 0.0 |
| -39 | 0.0 | 0.0 |  | -7 | 0.0 | 0.0 |  | 25 | 0.0 | 0.0 |  | 57 | 0.0 | 0.0 |
| -38 | 0.0 | 0.0 |  | -6 | 0.0 | 0.0 |  | 26 | 0.0 | 0.0 |  | 58 | 0.0 | 0.0 |
| -37 | 0.0 | 0.0 |  | -5 | 0.0 | 0.0 |  | 27 | 0.0 | 0.0 |  | 59 | 0.0 | 0.0 |
| -36 | -1.414 | -1.414 |  | -4 | 1.414 | 1.414 |  | 28 | -1.414 | -1.414 |  | 60 | 0.0 | 0.0 |
| -35 | 0.0 | 0.0 |  | -3 | 0.0 | 0.0 |  | 29 | 0.0 | 0.0 |  | 61 | 0.0 | 0.0 |
| -34 | 0.0 | 0.0 |  | -2 | 0.0 | 0.0 |  | 30 | 0.0 | 0.0 |  | 62 | 0.0 | 0.0 |
| -33 | 0.0 | 0.0 |  | -1 | 0.0 | 0.0 |  | 31 | 0.0 | 0.0 |  | 63 | 0.0 | 0.0 |

One period of the IFFT on the contents of  Table 20 is given in Table 21.

Table 21 - One period of IFFT of the short training preamble for optional mode of subcarrier arrangement

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.500 | 0.500 |  | 32 | 0.500 | 0.500 |  | 64 | 0.500 | 0.500 |  | 96 | 0.500 | 0.500 |
| 1 | -0.145 | 0.764 |  | 33 | -0.145 | 0.764 |  | 65 | -0.145 | 0.764 |  | 97 | -0.145 | 0.764 |
| 2 | -0.140 | 1.147 |  | 34 | -0.140 | 1.147 |  | 66 | -0.140 | 1.147 |  | 98 | -0.140 | 1.147 |
| 3 | -0.702 | 0.400 |  | 35 | -0.702 | 0.400 |  | 67 | -0.702 | 0.400 |  | 99 | -0.702 | 0.400 |
| 4 | -0.780 | 0.280 |  | 36 | -0.780 | 0.280 |  | 68 | -0.780 | 0.280 |  | 100 | -0.780 | 0.280 |
| 5 | 0.818 | 0.059 |  | 37 | 0.818 | 0.059 |  | 69 | 0.818 | 0.059 |  | 101 | 0.818 | 0.059 |
| 6 | 0.142 | -0.766 |  | 38 | 0.142 | -0.766 |  | 70 | 0.142 | -0.766 |  | 102 | 0.142 | -0.766 |
| 7 | -0.555 | 1.092 |  | 39 | -0.555 | 1.092 |  | 71 | -0.555 | 1.092 |  | 103 | -0.555 | 1.092 |
| 8 | 0.250 | 0.750 |  | 40 | 0.250 | 0.750 |  | 72 | 0.250 | 0.750 |  | 104 | 0.250 | 0.750 |
| 9 | 0.577 | -0.790 |  | 41 | 0.577 | -0.790 |  | 73 | 0.577 | -0.790 |  | 105 | 0.577 | -0.790 |
| 10 | 0.620 | -0.703 |  | 42 | 0.620 | -0.703 |  | 74 | 0.620 | -0.703 |  | 106 | 0.620 | -0.703 |
| 11 | 0.174 | -0.834 |  | 43 | 0.174 | -0.834 |  | 75 | 0.174 | -0.834 |  | 107 | 0.174 | -0.834 |
| 12 | -0.780 | 0.280 |  | 44 | -0.780 | 0.280 |  | 76 | -0.780 | 0.280 |  | 108 | -0.780 | 0.280 |
| 13 | -0.799 | -0.530 |  | 45 | -0.799 | -0.530 |  | 77 | -0.799 | -0.530 |  | 109 | -0.799 | -0.530 |
| 14 | 0.414 | -0.714 |  | 46 | 0.414 | -0.714 |  | 78 | 0.414 | -0.714 |  | 110 | 0.414 | -0.714 |
| 15 | -0.420 | 0.890 |  | 47 | -0.420 | 0.890 |  | 79 | -0.420 | 0.890 |  | 111 | -0.420 | 0.890 |
| 16 | -0.500 | -0.500 |  | 48 | -0.500 | -0.500 |  | 80 | -0.500 | -0.500 |  | 112 | -0.500 | -0.500 |
| 17 | 0.890 | -0.420 |  | 49 | 0.890 | -0.420 |  | 81 | 0.890 | -0.420 |  | 113 | 0.890 | -0.420 |
| 18 | -0.714 | 0.414 |  | 50 | -0.714 | 0.414 |  | 82 | -0.714 | 0.414 |  | 114 | -0.714 | 0.414 |
| 19 | -0.530 | -0.799 |  | 51 | -0.530 | -0.799 |  | 83 | -0.530 | -0.799 |  | 115 | -0.530 | -0.799 |
| 20 | 0.280 | -0.780 |  | 52 | 0.280 | -0.780 |  | 84 | 0.280 | -0.780 |  | 116 | 0.280 | -0.780 |
| 21 | -0.834 | 0.174 |  | 53 | -0.834 | 0.174 |  | 85 | -0.834 | 0.174 |  | 117 | -0.834 | 0.174 |
| 22 | -0.703 | 0.620 |  | 54 | -0.703 | 0.620 |  | 86 | -0.703 | 0.620 |  | 118 | -0.703 | 0.620 |
| 23 | -0.790 | 0.577 |  | 55 | -0.790 | 0.577 |  | 87 | -0.790 | 0.577 |  | 119 | -0.790 | 0.577 |
| 24 | 0.750 | 0.250 |  | 56 | 0.750 | 0.250 |  | 88 | 0.750 | 0.250 |  | 120 | 0.750 | 0.250 |
| 25 | 1.092 | -0.555 |  | 57 | 1.092 | -0.555 |  | 89 | 1.092 | -0.555 |  | 121 | 1.092 | -0.555 |
| 26 | -0.766 | 0.142 |  | 58 | -0.766 | 0.142 |  | 90 | -0.766 | 0.142 |  | 122 | -0.766 | 0.142 |
| 27 | 0.059 | 0.818 |  | 59 | 0.059 | 0.818 |  | 91 | 0.059 | 0.818 |  | 123 | 0.059 | 0.818 |
| 28 | 0.280 | -0.780 |  | 60 | 0.280 | -0.780 |  | 92 | 0.280 | -0.780 |  | 124 | 0.280 | -0.780 |
| 29 | 0.400 | -0.702 |  | 61 | 0.400 | -0.702 |  | 93 | 0.400 | -0.702 |  | 125 | 0.400 | -0.702 |
| 30 | 1.147 | -0.140 |  | 62 | 1.147 | -0.140 |  | 94 | 1.147 | -0.140 |  | 126 | 1.147 | -0.140 |
| 31 | 0.764 | -0.145 |  | 63 | 0.764 | -0.145 |  | 95 | 0.764 | -0.145 |  | 127 | 0.764 | -0.145 |

# ANNEX D Generation of long training preamble for optional mode of subcarrier arrangement

The long training symbol for antenna 1 at time 1,2,3,4, antenna 2 at time 1,3, antenna 3 at time 1,2 and antenna 4 at time 1,4 is described in the time domain in Table 22.

Table 22 - Time domain representation of the long training symbol (sequences P+L)

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.770 | 0.000 |  | 32 | 0.378 | 0.000 |  | 64 | 0.838 | 0.000 |  | 96 | 0.000 | 0.000 |
| 1 | -1.770 | -0.074 |  | 33 | 0.279 | -0.176 |  | 65 | -0.756 | -0.074 |  | 97 | 0.116 | -0.176 |
| 2 | 1.549 | 0.275 |  | 34 | 1.031 | 0.079 |  | 66 | -1.636 | 0.275 |  | 98 | 0.106 | 0.079 |
| 3 | 0.093 | 0.012 |  | 35 | 0.049 | -0.262 |  | 67 | -0.509 | 0.012 |  | 99 | -0.225 | -0.262 |
| 4 | -0.916 | 0.163 |  | 36 | -1.552 | -0.163 |  | 68 | 0.902 | 0.163 |  | 100 | -0.288 | -0.163 |
| 5 | 0.154 | 0.301 |  | 37 | -1.751 | -0.051 |  | 69 | 0.595 | 0.301 |  | 101 | 0.118 | -0.051 |
| 6 | -0.949 | -0.003 |  | 38 | -0.032 | -0.350 |  | 70 | 2.065 | -0.003 |  | 102 | 0.310 | -0.350 |
| 7 | 1.312 | 0.281 |  | 39 | 0.954 | -0.031 |  | 71 | 1.040 | 0.281 |  | 103 | 0.035 | -0.031 |
| 8 | 1.909 | 0.125 |  | 40 | -1.117 | -0.125 |  | 72 | 0.485 | 0.125 |  | 104 | -0.125 | -0.125 |
| 9 | -0.644 | -0.042 |  | 41 | 0.030 | -0.208 |  | 73 | 0.531 | -0.042 |  | 105 | -0.010 | -0.208 |
| 10 | -0.424 | 0.211 |  | 42 | 1.232 | 0.142 |  | 74 | -1.434 | 0.211 |  | 106 | -0.065 | 0.142 |
| 11 | 0.403 | -0.142 |  | 43 | 0.717 | -0.108 |  | 75 | 1.092 | -0.142 |  | 107 | -0.190 | -0.108 |
| 12 | 0.875 | -0.068 |  | 44 | 1.365 | 0.068 |  | 76 | 1.563 | -0.068 |  | 108 | 0.057 | 0.068 |
| 13 | 0.191 | 0.013 |  | 45 | 0.401 | 0.237 |  | 77 | 0.122 | 0.013 |  | 109 | 0.343 | 0.237 |
| 14 | -0.756 | -0.324 |  | 46 | -0.063 | -0.030 |  | 78 | 0.538 | -0.324 |  | 110 | 0.102 | -0.030 |
| 15 | -0.564 | -0.044 |  | 47 | -1.061 | 0.294 |  | 79 | -0.278 | -0.044 |  | 111 | -0.274 | 0.294 |
| 16 | 0.308 | -0.177 |  | 48 | -1.536 | 0.177 |  | 80 | 0.609 | -0.177 |  | 112 | -0.177 | 0.177 |
| 17 | -0.035 | -0.294 |  | 49 | -1.099 | 0.044 |  | 81 | -0.085 | -0.294 |  | 113 | 0.078 | 0.044 |
| 18 | -0.801 | 0.030 |  | 50 | -2.025 | 0.324 |  | 82 | -0.132 | 0.030 |  | 114 | 0.033 | 0.324 |
| 19 | 1.018 | -0.237 |  | 51 | 0.378 | -0.013 |  | 83 | 1.196 | -0.237 |  | 115 | 0.004 | -0.013 |
| 20 | 0.014 | -0.068 |  | 52 | 0.971 | 0.068 |  | 84 | 0.103 | -0.068 |  | 116 | 0.193 | 0.068 |
| 21 | -1.539 | 0.108 |  | 53 | -2.090 | 0.142 |  | 85 | 0.667 | 0.108 |  | 117 | 0.121 | 0.142 |
| 22 | -0.491 | -0.142 |  | 54 | -0.860 | -0.211 |  | 86 | 0.981 | -0.142 |  | 118 | -0.262 | -0.211 |
| 23 | -0.775 | 0.208 |  | 55 | 0.483 | 0.042 |  | 87 | 0.472 | 0.208 |  | 119 | -0.284 | 0.042 |
| 24 | -0.184 | 0.125 |  | 56 | -1.001 | -0.125 |  | 88 | -1.114 | 0.125 |  | 120 | 0.125 | -0.125 |
| 25 | -1.163 | 0.031 |  | 57 | -0.271 | -0.281 |  | 89 | -0.958 | 0.031 |  | 121 | 0.259 | -0.281 |
| 26 | -1.108 | 0.350 |  | 58 | 0.040 | 0.003 |  | 90 | 2.203 | 0.350 |  | 122 | 0.016 | 0.003 |
| 27 | 1.082 | 0.051 |  | 59 | -1.096 | -0.301 |  | 91 | -0.225 | 0.051 |  | 123 | -0.049 | -0.301 |
| 28 | 0.282 | 0.163 |  | 60 | 0.279 | -0.163 |  | 92 | -1.177 | 0.163 |  | 124 | 0.038 | -0.163 |
| 29 | 0.541 | 0.262 |  | 61 | 0.591 | -0.012 |  | 93 | 1.511 | 0.262 |  | 125 | -0.122 | -0.012 |
| 30 | 0.488 | -0.079 |  | 62 | 0.889 | -0.275 |  | 94 | -0.644 | -0.079 |  | 126 | -0.241 | -0.275 |
| 31 | -0.070 | 0.176 |  | 63 | 2.349 | 0.074 |  | 95 | -1.511 | 0.176 |  | 127 | 0.081 | 0.074 |

The long training symbol for antenna 2 at time 2,4, antenna 3 at time 3,4 and antenna 4 at time 2,3 is described in the time domain in Table 23.

Table 23 - Time domain representation of the long training symbol (sequences P-L)

| ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |  | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 2.478 | 0.000 |  | 32 | -0.378 | 0.000 |  | 64 | -0.131 | 0.000 |  | 96 | 0.000 | 0.000 |
| 1 | 1.932 | -0.074 |  | 33 | -0.048 | -0.176 |  | 65 | 0.918 | -0.074 |  | 97 | 0.116 | -0.176 |
| 2 | -2.031 | 0.275 |  | 34 | -0.820 | 0.079 |  | 66 | 1.154 | 0.275 |  | 98 | 0.106 | 0.079 |
| 3 | -0.337 | 0.012 |  | 35 | -0.498 | -0.262 |  | 67 | 0.265 | 0.012 |  | 99 | -0.225 | -0.262 |
| 4 | 0.993 | 0.163 |  | 36 | 0.976 | -0.163 |  | 68 | -0.825 | 0.163 |  | 100 | -0.288 | -0.163 |
| 5 | -0.252 | 0.301 |  | 37 | 1.986 | -0.051 |  | 69 | -0.692 | 0.301 |  | 101 | 0.118 | -0.051 |
| 6 | 0.981 | -0.003 |  | 38 | 0.653 | -0.350 |  | 70 | -2.033 | -0.003 |  | 102 | 0.310 | -0.350 |
| 7 | -0.794 | 0.281 |  | 39 | -0.884 | -0.031 |  | 71 | -0.522 | 0.281 |  | 103 | 0.035 | -0.031 |
| 8 | -1.659 | 0.125 |  | 40 | 0.867 | -0.125 |  | 72 | -0.235 | 0.125 |  | 104 | -0.125 | -0.125 |
| 9 | 0.077 | -0.042 |  | 41 | -0.051 | -0.208 |  | 73 | -1.098 | -0.042 |  | 105 | -0.010 | -0.208 |
| 10 | -0.100 | 0.211 |  | 42 | -1.362 | 0.142 |  | 74 | 0.911 | 0.211 |  | 106 | -0.065 | 0.142 |
| 11 | -0.160 | -0.142 |  | 43 | -1.098 | -0.108 |  | 75 | -0.849 | -0.142 |  | 107 | -0.190 | -0.108 |
| 12 | -0.490 | -0.068 |  | 44 | -1.250 | 0.068 |  | 76 | -1.178 | -0.068 |  | 108 | 0.057 | 0.068 |
| 13 | -0.182 | 0.013 |  | 45 | 0.284 | 0.237 |  | 77 | -0.114 | 0.013 |  | 109 | 0.343 | 0.237 |
| 14 | 0.822 | -0.324 |  | 46 | 0.267 | -0.030 |  | 78 | -0.472 | -0.324 |  | 110 | 0.102 | -0.030 |
| 15 | 0.720 | -0.044 |  | 47 | 0.513 | 0.294 |  | 79 | 0.433 | -0.044 |  | 111 | -0.274 | 0.294 |
| 16 | -0.662 | -0.177 |  | 48 | 1.183 | 0.177 |  | 80 | -0.963 | -0.177 |  | 112 | -0.177 | 0.177 |
| 17 | -0.513 | -0.294 |  | 49 | 1.255 | 0.044 |  | 81 | -0.463 | -0.294 |  | 113 | 0.078 | 0.044 |
| 18 | 1.005 | 0.030 |  | 50 | 2.091 | 0.324 |  | 82 | 0.336 | 0.030 |  | 114 | 0.033 | 0.324 |
| 19 | -0.333 | -0.237 |  | 51 | -0.369 | -0.013 |  | 83 | -0.510 | -0.237 |  | 115 | 0.004 | -0.013 |
| 20 | 0.101 | -0.068 |  | 52 | -0.586 | 0.068 |  | 84 | 0.012 | -0.068 |  | 116 | 0.193 | 0.068 |
| 21 | 1.158 | 0.108 |  | 53 | 2.333 | 0.142 |  | 85 | -1.047 | 0.108 |  | 117 | 0.121 | 0.142 |
| 22 | 0.361 | -0.142 |  | 54 | 0.337 | -0.211 |  | 86 | -1.112 | -0.142 |  | 118 | -0.262 | -0.211 |
| 23 | 0.754 | 0.208 |  | 55 | -1.051 | 0.042 |  | 87 | -0.492 | 0.208 |  | 119 | -0.284 | 0.042 |
| 24 | -0.066 | 0.125 |  | 56 | 1.251 | -0.125 |  | 88 | 0.864 | 0.125 |  | 120 | 0.125 | -0.125 |
| 25 | 1.232 | 0.031 |  | 57 | 0.790 | -0.281 |  | 89 | 1.027 | 0.031 |  | 121 | 0.259 | -0.281 |
| 26 | 1.729 | 0.350 |  | 58 | -0.007 | 0.003 |  | 90 | -1.582 | 0.350 |  | 122 | 0.016 | 0.003 |
| 27 | -0.846 | 0.051 |  | 59 | 0.998 | -0.301 |  | 91 | 0.460 | 0.051 |  | 123 | -0.049 | -0.301 |
| 28 | -0.859 | 0.163 |  | 60 | -0.203 | -0.163 |  | 92 | 0.601 | 0.163 |  | 124 | 0.038 | -0.163 |
| 29 | -0.990 | 0.262 |  | 61 | -0.835 | -0.012 |  | 93 | -1.961 | 0.262 |  | 125 | -0.122 | -0.012 |
| 30 | -0.277 | -0.079 |  | 62 | -1.371 | -0.275 |  | 94 | 0.855 | -0.079 |  | 126 | -0.241 | -0.275 |
| 31 | 0.301 | 0.176 |  | 63 | -2.187 | 0.074 |  | 95 | 1.742 | 0.176 |  | 127 | 0.081 | 0.074 |

# ANNEX E H1 matrix of LDPC

H1 matrix of PD-LDPC with n=1152, k=864, and rate of ¾ is listed in Table 24.

Table 24 - H1 matrix of PD-LDPC (1152, 864)

| ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 1 144 216 288 | 46 | 4 | 60 118 223 236 | 91 | 4 | 18 23 88 240 |
| 2 | 4 | 1 73 180 252 | 47 | 4 | 62 74 94 173 | 92 | 4 | 19 101 141 276 |
| 3 | 4 | 2 38 108 162 | 48 | 4 | 64 141 206 233 | 93 | 4 | 20 41 161 261 |
| 4 | 4 | 3 198 234 270 | 49 | 4 | 67 110 148 178 | 94 | 4 | 21 53 97 251 |
| 5 | 4 | 4 56 90 126 | 50 | 4 | 68 105 127 277 | 95 | 4 | 24 38 128 280 |
| 6 | 4 | 5 23 172 244 | 51 | 4 | 70 114 151 248 | 96 | 4 | 25 70 157 242 |
| 7 | 4 | 6 152 188 260 | 52 | 4 | 71 170 195 282 | 97 | 4 | 26 200 207 218 |
| 8 | 4 | 7 208 224 280 | 53 | 4 | 75 99 122 218 | 98 | 4 | 27 107 115 146 |
| 9 | 4 | 8 66 100 136 | 54 | 4 | 79 109 204 211 | 99 | 4 | 29 87 236 285 |
| 10 | 4 | 9 46 80 116 | 55 | 4 | 81 93 163 240 | 100 | 4 | 31 71 81 137 |
| 11 | 4 | 10 32 166 238 | 56 | 4 | 83 134 171 273 | 101 | 4 | 32 39 75 156 |
| 12 | 4 | 11 129 249 265 | 57 | 4 | 87 147 230 266 | 102 | 4 | 34 179 185 195 |
| 13 | 4 | 12 175 193 275 | 58 | 4 | 89 135 185 262 | 103 | 4 | 35 138 191 226 |
| 14 | 4 | 13 52 157 229 | 59 | 4 | 91 150 207 253 | 104 | 4 | 37 91 113 181 |
| 15 | 4 | 14 95 184 203 | 60 | 4 | 96 115 197 215 | 105 | 4 | 40 45 83 252 |
| 16 | 4 | 15 27 85 140 | 61 | 4 | 101 145 242 269 | 106 | 4 | 42 60 204 258 |
| 17 | 4 | 16 41 121 283 | 62 | 4 | 103 124 232 286 | 107 | 4 | 43 136 173 248 |
| 18 | 4 | 17 61 113 256 | 63 | 4 | 107 120 155 174 | 108 | 4 | 46 62 189 241 |
| 19 | 4 | 18 104 168 212 | 64 | 4 | 112 161 228 272 | 109 | 4 | 47 149 170 247 |
| 20 | 4 | 19 76 133 149 | 65 | 4 | 117 128 183 191 | 110 | 4 | 48 114 211 273 |
| 21 | 4 | 20 49 196 220 | 66 | 4 | 131 158 187 255 | 111 | 4 | 50 127 177 216 |
| 22 | 4 | 21 35 69 205 | 67 | 4 | 137 210 235 250 | 112 | 4 | 52 67 119 209 |
| 23 | 4 | 22 119 159 267 | 68 | 4 | 139 194 217 259 | 113 | 4 | 55 131 203 250 |
| 24 | 4 | 24 98 231 254 | 69 | 4 | 143 189 202 237 | 114 | 4 | 56 76 175 260 |
| 25 | 4 | 25 164 177 222 | 70 | 4 | 121 165 225 245 | 115 | 4 | 57 64 79 186 |
| 26 | 4 | 26 78 263 271 | 71 | 4 | 176 182 206 257 | 116 | 4 | 58 197 253 277 |
| 27 | 4 | 28 72 102 190 | 72 | 4 | 49 181 264 279 | 117 | 4 | 59 169 201 214 |
| 28 | 4 | 29 55 111 219 | 73 | 4 | 152 199 276 284 | 118 | 4 | 61 165 198 229 |
| 29 | 4 | 30 43 97 278 | 74 | 4 | 90 190 213 222 | 119 | 4 | 63 125 129 154 |
| 30 | 4 | 31 63 146 261 | 75 | 4 | 80 159 221 284 | 120 | 4 | 66 202 271 283 |
| 31 | 4 | 33 92 227 285 | 76 | 4 | 1 6 13 167 | 121 | 4 | 68 89 238 256 |
| 32 | 4 | 34 84 154 214 | 77 | 4 | 2 30 245 257 | 122 | 4 | 72 116 134 255 |
| 33 | 4 | 36 59 241 274 | 78 | 4 | 3 187 193 227 | 123 | 4 | 73 106 118 233 |
| 34 | 4 | 37 45 123 186 | 79 | 4 | 4 36 120 249 | 124 | 4 | 74 176 239 266 |
| 35 | 4 | 39 50 65 247 | 80 | 4 | 5 44 148 263 | 125 | 4 | 77 151 184 232 |
| 36 | 4 | 40 88 192 209 | 81 | 4 | 7 69 94 124 | 126 | 4 | 82 111 244 279 |
| 37 | 4 | 42 130 179 200 | 82 | 4 | 8 112 132 155 | 127 | 4 | 84 160 224 270 |
| 38 | 4 | 44 142 156 169 | 83 | 4 | 9 54 104 194 | 128 | 4 | 86 108 122 213 |
| 39 | 4 | 47 86 243 251 | 84 | 4 | 10 85 110 254 | 129 | 4 | 92 100 139 265 |
| 40 | 4 | 48 106 201 226 | 85 | 4 | 11 96 133 223 | 130 | 4 | 93 98 130 142 |
| 41 | 4 | 51 82 125 258 | 86 | 4 | 12 22 28 65 | 131 | 4 | 95 161 217 243 |
| 42 | 4 | 53 77 239 281 | 87 | 4 | 14 99 105 171 | 132 | 4 | 102 153 162 166 |
| 43 | 4 | 54 153 182 268 | 88 | 4 | 15 210 268 287 | 133 | 4 | 103 143 158 264 |
| 44 | 4 | 57 138 160 167 | 89 | 4 | 16 51 117 150 | 134 | 4 | 109 188 230 282 |
| 45 | 4 | 58 132 246 287 | 90 | 4 | 17 33 78 144 | 135 | 4 | 123 168 235 278 |

Table 24 - H1 matrix of PD-LDPC (1152, 864) (continued)

| ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 136 | 4 | 126 147 208 272 | 186 | 4 | 47 168 192 206 | 236 | 4 | 18 248 274 277 |
| 137 | 4 | 135 231 275 288 | 187 | 4 | 49 152 209 254 | 237 | 4 | 21 63 231 264 |
| 138 | 4 | 140 212 220 281 | 188 | 4 | 50 54 130 138 | 238 | 4 | 22 162 263 269 |
| 139 | 4 | 145 163 219 246 | 189 | 4 | 51 167 175 201 | 239 | 4 | 23 97 153 216 |
| 140 | 4 | 164 172 186 237 | 190 | 4 | 56 142 165 211 | 240 | 4 | 24 68 111 180 |
| 141 | 4 | 174 199 205 262 | 191 | 4 | 57 133 218 256 | 241 | 4 | 25 201 261 279 |
| 142 | 4 | 178 225 259 286 | 192 | 4 | 60 71 99 264 | 242 | 4 | 27 95 144 159 |
| 143 | 4 | 149 180 267 274 | 193 | 4 | 61 126 220 263 | 243 | 4 | 28 37 81 206 |
| 144 | 4 | 8 183 198 251 | 194 | 4 | 63 105 252 271 | 244 | 4 | 29 52 146 149 |
| 145 | 4 | 192 215 221 269 | 195 | 4 | 64 120 280 285 | 245 | 4 | 30 47 74 105 |
| 146 | 4 | 26 173 191 196 | 196 | 4 | 65 111 121 241 | 246 | 4 | 31 69 185 241 |
| 147 | 4 | 81 122 180 228 | 197 | 4 | 66 156 187 223 | 247 | 4 | 33 58 123 249 |
| 148 | 4 | 44 125 234 288 | 198 | 4 | 67 154 237 243 | 248 | 4 | 36 100 148 229 |
| 149 | 4 | 1 131 147 196 | 199 | 4 | 68 139 145 153 | 249 | 4 | 38 82 233 271 |
| 150 | 4 | 2 80 169 232 | 200 | 4 | 70 179 193 245 | 250 | 4 | 40 140 195 242 |
| 151 | 4 | 3 98 212 267 | 201 | 4 | 73 247 261 284 | 251 | 4 | 42 104 272 284 |
| 152 | 4 | 4 53 134 202 | 202 | 4 | 74 91 109 160 | 252 | 4 | 43 50 239 244 |
| 153 | 4 | 5 137 176 197 | 203 | 4 | 75 95 114 128 | 253 | 4 | 44 112 203 285 |
| 154 | 4 | 6 27 59 89 | 204 | 4 | 77 135 157 236 | 254 | 4 | 46 161 165 200 |
| 155 | 4 | 7 42 164 275 | 205 | 4 | 79 124 183 281 | 255 | 4 | 49 183 237 258 |
| 156 | 4 | 9 141 230 244 | 206 | 4 | 83 146 190 276 | 256 | 4 | 53 129 166 181 |
| 157 | 4 | 10 123 199 216 | 207 | 4 | 85 118 163 189 | 257 | 4 | 54 122 157 260 |
| 158 | 4 | 11 69 78 268 | 208 | 4 | 86 103 175 215 | 258 | 4 | 55 103 172 267 |
| 159 | 4 | 12 34 272 278 | 209 | 4 | 87 200 238 249 | 259 | 4 | 57 143 184 209 |
| 160 | 4 | 13 37 136 221 | 210 | 4 | 94 101 170 279 | 260 | 4 | 59 101 198 286 |
| 161 | 4 | 14 88 129 225 | 211 | 4 | 96 126 188 240 | 261 | 4 | 61 79 107 176 |
| 162 | 4 | 15 143 228 248 | 212 | 4 | 106 208 242 265 | 262 | 4 | 62 87 119 193 |
| 163 | 4 | 16 102 185 266 | 213 | 4 | 112 140 152 274 | 263 | 4 | 64 70 89 108 |
| 164 | 4 | 17 97 233 260 | 214 | 4 | 117 178 203 277 | 264 | 4 | 65 139 197 288 |
| 165 | 4 | 18 72 155 207 | 215 | 4 | 48 144 182 195 | 265 | 4 | 66 88 141 235 |
| 166 | 4 | 19 38 62 250 | 216 | 4 | 159 213 219 258 | 266 | 4 | 67 75 102 174 |
| 167 | 4 | 20 110 171 257 | 217 | 4 | 162 229 235 253 | 267 | 4 | 71 86 132 270 |
| 168 | 4 | 21 58 82 222 | 218 | 4 | 174 181 217 287 | 268 | 4 | 73 78 84 282 |
| 169 | 4 | 22 31 113 286 | 219 | 4 | 35 116 220 234 | 269 | 4 | 76 221 228 254 |
| 170 | 4 | 23 84 100 204 | 220 | 4 | 1 120 222 270 | 270 | 4 | 80 94 110 251 |
| 171 | 4 | 24 194 246 273 | 221 | 4 | 2 45 255 265 | 271 | 4 | 85 155 230 256 |
| 172 | 4 | 25 92 116 210 | 222 | 4 | 3 34 225 261 | 272 | 4 | 91 133 169 179 |
| 173 | 4 | 28 148 151 224 | 223 | 4 | 4 15 20 145 | 273 | 4 | 93 135 194 208 |
| 174 | 4 | 29 48 253 262 | 224 | 4 | 5 83 106 130 | 274 | 4 | 96 257 280 287 |
| 175 | 4 | 30 90 158 217 | 225 | 4 | 6 77 92 154 | 275 | 4 | 98 137 151 189 |
| 176 | 4 | 32 131 150 269 | 226 | 4 | 7 26 32 245 | 276 | 4 | 99 117 147 186 |
| 177 | 4 | 33 52 108 172 | 227 | 4 | 8 51 90 210 | 277 | 4 | 109 127 226 243 |
| 178 | 4 | 35 76 163 283 | 228 | 4 | 9 56 60 262 | 278 | 4 | 113 164 214 266 |
| 179 | 4 | 36 115 177 184 | 229 | 4 | 10 115 173 252 | 279 | 4 | 114 219 238 276 |
| 180 | 4 | 39 104 205 227 | 230 | 4 | 11 41 167 227 | 280 | 4 | 118 121 192 217 |
| 181 | 4 | 40 55 107 231 | 231 | 4 | 12 142 218 281 | 281 | 4 | 124 138 207 213 |
| 182 | 4 | 41 93 214 255 | 232 | 4 | 13 177 187 246 | 282 | 4 | 125 191 211 247 |
| 183 | 4 | 43 119 166 282 | 233 | 4 | 14 72 199 223 | 283 | 4 | 128 160 268 283 |
| 184 | 4 | 45 132 226 239 | 234 | 4 | 16 19 232 273 | 284 | 4 | 134 212 215 259 |
| 185 | 4 | 46 127 259 270 | 235 | 4 | 17 39 171 224 | 285 | 4 | 136 178 190 205 |

Table 24 - H1 matrix of PD-LDPC (1152, 864) (continued)

| ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 286 | 4 | 150 182 204 278 | 336 | 4 | 64 137 216 283 | 386 | 4 | 32 54 189 279 |
| 287 | 4 | 156 196 240 275 | 337 | 4 | 65 157 226 249 | 387 | 4 | 33 88 264 281 |
| 288 | 4 | 75 158 170 234 | 338 | 4 | 66 97 211 266 | 388 | 4 | 35 79 83 166 |
| 289 | 4 | 31 116 168 188 | 339 | 4 | 68 134 176 264 | 389 | 4 | 36 98 158 199 |
| 290 | 4 | 58 180 202 220 | 340 | 4 | 70 149 188 258 | 390 | 4 | 37 142 238 283 |
| 291 | 4 | 22 83 169 236 | 341 | 4 | 71 152 173 201 | 391 | 4 | 38 115 121 239 |
| 292 | 4 | 118 212 250 257 | 342 | 4 | 72 238 272 286 | 392 | 4 | 39 173 219 243 |
| 293 | 4 | 1 165 205 209 | 343 | 4 | 74 129 163 285 | 393 | 4 | 40 59 125 160 |
| 294 | 4 | 2 8 12 95 | 344 | 4 | 76 106 203 245 | 394 | 4 | 42 78 122 155 |
| 295 | 4 | 3 40 153 218 | 345 | 4 | 84 87 96 105 | 395 | 4 | 44 70 127 209 |
| 296 | 4 | 4 47 194 204 | 346 | 4 | 89 124 221 244 | 396 | 4 | 45 61 103 152 |
| 297 | 4 | 5 94 228 246 | 347 | 4 | 91 198 231 247 | 397 | 4 | 46 73 108 278 |
| 298 | 4 | 6 46 82 156 | 348 | 4 | 99 112 119 177 | 398 | 4 | 47 137 218 222 |
| 299 | 4 | 7 144 192 277 | 349 | 4 | 102 115 236 256 | 399 | 4 | 48 116 148 231 |
| 300 | 4 | 9 17 186 190 | 350 | 4 | 110 200 215 235 | 400 | 4 | 49 136 150 163 |
| 301 | 4 | 10 13 55 214 | 351 | 4 | 111 160 164 174 | 401 | 4 | 50 60 206 267 |
| 302 | 4 | 11 113 131 262 | 352 | 4 | 114 185 207 260 | 402 | 4 | 51 80 87 212 |
| 303 | 4 | 14 35 42 242 | 353 | 4 | 121 127 135 172 | 403 | 4 | 52 81 153 245 |
| 304 | 4 | 15 166 206 259 | 354 | 4 | 138 199 273 282 | 404 | 4 | 56 195 248 251 |
| 305 | 4 | 16 197 208 237 | 355 | 4 | 141 213 224 241 | 405 | 4 | 58 95 269 282 |
| 306 | 4 | 18 52 142 252 | 356 | 4 | 143 171 239 253 | 406 | 4 | 62 104 114 132 |
| 307 | 4 | 19 33 243 255 | 357 | 4 | 147 184 234 268 | 407 | 4 | 63 117 139 170 |
| 308 | 4 | 20 81 103 288 | 358 | 4 | 151 162 187 191 | 408 | 4 | 65 85 168 183 |
| 309 | 4 | 21 73 128 240 | 359 | 4 | 151 159 205 265 | 409 | 4 | 66 120 138 277 |
| 310 | 4 | 23 51 189 195 | 360 | 4 | 167 250 266 284 | 410 | 4 | 67 198 213 249 |
| 311 | 4 | 24 43 140 225 | 361 | 4 | 53 193 210 219 | 411 | 4 | 68 169 210 260 |
| 312 | 4 | 25 126 150 276 | 362 | 4 | 82 133 208 248 | 412 | 4 | 69 119 214 276 |
| 313 | 4 | 26 48 93 123 | 363 | 4 | 1 75 124 242 | 413 | 4 | 71 97 111 184 |
| 314 | 4 | 27 69 202 278 | 364 | 4 | 2 23 220 247 | 414 | 4 | 72 110 167 194 |
| 315 | 4 | 28 108 117 133 | 365 | 4 | 3 29 221 240 | 415 | 4 | 76 190 235 280 |
| 316 | 4 | 29 61 154 168 | 366 | 4 | 4 12 191 216 | 416 | 4 | 77 113 246 252 |
| 317 | 4 | 30 78 125 254 | 367 | 4 | 5 8 57 214 | 417 | 4 | 89 150 165 271 |
| 318 | 4 | 32 100 161 267 | 368 | 4 | 6 19 178 268 | 418 | 4 | 91 184 259 263 |
| 319 | 4 | 34 233 269 274 | 369 | 4 | 7 181 211 255 | 419 | 4 | 94 147 175 204 |
| 320 | 4 | 36 54 109 196 | 370 | 4 | 9 41 180 286 | 420 | 4 | 96 201 228 234 |
| 321 | 4 | 37 155 170 227 | 371 | 4 | 10 90 226 232 | 421 | 4 | 5 99 109 144 |
| 322 | 4 | 38 130 145 175 | 372 | 4 | 11 25 171 217 | 422 | 4 | 100 107 225 237 |
| 323 | 4 | 39 77 120 182 | 373 | 4 | 13 161 202 280 | 423 | 4 | 101 129 188 253 |
| 324 | 4 | 41 79 139 222 | 374 | 4 | 14 31 182 193 | 424 | 4 | 106 213 256 275 |
| 325 | 4 | 44 90 107 279 | 375 | 4 | 15 34 236 265 | 425 | 4 | 53 112 185 257 |
| 326 | 4 | 45 85 101 179 | 376 | 4 | 16 43 92 146 | 426 | 4 | 118 126 227 244 |
| 327 | 4 | 49 104 146 223 | 377 | 4 | 17 74 135 258 | 427 | 4 | 123 128 131 176 |
| 328 | 4 | 50 251 263 275 | 378 | 4 | 18 55 179 287 | 428 | 4 | 77 130 170 241 |
| 329 | 4 | 53 158 178 230 | 379 | 4 | 20 64 75 84 | 429 | 4 | 134 159 196 233 |
| 330 | 4 | 56 88 132 148 | 380 | 4 | 21 26 102 228 | 430 | 4 | 140 172 181 200 |
| 331 | 4 | 57 98 181 271 | 381 | 4 | 22 105 174 250 | 431 | 4 | 95 141 164 254 |
| 332 | 4 | 59 86 136 281 | 382 | 4 | 24 86 223 229 | 432 | 4 | 143 179 191 285 |
| 333 | 4 | 60 67 232 280 | 383 | 4 | 27 157 270 273 | 433 | 4 | 61 145 203 207 |
| 334 | 4 | 62 92 122 183 | 384 | 4 | 28 215 230 261 | 434 | 4 | 149 162 281 288 |
| 335 | 4 | 63 80 229 287 | 385 | 4 | 30 93 177 284 | 435 | 4 | 106 154 187 259 |

Table 24 - H1 matrix of PD-LDPC (1152, 864) (continued)

| ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 436 | 4 | 156 192 262 272 | 486 | 4 | 4 78 235 286 | 536 | 4 | 36 208 261 268 |
| 437 | 4 | 21 186 217 274 | 487 | 4 | 80 113 208 216 | 537 | 4 | 37 84 185 191 |
| 438 | 4 | 108 158 167 197 | 488 | 4 | 82 185 198 255 | 538 | 4 | 38 59 152 237 |
| 439 | 4 | 166 196 224 246 | 489 | 4 | 84 121 149 210 | 539 | 4 | 39 52 223 259 |
| 440 | 4 | 1 112 116 237 | 490 | 4 | 88 161 169 258 | 540 | 4 | 40 150 228 239 |
| 441 | 4 | 2 262 274 283 | 491 | 4 | 90 168 172 265 | 541 | 4 | 43 96 113 137 |
| 442 | 4 | 3 64 72 263 | 492 | 4 | 51 93 109 223 | 542 | 4 | 44 133 153 273 |
| 443 | 4 | 4 63 224 266 | 493 | 4 | 15 97 226 276 | 543 | 4 | 31 45 51 280 |
| 444 | 4 | 6 163 171 231 | 494 | 4 | 105 157 190 220 | 544 | 4 | 22 48 141 287 |
| 445 | 4 | 7 34 219 229 | 495 | 4 | 107 193 253 284 | 545 | 4 | 49 160 203 249 |
| 446 | 4 | 8 39 188 264 | 496 | 4 | 78 111 117 142 | 546 | 4 | 50 106 174 202 |
| 447 | 4 | 9 15 177 271 | 497 | 4 | 57 114 118 288 | 547 | 4 | 53 85 162 267 |
| 448 | 4 | 10 70 160 212 | 498 | 4 | 119 123 145 156 | 548 | 4 | 54 71 125 161 |
| 449 | 4 | 11 59 81 127 | 499 | 4 | 9 120 162 201 | 549 | 4 | 55 156 183 254 |
| 450 | 4 | 12 91 152 244 | 500 | 4 | 122 187 195 238 | 550 | 4 | 58 146 211 235 |
| 451 | 4 | 13 101 104 121 | 501 | 4 | 17 132 199 279 | 551 | 4 | 60 234 265 277 |
| 452 | 4 | 14 46 67 288 | 502 | 4 | 133 164 182 220 | 552 | 4 | 63 69 172 184 |
| 453 | 4 | 16 54 221 252 | 503 | 4 | 61 136 261 273 | 553 | 4 | 64 169 176 246 |
| 454 | 4 | 17 100 130 209 | 504 | 4 | 24 143 155 269 | 554 | 4 | 67 79 104 155 |
| 455 | 4 | 18 41 50 147 | 505 | 4 | 144 174 240 243 | 555 | 4 | 34 68 132 167 |
| 456 | 4 | 19 31 79 200 | 506 | 4 | 151 180 243 257 | 556 | 4 | 72 170 245 281 |
| 457 | 4 | 20 66 115 179 | 507 | 4 | 159 175 211 225 | 557 | 4 | 73 129 139 143 |
| 458 | 4 | 22 36 94 154 | 508 | 4 | 22 46 176 227 | 558 | 4 | 74 117 122 200 |
| 459 | 4 | 23 158 183 272 | 509 | 4 | 81 178 236 247 | 559 | 4 | 76 100 111 131 |
| 460 | 4 | 24 56 124 197 | 510 | 4 | 186 232 253 287 | 560 | 4 | 80 102 130 154 |
| 461 | 4 | 25 87 153 205 | 511 | 4 | 192 249 268 279 | 561 | 4 | 82 140 193 201 |
| 462 | 4 | 26 198 204 239 | 512 | 4 | 34 188 203 222 | 562 | 4 | 83 243 271 278 |
| 463 | 4 | 27 129 137 275 | 513 | 4 | 5 30 207 270 | 563 | 4 | 86 157 182 265 |
| 464 | 4 | 28 68 125 251 | 514 | 4 | 46 147 219 254 | 564 | 4 | 87 101 128 263 |
| 465 | 4 | 29 99 138 233 | 515 | 4 | 62 102 244 277 | 565 | 4 | 85 88 127 222 |
| 466 | 4 | 30 37 134 242 | 516 | 4 | 41 215 246 278 | 566 | 4 | 89 99 229 247 |
| 467 | 4 | 18 32 57 86 | 517 | 4 | 1 93 260 264 | 567 | 4 | 81 90 205 269 |
| 468 | 4 | 33 96 165 194 | 518 | 4 | 2 42 75 112 | 568 | 4 | 97 178 213 262 |
| 469 | 4 | 35 131 141 248 | 519 | 4 | 3 32 94 171 | 569 | 4 | 98 197 207 236 |
| 470 | 4 | 38 44 195 267 | 520 | 4 | 6 14 118 206 | 570 | 4 | 107 124 251 255 |
| 471 | 4 | 40 110 202 260 | 521 | 4 | 7 20 199 227 | 571 | 4 | 109 166 214 272 |
| 472 | 4 | 42 89 128 189 | 522 | 4 | 8 105 123 241 | 572 | 4 | 12 115 242 276 |
| 473 | 4 | 43 103 218 241 | 523 | 4 | 10 28 204 274 | 573 | 4 | 119 134 231 285 |
| 474 | 4 | 45 135 250 282 | 524 | 4 | 12 110 186 270 | 574 | 4 | 66 126 149 218 |
| 475 | 4 | 11 47 102 140 | 525 | 4 | 16 95 187 224 | 575 | 4 | 135 153 168 286 |
| 476 | 4 | 13 48 126 139 | 526 | 4 | 19 56 65 209 | 576 | 4 | 136 180 194 240 |
| 477 | 4 | 49 69 230 252 | 527 | 4 | 21 47 175 238 | 577 | 4 | 101 138 142 212 |
| 478 | 4 | 51 73 148 164 | 528 | 4 | 23 70 148 163 | 578 | 4 | 145 151 190 282 |
| 479 | 4 | 52 85 234 284 | 529 | 4 | 25 103 120 283 | 579 | 4 | 127 159 230 275 |
| 480 | 4 | 55 92 256 270 | 530 | 4 | 26 116 167 232 | 580 | 4 | 79 157 173 250 |
| 481 | 4 | 58 76 173 206 | 531 | 4 | 27 91 100 257 | 581 | 4 | 189 215 226 233 |
| 482 | 4 | 60 92 215 245 | 532 | 4 | 29 165 181 248 | 582 | 4 | 74 210 221 256 |
| 483 | 4 | 62 83 216 225 | 533 | 4 | 31 66 77 108 | 583 | 4 | 57 208 225 258 |
| 484 | 4 | 65 74 98 146 | 534 | 4 | 33 114 192 196 | 584 | 4 | 1 133 198 263 |
| 485 | 4 | 71 217 220 285 | 535 | 4 | 35 144 177 266 | 585 | 4 | 2 216 259 279 |

Table 24 - H1 matrix of PD-LDPC (1152, 864) (continued)

| ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 586 | 4 | 3 77 231 276 | 636 | 4 | 95 168 239 241 | 686 | 4 | 44 47 71 76 |
| 587 | 4 | 4 70 175 287 | 637 | 4 | 97 104 128 275 | 687 | 4 | 46 134 138 173 |
| 588 | 4 | 5 40 78 104 | 638 | 4 | 109 140 170 238 | 688 | 4 | 48 50 108 199 |
| 589 | 4 | 6 132 191 201 | 639 | 4 | 114 117 138 154 | 689 | 4 | 49 62 218 247 |
| 590 | 4 | 7 65 123 171 | 640 | 4 | 115 131 226 258 | 690 | 4 | 51 171 184 271 |
| 591 | 4 | 8 29 134 139 | 641 | 4 | 126 169 217 264 | 691 | 4 | 52 95 198 276 |
| 592 | 4 | 9 39 112 129 | 642 | 4 | 130 150 183 196 | 692 | 4 | 53 123 141 286 |
| 593 | 4 | 10 64 99 185 | 643 | 4 | 135 161 187 269 | 693 | 4 | 56 130 242 257 |
| 594 | 4 | 11 23 125 260 | 644 | 4 | 141 146 204 266 | 694 | 4 | 59 187 196 266 |
| 595 | 4 | 13 110 142 277 | 645 | 4 | 2 143 146 186 | 695 | 4 | 68 77 188 202 |
| 596 | 4 | 14 26 98 230 | 646 | 4 | 145 172 176 228 | 696 | 4 | 72 127 180 211 |
| 597 | 4 | 15 119 189 245 | 647 | 4 | 151 174 214 280 | 697 | 4 | 74 126 182 197 |
| 598 | 4 | 16 30 107 205 | 648 | 4 | 180 184 207 223 | 698 | 4 | 78 151 181 193 |
| 599 | 4 | 17 28 166 242 | 649 | 4 | 194 212 233 268 | 699 | 4 | 79 106 152 189 |
| 600 | 4 | 18 124 162 165 | 650 | 4 | 195 254 273 286 | 700 | 4 | 81 132 203 253 |
| 601 | 4 | 19 120 125 203 | 651 | 4 | 236 244 251 283 | 701 | 4 | 82 113 168 194 |
| 602 | 4 | 20 24 58 106 | 652 | 4 | 1 43 55 117 | 702 | 4 | 83 120 156 213 |
| 603 | 4 | 21 45 182 234 | 653 | 4 | 3 58 167 186 | 703 | 4 | 85 119 170 176 |
| 604 | 4 | 25 41 252 282 | 654 | 4 | 4 45 87 115 | 704 | 4 | 86 88 91 190 |
| 605 | 4 | 27 84 103 208 | 655 | 4 | 5 84 139 261 | 705 | 4 | 89 96 110 145 |
| 606 | 4 | 32 55 73 213 | 656 | 4 | 6 66 137 220 | 706 | 4 | 92 105 229 273 |
| 607 | 4 | 33 43 240 283 | 657 | 4 | 7 75 169 288 | 707 | 4 | 93 192 222 245 |
| 608 | 4 | 35 47 253 256 | 658 | 4 | 8 35 148 212 | 708 | 4 | 97 200 204 237 |
| 609 | 4 | 36 163 190 193 | 659 | 4 | 9 102 133 243 | 709 | 4 | 101 159 238 280 |
| 610 | 4 | 37 197 224 250 | 660 | 4 | 10 100 116 143 | 710 | 4 | 107 165 191 241 |
| 611 | 4 | 38 69 72 96 | 661 | 4 | 11 65 175 221 | 711 | 4 | 109 250 265 285 |
| 612 | 4 | 42 48 76 281 | 662 | 4 | 13 60 233 248 | 712 | 4 | 112 157 207 224 |
| 613 | 4 | 44 147 177 200 | 663 | 4 | 14 23 90 166 | 713 | 4 | 118 172 260 270 |
| 614 | 4 | 49 156 178 232 | 664 | 4 | 15 63 240 278 | 714 | 4 | 109 136 219 287 |
| 615 | 4 | 50 93 185 235 | 665 | 4 | 16 135 164 281 | 715 | 4 | 140 232 259 282 |
| 616 | 4 | 52 131 148 206 | 666 | 4 | 17 99 228 236 | 716 | 4 | 154 230 263 283 |
| 617 | 4 | 53 61 160 209 | 667 | 4 | 18 122 159 249 | 717 | 4 | 123 161 163 256 |
| 618 | 4 | 12 54 248 262 | 668 | 4 | 19 27 73 147 | 718 | 4 | 174 183 269 279 |
| 619 | 4 | 56 173 274 284 | 669 | 4 | 20 94 129 160 | 719 | 4 | 78 206 237 251 |
| 620 | 4 | 59 105 155 219 | 670 | 4 | 21 38 114 155 | 720 | 4 | 99 210 225 246 |
| 621 | 4 | 60 108 152 285 | 671 | 4 | 22 25 57 121 | 721 | 4 | 176 223 252 274 |
| 622 | 4 | 62 111 181 222 | 672 | 4 | 24 70 144 234 | 722 | 4 | 184 227 264 284 |
| 623 | 4 | 63 192 247 257 | 673 | 4 | 26 103 209 272 | 723 | 4 | 170 264 275 277 |
| 624 | 4 | 67 137 179 267 | 674 | 4 | 28 54 124 177 | 724 | 4 | 1 44 132 212 |
| 625 | 4 | 68 118 158 261 | 675 | 4 | 29 69 178 216 | 725 | 4 | 2 138 153 187 |
| 626 | 4 | 71 149 159 164 | 676 | 4 | 30 67 80 201 | 726 | 4 | 3 109 192 251 |
| 627 | 4 | 75 211 254 271 | 677 | 4 | 31 142 162 214 | 727 | 4 | 4 43 54 213 |
| 628 | 4 | 80 90 218 255 | 678 | 4 | 32 61 64 205 | 728 | 4 | 5 226 279 281 |
| 629 | 4 | 82 92 144 249 | 679 | 4 | 33 98 153 262 | 729 | 4 | 6 58 174 274 |
| 630 | 4 | 83 221 229 237 | 680 | 4 | 34 111 128 149 | 730 | 4 | 7 193 242 259 |
| 631 | 4 | 86 210 227 272 | 681 | 4 | 36 41 226 244 | 731 | 4 | 8 167 228 238 |
| 632 | 4 | 87 94 202 288 | 682 | 4 | 37 195 231 258 | 732 | 4 | 9 181 216 250 |
| 633 | 4 | 88 113 122 199 | 683 | 4 | 39 150 179 255 | 733 | 4 | 10 169 219 240 |
| 634 | 4 | 89 116 136 267 | 684 | 4 | 40 158 215 268 | 734 | 4 | 11 73 107 235 |
| 635 | 4 | 91 121 188 278 | 685 | 4 | 42 217 235 239 | 735 | 4 | 12 76 154 180 |

Table 24 - H1 matrix of PD-LDPC (1152, 864) (continued)

| ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column | ## | dc | Position of 1’s in each column |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 736 | 4 | 13 21 32 80 | 786 | 4 | 125 182 186 225 | 836 | 4 | 57 120 179 266 |
| 737 | 4 | 14 71 160 272 | 787 | 4 | 126 178 215 241 | 837 | 4 | 59 220 230 273 |
| 738 | 4 | 15 113 121 163 | 788 | 4 | 77 128 218 243 | 838 | 4 | 60 225 240 287 |
| 739 | 4 | 16 27 90 247 | 789 | 4 | 129 149 215 266 | 839 | 4 | 61 72 206 288 |
| 740 | 4 | 17 46 89 245 | 790 | 4 | 131 189 252 273 | 840 | 4 | 62 103 137 162 |
| 741 | 4 | 18 53 237 265 | 791 | 4 | 23 134 276 278 | 841 | 4 | 63 173 202 213 |
| 742 | 4 | 8 19 25 69 | 792 | 4 | 137 190 242 263 | 842 | 4 | 64 148 150 181 |
| 743 | 4 | 20 151 168 275 | 793 | 4 | 155 172 221 233 | 843 | 4 | 65 84 174 267 |
| 744 | 4 | 22 118 188 255 | 794 | 4 | 173 254 268 288 | 844 | 4 | 66 166 171 209 |
| 745 | 4 | 23 33 77 257 | 795 | 4 | 194 253 269 276 | 845 | 4 | 68 96 263 272 |
| 746 | 4 | 24 146 185 270 | 796 | 4 | 200 206 220 252 | 846 | 4 | 70 83 178 280 |
| 747 | 4 | 26 113 152 155 | 797 | 4 | 42 203 256 282 | 847 | 4 | 71 257 278 284 |
| 748 | 4 | 28 86 130 156 | 798 | 4 | 2 18 29 235 | 848 | 4 | 73 101 172 201 |
| 749 | 4 | 29 45 50 59 | 799 | 4 | 3 35 107 112 | 849 | 4 | 74 95 157 199 |
| 750 | 4 | 30 88 102 111 | 800 | 4 | 4 125 262 264 | 850 | 4 | 75 129 194 244 |
| 751 | 4 | 31 144 196 217 | 801 | 4 | 5 111 141 165 | 851 | 4 | 76 134 191 227 |
| 752 | 4 | 34 74 143 286 | 802 | 4 | 6 87 248 253 | 852 | 4 | 78 90 212 274 |
| 753 | 4 | 35 87 108 261 | 803 | 4 | 7 153 177 283 | 853 | 4 | 79 169 231 239 |
| 754 | 4 | 36 64 136 224 | 804 | 4 | 9 43 170 251 | 854 | 4 | 81 85 126 154 |
| 755 | 4 | 37 115 147 209 | 805 | 4 | 10 131 192 256 | 855 | 4 | 86 128 145 204 |
| 756 | 4 | 38 88 223 231 | 806 | 4 | 11 100 195 277 | 856 | 4 | 93 182 210 258 |
| 757 | 4 | 39 84 162 211 | 807 | 4 | 12 21 121 200 | 857 | 4 | 99 116 158 245 |
| 758 | 4 | 40 56 66 177 | 808 | 4 | 13 82 270 282 | 858 | 4 | 106 127 133 224 |
| 759 | 4 | 41 110 114 205 | 809 | 4 | 14 19 58 261 | 859 | 4 | 108 222 229 249 |
| 760 | 4 | 42 47 197 246 | 810 | 4 | 15 32 175 218 | 860 | 4 | 114 139 223 250 |
| 761 | 4 | 48 104 166 229 | 811 | 4 | 16 123 159 226 | 861 | 4 | 118 132 176 275 |
| 762 | 4 | 49 81 175 271 | 812 | 4 | 17 40 163 243 | 862 | 4 | 146 228 271 285 |
| 763 | 4 | 51 157 244 262 | 813 | 4 | 20 117 143 211 | 863 | 4 | 167 207 217 219 |
| 764 | 4 | 52 139 191 287 | 814 | 4 | 22 91 203 254 | 864 | 4 | 185 190 205 238 |
| 765 | 4 | 55 135 142 198 | 815 | 4 | 24 44 67 92 |  |  |  |
| 766 | 4 | 57 145 164 210 | 816 | 4 | 25 80 193 233 |  |  |  |
| 767 | 4 | 60 95 222 227 | 817 | 4 | 26 94 216 265 |  |  |  |
| 768 | 4 | 61 91 93 236 | 818 | 4 | 27 54 142 241 |  |  |  |
| 769 | 4 | 62 165 171 186 | 819 | 4 | 28 140 164 197 |  |  |  |
| 770 | 4 | 63 75 199 280 | 820 | 4 | 30 130 189 232 |  |  |  |
| 771 | 4 | 65 101 214 268 | 821 | 4 | 31 50 98 221 |  |  |  |
| 772 | 4 | 67 103 195 246 | 822 | 4 | 33 89 149 198 |  |  |  |
| 773 | 4 | 68 133 141 161 | 823 | 4 | 34 135 184 247 |  |  |  |
| 774 | 4 | 70 85 116 239 | 824 | 4 | 36 105 147 180 |  |  |  |
| 775 | 4 | 72 112 179 258 | 825 | 4 | 37 119 255 281 |  |  |  |
| 776 | 4 | 79 249 277 285 | 826 | 4 | 38 53 151 260 |  |  |  |
| 777 | 4 | 82 96 183 284 | 827 | 4 | 39 208 269 286 |  |  |  |
| 778 | 4 | 83 98 117 260 | 828 | 4 | 41 97 152 156 |  |  |  |
| 779 | 4 | 92 94 97 207 | 829 | 4 | 45 144 214 259 |  |  |  |
| 780 | 4 | 100 124 127 201 | 830 | 4 | 46 69 183 236 |  |  |  |
| 781 | 4 | 56 105 150 267 | 831 | 4 | 47 49 110 188 |  |  |  |
| 782 | 4 | 106 136 230 234 | 832 | 4 | 48 122 138 234 |  |  |  |
| 783 | 4 | 119 148 204 208 | 833 | 4 | 51 160 187 279 |  |  |  |
| 784 | 4 | 120 140 168 248 | 834 | 4 | 52 102 124 161 |  |  |  |
| 785 | 4 | 122 158 202 232 | 835 | 4 | 55 104 115 196 |  |  |  |

The fixed-beam reception mode in the uplink can be used either for reception of two multiplexed data streams from station reciprocal to the downlink, or for beamforming reception of one data streams from station.

The matrix can be viewed as an approximation of the downlink channel covariance matrix.