IEEE P802.11
Wireless LANs

TGn Sync Proposal FRCC Compliance

Date:	 March 4th, 2005

Author:	Syed Aon Mujtaba
	Agere Systems

555 Union Boulevard

Allentown, Pennsylvania 18109, U.S.A.
	Phone: +1 610 712 6616

Cell: +1 908 342 5251
	e-Mail: mujtaba@agere.com

Additional Authors:

Adrian P Stephens, Intel Corporation, adrian.p.stephens@intel.com,

Alek Purkovic, Nortel Networks, apurkovi@nortelnetworks.com,

Brian Johnson, Nortel Networks, brjohnso@nortelnetworks.com,

Daisuke Takeda, Toshiba, daisuke.takeda@toshiba.co.jp,

Dongjun Lee, Samsung, djthekid.lee@samsung.com,

Eldad Perahia, Cisco Systems, eperahia@cisco.com,

Huanchun Ye, Atheros, hcye@atheros.com,

Hui-Ling Lou, Marvell Semiconductor Inc., hlou@marvell.com,

James Chen, Marvell Semiconductor Inc., jamesc@marvell.com,

Jari Jokela, Nokia, jari.jokela@nokia.com,

Jeff Gilbert, Atheros, gilbertj@atheros.com,

Joe Pitarresi, Intel, joe.pitarresi@intel.com,

Joerg Habetha, Philips, joerg.habetha@philips.com,

John Sadowsky, Intel, john.sadowsky@intel.com,

Jon Rosdahl, Samsung, jon.rosdahl@partner.samsung.com,

Luke Qian, Cisco Systems, lchia@cisco.com,

Mary Cramer, Agere Systems, mecramer@agere.com,

Masahiro Takagi, Toshiba, masahiro3.takagi@toshiba.co.jp,

Nico van Waes, Nokia, nico.vanwaes@nokia.com,

Osama Aboul-Magd, Nortel Networks, osama@nortelnetworks.com,

Paul Feinberg, Sony, Paul.Feinberg@am.sony.com,

Pen Li, Philips, pen.li@philips.com,

Peter Loc, Marvell Semiconductor Inc., ploc@marvell.com,

Richard van  Leeuwen, Agere Systems, rleeuwen@agere.com,

Seigo Nakao, Sanyo, snakao@gf.hm.rd.sanyo.co.jp,

Sheung Li, Atheros, sheung@atheros.com,

Takushi Kunihiro, Sony, kuni@wcs.sony.co.jp,

Teik-Kheong (TK) Tan, Philips, tktan@philips.com,

Tomoko Adachi, Toshiba, tomo.adachi@toshiba.co.jp,

Tomoya Yamaura, Sony, yamaura@wcs.sony.co.jp,

Won-Joon Choi, Atheros Communications Inc., wjchoi@atheros.com,

Yasuhiro Tanaka, Sanyo, y_tanaka@gf.hm.rd.sanyo.co.jp,

Yoshiharu Doi, Sanyo, doi@gf.hm.rd.sanyo.co.jp,

Youngsoo Kim, Samsung, KimYoungsoo@samsung.com,

Yuichi Morioka, Sony, morioka@wcs.sony.co.jp

Abstract

This document demonstrates that the referenced proposal document submitted in response to the IEEE 802.11 TGn call for proposals is a complete proposal by showing compliance with TGn functional requirements and comparison criteria.

# Introduction

## Purpose of document

“IEEE 802 11-04/889, TGnSync Proposal Technical Specification” is submitted for consideration under the 802.11 TGn selection process [4] and is hereby declared complete based on its compliance with the functional requirements defined in [5] and supported by the results disclosed herein in accordance with the TGn comparison criteria defined in [6].

## References

[1] IEEE 802 11-04/891, TGnSync Proposal PHY Results

[2] IEEE 802 11-04/892, TGnSync Proposal MAC Results

[3] IEEE 802 11-04/889, TGnSync Proposal Technical Specification

[4] IEEE 802 11-03/665, TGn Selection Procedure

[5] IEEE 802 11-03/813, TGn Functional Requirements

[6] IEEE 802.11-03/814, TGn Comparison Criteria

# Additional Disclosures (AD)

## Coverage of Additional Disclosures

| Number | Name | Definition | Disclosure |  |
| --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of related IEEE submissions, both documents and presentations. |  | Section 2.2.1 |
| AD2 | TCP Model Parameters | Include a reference to the TCP protocol type (e.g. Reno) and the parameter values associated with that protocol used for all MAC simulations. |  | [2, Section 3] |
| AD3 | MAC simulation methodology | Include a description of the simulation methodology used for MAC simulations, including a description of how the PHY and its impairments are modeled. |  | [2, Section 3] |
| AD4 | MAC simulation occupied channel width | For each MAC simulation, report the total channel width occupied. |  | [2, Section 3] |
| AD5 | Justification of low PLR rates achieved | For each application with a PLR < 10^-4, proposal shall justify that the proposed PLR could be met. |  | [2, Section 3] |

## Additional Disclosures Responses

### AD1 Reference Submission

Below is a complete list of IEEE submissions, both documents and presentations, for the TGn Sync proposal.

IEEE 802 11-04/889, TGnSync Proposal Technical Specification

- Complete technical specification

IEEE 802 11-04/888, TGnSync Proposal Presentation

- Complete detailed presentation

IEEE 802 11-04/887, TGnSync Proposal Summary

- High level summary presentation

IEEE 802 11-04/890, TGnSync Proposal FRCC Compliance

- FRCC compliance

IEEE 802 11-04/892, TGnSync Proposal MAC Results

- Summary of MAC simulation results

IEEE 802 11-04/893, TGnSync Proposal MAC1 Simulation Results

- Detailed results from MAC1 simulator

IEEE 802 11-04/894, TGnSync Proposal MAC2 Simulation Results

- Detailed results from MAC2 simulator

IEEE 802 11-04/895, TGnSync Proposal MAC Simulation Methodology

- Description of MAC simulation methodology

IEEE 802 11-04/891, TGnSync Proposal PHY Results

- Summary of PHY simulation results

# Functional Requirements

## Coverage of Functional Requirements

| Number | Name | Coverage (Yes/No) | Reference |
| --- | --- | --- | --- |
| R1 | Single Link HT rate supported | Yes | [2, Section 2.8] |
| R2 | HT rate supported in 20MHz channel | Yes | [2, Section 2.9] |
| R3 | Supports 5GHz bands | Yes | [3, Section 11.1.1.1] |
| R4 | .11a backwards compatibility | Yes | Section 4.2.2 CC11 |
| R5 | .11g backwards compatibility | Yes | Section 3.2.1 |
| R6 | Control of support for legacy STA from .11n AP | Yes | Section 3.2.2 |
| R7 | .11e QoS support | Yes | Section 3.2.3 |
| R8 | Spectral Efficiency | Yes | [2, Section 2.2] |
| R9 | Compliance to PAR | Yes |  |

## Functional Requirement Responses

### R5 .11g backwards compatibility

An .11n STA is capable of operating as a .11g legacy device if it supports operation in that band as described in [3, Section 8.1.3.2].  It can send and receive legacy format PPDUs as described in [3, Section 10.1.1].

When operating in a high throughput mode, backwards compatibility is provided at the PHY level [3, Section 11.3.1.2] essentially by retaining the legacy short and long training fields and the legacy signal field.

Coexistence is managed within the MAC through the definition of an operating mode [3, section 7.1.1] and rules [3, Sections 7.1.3 and 7.1.4] for operation in each operating mode.

Protection mechanisms are defined using both PHY-level ("spoofing") [3, section 7.1.7.2] and MAC-level techniques [3, Section 7.1.7.1].

When operating in the presence of 802.11b clients, MAC-level techniques are used using 802.11b PPDU formats as described in [3, Section 7.1.7.1.3].

### FR6 Control of support for legacy STA from .11n AP

A HT AP can choose to make itself discoverable or not according to whether it transmits a beacon in pure mode or mixed capable mode as defined in [3, section 8.1.1].

Furthermore, the AP may choose to accept or deny an association from a legacy STA as defined in [3, 8.1.3.1].

### FR7 .11e QoS support

This is mandatory requirement of a HT STA.  This requirement is defined in [3, section 7.4.1].

# Comparison Criteria

## Comparison Criteria Coverage

| Number | Name | Mandatory / optional | Coverage (Yes/No) | Reference |
| --- | --- | --- | --- | --- |
| General |  |  |  |  |
| CC2 | Regulatory compliance | Mandatory | Yes | Section 4.2.1 |
| Marketability |  |  |  |  |
| CC3 | List of goodput results for usage models 1,4 and 6. | Mandatory | Yes | [2 , Section 2.2] |
| CC6 | PHY complexity | Optional | No |  |
| CC7 | MAC processing complexity | Optional | No |  |
| Backward Compatibility and Coexistence with Legacy Devices |  |  |  |  |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Mandatory | Yes | Section 4.2.2 |
| CC15 | Sharing of medium with legacy devices | Mandatory | Yes | [2, Section 2.2] |
| MAC Related |  |  |  |  |
| Performance Measurements at the MAC SAP |  |  |  |  |
| CC18 | HT Usage Models Supported (non QoS) | Mandatory | Yes | [2, Section 2.2] |
| CC19 | HT Usage Models Supported (QoS) | Mandatory | Yes | [2, Section 2.2] |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | Mandatory | Yes | [2, Section 2.2] |
| CC24 | MAC Efficiency | Mandatory | Yes | [2, Section 2.2] |
| CC27 | Throughput / Range | Mandatory | Yes | [2, Section 2.4] |
| CC28 | Throughput / Range in 20MHz | Mandatory | Yes | [2, Section 2.5] |
| MAC Changes |  |  |  |  |
| CC46 | MAC Compatibility and parameters. | Mandatory | Yes | [2, Section 2.6] |
| CC47 | MAC extensions | Mandatory | Yes | [2, Section 2.7] |
| PHY Related |  |  |  |  |
| PHY Rates and Preambles |  |  |  |  |
| CC51 | Data rates | Mandatory | Yes | Section 4.2.3 |
| CC42 | Preambles | Mandatory | Yes | Section 4.2.4 |
| Channelization |  |  |  |  |
| CC51.5 | Channelization | Mandatory | Yes | Section 4.2.5 |
| CC52 | Spectral Mask | Mandatory | Yes | Section 4.2.6 |
| Spectral Efficiency |  |  |  |  |
| CC58 | HT Spectral Efficiency | Mandatory | Yes | [2, Section 2.2] |
| PHY Performance |  |  |  |  |
| CC59 | AWGN PER performance | Mandatory | Yes | [1, Section 2] |
| CC67 | PER performance in non AWGN channels | Mandatory | Yes (set 1) | [1, Section 3] |
| CC67.2 | Offset Compensation | Mandatory | Yes | [1, Section 3] |
| PHY Changes |  |  |  |  |
| CC80 | Required changes to 802.11 PHY | Mandatory | Yes | Section 4.2.7 |

## Comparison Criteria Responses

### CC2 Regulatory Compliance

There are no known problems with regulatory compliance in USA, Europe, and China.

#### Japan Domain

This section refers the following Japanese Regulations.

RE:  Radio Equipment Regulations

RLE: Radio Law Enforcement Regulations

Known problems with Japan domain are as follows:

- Transmit burst length

According to RE: Article 49-20, 49-21, transmit burst length is defined as follows;

  - 2400-2483.5MHz

There is no definition of transmit burst length.

  - 5170, 5190, 5210, 5230MHz

Transmit burst length should be less than 4 milliseconds.

  - 4900-5000MHz, 5030-5090MHz

Transmit burst length should be less than 4 milliseconds.

An .11n device which obtains a type approval for Japan region should keep a 4ms burst length limitation.  Sections regarding the duration of aggregated PSDU and MAC frame sequences (Long NAV, Pairwise Spoofing, Single Ended Spoofing etc.) in [3] are relevant to the burst length.

- Occupied Bandwidth

According to RE: Article 6, Annex Table-2, occupied bandwidth is defined as follows;

  - 2400-2483.5MHz

    - Occupied bandwidth of transmitting equipment using FH, combination of DS-SS and FH, or combination of OFDM and FH shall not exceed 83.5MHz.

    - Occupied bandwidth of transmitting equipment except (1) shall not exceed 26MHz.

  - 5170, 5190, 5210, 5230MHz

Occupied bandwidth of transmitting equipment which utilizes 5170, 5190, 5210, and 5230MHz frequency band shall not exceed 18MHz.

  - 4900-5000MHz, 5030-5090MHz

Occupied bandwidth of a 5GHz band Wireless Access System shall be as follows;

for 20MHz system: 19.7MHz

for 10MHz system: 9MHz

for 5MHz system: 4.5MHz

An .11n device that obtains a type approval for the Japan region must keep its occupied bandwidth to 18MHz for the 5GHz band, and 26MHz for the 2.4GHz band.

- MIMO (Multiple-Input Multiple-Output)/Multiple Transmit

At this time, regarding the use of MIMO, there is no explicit definition in RE and RLE.

### CC11 Backward compatibility

Backwards compatibility with legacy .11a and .11g OFDM devices is provided at the PHY level.  The preamble begins with a legacy OFDM short training (STF), long training (LTF), and signal field (L-SIG).  Legacy OFDM stations will be able to perform start-of-packet detection, AGC, frequency estimation, timing offset estimation, channel estimation without any degradation.  Furthermore, legacy OFDM stations will be able to decode the legacy signal field and extract the RATE and LENGTH fields.  The RATE and LENGTH fields in the legacy signal field are “spoofed” such that a legacy OFDM RX can defer for the correct period of time.  Further details are described in [3, Sections 11.3.1.2 and 11.3.1.3].

Backward compatibility is managed within the MAC through the definition of an operating mode [3, Section 8.1.1] and rules [3, Sections 8.1.3 and 8.1.4] for operation in each operating mode.  Protection mechanisms are defined using both PHY-level ("spoofing") [3, Section 7.1.7.2] and MAC-level techniques [3, Section 7.1.7.1].

When operating in the presence of 802.11b clients, MAC-level techniques are used using 802.11b PPDU formats as described in [3, Section 11.3.1.2].

### CC51 Data Rates

#### Basic Modes

The following modes are the basic mode set.

|  |  |  |  | Full GI |  |  |  | Half GI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 20MHz |  | 40MHz |  | 20MHz |  | 40MHz |  |
| MCS Index | Number of spatial streams | Modulation | Coding rate | Rate (Mbps) | Mandatory/ Optional | Rate (Mbps) | Mandatory/ Optional | Rate (Mbps) | Mandatory/ Optional | Rate (Mbps) | Mandatory/ Optional |
| 0 | 1 | BPSK | 1/2 | 6.50 | Mandatory | 13.5 | Optional | 7.22 | Optional | 15 | Optional |
| 1 | 1 | QPSK | 1/2 | 13.00 | Mandatory | 27.0 | Optional | 14.44 | Optional | 30 | Optional |
| 2 | 1 | QPSK | 3/4 | 19.50 | Mandatory | 40.5 | Optional | 21.67 | Optional | 45 | Optional |
| 3 | 1 | 16-QAM | 1/2 | 26.00 | Mandatory | 54.0 | Optional | 28.89 | Optional | 60 | Optional |
| 4 | 1 | 16-QAM | 3/4 | 39.00 | Mandatory | 81.0 | Optional | 43.33 | Optional | 90 | Optional |
| 5 | 1 | 64-QAM | 2/3 | 52.00 | Mandatory | 108.0 | Optional | 57.78 | Optional | 120 | Optional |
| 6 | 1 | 64-QAM | 3/4 | 58.50 | Mandatory | 121.5 | Optional | 65.00 | Optional | 135 | Optional |
| 7 | 1 | 64-QAM | 5/6 | 65.00 | Mandatory | 135.0 | Optional | 72.22 | Mandatory | 150.0 | Optional |
| 8 | 2 | BPSK | 1/2 | 13.00 | Mandatory | 27 | Optional | 14.44 | Mandatory | 30 | Optional |
| 9 | 2 | QPSK | 1/2 | 26.00 | Mandatory | 54 | Optional | 28.89 | Mandatory | 60 | Optional |
| 10 | 2 | QPSK | 3/4 | 39.00 | Mandatory | 81 | Optional | 43.33 | Mandatory | 90 | Optional |
| 11 | 2 | 16-QAM | 1/2 | 52.00 | Mandatory | 108 | Optional | 57.78 | Mandatory | 120 | Optional |
| 12 | 2 | 16-QAM | 3/4 | 78.00 | Mandatory | 162 | Optional | 86.67 | Mandatory | 180 | Optional |
| 13 | 2 | 64-QAM | 2/3 | 104.00 | Mandatory | 216 | Optional | 115.56 | Mandatory | 240 | Optional |
| 14 | 2 | 64-QAM | 3/4 | 117.00 | Mandatory | 243 | Optional | 130.00 | Mandatory | 270 | Optional |
| 15 | 2 | 64-QAM | 5/6 | 130.00 | Mandatory | 270.0 | Optional | 144.44 | Mandatory | 300.0 | Optional |
| 16 | 3 | BPSK | 1/2 | 19.50 | Optional | 40.5 | Optional | 21.67 | Optional | 45 | Optional |
| 17 | 3 | QPSK | 1/2 | 39.00 | Optional | 81 | Optional | 43.33 | Optional | 90 | Optional |
| 18 | 3 | QPSK | 3/4 | 58.50 | Optional | 121.5 | Optional | 65.00 | Optional | 135 | Optional |
| 19 | 3 | 16-QAM | 1/2 | 78.00 | Optional | 162 | Optional | 86.67 | Optional | 180 | Optional |
| 20 | 3 | 16-QAM | 3/4 | 117.00 | Optional | 243 | Optional | 130.00 | Optional | 270 | Optional |
| 21 | 3 | 64-QAM | 2/3 | 156.00 | Optional | 324 | Optional | 173.33 | Optional | 360 | Optional |
| 22 | 3 | 64-QAM | 3/4 | 175.50 | Optional | 364.5 | Optional | 195.00 | Optional | 405 | Optional |
| 23 | 3 | 64-QAM | 5/6 | 195.00 | Optional | 405 | Optional | 216.67 | Optional | 450 | Optional |
| 24 | 4 | BPSK | 1/2 | 26.00 | Optional | 54 | Optional | 28.89 | Optional | 60 | Optional |
| 25 | 4 | QPSK | 1/2 | 52.00 | Optional | 108 | Optional | 57.78 | Optional | 120 | Optional |
| 26 | 4 | QPSK | 3/4 | 78.00 | Optional | 162 | Optional | 86.67 | Optional | 180 | Optional |
| 27 | 4 | 16-QAM | 1/2 | 104.00 | Optional | 216 | Optional | 115.56 | Optional | 240 | Optional |
| 28 | 4 | 16-QAM | 3/4 | 156.00 | Optional | 324 | Optional | 173.33 | Optional | 360 | Optional |
| 29 | 4 | 64-QAM | 2/3 | 208.00 | Optional | 432 | Optional | 231.11 | Optional | 480 | Optional |
| 30 | 4 | 64-QAM | 3/4 | 234.00 | Optional | 486 | Optional | 260.00 | Optional | 540 | Optional |
| 31 | 4 | 64-QAM | 5/6 | 260.00 | Optional | 540 | Optional | 288.89 | Optional | 600 | Optional |
| 32 | 1 | BPSK | 1/2 | N/A | N/A | 6 | Optional | N/A | N/A | 6.67 | Optional |

#### Extended Modes

The following modes are the extended mode set.  All the extended modes are optional.

| MCS Index | Code Rate | Stream Modulations |  |  |  | Full GI |  | Half GI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | M0 | M1 | M2 | M3 | 20MHz | 40MHz | 20MHz | 40MHz |
| 33 | 3/4 | 256-QAM | OFF | OFF | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 34 | 1/2 | 16-QAM | 16-QAM | OFF | OFF | 52.00 | 108.00 | 57.78 | 120.00 |
| 35 | 1/2 | 64-QAM | QPSK | OFF | OFF | 52.00 | 108.00 | 57.78 | 120.00 |
| 36 | 1/2 | 64-QAM | 16-QAM | OFF | OFF | 65.00 | 135.00 | 72.22 | 150.00 |
| 37 | 3/4 | 16-QAM | QPSK | OFF | OFF | 58.50 | 121.50 | 65.00 | 135.00 |
| 38 | 3/4 | 64-QAM | QPSK | OFF | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 39 | 3/4 | 64-QAM | 16-QAM | OFF | OFF | 97.50 | 202.50 | 108.33 | 225.00 |
| 40 | 3/4 | 256-QAM | QPSK | OFF | OFF | 97.50 | 202.50 | 108.33 | 225.00 |
| 41 | 3/4 | 256-QAM | 16-QAM | OFF | OFF | 117.00 | 243.00 | 130.00 | 270.00 |
| 42 | 3/4 | 256-QAM | 64-QAM | OFF | OFF | 136.50 | 283.50 | 151.67 | 315.00 |
| 43 | 3/4 | 256-QAM | 256-QAM | OFF | OFF | 156.00 | 324.00 | 173.33 | 360.00 |
| 44 | 1/2 | 16-QAM | QPSK | QPSK | OFF | 52.00 | 108.00 | 57.78 | 120.00 |
| 45 | 1/2 | 16-QAM | 16-QAM | QPSK | OFF | 65.00 | 135.00 | 72.22 | 150.00 |
| 46 | 1/2 | 64-QAM | QPSK | QPSK | OFF | 65.00 | 135.00 | 72.22 | 150.00 |
| 47 | 1/2 | 64-QAM | 16-QAM | QPSK | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 48 | 1/2 | 64-QAM | 16-QAM | 16-QAM | OFF | 91.00 | 189.00 | 101.11 | 210.00 |
| 49 | 1/2 | 64-QAM | 64-QAM | QPSK | OFF | 91.00 | 189.00 | 101.11 | 210.00 |
| 50 | 1/2 | 64-QAM | 64-QAM | 16-QAM | OFF | 104.00 | 216.00 | 115.56 | 240.00 |
| 51 | 1/2 | 256-QAM | QPSK | QPSK | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 52 | 1/2 | 256-QAM | 16-QAM | QPSK | OFF | 91.00 | 189.00 | 101.11 | 210.00 |
| 53 | 1/2 | 256-QAM | 16-QAM | 16-QAM | OFF | 104.00 | 216.00 | 115.56 | 240.00 |
| 54 | 1/2 | 256-QAM | 64-QAM | QPSK | OFF | 104.00 | 216.00 | 115.56 | 240.00 |
| 55 | 3/4 | 16-QAM | QPSK | QPSK | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 56 | 3/4 | 16-QAM | 16-QAM | QPSK | OFF | 97.50 | 202.50 | 108.33 | 225.00 |
| 57 | 3/4 | 64-QAM | QPSK | QPSK | OFF | 97.50 | 202.50 | 108.33 | 225.00 |
| 58 | 3/4 | 64-QAM | 16-QAM | QPSK | OFF | 117.00 | 243.00 | 130.00 | 270.00 |
| 59 | 3/4 | 64-QAM | 16-QAM | 16-QAM | OFF | 136.50 | 283.50 | 151.67 | 315.00 |
| 60 | 3/4 | 64-QAM | 64-QAM | QPSK | OFF | 136.50 | 283.50 | 151.67 | 315.00 |
| 61 | 3/4 | 64-QAM | 64-QAM | 16-QAM | OFF | 156.00 | 324.00 | 173.33 | 360.00 |
| 62 | 3/4 | 256-QAM | QPSK | QPSK | OFF | 117.00 | 243.00 | 130.00 | 270.00 |
| 63 | 3/4 | 256-QAM | 16-QAM | QPSK | OFF | 136.50 | 283.50 | 151.67 | 315.00 |
| 64 | 3/4 | 256-QAM | 16-QAM | 16-QAM | OFF | 156.00 | 324.00 | 173.33 | 360.00 |
| 65 | 3/4 | 256-QAM | 64-QAM | QPSK | OFF | 156.00 | 324.00 | 173.33 | 360.00 |
| 66 | 3/4 | 256-QAM | 64-QAM | 16-QAM | OFF | 175.50 | 364.50 | 195.00 | 405.00 |
| 67 | 3/4 | 256-QAM | 64-QAM | 64-QAM | OFF | 195.00 | 405.00 | 216.67 | 450.00 |
| 68 | 3/4 | 256-QAM | 256-QAM | QPSK | OFF | 175.50 | 364.50 | 195.00 | 405.00 |
| 69 | 3/4 | 256-QAM | 256-QAM | 16-QAM | OFF | 195.00 | 405.00 | 216.67 | 450.00 |
| 70 | 3/4 | 256-QAM | 256-QAM | 64-QAM | OFF | 214.50 | 445.50 | 238.33 | 495.00 |
| 71 | 3/4 | 256-QAM | 256-QAM | 256-QAM | OFF | 234.00 | 486.00 | 260.00 | 540.00 |
| 72 | 1/2 | 16-QAM | QPSK | QPSK | QPSK | 65.00 | 135.00 | 72.22 | 150.00 |
| 73 | 1/2 | 16-QAM | 16-QAM | QPSK | QPSK | 78.00 | 162.00 | 86.67 | 180.00 |
| 74 | 1/2 | 16-QAM | 16-QAM | 16-QAM | QPSK | 91.00 | 189.00 | 101.11 | 210.00 |
| 75 | 1/2 | 64-QAM | QPSK | QPSK | QPSK | 78.00 | 162.00 | 86.67 | 180.00 |
| 76 | 1/2 | 64-QAM | 16-QAM | QPSK | QPSK | 91.00 | 189.00 | 101.11 | 210.00 |
| 77 | 1/2 | 64-QAM | 16-QAM | 16-QAM | QPSK | 104.00 | 216.00 | 115.56 | 240.00 |
| 78 | 1/2 | 64-QAM | 16-QAM | 16-QAM | 16-QAM | 117.00 | 243.00 | 130.00 | 270.00 |
| 79 | 1/2 | 64-QAM | 64-QAM | QPSK | QPSK | 104.00 | 216.00 | 115.56 | 240.00 |
| 80 | 1/2 | 64-QAM | 64-QAM | 16-QAM | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 81 | 1/2 | 64-QAM | 64-QAM | 16-QAM | 16-QAM | 130.00 | 270.00 | 144.44 | 300.00 |
| 82 | 1/2 | 64-QAM | 64-QAM | 64-QAM | QPSK | 130.00 | 270.00 | 144.44 | 300.00 |
| 83 | 1/2 | 64-QAM | 64-QAM | 64-QAM | 16-QAM | 143.00 | 297.00 | 158.89 | 330.00 |
| 84 | 1/2 | 256-QAM | QPSK | QPSK | QPSK | 91.00 | 189.00 | 101.11 | 210.00 |
| 85 | 1/2 | 256-QAM | 16-QAM | QPSK | QPSK | 104.00 | 216.00 | 115.56 | 240.00 |
| 86 | 1/2 | 256-QAM | 16-QAM | 16-QAM | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 87 | 1/2 | 256-QAM | 16-QAM | 16-QAM | 16-QAM | 130.00 | 270.00 | 144.44 | 300.00 |
| 88 | 1/2 | 256-QAM | 64-QAM | QPSK | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 89 | 1/2 | 256-QAM | 64-QAM | 16-QAM | QPSK | 130.00 | 270.00 | 144.44 | 300.00 |
| 90 | 1/2 | 256-QAM | 64-QAM | 16-QAM | 16-QAM | 143.00 | 297.00 | 158.89 | 330.00 |
| 91 | 1/2 | 256-QAM | 64-QAM | 64-QAM | QPSK | 143.00 | 297.00 | 158.89 | 330.00 |
| 92 | 1/2 | 256-QAM | 256-QAM | 16-QAM | QPSK | 143.00 | 297.00 | 158.89 | 330.00 |
| 93 | 1/2 | 256-QAM | 256-QAM | 64-QAM | QPSK | 156.00 | 324.00 | 173.33 | 360.00 |
| 94 | 1/2 | 256-QAM | 256-QAM | 256-QAM | QPSK | 169.00 | 351.00 | 187.78 | 390.00 |
| 95 | 3/4 | 16-QAM | QPSK | QPSK | QPSK | 97.50 | 202.50 | 108.33 | 225.00 |
| 96 | 3/4 | 16-QAM | 16-QAM | QPSK | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 97 | 3/4 | 16-QAM | 16-QAM | 16-QAM | QPSK | 136.50 | 283.50 | 151.67 | 315.00 |
| 98 | 3/4 | 64-QAM | QPSK | QPSK | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 99 | 3/4 | 64-QAM | 16-QAM | QPSK | QPSK | 136.50 | 283.50 | 151.67 | 315.00 |
| 100 | 3/4 | 64-QAM | 16-QAM | 16-QAM | QPSK | 156.00 | 324.00 | 173.33 | 360.00 |
| 101 | 3/4 | 64-QAM | 16-QAM | 16-QAM | 16-QAM | 175.50 | 364.50 | 195.00 | 405.00 |
| 102 | 3/4 | 64-QAM | 64-QAM | QPSK | QPSK | 156.00 | 324.00 | 173.33 | 360.00 |
| 103 | 3/4 | 64-QAM | 64-QAM | 16-QAM | QPSK | 175.50 | 364.50 | 195.00 | 405.00 |
| 104 | 3/4 | 64-QAM | 64-QAM | 16-QAM | 16-QAM | 195.00 | 405.00 | 216.67 | 450.00 |
| 105 | 3/4 | 64-QAM | 64-QAM | 64-QAM | QPSK | 195.00 | 405.00 | 216.67 | 450.00 |
| 106 | 3/4 | 64-QAM | 64-QAM | 64-QAM | 16-QAM | 214.50 | 445.50 | 238.33 | 495.00 |
| 107 | 3/4 | 256-QAM | QPSK | QPSK | QPSK | 136.50 | 283.50 | 151.67 | 315.00 |
| 108 | 3/4 | 256-QAM | 16-QAM | QPSK | QPSK | 156.00 | 324.00 | 173.33 | 360.00 |
| 109 | 3/4 | 256-QAM | 16-QAM | 16-QAM | QPSK | 175.50 | 364.50 | 195.00 | 405.00 |
| 110 | 3/4 | 256-QAM | 16-QAM | 16-QAM | 16-QAM | 195.00 | 405.00 | 216.67 | 450.00 |
| 111 | 3/4 | 256-QAM | 64-QAM | QPSK | QPSK | 175.50 | 364.50 | 195.00 | 405.00 |
| 112 | 3/4 | 256-QAM | 64-QAM | 16-QAM | QPSK | 195.00 | 405.00 | 216.67 | 450.00 |
| 113 | 3/4 | 256-QAM | 64-QAM | 16-QAM | 16-QAM | 214.50 | 445.50 | 238.33 | 495.00 |
| 114 | 3/4 | 256-QAM | 64-QAM | 64-QAM | QPSK | 214.50 | 445.50 | 238.33 | 495.00 |
| 115 | 3/4 | 256-QAM | 64-QAM | 64-QAM | 16-QAM | 234.00 | 486.00 | 260.00 | 540.00 |
| 116 | 3/4 | 256-QAM | 64-QAM | 64-QAM | 64-QAM | 253.50 | 526.50 | 281.67 | 585.00 |
| 117 | 3/4 | 256-QAM | 256-QAM | QPSK | QPSK | 195.00 | 405.00 | 216.67 | 450.00 |
| 118 | 3/4 | 256-QAM | 256-QAM | 16-QAM | QPSK | 214.50 | 445.50 | 238.33 | 495.00 |
| 119 | 3/4 | 256-QAM | 256-QAM | 16-QAM | 16-QAM | 234.00 | 486.00 | 260.00 | 540.00 |
| 120 | 3/4 | 256-QAM | 256-QAM | 64-QAM | QPSK | 234.00 | 486.00 | 260.00 | 540.00 |
| 121 | 3/4 | 256-QAM | 256-QAM | 64-QAM | 16-QAM | 253.50 | 526.50 | 281.67 | 585.00 |
| 122 | 3/4 | 256-QAM | 256-QAM | 64-QAM | 64-QAM | 273.00 | 567.00 | 303.33 | 630.00 |
| 123 | 3/4 | 256-QAM | 256-QAM | 256-QAM | QPSK | 253.50 | 526.50 | 281.67 | 585.00 |
| 124 | 3/4 | 256-QAM | 256-QAM | 256-QAM | 16-QAM | 273.00 | 567.00 | 303.33 | 630.00 |
| 125 | 3/4 | 256-QAM | 256-QAM | 256-QAM | 64-QAM | 292.50 | 607.50 | 325.00 | 675.00 |
| 126 | 3/4 | 256-QAM | 256-QAM | 256-QAM | 256-QAM | 312.00 | 648.00 | 346.67 | 720.00 |

### CC42 Preamble

A detailed description of the preamble is given in [3, Sections 11.1.1.3 and 11.3.1.3].

The preamble is constructed as follows:

- Legacy short training field (8usec)

- Legacy long training field (8usec)

- Legacy signal field (one OFDM symbol, 4usec)

- High throughput signal field (two OFDM symbols, 8usec)

- High throughput short training field (2.4usec)

- High throughput long train fields (7.2usec per data stream)

The legacy OFDM short training (STF), long training (LTF), and signal field (L-SIG) ensure that legacy OFDM stations will be able to perform start-of-packet detection, AGC, frequency estimation, timing offset estimation, and channel estimation without any degradation.  Furthermore, legacy OFDM stations will be able to decode the legacy signal field and extract the RATE and LENGTH fields.  The RATE and LENGTH fields in the legacy signal field are “spoofed” such that a legacy OFDM RX can defer for the correct period of time.

In the 40MHz mode, the STF in the lower sub-channel is identical to legacy, whereas the STF in the upper sub-channel is a 90-degree phase rotation of the legacy STF, similarly for LTF. This rotation is performed to keep the PAPR of the STF low.  In a 40MHz transmission, the L-SIG in the upper and the lower sub-channels are rotated by 90 degrees, to be consistent with the LTF.

The High Throughput signal field (HT-SIG) contains information needed to correctly demap and decode the HT data fields (i.e. the MIMO-OFDM symbols). It also contains fields for supporting advanced techniques.

To achieve an accurate HT AGC measurement, a HT-STF is transmitted after the HT-SIG. The HT short training symbol uses 24 tones. The choice of 24 tones permits accurate AGC measurements across 4 transmit antennas. In the 40 MHz channel, 48 tones are used in the HT-STF. The 24 (48) tones are interleaved across the antennas. The length of the HT-STS is 1.6µs.  The purpose of HT-STF is purely for AGC purposes, and hence only one STS is needed. Together with the 0.8µs guard interval, the total length of the HT-STF is 2.4µs. It is anticipated that the latter portion of the 2.4s duration would act as a blanking interval to allow the RFIC transients to settle down.

After the AGC is adjusted for HT reception, the system is re-trained for channel estimation, fine frequency offset estimation etc. The number of HT-LTFs is equal to the number of spatial streams in the basic MIMO mode.  The HT-LTF is also tone interleaved in order to have the same power level as the AGC setting.  The HT-LTF consists of two Long Training Symbols (LTS) as in 802.11a/g and a regular guard interval of 0.8 µs. The total length of one HT-LTF is 7.2µs.

The mean and standard deviation (Std) of the peak to sidelobe ratio (in dB) of the autocorrelation function for the various components of the preamble are given in the table below.  For the HT-STF and HT-LTF a range of autocorrelation values are given covering one to four data streams.

|  | L-STF |  | L-LTF |  | HT-STF |  | HT-LTF |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | Std | Mean | Std | Mean | Std | Mean | Std |
| 20MHz | 10.2 | 11.5 | 15.7 | 17.8 | 10.2 – 13.8 | 11.5 – 17.0 | 13.4 – 15.1 | 12.4 – 17.1 |
| 40MHz | 15.1 | 16.5 | 19.7 | 21.2 | 13.4 – 17.1 | 12.8 – 17.3 | 14.9– 17.8 | 14.7 – 23.2 |

By using tone interleaving, the cross correlation is zero across antennas during HT-STF and HT-LTF.

The PAPR values (in dB) for the various components of the preamble are given in the table below.  For the HT-STF and HT-LTF a range of PAPR values are given covering one to four data streams.

|  | L-STF | L-LTF | HT-STF | HT-LTF |
| --- | --- | --- | --- | --- |
| 20MHz | 2.0 | 3.2 | 3.7-4.4 | 2.4-3.3 |
| 40MHz | 2.0 | 3.2 | 2.5-4.8 | 2.9-5.1 |

### CC51.5 Channelization

This proposal supports 10, 20, and 40MHz channelization.

### CC52 Spectral Mask

Transmit spectral mask requirements are given in [3, Section 11.3.1.7.1] for 20 and 40MHz operation and [3, Section 11.1.1.1] describes 10MHz operation.

### CC80 Required changes to 802.11 PHY

- Optional 40MHz channel

  - [3, Sections 11.1.1.2, 11.1.1.3, 11.3.1.5.9]

- MIMO spatial division multiplexing with up to 4 spatial streams

  - [3, Sections 11.1.1.2, 11.1.1.3, 11.3.1.5.4]

- High throughput preamble

  - [3, Sections 11.1.1.4, 11.3.1.1]

- Mandatory 5/6 coding

  - [3, Sections 11.1.1.2, 11.3.1.5.6]

- Mandatory ½ guard interval

  - [3, Sections 11.1.1.2, 11.1.2]

- Optional LDPC advanced coding

  - [3, Sections 11.1.1.6, 11.2.4.1, 11.2.4.2]

- Optional Transmit beamforming

  - [3, Sections 11.1.1.6, 11.2.2, 11.2.3]

- 56 tones

  - [3, Sections ]