Simulation Report 
for Draft Samsung MAC proposal

Aug. 12, 2004

Youngsoo Kim		(kimyoungsoo@samsung.com)

Kyunghun Jang	(khjang@samsung.com)

Jin-Bong Chang	(jinb.chang@samsung.com)

Dongjun Lee		(djthekid.lee@samsung.com)

Sunghyun Choi	        (schoi@snu.ac.kr)

Seongkwan Kim	(skim@mwnl.snu.ac.kr)

JeongGyun Yu		(jgyu@mwnl.snu.ac.kr)

Table of Contents

1.	Simulation MAC modes	4

1.1	Mode 1: Periodic MP frame	4

1.2	Mode 2: Aperiodic MP frame	4

2.	Simultion Methodology	5

2.1	Summary of PHY Layer Modeling	5

2.2	Higher Layer Assumptions	7

2.3	MAC Layer Parameters	7

2.3.1	Frame Size	7

2.3.2	Frame Aggregation	7

2.3.3	Rate vs. Distance	7

2.3.4	802.11e Support	8

2.3.5	Additional MAC Parameters	8

3.	Simulation Scenarios	10

3.1	Residential	10

3.2	Large Enterprise	10

3.3	Hot Spot	11

4.	Simulation Results	13

List of Figures

Figure 1 - RxSNR vs. Distance when BW=40MHz, TxPwr=50mW, NF=10dB, Channel B	5

Figure 2 - RxSNR vs. Distance when BW=40MHz, TxPwr=50mW, NF=10dB, Channel D	6

Figure 3 - RxSNR vs. Distance when BW=40MHz, TxPwr=50mW, NF=10dB, Channel E	6

# Simulation MAC modes

## Mode 1: Periodic MP frame

## Mode 2: Aperiodic MP frame

# Simultion Methodology

## Summary of PHY Layer Modeling

The following physical layer characteristics and parameters are assumed for all of the stations, for this evaluation.

| PHY Model Parameters | Parameter |
| --- | --- |
| Frequency Channel | 5.25[GHz] |
| Multi-path fading | Disable |
| Shadow fading | Disabled |
| Signal Bandwidth | 40.0[MHz] |
| Antenna Configuration | 2X2 |
| Transmit Power | 50.0[mW] |
| Antenna Gain (TX/RX) | 0.0[dBi] |
| Antenna Pattern (TX/RX) | Omni-Directional |
| Noise Figure | 10.0[dB] |
| CCA accuracy (energy detection) | RSSI >= -65[dBm] |
| CCA accuracy (preamble detection) | SINR >= 3.0[dB] |
| CCA delay | 6.0[usec] |

The Figure 1 shows relationship between Rx. SNR vs. distance between transmitter and receiver, based on channel model B.

Figure 1 - RxSNR vs. Distance when BW=40MHz, TxPwr=50mW, NF=10dB, Channel B

The green lines show the approximated boundaries between the selections of data rate classes used when Channel Model B is used.

Figure 2 - RxSNR vs. Distance when BW=40MHz, TxPwr=50mW, NF=10dB, Channel D

The Figure 2 shows relationship between Rx. SNR vs. distance between transmitter and receiver, based on channel model D. The green lines show the approximated boundaries between the selections of data rate classes used when Channel Model D is used.

Figure 3 - RxSNR vs. Distance when BW=40MHz, TxPwr=50mW, NF=10dB, Channel E

The Figure 3 shows relationship between Rx. SNR vs. distance between transmitter and receiver, based on channel model E. The green lines show the approximated boundaries between the selections of data rate classes used when Channel Model E is used.

## Higher Layer Assumptions

The type and parameter of the TCP model is as follows:

| TCP Factor | Parameter |
| --- | --- |
| TCP Model | Reno |
| Duplicate ACK threshold | 3 |
| ACK policy | Delayed ACK |
| Maximum ACK delay | 200msec |

## MAC Layer Parameters

The following section describes the MAC layer assumptions used in the simulator.

### Frame Size

Frame Sizes are based on the Samsung MAC Specification draft, D.10.0.

| Frame Size | Parameter |
| --- | --- |
| MP frame (Header + Payload) | 18+(5*n)+(4*m) [Bytes] |
| Smart Block ACK Request | 22 [Bytes] |
| Smart Block ACK | 32+(2*l) [Bytes] |
| MPDU Delimiter | 4 [Bytes] |

(* n : the number of the elements of D/L MAP, m : the number of the elements of U/L MAP, l : the number of erroneous MSDUs )

### Frame Aggregation

Two-level aggregation is implemented in the simulator. The total size of aggregate PPDU is variable. The PLCP in a PPDU has the Length (12 bits) and Rate (4 bits) field in the legacy SIGNAL field. The Length and Rate fields are virtually set in order to cover the duration of the PPDU. The length of an aggregate PPDU is limited so that its duration may not exceed 5.46 milliseconds, because the maximum duration of the PPDU in 802.11a is about 5.46 millisecond, which can be derived from the maximum length (4096 bytes) divided by the lowest rate (6 Mbps).

| Aggregation Factor | Parameter |
| --- | --- |
| Frame Aggregation | Two-level |
| Maximum size of aggregate PPDU | 5msec |

### Rate vs. Distance

This predicted rate on each scenario model is determined from the distance between the transmitter and receiver based on channel model. Table 1 presents the relationship between predicted rate and distance based on channel model on each scenario.

| Rate (Mbps) | Scenario 1 | Scenario 4 | Scenario 6 |
| --- | --- | --- | --- |
|  | Bound (m) | Bound (m) | Bound (m) |
| 243 | 6.5 | 15 | 18 |
| 216 | 7.5 | 18 | 23 |
| 162 | 9 | 22 | 28 |
| 121.5 | 23.5 | 33 | 41.5 |
| 108 | 28 | 37.5 | 50.5 |
| 81 | 32 | 44 | 59.5 |
| 54 | 46 | 63.5 |  |
| 40.5 | 47 | 65.5 |  |
| 27 | 65.5 |  |  |
| 13.5 | 80 |  |  |

### 802.11e Support

Parts of 802.11e are implemented inside the simulator.

| 802.11e Factor |  | Parameter |
| --- | --- | --- |
| Block ACK |  | Not Used |
| EDCF | AIFS | Incorporated (See Below) |
|  | CW | Incorporated (See Below) |
|  | TXOP | Not Used |
| HCCA |  | Not Used |
| DLP |  | Not Used |

AIFS is used as described below.

| Priority | AC | CWmin [Slot Time] | CWmax [Slot Time] | AIFSN |
| --- | --- | --- | --- | --- |
| Lowest ↓ ↓ Highest | AC_BK | aCWmin = 15 | aCWmax= 1023 | 7 |
|  | AC_BE | aCWmin = 15 | aCWmax = 1023 | 3 |
|  | AC_VI | (aCWmin + 1) / 2 – 1 = 7 | ACWmin = 15 | 2 |
|  | AC_VO | (aCWmin + 1) / 4 – 1 = 3 | (aCWmin + 1) / 2 – 1 = 7 | 2 |

*AIFS[AC] = AIFSN[AC] x aSlotTime + a SIFSTime

### Additional MAC Parameters

Additional MAC parameters are as follows.

| MAC related parameters | Value |
| --- | --- |
| Transmission Buffer Depth | 150[MSDU] |
| MP frame interval | 10msec [mode 1] / variable [mode 2] |
| CP duration | variable [mode 1] / 0msec [mode 2] |
| ASlotTime | 9.0[usec] |
| Bi-Directional | Enabled |
| Fragmentation | Disabled |
| Pair acknowledgment | Enabled |
| Phase acknowledgment | Disabled |
| Implicit acknowledgment | Enabled |

# Simulation Scenarios

This section describes the usage models used in the simulation.

## Residential

The Residential model is modeled as follows, with the AP at the center.  See the TGn Usage Model document for details.

| From | To | Mean Rate [Mbps] | MSDU Size [Bytes] | Access Category | Higher Layer |
| --- | --- | --- | --- | --- | --- |
| 0 (AP) | 1 | 19.2 | 1500 | AC_VI | UDP |
| 0 | 3 | 24 | 1500 | AC_VI | UDP |
| 0 | 4 | 4 | 1500 | AC_VI | UDP |
| 0 | 4 | 1 | 300 | AC_BE | TCP |
| 0 | 7 | 0.096 | 120 | AC_VI | UDP |
| 0 | 8 | 0.096 | 120 | AC_VI | UDP |
| 0 | 9 | 0.096 | 120 | AC_VI | UDP |
| 0 | 10 | 2 | 512 | AC_VI | UDP |
| 0 | 11 | 0.128 | 418 | AC_VI | UDP |
| 1 | 0 | 0.06 | 64 | AC_VI | UDP |
| 3 | 0 | 0.06 | 64 | AC_VI | UDP |
| 4 | 10 | 30 | 1500 | AC_BE | TCP |
| 5 | 6 | 0.5 | 512 | AC_VI | UDP |
| 6 | 5 | 0.5 | 512 | AC_VI | UDP |
| 7 | 0 | 0.096 | 120 | AC_VI | UDP |
| 8 | 0 | 0.096 | 120 | AC_VI | UDP |
| 9 | 0 | 0.096 | 120 | AC_VI | UDP |
| 10 | 0 | 1 | 512 | AC_VI | UDP |
| 11 | 10 | 0.5 | 50 | AC_VI | UDP |

## Large Enterprise

The Large Enterprise model is modeled as follows, with the AP at the center.  See the TGn Usage Model document for details.

| From | To | Mean Rate [Mbps] | MSDU Size [Bytes] | Access Category | Higher Layer |
| --- | --- | --- | --- | --- | --- |
| 0 (AP) | 1 | 1 | 300 | AC_BE | TCP |
| 0 | 2 | 1 | 300 | AC_BE | TCP |
| 0 | 3 | 1 | 300 | AC_BE | TCP |
| 0 | 4 | 1 | 300 | AC_BE | TCP |
| 0 | 5 | 1 | 300 | AC_BE | TCP |
| 0 | 6 | 10 | 300 | AC_BE | TCP |
| 0 | 7 | 1 | 512 | AC_VI | UDP |
| 0 | 8 | 1 | 512 | AC_VI | UDP |
| 0 | 9 | 2 | 512 | AC_BE | TCP |
| 0 | 10 | 2 | 512 | AC_BE | TCP |
| 0 | 11 | 30 | 1500 | AC_BE | TCP |
| 0 | 12 | 30 | 1500 | AC_BE | TCP |
| 0 | 13 | 30 | 1500 | AC_BE | TCP |
| 0 | 14 | 30 | 1500 | AC_BE | TCP |
| 0 | 15 | 30 | 1500 | AC_BE | TCP |
| 0 | 16 | 30 | 1500 | AC_BE | TCP |
| 0 | 17 | 30 | 1500 | AC_BE | TCP |
| 0 | 18 | 30 | 1500 | AC_BE | TCP |
| 0 | 19 | 30 | 1500 | AC_BE | TCP |
| 0 | 20 | 30 | 1500 | AC_BE | TCP |
| 0 | 25 | 0.096 | 120 | AC_VI | UDP |
| 0 | 26 | 0.096 | 120 | AC_VI | UDP |
| 0 | 27 | 0.096 | 120 | AC_VI | UDP |
| 0 | 28 | 0.096 | 120 | AC_VI | UDP |
| 0 | 29 | 0.096 | 120 | AC_VI | UDP |
| 0 | 30 | 0.096 | 120 | AC_VI | UDP |
| 1 | 0 | 0.256 | 64 | AC_BE | TCP |
| 2 | 0 | 0.256 | 64 | AC_BE | TCP |
| 3 | 0 | 0.256 | 64 | AC_BE | TCP |
| 4 | 0 | 5 | 1000 | AC_BE | TCP |
| 5 | 0 | 10 | 1500 | AC_BE | TCP |
| 6 | 0 | 0.256 | 64 | AC_BE | TCP |
| 7 | 0 | 1 | 512 | AC_VI | UDP |
| 8 | 0 | 1 | 512 | AC_VI | UDP |
| 21 | 0 | 30 | 1500 | AC_BE | TCP |
| 22 | 0 | 30 | 1500 | AC_BE | TCP |
| 23 | 0 | 30 | 1500 | AC_BE | TCP |
| 24 | 0 | 30 | 1500 | AC_BE | TCP |
| 25 | 0 | 0.096 | 120 | AC_VI | UDP |
| 26 | 0 | 0.096 | 120 | AC_VI | UDP |
| 27 | 0 | 0.096 | 120 | AC_VI | UDP |
| 28 | 0 | 0.096 | 120 | AC_VI | UDP |
| 29 | 0 | 0.096 | 120 | AC_VI | UDP |
| 30 | 0 | 0.096 | 120 | AC_VI | UDP |

## Hot Spot

The Hot Spot model is modeled as follows, with the AP at the center.  See the TGn Usage Model document for details.

| From | To | Mean Rate [Mbps] | MSDU Size [Bytes] | Access Category | Higher Layer |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 300 | AC_BE | TCP |
| 0 | 2 | 2 | 300 | AC_BE | TCP |
| 0 | 3 | 2 | 300 | AC_BE | TCP |
| 0 | 4 | 2 | 300 | AC_BE | TCP |
| 0 | 5 | 2 | 300 | AC_BE | TCP |
| 0 | 6 | 2 | 300 | AC_BE | TCP |
| 0 | 7 | 2 | 300 | AC_BE | TCP |
| 0 | 8 | 2 | 300 | AC_BE | TCP |
| 0 | 9 | 2 | 300 | AC_BE | TCP |
| 0 | 10 | 2 | 300 | AC_BE | TCP |
| 0 | 11 | 2 | 512 | AC_VI | UDP |
| 0 | 12 | 2 | 512 | AC_VI | UDP |
| 0 | 13 | 2 | 512 | AC_VI | UDP |
| 0 | 14 | 2 | 512 | AC_VI | UDP |
| 0 | 15 | 2 | 512 | AC_VI | UDP |
| 0 | 16 | 2 | 512 | AC_VI | UDP |
| 0 | 17 | 2 | 512 | AC_VI | UDP |
| 0 | 18 | 5 | 1500 | AC_VI | UDP |
| 0 | 19 | 5 | 1500 | AC_VI | UDP |
| 0 | 20 | 0.096 | 120 | AC_VI | UDP |
| 0 | 21 | 0.096 | 120 | AC_VI | UDP |
| 0 | 22 | 0.096 | 120 | AC_VI | UDP |
| 0 | 23 | 0.096 | 120 | AC_VI | UDP |
| 0 | 24 | 0.096 | 120 | AC_VI | UDP |
| 0 | 25 | 0.096 | 120 | AC_VI | UDP |
| 0 | 26 | 0.096 | 120 | AC_VI | UDP |
| 0 | 27 | 0.096 | 120 | AC_VI | UDP |
| 0 | 28 | 0.096 | 120 | AC_VI | UDP |
| 0 | 29 | 0.096 | 120 | AC_VI | UDP |
| 0 | 30 | 0.096 | 120 | AC_VI | UDP |
| 0 | 31 | 0.096 | 120 | AC_VI | UDP |
| 0 | 32 | 0.096 | 120 | AC_VI | UDP |
| 0 | 33 | 0.096 | 120 | AC_VI | UDP |
| 0 | 34 | 0.096 | 120 | AC_VI | UDP |
| 20 | 0 | 0.096 | 120 | AC_VI | UDP |
| 21 | 0 | 0.096 | 120 | AC_VI | UDP |
| 22 | 0 | 0.096 | 120 | AC_VI | UDP |
| 23 | 0 | 0.096 | 120 | AC_VI | UDP |
| 24 | 0 | 0.096 | 120 | AC_VI | UDP |
| 25 | 0 | 0.096 | 120 | AC_VI | UDP |
| 26 | 0 | 0.096 | 120 | AC_VI | UDP |
| 27 | 0 | 0.096 | 120 | AC_VI | UDP |
| 28 | 0 | 0.096 | 120 | AC_VI | UDP |
| 29 | 0 | 0.096 | 120 | AC_VI | UDP |
| 30 | 0 | 0.096 | 120 | AC_VI | UDP |
| 31 | 0 | 0.096 | 120 | AC_VI | UDP |
| 32 | 0 | 0.096 | 120 | AC_VI | UDP |
| 33 | 0 | 0.096 | 120 | AC_VI | UDP |
| 34 | 0 | 0.096 | 120 | AC_VI | UDP |

# Simulation Results

Detailed simulation results are presented in a separate “IEEE 802 11-04/921, Samsung MAC Proposal CC Summary” document.

| # | Name | Simulation Scenario |  | Mode1: Periodic MP frame | Mode 2: Aperiodic MP frame |
| --- | --- | --- | --- | --- | --- |
| CC18 | HT Usage Models Supported (non QoS) | SS1 (Mbps/ratio) |  | 9.7/0.31 | 27.4/0.88 |
|  |  | SS4 |  | 124.3/0.28 | 144.3/0.32 |
|  |  | SS6 |  | 14.1/0.7 | 36.9/1.84 * |
| CC19 | HT Usage Models Supported (QoS) | SS1 |  | 16/16 ** | 16/16 ** |
|  |  | SS4 |  | 18/18 | 18/18 |
|  |  | SS6 |  | 39/39 | 39/39 |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | SS1 | M1 | 61.7 | 79.4 |
|  |  |  | M2 | 61.7 | 79.4 |
|  |  |  | M3 | 61.7 | 79.4 |
|  |  | SS4 | M1 | 133.5 | 152.5 |
|  |  |  | M2 | 133.5 | 152.5 |
|  |  |  | M3 | 133.5 | 152.5 |
|  |  | SS6 | M1 | 58.9 | 81.8 |
|  |  |  | M2 | 58.9 | 81.8 |
|  |  |  | M3 | 58.9 | 81.8 |
| CC24 | MAC Efficiency | SS1 |  | 0.40 | 0.51 |
|  |  | SS4 |  | 0.56 | 0.63 |
|  |  | SS6 |  | 0.29 | 0.40 |

* TCP doesn’t have an offered load (limitation) in simulation

** Game control traffic is not used in simulation