IEEE P802.11
Wireless LANs

ETRI Proposal Coverage of Functional Requirements

Date:	August 13, 2004

Author:	Heejung Yu, Taehyun Jeon, Sok-Kyu Lee,
	Myung-Soon Kim, Eun-young Choi, and Seung-Ku Hwang

Electronics and Telecommunications Research Institute (ETRI)
	161 Gajeong-dong Yuseong-gu, Daejeon, 305-350, KOREA
	Phone : +82 42 860 1651
	Fax : +82 42 860 6732
	e-Mail: {heejung, thjeon, sk-lee, mskim75, eychoi, skhwang}@etri.re.kr

Abstract

This document contains coverage of function requirements (IEEE 802.11-02/813r12) for ETRI’s proposal (partial; PHY) to TGn. The coverage and their related references are summarized in the following tables.

# Introduction

In this document, coverage of functional requirements for ETRI PHY proposal is included. ETRI’s TGn proposal is a partial one that only covers the physical layer specification.

# Functional Requirements

The blow table is the list of functional requirements for TGn proposal.

| Number | Name | Requirement | Status of Requirement | Straw Poll on 13 Nov 2003 | Notes (informative) |
| --- | --- | --- | --- | --- | --- |
| 1 | Single Link HT rate supported | Demonstrate at least one set of conditions under which 100 Mbps at the top of the MAC SAP can be achieved. Provide all relevant information to document this. |  |  |  |
| 2 | HT rate supported in 20MHz channel | Proposal supports at least one mode of operation that supports 100Mbps throughput at the top of the MAC SAP in a 20MHz channel. Provide all relevant information to document this. |  |  | The regulatory bodies of many countries do not allow operation of a single device using several channels |
| 3 | Supports 5GHz bands | Protocol supports 5GHz bands (including those supported by .11a) | Straw poll: 92/1 on 12 Nov 2003 | 48/0 passes |  |
| 4 | .11a backwards compatibility | Some of the modes of operation defined in the proposal shall be backwards compatible with 802.11a. | 34+43/1 on 12 Nov 2003 | 48/1 passes |  |
| 5 | .11g backwards compatibility | If it supports 2.4 GHz operation, some of the modes of operation defined in the proposal shall be backwards compatible with 802.11g. | 25+44/1+8 | 50/0 passes |  |
| 6 | Control of support for legacy STA from .11n AP | A .11n AP can be configured to reject or accept associations from legacy STA because they are legacy STA. | WFA proposal 49/17 on 12 Nov 2003 | 33/9 passes |  |
| 7 | .11e QoS support | The proposal shall permit implementation of the 802.11e ammendment within a .11n STA | John K proposal 70/6 on 12 Nov 2003. | 39/0 passes |  |
| 8 | Spectral Efficiency | The highest throughput mode of the proposal shall achieve a spectral efficiency of at least 3 bps/Hz for the PSDU | Vote 73/0 on 13 Nov 2003 | 53/0 passes |  |
| 9 | Compliance to PAR | The proposal complies with all the mandatory requirements of the PAR [5] and 5 Criteria [6] | Voted yes 71/0 13 Nov 2003 | 56/0 passes |  |

# Coverage of Functional Requirements

The coverage and results reference columns (blue ones) are added in the provided template.

| Number | Name | Coverage (Yes/No) | Results Reference |
| --- | --- | --- | --- |
| R1 | Single Link HT rate supported | Yes (with assumption of 35% and 47% MAC efficiency in case of 288Mbps and 216Mbps, respectively.) | 288Mbps (optional) or 216Mbps (Mandatory) at PHY (details in Table.124D in PHY spec.) |
| R2 | HT rate supported in 20MHz channel | Yes (with assumption of 70% MAC efficiency) | 144Mbps (optional) |
| R3 | Supports 5GHz bands | Yes |  |
| R4 | .11a backwards compatibility | Yes |  |
| R5 | .11g backwards compatibility | No | There is no 11b modes |
| R6 | Control of support for legacy STA from .11n AP | N/A | Only PHY proposal |
| R7 | .11e QoS support | N/A | Only PHY proposal |
| R8 | Spectral Efficiency | Yes | 288Mbps/40MHz = 7.2bits/sec/Hz |
| R9 | Compliance to PAR | Yes | 100Mbps at MAC SAP. Compatibility with 11a/g |

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/813r12, TGn Functional Requirements

[3] IEEE 802 11-03/814r31, TGn Comparison Criteria

[4] IEEE 802 11-03/802, TGn Usage Models

[5] IEEE 11-03/940r2, TGn Channel Models

[6] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[7] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)