IEEE 802.11n 
Wireless LANs

Partial Proposal (Physical Layer) for extending the 802.11-1999 standard.

Date:	August 13th, 2004

Authors:	Patrik Eriksson, Anders Edman, Christian Kark
	Wavebreaker AB,
	Bredgatan 34, SE-602 21, Norrkoping, Sweden.
	Phone: +46 11 36 36 00
	Fax: +46 11 36 36 01
	e-Mail: patrik.eriksson@acreo.se

Scott Leyonhjelm (Editor), Mike Faulkner,  Melvyn Pereira,  Jason Gao, Aaron Reid, Tan Ying, Vasantha Crabb.
	Australian Telecommunication Co-operative Research Centre,
	P.O Box 14428, Melbourne, Vic 8001, Australia.
	Phone: +61 3 9688 4247
	Fax: +61 3 9688 4058
	e-Mail: scott.leyonhjelm@vu.edu.au

Abstract:	A “High Throughput” physical layer extension to the 802.11-1999 standard that demonstrates physical layer data rates of up to 216Mbps in 20MHz bandwidth and demonstrates throughputs in excess of 100Mbps at the MAC SAP is proposed. The key benefits of the proposal are:

Higher data throughput due to use of higher order QAM modulation and multiple antennas:

- 256QAM, specifically 64Mbps (256QAM @ R2/3) and 72Mbps (256QAM @ R¾)

- up to 3 transmit antennas (mandatory), 4 antennas (optional)

Higher data throughput due to reduced overhead achieved by

- Shorter allowed SIFS, down to 8 us.

- Frames without short and long training sequences

- Frame aggregation – max 16kbyte packet size

Higher data throughput due to adaptive loading achieved by

- Adapting the data rate on a per layer (mandatory) or per a subgroup (optional) level

- Fast adaptation to best rates as each frame reports back a recommended rate.

All structures are simple extensions to 11a.

- Enables reuse of design and logic between 11a and 11n

- lla STS and LTS sequences are used in conjunction with progressive cyclic delay per antenna

Frame format designed to increase available time for inverting channel estimate.

- Simplified hardware for the matrix inversion made possible.

| Notice | This document has been prepared to assist the IEEE P802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| --- | --- |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.11. |

Table of Contents

1	Abbreviations	4

2	Physical Layer Proposal	5

2.1	Introduction	5

2.2	MIMO-OFDM-AdaptLoad PLCP sublayer	5

2.2.1	Introduction	5

2.2.2	PLCP frame formats	6

2.2.2.1	RATE dependent options	7

2.2.3	PLCP preamble	8

2.2.4	Signal (Sig) Symbol	8

2.2.4.1	Short Rate field (Short Rate)	8

2.2.4.2	High Throughput field (HT)	9

2.2.5	Signal 2 (Sig2) Symbol	9

2.2.5.1	Adaptive Loading Mode field (AL)	9

2.2.5.2	Channel State Information Indication field (CSI)	9

2.2.5.3	High Throughput field (HT)	9

2.2.5.4	802.11n Transmission Mode field (Mode)	10

2.2.5.5	Sig3 Symbol On field (Sig3On)	10

2.2.5.6	Sig3 Symbol Rate field (Sig3R)	10

2.2.5.7	Channel Bonding field (CB)	10

2.2.5.8	Reserved (Res), Parity (P) and Tail (Tail) fields	11

2.2.6	Signal 3 (Sig3) Symbols	11

2.2.6.1	Reverse Link Channel State Information field (Rev CSI)	11

2.2.6.2	Forward link rate field (FWD Rate)	11

2.2.6.3	Request for Training MIMO Type 1 Frame field (Req)	11

2.2.6.4	MIMO 802.11n data length field (11nLength)	12

2.2.6.5	Pad with Zeros field (Pad)	12

2.2.6.6	Reserved (Res), Parity (P) and Tail (Tail) fields	12

2.2.7	Data Symbols	12

2.2.7.1	Subcarrier Modulation Mapping Additions	12

2.2.8	PMD operating specifications	13

2.2.8.1	Parallel Spatial Multiplexing Transmitter Architecture	13

2.2.8.2	Adaptive Loading Overview	13

2.3	MIMO-OFDM-AdaptLoad Frame Transmission Modes	14

2.3.1	5 GHz Band Proposal	14

2.3.1.1	Single Packet Transmission Mode	14

2.3.1.2	RTS/CTS Transmission Mode	14

2.3.2	2.4 GHz Band Proposal	16

2.3.2.1	Single Packet Transmission Mode	16

2.3.2.2	RTS/CTS Transmission Mode	16

2.3.3	Implementation Details of the Proposed Frame Format	17

2.3.3.1	802.11n Channel Environment	18

2.3.3.2	Tracking Algorithm Simulations	20

3	Addressing the Functional Requirements	21

4	Addressing Section 3 of the Comparision Criteria	22

4.1	AD1 – Reference Submissions	22

4.2	AD2 – TCP Model Parameters	22

4.3	AD3 – MAC Simulation Methodology	22

4.4	AD4 – MAC Simulation occupied channel width	22

4.5	AD5 – Justification of low packet loss rate (PLR) rates achieved	22

5	Addressing Section 4 of the Comparision Criteria	23

5.1	General	23

5.1.1	CC2 - Regulatory Compliance	23

5.2	Marketability	23

5.2.1	CC3 - List of goodput results for usage models 1,4 and 6.	23

5.2.2	CC6 – PHY complexity	23

5.2.3	CC7 – MAC processing complexity	23

5.3	Backward Compatibility and Coexistence with Legacy Devices	23

5.3.1	CC11 – Backward compatability with 802.11-1999 (Rev 2003) and 802.11g	23

5.3.2	CC15 – Sharing of medium with legacy devices	23

5.4	MAC Related	24

5.4.1	CC46 – MAC Compatibility and Parameters	24

5.4.2	CC47 – MAC extensions	24

5.5	PHY related	25

5.5.1	PHY Rates and Preambles	25

5.5.1.1	CC51 - Data Rates	25

5.5.1.2	CC42 – Preambles	25

5.5.2	Channelisation	26

5.5.2.1	CC51.5 - Channelisation	26

5.5.2.2	CC52 – Spectral Mask	26

5.5.3	Spectral Efficiency	27

5.5.3.1	CC58 – HT spectral Efficiency	27

5.5.4	PHY Performance	33

5.5.4.1	CC59 – AWGN PER performance	33

5.5.4.2	CC67 – PER performance in non AWGN channels	36

5.5.4.3	CC67.2 – Offset Compensation	38

5.5.5	PHY Changes	38

5.5.5.1	CC80 – Required Changes to 802.11 PHY	38

6	References	38

7	Appendices	39

7.1	Appendix A – Background to Cyclic Delay Technique	39

7.2	Appendix B – Example of Length Calculation of the Sig3 field	42

7.2.1	Transmitter to Receiver	42

7.2.2	Receiver to Transmitter	42

7.3	Appendix C – Frame Aggregation	43

7.4	Appendix D - CC58 Calculation Technique and Assumptions	44

7.5	Appendix E – Rate Estimation for Channel Bonding	46

# Abbreviations

| ACK | Acknowledgement |
| --- | --- |
| AGC | Automatic gain control |
| AL | Adaptive loading |
| AOD | Angle of departure |
| AP | Access point |
| AWGN | Additive white gaussian noise |
| BPSK | Binary phase shift keying |
| BW | Bandwidth |
| CB | Channel bonding |
| CP | Cylic prefix |
| CSI | Channel state information |
| CTS | Clear to send |
| DSSS | Direct spread spectrum sequence |
| EVM | Error Vector Magnitude |
| FFT | Fast fourier transform |
| FWD | Forward |
| HCF | Hybrid co-ordination function |
| HT | High Throughput |
| LAN | Local area network |
| LOS | Line of sight |
| LTS | Long training sequence |
| MAC | Medium access controller |
| MIMO | Multiple Input Multiple Output |
| MSE | Minimium Square Error |
| N.A. | Not applicable |
| NAV | Network allocation vector |
| OFDM | Orthogonal Frequency Division Multiplexing |
| P | Parity bits |
| Pad | Padding bits |
| PAR | Project Authoritation Request |
| PER | Packet error rate |
| PHY | Physical layer |
| PLCP | Physical layer convergence procedure |
| PMD | Physical medium dependent |
| PSDU | PHY sublayer service data unit |
| QAM | Quadrature Amplitude Modulation |
| QoS | Quality of service |
| QPSK | Quadrature phase shift keying |
| RTS | Request to send |
| RX | Receiver/reception |
| SAP | Service access point |
| SIFS | Short InterFrame Space |
| SISO | Single input single output |
| SNR | Signal to noise ratio |
| STA | Station |
| STS | Short training sequences |
| TX | Transmitter/transmission |
| TXOP | Transmission opportunity |

# Physical Layer Proposal

## Introduction

This clause specifies the PHY entity for a multiple input multiple output antenna orthogonal frequency division multiplexing (MIMO-OFDM) system with adaptive loading (AdaptLoad) mechanisms and the additions that have to be made to the base standard (802.11-1999 revision 2003) to accommodate the MIMO-OFDM-AdaptLoad PHY. The radio frequency LAN system is initially aimed for both the

- 5GHz band in Europe, Asia and the Americas.

- 2.4GHz band (Optional)

The OFDM system provides a wireless LAN with data payload communication capabilities of up to 3*72 = 216 Mbps in the mandatory modes and with extension up to 4*72 = 288 Mbps in optional mode. The system uses a total of 52 sub-carriers per transmit antenna that are modulated using BPSK, QPSK, 16QAM, 64QAM, and 256QAM in 20 MHz bandwidth. The proposal supports 1-4 transmit antennas (layers), where support for 1-3 antennas is mandatory. Forward error correction coding (convolutional coding) is used with coding rates of ½, 2/3 and ¾.

## MIMO-OFDM-AdaptLoad PLCP sublayer

### Introduction

In addition to the frame formats defined in IEEE stds 802.11a-1999 and 802.11g, 3 new MIMO frame types are proposed:

- MIMO - Type 1 frames with Training. Note that the STS, LTS and Sig2 sequence can be received by legacy equipment.

- MIMO - Type 2 frames without Training. Note that channel tracking algorithms will be required.

- MIMO – Type 3 frames with Training used only in 5GHz band.

Extt

D1

D2

Note: Sig3 and Data Symbols can be turned off.

802.11n MIMO - Type 1

802.11n MIMO - Type 2

STS1

LTS1

Sig2

LTS1a

LTS1b

LTS1c

STS2

LTS2

Sig2

LTS2a

LTS2b

LTS2c

STS3

LTS3

Sig2

LTS3a

LTS3b

LTS3c

STS4

LTS4

Sig2

LTS4a

LTS4b

LTS4c

Sig3

Sig3

Sig3

Sig3

D1

D1

D1

D1

D2

D2

D2

D2

Dn

Dn

Dn

Dn

Sig3

Sig3

Sig3

Sig3

D1

D1

D1

D1

D2

D2

D2

D2

Dn

Dn

Dn

Dn

802.11n MIMO - Type 3

Sig2

LTS1a

LTS1b

LTS1c

Sig2

LTS2a

LTS2b

LTS2c

Sig2

LTS3a

LTS3b

LTS3c

Sig2

LTS4a

LTS4b

LTS4c

Sig3

Sig3

Sig3

Sig3

D1

D1

D1

D1

D2

D2

D2

D2

STS1

LTS1

Sig

STS2

LTS2

Sig

STS3

LTS3

Sig

STS4

LTS4

Sig

D1

D1

D1

D1

D2

D2

D2

D2

Dn

Dn

Dn

Dn

802.11a OFDM Frame format

802.11g DSSS Frame format

802.11g OFDM Frame format

STS

LTS

Sig

D1

D2

Preamble

Head

STS

LTS

Sig

D1

D2

Figure 1: Overview of Proposed Frame Formats – All symbols are assumed to be 4us except for STS and LTS which are 8us.

MIMO - Type 3 frames are only used for single frame transmissions, they are backward compatible with SISO transmissions, and they contain MIMO training and Data. However, no adaptive loading is activated for single frame transmissions. Adaptive loading is activated in a RTS/CTS transmission mode where the other 2 new frame types are used. The RTS/CTS frames are Type 3 and they are backward compatible with 11a/g frames and contain MIMO training (but no data) which is used to calculate the initial bit loading for transmission. A sequence of Type 2 frames, essentially only carrying data, follow. It is assumed free-wheeling tracking algorithms are needed to maintain time, frequency and channel estimates. The Type 1 frames are used to re-synchronise during a longer transmission. These frames are similar to Type 3 frames but they are only backward compatible with SISO transmissions up until the Sig symbol.

Note that the Sig3 symbol has been placed between the MIMO long training sequences (LTS) and the MIMO data, which increases the time available for inverting the channel matrix with one symbol period. This reduces the hardware implementation cost.  It is a basic assumption that the Type 2 frames will occur more often than the Type 1 and 3 frames, otherwise 100Mbps data throughput at the MAC layer will not be possible unless frame aggregation is used. Also the Sig3 and Data symbols can be turned off in type 1 and 3 frames. The different frame types are illustrated above in Figure 1 for up to 4 antennas. The next section defines the new symbols which should make it clear to the reader that the proposal is scalable for more than 4 antennas. Note that the 4 pilot tones are continuous throughout all the symbols, and are as defined in the 802.11a standard. Further, cyclic delay is implemented at a rate of 1 sample/antenna for symbols transmitted with same information on each antenna, see Appendix A, Section 7.1.

### PLCP frame formats

The different Signal and Data Symbol definitions used in the frame formats are summarized in Table 1.

Table 1: Signal and Data Symbol Definitions.

| Symbol | Definition | Used For |
| --- | --- | --- |
| Sig (BPSK R1/2) | Same 11a Signal Symbol field format transmitted on each antenna but; - Rate set to indicate SISO data transmission rate - R reserved bit indicate MIMO mode i.e. 1 - Length to indicate data payload size in RTS/CTS transmission TX Power per antenna reduced proportional to Ntx Cyclic delay at a rate 1sample/antenna For field definitions see Figure 3. | - Coexistance requirement due to indication that transmission is a MIMO (HT field) and Length field can be detected to defer during transmission. - Backward Compatible requirement due to indication that transmission is not a SISO mode (R field) and Length field can be detected to defer during remainder of RTS/CTS transmission. |
| D1,D2 | Same 11a RTS data (service +20byte PSDU) or CTS data (service +14byte PSDU) transmitted on each TX antenna. TX Power per antenna reduced proportional to Ntx Cyclic delay at a rate 1sample/antenna | - Backward Compatible requirement due to 11a user can interpret destination address for forthcoming transmission. |
| Sig2 (BPSK R1/2) | Same MIMO Signal Symbol transmitted on each antenna. TX Power per antenna reduced proportional to Ntx Cyclic delay at a rate 1sample/antenna . For field definitions see Figure 2 & Figure 3. | - Specifies MIMO mode to receiver indicating Transmission mode for Data and ACK frames. - Backward Compatible Reserved bit in Sig symbol same location as bit which is always set to 1 |
| Sig3 (see Sig2) | Adaptive Loading Symbol transmitted on each antenna. TX Power per antenna reduced proportional to Ntx Cyclic delay at a rate 1sample/antenna . For field definitions see Figure 2 & Figure 3. | - DATA frame : Indicates Actual Adaptive Loading being used for every subgroup on each layer. - ACK/CTS frame : Indicates Adaptive Loading Maximum rates to be used for every subgroup on each layer. |

Figure 2: 802.11n MIMO Type 1 frame showing Sig2 and Sig3 fields.

Figure 3: 802.11n MIMO Type3 frame showing Sig, Sig2 and Sig3 symbol fields.

#### RATE dependent options

The additional modulation parameters dependent on data rate used shall be set according to Table 2.

Table 2: Additional Rate dependent parameters

| Rate (Mbits/s) | Modulation | Coding Rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) |
| --- | --- | --- | --- | --- | --- |
| 0 | N.A. | N.A. | 0 | 0 | 0 |
| 64 | 256-QAM | 2/3 | 8 | 384 | 256 |
| 72 | 256-QAM | 3/4 | 8 | 384 | 288 |

### PLCP preamble

The PLCP preamble fields used for synchronisation are based on those defined in the IEEE stds 802.11a-1999. The definitions are given in Table 3.

Table 3: PLCP preamble field definitions.

| PLCP preamble field | Definition | Used For |
| --- | --- | --- |
| STS1-4 | Same 11a STS on each antenna TX Power per antenna reduced proportional to Ntx Cyclic delay at a rate 1sample/antenna | - AGC - Timing detection - Coarse freq offset estimation |
| LTS1-4 | Same 11a LTS on each antenna TX Power per antenna reduced proportional to Ntx Cyclic delay at a rate 1sample/antenna | - Channel estimation - Fine frequency estimation |
| LTS1-4,a-d | Same 11a LTS phase loaded onto each antenna e.g. 2TX: LTS1a=11a, LTS2a=11a *exp(j180o), all others not required. 3TX: LTS1a=11a, LTS2a=11a *exp(j120o), LTS3a = 11a *exp(j240o). LTS1b=11a, LTS2b=11a *exp(j240o), LTS3b = 11a *exp(j120o), all others not required. 4TX: etc TX Power per antenna reduced proportional to Ntx Cyclic delay at a rate 1 sample/antenna | - Allows for MIMO channel estimation |

### Signal (Sig) Symbol

The Sig Symbol fields are defined according to the existing 802.11a standard, except for the rate and reserved fields. Table 4 shows the standard rate field according to the 802.11a standard.

Table 4: 802.11a Standard Rate field

| Rate (Mbps) | R0-R3 | Modulation, Code Rate |
| --- | --- | --- |
| 6 | 1101 | BPSK, ½ |
| 9 | 1111 | BPSK, ¾ |
| 12 | 0101 | QPSK, ½ |
| 18 | 0111 | QPSK, ¾ |
| 24 | 1001 | 16QAM, ½ |
| 36 | 1011 | 16QAM, ¾ |
| 48 | 0001 | 64QAM, 2/3 |
| 54 | 0011 | 64QAM, ¾ |

As R3 is always a one we can ensure backward compatibility by reducing the existing 802.11a Sig Symbol rate field to 3 bits (R0-R2) and include an aditional HT field that has two valid responses 10 and 11, see Figure 3. Note that this is equivalent to the current definition of the Sig Symbol, where bits R0-R3 are defined as the rate field and R4 is the reserved bit which is set to zero for SISO and one for MIMO.

#### Short Rate field (Short Rate)

The new Short Rate field is specified in Table 5 below.

Table 5: Contents of Short Rate field

| Short Rate (Mbits/s) | R0-R2 | Modulation, Code Rate |
| --- | --- | --- |
| 6 | 110 | BPSK, ½ |
| 9 | 111 | BPSK, ¾ |
| 12 | 010 | QPSK, ½ |
| 18 | 011 | QPSK, ¾ |
| 24 | 100 | 16QAM, ½ |
| 36 | 101 | 16QAM, ¾ |
| 48 | 000 | 64QAM, 2/3 |
| 54 | 001 | 64QAM, ¾ |

#### High Throughput field (HT)

The new High Throughput field is defined in Table 6 below.

Table 6: Contents of High Throughput field

|  | HT | Comment |
| --- | --- | --- |
| Sig-field according to 11a | 10 | 802.11a compliant |
| N.A | 01 |  |
| Type 3 MIMO Frame | 11 | Indicates a MIMO transmission instead of an 802.11a |
| N.A. | 00 | This combination will cause an invalid rate for 11a |

### Signal 2 (Sig2) Symbol

The Signal 2 symbol’s purpose is to specify the MIMO transmission mode that is to be employed, see Figure 2 and Figure 3. It contains information relating to;

- Adaptive Loading mode

- MIMO mode

- Indication if a Signal 3 symbol is appended

- Indication if CSI information is contained in Signal 3 symbol

- Indication if channel bonding is used

#### Adaptive Loading Mode field (AL)

The Adaptive Loading Mode field assists in determining the length of the Rev CSI and FWD Rate fields in the Sig3 symbol. The field is specified in Table 7 below.

Table 7: Contents of field

|  | AL | Comment |
| --- | --- | --- |
| No Adaptive Loading | 00 | Same rate for all layers |
| Layered Adaptive Loading | 01 | 1 rate per layer |
| Subgroup Adaptive Loading | 10 | 4 subgroups (rates) per layer |
| Reserved | 11 |  |

#### Channel State Information Indication field (CSI)

A zero in this field represents that the Channel State Information field in the Sig3 (Rev CSI) contains no information (length =0 in the present frame). When a RTS frame or a training frame is transmitted after a missed ACK there is no valid knowledge of the CSI length, thus any frame that is commencing or restarting a packet transmission should have this bit set to zero when the CSI is unknown. All Sig3 symbols thereafter will have a CSI length that can be calculated from the information in the AL and Mode fields of Sig2 in the opposite link direction. See Section 7.2.1 for the length calculation.

#### High Throughput field (HT)

The High throughput field is used in the Sig2 symbol  to indicate a Type 1 or Type 3 MIMO Frame. The definitions are according to Table 8 below.

Table 8: Contents of  High Throughput (HT) field

|  | HT | Comment |
| --- | --- | --- |
| N.A. | 10 |  |
| Type 1 MIMO Frame | 01 |  |
| Type 3 MIMO Frame | 11 |  |
| Reserved | 00 |  |

#### 802.11n Transmission Mode field (Mode)

The Mode field of Sig2 specifies the transmitter architecture configuration. The definition is given in Table 9 below.

Table 9: Contents of Mode  field

|  | Mode | Nlayers | Comment |
| --- | --- | --- | --- |
| Reserved | 0000 |  |  |
| 802.11n frames | 0001 | 1 | Type 1,2 & 3 MIMO frames |
| 802.11n frames with Parallel SMX Architecture | 0010 | 2 | Type 1,2 & 3 MIMO frames |
| 802.11n frames with Parallel SMX Architecture | 0011 | 3 | Type 1,2 & 3 MIMO frames |
| 802.11n frames with Parallel SMX Architecture | 0100 | 4 | Type 1,2 & 3 MIMO frames |
| Reserved | 0100- 1111 |  |  |

The difference between mode 1 above and 11a is longer packets, less preamble overhead, shorter SIFS, frame aggregation, and faster adaptation to best rate.

#### Sig3 Symbol On field (Sig3On)

A one represents that the Sig3 symbol is present. A zero represents that the transmission ends after the training sequence (no Sig3 or Data Symbols appended). This mode is used for obtaining CSI at the receiver, which is then immediately transmitted back in the acknowledgement, so that in the next transmission the layers can be loaded in an optimal way.

#### Sig3 Symbol Rate field (Sig3R)

R0-R3 define the rate for the sig3 symbol in the same way as is done for the existing 802.11a standard. The definitions are given in Table 10.  Note however this information is only valid if the Sig3on field is set.

Table 10: Contents of Sig3R field

| Rate (Mbits/s) | R0-R3 | Modulation, Code Rate |
| --- | --- | --- |
| 6 | 1101 | BPSK, ½ |
| 9 | 1111 | BPSK, ¾ |
| 12 | 0101 | QPSK, ½ |
| 18 | 0111 | QPSK, ¾ |
| 24 | 1001 | 16QAM, ½ |
| 36 | 1011 | 16QAM, ¾ |
| 48 | 0001 | 64QAM, 2/3 |
| 54 | 0011 | 64QAM, ¾ |

#### Channel Bonding field (CB)

The CB field provides a mechanism for signaling if channel bonding is used or not. The definitions are given in Table 11.

Table 11: Contents of  CB field

|  | CB |
| --- | --- |
| No Channel Bonding | 00 |
| Channel Bonding with Upper Adjacent channel | 01 |
| Channel Bonding with Lower Adjacent channel | 10 |
| Reserved | 11 |

#### Reserved (Res), Parity (P) and Tail (Tail) fields

One reserved bit currently exists. Parity is defined as the Even parity for bits 0-16 in Sig2 Symbol for a Type 3 & 1 MIMO frame. The 6 Tail bits are all set to zero.

### Signal 3 (Sig3) Symbols

The Signal 3 symbol’s purpose is to support the use of adaptive loading, and includes the length of the MIMO data to be transmitted, see Figure 2 and Figure 3. It contains information relating to;

- Reverse Link Channel State Information

- Data rate used in transmitting following data symbols

- Request for retraining

- Data length of following data transmission

#### Reverse Link Channel State Information field (Rev CSI)

The receiver transmits the Channel State Information to the transmitter. The length of this field can be determined from the AL and Mode fields of the sig2 symbol in the opposite link direction or is set to zero by the CSI bit in the sig2 field of the current packet. The information can take different forms. For example;

- Transmit which rate should be used on the next transmission of data symbols. Obviously anything less can be used by the transmitter.

- Transmit the margin to indicate to the transmitter that it can increase or decrease the rate. Instead of repeating the same rate as in previous packet, a margin code is transmitted (optional)

An example of the number of bits needed here is equal to the Number of Data Layers * Number of Rates per Layer). For example:

- Maximum: No of layers = 4, No of rates per layer = 4, 4bits per rate => 4*4*(4) = 64bits

- Minimum: No of layers = 1, No of rates per layer = 1, 4bits per rate => 1*1*(4) = 4bits

The rate definitions are according to Table 12 below.

Table 12: Contents of RevCSI field

| Rate (Mbits/s) | R0-R3 | Modulation, Code Rate |
| --- | --- | --- |
| 6 | 1101 | BPSK, ½ |
| 9 | 1111 | BPSK, ¾ |
| 12 | 0101 | QPSK, ½ |
| 18 | 0111 | QPSK, ¾ |
| 24 | 1001 | 16QAM, ½ |
| 36 | 1011 | 16QAM, ¾ |
| 48 | 0001 | 64QAM, 2/3 |
| 54 | 0011 | 64QAM, ¾ |
| 0 | 0000 | none |
| 64 | 1010 | 256QAM, 2/3 |
| 72 | 1100 | 256QAM, ¾ |

#### Forward link rate field (FWD Rate)

This field specifies the data rate used in transmitting the following data symbols. The length of this field can be determined from the AL and Mode fields of the sig2 symbol. The rates are as defined in the previous section.

No of bits needed here is equal to the Number of Data Layers * Number of Rates per Layer * Number of bits per Rate. For example:

- Maximum: No of layers = 4, No of rates per layer = 4 &4bits per rate => 4*4*4 = 64bits

- Minimum: No of layers = 1, No of rates per layer = 1, 4bits per rate => 1*1*4 = 4bits

#### Request for Training MIMO Type 1 Frame field (Req)

This field is set in MIMO Type2 frames to indicate that the next frame should be a MIMO Type 1 frame, which includes a training sequence.

#### MIMO 802.11n data length field (11nLength)

Unsigned 14 bit integer indicates the numbers of octets in the data symbols following the Sig3 Symbol. Length of zero indicates that there will not be any data symbols following.

#### Pad with Zeros field (Pad)

The number of bits here are used for padding to ensure that the symbol is filled. See Appendix B, for an example on length calculation.

#### Reserved (Res), Parity (P) and Tail (Tail) fields

The reserved bits are for future use.

Parity is defined as the even parity for bits 0-X in Sig3 Symbol, where X is the length of the Sig3 field.

The 6 Tail bits are all set to zero.

### Data Symbols

The only modification to IEEE stds 802.11a-1999 is that subcarrier modulation mapping for 256-QAM and a zero rate option has been added. The complete table is shown below.

Table 13: Rate-dependent parameters for 802.11n

| Data rate (Mbit/s) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) |
| --- | --- | --- | --- | --- | --- |
| 0 | N.A. | N.A. | 0 | 0 | 0 |
| 6 | BPSK | 1/2 | 1 | 48 | 24 |
| 9 | BPSK | 3/4 | 1 | 48 | 36 |
| 12 | QPSK | 1/2 | 2 | 96 | 48 |
| 18 | QPSK | 3/4 | 2 | 96 | 72 |
| 24 | 16-QAM | 1/2 | 4 | 192 | 96 |
| 36 | 16-QAM | 3/4 | 4 | 192 | 144 |
| 48 | 64-QAM | 2/3 | 6 | 288 | 192 |
| 54 | 64-QAM | 3/4 | 6 | 288 | 216 |
| 64 | 256-QAM | 2/3 | 8 | 384 | 256 |
| 72 | 256-QAM | 3/4 | 8 | 384 | 288 |

#### Subcarrier Modulation Mapping Additions

An additional mapping constellation for 256-QAM has been added. The conversion of the 8 bits to the constellation points shall be performed according to the Gray-coded constellation mapping, according to Table 14 below. The scaling factor, kMOD, is 1/sqrt(170).

Table 14: 256-QAM encoding table

| Input bits (b0b1b2b3) | I-out |  | Input bits (b4b5b6b7) | Q-out |
| --- | --- | --- | --- | --- |
| 0000 | -15 |  | 0000 | -15 |
| 0001 | -13 |  | 0001 | -13 |
| 0011 | -11 |  | 0011 | -11 |
| 0010 | -9 |  | 0010 | -9 |
| 0110 | -7 |  | 0110 | -7 |
| 0111 | -5 |  | 0111 | -5 |
| 0101 | -3 |  | 0101 | -3 |
| 0100 | -1 |  | 0100 | -1 |
| 1100 | 1 |  | 1100 | 1 |
| 1101 | 3 |  | 1101 | 3 |
| 1111 | 5 |  | 1111 | 5 |
| 1110 | 7 |  | 1110 | 7 |
| 1010 | 9 |  | 1010 | 9 |
| 1011 | 11 |  | 1011 | 11 |
| 1001 | 13 |  | 1001 | 13 |
| 1000 | 15 |  | 1000 | 15 |

### PMD operating specifications

#### Parallel Spatial Multiplexing Transmitter Architecture

Figure 4 illustrates the scalable transmitter architecture, that in its single antenna configuration is identical to an 802.11a/g transmitter. The modifications required for 802.11n mode are;

- The scalable architecture supports up to 3 (mandatory) or 4 (optional) antennas (although more are possible)

- The adaptive loading modifies the puncturing and Constellation Mapping on a layer basis, see Section 2.2.5.1. The STA determine the maximum rate per layer (mandatory) or subgroup of carriers (optional) and this is communicated back to the AP, and vice-versa.

- The mapping function has an expanded rate set, see Section 2.2.6.1.

- Cyclic Delay is implemented with a progressive 1 sample delay /per antenna, see Section 7.1.

- The LTS preambles are modified versions of the 802.11a/g defined sequences, see Section 2.2.2.

Demux

Data Bits

Scramble

Encode

Encode

Encode

Encode

Punct

Punct

Punct

Punct

Inter.

Map

Inter.

Inter.

Inter.

FFT

CP

Cyclic Delay

To DACs

Map

FFT

CP

Cyclic Delay

Map

FFT

CP

Cyclic Delay

Map

FFT

CP

Cyclic Delay

Adaptive Loading Info from Sig3 Symbol ‘Rev CSI’ field

Mux

STS and LTS Preambles

Mux

Pilots

Data Bits

Cyclic Delay=0

Adaptive Loading Info from Sig3 Symbol ‘Rev CSI’ field

Demux

Scramble

Encode

Encode

Encode

Encode

Punct

Punct

Punct

Punct

Inter.

Map

Inter.

Inter.

Inter.

FFT

CP

Cyclic Delay=3

To Antennas

Map

FFT

CP

Cyclic Delay=2

Map

FFT

CP

Cyclic Delay=1

Map

FFT

CP

Adaptive Loading Info from Sig3 Symbol ‘CSI’ field

Mux

STS and LTS Preambles

Mux

Pilots

Figure 4: Digital Transmitter functional block diagram

#### Adaptive Loading Overview

The Adaptive Loading technique (or closed loop rate adaptation) is based on the receiver determining the maximum rate on a per layer (mandatory) or subgroup (optional) basis that can be supported on the Forward Link. This rate recommendation is based on SNR estimates computed in the channel estimation algorithm. These rates are then sent back to the transmitter on the Reverse Link in the sig3 Symbol. This is illustrated in Figure 5, and typical frame transmision sequences are shown in Section 2.3.1.2.

Punct/ Map

Data Bits

Tx

Channel Estimation

Rx

Data Bits

Forward Link

SNR (Link Margin/layer)

Calculate Maximum Rate Possible on a per layer basis

Decode Sig3 Symbol ‘Rev CSI’ field

Reverse Link

Encode Sig3 Symbol ‘Rev CSI’ field

Figure 5: Adaptive Loading functional block diagram

## MIMO-OFDM-AdaptLoad Frame Transmission Modes

### 5 GHz Band Proposal

#### Single Packet Transmission Mode

This mode will allow 100Mbps throughput at the MAC layer with frame aggregation, see Section 7.3 and it allows faster transmission than SISO transmission once the Data payload of the frame exceeds 400 bytes. It only employs Type 3 MIMO frames.

- The duration field in the 802.11a compatible part sets a dummy duration, that a legacy device would interpret and set the NAV accordingly.

- The Acknowledgement is transmitted in SISO mode

- Capabilities of Destination is known at Source and vice-versa (how many antennas, which modes supported etc.)

- SIFS can take a value between 8-16us, meaning that the receiver must be ready to receive after 8us but a transmitter is allowed to wait up to 16us before starting.

D1

D2

Sig

LTS3

STS3

D1

D2

Sig

LTS4

STS4

Sig2

LTS4a

LTS4b

LTS4c

Dn

Sig2

LTS3a

LTS3b

LTS3c

Dn

Sig2

LTS2a

LTS2b

LTS2c

Dn

Sig3

D1

D2

Sig3

D1

D2

Sig33

D1

D2

D1

D2

Sig

Sig2

LTS1a

LTS1b

LTS1c

Dn

D1

D2

Sig

LTS1

STS1

Sig3

D1

D2

LTS

Data – Type 3

ACK – 802.11a

11a compatible

STS

D1

D2

Sig

LTS2

STS2

11a compatible

MIMO Specific

Figure 6: MIMO frame sequence for 5GHz band 11a compatible transmission.

#### RTS/CTS Transmission Mode

Figure 7 illustrates the RTS/CTS transmission mode which in this example is 4*4 MIMO mode on the downlink with a 2*2 MIMO mode on the uplink. This mode employs all 3 types of MIMO frames. The characteristics of the RTS/CTS mode are;

- RTS/CTS are 802.11a compatible

- The duration field in the RTS/CTS would be set to reserve the medium for a specified time, up to a maximum of 32ms (8125 symbol periods, at 3*36Mbps = 108Mbps data rate => 438kbytes of data can be transferred during this time).

- Extending the reservation of the medium (duration field) can be accomplished by using the protection mechanism stated in the 802.11g standard, Section 9.10 ‘Protection mechanism’, by sending a ‘CTS to self’.

- MIMO channel estimation occurs in RTS so that adaptive loading information can be returned in CTS.

- Type 1 MIMO frames can be sent on request or by own initiation. These frames should always reset synchronisation:  Rules (A is the data source and initiator, whilst  B is the data sink)

    - A and B are always prepared to receive STS/LTS

    - The transmitter is always synchronous to last STS/LTS

    - When time shift between A and B is too large, B transmits a STS/LTS

    - If A transmits STS/LTS, B always answers with a STS/LTS to reset timing

- A missed ACK results in a retransmission. This retransmission is sent using a Type 1 MIMO frame and transmits the data at a backed-off rate.

- Capabilities of Destination is known at Source and vice-versa (how many antennas, which modes supported etc)

- SIFS can take a value between 8-16us, meaning that the receiver must be ready to receive after 8us but a transmitter is allowed to wait up to 16us before starting.

CTS – Type 3 with Data Off

Type 2 -Frame0

ACK0 – Type 2

D1

D2

Dn

Sig3

D1

D2

Dn

Sig3

D1

D2

Dn

Sig3

D1

D2

Dn

Sig3

D1

Sig3

D1

Sig3

11a compatible

MIMO Specific

11a compatible

Medium reserved for MIMO Specific operation.

Duration indicated in RTS

Duration indicated in CTS

Type 1 - Frame n

ACK n

ACK n-1 – Request Training Sequence

D1

Sig3

D1

Sig3

Medium reserved for MIMO Specific operation.

Duration indicated in RTS

Duration indicated in CTS

Medium reserved for MIMO Specific operation.

Duration indicated in RTS

Duration indicated in CTS

Type 3: CTS to self

Type 3 -ACK

Extend Duration

Extended Duration

Extended Duration

11a compatible

11a compatible

Note: Training can also be initiated by the sender e.g. if missed ACK

STS1

LTS1

Sig2

LTS1a

LTS1b

LTS1c

STS2

LTS2

Sig2

LTS2a

LTS2b

LTS2c

STS3

LTS3

Sig2

LTS3a

LTS3b

LTS3c

STS4

LTS4

Sig2

LTS4a

LTS4b

LTS4c

Sig3

Sig3

Sig3

Sig3

D1

D1

D1

D1

D2

D2

D2

D2

Dn

Dn

Dn

Dn

Sig2

LTS1a

LTS1b

LTS1c

Sig2

LTS2a

LTS2b

LTS2c

Sig2

LTS3a

LTS3b

LTS3c

Sig2

LTS4a

LTS4b

LTS4c

D1

D1

D1

D1

D2

D2

D2

D2

STS1

LTS1

Sig

STS2

LTS2

Sig

STS3

LTS3

Sig

STS4

LTS4

Sig

Sig2

LTS1a

Sig2

LTS2a

Sig3

Sig3

D1

D1

D2

D2

STS1

LTS1

Sig

STS2

LTS2

Sig

Sig2

LTS1a

Sig2

LTS2a

Sig3

Sig3

D1

D1

D2

D2

STS1

LTS1

Sig

STS2

LTS2

Sig

Sig2

LTS1a

LTS1b

LTS1c

Sig2

LTS2a

LTS2b

LTS2c

Sig2

LTS3a

LTS3b

LTS3c

Sig2

LTS4a

LTS4b

LTS4c

Sig3

Sig3

Sig3

Sig3

D1

D1

D1

D1

D2

D2

D2

D2

STS1

LTS1

Sig

STS2

LTS2

Sig

STS3

LTS3

Sig

STS4

LTS4

Sig

STS1

LTS1

Sig2

LTS1a

STS2

LTS2

Sig2

LTS2a

Sig3

Sig3

D1

D1

Timing Reset due to STS transmission

RTS – Type 3 with Sig3 & Data Off

MIMO Specific

Figure 7: MIMO transmission frame sequence for 5GHz band 11a compatible operation.

### 2.4 GHz Band Proposal

#### Single Packet Transmission Mode

Given the long preambles (192us or 96us) necessary for this band to ensure backward compatibility, single packet transmission in a MIMO mode is not considered a sensible solution.

#### RTS/CTS Transmission Mode

This mode employs 2 types of MIMO frames. It should be noted that the throughput in the 2.4GHz band will not be as high as that in the 5GHz band due to the longer preamble. The characteristics of the mode are;

- RTS/CTS are 802.11g compatible and are transmitted from one antenna with DSSS modulation.

- No mixing of DSSS and MIMO OFDM modulations in same frame.

- The duration field in the RTS/CTS would be set to reserve the medium for a specified time, up to a maximum of 32ms (8125 symbol periods, at 3*36Mbps = 108Mbps data rate => 438kbytes of data can be transferred during this time).

- Extending the duration field can be accomplished by using the protection mechanism stated in the 802.11g standard, Section 9.10, ‘Protection mechanism’, by sending a ‘CTS to self’.

- MIMO channel estimation occurs in first transmission after the RTS so that the adaptive loading information can be returned in the ACK.

- Type 1 MIMO frames can be sent on request or by own initiation. These frames should always reset synchronization:  Rules (A is the data source and initiator, whilst  B is data sink)

  - A and B are always prepared to receive STS/LTS

  - The transmitter is always synchronous to last STS/LTS

  - When time shift between A and B is too large, B transmits a STS/LTS

  - If A transmits STS/LTS, B always answers with a STS/LTS to reset timing

- A missed ACK results in a retransmission. This retransmission is sent using a Type 1 MIMO frame and transmits the data at a backed off rate.

- Capabilities of Destination is known at Source and vice-versa (how many antennas, which modes supported etc)

- SIFS can take a value between 8-16us, meaning that the receiver must be ready to receive after 8us but a transmitter is allowed to wait up to 16us before starting.

Figure 8: MIMO transmission frame sequence for 2.4GHz Band for .11g compatible operation.

### Implementation Details of the Proposed Frame Format

The key to high throughput in the RTS/CTS frame transmission  mode is the use of rate feedback for good MIMO performance and the reduction of overhead. For a forward (reverse) transmission, synchronisation and channel training data for the whole packet is sent initially by the AP (STA).  This is attached to the RTS frame using the MIMO type 3 frame, and allows the receiving STA (AP) to estimate the channel and return the bit-loading profile to the AP (STA) as part of the CTS frame payload.  The data frame is then sent from the AP (STA) without any further STS and LTS training sequences.  Synchronisation and training is therefore based on the previous RTS transmission.

For the proposed frame format scheme to be successful, the receiving STA (AP) must therefore be able to fly-wheel over the non-reception period between the end of the RTS and the start of the next data frame. A similar fly-wheel requirement is necessary between two data frames when ACKs are being transmitted.  This is possible since the defined 802.11n channels are slowly changing.

#### 802.11n Channel Environment

Figure 9 illustrates the channel changing in time for three channel models with fd=6Hz; (channel B, channel E +fluorescent effects, and channel F +the 40kph high speed Doppler component).  The MSE between the current channel and the earlier estimate of the channel is plotted against the time difference.  A MSE of less than -35dB (reasonable assumption for implementation) is possible for time delays up to 0.2ms (50 OFDM symbol periods) for channels including Channel F.  Relaxing the MSE to -25dB increases the delay to 0.8ms (200 OFDM symbol periods).  These times are doubled for the channels without the high speed Doppler component.  Since the time for the CTS or ACK frames is typically less than 20 symbols, the effect of channel variations will not affect performance over this period.

0

0.2

0.4

0.6

0.8

1

10

-3

-70

-60

-50

-40

-30

-20

-10

MSE(t) dB  fd=6Hz, b-chF(+hs doppler) , r-chE(+Fluoro), g-chB,

Time

Figure 9 : Channel variation with time. The change in channel is shown as a MSE. All channels have a Doppler of 6Hz (green). 6Hz Doppler + fluorescent light effect (red); 6Hz Doppler + high Doppler component (blue).

The frame format proposal still does raise a number of other technical issues.

- Channel Tracking is required in this proposal for multi-frame transmission. Decision directed or other  tracking schemes will follow the slow variations in the channel and remove the need for any further channel training.

- Timing and Synchronisation. The first STS and LTS sets up the timing grid for the rest of the forward (reverse) transmission.  Packets must always be transmitted on a symbol boundary.  The receiver therefore knows that a transmission will start on a timing grid quantized to 4us (one symbol period).  When there is significant path delay, the transmission will be delayed to the next symbol timing boundary. The receiver must detect the presence of the first symbol of the frame using power or other means.

The example below illustrates some of the important features. 1) The A party initiates an RTS.  2) The B party sets up a timing grid based on the A party STS/LTS header. 3) The B party replies some 8s later with a CTS frame. 4) The A party sets up its receive timing grid based on the STS/LTS of the CTS frame.  The first Data frame from the A party is transmitted some 8s + Tus later. This enables the data frame to have the same timing grid of the RTS frame and alleviate the need for additional channel and timing preambles.  5) The B party receiver can expect the first data frame to arrive 8s after the CTS transmission if the transmission delay (time of flight) is zero. Generally this is not the case, and so the data frame will arrive an integer number of symbols later depending on the transmission delay. Note: The A party can use the time shift between the Rx and Tx grids to estimate the distance to the B party i.e. location services.

STS

LTS

RTS

CTS

Data

ACK

Data2

B Party

A Party

A Party Tx Timing Grid

A Party Rx Timing Grid

B Party Rx & Tx Timing Grid

STS

LTS

Tgrid

Data

Figure 10: An example of timing in the RTS/CTS mode

- Frequency offset and timing slip must be corrected and fly-wheel over non transmission periods. The 4 pilot tones can aid in this respect.

Timing slip effects the outside sub-carriers most.  The phase shift k in bin k is given by:

ppm = effective timing slip, L is CP length (16 samples), N=fft size(64), S=symbol number.

The effective timing slip can be reduced using the frequency offset estimation. For a frequency offset error of 1% sub carrier spacing, (3.1kHz) the effective ppm  becomes  0.56ppm (= 40*3.1/220).  The number of symbols before the MSE (in bins 26) grows beyond the -35dB or -25dB becomes S @-35dB  = 155 symbols or S @-25dB  = 489 symbols. This indicates the fly-wheel limit before corrective action should be taken (using the four pilot tones for example).

The proposal also includes the possibility of using the same structure in the reverse (forward) link with timing based on the first STS and LTS of the CTS transmission.  This would eliminate the preamble length on subsequent ACKs and BlockACKs.   Maintaining tracking on the reverse (forward) link at first sight appears difficult because of the long fly-wheel times and short ACK frames (as low as 2 symbols) that limit the training possibilities.  However the reverse link is operated with robust modulation (BPSK, QPSK) and can tolerate much higher MSE’s.  If it is operated in SISO (suggested) with delay diversity, channel tracking becomes easier and quicker.  An alternate solution is to operate it in SISO with preamble. SISO preambles are much shorter than MIMO preambles.

#### Tracking Algorithm Simulations

To verify that the tracking of time, frequency and the channel is possible in the presence of impairments IM1-6 with the current frame format proposal, two simulations have been completed :

- Forward Link

- Reverse Link

The parameters for  the simulation are illustrated in Table 15, one should also reference Figure 7. Since the EVM is plotted against time the phase noise and channel noise is turned off so that the tracking characteristic of the algorithms can be observed.

Table 15: Parameters for Tracking Algorithm Simulations

| Configuration | 2x2 (forward and reverse links) |
| --- | --- |
| Frequency Tolerance | 20 ppm |
| RTS length –Type 3 | 10 symbols |
| CTS length –Type 3 | 11 symbols |
| DATA length (64QAM) | 23 symbols |
| ACK length (BPSK) | 4 symbols |
| SIFS | 4 symbols (16us) |
| Channel Model | E (1.2km/h) |
| Channel tracking | On (dotted) or Off (continuous) |
| Fine Freq/Time tracking | On (dotted) or Off (continuous) |

Figure 11(a) shows the simulated EVM (MSE) error of the forward path.  The EVM error is measured after the Rx signal is equalised with the channel inverse (H-1).  Impairments include frequency and timing offset (20ppm). Fine frequency offset and sample time offset corrections are included in the channel tracking algorithm. The measurement starts at the first data frame after the RTS and CTS frame transmissions.  The discontinuities represent the ACK positions.  The curve with channel tracking off is also shown.  Key features of the plot show the reduction in MSE by the tracking algorithm at the start of the transmission.  The -35dB floor of the MSE is dominated by ISI from channel E and the system filters.  Tracking stops during the ACK transmission and this is shown by the MSE increase at the start of the next data frame.

Figure 11(b) shows the EVM (MSE) plot for the reverse link transmission when channel tracking is on /off.  The longer free-wheel period and reduced ACK frame length reduce convergence performance by approximately 5dB.  The EVM is only maintained to better than -25dB. The usable data rate on the reverse link is less than that of the forward link.

Figure 11 (a) EVM vs time plot for Forward Link (b) EVM vs time plot for Reverse Link

# Addressing the Functional Requirements

| Number | Name | Requirement | Coverage (Yes/No) | Results Reference |
| --- | --- | --- | --- | --- |
| 1 | Single Link HT rate supported | Demonstrate at least one set of conditions under which 100 Mbps at the top of the MAC SAP can be achieved. Provide all relevant information to document this. | Yes | See Sections 5.5.3.1, 5.5.4.1 & 5.5.4.2 |
| 2 | HT rate supported in 20MHz channel | Proposal supports at least one mode of operation that supports 100Mbps throughput at the top of the MAC SAP in a 20MHz channel. Provide all relevant information to document this. | Yes | See Sections 5.5.3.1, 5.5.4.1 & 5.5.4.2 |
| 3 | Supports 5GHz bands | Protocol supports 5GHz bands (including those supported by .11a) | Yes | See Section 5.1.1 |
| 4 | .11a backwards compatibility | Some of the modes of operation defined in the proposal shall be backwards compatible with 802.11a. | Yes | See Section 2.3.1 |
| 5 | .11g backwards compatibility | If it supports 2.4 GHz operation, some of the modes of operation defined in the proposal shall be backwards compatible with 802.11g. | Yes | See Section 2.3.2 |
| 6 | Control of support for legacy STA from .11n AP | A .11n AP can be configured to reject or accept associations from legacy STA because they are legacy STA. | Yes | No details. |
| 7 | .11e QoS support | The proposal shall permit implementation of the 802.11e ammendment within a .11n STA | Yes | Single or RTS/CTS frame transmision mode can operate within a TXOP. |
| 8 | Spectral Efficiency | The highest throughput mode of the proposal shall achieve a spectral efficiency of at least 3 bps/Hz for the PSDU | Yes | See Section 5.5.3.1. |
| 9 | Compliance to PAR | The proposal complies with all the mandatory requirements of the PAR [6] and 5 Criteria [7] | Yes |  |

# Addressing Section 3 of the Comparision Criteria

Sections 4.1 to 4.5 address the additional disclosures as specified by section 3 of 11-03-0814-30-000n.

## AD1 – Reference Submissions

- ‘11-04-0020-00-000n-low-pilot-overhead.ppt’, Igor Tolochko & Mike Faulkner , Jan 2004.

- ’11-04-xxxx-00-000n-Eriksson-et-al-proposal.ppt’,

## AD2 – TCP Model Parameters

Not Applicable.

## AD3 – MAC Simulation Methodology

Not Applicable.

## AD4 – MAC Simulation occupied channel width

Not Applicable.

## AD5 – Justification of low packet loss rate (PLR) rates achieved

Not Applicable.

# Addressing Section 4 of the Comparision Criteria

## General

### CC2 - Regulatory Compliance

The proposal has no known problems with regulatory compliance in the USA, Japan, Europe, or China.

## Marketability

### CC3 - List of goodput results for usage models 1,4 and 6.

Not Applicable.

### CC6 – PHY complexity

Most baseband processing blocks (e.g. filter, FFT, frequency correction, mapping, demapping) will have a configuration as in 11a. However calculations will be done N times in an N channel system. Thus, assuming N times higher bit-rate, energy per bit for these parts remain constant compared to 11a. This is also true for the Viterbi decoder.

The computational complexity in the channel correction unit grows faster than throughput for MIMO, but its computational complexity is about the same as for FFTs, even for a 3*3 MIMO system.

The calculation needed for generating channel correction coefficients increases rapidly with the number of channels. The increased length of payload, and transmission of frames without preambles keep the power cost for this operation at a reasonable level. The placement of the Sig3 symbol after the last preamble increases available time for computation with one symbol period.  This reduces the required complexity of the logic for this function.

In total the digital baseband computational complexity for a 3-layer systems is roughly 4 times that of an 11a system.

The analog front-end grows slower in area and power consumption as some units are reused for all channels. For a 3-layer system the area and power is roughly 2.5 times that of an 11a analog front-end.

### CC7 – MAC processing complexity

Not applicable.

## Backward Compatibility and Coexistence with Legacy Devices

### CC11 – Backward compatability with 802.11-1999 (Rev 2003) and 802.11g

Backward Compatability is ensured by

- Operation within a 20MHz bandwidth with the same 802.11a/g spectral mask.

- Single and RTS/CTS frame transmission modes are fully compatible with legacy 802.11a/g devices, see Section 2.3.

### CC15 – Sharing of medium with legacy devices

Not Applicable.

## MAC Related

Not applicable except for CC46 and CC47.

### CC46 – MAC Compatibility and Parameters

The SIFS parameter is proposed to take a value between 8-16us, meaning that the receiver must be ready to receive after 8us but a transmitter is allowed to wait up to 16us before starting.

### CC47 – MAC extensions

In order to ensure the best chance of >100Mbps data throughput at the MAC SAP, then it would be beneficial to provide for frame aggregation such that the MAC efficiency is improved, see Section 7.3. Up to a maximum of 16kbytes is proposed, see Section 2.2.6.4.

## PHY related

### PHY Rates and Preambles

#### CC51 - Data Rates

The PHY data rates allowed on a per layer or subgroup basis are defined in Section 2.2.6.1. They can be summarised as:

- 1 to 3 antennas (Mandatory), 4 antennas (optional)

- Bandwidth = 20MHz (Mandatory), 40MHz (Optional)

Table 16: Mandatory Rates

| Rate (Mbits/s) | Modulation, Code Rate |
| --- | --- |
| 6 | BPSK, ½ |
| 9 | BPSK, ¾ |
| 12 | QPSK, ½ |
| 18 | QPSK, ¾ |
| 24 | 16QAM, ½ |
| 36 | 16QAM, ¾ |
| 48 | 64QAM, 2/3 |
| 54 | 64QAM, ¾ |
| 0 | none |
| 64 | 256QAM, 2/3 |
| 72 | 256QAM, ¾ |

- Adaptive rate  can vary from 0Mbit/s through to Ntx*72Mbits/s (216Mbits/s for 3 transmit antennas )

#### CC42 – Preambles

The preambles are defined in Section . The short and long training sequences are the same as the 802.11a/g defined training sequences with the following modifications:

- Both the STS and LTS sequences have a progressive cyclic delay of 1 sample per antenna applied, see also Appendix A, Section 7.1

- The LTS sequences are also phase loaded on a per antenna basis.

The STS characteristics are summarised in Table 17. In terms of the auto-correlation and cross-correlation the mean sidelobe ratio has a small discrimination ratio with larger number of antennas.

Table 17 : STS Characteristics

|  | 1x1 | 2x1 | 3x1 | 4x1 |
| --- | --- | --- | --- | --- |
| Auto Correlation – mean side lobe (linear) / std of sidelobe (linear) | 0.125 / 0.085 | 0.145 / 0.007 | 0.2416 / 0.023 | 0.3412 / 0.016 |
| Cross Correlation – mean side lobe (linear) / std of sidelobe (linear) | 0.125 / 0.085 | 0.143 / 0.044 | 0.179 / 0.078 | 0.327 / 0.09 |
| Peak to Average Power Ratio/antenna (dB) | 2.1 | 2.1 | 2.1 | 2.1 |

The LTS characteristics summarised in Table 18. The effect on the Long Training Sequences (LTS) is similar to STS, except that the longer waveform of 64 leads to a much lower sidelobe value. The shape of the peaks is unaffected (same as for STS).

Table 18 : LTS Characteristics

|  | 1x1 | 2x1 | 3x1 | 4x1 |
| --- | --- | --- | --- | --- |
| Auto Correlation – mean side lobe (linear) / std of sidelobe (linear) | 0.0415 / 0.0444 | 0.0328 / 0.0038 | 0.0522 / 0.0127 | 0.068 / 0.0108 |
| Cross Correlation – mean side lobe (linear) / std of sidelobe (linear) | 0.0415 / 0.0444 | 0.0342 / 0.0124 | 0.0472 / 0.0188 | 0.0675 / 0.0225 |
| Peak to Average Power Ratio/antenna (dB) | 3.2 | 3.2 | 3.2 | 3.2 |

### Channelisation

#### CC51.5 - Channelisation

The PHY uses the 2.4 GHz frequency band (optional) and channelisation plan specified in 18.4.6 and the 5 GHz frequency band and channelisation plan specified in 17.3.8. of IEEE Std 802.11, 1999.

#### CC52 – Spectral Mask

The transmit spectral mask for the PHY operating in the 2.4 GHz & 5GHz band with OFDM modulation is unchanged as illustrated in Figure 12.

Figure 12: Transmit Spectrum mask and a typical Signal Spectrum modulated with  256QAM with 8dB back-off in the PA

### Spectral Efficiency

#### CC58 – HT spectral Efficiency

Table 19 summarises the results for CC58. The RTS/CTS frame transmission mode achieves a goodput of more than 100Mbps, requiring at least 144Mbps average PHY data rate. That is, the MAC efficiency is  approximately 70%.

The single frame transmission mode achieves a maximum goodput of 80Mbps when the average PHY data rate is 288Mbps. The goodput can be increased by at least two known techniques :

- With frame aggregation a 5.5kbyte packet size transmitted at a average PHY data rate of 144Mbps can achieve a goodput of 100Mbps, see Appendix C.

- With channel bonding (optional) the average PHY data rate is increased by a factor 1.8 which would achieve more than 100 Mbps at the MAC SAP, see Appendix E.

Table 19: CC58 Summary

| Configuration |  | Average PHY Data rate to achieve Goodput >100Mbps | bps/Hz |
| --- | --- | --- | --- |
| 2*2 MIMO, Channel B | Single Frame Mode | N.A. | N.A. |
|  | RTS/CTS Mode | 144 Mbps | 7.2 |
| 3*3 MIMO, Channel B | Single Frame Mode | N.A. | N.A. |
|  | RTS/CTS Mode | 144-216Mbps | 7.2-10.8 |
| 4*4 MIMO, Channel B | Single Frame Mode | N.A. | N.A. |
|  | RTS/CTS Mode | 144-288Mbps | 7.2-14.4 |
| 3*3 MIMO, Channel D | Single Frame Mode | N.A. | N.A. |
|  | RTS/CTS Mode | 144-216Mbps | 7.2-10.8 |
| 4*4 MIMO, Channel D | Single Frame Mode | N.A. | N.A. |
|  | RTS/CTS Mode | 144-288Mbps | 7.2-14.4 |

Table 20: CC58 Simulation Conditions

| Parameter | Parameter value | Comment |
| --- | --- | --- |
| Simulation length | Max. 6000 packets, Min 300. | Vary number depending on SNR. PER to 10-2 |
| Packet Payload | 1500 bytes |  |
| TX architecture | Parallel |  |
| Shaping Filters | Transmitter - Interpolation by 5 then 71 tap RRC filter, a=0.2 Receiver - 71 tap RRC filter, a=0.2 followed by decimate by 5 |  |
| TX/RX antennas | 2/2, 3/3, 4/4(optional) |  |
| Rate/layer | Average rate as shown on plots |  |
| Interleaver | 802.11a |  |
| Channel Model | B, D (with fluorescent effect on) | @ 100MHz |
| Shadowing variance | 0 |  |
| Time & Freq. Synchronization | Ideal |  |
| Channel Estimation | Ideal |  |
| IM1 - PA Model | On | PA backoff at 10dB |
| IM2 – Carrier frequency offset | 0Hz |  |
| IM4 – Phase noise | Off |  |
| IM5 – Noise Figure | 10dB |  |
| IM6 – Antenna Config. | ½ at Tx and Rx |  |
| TX-RX Spacing | 10m |  |

Figure 13: CC58  2*2 MIMO, Channel B

Each curve in the above figures represents an average PHY goodput achieved when adaptive loading is enabled. That is the data rate of each of the two layers varies independently on a packet by packet basis, whilst the sum of the data rate for the two layers is constrained to be constant for each packet.

Please refer to Appendix D, Section 7.4, for a description of the calculation process, which is indicated by the flow of arrows in the above figures.

Figure 14: CC58 3*3 MIMO, Channel B

Figure 15: CC58 4*4 MIMO, Channel B (optional mode)

Figure 16: CC58 3*3 MIMO, Channel D

Figure 17: CC58 4*4 MIMO, Channel D (optional mode)

### PHY Performance

#### CC59 – AWGN PER performance

The results for CC59 are illustrated in Figure 18 to Figure 21 for 1*1 to 4*4 configurations respectively. One observation that can be made is that for a AWGN channel the capacity is a linear function of the number of transmit antennas. That is for a PER of 10% the SNR equaled ~17dB for  54Mbps, 108Mbps, 162Mbps and 216Mbps for 1*1, 2*2, 3*3, 4*4 configurations respectively.

It should be noted that in these simulations each layer had the same rate, even if the adaptive loading algorithm was switched on.

Table 21: Simulation conditions

| Parameter | Parameter value | Comment |
| --- | --- | --- |
| Simulation length | Max. 10000 packets Min 300 | Vary number depending on SNR. |
| Packet Payload | 1000 bytes |  |
| TX architecture | Parallel |  |
| Shaping Filters | Transmitter - Interpolation by 5 then 71 tap RRC filter, a=0.2 Receiver - 71 tap RRC filter, a=0.2 followed by decimate by 5 |  |
| TX/RX antennas | 1/1, 2/2, 3/3, 4/4 |  |
| Rate/layer | Average rate as shown on plots. Same rate per layer. | . |
| Interleaver | 802.11a |  |
| Channel Model | AWGN | @ 100MHz |
| Shadowing variance | 0 |  |
| Time & Freq. Synchronization | Ideal |  |
| Channel Estimation | Ideal |  |
| IM1 - PA Model | On | PA back-off at 10dB |
| IM2 – Carrier frequency offset | 0Hz |  |
| IM4 – Phase noise | Off |  |
| IM5 – Noise Figure | Not Applicable |  |
| IM6 – Antenna Config. | ½ at Tx and Rx |  |
| TX-RX Spacing | 10m |  |

Figure 18: 1x1 AWGN channel

Figure 19: 2x2 AWGN channel

Figure 20: 3x3 AWGN channel

Figure 21: 4x4 AWGN channel

#### CC67 – PER performance in non AWGN channels

The results for CC67 are illustrated in Figure 22 to Figure 24 for the 3*3 configuration. It should be noted that IM2 (frequency offset) was not activated as this function was not yet supported by the PER simulator. Also, channel D was simulated with the fluorescent effect on and one curve was simulated with it turned off (back to front as was required).

An observation that can be made is that a performance floor is evident for higher rates for all channels. This can be explained by the channel impulse response energy that exceeds the CP length (ISI) for channels E and D and also the phase noise. For channel B it is the phase noise alone that generates the performance floor.

Table 22: Simulation Conditions

| Parameter | Parameter value | Comment |
| --- | --- | --- |
| Simulation length | Max. 10000 packets Min 300 packets | Vary number depending on SNR. |
| Packet Payload | 1000 bytes |  |
| TX architecture | Parallel |  |
| Shaping Filters | Transmitter - Interpolation by 5 then 71 tap RRC filter, a=0.2 Receiver - 71 tap RRC filter, a=0.2 followed by decimate by 5 |  |
| TX/RX antennas | 3/3 |  |
| Rate/layer | As illustrated | Adaptive loading on |
| Interleaver | 802.11a |  |
| Channel Model | B, D with fluorescent (one rate without) & E | @ 100MHz |
| Shadowing variance | 0 |  |
| Time & Freq. Synchronization | Time Synchronisation Only | No frequency compensation required as IM2 is OFF |
| Channel Estimation | On |  |
| IM1 - PA Model | On | PA back-off at 12dB |
| IM2 – Carrier frequency offset | 0Hz | Not implemented in version of simulator |
| IM4 – Phase noise | On |  |
| IM5 – Noise Figure | 10dB |  |
| IM6 – Antenna Config. | ½ at Tx and Rx |  |
| TX-RX Spacing | 10m |  |

Figure 22: 3x3 Channel B

Figure 23: 3x3 Channel D with fluorescent effect (144Mbps rate shown without fluorescent effect)

Figure 24: 3x3 Channel E

#### CC67.2 – Offset Compensation

Not completed as the PER simulator does not yet support reference frequency offset at transmitter and receiver i.e. frequency offset and sampling time offset compensation. However EVM simulations have been shown in Section 2.3.3.2.

### PHY Changes

#### CC80 – Required Changes to 802.11 PHY

The modifications required for a legacy 802.11 PHY are;

- The scalable architecture supports up to 3 (mandatory) or 4 (optional) antennas (although more are possible), see section 2.2.8.1

- The adaptive loading modifies the puncturing and Constellation Mapping on a layer basis, see Section 2.2.5.1. The STA determine the maximum rate per layer (mandatory) or subgroup of carriers (optional) and this is communicated back to the AP, and vice-versa.

- The mapping function has an expanded rate set, see Section 2.2.6.1.

- Cyclic Delay is implemented with a progressive 1 sample delay /per antenna, see Section 7.1.

- The LTS preambles are modified versions of the 802.11a/g defined sequences, see Section 2.2.2.

# References

[1]	Call for Proposals for 802.11n, IEEE 802.11-03/858r6.

[2]	Functional Requirements, IEEE 802.11-03-0813-12-000n

[3]	Comparision Criteria, IEEE 802.11-03-0814-30-000n

[4]	Usage Models, IEEE 802.11-03-0802-23-000n

[5]	Throughput Enhancement via Frame Aggregation – A Sequel,  IEEE 802.11-03/567r0

[6] 	IEEE 802.11-02-798r7, 802.11 HT PAR

[7]	IEEE 802.11-02-799r6, 802.11 HT 5 Criteria

# Appendices

## Appendix A – Background to Cyclic Delay Technique

When one wishes to transmit the same information simultaneously on all Tx antennas in a channel with a dominant LOS characteristic, then there is a problem of the MIMO transmit antennas exhibiting a directional pattern (of both gain and attenuation).  For example in Figure 25(a) a lamda/2 array with ideal matched up-conversion, the number of nulls is shown to equal the number of TX antennas minus 1.  A 3 antenna system has nulls at 42 degrees and -42 degrees. In the case when the Tx chains have random phase errors, see Figure 25(b), then there is always one direction that gives a null greater than 20dB in the 2Tx antenna case.  Also 3Tx and 4Tx antennas can have this problem, but are more likely to have many directions giving over 5dB attenuation.

The implications of this are that in certain directions the AGC will give the wrong setting, the 802.11a Signal and Data symbols transmitted simultaneously on each antenna might not be decoded, even though the MIMO section of the packet can be decoded.

Figure 25: Antenna Array (sp=0.5) gain dB vs AOD.  (a) Steering coefficients = 1exp(j*0). (b) Steering coefficients = 1exp(j*pi*rand). The single antenna performance (with same Tx power) is at 0dB

One way of circumventing this problem is to introduce a progressive cyclic delay per antenna. The cyclic delay can be implemented by a cyclic shift immediately after the IFFT of 0 samples for the first antenna, 1 sample for the 2nd antenna and 2 samples for the third antenna etc.   It should be applied to every symbol.

The Table below illustrates the beneficial effects of introducing the cyclic delay shift. The path loss (dB) has been calculated in the situation when 3Tx antennas are transmitting the same signal to 1 Rx antenna with and without delay diversity. The result is that the average path loss is less with cyclic delay due to the delay diversity effect. It is applicable to channels with a LOS or high level of correlation.  It stops these dominant rays from either reinforcing or cancelling (flat fading component). As such it improves the performance of synchronisation when the same information is to be simultaneously sent on all Transmit antennas. In summary;

- Training using phase loaded LTSs will be improved because each LTS will have the same average power.

- AGC levels will be correctly set using the STS.

- It has no effect in non-correlated NLOS channels.

Table 23: Effect of Cyclic Delay on Average Path Loss(dB)

|  | LOS (19m,E) Corr(0.5) | LOS(19m,E) Ncorr(10) | NLOS(30m,E) + Corr C(0.5) | NLOS(30m,E) NonCor(10) |
| --- | --- | --- | --- | --- |
| Path loss – No cyclic Delay | -75.3 | -68.2 | -83.4 | -78.7 dB |
| Path Loss – Cyclic Delay | -70.3 | -70.2 | -79.8 | -79.5dB |
| Improvement | 5dB | -2dB | 3.6dB | -0.8dB |

The introduction of cyclic delay effects the cross correlation and auto correlation of the ‘short-training sequences’, by synthetically increasing the delay spread. Synchronisation performance is known to degrade with delay spread.

Figure 26: Cross correlation (top) with the standard 11a STS. Standard-STS(blue curve), delayed-STS(red curve) and (combined) DD-STS(black curve). Auto-correlation (bottom) with standard-STS(blue curve) and DD-STS(black curve) (un-normalised).  Correlations are summed over 16 samples.

Figure 26 & Figure 28 shows the two peaks of the cross-correlated waveform of the STS and LTS respectively, representing two signals at the transmit antenna with a progressive cyclic delay of 1 sample per antenna.  The auto correlation result shown in Figure 26 & Figure 28 for STS and LTS respectively, shows a broadening of the peak, this will make synchronisation less accurate. , As the number of antennas are increased the synchronisation and channel estimation algorithms will degrade in performance unless they are reconfigured to take advantage of the diversity  , this also implies more computation.

Figure 27 : Top: Cross correlation for 3 transmit antennas showing the component parts and composite result. Bottom: Auto correlation for 1x1 (blue),2x1(red),3x1(green),4x1(black) antennas. Note curves will change with the channel.

Figure 28 : Top: Cross correlation for 3 transmit antennas showing the component parts and composite result. Bottom: Auto correlation for 1x1 (blue),2x1(red),3x1(green),4x1(black) antennas. Note curves will change with the channel.

## Appendix B – Example of Length Calculation of the Sig3 field

### Transmitter to Receiver

Table 24: Calculation of # pad bits

| AL | Mode | Sig3R [Mbps] | Rev CSI field Length* [bits] | FWD Rate field Length [bits] | Sig3 field length [bits] | # OFDM symbols | Pad bits | Total length of Sig3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Same on all layers | 11n; any # of layers | 6 | 4 | 4 | 36+Pad | 2 | 12 | 48 |
| 1 rate per layer | 11n; 2 layers | 6 | 4 | 8 | 40+Pad | 2 | 8 | 48 |
|  | 11n; 3 layers | 6 | 4 | 12 | 42+Pad | 2 | 4 | 48 |
|  | 11n; 4 layers | 6 | 4 | 16 | 48+Pad | 2 | 0 | 48 |
| 4 sub-groups per layer | 11n; 2 layers | 9 | 4 | 32 | 62+Pad | 2 | 10 | 72 |
|  | 11n; 3 layers | 12 | 4 | 48 | 80+Pad | 2 | 16 | 96 |
|  | 11n; 4 layers | 12 | 4 | 64 | 96+Pad | 2 | 0 | 96 |

*) The Rev CSI field is empty for the first transmitted frame; the RTS frame.

If we allow the sig3 field to occupy two OFDM-symbols it’s not necessary to go higher than 12 Mbps (QPSK, R=1/2) and yet have 6 reserved bits. Then the number of padbits is between 0 and 16.

### Receiver to Transmitter

The length of the Sig3 field in the up-link direction is the same as in the down-link. The difference is that the “FWD Rate” field consists of four bits and the “Rev CSI” field varies in the same manner, See Table 24 above, as the “FWD Rate” field of the previously received frame did in the down-link.

## Appendix C – Frame Aggregation

Frame aggregation increases the MAC efficiency and ‘seems to be a good solution to improve throughput when the distance between sender and receiver is relatively short’ [5]. In terms of the frame format proposed in this document the expected MAC throughput rates are illustrated in Figure 29. The techniques and assumptions that are compared are:

- MIMO single frame mode: An average of 144Mbps on Data frame, 12Mbps on ACK frame and an 8us SIFS.

- MIMO RTS/CTS mode : An average of 144Mbps on Type 2 MIMO Data frame, 12Mbps on ACK frame and an 8us SIFS. PER of 10 % and adaptive loading overhead included.

Figure 29(a) illustrates the upper bound which is represented by the MIMO single frame transmissions, where 100Mbps Data throughput rate for payload sizes greater than 5.5kbytes.

Figure 29(a) also illustrates the MAC data throughput considering also the transmission of the RTS/CTS frames given a PER of 10%, MIMO type 3 training frame and adaptive loading overhead. It can be seen that the conditions needed to achieve 100Mbps are that the payload size per frame is greater than 2kbytes, and the transmission length is at least 10kbytes. By increasing the payload size per frame to 9kbytes then the transmission length can be reduced to around 7kbytes, and it obviously has a much increased MAC efficiency over payload sizes of 1500bytes.

Figure 29: Comparison of  MAC data throughputs versus transmission length for different frame format assumes a PER of 10%.

## Appendix D - CC58 Calculation Technique and Assumptions

##### Calculation Assumptions

MSDU = 1500 bytes = 12000 bits

Transmission time per symbol, TS = 4s

- Calculation Process

  - Generate PER vs SNR curve using adapive loading for a given PHY data rate e.g.  RatePHY = 144Mbps on average using adaptive loading.

  - Convert SNR axis to distance axis

The relationship between SNR at the receiver and the channel path loss as a function of distance, L(d), can be calculated:

SNR = PTX – L(d) – (-174+10+10log10(BW))  dB

Assuming that

- The transmit power is  17dBm

- The receiver has a noise figure of 10dB

- The channel Bandwidth is 20MHz

Then

SNR = 108 – L(d) dB

From IEEE 802.11 MIMO channel models [3], the path loss model consists of the free space loss LFS (slope of 2) up to a breakpoint distance  and slope of 3.5 after the breakpoint distance, which can be expressed as

(4)

Free space loss is given by

dB					(A)

where  is the distance between transmitter and receiver,  is the carrier wavelength. dBP for IEEE channel models A to F are shown below.

|  | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| (m) | 5 | 5 | 5 | 10 | 20 | 30 |

Figure 30: (a) SNR vs Distance for different channel models (b) conversion for model B

  - Convert PER axis to Goodput axis for Single Frame transmission

Goodput takes into account retransmissions and overhead  i.e :

- Retransmissions: PER = 10%, 10% retransmissions,

Rategoodput = RatePHY,OH*(1-PER)………………………………………(1)

- Overhead: Total = NP3 = 29 symbols where

  - Type 3 frame overhead = 18 symbols

  - SIFS at 8us = 2 symbols

  - ACK = 7 symbols

  - SIFS = 2 symbols

- Calculate number of data symbols

Average number of data symbols for a 1500 byte payload

| RatePHY | NS |  |  |
| --- | --- | --- | --- |
|  | 2*2 | 3*3 | 4*4 |
| 108 |  | 29 |  |
| 144 | 22 | 22 | 22 |
| 162 |  | 19 | 19 |
| 192 |  | 17 | 17 |
| 216 |  | 15 | 15 |
| 256 |  |  | 13 |
| 288 |  |  | 11 |

Average PHY data rate = 144 Mbps => NS = 22 data symbols

The PHY rate including the overhead is calculated as:

RatePHY,OH = RatePHY * NS /(NS + NP3) = RatePHY * 22/(22+29)………………………..(2)

Insert (2) into (1) to get

Total Goodput (Single frame Only)= Rategoodput= RatePHY * 22/(22+29) *(1-PER)

  - Convert PER axis to Goodput axis for RTS/CTS Frame transmission

This is based on the assumption that the time, frequency and Channel Tracking Algorithm performance does not impact performance -

Goodput takes into account retransmissions and overhead. Assume transmiting successfully 50*1500byte packets = 75kbytes of transmission

- Retransmissions: PER = 10%, Total packets re-transmitted = floor(50*PER / (1-(PER)) = 5. Note that a retransmission includes a packet with a training sequence.

- Calculate Average Overhead:  assume reserve medium for a transmission of 75kbytes e.g. Ntx = Nrx = 3. Total Overhead = 499 symbols

  - RTS Type 3 frame 	= 8+2*(Ntx-1)+ 2 symbols	= 14

  - CTS Type 3 frame 	= 8+2*(Nrx-1)+ 1 +2 symbols	= 15

  - 50 Type 2 and 	= (1 + 2)*50			= 150

  - 50 ACK frames   	= (2 + 2)*50			= 200

  - 5 Type 1 frame and = (5+2*(Ntx-1)+1 +2)*5	= 60

  - 5 Type 1 ACK frame = (5+2*(Nrx-1)+1 +2	)*5	= 60

- Calculate number of data symbols transmitted @ 144Mbps

Total number data symbols in type 2 and type 1 frame= (50+5) * 22= 1210 symbols

- Calculate Goodput

Rategoodput = Total successful bits at MAC / Total Simulation time =
= 50*1500*8 / (4e-6*(1210+499)) = 87.8 Mbps

- Note for CC67: Data throughput excludes the overheads. Assuming the parameters above:

Ratethroughput = Total successful bits at MAC / Total Simulation time without overheads =
= 50*1500*8 /(4e-6*(1210)) = 124Mbps

  - From graph choose points that are greater than 100Mbps and calculate bps/Hz = RatePHY /20MHz

## Appendix E – Rate Estimation for Channel Bonding

Apart from using multiple antennas, channel bonding can also increase data rates of OFDM system. Estimation of the rate increase is as follows. First, each OFDM symbol has 48 data carrying subchannels. As a result of channel bonding, the total number of data carrying subchannels is  where the extra 12 subchannels are recovered from guard band in the reference SISO-OFDM system. Second, in order to maintain the same transmitted power, the total power per subchannel drops by approximately a half (or 3 dB) which results in increase of BER.  One way of maintaining the same bit error rate is to reduce the number of bits per QAM symbol by one.

Therefore, assuming 6 bits on each subchannel in reference SISO system, channel bonding can transmit the total of  bits per OFDM symbol, comparing to bits per OFDM symbol without. This represents an increase by a factor of 1.8.  This increase factor also applies to the case of MIMO systems.

The value of  ranges from 5 to 30 m, depends on the channel models A to F. More details can be found in the reference provided.

We have not included the extra 12 subchannels in the power computation.