IEEE P802.11
Wireless LANs

ITRI Proposal Response to Comparison Criteria

Date:	August 13, 2004

Author:	Yung-Yih Jian(1), Pangan Ting, Wei-Ping Chuang,

Chun-Chun Kuo, Chang-Lung Hsiao

Wei-De Wu(2)

Chih-Kai Chen(3)

(1)Industiral Technology Research Institute (ITRI)

Computer and Communications Research Laboratories (CCL)

Bldg.14, 195 Sec. 4, Chung  Hsing Rd. Chutung, Hsinchu, Taiwan 310, R.O.C

Phone : +886-3-5914707
	Fax : +886-3-5820371
	e-Mail: {yyjian, pating, joeChuang, godeman, clhsiao}@itri.org.tw

(2)National Tsing Hua University

Institute of Communications Engineering

welder@micro.ee.nthu.edu.tw

(3)Stanford University

Department of Electrical Engineering

chihkai@ stanford.edu

Abstract

This document contains responses to the comparison criteria (IEEE 802.11-03/814r31) for ITRI’s proposal (partial proposal; preamble design) to TGn. The responses are summarized in the following tables.

# Additional Disclosures

| Number | Name | Definition | Status of this AD | Notes | Responses |
| --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of related IEEE submissions, both documents and presentations. |  |  | ITRI preamble spec. ITRI CC response ITRI FR response |
| AD2 | TCP Model Parameters | Include a reference to the TCP protocol type (e.g. Reno) and the parameter values associated with that protocol used for all MAC simulations. |  |  | N/A |
| AD3 | MAC simulation methodology | Include a description of the simulation methodology used for MAC simulations, including a description of how the PHY and its impairments are modeled. |  |  | N/A |
| AD4 | MAC simulation occupied channel width | For each MAC simulation, report the total channel width occupied. |  |  | N/A |
| AD5 | Justification of low PLR rates achieved | For each application with a PLR < 10^-4, proposal shall justify that the proposed PLR could be met. | 17/1 on 10 May, 2004 |  | N/A |

# Response to Comparison Criteria

| Number | Name | Definition | Simulation Scenario / Impairments | Mandatory / optional | Disclosure | Response |
| --- | --- | --- | --- | --- | --- | --- |
| CC2 | Regulatory compliance | The proposal shall state any known problems with regulatory compliance with at least the following domains: USA, Japan, Europe, China. | None required | Mandatory | Textual Statement | N/A (In this proposal, we adopt the same frequency band and channalization as 802.11a) |
| CC3 | List of goodput results for usage models 1,4 and 6. | List the goodput results (CC20, metric 1) for scenarios 1,4 and 6. | Scenarios 1,4,6 Impairments as defined for CC20 | Mandatory | List of Goodput results. | N/A (PHY partial proposal, preamble design ) |
| CC6 | PHY complexity | Give an indication of the PHY complexity, relative to current 802.11a PHYs (e.g. gate counts, MIPS, filtering, clock rate, etc.). | None required | Optional | Numeric value in any suitable form | N/A (PHY partial proposal, preamble design ) |
| CC7 | MAC processing complexity | Give an indication of the MAC processing complexity, relative to implementations of 802.11-1999 (Rev 2003), and published amendments (e.g. gate counts, MIPS, memory, clock rate, etc.) | None required | Optional | Numeric value in any suitable form | N/A (PHY partial proposal, preamble design ) |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Provide a summary description of the means, if any, by which backward compatibility with 802.11-1999 (Rev 2003) and 802.11g is achieved in the band(s) covered by the proposal, including references to the sections in the technical proposal document where the complete details are given. | None required | Mandatory | Bullet-point summary and reference to section in technical specification. | With the proposed preamble design, the system is backward compatible with 802.11a. Please refer to Section 3.2.1 of “ITRI preamble spec.”. |
| CC15 | Sharing of medium with legacy devices | Goodput of legacy device in HT network and impact of legacy device on the goodput of the HT devices. Report the following measurements : --T1: the goodput reported in simulation scenario 17. --T2: the goodput reported in simulation scenario 18. --T3: the goodput reported for STA1 in simulation scenario 19. --T4: the goodput reported for STA2 in simulation scenario 19. | Simulation scenarios 17-19. No impairments | Mandatory | Report values of metrics T1-T4. | N/A (PHY partial proposal, preamble design ) |
| CC18 | HT Usage Models Supported (non QoS) | This metric relates to the ability of the proposal to support the non-QoS application service level of each usage model, as defined by its simulation scenario. For each simulated scenario: Report Goodput per non-QoS flow. Report aggregate goodput for non-QoS flows. Report the ratio of aggregate Goodput for non-QoS flows to the total offered load for non-QoS flows. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. Note, backward TCP Ack flows shall be counted as non-QoS flows. | Simulation Scenarios 1, 4, and 6. Note, this is measured with QoS flows turned on. IM1,5,6 Simulations should be conducted using perfect synchronization and perfect channel estimation | Mandatory | For each simulated scenario report results as defined in this CC. | N/A (PHY partial proposal, preamble design ) |
| CC19 | HT Usage Models Supported (QoS) | This metric relates to the ability of the proposal to support the QoS application service level of each usage model, as defined by its simulation scenario. For each QoS flow, the proposal shall report the packet loss rate (defined below) and compare it to the specified QoS objective. The proposal shall also report the number of these flows that satisfy their QoS objective . Also report the fraction of QoS flows that satisfy their QoS objective. For the purpose of this criterion, packet loss rate (PLR) for a QoS flow is defined as the number of MSDUs that are not delivered at the Rx MAC SAP within the specified delay bound, divided by the total number of MSDUs offered at the Tx MAC SAP for that flow during the simulation . | Simulation Scenarios 1, 4 and 6. Note, this is measured with non- QoS flows turned on. IM1,5,6 Simulations should be conducted using perfect synchronization and perfect channel estimation | Mandatory (Vote in TGn 10 May 2004) | For each simulated scenario report results as defined in this CC. | N/A (PHY partial proposal, preamble design ) |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | Three aggregate goodput metrics are to be reported. Metric 1 is defined as the sum of goodput across all flows in the simulation scenario. Metric 2 is defined as the aggregate number of bits in MSDUs that are delivered at the Rx MAC SAP within the specified delay bound of the flow’s defined QoS, divided by the simulation duration. Metric 3 is defined as the sum of goodput across all flows that meet their QoS objective in the simulation scenario. Notes: Metric 1 includes flows that fail to meet their QoS objectives. Metric 2 excludes MSDUs that exceed the delay bound. Metric 3 excludes all MSDUs from flows that fail to meet their QoS objectives. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. | Simulation Scenarios 1, 4 and 6. IM1,5,6 Simulations should be conducted using perfect synchronization and perfect channel estimation | Mandatory | For each simulated scenario, report values of metrics 1-3 defined in here | N/A (PHY partial proposal, preamble design ) |
| CC24 | MAC Efficiency | MAC efficiency is defined as the aggregate Metric 2 goodput in CC20 divided by the average physical layer data rate. Let r(n) denote the rate at which the nth successful Data MPDU is transmitted. Let t(n) denote the PPDU transmission time of the nth successfully transmitted Data MPDU (i.e. including preamble, PLCP header). Define the average PHY data rate r as r = ∑ r(n)t(n)/ ∑t(n) | Simulation Scenarios 1, 4, and 6. IM1,5,6 Simulations should be conducted using perfect synchronization and perfect channel estimation | Mandatory | For each simulated scenario, Report MAC efficiency metric defined in here | N/A (PHY partial proposal, preamble design ) |
| CC27 | Throughput / Range | Report curves of Goodput (bps) vs. range (m). Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [4]: Model B Residential Model D Typical Office | Simulation Scenario 16 IM1,5,6 Simulations should be conducted using perfect synchronization and perfect channel estimation | Mandatory | For the simulated channel models, Present the required curves. Also provide, textual description of parameter and setting values. | N/A (PHY partial proposal, preamble design ) |
| CC28 | Throughput / Range in 20MHz | Report curves of Goodput (bps) vs. range (m) , when operating in a 20 MHz bandwidth. Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [4]: Model B Residential Model D Typical Office | Simulation scenario 16 IM1,5,6 Simulations should be conducted using perfect synchronization and perfect channel estimation | Mandatory | For the specified channel models from [3]: Present curves of Goodput (bps) vs. range (ml, when operating in a 20 MHz bandwidth. Disclose all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. | N/A (PHY partial proposal, preamble design ) |
| CC46 | MAC Compatibility and parameters. | Provide a list of optional features of 802.11a/b/d/e/g/h/i/j that are required for HT operation, and a summary description of the manner in which they are used. Include references to the sections in the technical proposal document where the complete details are given. | None required | Mandatory | Text list of features plus references to sections in proposed technical specification document. | N/A (PHY partial proposal, preamble design ) |
| CC47 | MAC extensions | Provide a summary description of MAC extensions beyond 802.11a/b/d/e/g/h/i/j that are required for HT operation. Include references to the sections in the technical proposal document where the complete details are given. | None required | Mandatory | Text list of features plus references to sections in proposed technical specification document. | N/A (PHY partial proposal, preamble design ) |
| CC51 | Data rates | A list of PHY layer data rates, and for each data rate, specify the used modulation techniques, number of Tx antennas, coding rate and bandwidth. Specify which of the rates are mandatory and which are optional. For adaptive rate proposals, specify the range of achievable rates or, if possible, state the achievable rates in a closed form expression. | None required | Mandatory | Text list of rates supported, marked mandatory or optional plus additional parameters as described in this CC. For adaptive rate proposals, the range of achievable rates. | N/A (PHY partial proposal, preamble design ) |
| CC42 | Preambles | Specify the proposed preambles. Summarize the important properties of each part the proposed preambles. Include references to the sections in the technical proposal document where the complete details are given. Specify how the use of any new preamble affects reception by legacy STA. | None required | Mandatory | Reference to section in technical specification defining preambles. For each preamble type supported: Mean and std of peak to sidelobe ratio of the autocorrelation function PAPR values. Description and evaluation of cross-correlation properties. | In this report, we propose a novel preamble structure for IEEE 802.11n standard. The proposed preamble structure is compliant with the FRCC documents and has good properties for frame detection and channel estimation. The structure consists of 10 periods of 32-sample short training symbols (STSs), and 4 periods of 64-sample long training symbols (LTSs). For details please refer to “ITRI Preamble spec.” |
| CC51.5 | Channelization | Specify the channelization – i.e. the adjacent channel spacing. | None required | Mandatory | Minimum channel spacing for all operational modes | The same as the specification of 802.11a. |
| CC52 | Spectral Mask | Define the transmit spectral mask requirements for each channelization of the proposal. This must be under the same PA backoff used for performance simulations. | None required IM1 | Mandatory | For each channelization: graph of spectral output (dBm) vs. frequency offset from center frequency (MHz). | The same as the specification of 802.11a. |
| CC58 | HT Spectral Efficiency | The number of bps/Hz during the PSDU carrying a Data MPDU when demonstrating a goodput value of at least 100Mbps. Specify the phy data rate used during this test. | Using simulation scenario 16 defined in [3]. IM1,5,6 | Mandatory | The number of bps/Hz and specified data rate. | N/A (PHY partial proposal, preamble design ) |
| CC59 | AWGN PER performance | Identify the performance in an idealized channel for a PSDU length of 1000B. The rows or columns of the channel shall be orthogonal to each other as follows: take the first Nr x Nt elements of the Fourier matrix with dimension max(Nr,Nt). Show PER versus SNR curves for 5 supported data rates representative of the proposal's rate set, including the maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Averaging should occur over a minimum of 100 packet errors down to 1% PER Refer to [7] for a definition of the Fourier matrix. Note: SNR is defined in section 2. | None required None Simulations should be conducted using perfect synchronization and perfect channel estimation | Mandatory | For proposals which have any mode generating 2 or more independent streams, show 4 graphs for the values Nr=Nt from 1 to 4. For proposals that do not have any mode generating 2 or more independent streams, show a single graph for Nr=Nt=1. For each graph: Plot a curve of log PER vs. SNR (dB) for each of 5 supported data rates on the same axes. | N/A (PHY partial proposal, preamble design ) |
| CC67 | PER performance in non AWGN channels | Show either or both of the following two sets of performance curves: 1.) Show the PER curves for 5 supported data rates representative of your rate set including your maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Plot PER versus SNR averaged over time per receive antennas for PSDUs of length 1000B. Averaging should occur over a minimum of 100 MPDU errors down to 1% PER. Each packet should use an independent channel realization. 2.) Show curves for both achieved average physical layer data throughput and PER, as a function of total SNR for 1000B PSDUs. These results should be generated with a rate selection algorithm active. Data throughput is defined as the total number of bits successfully received in the data portion of the PPDU, divided by total transmission time, not including overheads such as preamble and backoff. The throughput shall be averaged over at least 100 independent realizations of the channel, each realization long enough to allow simulation of rate adaptation with subsequent transmission of one or more PPDUs while the Doppler process evolves. The same number of PPDUs shall be simulated for each channel realization. In addition a minimum number of PPDUs, equal to 100 divided by the target FER, shall be simulated to determine throughput. Total received signal power is summed over all transmit antennas. There shall not be any a priori knowledge of the channel or synchronization at the receiver or transmitter. This should be simulated for channel models B, D, and E. The simulations should all include the Doppler effect as specified in the text of the channel model document. All models should be run without the fluorescent effect but additionally model D should be run with the fluorescent effect on the highest data rate. The interferer to carrier energy ratio of the fluorescent effect shall be equal to its average value (0.0203). The shadowing variance should be 0. These simulations are performed using the NLOS version of the specified channel models. Note: SNR is defined in section 2. | None required IM1-6 | Mandatory | For each of channel models B, D and E: Plot a curve of log PER vs. SNR (dB) for each of 5 supported data rates. Additionally, for channel model D: Plot a curve of log PER vs. SNR (dB) for the highest supported data rate incorporating the Fluorescent effect on the same graph as the other curve for channel model D. | N/A (PHY partial proposal, preamble design ) |
| CC67.2 | Offset Compensation | Provide the impact on PER of carrier frequency offset and symbol clock offset by comparing to the PER achieved at the lowest average SNR that achieves a 10% PER for PSDUs of length 1000 bytes in channel E (NLOS) with no carrier and symbol clock offset. The symbol clock shall have the same relative frequency offset as the carrier frequency offset. Also, provide that same impact on PER using an SNR of 50dB in channel E (LOS). The carrier offset difference at the receiver relative to the transmitter shall be -40ppm and +40ppm. The results shall be presented in such a manner that it is clear whether there are specific values of offset for which the proposed system has better or worse performance relative to no offset. Simulations shall be performed under the same conditions as those in CC 59 & 67. | None required IM1-5 | Mandatory | Numeric results | N/A (PHY partial proposal, preamble design ) |
| CC80 | Required changes to 802.11 PHY | Give a summary description of changes to a legacy 802.11 PHY. Give references to sections in your specification that give the complete details. | None required | Mandatory | A bullet-point list of main features with reference to the section in the technical specification. | N/A (PHY partial proposal, preamble design ) |