IEEE P802.11
Wireless LANs

Responses to Functional Requirements and Comparison Criteria

Date:				August 13, 2004

Authors:

John M. Kowalski

5750 NW Pacific Rim Blvd., Camas, Washington 98607

Phone: +1-360-834-8696

e-Mail: Kowalski@sharplabs.com

Atsushi Ohta

Access Service Systems Laboratories, NTT corporation

1-1, Hikarino-oka, Yokosuka-shi, Kanagawa, Japan

Phone: +81-46-859-2903

e-Mail:  Ohta.Atsushi@ansl.ntt.coAbstract

Abstract

This document is a partial proposal for the IEEE802.11n standard.

# Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| John M. Kowalski | Sharp | 5750 NW Pacific Rim Blvd., Camas, Washington 98607 | +1-360-817-7520 | +1-360-834-8696 | kowalskj@sharplabs.com |
| Srinivas Kandala | Sharp | 5750 NW Pacific Rim Blvd., Camas, Washington 98607 | +1-360-817-7520 | +1-360-834-8696 | srini@sharplabs.com |
| Morgan H. Miki | Sharp | 2613-1 Ichinomoto, Tenri, Nara, 632-8567, Japan | +81-743-65-4529 | +81-743-65-0823 | miki@slab.tnr.sharp.co.jp |
| Yoshihiro Ohtani | Sharp | 2613-1 Ichinomoto, Tenri, Nara, 632-8567, Japan | +81-743-65-4529 | +81-743-65-0823 | ohtani@slab.tnr.sharp.co.jp |
| Wataru Gohda | Sharp | 5750 NW Pacific Rim Blvd., Camas, Washington 98607 | +1-360-817-7520 | +1-360-834-8696 | wgohda@sharplabs.com |
| Richard Bennett | Sharp | 5750 NW Pacific Rim Blvd., Camas, Washington 98607 | 1-360-817-7439 | +1-360-834-8696 | rbennett@sharplabs.com |
| Naoki Urano | Sharp | 2613-1 Ichinomoto, Tenri, Nara, 632-8567, Japan | +81-743-65-4529 | +81-743-65-0823 | urano@slab.tnr.sharp.co.jp |
| Atsushi Ohta | NTT | 1-1, Hikarino-oka, Yokosuka-shi, Kanagawa, Japan | +81-46-859-2903 | +81-46-855-1752 | Ohta.Atsushi@ansl.ntt.co.jp |
| Yusuke Asai | NTT | 1-1, Hikarino-oka, Yokosuka-shi, Kanagawa, Japan | +81-46-859-3494 | +81-46-855-1752 | asai.yusuke@lab.ntt.co.jp |
| Yasuhiko Inoue | NTT | 1-1, Hikarino-oka, Yokosuka-shi, Kanagawa, Japan | +81-46-859-5097 | +81-46-855-1752 | yinoue@ansl.ntt.co.jp |
| Takeshi Onizawa | NTT | 1-1, Hikarino-oka, Yokosuka-shi, Kanagawa, Japan | +81-46-859-2453 | +81-46-855-1752 | onizawa@ansl.ntt.co.jp |
| Daisei Uchida | NTT | 1-1, Hikarino-oka, Yokosuka-shi, Kanagawa, Japan | +81-46-859-2369 | +81-46-859-3351 | uchida.daisei@lab.ntt.co.jp |
| Takafumi Fujita | NTT | 1-1, Hikarino-oka, Yokosuka-shi, Kanagawa, Japan | +81-46-859-8423 | +81-46-855-1497 | fujita.takafumi@lab.ntt.co.jp |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Initial version | Aug 13th, 2004 |

Abstract

This document summarizes the responses to the Functional Requirements and Comparison Criteria as required by the 802.11n Call for Proposals.  The information in this document is also contained in document 04/938r0, the partial proposal for 802.11n.

The first section is a summary of the simulation results for the CC; followed by specific responses to the FR and CC. References to tables and figures are to those contained in document 04/938r0.  Should there be any discrepancy between 938r0 and this document, 938r0 takes precedence.

# Simulation results for items in Comparison Criteria document

In the following section, simulation results that are required in Comparison Criteria document are presented.  To understand the upper limit of proposal’s performance, theoretical throughput performance is also evaluated.

## Theoretical throughput performance

Figure 1 shows theoretical relationship between number of aggregated frames and throughput performance under error free environment.  For simplicity, let us assume following condition;

- Error free environment,

- UDP traffic source with bit rate of more than 126 Mbps,

- Fixed length Ethernet frame whose payload size is 1500 Bytes,

- CWmin=15,

- Rate on PHY layer is 126 Mbps where the number of Tx chains is 2 and the PHY mode is 64QAM R=7/8,

where the random backoff period is averaged to be 67.5 ms.  When pilot preamble is not inserted, the maximum throughput is 116.4 Mbps and its MAC efficiency is more than 92%.  When inserted pilot preamble intervals are 64 and 128 OFDM symbols, the maximum throughput are 112.9 Mbps and 114.5 Mbps, respectively.

Figure 1  Theoretical throughput performance

## MAC related simulation items

MAC related simulation items in comparison criteria document are CC18, CC19, CC20 and CC24 for SS1 and SS6, as well as CC27 and CC28 for SS16.

### SS1 and SS6

Simulation results for CC18, CC19, CC20 and CC24 for SS1 and SS6 are in Appendix A.

There are 3 results for SS1, two for EDCA and one for HCCA. Also, there are 2 results for SS6, one for EDCA and one for HCCA.

### SS16

Table 1 and Table 2 shows the Goodput result for SS16 in Mbps, obtained with the use of 2Tx MIMO mode for both Channel Models B and E respectively. Figure 2 shows the curves of Goodput(Mbps) vs. range(m) of both tables.

Table 1  Goodput for SS16 (Mbps), Channel Model B

| Channel Model |  | B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pilot Preamble Interval |  | 0(*1) |  |  |  | 16(*2) |  |  |  |
| Max Frame Duration |  | 1.5ms |  | 2.5ms |  | 1.5ms |  | 2.7ms |  |
|  |  | Goodput (Mbps) | mean PHY rate (Mbps) | Goodput (Mbps) | mean PHY rate (Mbps) | Goodput (Mbps) | mean PHY rate (Mbps) | Goodput (Mbps) | mean PHY rate (Mbps) |
| Range | 4 | 109.68 | 126 | 116.69 | 126 | 108.96 | 126 | 115.85 | 126 |
| (m) | 8 | 108.25 | 125.3 | 115.16 | 125.3 | 107.53 | 125.4 | 114.44 | 125.4 |
|  | 12 | 94.68 | 112.2 | 100.62 | 112.2 | 94.06 | 112.2 | 99.89 | 112.2 |
|  | 16 | 73.82 | 87.3 | 78.36 | 87.3 | 73.28 | 87.3 | 77.83 | 87.3 |
|  | 20 | 58.57 | 68.8 | 62.2 | 68.8 | 58.21 | 68.8 | 61.72 | 68.8 |
|  | 24 | 48.67 | 57.1 | 51.68 | 57.1 | 48.35 | 57.1 | 51.29 | 57.1 |

(*1) No pilot preambles

(*2) Pilot preamble interval is 8*16=128 OFDM symbols

Table 2  Goodput for SS16 (Mbps), Channel Model B

| Channel Model |  | E |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pilot Preamble Interval |  | 0(*1) |  |  |  | 16(*2) |  |  |  |
| Max Frame Duration |  | 1.5ms |  | 2.5ms |  | 1.5ms |  | 2.7ms |  |
|  |  | Goodput (Mbps) | mean PHY rate (Mbps) | Goodput (Mbps) | mean PHY rate (Mbps) | Goodput (Mbps) | mean PHY rate (Mbps) | Goodput (Mbps) | mean PHY rate (Mbps) |
| range | 4 | 109.68 | 126 | 116.69 | 126 | 108.96 | 126 | 126 | 126 |
| (m) | 8 | 109.67 | 126 | 116.68 | 126 | 108.93 | 126 | 126 | 126 |
|  | 12 | 109.38 | 126 | 116.37 | 126 | 108.65 | 126 | 126 | 126 |
|  | 16 | 107.88 | 125.7 | 114.73 | 125.7 | 107.14 | 125.8 | 125.8 | 125.8 |
|  | 20 | 92.36 | 108.6 | 98.1 | 108.6 | 91.7 | 108.6 | 108.6 | 108.6 |
|  | 24 | 79.21 | 93.6 | 83.93 | 93.6 | 78.65 | 93.6 | 93.6 | 93.6 |
|  | 32 | 56.8 | 67 | 60.36 | 67 | 56.43 | 67 | 67 | 67 |
|  | 40 | 46.38 | 54.2 | 49.18 | 54.2 | 46.07 | 54.2 | 54.2 | 54.2 |
|  | 48 | 41.13 | 49.1 | 43.79 | 49.1 | 40.88 | 49.1 | 49.1 | 49.1 |
|  | 56 | 33.65 | 40 | 35.78 | 40 | 33.43 | 40 | 40 | 40 |

(*1) No pilot preambles

(*2) Pilot preamble interval is 8*16=128 OFDM symbols

Figure 2  Goodput for SS16

## PHY related simulation items

PHY related simulation items in comparison criteria document are CC59, CC67 and CC67.2.  The simulation parameters for them are listed in Error: Reference source not found and Error: Reference source not found.

### AWGN PER performance (CC59)

Figure 3(a) and Figure 3(b) show the evaluated results for Nt=Nr=1 (SISO) and Nt=Nr=2 (2 Tx chains MIMO) of CC59, respectively.

(a) Nt=Nr=1 (SISO)

(b) Nt=Nr=2 (2 Tx chains MIMO)

Figure 3  Evaluation results for CC59

### PER performance in non AWGN channels (CC67)

Figure 4(a), Figure 4 (b) and Figure 4 (c) show the evaluated PER performance for Channel model B, D and E with Nt=Nr=1 (SISO) of CC67(1), respectively.

(a) Channel model B

(b) Channel model D

(c) Channel model E

Figure 4  Evaluation Results for CC67(1) with Nt=Nr=1

Figure 5(a), Figure 5 (b) and Figure 5 (c) show the evaluated PER performance for Channel model B, D and E with Nt=Nr=2 (2 Tx chains MIMO) of CC67(1), respectively.

(a) Channel model B

(b) Channel model D

(c) Channel model E

Figure 5  Evaluation Results for CC67(1) with Nt=Nr=2

Figure 6(a), Figure 6 (b) and Figure 6 (c) show the evaluated throughput performance with rate adaptation control for Channel model B, D and E with Nt=Nr=2 (2 Tx chains MIMO) and Nt=Nr=1 (SISO) of CC67(2), respectively.

(a) Channel model B

(b) Channel model D

(c) Channel model E

Figure 6  Throughput performance with rate adaptation control (CC67(2))

Figure 7(a), Figure 7 (b) and Figure 7 (c) show the evaluated PER performance with rate adaptation control for Channel model B, D and E with Nt=Nr=2 (2 Tx chains MIMO) and Nt=Nr=1 (SISO) of CC67(2), respectively.

(a) Channel model B

(b) Channel model D

(c) Channel model E

Figure 7  PER performance with rate adaptation control (CC67(2))

### Offset Compensation (CC67.2)

In CC67.2, the impact of carrier frequency offset and symbol clock offset is evaluated.  Table 3 shows the impact around PER=10% and Table 4 shows that for SNR=50dB.  The evaluated results show that there is only negligible performance gap and this proposal has a wide compensating range with +/- 40ppm, which is caused by carrier frequency offset and clock offset.

Table 3  Impact of carrier frequency offset for PER=10%

Table 4  Impact of carrier frequency offset for SNR=50dB

# Response to Functional Requirements

| Number | Name | Definition | Coverage | Disclosure |
| --- | --- | --- | --- | --- |
| 1 | Single Link HT rate supported | Demonstrate at least one set of conditions under which 100 Mbps at the top of the MAC SAP can be achieved. Provide all relevant information to document this. | Yes | In section 1.1, the throughput at MAC-SAP is evaluated. As Figure 1 shows, more than 116 Mbps can be achieved under error free environment. The results for CC27 show that the proposal can also achieve more than 100 Mbps at MAC-SAP with system level simulation. |
| 2 | HT rate supported in 20MHz channel | Proposal supports at least one mode of operation that supports 100Mbps throughput at the top of the MAC SAP in a 20MHz channel. Provide all relevant information to document this. | Yes | This proposal supports only 20 MHz operation. As described in FR1, by using 2 Tx chains MIMO and 64QAM R=7/8 on 20 MHz channel, more than 100Mbps can be achieved. |
| 3 | Supports 5GHz bands | Protocol supports 5GHz bands (including those supported by .11a) | Yes | This proposal supports 5 GHz band and it also contains all IEEE 802.11a modes. |
| 4 | .11a backwards compatibility | Some of the modes of operation defined in the proposal shall be backwards compatible with 802.11a. | Yes | As described in section Error: Reference source not found, this proposal is completely backward compatible with IEEE802.11a. |
| 5 | .11g backwards compatibility | If it supports 2.4 GHz operation, some of the modes of operation defined in the proposal shall be backwards compatible with 802.11g. | No | We did not consider support for 2.4GHz operation, but elements of this proposal may be adapted for use in that band. |
| 6 | Control of support for legacy STA from .11n AP | A .11n AP can be configured to reject or accept associations from legacy STA because they are legacy STA. | Yes | There is nothing in our proposal to forbid legacy associations, but they can always be limited by policy from the AP. |
| 7 | .11e QoS support | The proposal shall permit implementation of the 802.11e ammendment within a .11n STA | Yes | This proposal does indeed permit 802.11e, as expressed in D9.0, to be able to be deployed coincident with an 802.11n station. |
| 8 | Spectral Efficiency | The highest throughput mode of the proposal shall achieve a spectral efficiency of at least 3 bps/Hz for the PSDU | Yes | When 2 Tx chains MIMO and 64QAM R=7/8 are applied, the physical rate is 126 Mbps. Therefore, the spectral efficiency is 6.3 bits/s/Hz (=126Mbit/s /20MHz). |
| 9 | Compliance to PAR | The proposal complies with all the mandatory requirements of the PAR [5] and 5 Criteria [6] | Yes | 5 Criteria require (1) Broad Market Potential, (2) Compatibility, (3) Distinct Identity, (4) Technical Feasibility and (5) Economic Feasibility. The aim of this proposal is to create a standard that limits mandatory requirements to feasible technologies and minimizing the difference from the legacy standards as described in section Error: Reference source not found, it complies with Criteria (1), (2), (4) and (5). The maximum spectral efficiency with mandatory PHY mode is 6.3 bits/s/Hz, and it can be improved with optional PHY modes up to 12.6 bits/s/Hz. The coverage is much wider than PAN. Therefore this proposal also complies with PAR and Criteria (3). |

# Responce to Comparison Criteria

## Additional Disclosures

| Number | Name | Definition | Disclosure |
| --- | --- | --- | --- |
| AD1 | Reference submissions | A list of related IEEE submissions, both documents and presentations. | 802.11-1999; 802.11e D9.0; Document 04/775. |
| AD2 | TCP Model Parameters | Include a reference to the TCP protocol type (e.g. Reno) and the parameter values associated with that protocol used for all MAC simulations. | Own implementation of RFC 793. Timeout considered infinite since this is WLAN with MAC level acknowledgement. One TCP Ack per maximum of 4 frame aggregation. |
| AD3 | MAC simulation methodology | Include a description of the simulation methodology used for MAC simulations, including a description of how the PHY and its impairments are modeled. | Simulation methodology that is used to execute system level and MAC simulations is described in section Error: Reference source not found. |
| AD4 | MAC simulation occupied channel width | For each MAC simulation, report the total channel width occupied. | This proposal occupies only 20MHｚ channel and it is identical to IEEE802.11a. |
| AD5 | Justification of low PLR rates achieved | For each application with a PLR < 10^-4, proposal shall justify that the proposed PLR could be met. | PLR less than required PLR is achieved for number of transmitted packets equal to at least 10 / ( Required PLR) |

## General and etc.

| Number | Name | Definition | Disclosure |
| --- | --- | --- | --- |
| CC2 (M) | Regulatory compliance | The proposal shall state any known problems with regulatory compliance with at least the following domains: USA, Japan, Europe, China. | Because this proposal uses the same channelization with IEEE802.11a, there is basically no problem with regulatory compliance. The major differences from 802.11a specification are to apply MIMO communication on PHY layer and frame aggregation on MAC layer. The MIMO communication generally increases total transmission power and it makes interference between wireless systems including satellite communication systems larger. Therefore, total transmission power shall be capped as it is compliant with the rule for the transmission power from a single antenna system. On the other hand, the frame aggregation makes occupied duration for transmitting a frame longer. To share the frequency resource fairly, the maximum duration also shall be capped. The rule is described in section Error: Reference source not found. In Japan, a regulatory provides that signal transmission from any wireless terminals on 5 GHz band shall not exceed 4 ms. Our proposal is also compliant with this Japanese regulatory. |
| CC3 (M) | List of goodput results for usage models 1,4 and 6. | List the goodput results (CC20, metric 1) for scenarios 1,4 and 6. | SS4 was not performed. SS1 EDCA simulation 1/2: 58.878Mbps SS1 EDCA simulation 2/2: 61.56 Mbps SS1 HCCA simulation: 71.039Mbps SS6 EDCA simulation: 48.029Mbps SS6 HCCA simulation: 65.18Mbps |
| CC6 (O) | PHY complexity | Give an indication of the PHY complexity, relative to current 802.11a PHYs (e.g. gate counts, MIPS, filtering, clock rate, etc.). | As described in section Error: Reference source not found, our proposal offers simple solution with less impact for modification from IEEE802.11a specification. The most of all specification on PHY layer except for MIMO communication is the same with IEEE802.11a and it is scalable for increasing the number of Tx chains on MIMO communication. This is one of the advantages of our proposal. |
| CC7 (O) | MAC processing complexity | Give an indication of the MAC processing complexity, relative to implementations of 802.11-1999 (Rev 2003), and published amendments (e.g. gate counts, MIPS, memory, clock rate, etc.) | The MAC complexity is comparable to complexity of the Burst Acknowledgement mechanism present in 802.11e. (Separately: for synchronization, the capability in gate count is approximately the same, but there would be additional precision of components required.) |
| CC11 (M) | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Provide a summary description of the means, if any, by which backward compatibility with 802.11-1999 (Rev 2003) and 802.11g is achieved in the band(s) covered by the proposal, including references to the sections in the technical proposal document where the complete details are given. | On this proposal, MIMO signal is encapsulated after legacy PLCP header. In addition, by accommodating fake information in legacy Signal, legacy devices can recognize the duration of received frame. Because this proposal occupies only 20 MHz channel, no additional functions to keep backward compatibility is required. |
| CC15 (M) | Sharing of medium with legacy devices | Goodput of legacy device in HT network and impact of legacy device on the goodput of the HT devices. Report the following measurements : --T1: the goodput reported in simulation scenario 17. --T2: the goodput reported in simulation scenario 18. --T3: the goodput reported for STA1 in simulation scenario 19. --T4: the goodput reported for STA2 in simulation scenario 19. | SS 17, 18, and 19 are not performed. |

## MAC Related

| Number | Name | Definition | Disclosure |
| --- | --- | --- | --- |
| CC18 (M) | HT Usage Models Supported (non QoS) | This metric relates to the ability of the proposal to support the non-QoS application service level of each usage model, as defined by its simulation scenario. For each simulated scenario: Report Goodput per non-QoS flow. Report aggregate goodput for non-QoS flows. Report the ratio of aggregate Goodput for non-QoS flows to the total offered load for non-QoS flows. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. Note, backward TCP Ack flows shall be counted as non-QoS flows. | All results for SS1 and SS6 are in Appendix A, including per non-QoS flow Goodput. Aggregate goodput for non-QoS flow: SS1 EDCA simulation 1/2: 6.358Mbps SS1 EDCA simulation 2/2: 9.036Mbps SS1 HCCA simulation: 18.53Mbps SS6 EDCA simulation: 3.161Mbps SS6 HCCA simulation: 20.35Mbps Ratio of aggregate Goodput for non-QoS flows to the total offered load for non-QoS flows: SS1 EDCA simulation 1/2: .205 SS1 EDCA simulation 2/2: .291 SS1 HCCA simulation: .6 SS6 EDCA simulation: .158 SS6 HCCA simulation: 1.02 |
| CC19 (M) | HT Usage Models Supported (QoS) | This metric relates to the ability of the proposal to support the QoS application service level of each usage model, as defined by its simulation scenario. For each QoS flow, the proposal shall report the packet loss rate (defined below) and compare it to the specified QoS objective. The proposal shall also report the number of these flows that satisfy their QoS objective . Also report the fraction of QoS flows that satisfy their QoS objective. For the purpose of this criterion, packet loss rate (PLR) for a QoS flow is defined as the number of MSDUs that are not delivered at the Rx MAC SAP within the specified delay bound, divided by the total number of MSDUs offered at the Tx MAC SAP for that flow during the simulation . | All results for SS1 and SS6 are in Appendix A, including per QoS flow PLR. All flows in all simulations (EDCA & HCCA) satisfy their PLR QoS objectives Number of QoS flows that satisfy their QoS objective, and the fraction of QoS flows that satisfy their QoS objectives: SS1EDCA simulation 1/2: 17 flows (100%) SS1: EDCA simulation 2/2: 17 flows (100%) SS1: HCCA simulation: 17 flows (100%) SS6 EDCA simulation: 39 flows (100%) SS6 HCCA simulation: 39 flows (100%) |
| CC20 (M) | BSS Aggregate Goodput at the MAC data SAP | Three aggregate goodput metrics are to be reported. Metric 1 is defined as the sum of goodput across all flows in the simulation scenario. Metric 2 is defined as the aggregate number of bits in MSDUs that are delivered at the Rx MAC SAP within the specified delay bound of the flow’s defined QoS, divided by the simulation duration. Metric 3 is defined as the sum of goodput across all flows that meet their QoS objective in the simulation scenario. Notes: Metric 1 includes flows that fail to meet their QoS objectives. Metric 2 excludes MSDUs that exceed the delay bound. Metric 3 excludes all MSDUs from flows that fail to meet their QoS objectives. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. | Detailed results for SS1 and SS6 are in Appendix A. All QoS objectives are met by all flows. SS1 Metrics 1 and 3: EDCA simulation 1/2: 58.88Mbps EDCA simulation 2/2: 61.56Mbps HCCA simulation: 71.042Mbps SS1 Metric 2: EDCA simulation 1/2: 58.88Mbps EDCA simulation 2/2: 61.56Mbps HCCA simulation: 71.039Mbps SS6 Metrics 1 and 3: EDCA simulation: 48.04Mbps HCCA simulation: 65.23Mbps SS6 Metrics 2: EDCA simulation: 48.029Mbps HCCA simulation: 65.18Mbps |
| CC24 (M) | MAC Efficiency | MAC efficiency is defined as the aggregate Metric 2 goodput in CC20 divided by the average physical layer data rate. Let r(n) denote the rate at which the nth successful Data MPDU is transmitted. Let t(n) denote the PPDU transmission time of the nth successfully transmitted Data MPDU (i.e. including preamble, PLCP header). Define the average PHY data rate r as r = ∑ r(n)t(n)/ ∑t(n) | Detailed results for SS1 and SS6 are in Appendix A. SS1 EDCA simulation 1/2: 55.6% SS1 EDCA simulation 2/2: 60.1% SS1 HCCA simulation: 75.7% SS6 EDCA simulation: 43.6% SS6 HCCA simulation: 60.85% |
| CC27 (M) | Throughput / Range | Report curves of Goodput (bps) vs. range (m). Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [4]: Model B Residential Model D Typical Office | All results for SS16 channel Model B and E are in Error: Reference source not found and 1.2.2. |
| CC28 (M) | Throughput / Range in 20MHz | Report curves of Goodput (bps) vs. range (m) , when operating in a 20 MHz bandwidth. Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [4]: Model B Residential Model D Typical Office | Because this proposal uses only 20 MHz channelization, the result for CC28 is identical to the result of CC27. |
| CC46 (M) | MAC Compatibility and parameters. | Provide a list of optional features of 802.11a/b/d/e/g/h/i/j that are required for HT operation, and a summary description of the manner in which they are used. Include references to the sections in the technical proposal document where the complete details are given. | In 802.11e, HCCA and EDCA are recommended as indicated within the standard, but not absolutely essential. The QoS control field from 802.11e is essential however. |
| CC47 (M) | MAC extensions | Provide a summary description of MAC extensions beyond 802.11a/b/d/e/g/h/i/j that are required for HT operation. Include references to the sections in the technical proposal document where the complete details are given. | Frame aggregation (Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found). Channel resource signaling for power save (Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found). |

## PHY Related

| Number | Name | Definition | Disclosure |
| --- | --- | --- | --- |
| CC51 (M) | Data rates | A list of PHY layer data rates, and for each data rate, specify the used modulation techniques, number of Tx antennas, coding rate and bandwidth. Specify which of the rates are mandatory and which are optional. For adaptive rate proposals, specify the range of achievable rates or, if possible, state the achievable rates in a closed form expression. | The proposed PHY modes are listed in Error: Reference source not found (mandatory modes) and Error: Reference source not found (optional modes). All modes occupy 20 MHz bandwidth. |
| CC42 (M) | Preambles | Specify the proposed preambles. Summarize the important properties of each part the proposed preambles. Include references to the sections in the technical proposal document where the complete details are given. Specify how the use of any new preamble affects reception by legacy STA. | This proposal uses two kinds of preambles; one is legacy PLCP preamble and another is MIMO preamble that is encapsulated after the legacy PLCP preamble. Therefore, legacy devices can recognize that it is IEEE802.11 based signal while they can not demodulate the received signal. It is just like that a STA that supports only mandatory PHY modes cannot receive 54Mbps signal. The proposed MIMO preamble is described in section Error: Reference source not found. The pilot preamble that is identical to the MIMO preamble or legacy long preamble can be inserted to re-estimate the channel in a burst. The pilot preamble is also described in section Error: Reference source not found. |
| CC51.5 (M) | Channelization | Specify the channelization – i.e. the adjacent channel spacing. | The channelization of our proposal is identical to that of IEEE802.11a. |
| CC52 (M) | Spectral Mask | Define the transmit spectral mask requirements for each channelization of the proposal. This must be under the same PA backoff used for performance simulations. | The spectral mask requirement is completely the same as IEEE802.11a. |
| CC58 (M) | HT Spectral Efficiency | The number of bps/Hz during the PSDU carrying a Data MPDU when demonstrating a goodput value of at least 100Mbps. Specify the phy data rate used during this test. | When 2 Tx chains MIMO and 64QAM R=7/8 are applied, the physical rate is 126 Mbps. Therefore, the spectral efficiency is 6.3 bits/s/Hz (=126Mbit/s /20MHz). |
| CC59 (M) | AWGN PER performance | Identify the performance in an idealized channel for a PSDU length of 1000B. The rows or columns of the channel shall be orthogonal to each other as follows: take the first Nr x Nt elements of the Fourier matrix with dimension max(Nr,Nt). Show PER versus SNR curves for 5 supported data rates representative of the proposal's rate set, including the maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Averaging should occur over a minimum of 100 packet errors down to 1% PER Refer to [7] for a definition of the Fourier matrix. Note: SNR is defined in section 2. | The simulation results for Nr=Nt=1 and Nr=Nt=2 are presented in section 1.3.1. Note that the mandatory requirement for the number of Tx chains is up to two in this proposal. So, the evaluated PHY modes are as follows; (1) Nr=Nt=2 - 64QAM R=7/8 (126Mbps) - 64QAM R=3/4 (108Mbps) - 64QAM R=2/3 (96Mbps) - 16QAM R=3/4 (72Mbps) - BPSK R=1/2 (12Mbps) (2) Nr=Nt=1 - 64QAM R=7/8 (63Mbps) - 64QAM R=3/4 (54Mbps) - 64QAM R=2/3 (48Mbps) - 16QAM R=3/4 (36Mbps) - 16QAM R=1/2 (24Mbps) - QPSK R=1/2 (12Mbps) - BPSK R=1/2 (6Mbps) |
| CC67 (M) | PER performance in non AWGN channels | Show either or both of the following two sets of performance curves: 1.) Show the PER curves for 5 supported data rates representative of your rate set including your maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Plot PER versus SNR averaged over time per receive antennas for PSDUs of length 1000B. Averaging should occur over a minimum of 100 MPDU errors down to 1% PER. Each packet should use an independent channel realization. 2.) Show curves for both achieved average physical layer data throughput and PER, as a function of total SNR for 1000B PSDUs. These results should be generated with a rate selection algorithm active. Data throughput is defined as the total number of bits successfully received in the data portion of the PPDU, divided by total transmission time, not including overheads such as preamble and backoff. The throughput shall be averaged over at least 100 independent realizations of the channel, each realization long enough to allow simulation of rate adaptation with subsequent transmission of one or more PPDUs while the Doppler process evolves. The same number of PPDUs shall be simulated for each channel realization. In addition a minimum number of PPDUs, equal to 100 divided by the target FER, shall be simulated to determine throughput. Total received signal power is summed over all transmit antennas. There shall not be any a priori knowledge of the channel or synchronization at the receiver or transmitter. This should be simulated for channel models B, D, and E. The simulations should all include the Doppler effect as specified in the text of the channel model document. All models should be run without the fluorescent effect but additionally model D should be run with the fluorescent effect on the highest data rate. The interferer to carrier energy ratio of the fluorescent effect shall be equal to its average value (0.0203). The shadowing variance should be 0. These simulations are performed using the NLOS version of the specified channel models. Note: SNR is defined in section 2. | The simulation results for Nr=Nt=1 and Nr=Nt=2 are presented in section 1.3.2. The evaluated PHY modes are as follows; (1) Nr=Nt=2 - 64QAM R=7/8 (126Mbps) - 64QAM R=3/4 (108Mbps) - 64QAM R=2/3 (96Mbps) - 16QAM R=3/4 (72Mbps) - BPSK R=1/2 (12Mbps) (2) Nr=Nt=1 - 64QAM R=7/8 (63Mbps) - 64QAM R=3/4 (54Mbps) - 64QAM R=2/3 (48Mbps) - 16QAM R=3/4 (36Mbps) - 16QAM R=1/2 (24Mbps) - QPSK R=1/2 (12Mbps) - BPSK R=1/2 (6Mbps) |
| CC67.2 (M) | Offset Compensation | Provide the impact on PER of carrier frequency offset and symbol clock offset by comparing to the PER achieved at the lowest average SNR that achieves a 10% PER for PSDUs of length 1000 bytes in channel E (NLOS) with no carrier and symbol clock offset. The symbol clock shall have the same relative frequency offset as the carrier frequency offset. Also, provide that same impact on PER using an SNR of 50dB in channel E (LOS). The carrier offset difference at the receiver relative to the transmitter shall be -40ppm and +40ppm. The results shall be presented in such a manner that it is clear whether there are specific values of offset for which the proposed system has better or worse performance relative to no offset. Simulations shall be performed under the same conditions as those in CC 59 & 67. | The simulation results for Nr=Nt=1 and Nr=Nt=2 are presented in section 1.3.3. The evaluated PHY modes are as follows; (1) Nr=Nt=2 - 64QAM R=7/8 (126Mbps) - 64QAM R=3/4 (108Mbps) - 64QAM R=2/3 (96Mbps) - 16QAM R=3/4 (72Mbps) - BPSK R=1/2 (12Mbps) |
| CC80 (M) | Required changes to 802.11 PHY | Give a summary description of changes to a legacy 802.11 PHY. Give references to sections in your specification that give the complete details. | The most of all PHY specifications except for following items are identical to IEEE802.11a specifications. - Adding Extended Signal, - Encapsulation of MIMO signal after legacy Signal, - Introducing a new PHY mode of 64QAM R=7/8, - Introducing pilot preamble insertion. |

# References

[1] IEEE 802.1-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)

[2] IEEE 802.11-1999 Std. 2003 Edition.

[3] Julian Dunn: “Sample clock jitter and real-time audio over the IEEE1394 high performance serial bus”, Nanophon Limited

[4] Measuring and Controlling Jitter in Digital Video Transmission Systems, A Tiernan White Paper, http://www.radynecomstream.com/doc/whitepaper.html

[5] A. Nishikawa, H. Fujii, H. Azuma, Y. Ohtani, N. Urano: “QoS control technology in Wireless AV Communication System”, Sharp Technical Journal, vol. 87, Dec. 2003.

Appendix A

Simulation parameters for SS1 and SS6 in both EDCA and HCCA as well as simulation results are listed here. Details are at flow level as well as aggregate flow level. Results for CC18, CC19, CC20, and CC24 are also shown. Clicking on the table opens up an Excel spreadsheet.