IEEE 802.11k
Wireless LANs

No Probe Channel Report information element

Date:	September 9th, 2004

Authors:	Floyd Simpson, Steve Emeott, Stephen Wang, Walter L. Johnson

Email: <floyd.simpson, steve.emeott, stephen.wang, walter.johnson>@motorola.com

Abstract

This paper proposes normative text changes needed to improve the roaming capability of a STA using a newly defined No Probe Channel Report element within IEEE P802.11k/D1.0. This element will be present within a beacon frame or probe response when dot11RadioMeasurementEnabled is true.

The proposal provides stations with a list of channels that they should not active probe. This will assist a STA in terms of time and power for discovering Neighbor AP roaming candidates.  A STA is allowed to actively probe channels not listed in the No Probe Channel Report element.

The following additions were required to IEEE P802.11k/D1.0 draft in order to make this proposal complete:

- Added No Probe Channel Report element as another information element transmitted in beacon and probe response frames.

- Added update to Table 20 which list information elements.

- Added new clause 7.3.2.27 which defines the No Probe Channel Report element.

- Updated the PICS table.

- Added new No Probe Channel Report Table called dot11NoProbeChannelReport Table to Annex D to store required information.

# Proposed Normative Text

The following are the proposed changes and instructions to the editor.

#### 7.2.3.1 Beacon frame format

Add order 20 field as shown to Table 5:

Table 5 – Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 20 | No Probe Channel Report | The No Probe Channel Report information element shall be present if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Add order 23 field and update order 23-n as shown in Table 12:

Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 23 | No Probe Channel Report | The No Probe Channel Report information element shall be present if dot11RadioMeasurementEnabled is true. |
| 2124-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

### 7.3.2 Information Elements

Insert Element ID TBD-TBD into Table 20 and change the reserved row accordingly:

Table 20 – Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | TBD |
| Neighbor Report | TBD |
| RCPI | TBD |
| No Probe Channel Report | TBD |
| Reserved | 51TBD-255 |

Add the No Probe Channel Report element as clause7.3.2.27 and number the figure appropriately for the TGk draft, renumbering as necessary.

# 7.3.2.27 No Probe Channel Report element

The No Probe Channel Report element contains a list of channels for which a STA is not allowed to active probe and for channels on which no information is available. The format of the No Probe Channel Report element is shown in Figure kXX.

|  | Element ID | Length | Channel Band | Channel List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure kXX—No Probe Channel Report element format

The Element ID field shall be equal to the No Probe Channel Report value in Table 20.

The Length field is dependent on the number of channels reported in the Channel List.

Channel Band shall contain an enumerated value from Table k1, specifying the frequency band in which the Channel List is valid. A No Probe Channel Report shall only report channels for a single frequency band. Multiple No Probe Channel Report elements may be used to report channels in more than one frequency band.

The Channel List shall contain a number of octets, where each octet describes a single channel number for which active probing is not allowed. Channel numbering shall be dependent on Channel Band according to Table k1.

The No Probe Channel Report contents shall be derived from dot11NoProbeChannelReportTable. A No Probe Channel Report shall only include channels that are valid for the regulatory domain in which the AP transmitting the element is operating and consistent with those specified by any Country element present.

Annex A

Insert this new item in A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM19 | No Probe Channel Report | 7.3.2.9, 7.3.2.27 | CF1:M | Yes, No, N/A |

# Annex D – ASN.1 encoding of the MAC and PHY MIB

Add the new dot11NoProbeChannelReport after the dot11APChannelListReport within AnnexD:

-- ********************************************************************

-- * dot11NoProbeChannelReport TABLE

-- ********************************************************************

dot11NoProbeChannelReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11NoProbeChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"No Probe Channel Report information, in tabular form."

::= { dot11RRMConfig 2 }

dot11NoProbeChannelReportEntry OBJECT-TYPE

SYNTAX Dot11NoProbeChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11NoProbeChannelReportTable. Each entry in the table is indexed by dot11NoProbeChannelReportIndex."

INDEX { dot11NoProbeChannelReportIndex }

::= { dot11NoProbeChannelReportTable 1 }

Dot11NoProbeChannelReportEntry ::=

SEQUENCE {

dot11NoProbeChannelReportIndex Unsigned32,

dot11NoProbeChannelReportIfIndex InterfaceIndex,

dot11NoProbeChannelReportChannelBand INTEGER,

dot11NoProbeChannelReportChannelList OCTET STRING }

dot11NoProbeChannelReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for No Probe Channel Report entry in dot11NoProbeChannelReportTable, greater than 0."

::= { dot11NoProbeChannelReportEntry 1 }

dot11NoProbeChannelReportIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex this row of No Probe Channel Report entry belongs to."

::= { dot11NoProbeChannelReportEntry 2 }

dot11NoProbeChannelReportChannelBand OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This corresponds to the frequency band for the channel list being reported as defined in 7.3.2.21.4."

::= { dot11NoProbeChannelReportEntry 3 }

dot11NoProbeChannelReportChannelList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (16))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute corresponds to the specific channels being reported. Each octet corresponds to a different channel within the specified Channel Band"

::= { dot11NoProbeChannelReportEntry 4 }

-- ********************************************************************

-- * End of dot11NoProbeChannelReportTable TABLE

-- ********************************************************************