IEEE P802.11
Wireless LANs

Neighbor Report Modifications to Include Regulatory Information for Active Scanning

Date:	September 13, 2004

Author:	A. Soomro 
	Philips
	345 Scarborough Road
	Phone: +1 914 945 6319
	Fax: +1 914 945 6580
	e-Mail: amjad.soomro@philips.com

Zhun Zhong
	e-Mail: zhun.zhong@gmail.com

Abstract

This submission proposes to include regulatory information about transmit power and radar detection measurement in Site Report. The knowledge of this information would enable STAs to perform active scanning in regulatory domains where requirements exist to acquire this information either from their own measurements or from measurement results transmitted by an access point. This submission proposes changes in draft normative text to add such information in Site Report.

Insert in the Neighbor Report Element subclause 7.3.2.26 as follows:

#### 7.3.2.26 Neighbor Report element

|  | Element ID | Length |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |  |  |  |  |
|  | BSSID | BSSID Match Status | Current Channel | Channel Band | PHY Options | Neighbor TBTT Offset (optional) | Beacon Interval (optional) |  |  |
| Octets: | 6 | 2 | 1 | 1 | 1 |  |  |  |  |
|  | Maximum Transmit Power Level | Local Power Constraint | Signal/Map Field |  |  | The element contains zero or more 8-tuples |  |  |  |
| Octets: | 1 | 2 | 1 |  |  |  |  |  |  |

Figure 2 –Neighbor Report element format

The Maximum Transmit Power Level field is a signed number and shall be 1 octet in length. It shall indicate the maximum power, in dBm, allowed to be transmitted.

The Local Power Constraint field shall be coded as defined in IEEE 802.1h amendment subclause 7.3.2.13.
[The Local Power Constraint field shall be set equal to a value that allows the mitigation requirements to be satisfied in the current channel. The field is coded as an unsigned integer in units of dB. The local maximum transmit power for a channel is thus defined as the Maximum Transmit Power Level specified for the channel in the Country element minus the Local Power Constraint specified for the channel (from the MIB) in the Power Constraint element].

The Map field is coded as a bit field, as defined in IEEE 802.11h amendment subclause 7.3.2.20.1

[The Map field is coded as a bit field, as shown in Figure 16, and shall contain a:

·  BSS bit, which shall be set equal to 1 when at least one valid MPDU was received in the channel during the measurement period from another BSS or IBSS. Otherwise, the BSS bit shall be set equal to 0.

·  OFDM Preamble bit, which shall be set equal to 1 when at least one sequence of short training symbols,as defined in 17.3.3, was detected in the channel during the measurement period without a subsequent valid Signal field. This may indicate the presence of an OFDM preamble, such as HIPERLAN/2. Otherwise, the OFDM Preamble bit shall be set equal to 0.

·  Unidentified Signal bit, which may be set equal to 1 when significant power is detected in the channel during the measurement period that cannot be characterized as radar, an OFDM preamble or a valid MPDU. Otherwise, the Unidentified Signal bit shall be set equal to 0. The definition of significant power is implementation dependent].

Insert the following MIB definitions in Annex D

Dot11RRMNeighborReportEntry ::=

SEQUENCE {

Dot11RRMNeighborReportIndex

dot11RRMNeighborReportIfIndex		ifIndex,

dot11RRMNeighborReportBSSID		 	MacAddress,

dot11RRMNeighborReportMatchStatus		Unsigned32,

dot11RRMNeighborReportCurentChannel	INTEGER,

dot11RRMNeighborReportPhyType		INTEGER,

dot11RRMNeighborReportMaximumTransmitPower INTEGER,

dot11RRMNeighborReportLocalPowerConstraint INTEGER,

dot11RRMNeighborReportMap		INTEGER,

dot11RRMNeighborReportRowStatus		RowStatus

}

dot11RRMNeighborReportRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains the row status of the Neighbor report, essentially used for indicating whether the row has all valid attributes filled in. Then set to active to be used in Neighbor report information elements. If any parameter is invalid, the SME shall set this attribute back to notReady. It is the responsibility of the manager to correct the parameters."

::= { dot11RRMNeighborReportEntry 10 }

Insert the following MIB definitions in Annex D

dot11RRMNeighborReportMaximumTransmitPower OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The maximum allowable transmit power in the channel of operation of the reporting STA."

::= { dot11RRMNeighborReportEntry 7 }

dot11RRMNeighborReportLocalPowerConstraint OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The value that allows the mitigation requirements to be satisfied in the current operating channel of the reporting STA."

::= { dot11RRMNeighborReportEntry 8 }

dot11RRMNeighborReportMap OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Map field indicating presence of radars and other users present in the current operating channel of the reporting STA."

::= { dot11RRMNeighborReportEntry 9 }