IEEE P802.11
Wireless LANs

| Proposed Normative Text for NAVs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2004-11-23 |  |  |  |  |
| Author(s): Mathilde Benveniste |  |  |  |  |
| Name | Company | Address | Phone | email |
| M. Benveniste | Avaya Labs | 233 Mt. Airy Road Basking Ridge, NJ USA | 973-761-6105 | benveniste@ieee.org |

Abstract

This document contains the material proposed to GTE for inclusion in the draft in the form of insertions into and replacements for material in IEEE Std 802.11-1999, as updated by IEEE Std 802.11a-1999, IEEE Std 802.11b-1999 (already present in TGE Draft 12).

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:" Added text appears in blue italics with an underline.  Deleted text appears in bold red with a double with a cross-through line.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Modify Clause 9.2.2.1 as follows:

9.9.2.2.1 NAV operation during a TXOP

HC shall set its own NAV to prevent its transmitting during a TXOP that it has granted to a QSTA through an HCCA poll. However, HC may reclaim the TXOP in the event of a QSTA not using it, or ending the TXOP early (see 9.9.2.1.3).

A QSTA may track two NAV values.  The NAV setting of a QSTA tracking two NAV values is the maximum of the NAV values for the QSTA; therefore, the QSTA can attempt to access the channel only if its NAV values are all 0. When a QSTA retains a new NAV value, that QSTA shall also save the BSSID from the frame that is setting the NAV value.   A QSTA may retain a subset consisting of the longest NAV values; the shorter are discarded. A NAV value (and the saved source address) is removed when the NAV value expires.

In a CFP or CP, if the HC has no more QSTAs to poll and it has no more data, management, BlockAckReq or BlockAck frames to send, it may reset the NAVs of all QSTAs in the QBSS that track two NAV values by sending a QoS CF-Poll with the RA matching its own MAC address and with the Duration/ID set to zero. When the QAP contains a PC, during the CFP, it may reset the NAVs of all STAs within the QBSS (i.e., corresponding to the same BSSID) by sending a CF-End frame, regardless of how the NAVs have been originally set.

A non-AP QSTA that tracks two NAV values and receives a CF-End frame containing the BSSID of the QBSS, in which the non-AP QSTA is associated, shall reset the NAV value corresponding to the saved BSSID that matches the AP’s MAC address to 0. A non-AP QSTA that tracks two NAV values and receives a QoS (+) CF-Poll with a MAC address in the Address 1 field that matches the HC’s MAC address and the Duration/ID value equal to zero shall reset the NAV value corresponding to the saved source address that matches the HC’s MAC address to zero.

When a QSTA receives a QoS (+)CF-Poll containing the BSSID of the QBSS in which the QSTA is associated, that QSTA shall update the NAV if necessary and shall save the TXOP holder address for that QBSS, which is the MAC address from the Address1 field of the frame containing the QoS (+)CF-Poll. If an RTS frame is received with the RA address matching the MAC address of the QSTA and the MAC address in the TA field in the RTS frame matches the saved TXOP holder address, then the QSTA shall send the CTS frame after SIFS, without regard for, and without resetting, its NAV. The saved TXOP holder address shall be cleared whenever the NAV is reset or when the NAV counts down to 0.

When a QSTA receives a frame addressed to it and requires an acknowledgement, it shall respond with an

ACK or QoS +CF-Ack independent of its NAV. A non-AP QSTA shall accept a polled TXOP by initiating

a frame exchange sequence independent of its NAV.