IEEE P802.11
Wireless LANs

Flexible Coding for 802.11n MIMO Systems Summary

Date:	November 5, 2004

Author:	Paul Gray
	TrellisWare Technologies, Inc.
	16516 Via Esprillo, Ste. 300, San Diego, CA, 92127, USA
	Phone: +1 858 753 1611
	Fax: +1 858 753 1640
	e-Mail: pgray@trellisware.com

Abstract

This is a summary of the key points of the partial proposal entitled “Flexible Coding for 802.11n MIMO Systems.”

# Flexible Coding for 802.11n MIMO Systems – 953r4

In the presentation “Flexible Coding for 802.11n MIMO Systems” (11-04-0953-04-000n-flexible-coding-802-11n-mimo-systems.ppt) TrellisWare Technologies made a partial proposal for the FEC to be used in 802.11n.

The code proposed for the 802.11n FEC is a Flexible Low Density Parity Check (F-LDPC) code.  This is a structured-LDPC code developed by TrellisWare Technologies that offers greater flexibility and lower memory requirements, while retaining the same performance advantages of the other structured-LDPC codes proposed.

The proposal was divided into the following sections:

- TrellisWare’s Flexible-Low Density Parity Check (F-LDPC) Codes

  - FEC Requirements for IEEE 802.11n

  - Introduction to F-LDPC Codes

  - F-LDPC Turbo/LDPC alternative interpretations

- Example Applications of F-LDPC Codes to the IEEE 802.11n PHY Layer

  - SVD-based MIMO-OFDM with Adaptive Rate Allocation

  - Open-loop Spatial Multiplexing MIMO-OFDM

    - MMSE Spatial Demultiplexing

- Conclusions

## F-LDPC Features

F-LDPC codes consist of the concatenation of very simple elements: a 2-state convulutional code, a single parity check code, and a 2-state accumulator, coupled with a flexible algorithmic interleaver.  This structure allows both a high degree of flexibility and low complexity decoder implementations.  Furthermore, this structure can be viewed as either a Turbo code or as a structured LDPC.  This means that there are many decoder architectures possible, and more opportunities to optimize decoder implementations.

The salient features of F-LDPC codes are:

- Unparalleled flexibility without complexity penalty

  - Input Block Sizes: 3 bytes to 1000 bytes in single byte increments, supporting variable length MAC PDUs on the fly

  - Code Rate: ½ to 32/33 with virtually any rate in between

- Uniformly good performance over these modes

  - ~< 1 db of SNR from random coding bounds (the best possible single point designs are 0.5 dB)

- Low complexity traits of LDPC codes

  - Similar edge complexity

  - Lower  memory requirements and simpler memory design and access

- Proven high-speed hardware implementation

  - 300 Mbps single FPGA prototype

  - F-LDPC code is simplification of TrellisWare’s FlexiCode ASIC design

  - Options for architectures associated with LDPC decoders and Turbo decoders

## F-LDPC Advantages

- Frame size flexibility

  - Packets from MAC can be segmented into codewords of any number of bytes

  - Packets can be only a few bytes in length

  - Byte-length granularity in packet sizes rather than OFDM symbol

- Code rate flexibility

  - Fine rate control allows efficient use of the available capacity

- Good performance

  - Can operate close to theory for finite block size and constellation constraint

- High Speed

  - Decoders can be constructed that can operate up to 300-500 Mbps

- Low Complexity

  - Achieves all of this without being excessively complex

- Proven Technology

  - Existing high-speed hardware implementations