IEEE P802.11
Wireless LANs

| AP Scanning Proposal Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2004-12-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Fujio Watanabe Moo Ryong Jeong Toshiro Kawahara | DoCoMo USA Labs | 181 Metro Drive #300, San Jose, CA 95110 | +1-408-451-4726 | {watanabe, jeong, kawahara}@docomolabs-usa.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).   The material in this document is subject to change in form and content after further study.  The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group.  If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document contains instructions to the editor to edit normative text concerning a fast active scanning. The fast active scanning can be effectively used to reduce channel scanning time.

### 9.7 Frame exchange sequences

Insert the following row into the Table 21:

Table 21 – Frame sequences

| Sequence | Frames in sequence | Usage |
| --- | --- | --- |
| fast probe request -- ACK | 2 | Defered probe response |

Insert the following text at the end of LEGEND:

24 – “fast probe request” represents a management frame of subtype Probe Request with an individual address in the Address 1 field.

### 10.3.2.1.2 Semantics of the service primitive

Change part of Table contents as follows:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BSSID | MAC address | Any valid individual or broadcast MAC address. Only individual MAC address is allowed if ScanType is Fast Active. | Specifies the desired BSSID or the broadcast BSSID. |
| ScanType | Enumeration | ACTIVE, PASSIVE, FAST ACTIVE | Indicates one of three scanning types: active scanning, passive scanning, and fast active scanning |
| ProbeDelay | Integer | N/A | Delay (in us) to be used prior to transmitting a Probe frame during active and fast active scanning |
| ChannelList | Ordered set of integers | Each channel will be selected from the valid channel range for the appropriate PHY and carrier set | Specifies a list of channels that are examined when scanning for a BSS. If ScanType is Fast Active, contains only a single channel, which is the current channel of the desired BSS. |

### 10.3.3.1.2 Semantics of the service primitive

Change a part of Table contents as follows:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ProbeDelay | Integer | N/A | Delay (in us) to be used prior to transmitting a Probe frame during active and fast active scanning |

### 10.3.10.1.2 Semantics of the service primitive

Change a part of Table contents as follows:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ProbeDelay | Integer | N/A | Delay (in us) to be used prior to transmitting a Probe frame during active and fast active scanning |

### 11.1.3 Acquiring synchronization, scanning

Change the first three paragraphs as follows:

A STA shall operate in one of three scanning modes: Passive Scanning mode, Active Scanning mode, and Fast Active Scanning mode depending on the current value of the ScanType parameter of the MLME-SCAN.request primitive.

Upon receipt of the MLME-SCAN.request primitive, a STA shall perform scanning. The SSID and BSSID parameters indicate the SSID and BSSID for which to scan, respectively.

To become a member of a particular ESS using passive scanning, a STA shall scan for Beacon frames containing that ESS’s SSID, returning all Beacon frames matching the desired SSID in the BSSDescriptionSet parameter of the corresponding MLME-SCAN.confirm primitive with the appropriate bits in the Capabilities Information field indicating whether the beacon came from an Infrastructure BSS or IBSS. To actively scan, the STA shall transmit Probe frames containing the desired SSID. To become a member of a particular BSS using fast active scanning, a STA shall transmit Probe frames containing the desired BSSID in Address 1 field and BSSID field. Upon completion of scanning, an MLME-SCAN.confirm is issued by the MLME indicating all of the BSS information received.

### 11.1.3.2.1 Sending a probe response

Change the first paragraph as follow:

STAs, subject to criteria below, receiving Probe Request frames shall respond with a probe response only if the SSID in the probe request is the broadcast SSID or matches the specific SSID of the STA. Probe Response frame shall be sent as a directed frame to the address of the STA that generated the probe request. If the Address 1 and BSSID in the probe request are same as the MAC address of the STA and the STA’s dot11RadioMeasurementEnabled is true, the STA shall  acknowledge the probe request and response with the corresponding probe response at a later time if the direct probe request is not a fast active. If the Address 1 is broadcast address or the STA’s dot11RadioMeasurementEnabled is false, the probe response shall be sent using normal frame transmission rules. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, the STA that generated the last beacon shall be the STA that responds to a probe request.

Insert the following three sections after Section 11.1.3.4 and renumber figures and tables as necessary:

### 11.1.3.5 Fast active scanning

Fast active scanning is a way for a STA to reduce channel-scanning time when the STA knows the BSSID and current channel of the AP to be scanned prior to the commencement of fast active scanning. The BSSID and current channel of an AP can be obtained from Site Report or scanning results of previous passive or active scanning.

The details of the fast active scanning procedures are as specified in the following subclauses.

### 11.1.3.5.1 Sending a probe response

STAs receiving Probe Request frames, with the Address 1 and BSSID in the probe request being same as the MAC address of the STA and the STA’s dot11RadioMeasurementEnabled is true,  shall respond with the corresponding acknoweldgement immediately after SIFS (See Figure 0-1)..

### 11.1.3.5.2 Fast active scanning procedure

Upon receipt of the MLME-SCAN.request with ScanType indicating a fast active scan, a STA shall use the following procedure:

For the current channel of the desired BSS to be scanned,

a) Wait until the ProbeDelay time has expired or a PHYRxStart.indication has been received;

b) Perform the Basic Access procedure as defined in 9.2.5.1;

c) Send a Probe Request frame with the desired BSSID in destination and BSSID filed, and SSID;

d) Clear and start a ProbeTimer at the PHY-TXEND.confirm;

e) If a PHY-RXSTART.indication does not occur before the Probe Timer reaches MinChannelTime, then proceed to step h), else when a PHY-RXSTART.indication does occur, the STA shall wait for the corresponding PHY-RXEND.indication to determine whether the received frame is an appropriate frame. The recognition of valid Probe Response frame or ACK frame sent by the recipient of the Probe Request frame, corresponding to this PHY-RXEND.indication, shall be interpreted as appropriate.

f) If a valid ACK frame was recognized in step e), the channel scanning is ended.

g) Clear NAV and issue an MLME-Scan.confirm with the BSSDescriptionSet containing the information gathered during the scan, if any.

Figure 0-  Probe response that is sent immediately after probe request in fast active scanning

Figure 0-  Probe response that is sent with DIFS after sending immediate ACK to probe request in fast active scanning

References: