IEEE P802.11
Wireless LANs

| TGT Terminology and Concepts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Steve Shellhammer | Intel | 13290 Evening Creek Drive San Diego, CA 92128 | (858) 391-4570 | shellhammer@ieee.org |
| Uriel Lemberger | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-5701 | uriel.lemberger@intel.com |
| Sasha Tolpin | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-5430 | alexander.tolpin@intel.com |
| Craig Warren | Intel | 13290 Evening Creek Drive San Diego, CA 92128 | (858) 375-7143 | craig.warren@intel.com |
| Neeraj Sharma | Intel | 13290 Evening Creek Drive San Diego, CA 92128 | (858) 385-4112 | neeraj.k.sharma@intel.com |
| Nir Alon | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-6621 | nir.alon@intel.com |

Abstract

This document introduces some Wireless Performance terminology and concepts.  The reason for writing this document is to attempt to clarify some terminology which can be used to discuss the objectives of 802.11 Task Group T.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Test Environments

Some measurements are made using conducted measurements and some are made over-the-air (OTA).  The over-the-air (OTA) measurements can be performed in a variety of environments.  Hence there are several OTA environments defined.

| Term | Description |
| --- | --- |
| Conducted (CON) | Tests that are performed using conducted measurements |
| Over-the-air (OTA) | Tests are performed over the air in one of a possible set of environments. Below are several specific OTA tests environments |
| Chamber | Tests that are performed over the air in a chamber environment to prevent interference from other systems |
| Indoor LOS | Tests that are performed over the air in an indoor environment where there is a line-of-site (LOS) between the AP and the client STA |
| Indoor NLOS | Tests that are performed over the air in an indoor environment where there is a not a line-of-site (NLOS) between the AP and the client STA |
| Outdoor | Tests that are performed over the air in an outdoor environment where there is a line-of-site (LOS) between the AP and the client STA |

# Primary and Secondary Performance Metrics

There are many possible performance metrics that can be considered.  There is value in classifying the metrics into one of two categories: primary and secondary metrics.  The reason for this is to attempt to minimize the number of metrics that need to be considered when evaluating wireless performance.  A primary metric is a metric that directly impacts the user experience.  A primary metric is therefore directly observable by the user.  A secondary metric does not directly impact the user experience.  A secondary metric is likely to indirectly affect the user experience, often by affecting a primary metric which in tern affects the user experience.

The distinction between primary and secondary metrics is a judgment call.  However, it is useful to classify performance metrics into these categories.  An example of a primary metric is throughput since the user can easily detect the affect of different levels of throughput.  An example of a secondary metric is receiver sensitivity.  The user cannot easily relate the receiver sensitivity to the performance of a user application, but it indirectly affects range and hence is observable through the range primary metric.

| Term | Description |
| --- | --- |
| Primary Metric | A metric that directly affect the user’s application performance. These metrics tend to be measured higher in the ISO stack and closer to the application layer. |
| Secondary Metric | A metric that does not directly affect the user’s application performance. These metrics tend to be measured lower in the ISO stack farter from the application layer. |

Both the primary and the secondary metrics are wireless performance metrics and not application layer metrics.  These primary and secondary metrics affect the application layer metrics.  Application layer metrics are outside the scope of TGT.

# Usage Models

The relevance of a given performance metric depends on the usage model that one is considering.  The following three usage models are defined. It is claimed that there three usage models are representative of the majority of usage models

| Term | Description |
| --- | --- |
| Data Usage Model | This model represents data transfer between an AP and a client. There are no strict QoS requirements other than a reasonable user experience in terms of not having to wait too long |
| Voice Usage Model | This model represents VoIP running on a WLAN. This usage model represents specific QoS requirements primarily in the area of low latency and packet loss. |
| Video Usage Model | This model represents video streaming running over the WLAN. This is not intended to model a Video Conference with two-way interactive video. It is intended to model video streaming for viewing of high quality video. This model has specific QoS requirements primarily in the area of throughput and packet loss |

# Canonical or Minimal Set of Primary Metrics

For each usage model the goal is to define a minimum set of primary metrics, call the canonical set that sufficiently represents the performance for that usage model.

| Term | Description |
| --- | --- |
| Canonical Set of Primary Metrics | This is the minimum set of primary metrics that represent the performance of a given usage model. |

# Correlation and Prediction

The primary metrics in the canonical set of metrics for each usage model should have a strong correlation with some of the secondary metrics.  For example, there is a high correlation between the receiver sensitivity (secondary metric) and range (primary metric).  In this context correlation is the mathematical correlation between two random variables.  Given that there is a correlation between primary and secondary metrics it is possible to make a prediction of a primary metric from several of the secondary metrics along with some other parameters, like environmental variables.

| Term | Description |
| --- | --- |
| Correlation | The mathematical correlation between a primary metric and a secondary metric |
| Prediction | The process of predicting the value of a primary metric from one or more secondary metrics |

# Repeatability in Time and Location

It is important to separate two aspects of repeatability: repeatability in time and repeatability in location.  The test environment may affect the repeatability of the tests.

| Term | Description |
| --- | --- |
| Repeatable in Time | A test is repeatable in time if it can be repeated in the future time and the results are same as in the previous test, to within the specified accuracy of the test |
| Repeatable in Location | A test is repeatable in location if it can be repeated in a different location and the results are the same as in the first location, to within the specified accuracy of the test |

Ideally we would like all tests to be repeatable in both time and location to a high level of accuracy.  However, it is also important to include test environments that are representative of the user environment.

Conducted test are repeatable in both time and location.  So the results of a conducted test can be repeated in a different laboratory test in a subsequent experiment.

Over-the-air test are repeatable it time but typically not repeatable in location.  In other words, an experiment can be repeated in the same facility and the results will be repeatable.  However, since it is difficult to replicate the exact test environment in a different facility the over-the-air tests are not likely to be repeatability in location.

None the less it is important to include OTA tests since many of the primary tests are likely to be OTA tests.

# Examples of Metrics

As was mentioned previously, for each usage model the task group needs to define the canonical set of primary metrics.  Section 7.1 gives some examples of potential primary metrics associated with each of the proposed usage models.  Some examples of secondary metrics are giving in Section 7.2.  Finally, the correlation between the primary and secondary metrics is illustrated in Section 7.3.

## Primary Metrics

This section gives some examples of primary metrics.  Table 1 gives a list of primary metrics as well as which usage models each primary metric applies to.

| Metric | Use Cases |  |  |  | comments |
| --- | --- | --- | --- | --- | --- |
|  | DATA | VOICE | VIDEO | GENERAL |  |
| TPT & Range | + |  | + |  |  |
| FLR - Frame Lost Rate- FLR (Transmitted - Delivered)/Delivered packets (%of retries, %of TX Failures) |  | + | + |  |  |
| Latency (delay) - min/max/average time it takes for a packet to cross a network connection, from sender to receiver over the MAC layer. |  | + |  |  |  |
| Jitter (a variance of latency/delay) |  | + | + |  |  |
| Video quality (excellent, good, fair, poor, bad) |  |  | + |  | Application layer |
| Voice quality (excellent, good, fair, poor, bad) |  | + |  |  | Application layer |
| Number of concurrent flows failing to meet QoS objectives/ TSPECs |  | + | + |  | Need to clarify |
| Ratio of amounts of data in each AC for EDCA |  | + | + |  | Need to clarify |
| Few NICs coexistence/co-working/ WL media sharing (BSS) |  |  |  | + | infrastructure |
| Noise tolerance (Adjacent/Alternant Channel rejection, CW) | + | + | + |  |  |
| Power Consumption for TX, RX, Idle Associated, Idle Non-Associated, disabled, off, RF-kill, WoWLAN etc. |  |  |  | + | platform |

Table 1: Examples of Primary Metrics

## Secondary Metrics

This section includes some examples of secondary metrics. Table 2 gives examples of secondary metrics.

| Metric | Use Cases |  |  |  | comments |
| --- | --- | --- | --- | --- | --- |
|  | DATA | VOICE | VIDEO | GENERAL |  |
| Receiver Sensitivity | + | + | + |  |  |
| TX Power | + | + | + |  |  |
| TX EVM | + |  | + |  |  |
| RX EVM | + |  | + |  |  |
| RX PER (Packet Error Rate for PHY) | + | + | + |  |  |
| Antenna Diversity | + | + | + |  |  |
| Auto detect ability (OFDM, CCK, 11n) - % of time correctly detected | + | + | + |  |  |
| Client QoS queue latency – min/max/average time from the frame is queued till the frame is sent to the air |  |  | + |  |  |
| Client QoS queue jitter – variance of Client QoS queue latency |  | + | + |  |  |

Table 2: Examples of Secondary Metrics

## Correlation between Primary and Secondary Metrics

Some of the secondary metrics have a strong affect on the primary metrics and some have limited affect.  Table 3 gives the correlation between the example primary and secondary metrics.  The correlation is specified as high (H), medium (M) or low (L).

| Correlation (High, Medium, Low) | Receiver Sensitivity | TX Power | TX EVM | RX EVM | RX PER | Antenna Diversity | Client QoS queue latency | Client QoS queue jitter | Auto detect ability (OFDM, CCK, 11n) | Frame lost rate due to ACK failure | Frame lost rate due to RX failure |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TPT vs. ATT | H | H | M | M | M |  |  |  | H | M | M |
| TPT vs. Range | H | H | M | M | M |  |  |  | H | M | M |
| TPT NLOS | H | H | M | M | M | H |  |  | H | M | M |
| Adjacent channel rejection & Alternate and Far channel rejection, in-band interference | L | L | L | L | L |  |  |  |  |  |  |
| Noise tolerance (Adjacent Channel rejection, CW, in-band interference) | L | L | L | L | L | M |  |  |  |  |  |
| FLR- Frame Lost Rate (Transmitted -Delivered)/Delivered packets (% of Retries, % of TX Failures) | H | H | H | H | H | M |  |  |  | H | H |
| Few NICs coexistence/co-working/ WL media sharing (BSS) | H | H | L | L | H | H |  |  | M |  |  |
| Number of flows failing to meet QoS objectives |  |  |  |  |  |  | H | H |  |  |  |
| Ratio of amounts of data in each AC for EDCA |  |  |  |  |  |  | H | H |  |  |  |
| System/Application latency (Delay) - min/max/average time it takes for a packet to cross a network connection, from sender to receiver over the MAC layer. |  |  |  |  |  |  | H | H |  |  |  |
| Jitter ( variation of Delay) |  |  |  |  |  |  | H | H |  |  |  |
| Video quality | M | M | M | M | M |  | L | H |  | H | H |
| Voice quality | L | L |  |  | M | M | H | H |  | H | H |

Table 3: Correlation of Primary and Secondary Metrics