IEEE P802.11
Wireless LANs

| Neighbour Report MLME Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2004-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Simon Black | Nokia |  | +44 1353 648567 | simon.black@ intalk2k.com |

Abstract

This paper proposes a set of MLME primitives for the neighbor report request-response protocol. It addresses comments 1, 2, 3 and 4 from LB71 on IEEE802.11k/D1.0 (comment numbering from 04/0964). The information in this document is based on that previously proposed in documents 03/0580, 04/0400 and 04/0521. Section numbering for new sections is based on approved amendments.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Insert the following new subsections following 10.3.23

### 10.3.24 Neighbor Report

The following MLME primitives support the signaling of Neighbor Reports.

#### 10.3.24.1 MLME-NEIGHBORREP.request

#### 10.3.24.1.1 Function

This primitive requests that a Neighbor Report Request frame be sent to the AP with which the STA is associated. It is valid only at a Radio Measurement capable non-AP STA.

#### 10.3.24.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREP.request	(
DialogToken,
NeighborReportRequestType,
NeighborReportFailureTimeout
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the neighbor report transaction. |
| NeighborReport RequestType | Enumeration | BASIC, TBTT_OFFSET | Set to TBTT_OFFSET to request that neighbor list entries contain TSF information where available. |
| NeighborReport FailureTimeout | Integer | ≥ 1 | Specifies a time limit (in TU) after which the neighbor report procedure will be terminated. |

#### 10.3.24.1.3 When Generated

This primitive is generated by the SME to request that a Neighbor Report Request frame be sent to the AP with which the STA is associated to request a Neighbor Report.

#### 10.3.24.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Neighbor Report Request Action management frame. The STA then attempts to transmit this to the AP with which it is associated.

#### 10.3.24.2 MLME-NEIGHBORREP.confirm

#### 10.3.24.2.1 Function

This primitive reports the result of a neighbor report request. It is valid only at a Radio Measurement capable non-AP STA.

#### 10.3.24.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREP.confirm	(
ResultCode,
DialogToken,
BSSInformationSet
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, REFUSED, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-NEIGHBORREP.request. |
| DialogToken | Integer | As defined in the corresponding MLME-NEIGHBORREP. request | The Dialog Token to identify the neighbor report transaction. |
| NeighborListSet | Set of Neighbor List Entries each as defined in the Neighbor Report element format | Set of Neighbor List Entries each as defined in the Neighbor Report element format | A set of Neighbor List Entries each representing a neighboring AP being reported as defined in the Neighbor Report element format. Only present if ResultCode = SUCCESS |

#### 10.3.24.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-NEIGHBORREP.request and indicates the results of the request.

This primitive is generated when the MLME-NEIGHBORREP.request contains invalid parameters, when a timeout occurs, or when the STA receives a Neighbor Report Response frame from the AP.

#### 10.3.24.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode and may use the reported data.

#### 10.3.24.3 MLME-NEIGHBORREP.indication

#### 10.3.24.3.1 Function

This primitive indicates that a Neighbor Report Request frame was received from a non-AP STA. It is valid only at a Radio Measurement capable AP.

#### 10.3.24.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREP.indication	(
PeerSTAAddress,
DialogToken,
NeighborReportRequestType
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Neighbor Report Request frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token in the Neighbor Report Request frame that was received. |
| NeighborReport RequestType | Enumeration | BASIC, TBTT_OFFSET | The value TBTT_OFFSET requests that neighbor list entries contain TSF information where available. |

#### 10.3.24.3.3 When Generated

This primitive is generated by the MLME when a valid Neighbor Report Request frame is received.

#### 10.3.24.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.8.2.

#### 10.3.24.1 MLME-NEIGHBORREP.response

#### 10.3.24.1.1 Function

This primitive is generated in response to an MLME-NEIGHBORREP.indication requesting a neighbor report be sent to a nonAP STA. It is valid only at a Radio Measurement capable AP.

#### 10.3.24.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREP.response	(
PeerSTAAddress,
ResultCode,
DialogToken,
NeighborListSet
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Neighbor Report Request frame was received. |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, REFUSED, UNSPECIFIED_ FAILURE | Indicates the results of the corresponding MLME-NEIGHBORREP.indication |
| DialogToken | Integer | As defined in the corresponding MLME-NEIGHBORREP. indication | The Dialog Token to identify the neighbor report transaction. |
| NeighborListSet | Set of Neighbor List Entries each as defined in the Neighbor Report element format | Set of Neighbor List Entries each as defined in the Neighbor Report element format | A set of Neighbor List Entries each representing a neighboring AP being reported as defined in the Neighbor Report element format. Only present if ResultCode = SUCCESS. |

#### 10.3.24.1.3 When Generated

This primitive is generated by the SME in response to an MLME-NEIGHBORREP.indication requesting a neighbor report be sent to a non-AP STA. If the returned ResultCode is SUCCESS, the primitive contains the neighbor report entries to be sent to the STA.

#### 10.3.24.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Neighbor Report Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the Non-AP MAC Address parameter.